3GPP TSG RAN WG1 #104-bis-e					    			    R1-2103203

e-Meeting, April 12th – 20th, 2021

Agenda Item:	8.13.3

Source:	InterDigital, Inc.

Title:	Fast SCell Activation

Document for:	Discussion and Decision

# Introduction

The work item on dynamic spectrum sharing (DSS) between LTE and NR aims to enable scheduling enhancements, including fast Scell activation and SCell scheduling PDSCH/PUSCH on PCell/PSCell. In RAN1#103e and 104e meetings, the following issues were discussed for fast activation:

- Issue-1: Triggering command for SCell activation/de-activation and temporary RS

- Issue-2: TRS structure for temporary RS

- Issue-3: QCL configuration of TRS

- Issue-4: Time-domain property of TRS

- Issue-5: Additional Functionality of temporary RS during the SCell activation

- Issue-6: Timeline for temporary RS

For efficient SCell activation, RAN1 to down select from:

- Option 1a: MAC CE(s) contained in a single PDSCH to trigger both SCell activation and corresponding temporary RS(s)

Details FFS including timeline design for receiving temporary RS

- Option 1b: A single DCI to trigger both SCell activation and corresponding temporary RS(s)

Details FFS including potential impact on SCell activation related procedures and, e.g. timeline design for SCell activation and for receiving temporary RS

FFS: The same DCI for SCell deactivation

- Option 2: A Rel-15/16 SCell activation MAC-CE to trigger SCell activation and a Rel-15/16 DCI to trigger corresponding temporary RS(s) with enhancement of timeline

Details FFS including timeline design for receiving a DCI trigger of temporary RS, and for receiving temporary RS

This contribution discusses remaining open issues to enable fast activation in NR DSS.

# Discussion

The time required to activate an SCell can be summarized as the sum of THARQ, Tactivation_time, and TCSI_Reporting. As illustrated in Figure 1.

Figure 1: breakdown of SCell activation time.

Tactivation_time:

To enable fast activation, automatic gain control (AGC) has to settle before a symbol can be sampled correctly. That means, the optimal point for the dynamic range in the receiver’s amplifier is set after a reading of the energy in the channel and some pilot RS (e.g. temporary RS) for correct setting. The number of temporary RS (and importance of it) is more profound for inter-band cell activation. To reduce Tactivation_time, it was agreed in RAN1#103e meeting to introduce TRS.

THARQ:

Given the activation command is delivered via PDSCH on a MAC CE, this time is associated with the HARQ RTT to successfully receive the TB containing the activation command MAC CE. The following option were identified in RAN1#103e to reduce THARQ:

- MAC CE (triggering only for SCell activation)

- MAC CE (triggering for both SCell activation and temporary RS)

- DCI (triggering only for SCell activation)

- DCI (triggering for both SCell activation and temporary RS)

Activation via DCI only reduces the time needed for activation, as the HARQ RTT time is reduced, but suffers from potentially increased DCI signaling overhead and the possibility of DCI misdetection. On the other hand, MAC CE requires less overhead compared to DCI, and is already used for cell activation. Further, the number of TRS required can be more than 1, and this would be too much overhead for DCI to carry.

Observation 1:  SCell activation by DCI (option 1b) increases DCI overhead, is prone to DCI misdetection, and requires re-designing the DCI format.

It is therefore safer to keep activation by MAC CE, and the TRS can also be contained in such MAC CE to reduce the activation time. A single PDSCH TB containing one MAC-CE can contain triggers for both Scell activation command and temporary RS. Alternatively, the two triggers can be delivered on different MAC CEs. This is inline with option 1a from the agreement last meeting.

Proposal 1:  For efficient SCell activation, the trigger of temporary RS is integrated into a single triggering signaling with the trigger of SCell activation transmitted on an activated cell (option 1a), by either:

- A single PDSCH TB containing two respective MAC-CEs for both triggers, one MAC-CE for both triggers; or

- A single PDSCH TB and its scheduling DL grant, e.g. MAC-CE for activation and DL assignment for temporary RS

One potential drawback of option 1a is that the UE must first decode the PDSCH prior to measuring the TRS. To reduce Tactivation_time even further, option 2 can be considered. With option 2, the SCell is still activated by MAC CE and thus there is no misdetection issues, but the preparation timer for activating the cell can happen faster on a separate DCI and potentially before correctly decoding the activation MAC CE, especially if the PDSCH needs to be retransmitted. In case the total SCell activation time needs to be reduced further, option 2 can be considered.

For the TRS periodicity, the following options where identified: TRS can be either aperiodic, periodic, or semi-persistent. Given TRS is coupled with the activation command, which is aperiodic by nature, aperiodic TRS should be supported.

Proposal 2:  Support aperiodic TRS allocation.

Further, the following options were identified for the TRS timeline in the feature lead summary [2]:

- Opt 4.1

“The TRS is triggered r slots after the UE sends HARQ-ACK to the triggering MAC CE, plus 0.5ms MAC-to-PHY processing delay, where r is configured by RRC or indicated by MAC CE.”

- Opt 4.2

“Offset between Scell activation and temporary RS can be configured by RRC singling and starting point of the offset is the HARQ-ACK feedback slot of triggering command”

- Opt 4.3

“The actual slot for the triggered TRS can be r slot after the slot the UE sends HARQ-ACK for the PDSCH converting TRS triggering MAC CE, where the r can be configured by RRC, or more flexibly, indicated by the MAC CE.”

- Opt 4.4

“The timing of A-TRS transmission is defined relative to the PUCCH transmission that carries the HARQ-ACK for triggering command, and the offset value of TRS transmission is indicated in triggering command.”

If option 1a is agreed, option 4.3 is suitable, as the TRS timeline can be configured per UE from the time a successful HARQ-ACK is received for the TB containing the activation command and the TRS. This time can different from one UE to another, i.e. to accommodate for the L1/L2 processing times and bandwidth switching times, which can be capability dependent.

Proposal 3:  For the TRS timeline, support option 4.3:

“The actual slot for the triggered TRS can be r slot after the slot the UE sends HARQ-ACK for the PDSCH converting TRS triggering MAC CE, where the r can be configured by RRC, or more flexibly, indicated by the MAC CE.”

# Conclusion

In this contribution we discuss the different alternatives and means to support fast activation. The following is proposed:

Observation 1:  SCell activation by DCI (option 1b) increases DCI overhead, is prone to DCI misdetection, and requires re-designing the DCI format.

Proposal 1:  For efficient SCell activation, the trigger of temporary RS is integrated into a single triggering signaling with the trigger of SCell activation transmitted on an activated cell (option 1a), by either:

- A single PDSCH TB containing two respective MAC-CEs for both triggers, one MAC-CE for both triggers; or

- A single PDSCH TB and its scheduling DL grant, e.g. MAC-CE for activation and DL assignment for temporary RS

Proposal 2:  Support aperiodic TRS allocation.

Proposal 3:  For the TRS timeline, support option 4.3:

“The actual slot for the triggered TRS can be r slot after the slot the UE sends HARQ-ACK for the PDSCH converting TRS triggering MAC CE, where the r can be configured by RRC, or more flexibly, indicated by the MAC CE.”

# References

- RP-192678, “New WID on NR Dynamic spectrum sharing (DSS)”, Ericsson

- R1-2101933, “Summary#2 of efficient SCell activation/de-activation mechanism of NR CA”, Moderator (Huawei)

- R1-200xxxx, “Summary#1 of efficient SCell activation/de-activation mechanism of NR CA”, Moderator (Huawei)

# Appendix: Relevant Agreements

Agreements from RAN1#104e meeting:

Agreement

For efficient activation of SCells, down select at least one option from below:

- Option 1a: MAC CE(s) contained in a single PDSCH to trigger both SCell activation and corresponding temporary RS(s)

  - Details FFS including timeline design for receiving temporary RS

- Option 1b: A single DCI to trigger both SCell activation and corresponding temporary RS(s)

  - Details FFS including potential impact on SCell activation related procedures and, e.g. timeline design for SCell activation and for receiving temporary RS

  - FFS: The same DCI for SCell deactivation

- Option 2: A Rel-15/16 SCell activation MAC-CE to trigger SCell activation and a Rel-15/16 DCI to trigger corresponding temporary RS(s) with enhancement of timeline

  - Details FFS including timeline design for receiving a DCI trigger of temporary RS, and for receiving temporary RS

- Note: Companies are encouraged to provide complete solutions for fast SCell activation.

- Note: the previous agreement on the definitions of Alt 1 and Alt 2 is still effective

Working Assumption

For efficient SCell activation with assistance of temporary RS, a SSB of the to-be-activated SCell can be indicated as a QCL source for the temporary RS in case of known SCell

- FFS: QCL type

- FFS: the case of unknown SCell

- FFS: other QCL source, e.g. the SSB/P-TRS of another active cell

Agreements from RAN1#103e meeting:

Agreements:

As working assumption, with respect to efficient SCell activation, reuse existing Rel-15/16 TRS structure for temporary RS

- FFS: how many burst/symbols are required for both AGC settling and Time/Frequency tracking for different cases, e.g. FR1 and FR2, known and unknown SCell

  - A burst of temporary RS is notated as in S5.1.6.1.1 of TS 38.214

    - “2-slot with four CSI-RSs resources (4 samples)” for FR1

    - either “1-slot with two CSI-RSs resources (2 samples)” or “2-slot with four CSI-RSs resources (4 samples)” for FR2

- The working assumption can be confirmed after RAN4 check. (A LS for such request is planned).

Agreements:

For efficient SCell activation, discuss and agree from the following alternatives at RAN1#104-e

- Alt 1: the trigger of temporary RS is integrated into a single triggering signaling with the trigger of SCell activation transmitted on an activated cell.

  - FFS detailed design of this integrated triggering signaling.

  - Potential examples of single triggering signaling for further discussions

  - A PDSCH TB, e.g. containing two respective MAC-CEs for both triggers, one MAC-CE for both triggers

  - A DCI for both triggers

  - A PDSCH TB and its scheduling DL grant, e.g. MAC-CE for activation and DL grant for temporary RS

  - A DL grant and a UL grant received in the same slot/OFDM symbols of PDCCH where the DL grant is scheduling a MAC-CE for SCell activation and the UL grant is triggering the RS.

  - Rel-15/16 SCell activation MAC-CE and a specific configuration of temporary RS being implicitly triggered as well

- Alt2: Triggering of temporary RS separately from SCell activation command is not precluded and both ‘separate’ triggers (examples below) and ‘integrated’ triggers (examples in Alt 1) are considered for SCell activation

  - FFS detailed design of separate triggering signaling.

  - Potential examples of separate triggering signaling for further discussions

  - Rel-15/16 SCell activation MAC-CE and Rel 15/16 DCI triggering

  - Rel-15/16 SCell activation MAC-CE and new DCI triggering for temporary RS

- Note: temporary RS should be triggered by DCI or MAC-CE.

- Note: the final mechanism of trigger signaling targets at applicability to one or more SCell activation.

- FFS handling of  SCell activation by existing Rel15/16 CA activation command when temporary RS is configured and triggered/not triggered

Working Assumption:

At least for the case of known cell, temporary RS is supported to expedite the activation process during the SCell activation procedure for efficient SCell activation for both FR1 and FR2:

- The temporary RS should provide at least the functionalities of AGC settling and time/frequency tracking during SCell activation procedure.

- FFS potential functionalities of CSI measurement/acquisition and cell search

Agreements:

TRS is selected as temporary RS for Scell activation

- If more functionalities are confirmed to be supported by temporary RS, other RS candidates, e.g. aperiodic CSI-RS, P/SP-CSI RS, SRS and RS based on SSS/PSS, are not precluded.

- The TRS should be triggered by DCI or MAC-CE. FFS which exact triggering command.

Agreements:

UEs measure the triggered temporary RS during Scell activation procedure no earlier than a slot m:

- FFS timeline values m which may need coordination with RAN4.

- FFS If the triggered temporary RS can be associated with a BWP, then the measurement above is independent of the activation state of the BWP.

Agreements:

Companies are encouraged to provide design details of temporary RS next meeting, at least including:

- TRS structure, e.g. whether to fully reuse existing Rel-15/16 TRS structure and configuration restriction (refer to S5.1.6.1.1 of TS 38.214), or any modification

- QCL information, if any

- Triggering command: DCI format/fields or MAC-CE fields

- Triggering timeline/scheduling offset