3GPP TSG RAN WG1 #104b-e			R1-2103225

e-Meeting, April 12th – 20th, 2021

Agenda item:	8.1.2.4

Source: 	Samsung

Title: 	Enhancements on HST-SFN

Document for:	Discussion and Decision

Introduction

Until RAN1#104-e meeting, the following agreements were made for HST-SFN related enhancements [1], [2], [3]:

Agreement For the discussion purpose consider the following categorization of the enhanced DL transmission schemes - Scheme 1: - TRS is transmitted in TRP-specific / non-SFN manner - DM-RS and PDCCH/PDSCH from TRPs are transmitted in SFN manner - Scheme 2: - TRS and DM-RS are transmitted in TRP-specific / non-SFN manner - PDSCH from TRPs is transmitted in SFN manner Agreement For discussion purpose consider the following three steps for TRP-based frequency offset pre-compensation scheme: - 1st step: Transmission of the TRS resource(s) from TRP(s) without pre-compensation - 2nd step: Transmission of the uplink signal(s)/channel(s) with carrier frequency determined based on the received TRS signals in the 1st step - 3rd step: Transmission of the PDCCH/PDSCH from TRP(s) with frequency offset pre-compensation determined based on the received signal/channel in the 2nd step - Note: A second set of TRS resource(s) may be transmitted at 3rd step. Agreement Study TRP-based frequency offset pre-compensation including the following aspects: - Aspects related to indication of the carrier frequency determined based on the received TRS resource(s) in the 1st step - Option 1: Implicit indication of the Doppler shift(s) using uplink signal(s) transmitted on the carrier frequency acquired in the 1st step - Indication for QCL-like association of the resource(s) received in the 1st step with UL signal transmitted in the 2nd step - Type of the uplink reference signals / physical channel used in the 2nd step, necessity of new configuration and corresponding signaling details - Option 2: Explicit reporting of the Doppler shift(s) acquired in the 1st step using CSI framework - FFS: Indication for QCL-like association of the resource(s) received in the 1st step with UL signal transmitted in the 2nd step - CSI reporting aspects, configuration, quantization, signalling details, etc. - New QCL types/assumption for TRS with other RS (e.g., SS/PBCH), when TRS resource(s) is used as target RS in TCI state - New QCL types/assumptions for TRS with other RS (e.g., DM-RS), when TRS resource(s) is used as source RS in the TCI state - Target physical channels (e.g., PDSCH only or PDSCH/PDCCH) and reference signals that should be supported for pre-compensation - Signaling/procedural details on whether/how the pre-compensation is applied to target channels - Whether multiple sets of TRS and pre-compensation on TRS is needed in 3rd step. - Note: Other aspects/schemes are not precluded

Agreement When the same DMRS port(s) are associated with two TCI states containing TRS as source reference signal, at least one variant is supported for Rel-17 HST-SFN scenario based on further evaluations - Variant A: One of the TCI state can be associated with {average delay, delay spread} and another TCI states can be associated with {average delay, delay spread, Doppler shift, Doppler spread} (i.e., QCL-TypeA) - Variant B: One of the TCI state can be associated with {average delay, delay spread} and another TCI state with {Doppler shift, Doppler spread} (i.e., QCL-TypeB) - Variant C: One of the TCI state can be associated with {delay spread} and another TCI states can be associated with {average delay, delay spread, Doppler shift, Doppler spread} (i.e., QCL-TypeA) - Variant E: Both TCI states can be associated with {average delay, delay spread, Doppler shift, Doppler spread} (i.e., QCL-TypeA) - FFS: Indication method to apply QCL, e.g., via new QCL-type, or reuse existing QCL-type while UE to ignore certain QCL properties - Note: Each TCI state in the above variants may be additionally associated with {Spatial Rx parameter} (i.e., QCL-TypeD) - Note: Companies are encouraged to provide evaluation results for the above variants based on agreed EVM from RAN1#102e meeting - Note: Above variants are applicable to scheme 1 and/or TRP based pre-compensation as a reference for evaluation. - This agreement is for the purpose of evaluation and does not imply the support or lack of support of scheme 1 and/or TRP based pre-compensation Agreement Scheme 1 is supported in Rel-17 - TRS is transmitted in TRP-specific / non-SFN manner - DM-RS and PDCCH/PDSCH from TRPs are transmitted in SFN manner - FFS other details Agreement For scheme 1 and SFN transmission of PDCCH support Variant E for QCL assumption in TCI state when TRS is used as source RS Agreement Two TCI states are supported for scheme 1 in FR2 Agreement - Support MAC CE activation of two TCI states for PDCCH - FFS other details Agreement For HST-SFN scenario: - Support semi-static (RRC based) switching of scheme 1 (PDSCH) with 2a, 2b, 3, 4 - FFS all other details including RRC signaling, possible RAN4 impact (if any), etc.

This contribution provides Samsung’s view on the topics above.

UE-based enhancements for HST-SFN

A typical deployment scenario for HST system is to place multiple RRHs which are connected to a BBU and distributed along the railway. In such scenario, the NW can transmit DL data to an HST UE via SFN manner due to the difficulty to track accurate position of the UE. This type of transmission provides more reliable transmission than using one nearest RRH. In addition, received signal power at the UE side can be boosted via SFN transmission.

To reap the benefit of SFN transmission via HST, the set of participating RRHs in the transmission should be selected carefully. For example, the following can be considered:

i) Transmission delay of each RRH-UE link: It is beneficial to form the set of RRHs such that their aggregate SFN delay spread to UE is within a CP, otherwise UE would experience ISI.

ii) UE complexity: UE experiences multiple Doppler shifts, each coming from an RRH participating in the SFN transmission. As the number of RRHs increases, UE processing complexity also increases to track increased number of Doppler shifts.

As a result, NW firstly forms multiple SFN combinations, and then chooses an appropriate combination for transmission depending on the position and processing capability of UE. Figure 1 shows an example of transmission scenario when 4 RRHs are in the network and located equi-distantly with D. In this example, it is assumed that each RRH uses bi-directional beams, each with separate TRS and up to 4 adjacent RRHs (that is RRHs within 2D distance from a UE) participate in SFN transmission. In this set-up, the list of all possible SFN combinations can be found in Table 1.

Figure 1. Example of transmission scenario in bi-directional 4-RRH network

Table 1. List of possible SFN combinations in Figure 1

| # of RRHs for SFN | List of SFN combinations | # of combinations |
| --- | --- | --- |
| Single RRH (DPS) | #0L, #0R, #1L, #1R, #2L, #2R, #3L, #3R | 8 |
| Two RRHs | {#0L,#1L}, {#0R,#1L}, {#0R,#1R}, {#1L,#2L}, {#1R,#2L}, {#1R,#2R}, {#2L,#3L}, {#2R,#3L}, {#2R,#3R} | 9 |
| Three RRHs | {#0R,#1L,#2L}, {#0R,#1R,#2L}, {#1R,#2L,#3L}, {#1R,#2R,#3L} | 4 |
| Four RRHs | {#0R,#1R,#2L,#3L} | 1 |
| Total # of combinations |  | 22 |

When a NW relies on Rel-16 QCL framework, the Doppler shift, and Doppler spread property of SFNed target channel are inferred from a single TRS. It implies that when the NW is to use multiple SFN combinations for data and RS transmission it needs to configure the corresponding number of TRSs. In other words, if the NW is to use all SFN combinations according to Table 1, it needs to configure 22 TRSs. This would result in excessive use of NW resources only for TRS transmissions. According to our analysis, TRS overhead ratio from the total REs in NW is 36.5% for N = 8 RRHs as shown in Table 2, where the detailed assumptions are provided in the appendix. Note that the overhead ratio would further increase as the number of RRHs and/or the number of Tx beams per RRH increases, which can be excessive especially in FR2.

Observation 1. Rel-16 QCL framework requires excessive TRS overhead to support HST-SFN.

To solve the TRS overhead problem, we propose an extension of the QCL framework to allow the QCL properties, e.g., Doppler shift and Doppler spread associated with PDSCH can be inferred from multiple TRSs. In this extended QCL framework, NW does not need to configure separate TRS per each SFN combination. Rather, it only needs to configure one TRS per RRH per beam. Then NW indicates UE about QCL property of a SFNed target channel via a list of TRSs corresponding to the RRHs participating in SFN transmission. TRS overhead reduction according to the QCL framework enhancement is analyzed in Table 2 for the bi-directional N-RRH network. Note that the TRS overhead can be reduced from 36.5 % to 15.4% (57.9% ratio reduction) for N = 8.

Observation 2. QCL framework enhancement can reduce TRS overhead significantly compared to Rel-16 QCL framework for HST-SFN.

Table 2. TRS overhead analysis for Rel-16 QCL framework vs. extended QCL framework

|  | # of TRSs |  | TRS overhead |  | TRS overhead ratio |  | Overhead ratio reduction |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Rel-16 QCL | QCL enhancement | Rel-16 QCL | QCL enhancement | Rel-16 QCL | QCL enhancement |  |
| N = 4 | 22 | 8 | 13,200 REs | 4,800 REs | 21.2% | 7.7% | 63.7% |
| N = 6 | 30 | 12 | 18,000 REs | 7,200 REs | 28.8% | 11.5% | 60.0% |
| N = 8 | 38 | 16 | 22,800 REs | 9,600 REs | 36.5% | 15.4% | 57.9% |

The extended QCL framework is also beneficial in reducing UE complexity in estimating Doppler shifts from TRS. In Rel-16 HST-SFN, the gNB transmits TRS in an SFNed manner and the UE has to track multiple Doppler shifts from the TRS. However, QCL framework enhancement allows gNB to transmit non-SFNed TRS and the UE is sufficient to track only a single Doppler shift per TRS. Figure 3 and Table 3 show the performance of Doppler shift tracking by a single SFNed TRS vs. three non-SFNed TRS in a 3-RRH network (estimation accuracy is improved by 64%). Assume that the UE tracks Doppler shifts of two nearest RRHs at a time. By using multiple non-SFNed TRS, the UE can achieve better Doppler shift tracking performance than SFNed TRS while requiring lower complexity. For example, the UE may suffer from large estimation errors from SFNed TRS as marked with red circles in the left figure. Such error occurs in the case that the transmission delays from RRHs to UE are almost the same so that their Doppler shifts are non-resolvable from SFNed TRS. If we use separate TRS per RRH, such error would not occur.

Figure 2. Doppler shift estimation in a 3-RRH network

Table 3. Mean absolute error of estimated Doppler shift

| SFNed TRS | Separate TRS per RRH | Error reduction by using separate RRH |
| --- | --- | --- |
| 113.4 Hz | 40.8 Hz | 72.6 Hz (64.0%) |

Observation 3. QCL framework enhancement that allows UE to track only a single Doppler shift per TRS in HST-SFN results in better estimation performance and lower complexity compared to Rel-16.

Until the RAN1#104-e meeting, it was under discussion in which target channels are applied the QCL framework enhancement for TRS. Rel-16 QCL framework defines that QCL target channel of TRS can be one of i) PDSCH, ii) PDCCH, iii) aperiodic (AP) TRS, iv) CSI-RS for CSI, v) CSI-RS for beam management (BM). Since all the aforementioned channels are supported for Rel-16 spec-transparent SFN, there’s no reason to exclude any of them from the QCL enhancement. On the contrary, estimation performance of all the aforementioned channels would be improved via the QCL framework enhancement because of the improvement of tracking accuracy for QCL referred TRS, as shown in Table 3. For the discussion of which target RSs are supported, PDCCH and PDSCH can be dealt with the first priority for HST-SFN, and the other reference signals, e.g., AP TRS, CSI-RS for CSI, and CSI-RS for BM, can be also dealt with the next priority and which agenda where this issue will be discussed can be also decided, e.g., HST-SFN or multi-TRP CSI.

Proposal 1. Support QCL target channel with a certain priority as follows.

- PDSCH / PDCCH

- AP TRS, CSI-RS for CSI, and CSI-RS for BM (discussed with a low priority or decide where to discuss under between AI 8.1.2.4 HST-SFN or AI 8.1.4 multi-TRP CSI)

During RAN1#104-e meeting, the switching mechanism between SFN scheme 1 and several Rel-16 multi-TRP schemes. It was agreed that RRC based semi-static switching between SFN scheme 1 and Rel-16 multi-TRP URLLC scheme 2a, 2b, 3, and 4. However, the controversial point is the switching among SFN scheme 1, Rel-16 multi-TRP scheme 1a, and a single-TRP transmission dynamically. In our view, both Rel-16 multi-TRP scheme 1a and SFN scheme 1 are beneficial for HST scenario, where Rel-16 multi-TRP scheme 1a can provide high throughput when UE is relatively slow so that its location can be accurately tractable, while SFN scheme 1 can provide high reliability when UE is fast. However, although the UE speed would change dynamically, it is controversial and doubtful how often the UE speed changes. Hence, although it is beneficial for NW to support dynamic switching between Rel-16 multi-TRP scheme 1a and SFN scheme 1, we first focus on the dynamic switching between SFN scheme 1 and a single TRP transmission. For the scheduling flexibility between SFNed transmission on PDCCH/PDSCH and dynamic point selection, it is necessary to support at least dynamic switching between SFN scheme 1 and single-TRP transmission. Moreover, since the switching can be used as a fallback operation from SFN scheme 1, a UE reporting the UE capability to support SFN scheme 1, which will be an optional, should support a single-TRP transmission, too. Therefore, it is not necessary to introduce additional UE capability for this switching.

Proposal 2. Support at least dynamic switching between scheme 1 and single-TRP transmission without additional UE capability.

Until RAN1#104-e meeting, it was under discussion whether PDSCH DM-RS needs to be transmitted in per-TRP manner as well as TRS. Per-TRP DM-RS transmission implies that each TRP participating in SFN are allocated different DM-RS ports, resulting in doubled number of DM-RS ports and doubled DM-RS overhead. Since per-TRP channel properties, such as Doppler and delay properties, can be already obtainable by per-TRP TRS, the necessity to introduce per-TRP DM-RS transmission is unclear considering its high overhead.

Proposal 3. Do not support Scheme 2 for UE enhancement.

gNB-based enhancements for HST-SFN

If the gNB can know the Doppler shift of the channel between each RRH and the UE, the gNB can pre-compensate Doppler shift at each RRH such that the UE experiences only a small residual Doppler shift at the received signal. Such pre-compensation can reduce UE complexity to track multiple Doppler shifts and improve the reliability of the received signal. Also, if the solution for HST scenario is only based on UE-side, it seems the incomplete solution as not all UEs will support the SFN scheme 1. Hence, gNB which has the functionality to pre-compensate Doppler effect can assist the UEs in HST scenario.

Figure 3 shows the PDSCH throughput comparison between UE-based enhancement scheme (per-TRP TRS), i.e., SFN scheme 1, and gNB enhancement scheme (gNB pre-compensation), with respect to the UE distance from the first TRP where there are two TRPs in the network. Based on the Figure 3, it is shown that gNB pre-compensation can improve the UE throughput significantly compared to the SFN scheme 1.

Figure 3. Throughput comparison between UE enhancement and gNB enhancement schemes
with respect to distance.

Observation 4. gNB pre-compensation can provide significant PDSCH throughput improvement compared to UE-based enhancement scheme.

Proposal 4. Support gNB pre-compensation scheme for HST-SFN scenario.

In RAN1#102-e, the following three steps were defined for gNB pre-compensation procedures:

Agreement For discussion purpose consider the following three steps for TRP-based frequency offset pre-compensation scheme: - 1st step: Transmission of the TRS resource(s) from TRP(s) without pre-compensation - 2nd step: Transmission of the uplink signal(s)/channel(s) with carrier frequency determined based on the received TRS signals in the 1st step - 3rd step: Transmission of the PDCCH/PDSCH from TRP(s) with frequency offset pre-compensation determined based on the received signal/channel in the 2nd step - Note: A second set of TRS resource(s) may be transmitted at 3rd step.

In the second step, UE informs Doppler shifts to the TRPs participating in SFN transmission via the two options:

- Option 1. UE transmits UL signal, e.g., SRS, and each TRP measures the UL signal to obtain Doppler shift

- Option 2. UE reports the measured Doppler shift from the 1st step via CSI report

Option 1 can be supported by gNB implementation by measuring Doppler shift from existing SRS for UL sounding purpose, so that no additional specification impact or RS overhead are needed. However, Option 2 requires additional specification support to define new UCI for Doppler shift reporting, and also requires additional UL resource for the new UCI. Hence in our view, Option 1 is preferred rather than Option 2.

Overall Doppler shift measurement procedure according to Option 1 is depicted in Figure 4. In Figure 4, UE receives separate TRSs from TRP#0 and TRP#1. Then, UE transmits an UL signal with frequency fc which are received with frequencies fm,0=fc+f0 and fm,1=fc+f1 at TRP#0 and TRP#1, respectively, where f0 and f1 are Doppler shifts at TRP#0 and TRP#1, respectively. From the UL signal, gNB obtains the frequency offset between TRP#0 and TRP#1, Δf=fm,0-fm,1=f0-f1, to be used for pre-compensation. In this procedure, when multiple TRSs are received with different frequencies the value of UL frequency fc is not defined in current specification so that it is up to UE implementation how to determine the value of fc.

There were discussions in RAN1#102-e meeting whether it is needed to indicate the UL frequency fc for UL signals used in pre-compensation purpose. In our view, indicating fc is not needed because the frequency offset measured at gNB, Δf, does not contain the component fc such that the pre-compensated value at gNB won’t differ regardless of fc.

Figure 4. Pre-compensation procedure according to Option 1.

Observation 5. In HST-SFN environment configured with multiple per-TRP TRSs, it is up to UE implementation how to determine UL frequency.

Proposal 5. Regarding the 2nd step for gNB pre-compensation procedure:

- Support Option 1 (Doppler shift measurement at TRP side)

- There’s no need to indicate UL frequency to UL signals/channels for pre-compensation purpose

In the third step, TRP#0 transmits signal with frequency fc while TRP#1 transmits signal with frequency fc-Δf for pre-compensation purpose. Then UE would receive SFNed signal from the two TRPs where frequency offset observed in the second step is removed.

One issue on the third step is to determine the target channel where the pre-compensation applies. In our view, it is needed to apply pre-compensation for PDSCH and PDCCH, and furthermore for TRS. As Rel-16 QCL framework mandates PDSCH and PDCCH to be in QCL linkage with TRS with QCL-TypeA, implying that Doppler shift and spread properties of PDSCH/PDCCH are inferred from the TRS, it is evident that TRS should be pre-compensated first to support pre-compensated PDSCH/PDCCH.

A drawback of supporting pre-compensated TRS is that the TRS needs to be UE specific since each UE faces different relative speed, resulting in the necessity of different pre-compensation value. For the system with lots of UEs, periodically transmitting such TRS imposes high RS overhead. An effective way to relieve such overhead is using AP-TRS only for pre-compensation, but it needs enhancement from the current spec. In Rel-16, the AP-TRS is restricted to be QCL’ed with P-TRS according to QCL-TypeA which implies that the AP-TRS cannot have different Doppler shift from its QCL’ed P-TRS, i.e., AP-TRS alone cannot be pre-compensated.

For pre-compensation of AP-TRS, we can introduce a new QCL type which does not contain Doppler shift property, i.e., Variant A or Variant E with dropping some QCL parameters as discussed in RAN1#103-e. Then, pre-compensated AP-TRS can be QCL’ed with non pre-compensated TRSs via the new QCL type as shown in Figure 4. Pre-compensated PDSCH or PDCCH can be QCL’ed with the AP-TRS via existing QCL type, e.g., QCL-TypeA and QCL-TypeD, which can be a case of Variant E.

Proposal 6. Regarding the 3rd step for gNB pre-compensation procedure:

- Support non pre-compensated P-TRS, which Variant A or E can be applied, to be used as QCL reference of pre-compensated AP-TRS

- Support pre-compensated AP-TRS, which Variant E can be applied, to be used as QCL reference of pre-compensated PDCCH/PDSCH.

SFN transmission of PDCCH from multi-TRP

Regarding SFN transmission of PDCCH from multi-TRP, the current default spatial relation and pathloss reference RS (PL-RS) for PUCCH, PUSCH, and SRS would be affected. As a result of the possibility of SFN transmission of PDCCH from multi-TRP, the existing method based on current specification to derive a default spatial setting and PL-RS for PUCCH cannot directly be applied. This is a direct consequence of the existence of multiple TCI states associated to the SFNed CORESET with the lowest ID. Therefore, it is required to resolve this ambiguity.

However, the current specification considers the only case of a single default spatial relation and PL-RS for PUCCH, PUSCH, and SRS. In Rel-17, the multiple default spatial relations and PL-RSs can be also considered at least for multi-TRP based PUCCH and PUSCH repetition. For PUCCH or PUSCH repetition intended towards multi-TRP, if certain conditions are fulfilled, it is natural to select multiple default spatial relations so that each spatial relation can be mapped to the PUCCH or PUSCH repetitions towards a corresponding TRP similar with two default TCI states in Rel-16 PDSCH TDMed repetition. Otherwise, for example, if a single default PL-RS is selected based on only one TCI state (spatial relation from a certain TRP), then the computed PL may not be accurate to compensate PL in PUCCH or PUSCH repetitions towards the remaining TRPs. Similarly, if a spatial setting is selected based on a single TCI states, then the PUCCH transmissions towards the remaining TRPs may not be correctly beamformed. Therefore, it is essential to consider multiple default beams for multi-TRP based PUCCH or PUSCH repetitions.

Proposal 7. Support to study the default spatial relation and PL-RS for PUCCH, PUSCH, and SRS for SFN transmission of PDCCH.

Proposal 8. Support to study the two default spatial relations and PL-RSs at least for PUCCH and PUSCH for both SFN transmission of PDCCH and multi-TRP PUCCH/PUSCH repetition.

Conclusion

In this contribution, the following observations and proposals are given:

Observation 1. Rel-16 QCL framework requires excessive TRS overhead to support HST-SFN.

Observation 2. QCL framework enhancement can reduce TRS overhead significantly compared to Rel-16 QCL framework for HST-SFN.

Observation 3. QCL framework enhancement that allows UE to track only a single Doppler shift per TRS in HST-SFN results in better estimation performance and lower complexity compared to Rel-16.

Observation 4. gNB pre-compensation can provide significant PDSCH throughput improvement compared to UE-based enhancement scheme.

Observation 5. In HST-SFN environment configured with multiple per-TRP TRSs, it is up to UE implementation how to determine UL frequency.

Proposal 1. Support QCL target channel with a certain priority as follows.

- PDSCH / PDCCH

- AP TRS, CSI-RS for CSI, and CSI-RS for BM (discussed with a low priority or decide where to discuss under between AI 8.1.2.4 HST-SFN or AI 8.1.4 multi-TRP CSI)

Proposal 2. Support at least dynamic switching between scheme 1 and single-TRP transmission without additional UE capability.

Proposal 3. Do not support Scheme 2 for UE enhancement.

Proposal 4. Support gNB pre-compensation scheme for HST-SFN scenario.

Proposal 5. Regarding the 2nd step for gNB pre-compensation procedure:

- Support Option 1 (Doppler shift measurement at TRP side)

- There’s no need to indicate UL frequency to UL signals/channels for pre-compensation purpose

Proposal 6. Regarding the 3rd step for gNB pre-compensation procedure:

- Support non pre-compensated P-TRS, which Variant A or E can be applied, to be used as QCL reference of pre-compensated AP-TRS

- Support pre-compensated AP-TRS, which Variant E can be applied, to be used as QCL reference of pre-compensated PDCCH/PDSCH.

Proposal 7. Support to study the default spatial relation and PL-RS for PUCCH, PUSCH, and SRS for SFN transmission of PDCCH.

Proposal 8. Support to study the two default spatial relations and PL-RSs at least for PUCCH and PUSCH for both SFN transmission of PDCCH and multi-TRP PUCCH/PUSCH repetition.

References

- 3GPP RAN1, RAN1#102-e Chairman’s Note

- 3GPP RAN1, RAN1#103-e Chairman’s Note

- 3GPP RAN1, RAN1#104-e Chairman’s Note

- Appendix

Table 4. Evaluation assumptions

| Parameters | Values |
| --- | --- |
| TRP layout | Ds = 700 m, Dmin = 150 m |
| Channel model | HST-SFN channel model in TS36.101 |
| Carrier frequency / SCS | 3.5 GHz / 30 kHz |
| UE speed | 500km/h |
| gNB / UE antenna configuration | 2 Tx ports / 2 Rx ports |
| Number of RBs | 50 RBs |
| SNR | 10 dB |
| TRS periodicity / pattern | 10 ms, 2 slot pattern |