3GPP TSG RAN WG1 #104b-e	R1-2103241

e-Meeting, April 12th –20th, 2021

Agenda item:	8.4.1

Source: 	Samsung

Title: 	Timing relationship enhancements for NTN

Document for:	Discussion and Decision

Introduction

A new work item on “Solutions for NR to support non-terrestrial networks (NTN)” was approved in RAN#86 [1]. In RAN1#104-e, the following agreements were made.

Agreement: Confirm the following working assumption: K_offset can be applied to indicate the first transmission opportunity of PUSCH in Configured Grant Type 2 in the same way as K_offset is applied to the transmission timing of DCI scheduled PUSCH. Agreement: Update of K_offset after initial access is supported Agreement: For unpaired spectrum, extend the value range of K1 from (0..15) to (0..31) FFS: Whether there is an impact on the size of the PDSCH-to-HARQ_feedback timing indicator field in DCI. Working assumption: Introduce K_offset to enhance the adjustment of uplink transmission timing upon the reception of a corresponding timing advance command.

In this contribution, we continue to discuss the timing relationship enhancements for NTN.

Beam specific Koffset value in initial access

For scheduling offset value, RAN1 had discussion on whether the offset value is cell specific or beam specific. Cell specific configuration for the offset value has advantages in that it requires less signaling overhead and leads to a simple solution. Of course, beam specific configuration enables finer control of the offset value. However, even though the granularity of the offset value is course with cell specific configuration, the gNB can handle the residual scheduling offset by using different delay indicated by DCI.

Proposal 1: Support cell specific Koffset value only.

Configuration of Koffset

For configuration of Koffset, there are following candidates:

- Cell specific Koffset, which can be derived based on the maximum full TA within one cell.

- Beam specific Koffset, which can be derived based on the maximum full TA within one beam footprint.

- Beam-group specific Koffset, which can be derived based on the maximum full TA within one group of neighboring beam footprints

- UE specific Koffset, which can be derived based on UE specific full TA

Considering different scenarios and deployments, more than one of above configurations can be supported, and using which one can be dependent on gNB configuration. For example, for a small cell range wherein the TA variation range is small, cell specific Koffset is preferred. For a large cell range consisting of multiple small beam footprints, beam specific Koffset or beam-group specific Koffset is preferred. For a large beam footprint wherein the TA variation range is large, UE specific Koffset is preferred. Therefore, above configurations are suitable for different scenarios, and more than one configurations can be specified.

Proposal 2: More than one of above Koffset configurations can be supported, and using which one is dependent on gNB configuration.

In RAN1#104-e, the following was agreed.

Agreement:

Update of K_offset after initial access is supported

Different solutions and options on how to update and indicate K_offset t after initial access have been discussed in the previous meetings. The following summarizes some of the options: 1)	RRC configuration, 2)	 MAC CE, 3)	Group common DCI, 4)	Signaling multiple K_offset values in a non-UE specific way which are used to update the UE applied value over time, and 5)	UE updates the value of K_offset based on predefined rules. Compared to other approaches, using RRC configuration could be the simplest solution to update K_offset after initial access procedure because a UE already has a connection with the gNB.

Proposal 3: The update of K_offset value after initial access is done by RRC configuration.

Timing for configured grant scheduled PUSCH

3GPP has specifies two categories of Configured Grant: 1) Configured Grant Type 1; and 2) Configured Grant Type 2.

Configured Grant Type 1

When using Configured Grant Type 1, a transmission occasion starts when the following equation is satisfied:

| [(SFN × numberOfSlotsPerFrame × numberOfSymbolsPerSlot) + (slot number in the frame × numberOfSymbolsPerSlot) + symbol number in the slot] = (timeReferenceSFN-r16 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot + timeDomainOffset × numberOfSymbolsPerSlot + S + N × periodicity) modulo (1024 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot). | (1) |
| --- | --- |

where timeDomainOffset and periodicity are both specified within the ConfiguredGrantConfig parameter structure. S corresponds to the starting symbol deducted from the timeDomainAllocation IE. N ≥ 0 is an integer corresponding to the Nth transmission occasion. As seen in Figure 1, gNB should configure UE such that the first transmission occasion (N = 0) happens after arrival of slot n at UE, i.e. the configured symbol for N = 0 transmission in equation (1) should happen in time in an amount of greater than or equal to TA after the current slot n. This can be done using two options:

- Add Koffset to equation (1), where Koffset is greater than or equal TA.  Note that in the other timing relationships discussed earlier, it is expected that the value of Koffset there is selected equal to TA. So the Koffset for configured grant could be the same value.  Of course to introduce it to equation (1) it has to be converted to symbol duration, i.e. Koffset × numberOfSymbolsPerSlot should be added to equation (1).

- Since in Rel-16 a new optional parameter namely timeReferenceSFN-r16 was introduced in ConfiguredGrantConfig IE, the value of Koffset can be implicitly included in this parameter by gNB.

Figure 1. Configured grant Type 1 timing

For other timing relationships for NTN a value for Koffset is being defined and hence Option 1 above will be practical and feasible.  However, Option 2 offers more flexibility and it leaves it to the Network to manage the timing with more freedom.

Proposal 4: The timing relationship for Configured Grant Type 1 should be left to Network implementation.

HARQ timing indicator field

In RAN1#104-e, the following was agreed.

Agreement:

For unpaired spectrum, extend the value range of K1 from (0..15) to (0..31)

FFS: Whether there is an impact on the size of the PDSCH-to-HARQ_feedback timing indicator field in DCI.

For Rel-15/16, depending on how many values for k are configured, PDSCH-to-HARQ_feedback timing indicator field in a DCI format could have up to 3 bits. This can provide a flexibility of the timing indication to the gNB. Even though the range of the K1 value is extended, there is no reason to increase the size of PDSCH-to-HARQ_feedback timing indicator field in a DCI format which should result in the performance degradation of DCI decoding.

Proposal 5: Do not change the size of the PDSCH-to-HARQ_feedback timing indicator field in DCI.

Conclusions

In this contribution, we discussed the timing relationship enhancements for NTN. Our proposals are summarized as below.

Proposal 1: Support cell specific Koffset value only.

Proposal 2: More than one of above Koffset configurations can be supported, and using which one is dependent on gNB configuration.

Proposal 3: The update of K_offset value after initial access is done by RRC configuration.

Proposal 4: The timing relationship for Configured Grant Type 1 should be left to Network implementation.

Proposal 5: Do not change the size of the PDSCH-to-HARQ_feedback timing indicator field in DCI.

- References

- RP‑210908, “Revised WID: Solutions for NR to support non-terrestrial networks (NTN).”