3GPP TSG RAN WG1 #104b-e	R1- 2103266

# e-Meeting, April 12th – 20th, 2021

Agenda Item:		8.15.1

Source:				Samsung

Title:					Initial link budget evaluation for NB-IoT/eMTC

Document for:	Discussion

# Introduction

One objective of the Study on NB-IoT/eMTC support for Non-Terrestrial Network approved in RAN#86 [1] is to identify scenarios applicable to NB-IoT/eMTC. Agreements on link budget analysis reached in RAN1#104e are listed in the Appendix.

This contribution discusses initial link budget evaluation based on the assumptions agreed in RAN1#104-e.

# Discussion

Following up to the agreements of the NTN IoT reference scenarios and parameters in previous RAN1 meetings, feasibility of NB-IoT and eMTC support on NTN is investigated based on a link budget analysis. It is noted that performance evaluation of NB-IoT and eMTC was done in the IMT-2020 5G performance requirements [3] and earlier in the MTC study in Rel-12 [4], and the methodologies used in these reports can be reused for the evaluation of IoT over NTN. Tables similar to the ones in in Sec.6.1 [2] for link/system evaluation and link budget calculation can be adapted and reused.

Agreements in RAN1#104-e are listed in the Appendix. Initial link budget analysis is presented for the following agreed sets of parameters. Uplink results consider a 23dBm transmit power. A noise figure of 7dB is used for the DL results. The carrier frequency is 2GHz.

- Set 1 satellite parameters (based on TR 38.821, Table 6.1.1.1-1)

- Set 2 satellite parameters (based on TR 38.821, Table 6.1.1.1-2)

- Set 3 satellite parameters (Eutelsat R1-2101146 with central beam edge elevation 12.5 degrees for GEO, and 30 degrees for LEO-600 km and 1200 km)

- Set 4 satellite parameters (Thales, Sateliot, Gatehouse R1-2101019)

| SET 1 - Downlink link budget | GEO 35786 km | LEO 1200 km | LEO 600 km | units |
| --- | --- | --- | --- | --- |
| Elevation angle | 12.5 | 30 | 30 | degree |
| EIRP Density | 59 | 40 | 34 | dBW/MHz |
| EIRP per spot (1080 kHz) | 59.3 | 40.3 | 34.3 | dBW |
| EIRP per spot (180 kHz) | 51.6 | 32.6 | 26.6 | dBW |
| RX antenna gain | 0 | 0 | 0 | dBi |
| Path length UE-Satellite | 40316.7 | 1998.9 | 1075.1 | Km |
| FSPL | 190.6 | 164.5 | 159.1 | dB |
| FPSL + other losses | 199.0 | 172.8 | 167.4 | dB |
| Received power | -117.1 | -109.9 | -110.5 | dBW |
| (C/N)_DL (1080 kHz) | -3.0 | 4.2 | 3.6 | dB |
| (C/N)_DL (180 kHz) | -3.0 | 4.2 | 3.6 | dB |
| G/T = Ga – NF – 10*LOG (To+(Ta-To)/(100.1*NF)) | -31.6 | -31.6 | -31.6 | dB/K |

| SET 1 - Uplink link budget | GEO 35786 km | LEO 1200 km | LEO 600 km | units |
| --- | --- | --- | --- | --- |
| Elevation angle | 12.5 | 30 | 30 | degree |
| Transmitted power | -7 | -7 | -7 | dBW |
| TX antenna gain | 0 | 0 | 0 | dBi |
| EIRP | -7 | -7 | -7 | dBW |
| RX antenna gain | 51 | 24.1 | 24.1 | dBi |
| Path length UE-Satellite | 40316.7 | 1998.9 | 1075.1 | Km |
| FSPL | 190.6 | 164.5 | 159.1 | dB |
| FPSL + other losses | 199.0 | 172.8 | 167.4 | dB |
| Received power | -155.0 | -155.7 | -150.3 | dBW |
| (C/N)_UL (45 kHz) | -7.9 | 0.4 | 5.8 | dB |
| (C/N)_UL (15 kHz) | -3.1 | 5.2 | 10.5 | dB |
| (C/N)_UL (3.75 kHz) | 2.9 | 11.2 | 16.6 | dB |
| G/T [TR 38.821 SET1, NR NTN] | 19 | 1.1 | 1.1 | dB/K |

| SET 2 - Downlink link budget | GEO 35786 km | LEO 1200 km | LEO 600 km | units |
| --- | --- | --- | --- | --- |
| Elevation angle | 12.5 | 30 | 30 | degree |
| EIRP Density | 53.5 | 34 | 28 | dBW/MHz |
| EIRP per spot (1080 kHz) | 53.8 | 34.3 | 28.3 | dBW |
| EIRP per spot (180 kHz) | 46.1 | 26.6 | 20.6 | dBW |
| RX antenna gain | 0 | 0 | 0 | dBi |
| Carrier frequency | 2 | 2 | 2 | GHz |
| Path length UE-Satellite | 40316.7 | 1998.9 | 1075.1 | Km |
| FSPL | 190.6 | 164.5 | 159.1 | dB |
| FPSL + other losses | 199.0 | 172.8 | 167.4 | dB |
| Received power | -122.6 | -115.9 | -116.5 | dBW |
| (C/N)_DL (1080 kHz) | -8.5 | -1.8 | -2.4 | dB |
| (C/N)_DL (180 kHz) | -8.5 | -1.8 | -2.4 | dB |
| G/T = Ga – NF – 10*LOG (To+(Ta-To)/(100.1*NF)) | -31.6 | -31.6 | -31.6 | dB/K |

| SET 2 - Uplink link budget | GEO 35786 km | LEO 1200 km | LEO 600 km | units |
| --- | --- | --- | --- | --- |
| Elevation angle | 12.5 | 30 | 30 | degree |
| Transmitted power | -7 | -7 | -7 | dBW |
| TX antenna gain | 0 | 0 | 0 | dBi |
| EIRP | -7 | -7 | -7 | dBW |
| RX antenna gain | 51 | 24.1 | 24.1 | dBi |
| Path length UE-Satellite | 40316.7 | 1998.9 | 1075.1 | Km |
| FSPL | 190.6 | 164.5 | 159.1 | dB |
| FPSL + other losses | 199.0 | 172.8 | 167.4 | dB |
| Received power | -155.0 | -155.7 | -150.3 | dBW |
| (C/N)_UL (45 kHz) | -12.9 | -5.6 | -0.2 | dB |
| (C/N)_UL (15 kHz) | -8.1 | -0.8 | 4.5 | dB |
| (C/N)_UL (3.75 kHz) | -2.1 | 5.2 | 10.6 | dB |
| G/T [TR 38.821 SET1, NR NTN] | 14 | -4.9 | -4.9 | dB/K |

| SET 3 - Downlink link budget | GEO 35786 km | LEO 1200 km | LEO 600 km | units |
| --- | --- | --- | --- | --- |
| Elevation angle | 12.5 | 30 | 30 | degree |
| EIRP Density | 59.8 | 33.7 | 28.3 | dBW/MHz |
| EIRP per spot (1080 kHz) | 60.1 | 34.0 | 28.6 | dBW |
| EIRP per spot (180 kHz) | 52.4 | 26.3 | 20.9 | dBW |
| RX antenna gain | 0 | 0 | 0 | dBi |
| Path length UE-Satellite | 40316.7 | 1998.9 | 1075.1 | Km |
| FSPL | 190.6 | 164.5 | 159.1 | dB |
| FPSL + other losses | 199.0 | 172.8 | 167.4 | dB |
| Received power | -116.3 | -116.2 | -116.2 | dBW |
| (C/N)_DL (1080 kHz) | -2.2 | -2.1 | -2.1 | dB |
| (C/N)_DL (180 kHz) | -2.2 | -2.1 | -2.1 | dB |
| G/T = Ga – NF – 10*LOG (To+(Ta-To)/(100.1*NF)) | -31.6 | -31.6 | -31.6 | dB/K |

| SET 3 - Uplink link budget | GEO 35786 km | LEO 1200 km | LEO 600 km | units |
| --- | --- | --- | --- | --- |
| Elevation angle | 12.5 | 30 | 30 | degree |
| Transmitted power | -7 | -7 | -7 | dBW |
| TX antenna gain | 0 | 0 | 0 | dBi |
| EIRP | -7 | -7 | -7 | dBW |
| RX antenna gain | 51 | 24.1 | 24.1 | dBi |
| Path length UE-Satellite | 40316.7 | 1998.9 | 1075.1 | Km |
| FSPL | 190.6 | 164.5 | 159.1 | dB |
| FPSL + other losses | 199.0 | 172.8 | 167.4 | dB |
| Received power | -155.0 | -155.7 | -150.3 | dBW |
| (C/N)_UL (45 kHz) | -10.2 | -13.5 | -8.1 | dB |
| (C/N)_UL (15 kHz) | -5.4 | -8.7 | -3.4 | dB |
| (C/N)_UL (3.75 kHz) | 0.6 | -2.7 | 2.7 | dB |
| G/T [EUTELSAT (NB-IoT)] | 16.7 | -12.8 | -12.8 | dB/K |

| SET 4 - Downlink link budget | SINGLE BEAM LEO 600 km | MULTIPLE BEAM LEO 600 km | units |
| --- | --- | --- | --- |
| Elevation angle | 56.8 | 30 | degree |
| EIRP Density | 21.45 | 21.45 | dBW/MHz |
| EIRP per spot (1080 kHz) | 21.8 | 21.8 | dBW |
| EIRP per spot (180 kHz) | 14.0 | 14.0 | dBW |
| RX antenna gain | 0 | 0 | dBi |
| Path length UE-Satellite | 704.3 | 1075.1 | Km |
| FSPL | 155.4 | 159.1 | dB |
| FPSL + other losses | 163.7 | 167.4 | dB |
| Received power | -119.4 | -123.1 | dBW |
| (C/N)_DL (1080 kHz) | -8.3 | -12.0 | dB |
| (C/N)_DL (180 kHz) | -8.3 | -12.0 | dB |
| G/T = Ga – NF – 10*LOG (To+(Ta-To)/(100.1*NF)) | -31.6 | -31.6 | dB/K |

| SET 4 - Uplink link budget | SINGLE-BEAM LEO 600 km | MULTIPLE BEAM LEO 600 km | units |
| --- | --- | --- | --- |
| Elevation angle | 56.8 | 30 | degree |
| Transmitted power | -7 | -7 | dBW |
| TX antenna gain | 0 | 0 | dBi |
| EIRP | -7 | -7 | dBW |
| RX antenna gain | 24.1 | 24.1 | dBi |
| Path length UE-Satellite | 704.3 | 1075.1 | Km |
| FSPL | 155.4 | 159.1 | dB |
| FPSL + other losses | 163.7 | 167.4 | dB |
| Received power | -146.6 | -150.3 | dBW |
| (C/N)_UL (45 kHz) | -6.6 | -13.2 | dB |
| (C/N)_UL (15 kHz) | -1.8 | -8.5 | dB |
| (C/N)_UL (3.75 kHz) | 4.2 | -2.4 | dB |
| G/T [SATELIOT (NB-IoT)] | -17.9 | -20.9 | dB/K |

# Conclusion

This contribution provides an initial NTN-IoT link budget evaluation for sets 1 to 4 of the satellite parameters.

# References

- RP-193235, New Study on NB-IoT/eMTC support for NTN, RAN#86, Dec. 2019.

- 3GPP TR 38.821 V16.0.0 (2019-12), Solutions for NR to support non-terrestrial networks (NTN), Rel-16.

- ITU-R, Report ITU-R M.2411-0 “Requirements, evaluation criteria and submission template for the development of IMT-2020”, ITU-R.

- TR 36.888, Machine-Type Communications (MTC) User Equipments (UEs) based on LTE, 3GPP, V12.0.0, June 2013.

# Appendix – RAN1#104-e agreements

Agreement: The following assumptions are agreed for a common set of link budget parameters: - UE power class (PC5=20 dBm) - UE Noise Figure (NF=9 dB) - Channel Bandwidth for NB-IoT and eMTC as was included in IoT NTN reference scenario parameters agreed in RAN1#103e - NB-IoT 180 kHz (DL), Up to 180 kHz with all permissible smaller resource allocations 12*15 kHz, 6*15 kHz, 3*15 kHz, 1*15 kHz, 1*3.75 kHz - eMTC: 1080 kHz (DL), Up to 1080 kHz with all permissible smaller resource allocations, including 2*180 kHz, 180 kHz, 2*15 kHz or 3*15 kHz or 6*15 kHz (UL) - Other losses Other Losses; GEO (35786 km); LEO (1200 km); LEO (600 km). Scintillation losses; 2.2; 2.2; 2.2. Atmospheric losses; 0.2; 0.1; 0.1. Polarization loss; 3; 3; 3. Shadow margin; 3; 3; 3. NOTE 1: With PC3 (23 dBm) there is a 3dB gain compared to the PC5 (20 dBm) assumption on UL. NOTE 2: With NF=7 dB, there is a 2 dB improvement compare to NF=9 dB on DL. NOTE 3: Link budgets with other link budget parameters are not excluded from being captured in the TR. NOTE 4: These parameters are only for the purpose of link budget calculations. NOTE 5: Atmospheric losses are a function of elevation angle. Agreement: Link budget analysis assumes 3 dB polarization loss for DL and 3 dB polarization loss on UL for satellite parameters Set 1, Set 2, Set 3, and Set 4 Agreement: Include in TR 36.763, the 3 dB beam width (HPBW), central beam center elevation and central beam edge elevation in the satellite parameter set(s) to be used in link budget calculations – (Corresponding satellite parameter Set 3 and Set 4 are given in Section 9.4) SET 3; GEO 35786 km; LEO-600 km; LEO-1200 km. 3 dB Beam width (HPBW); 0.735 degree; 22.0631 degree; 22.0631 degree. Central beam center elevation; 20.88 degree; 43.78 degree; 46.05 degree. Central beam edge elevation; 12.5 degree; 30 degree; 30 degree. Central beam edge satellite-UE distance; 40316 km; 1074 km; 1998 km. SET 4; LEO-600 km. 3 dB Beam width (HPBW); 104.7 degree. Central beam center elevation; 90 degree. Central beam edge elevation; 30 degree. Central beam edge satellite-UE distance; 1076 km. NOTE 1: The 3 dB beam width (HPBW) is already included in satellite parameter set 1 and Set 2 in TR 38.821 Table 6.1.1.1-1 and Table 6.1.1.1-2, respectively. The central beam center elevation for Set-1 and Set-2 is defined as the target elevation angle that is included in in TR 38.821 Table 6.1.3.2-1. The central beam edge satellite-UE distance can be derived from the central beam edge elevation and does not need to be included. NOTE 2: Central beam center elevation is the beam center elevation of the central beam in the beam layout. NOTE 3: Central beam edge elevation is the minimum beam edge elevation of the central beam in the beam layout. NOTE 4 In SLS evaluation with a multiple beam layout, the central beam is the serving beam for UEs. The outer beams have beam center elevation that is different from the central beam center elevation. For the interference modelling, the interference due to the outer beams is determined by using their respective beam center elevations. NOTE 5: For the multiple-beam satellite cell, the longest beam edge distance will correspond to the minimum beam edge elevation of the most outer beam as illustrated in figure below. Agreement: Include the following tables in TR 36.763: - Set 1 satellite parameters (based on TR 38.821, Table 6.1.1.1-1) - Set 2 satellite parameters (based on TR 38.821, Table 6.1.1.1-2) - Set 3 satellite parameters (Eutelsat R1-2101146 with central beam edge elevation 12.5 degree for GEO, and 30 degree for LEO-600 km and 1200 km) Satellite orbit; GEO; LEO-1200; LEO-600. Satellite altitude; 35786 km; 1200 km; 600 km. Central beam edge elevation; 12.5 degree; 30 degree; 30 degree. Central beam center elevation; 20.9 degree; 46.05 degree; 43.8 degree. Payload characteristics for DL transmissions. Equivalent satellite antenna aperture (NOTE 1); S-band (i.e. 2 GHz); 12 m; 0.4m; 0.4 m. Satellite EIRP density; 59.8 dBW/MHz; 33.7 dBW/MHz; 28.3 dBW/MHz. Satellite Tx max Gain; 45.7 dBi; 16.2 dBi; 16.2 dBi. 3dB beam width (HPBW); 0.7353 degree; 22.1 degree; 22.1 degree. Satellite beam diameter (NOTE 2); 459km; 470 km; 234 km. Payload characteristics for UL transmissions. Equivalent satellite antenna aperture (NOTE 1); S-band (i.e. 2 GHz); 12 m; 0.4 m; 0.4 m. G/T; 16.7dB K-1; -12.8 dB K-1; -12.8 dB K-1. Satellite Rx max Gain; 45.7 dBi; 16.2 dBi; 16.2 dBi. NOTE 1: This value is equivalent to the antenna diameter in Sec. 6.4.1 of TR 38.811 NOTE 2: Satellite beam diameter is at Nadir point NOTE 3: Central beam center elevation is referred to as central beam elevation in TR 38.821 NOTE 4: Central beam edge elevation is the minimum beam edge elevation of the central beam in the beam layout. - Set 4 satellite parameters (Thales, Sateliot, Gatehouse R1-2101019) Satellite orbit; LEO-600. Satellite altitude; 600 km. Central beam edge elevation; 30 degree. Central beam center elevation; 90 degree. Payload characteristics for DL transmissions. Equivalent satellite antenna aperture (NOTE 1); S-band (i.e. 2 GHz); 0.097 m. Satellite EIRP density; 21.45 dBW/MHz. Satellite Tx max Gain; 11 dBi. 3dB beam width (HPBW); 104.7 degree. Satellite beam diameter (Note 2); 1700 km. Payload characteristics for UL transmissions. Equivalent satellite antenna aperture (Note1); S-band (i.e. 2 GHz); 0.097 m. G/T; - 18.6 dB·K-1. Satellite Rx max Gain; 11 dBi. NOTE 1: This value is equivalent to the antenna diameter in Sec. 6.4.1 of TR 38.811 NOTE 2: Satellite beam diameter is at Nadir point NOTE 3: Central beam center elevation is referred to as central beam elevation in TR 38.821 NOTE 4: Central beam edge elevation is the minimum beam edge elevation of the central beam in the beam layout.