3GPP TSG RAN WG1 #104bis-e			R1-2103270

e-Meeting, April 12th – 20th, 2021

Agenda Item:	7.2.5

Source:	Apple Inc.

Title:	Remaining issues on UCI enhancements for eURLLC

Document for:	Discussion/Decision

# 1. Introduction

In this contribution, we discuss some remaining issues on UCI enhancements for eURLLC.

# 2. UCI Enhancements

## 2.1 Sub-slot-based HARQ-ACK feedback and Multi-DCI based M-TRP

In Rel-15, a UE is not expected to transmit two HARQ-ACK PUCCH in the same slot. As part of Rel-16 Multi-DCI based Multi-TRP enhancement, separate HARQ-ACK PUCCH within a slot for different TRPs is supported in which UE can be configured to transmit two HARQ-ACK PUCCH in TDM way in the same slot. Correspondingly, FG16-2a-4 is agreed to indicate whether UE supports separate HARQ-ACK PUCCH for Multi-DCI based Multi-TRP operation with candidate values of bitmap {LongAndLong, LongAndShort, ShortAndShort} for the supported combinations of long/short PUCCH formats within a slot.

At the same time, Rel-16 URLLC enhancement agreed on sub-slot based HARQ-ACK feedback, where there can be one HARQ-ACK PUCCH per sub-slot. With this feature, the UE can support up to 2 or 7 HARQ-ACK PUCCHs per slot. Consequently, UE capability is designed to indicate whether UE supports sub-slot based HARQ-ACK feedback procedure, including FG11-3, FG11-3c/d/e/f, etc.

There was no discussion whether UE should support Multi-DCI based Multi-TRP operation  with sub-slot based HARQ-ACK feedback since these two features were designed independently by two different groups. As results, this is an unclear area without explicitly UE capability indication. As an example, if these two features can be used together, the UE would be required to support two HARQ-ACK PUCCHs within each sub-slot if separate HARQ-ACK feedback is configured for Multi-DCI based Multi-TRP, which is not covered by any of the existing UE FGs.

To resolves the issues, in our view, there are two alternatives:

- Alt 1: Introduce UE capability to indicate whether UE supports sub-slot based HARQ-ACK PUCCH when Multi-DCI based Multi-TRP operation is configured

- Alt 2: When UE is configured with Multi-DCI based Multi-TRP operation, the UE is not expected to be configured with sub-slot based HARQ-ACK PUCCH

In summary, we have the following proposal

Proposal 1: In terms of whether UE supports sub-slot based HARQ-ACK PUCCH when UE is configured with Multi-DCI based Multi-TRP operation, adopt one of the following solutions:

- Alt 1: Introduce UE capability to indicate whether UE supports sub-slot based HARQ-ACK feedback when Multi-DCI based Multi-TRP operation is configured

- Alt 2: When UE is configured with Multi-DCI based Multi-TRP operation, the UE is not expected to be configured with sub-slot based HARQ-ACK feedback. TP to 38.213 as follows

# 9 UE procedure for reporting control information <unchanged part omitted> In the remaining of this Clause, if a UE is provided subslotLengthForPUCCH-r16, a slot for an associated PUCCH transmission includes a number of symbols indicated by subslotLengthForPUCCH-r16. If, for an active DL BWP of a serving cell, the UE is not provided CORESETPoolIndex or is provided CORESETPoolIndex with value 0 for one or more first CORESETs, and is provided CORESETPoolIndex with value 1 for one or more second CORESETs, the UE is not expected to be configured with subslotLengthForPUCCH-r16 in any PUCCH-config of the active UL BWP. <unchanged part omitted>

## 2.2 Two HARQ-ACK codebooks and Multi-DCI based M-TRP

Rel-16 URLLC enhancement allows two HARQ-ACK codebooks to be configured with different priorities with corresponding UE capability FG11-4 and FG11-4a. It has not been discussed whether Multi-TRP Multi-DCI supports two HARQ-ACK codebooks with different priorities, especially when UE supports and is configured with ACKNACKFeedbackMode = SeparateFeedback. More importantly, the corresponding UE capability was not discussed during the Rel-16 UE feature discussion. If these two features can be used together, the UE would be required to support up to 4 HARQ-ACK PUCCHs in a slot, which is not covered by any of the existing UE FGs. In order to resolve the issue, we have the following proposal:

Proposal 2: In terms of whether UE supports two HARQ-ACK codebooks with different priorities when UE is configured with Multi-DCI based Multi-TRP operation, adopt one of the following solutions

- Alt 1: Introduce UE capability to indicate whether UE supports two HARQ-ACK codebooks with different priorities when Multi-DCI based Multi-TRP operation is configured

- Alt 2: When UE is configured with Multi-DCI based Multi-TRP operation, the UE is not expected to be configured with two HARQ-ACK codebooks with different priorities. TP to 38.213 as follows

## 9.1 HARQ-ACK codebook determination If a UE is provided pdsch-HARQ-ACK-Codebook-List, the UE can be indicated by pdsch-HARQ-ACK-Codebook-List to generate one or two HARQ-ACK codebooks. If the UE is indicated to generate one HARQ-ACK codebook, the HARQ-ACK codebook is associated with a PUCCH of priority index 0. If a UE is provided pdsch-HARQ-ACK-Codebook-List, the UE multiplexes in a same HARQ-ACK codebook only HARQ-ACK information associated with a same priority index. If the UE is indicated to generate two HARQ-ACK codebooks - a first HARQ-ACK codebook is associated with a PUCCH of priority index 0 and a second HARQ-ACK codebook is associated with a PUCCH of priority index 1 - the UE is provided first and second for each of {PUCCH-Config, UCI-OnPUSCH, PDSCH-codeBlockGroupTransmission} by {PUCCHConfigurationList, UCI-OnPUSCH-List, PDSCH-CodeBlockGroupTransmission-List}, respectively, for use with the first and second HARQ-ACK codebooks, respectively If, for an active DL BWP of a serving cell, the UE is not provided CORESETPoolIndex or is provided CORESETPoolIndex with value 0 for one or more first CORESETs, and is provided CORESETPoolIndex with value 1 for one or more second CORESETs, the UE is not expected to be configured to generate two HARQ-ACK codebooks. <unchanged part omitted>

For proposals 1 and 2, we have a preference of Alt 2, not supporting the simultaneous configurations of multi-DCI based multi-TRP operation and sub-slot-based HARQ-ACK feedback/two HARQ-ACK codebooks in Rel-16 for simplicity.

- We are at a late stage of Rel-16 maintenance, and the UE features are frozen already (even though new FG can still be added if necessary). Defining new FGs according to Alt 1 may not be very straightforward, because we have FG 11-3c/d/e/f and FG 11-4c/d/e/f/g/h that define the PUCCH capabilities (numbers and format combinations) for eURLLC features, and short/long PUCCH combinations defined in FG 16-2a-4. Careful investigation would be required to go through the list and identify the valid overall combinations.

- Multi-DCI based multi-TRP operation was designed for eMBB, not URLLC. The consequence of not allowing joint operation of the features is that multi-DCI based multi-TRP operation may not be used for eMBB traffic when a UE has mixed eMBB/URLLC traffic. So the impact is relatively limited.

# 4. Conclusions

In this contribution, we discussed the interaction between eURLLC HARQ-ACK enhancement features and multi-DCI multi-TRP, and remaining issues for PUSCH repetition Type B. We proposed:

Proposal 1: In terms of whether UE supports sub-slot based HARQ-ACK PUCCH when UE is configured with Multi-DCI based Multi-TRP operation, adopt one of the following solutions:

- Alt 1: Introduce UE capability to indicate whether UE supports sub-slot based HARQ-ACK feedback when Multi-DCI based Multi-TRP operation is configured

- Alt 2: When UE is configured with Multi-DCI based Multi-TRP operation, the UE is not expected to be configured with sub-slot based HARQ-ACK feedback. TP to 38.213 as follows

# 9 UE procedure for reporting control information <unchanged part omitted> In the remaining of this Clause, if a UE is provided subslotLengthForPUCCH-r16, a slot for an associated PUCCH transmission includes a number of symbols indicated by subslotLengthForPUCCH-r16. If, for an active DL BWP of a serving cell, the UE is not provided CORESETPoolIndex or is provided CORESETPoolIndex with value 0 for one or more first CORESETs, and is provided CORESETPoolIndex with value 1 for one or more second CORESETs, the UE is not expected to be configured with subslotLengthForPUCCH-r16 in any PUCCH-config of the active UL BWP. <unchanged part omitted>

Proposal 2: In terms of whether UE supports two HARQ-ACK codebooks with different priorities when UE is configured with Multi-DCI based Multi-TRP operation, adopt one of the following solutions

- Alt 1: Introduce UE capability to indicate whether UE supports two HARQ-ACK codebooks with different priorities when Multi-DCI based Multi-TRP operation is configured

- Alt 2: When UE is configured with Multi-DCI based Multi-TRP operation, the UE is not expected to be configured with two HARQ-ACK codebooks with different priorities. TP to 38.213 as follows

## 9.1 HARQ-ACK codebook determination If a UE is provided pdsch-HARQ-ACK-Codebook-List, the UE can be indicated by pdsch-HARQ-ACK-Codebook-List to generate one or two HARQ-ACK codebooks. If the UE is indicated to generate one HARQ-ACK codebook, the HARQ-ACK codebook is associated with a PUCCH of priority index 0. If a UE is provided pdsch-HARQ-ACK-Codebook-List, the UE multiplexes in a same HARQ-ACK codebook only HARQ-ACK information associated with a same priority index. If the UE is indicated to generate two HARQ-ACK codebooks - a first HARQ-ACK codebook is associated with a PUCCH of priority index 0 and a second HARQ-ACK codebook is associated with a PUCCH of priority index 1 - the UE is provided first and second for each of {PUCCH-Config, UCI-OnPUSCH, PDSCH-codeBlockGroupTransmission} by {PUCCHConfigurationList, UCI-OnPUSCH-List, PDSCH-CodeBlockGroupTransmission-List}, respectively, for use with the first and second HARQ-ACK codebooks, respectively If, for an active DL BWP of a serving cell, the UE is not provided CORESETPoolIndex or is provided CORESETPoolIndex with value 0 for one or more first CORESETs, and is provided CORESETPoolIndex with value 1 for one or more second CORESETs, the UE is not expected to be configured to generate two HARQ-ACK codebooks. <unchanged part omitted>

We prefer Alt. 2 for both cases.

# References