3GPP TSG RAN WG1 #104-bis-e		                               R1-2103310

e-Meeting, April 12th – April 20th, 2021

Source: 	Sony

Title:	Discussion on potential paging enhancements

Agenda Item:	8.7.1.1

Document for:	Discussion

# Introduction

In RAN1 #104e-meeting [1], the following observations and proposals were agreed and concluded.

Agreements:

- Carrying UE subgroups information is considered in physical layer design for paging enhancement

Agreements:

For the evaluation and comparison of PEI candidate designs based on PDCCH, TRS/CSI-RS and SSS, the following are assumed:

- Behv-A:

  - PEI indicates UE should monitor a PO if UE’s group/subgroup is paged

  - UE is not required to monitor a PO if UE does not detect PEI at all PEI occasion(s) for the PO

- Behv-B:

  - PEI indicates whether or not UE should monitor a PO

  - UE is required to monitor a PO if UE does not detect PEI at all PEI occasion(s) for the PO

Agreements:

For the evaluation and comparison of PEI candidate designs, companies to report

- Description of how PEI design can co-exist with existing channels/signals, and impact to legacy UEs.

  - Rel-15 designs for multiplexing PEI with legacy channels/signals are assumed as baseline

o   Other multiplexing method with legacy channels/signals can be additionally reported with justification

Agreement:

- Take Alt 1 as mandatory, and Alt 2 as optional

Alt 1

For the performance evaluations of PEI candidate designs based on PDCCH, TRS/CSI-RS and SSS,

- The following are assumed, at the SNR where the Miss-Detection Rate (MDR) of paging PDSCH is 1%,

  - When Behv-A is assumed:

    - The joint miss-detection rate (MDR) of PEI and paging PDCCH defined below should be no worse than 1%:

MDR_Joint_A = MDR_PEI + (1 – MDR_PEI) MDR_PagingPDCCH

    - The False-Alarm Rate (FAR) of PEI should be no larger than [1%]

  - When Behv-B is assumed:

    - The joint miss-detection rate (MDR) of PEI and paging PDCCH defined below should be no worse than 1%:

MDR_Joint_B = FAR_PEI + (1 – FAR_PEI) MDR_PagingPDCCH

    - The MDR of PEI should be no larger than [1%]

  - Note: The CFO is modeled at the input of PEI detection and based on LLS assumptions agreed in RAN1 #102-e. Companies should justify the applied random range for the CFO.

- Companies to provide:

  - Information on the utilized detection method for each PEI candidate design (e.g., non-coherent detection or coherent detection)

  - The required #REs to comply with the performance assumptions

  - The maximum number of subgroups that can be carried in PEI, subject to the performance assumptions

Alt 2

For the performance evaluations of PEI candidate designs based on PDCCH, TRS/CSI-RS and SSS,

- The following are assumed, at the SNR where the Miss-Detection Rate (MDR) of paging DCI is 1%,

  - When Behv-A is assumed:

    - The MDR of PEI should be no larger than 0.1%

    - The False-Alarm Rate (FAR) of PEI should be no larger than 1%

  - When Behv-B is assumed:

    - The FAR of PEI should be no larger than 0.1%

    - The MDR of PEI should be no larger than 1%

  - Note: The CFO is modeled at the input of PEI detection and based on LLS assumptions agreed in RAN1 #102-e. Companies should justify the applied random range for the CFO.

- Companies to provide:

  - Information on the utilized detection method for each PEI candidate design (e.g., non-coherent detection or coherent detection)

  - The required #REs to comply with the performance assumptions

  - The maximum number of subgroups that can be carried in PEI, subject to the performance assumptions

Agreements:

For the evaluation of resource overhead with PEI candidate designs based on PDCCH, TRS/CSI-RS and SSS, companies to provide estimated overheads for PEI candidate designs based on the following factors:

- Assumption of Behv-A/B

- Required #REs from performance evaluations

- 10% group paging rate per PO as baseline; other group paging rates can be optionally considered

and based on the following assumptions with justification (up to each company)

- Whether and how coexistence with legacy UEs is considered

- Whether and how indication(s) to multiple POs and/or UE subgroups by one PEI is considered

- Whether and how multi-beam transmission is considered

Agreements:

Further study the design on how to provide the indications for UE subgroups over PEI and/or paging PDCCH, subject to the metrics agreed in RAN1 102e.

In this contribution, we discuss candidate designs for paging early indication (PEI) and address aspects related to UE sub-grouping. We also discuss aspects related to co-existence of PEI with existing channels/signals and impact to legacy UEs.

# Discussion

During IDLE/INACTIVE mode, an NR UE needs to monitor the channel for potential paging and performs measurements and checks the signal strength of the serving cell and if necessary, neighbor cells for cell re-selection.

When monitoring for paging, the UE needs to first monitor the PO for a paging DCI, i.e., a DCI whose CRC is scrambled with P_RNTI. If a paging DCI is detected, the UE continues to the second step where the UE decodes the PDSCH for its TMSI (5G-S-TMSI). Details of the frequency and time resource allocation of PDSCH are provided in the paging DCI. The UE knows whether it is paged or not only after it successfully decodes PDSCH (paging message). Both idle-channel monitoring and paging-DCI overhearing lead to extra costs at the UE. Since the UE needs to be fully synchronized to be able to decode paging DCI and a paging message, these additional costs can become significant for instance for scenarios where the UE is not in very good coverage. The total extra costs depend on the paging probability and how PO/DRX of UEs with different paging probability are configured. Figure 1 shows an example power consumption break-down of a UE for two paging group rates (or paging rate per PO) of 10% and 60% and when at cell edge. The group paging rate is calculated according to 1-(1-Pue)N , where the number of UEs, N, is 10 or 100 and the UE’s paging rate Pue is 1%. 3 SSBs are considered for synchronization.

Figure 1 - Power consumption break-down of a UE in bad coverage

As can be seen, the cost of transition from/to sleep states and synchronization necessary to decode paging DCI and paging message is about 50% - 60% of the total average power consumption in one DRX cycle. This means that even though the cost for idle PO monitoring and overhearing/false-paging is relatively low, the actual total cost can become significant when the synchronization and transition cost are taken into account.

Observation 1 – The cost of transition from/to deep sleep as well as synchronization cost are dominant sources of power/energy consumption.

To reduce the cost of unnecessary paging and for paging enhancement, we need to provide solutions where the UE can avoid entering power consuming transition and synchronization states when there is no paging for the target UE.

Proposal 1 – Paging enhancement schemes should avoid the UE unnecessarily transitioning from/to sleep states and from/to synchronization states, when there is no paging for a target UE.

## Paging early indication (PEI)

Paging early indication before a paging occasion is supported from the RAN1 perspective for NR idle/inactive-mode paging enhancement. In this scheme an indicator before a target PO is transmitted. Two types of information can be carried in the indicator: i) a wake-up signal (referred to as Behv-A in the above agreement) or ii) a wake-up/go-to-sleep signal (referred to as Behv-B in the above agreement). In Behv-A the PEI is only transmitted when there is a paging for a UE or group of UEs and after detection of the PEI, UE continues to monitor for PDCCH scrambled with P-RNTI and a paging message at the PO. In Behv-B, however, the PEI is always transmitted to indicate to the UE whether to wake-up and continue to monitor for PDCCH scrambled with P-RNTI and a paging message at the PO or to go back to sleep. While system overhead and impact on co-existence with other signals/channels and legacy UEs for the two behaviours are significantly different, the energy cost of channel monitoring for PEI at the UE for the two behaviours is the same when the same design is selected among the choices of signal design for PEI.

Observation 2 – While system overhead and impact on co-existence with other signals/channels and legacy UEs for the two behaviours are significantly different, the energy cost of channel monitoring for PEI at the UE for the two behaviours is the same when the same design is selected among the choices of signal design for PEI.

Details of the PEI physical layer design is still under discussion. In this section we discuss the three candidate designs, PDCCH/DCI-based, SSS-based, or TRS/CSI-RS-based designs. The analysis and discussion below apply to both behaviors. Details related to co-existence and system overhead are discussed in section 2.3.

- In a DCI-based design, the UE needs to be fully synchronized to the network to be able to decode the DCI-based PEI. This means that the UE needs to decode SSB prior to its PEI decoding/detection. Even if the DCI-based PEI is placed as close as possible to an SSB signal, the UE still needs to receive and decode both SSB and DCI-based PEI before it decides whether or not there is paging. This means a DCI-based PEI design does not fulfil the condition of proposal 1 as it needs to receive synchronization in advance for its detection, leading to extra unnecessary power consumption.

- A TRS/CSI-RS based design requires a longer time and wider bandwidth than the other two designs to accommodate the signal content/information. This means longer time and higher received power consumption and consequently relatively higher power consumption for its detection, compared to the other candidates.

- By using a sequence-based PEI design, having similar characteristics as SSS, the UE does not need to do any synchronization prior to its PEI detection. The UE proceeds to read SSB and PO only if the SSS-based PEI is detected. This basically means that the SSS-based PEI scheme fulfils the condition in proposal one, resulting in higher power saving gain compared to a DCI-based solution.

Observation 3 – Sequence-based (e.g. SSS-based) early paging indication fulfils the condition that paging enhancement schemes should avoid the UE unnecessarily transitioning from/to deep sleep and from/to synchronization states, when there is no paging for target UE.

Practically/in principle only a correlator is needed to decode/detect an SSS-based PEI. This means that the power consumption of the receiver when decoding/detecting SSS-based PEI is basically lower than the power consumption of the receiver when receiving an SSB signal, as assumed above. Moreover, the SSS-based PEI can also be used for synchronization purposes. This also means fewer SS bursts are needed for synchronization prior to PO for decoding of paging DCI and a paging message. Lower power consumption for PEI detection and fewer SS burst for synchronization before a PO can result in a higher power saving gain than the other designs.

Proposal 2 – Use sequence-based early paging indication as a paging enhancement scheme to reduce idle PO monitoring cost at the UE.

## UE Sub-grouping

UE sub-grouping is considered as another paging enhancement technique and is used to reduce unnecessary paging reception resulting from false paging or overhearing. In this technique, UEs monitoring the same PO are divided into subgroups, and the network needs to indicate whether a subgroup of UEs needs to monitor the paging message.

In RAN2#112e-meeting [4], the following candidate UE subgrouping schemes were agreed, for further investigation:

- cross-slot scheduling for UE subgroup,

- paging early indication or wake-up signal (WUS) for UE subgroup

- paging indication for UE subgroup using paging DCI,

while the other candidate solutions, i.e., using additional reception occasion in time/frequency domain for UE subgrouping and P-PNTI based subgroup discrimination were deprioritized. Below we discuss the subgrouping schemes from a signalling and power saving perspective. When discussing, we assume the PEI design has SSS-type characteristics since it has the lowest power consumption compared to the other two designs. The analysis applies to both Behv-A and Behv-B.

- A UE being sub-grouped using cross-slot scheduling needs to be fully synchronized with the network to be able to decode the paging DCI and decide whether there is a paging DCI for its target subgroup. This makes entering synchronization states as well as transition to/from deep sleep unavoidable. Consequently, the power saving gain of this scheme compared to the same slot-scheduling is very limited as it cannot fulfil the conditions in proposal one. An additional important disadvantage is that this approach is not always backward compatible.

- UE subgrouping in an SSS-based PEI is done by using different sequences for different UE subgroups in the indicator. Since each sequence carries only one bit of information, in order to allow paging of different UEs subgroups at the same time the sequences, at transmission time, need to be time and/or frequency and/or code multiplexed. Additionally, a common sequence known to all UEs can be used for scenarios where all UEs need to be paged at the same time, e.g. for notification of SI change and ETWS indication.

- UE subgrouping using legacy paging DCI is an approach combined with PEI. In this approach, the SSS-based PEI carries only one-bit of information and is used for wake-up. The reserved bits in the legacy paging DCI are used to further split UEs listening to the same POs. Only after decoding of the paging DCI can a UE decide whether its subgroup is paged. When the paging rate per PO is low, the average power saving consumption of this approach is very similar to when sub-grouping is carried by SSS-based PEI. The power consumption increases when the paging rate per PO increases. This is since a UE only after synchronization and decoding of subgrouping paging DCI can decide whether its subgroup is paged.

Observation 4 – Among the three sub-grouping candidates, UE sub-grouping using sequenced-based/SSS-based early paging indication fulfils the condition that paging enhancement schemes should avoid the UE unnecessarily transitioning from/to deep sleep and from/to synchronization states, when there is no paging for a target UE.

### Power saving gain analysis

We have done further evaluation where we compare the power saving gains of these schemes with respect to the baseline. Again, the power saving gains are calculated for two paging rates per PO of 10% and 60% and for two coverage conditions. In our calculation, we assume

- 1 SS burst for synchronization before PO reception for good coverage and 3 SS bursts for UEs in bad coverage,

- a time duration TPEI of two OFDM symbols and a power consumption the same level as PSSB for SSS-based PEI (practically this can be lower since only a correlator is used for its detection),

- An average length/duration (TPEINbeam2) of 8 and 32 OFDM symbols for monitoring the channel for PEI detection in good and bad coverage, respectively (for this we assume 8 beams and no SSB is considered prior to PEI for synchronization.).

- miss detection and false-alarm probabilities of 1% for both PEI detection as well as subgrouping.

UEs with 10% paging rate per PO are further divided to two sub-groups each containing 5 UEs and UEs with 60% paging rate per PO to five sub-groups, each containing 20 UEs. Through sub-grouping the paging rate per PO for each sub-group is reduced from 10% and 60% to 4.5% and 18%, respectively. All the other parameters are set according to evaluation assumptions in [5].

The power saving gains for these studied scenarios are listed in Table 2. As expected, the amount of power saving gain resulting from cross-slot scheduling is very limited. For both SSS-based PEI UE grouping and combined PEI and UE sub-group using paging DCI, the amount of power saving gain for 10% paging rate per PO is very similar. This is because the amount of power saving is more related to the reduction of listening to idle paging rather than to a reduction of false paging. Increasing the paging rate to 60%, the cost of false paging increases and thereby the amount of power saving gain for UE subgrouping using paging DCI becomes up to ~13% lower than the one using PEI for subgrouping.

Table 2 - Summary of power saving gains for different UE subgrouping techniques in percentage

| UE sub-grouping technique | Low SINR/bad coverage |  | High SINR/good coverage |  |
| --- | --- | --- | --- | --- |
|  | N=10/paging rate per PO = 10% | N=100/paging rate per PO = 60% | N=10/paging rate per PO = 10% | N=100/paging rate per PO = 60% |
| Cross-slot scheduling | 0.32 | 3.0 | 0.1 | 0.99 |
| SSS-based PEI carries UE subgrouping info. | 12.66 | 13.67 | 27.56 | 25.02 |
| SSS-based PEI + UE subgroup using paging DCI | 12.58 | 9.93 | 26.28 | 11.77 |

Observation 5 – The amount of power saving gain for high paging rate per PO is much higher when UE grouping is carried in a sequence-based early paging indicator than when UE sub-grouping is done using paging DCI.

Proposal 3 – Use sequence-based early paging indicator with sub-grouping for paging enhancement and reducing unnecessary paging reception.

## System overhead and co-existence

Above we have shown that SSS-based PEI with sub-grouping can lead to higher power savings compared to all the other signal designs and subgrouping schemes. In this section we discuss SSS-based PEI for the two behaviors, i.e., Behv-A and Behv-B from system overhead and co-existence with legacy UEs and other existing signaling perspectives.

### System overhead

As described in section 2.1, PEI in Behv-A is only transmitted when there is a paging for a UE or group of UEs and after detection of the PEI, the UE continues to monitor for PDCCH scrambled with P-RNTI and a paging message at the PO. In Behv-B, however, the PEI is always transmitted to indicate to the UE whether to wake-up and continue to monitor for PDCCH scrambled with P-RNTI and a paging message at the PO or go back to sleep. While the two behaviours lead to increasing of the resources compared to legacy, the amount of resource overhead in Behv-B is much higher than Behv-A. For a certain number of UEs forming a sub-group the resource overhead for Behv-A and Behv-B become:

- resource overhead of a certain UEs sub-group in Behv-A:

(subgroup paging rate)×(resources for PEI TX + resources for paging DCI and paging message TX)(paging rate) ×(resources for paging DCI and paging message TX)

- resource overhead of a certain UEs sub-group in Behv-B: resources for PEI TX + (subgroup paging rate)× resources for paging DCI and paging message TX)(paging rate) ×(resources for paging DCI and paging message TX)

We use the paging rates and sub-group paging rate in the example in section 2.2 and calculate the resource overhead based on the above two formulations. For UEs with 10% paging rate per PO and sub-group rate of 4.5%, resource overhead is around 1.35 times higher for Behv-A compared to legacy. Using an indication based on Behv-B the resource overhead increases 20.45 times compared to the one in legacy, around 15 times higher than the overhead in Behv-A. Similarly, for UEs with 60% paging rate per PO and sub-group rate of 18% the resource overhead increases 5.4 times for Behv-A. Using an indication based on Behv-B the resource overhead becomes 22 times higher, around 4 times higher compared to Behv-A.

Observation 6 – Resource overhead in Behv-A is lower than the one in Behv-B since the PEI does not need to always be transmitted.

Proposal 4 – Use an SSS-based PEI based on Behv-A, i.e., use a PEI where the indicator is only transmitted when there is a paging available for the target UE or sub-group of UEs.

### Co-existence with existing signals/channels and legacy UEs

As shown in Fig. 3, a PEI is transmitted with a certain time offset (here it is 50 msec) before a PO. When scheduling the PEI, there is a certain probability that the PEI transmission coincides with transmission of existing signals/channels such as SSB or SIB of other UEs. There is also a certain probability that the PEI transmission coincides with DL transmission of legacy UEs.

Figure 3 - Early paging indication for a UE in good coverage – with paging

Observation 7 - There is a certain probability that PEI transmission may potentially collide with DL transmission to a legacy UE or DL transmission of existing signals/channels to other UEs.

To allow a flexible scheduling and avoid any blocking when a DL transmission coincides with PEI a new mechanism needs to be added to the ongoing PEI scheme. A potential enhancement to the PEI scheme is inclusion of a timing (or timing and frequency) window for scheduling/monitoring of PEI. This means that instead of having only one occasion for PEI transmission, a window with multiple opportunities for PEI transmission/reception is considered for supporting co-existence of resources that can be used for other traffic. From these multiple opportunities, basically, the gNB selects the one that the PEI transmission would not block other legacy transmissions.

Proposal 5 – A window for PEI transmission/reception prior to PO is supported to avoid any blocking when other DL transmissions coincide with PEI.

Proposal 6 – Signaling aspects on conveying the configuration of the PEI transmission/reception window and UE/network behavior on PEI reception/transmission are FFS.

## Paging early indication (PEI) for UEs operating in FR2

## All the analysis above is done for the scenarios where multiplexing pattern 1 for SS/PBCH block and CORESET multiplexing is used (SS/PBCH block and CORESET are TDMed). When a UE operates in FR2, SS/PBCH block and CORESET multiplexing can be FDMed, referred to as multiplexing pattern 2 and pattern 3. The use of sequence-based early paging indicator together with UE sub-grouping is also beneficial here since the UE does not need to monitor a wider bandwidth for its paging detection. For these two settings the early paging indicator can be TDMed and the UE continues to synchronization and paging DCI/paging message reception states if a sequence is transmitted and successfully detected.

Observation 8 – The use of sequence-based early paging indicator is also beneficial for the FR2 case since the UE does not need to monitor a wider bandwidth for its paging detection.

Proposal 7 - Use sequence-based early paging indicator with sub-grouping for paging enhancement for FR2 operation.

# Conclusion

In this contribution, the following observations and proposals are made:

Observation 1 – The cost of transition from/to deep sleep as well as synchronization cost are dominant sources of power/energy consumption.

Observation 2 – While system overhead and impact on co-existence with other signals/channels and legacy UEs for the two behaviours are significantly different, the energy cost of channel monitoring for PEI at the UE for the two behaviours is the same when the same design is selected among the choices of signals design for PEI.

Observation 3 – Sequence-based (e.g. SSS-based) early paging indication fulfils the conditions: that paging enhancement schemes should avoid the UE unnecessarily transitioning from/to deep sleep and from/to synchronization states, when there is no paging for target UE.

Observation 4 – Among the three sub-grouping candidates, UE sub-grouping using sequenced-based/SSS-based early paging indication fulfils the condition that paging enhancement schemes should avoid the UE.

Observation 5 – The amount of power saving gain for high paging rate per PO is much higher when UE grouping is carried in a sequence-based early paging indicator than when UE sub-grouping is done using paging DCI.

Observation 6 – Resource overhead in Behv-A is lower than the one in Behv-B since the PEI does not need to always be transmitted.

Observation 7 - There is a certain probability that PEI transmission may potentially collide with Dl transmission to a legacy UE or DL transmission of existing signals/channels to other UEs.

Observation 8 – The use of sequence-based early paging indicator is also beneficial for the FR2 case since the UE does not need to monitor a wider bandwidth for its paging detection.

Proposal 1 – Paging enhancement schemes should avoid the UE unnecessarily transitioning from/to sleep states and from/to synchronization states, when there is no paging for a target UE.

Proposal 2 – Use sequence-based early paging indication as a paging enhancement scheme to reduce idle PO monitoring cost at the UE.

Proposal 3 – Use sequence-based early paging indicator with sub-grouping for paging enhancement and reducing unnecessary paging reception.

Proposal 4 – Use an SSS-based PEI based on Behv-A, i.e., use a PEI where the indicator is only transmitted when there is a paging available for the target UE or sub-group of UEs.

Proposal 5 – A window for PEI transmission/reception prior to PO is supported to avoid any blocking when other DL transmissions coincide with PEI.

Proposal 6 – Signaling aspects on conveying the configuration of the PEI transmission/reception window and UE/network behavior on PEI reception/transmission are FFS.

Proposal 7 – Use sequence-based early paging indicator with sub-grouping for paging enhancement for FR2 operation.

# References

- Chairman’s Notes RAN1#104e

- TR 38.840, “Study on UE power saving”, RAN1.

- R1-2008368 - Considerations on paging enhancements

- Minutes RAN2#112e

- Chairman's Notes RAN1#102-e final