3GPP TSG RAN WG1 #104b-e                                       R1-2103326

e-Meeting, April 12th – 20th, 2021

Source: 	ETRI

Title:	Enhancements for unlicensed band URLLC/IIoT

Agenda Item:	8.3.2   Enhancements for unlicensed band URLLC/IIoT

Document for:	Discussion/Decision

Introduction

In this contribution, we discuss remaining issues on enhancement of channel access and CG-PUSCH transmission for support of unlicensed URLLC operation in FBE.

Channel access enhancements for FBE

FFP period and offset for UE-initiated COT

Agreement: (RAN1#104-e) - In semi-static channel access mode, UE FFP periodicity is chosen from the following set of values in ms: {1, 2, 2.5, 4, 5,10}. - FFS on other values Agreement: (RAN1#104-e) - In semi-static channel access mode: - An FFP period for UE-initiated COT is configured as the same, integer multiple of, or inter-factor of the FFP period configured for gNB-initiated COT - FFP period for UE-initiated COT can be configured independently from FFP period of gNB-initiated COT, if the UE indicates the corresponding capability - FFP offset for UE-initiated COT is the starting point of first UE FFP relative to the radio frame X boundary. - The offset value range is 0 ≤ offset ＜FFP period of UE-initiated COT - FFS on X (e.g. X=0, or X= even index number)

As captured above [1], the supported FFP periodicity values for UE-initiated COT were decided as {1, 2, 2.5, 4, 5, 10} ms. Accordingly, for the value of X, taking X=even index number is a natural choice. Adding more value(s) for the periodicity may increase the flexibility of CG-PUSCH configuration to some extent but is not considered necessary. For the FFP offset, it is good to clarify that any value within 0 <= offset < UE FFP period with 1-symbol granularity can be configured. When multiple numerologies are configured for a cell, a numerology having the smallest SCS can be used as the reference numerology similar to the approach that was used for slot format configuration/indication in Rel-16.

Proposal 1: FFP offset for UE-initiated COT can be any value within 0 <= offset < UE FFP period with 1-symbol granularity based on the smallest SCS configured in the cell.

Proposal 2: FFP offset for UE-initiated COT is the starting point of first UE FFP relative to the boundary of the even indexed radio frame.

RRC signalling enabling/disabling COT initiation

In the last meeting, the following was discussed [2].

Updated Proposal 3-2: - In semi-static channel access mode, down-select one of the following alternatives: - Alt-1: As part of a CG UL configuration, enable/disable UE COT-initiating functionality for the CG UL transmissions aligned with the configured UE FFP boundaries. Details FFS. - Alt-2: As part of UE FFP configuration, enable/disable UE COT-initiating functionality for the UL transmissions aligned with a set of UE FFP boundaries. Details FFS. - FFS on other alternatives.

gNB may want to cancel/disable UE’s COT initiation in certain UE FFPs even if a valid configured UL is allocated in the beginning. For this purpose, UE can be informed a set of UE FFPs where COT initiation is enabled/disabled based on RRC signalling. In our view, that information should be as part of UE FFP configuration to be generally applicable for all UL transmissions.

Proposal 3: As part of UE FFP configuration, enable/disable UE COT-initiating functionality for the UL transmissions aligned with a set of UE FFP boundaries.

COT-initiator determination

Agreement: (RAN1#104-e) In semi-static channel access mode when a UE can operate as UE-initiated COT, - Select one of the following alternatives to determine whether a configured UL transmission that is aligned with a UE FFP boundary and ends before the idle period of that UE FFP, is based on UE-initiated COT or sharing a gNB-initiated COT: - Alt-a: If the transmission is confined within a gNB FFP before the idle period of that gNB FFP, and the UE has already determined that gNB is initiated that gNB FFP, UE assumes that the configured UL transmission corresponds to gNB-initiated COT. Otherwise, UE assumes that the configured UL transmission corresponds to UE-initiated COT - Alt-b: The UE assumes that the configured UL transmission corresponds to UE-initiated COT.

To move forward on this issue, i.e., the issue of COT initiator determination for a configured UL that is aligned with a UE FFP boundary, down-selection between Alt-a and Alt-b is required [1]. According to Alt-a, UE should check before the configured UL transmission 1) whether the configured UL is configured within a gNB COT, 2) whether a DL burst in the gNB COT was already detected. Depending on the outcome, UE may or may not perform channel sensing to initiate a COT in the UE FFP. However, in Alt-b, such procedure is not required. UE can simply initiate a COT and transmit the configured UL. gNB can still disable the UE’s COT initiation as needed by allocating the configured UL not to be aligned with the FFP boundary, or potentially by enabling/disabling the UE’s COT initiation based on signalling (if such signalling is introduced). The only risk we observe with Alt-b is that UE may fail to succeed in the CCA, but the probability of that event would be very small in controlled environments.

When the UL repetition transmission is considered, one clarification seems necessary: whether the configured UL in the agreement stands for each repetition/instance or the whole repetitions/instances. In our understanding, the COT initiator needs to be determined per each repetition. Assuming that, it would also be desirable that UE expects a configured UL (each repetition) starting from a UE FFP boundary to be always confined within the UE-initiated COT.

Proposal 4: If a configured UL transmission is aligned with a UE FFP boundary and ends before the idle period of that UE FFP (and if COT initiation is allowed in that UE FFP (up to Proposal 3)), UE assumes that the configured UL transmission corresponds to UE-initiated COT (Alt-b), with a clarification that “a/the configured UL transmission” is each UL repetition in case of UL repetition transmission.

Proposal 5: UE expects that a configured UL starting from a UE FFP boundary is confined within the UE-initiated COT.

For the scheduled UL transmission, the following agreement was made in the last meeting [1]:

Agreement: In semi-static channel access mode when a UE can operate as initiating device, - Select one of the following alternatives to determine whether a scheduled UL transmission is based on UE-initiated COT or sharing a gNB-initiated COT: - Alt-a: Determination based on the content in the scheduling DCI - FFS on whether the corresponding field(s) can be absent in DCI - If absent, determination based on the rules applied for configured UL transmissions is applied - FFS whether/how to handle the case when the gNB schedules an UL transmission in the next gNB’s FFP period - Alt-b: Determination based on the rules applied for a configured UL transmission

Basically, since a scheduled UL transmission is initiated by the scheduling DCI, it is no harm to add the COT-initiator information to the scheduling DCI to dynamically control the UE’s COT initiation, instead, it provides more operation options to gNB scheduler. UE can just follow the gNB’s instruction without checking any condition (that is required for the configured UL). Moreover, the COT initiation indication can be realized with no additional DCI overhead by reusing the ChannelAccess-CPext field. However, it is noted that the ChannelAccess-CPext field is an optional field, and not all scheduling DCIs have that field. For example, DCI 2_3 for SRS request does not have the field. It is not forbidden for now that UE-initiated COT starts with an aperiodic SRS triggered by DCI 2_3, and in that case some fallback mechanism is required to UE to determine whether to initiate a COT or not. If the field is absent in the DCI, the COT initiator determination can be done based on predetermined rules, e.g., the rules applied for configured UL.

Proposal 6: For a scheduled UL transmission, the COT initiator determination is based on the content in the scheduling DCI. If the content is absent, the determination is based on the rules applied for configured UL transmissions.

Indication of gNB-to-UE COT sharing

When UE detects a DL burst from gNB, it needs to determine whether the DL burst is transmitted according to a gNB COT or a shared UE COT. The following behaviour can be considered.

- If UE determines that the DL burst belongs to a gNB COT, it can share the gNB COT and perform UL transmission in the remaining gNB COT duration.

- If UE determines that the DL burst does not belong to a gNB COT,

  - If the UE initiated a COT including the DL burst before the DL burst reception (as UE A in Fig. 1), it assumes that the gNB shared that UE-initiated COT (regardless of whether the gNB actually shared that UE’s COT or other UE’s COT (UE cannot distinguish them)).

  - Otherwise (as UE B in Fig. 1), the UE assumes that the gNB shared other UE’s COT, and does not transmit UL according to the gNB COT in the remaining duration.

Fig. 1. Ambiguity in DL burst detection

It seems difficult for UE to distinguish 1) and 2) by itself. Thus, the gNB needs to indicate to UE whether the DL burst is transmitted according to a gNB COT or not. For the signalling method, several alternatives can be considered:

- Alt. 1: Implicitly known to UE based on position of the DL burst. For example, UE assumes that the DL burst belongs to a gNB COT if it detects the DL burst (or DCI 2_0) from the beginning of the gNB FFP.

- Alt. 2: Based on existing field in DCI 2_0, e.g., by assigning zero to the CO duration indication field.

- Alt. 3: Based on DCI 2_0 introducing a new field.

Among the alternatives, Alt. 1 is slightly preferred since it does not mandate transmission of DCI 2_0.

Proposal 7: For gNB-to-UE COT sharing in FBE, UE determines whether a detected DL burst was transmitted according to a gNB-initiated COT or a UE-initiated COT based on (implicit or explicit) indication from gNB.

Channel access rule when both gNB and UE FFPs are configured

In a past meeting, it was recommended to discuss further on the following:

For semi-static channel access mode, decide which of the following rules are supported: - A UE can initiate a COT within a Gnb-initiated COT, and Gnb can initiate a COT within a UE-initiated COT - As initiating device, the Gnb can transmit during any UE FFP idle periods. - As initiating device, a UE can transmit during other UEs FFP idle periods. - A responding device can still transmit in the shared COT even if its transmission collides with the idle period of FFP configured for the responding device - A UE may operate as an initiating device within a Gnb’s FFP only in case that Gnb has not initiated that FFP. - By default, a UE is not allowed to transmit during the idle period of any FFP associated with the serving Gnb (regardless whether the serving Gnb initiates a COT in that FFP).

Based on the progress so far, we think that a)~d) need to be supported, but e) and f) are unnecessary restrictions. If there is no expected spec impact, they can be drawn as conclusion.

Proposal 8: For FBE, the following rules are supported:

- A UE can initiate a COT within a gNB-initiated COT, and gNB can initiate a COT within a UE-initiated COT.

- As initiating device, the gNB can transmit during any UE FFP idle periods.

- As initiating device, a UE can transmit during other UEs FFP idle periods.

- A responding device can still transmit in the shared COT even if its transmission collides with the idle period of FFP configured for the responding device.

UE-initiated COT for idle/inactive mode

There is continuing discussion on support of UE-initiated COT for idle/inactive UEs. In our view, enhancement of the idle mode operation is not justified by the WID. In addition, it is not clear whether supporting UE-initiated COT in idle mode improves the initial access performance. As an implementation, gNB may initiate its COT (rather than relying on UE-initiated COT) in FFPs where PRACH resources are configured, which seems not a big burden but is a typical operation to be ready to serve aperiodic URLLC traffic immediately. Accordingly, it is preferred to not support the UE-initiated COT for idle/inactive UEs at least in Rel-17.

Proposal 9: For FBE, do not support UE-initiated COT for idle/inactive UEs.

Cross-FFP scheduling

There was discussion on whether cross-FFP scheduling is allowed or not by regulation. In our understanding, there is no clue to prohibit cross-FFP scheduling in the ETSI specification. Rather, UL/DL scheduling needs not be restricted by the channel access procedure. For example, UL transmission can be configured without UL grant, or scheduled by UL grant. In the latter case, the UL grant and the scheduled UL can be transmitted on the same FFP or different FFPs, and on the same channel or different channels. Taking the PUSCH as an example, we think that all four cases in Fig. 2 need to be not precluded. Besides, if a UL is transmitted based on the repetition, the scheduling rule may be applied to each UL repetition.

Proposal 10: Clarify that for scheduled UL, the FFP/channel where a UL (each UL repetition in case of repetition) is scheduled can be same or different from the FFP/channel where an associated UL grant is transmitted.

Fig. 2. Cases for PUSCH transmission in FBE

Processing time for gNB-to-UE COT sharing

Referring to Case 1 in Fig. 2, the PUSCH transmission is validated by the detection of the UL grant. Thus, a separate DL detection is not required for UE to transmit the PUSCH. However, in the remaining three cases, the UL grant is not present in the same gNB COT. Therefore, UE is required to detect other DL signal/channel before transmitting the PUSCH for its validation.

On the other hand, the completion of the DL detection requires some processing time to UE. If not defined in the spec, the processing time for DL detection would be different among UEs depending on their capability or implementation. Then, gNB may not know the exact timing from which UE is ready to transmit UL in a shared gNB COT. This issue is illustrated in Fig. 3.

Fig. 3. Processing time for UE’s DL detection

In Fig. 3, UE A may be capable to decode the DL signal in a shared COT until before the starting position of the CG-PUSCHs. Thus, UE A can transmit the CG-PUSCHs. However, UE B may not able to finish the DL detection at the starting position of the 1st CG-PUSCH. Then, UE B may not transmit the 1st CG-PUSCH. Depending on cases, UE B may drop the whole CG-PUSCH transmission including possible repetitions, which may severely degrade the UL delay performance for a TB (and also for piggybacked UCI). In Rel-16 NR-U, the delay due to UL dropping may not be critical because eMBB is the main use case. However, such a relaxed delay requirement is not applicable for high end URLLC applications. Furthermore, the gNB may perform blind detection for UL reception because it does not know whether the UL is actually transmitted or not, especially if the UL is allocated in the earlier part of the gNB COT.

Therefore, in order to avoid the UL scheduling uncertainty, it is necessary to define the processing time for UE’s DL detection (i.e., for validation of each UL transmission in a shared COT). Although this issue is not directly related to introduction of UE-initiated COT, it is considered essential because it may highly impact the UL reliability performance for unlicensed URLLC.

Observation 1: The UL reliability performance of unlicensed URLLC can be severely degraded if UE’s processing time for DL detection to share a COT is unknown to gNB.

Proposal 11: For gNB-to-UE COT sharing, define UE’s processing time required for DL burst detection and UL transmission preparation.

The same issue is identified in the UE-to-gNB COT sharing. Since gNB should spend some time for UL burst detection, gNB can realize whether sharing a UE COT is possible or not after some delay from the UL reception. If UE can know the gNB’s processing time for UL detection, UE may skip the DL reception operation (e.g., micro-sleep) during that time for power saving purpose.

Proposal 12: For UE-to-gNB COT sharing, consider defining processing time for gNB’s UL burst detection for UE power saving purpose.

Fig. 4. Processing time for gNB’s UL detection

CP extension for CG-PUSCH

In Rel-16 NR-U, CP extension can be applied to CG-PUSCH to allow PUSCH overloading. Different UEs can apply different CP extensions for their CG-PUSCH transmissions starting from the same symbol. Then, even if multiple UEs try to access the channel, the collision can be resolved by the LBT mechanism. The same benefit should be available for FBE. For FBE, a potential issue is how to handle the CP extension for CG-PUSCH starting from the beginning of a UE FFP (Fig 5).

Proposal 13: Discuss how to handle the CP extension when configured for CG-PUSCH that starts from the beginning of a UE FFP.

Fig. 5. CP extension for CG-PUSCH in FBE

Harmonizing Rel-16 enhancements for UL configured-grant

Harmonization of CG features

For harmonizing NR-U CG-PUSCH and URLLC CG-PUSCH mechanisms, the following agreements were made in the past meetings.

Agreements:

- At least for FBE, configuration of (cg-RetransmissionTimer) should not be mandated when configured grant Type 1 or Type 2 are configured on unlicensed spectrum.

Agreements:

Down-select one of the following options (target RAN1#104-e):

- Option 1: Both “CG-UCI based procedures” and “CG-DFI based procedures” are enabled or disabled for unlicensed using one RRC parameter i.e. cg-RetransmissionTimer-r16.

- Option 2-a: “CG-UCI based procedures” and “CG-DFI based procedures” are independently enabled or disabled for unlicensed using respective RRC parameter, i.e. new parameter X and cg-RetransmissionTimer-r16, respectively.

- Option 2-b: “CG-UCI based procedures” and “CG-DFI based procedures” are independently enabled or disabled for unlicensed using respective RRC parameter, i.e. new parameter X and new parameter Y, respectively, where X and Y are different from cg-RetransmissionTimer-r16.

- Option 3: CG-UCI based procedures are supported for unlicensed. CG-DFI based procedures are enabled or disabled for unlicensed using one RRC parameter i.e. cg-RetransmissionTimer-r16

- Note: Procedures based on CG-UCI rely on UE including CG-UCI in CG PUSCH at least as in Rel-16 where the values of the respective fields of CG-UCI are decided by UE.

- Note: Procedures based on CG-DFI rely on automatic re-transmission on CG configuration and reception of CG downlink feedback information (DFI) in DCI for re-transmissions.

In our view, it is sufficient to support CG features of Rel-16 URLLC (with slight modification) for unlicensed URLLC operation, and neither CG-UCI based procedures nor CG-DFI based procedures contribute much for unlicensed URLLC. Hence, our preference is to simply adopt Option 1.

Proposal 14: Both “CG-UCI based procedures” and “CG-DFI based procedures” are enabled or disabled for unlicensed using one RRC parameter i.e. cg-RetransmissionTimer-r16.

Enhancement on Rel-16 URLLC CG features

For PUSCH repetition type B, a nominal PUSCH repetition may include symbol(s) overlapping with the idle period, i.e., idle symbol(s), that belongs to an associated FFP. In that case, rather than being dropped, the nominal PUSCH repetition can be segmented into one or more actual PUSCH(s) by the idle symbol(s). That is, the idle symbol(s) can be regarded as invalid symbol for PUSCH repetition type B.

Proposal 15: For FBE, when cg-RetransmissionTimer is not configured, a symbol overlapping with idle period of a FFP associated to PUSCH transmission is regarded as invalid symbol for PUSCH mapping type B.

Conclusion

In this contribution, we provide our view on channel access and CG-PUSCH for Rel-17 FBE for unlicensed URLLC. The following proposals and an observation are drawn:

Proposal 1: FFP offset for UE-initiated COT can be any value within 0 <= offset < UE FFP period with 1-symbol granularity based on the smallest SCS configured in the cell.

Proposal 2: FFP offset for UE-initiated COT is the starting point of first UE FFP relative to the boundary of the even indexed radio frame.

Proposal 3: As part of UE FFP configuration, enable/disable UE COT-initiating functionality for the UL transmissions aligned with a set of UE FFP boundaries.

Proposal 4: If a configured UL transmission is aligned with a UE FFP boundary and ends before the idle period of that UE FFP (and if COT initiation is allowed in that UE FFP (up to Proposal 3)), UE assumes that the configured UL transmission corresponds to UE-initiated COT (Alt-b), with a clarification that “a/the configured UL transmission” is each UL repetition in case of UL repetition transmission.

Proposal 5: UE expects that a configured UL starting from a UE FFP boundary is confined within the UE-initiated COT.

Proposal 6: For a scheduled UL transmission, the COT initiator determination is based on the content in the scheduling DCI. If the content is absent, the determination is based on the rules applied for configured UL transmissions.

Proposal 7: For gNB-to-UE COT sharing in FBE, UE determines whether a detected DL burst was transmitted according to a gNB-initiated COT or a UE-initiated COT based on (implicit or explicit) indication from gNB.

Proposal 8: For FBE, the following rules are supported:

- A UE can initiate a COT within a gNB-initiated COT, and gNB can initiate a COT within a UE-initiated COT.

- As initiating device, the gNB can transmit during any UE FFP idle periods.

- As initiating device, a UE can transmit during other UEs FFP idle periods.

- A responding device can still transmit in the shared COT even if its transmission collides with the idle period of FFP configured for the responding device.

Proposal 9: For FBE, do not support UE-initiated COT for idle/inactive UEs.

Proposal 10: Clarify that for scheduled UL, the FFP/channel where a UL (each UL repetition in case of repetition) is scheduled can be same or different from the FFP/channel where an associated UL grant is transmitted.

Observation 1: The UL reliability performance of unlicensed URLLC can be severely degraded if UE’s processing time for DL detection to share a COT is unknown to gNB.

Proposal 11: For gNB-to-UE COT sharing, define UE’s processing time required for DL burst detection and UL transmission preparation.

Proposal 12: For UE-to-gNB COT sharing, consider defining processing time for gNB’s UL burst detection for UE power saving purpose.

Proposal 13: Discuss how to handle the CP extension when configured for CG-PUSCH that starts from the beginning of a UE FFP.

Proposal 14: Both “CG-UCI based procedures” and “CG-DFI based procedures” are enabled or disabled for unlicensed using one RRC parameter i.e. cg-RetransmissionTimer-r16.

Proposal 15: For FBE, when cg-RetransmissionTimer is not configured, a symbol overlapping with idle period of a FFP associated to PUSCH transmission is regarded as invalid symbol for PUSCH mapping type B.

References

[1] Chairman's Notes RAN1#104-e.

[2] R1-2102175, “Summary#6 - URLLC/IIoT operation on Unlicensed Band,” Moderator (Ericsson), RAN1 #104-e.