3GPP TSG RAN WG1 #104b-e		                         R1- 2103355

e-Meeting, April 12th – April 20th, 2021

Agenda Item:	8.7.1.1

Source: 	LG Electronics

Title: 	Discussion on potential paging enhancements

Document for:	Discussion and decision

- Introduction

According to WID on UE Power Saving Enhancements, followings will be studied and specified in UE Power Saving Enhancements work item [1];

- Specify enhancements for idle/inactive-mode UE power saving, considering system performance aspects [RAN2, RAN1] - Study and specify paging enhancement(s) to reduce unnecessary UE paging receptions, subject to no impact to legacy UEs [RAN2, RAN1] - NOTE: RAN1 to check and update, if needed, evaluation methodology in RAN1 #100 meeting

In this contribution, we discuss and provide our view on potential paging enhancement for idle/inactive mode UE power saving.

- Discussion

  - Information on PEI

UE sub-group indication

To enhance the UE power saving by reducing unnecessary wake up due to the other UE paging, group-WUS was introduced in Rel-16 LTE MTC/NB-IoT. In a similar way, supporting UE sub-group indication, which allow UEs to skip PDSCH reception if there is no paging message intended, would be useful for NR power saving as well. In NR, a number of UE_IDs are assigned at a PO and a detailed indication of the paged UE (i.e. NG-5G-S-TMSI) is conveyed by the PDSCH scheduled by the DCI. Thus the Rel-16 NR UE that obtains PDSCH scheduling information by the paging PDCCH shall try to decode the scheduled PDSCH regardless of whether the UE is actually paged or not.

According to the agreement in the last meeting, UE sub-group indication over PEI and/or paging PDCCH can be considered further. One of the candidates that can convey the UE sub-group information is a DCI for paging at a PO. In a DCI format 1_0 with CRC scrambled by P-RNTI, there are 6 reserved bits and they can be used to indicate UE sub-group indication; whether a UE’s associated UE sub-group is paged or not. Moreover, depending on the bit representation of short message indicator, short message field can be used to convey additional information for further sub-grouping. However it should be noted that UE cannot assume cross slot scheduling for paging with this method; UE should prepare same slot scheduling, which means preparing PDSCH decoding concurrently with PDCCH decoding, for paging reception regardless it is actually paged or not. According to the observation that was agreed in the last RAN1 meeting, up to 1.1% power saving gain can be expected with UE sub-group indication within a PO when group paging rate is 10%.

Another candidate to inform the UE of sub-group indication is to use PEI that was agreed to support from RAN1 perspective in a previous meeting. UE that monitors PEI does not need to prepare PDSCH decoding. Only the UE sub-group that is indicated by the PEI is required to monitor PO for PDCCH and PDSCH decoding, otherwise the UE sub-group can skip monitoring the corresponding PO. Figure 1 shows an example of UE sub-group indication using PEI and paging DCI. In Figure 1 (a), PEI conveys UE sub-group information. It is obvious that UE can turn into the deep sleep mode faster than other methods, so the best power saving gain can be expected among examples. In Figure 1 (b), PEI is used to indicate paging transmission but it does not convey UE sub-group information. In this example, UE should wake up both at PEI monitoring occasion and PO to check UE sub-group information. In Figure 1 (c), paging DCI conveys the UE sub-group information and PEI is not used in this example. Although it can save the power consumption due to the PEI monitoring, it still requires power consumption for PO monitoring as conventional paging UE. According to the observation that was agreed in the last RAN1 meeting, up to 46% power saving gain can be expected with UE sub-group indication carried in PEI when group paging rate is 10%.

Observation 1: The UE sub-group indication using PEI outperforms UE sub-group indication within a PO.

Furthermore, UE sub-group indication using both PEI and paging PDCCH can be considered. For example, carrying part of the UE sub-group information in the PEI and the remaining part in the paging PDCCH. This approach may be useful if PEI is not enough to afford conveying sub-group information. However, since UE sub-grouping using PEI shows better power saving gain than paging PDCCH based solution, using PEI for UE sub-group indication should be discussed first with higher priority.

Figure 1

Short message

In NR, short message field in paging DCI can be used to convey information on direct indication (e.g. ETWS/CMAS and SI update). The paging DCI can be used to convey short message only without scheduling information for paging message. To inform short message to UE that monitors PEI, two candidate methods can be considered. One possible way to provide short message is to indicate UEs wake-up behaviour via PEI so that the UE can monitor the PO. This method can be used without additional specification work if PEI is introduced. However, UE have to monitor both PEI and paging PDCCH to acquire short message, which would degrade UE power consumption efficiency. Moreover, according to the SI change indication procedure described in RRC protocol specification, repetitions of SI change indication may occur within preceding modification period. Therefore, even if there is no paging message intended, UE may wake up several times if SI change indication is repeated within the modification period.

Alternatively, to reduce the unnecessary UE wake up due to the short message acquisition, containing the short message over the PEI can be considered. If so, even a UE that is not indicated to wake up by the PEI can get short message information without PO monitoring. Note that two bits information are required for short message to represent SI change indication and PWS notification separately.

Observation 2: Once the SI change indication is transmitted, repetitions of SI change indication may occur within preceding modification period.

Observation 3: Informing short messages over the PEI avoids unnecessary UE wake ups at PO.

TRS/CSI-RS availability indication

Another candidate that can be considered for PEI contents is an indication of actual transmission of TRS/CSI-RS for idle/inactive mode UEs. If method for indicating actual TRS/CSI-RS transmission is adopted, it would be beneficial to convey such information in PEI. However, whether and how to inform UE of the actual TRS/CSI-RS transmission should be discussed in other sub-agenda item.

Multiple PO

To reduce the network overhead due to the PEI transmission, indicating wake up operation for multiple UE groups using a single PEI can be considered. For example, when sequence-based PEI is considered, assigning different sequence to each group of UEs can be considered to inform UE group information. Alternatively, configuring UE group indication field may be useful to individually control UE group information when DCI based PEI is considered. Although network overhead can be saved with this approach, number of POs associated with a PEI shall be considered carefully as individual control of wake up indication for each UE group will cause additional PEI payload. Moreover, it should be noted that larger gap between PEI and PO may cause lower UE power consumption efficiency, and different PO may have different PEI-to-PO gap when multiple POs are associated with one PEI.

Proposal 1: PEI should at least convey the information on UE sub-group indication and short message

– FFS: TRS/CSI-RS availability indication and/or UE group indication via PEI

– FFS: UE sub-group indication at a PO with same slot scheduling

  - PEI design

In Rel-15 LTE, WUS was introduced to support MTC/NB-IoT UE in idle mode. When WUS is configured by eNB, UE can expect to receive paging at a PO if a WUS is detected within the corresponding WUS duration. If UE does not detect WUS within the WUS duration, it can skip monitoring the associated PO for saving unnecessary power consumption. To reduce the WUS monitoring duration due to the repetition number and reduce the implementation cost, NSSS based sequence design was adopted for MTC/NB-IoT WUS.

To design efficient UE group indication for NR paging, different characteristics from LTE MTC/NB-IoT should be considered carefully. First, repetition is not used for PDCCH transmission and supporting extreme coverage level is not a target in NR power saving. Thus PEI with 1~3 OFDM symbols duration would be enough for NR paging enhancement. Also it should be considered that paging rate of normal NR UE would be much higher than MTC/NB-IoT UE. In MTC/NB-IoT, common WUS can be used to wake up more than one UE groups at the same time. The common WUS would be transmitted more frequently when UEs have higher paging rate, and it causes degradation of the benefit from UE group indication. Moreover, as discussed in another section, it would be worth conveying various types of information for power saving efficiency. In these points of view, sequence based solution for UE group indication is not preferable in NR. Instead, DCI based UE group indication method can be considered for paging enhancement. Note that DCI based power saving scheme that can indicate UE dedicated information was introduced for UEs in connected mode in Rel-16 NR. Similarly, assigning dedicated bits in DCI to UE groups can be considered for idle/inactive mode UE group indication.

Proposal 2: Conveying PEI information using PDCCH is supported

DCI format

The paging information is carried by the paging DCI (i.e. DCI format 1_0 with CRC scrambled by P-RNTI). In terms of UE implementation complexity and specification impact, extension of paging DCI can be considered. For example, some reserved bits in the paging DCI can be assigned to convey PEI information, such as wake up and/or UE sub-group indication. Furthermore, if same DCI format and P-RNTI is used, a PO for other UE group can be used as a PEI occasion. If so, both PEI for one UE group and paging for the other UE group can be informed at once using a single DCI transmission without additional DL resource overhead. On the other hand, it should be noted that DCI size for PEI with this method is same with the paging DCI. So, the number of SSB that is required for the time/frequency tracking and decoding performance for the PEI should be guaranteed to the same level as paging DCI decoding.

Alternatively, introducing new DCI format can be considered. If DCI format 1_0 with CRC scrambled by P-RNTI is used for PEI, maximum size of reserved bits which can be used for PEI is 6 bits only. Thus number of UE sub-groups that can be configured would be constrained by the number of reserved bits. Moreover, as discussed in a previous section, additional information other than UE sub-group indication might be useful for further UE power saving. To achieve better power saving gain with more PEI information, introducing new DCI format would be useful. Note that DCI size for the new format can be discussed further, and compact DCI format which have smaller size than paging DCI can be considered to obtain better power saving gain in terms of time/frequency tracking and decoding performance. From these perspectives, it is preferable to introduce new compact DCI format for PEI.

PEI monitoring occasion

As we discussed in a previous section, using a paging PDCCH for other UE group as a PEI can be considered as a candidate for the PEI design. In this case, existing configuration of PO can be used to determine PEI monitoring occasion for the UE. For example, a PO conveys the PEI information for the nearest subsequent PO of another UE group.

In another case, if compact DCI format is adopted for PEI, introducing new monitoring occasions for PEI can be considered. One of the simplest way to configure PEI monitoring occasion is to configure the occasion with the offset from the associated PO. In LTE, an offset between the PO and the last subframe of the WUS duration can be used to determine the WUS monitoring duration. In NR, the start of WUS monitoring window for connected mode UE can be configured by an offset to the starting of DRX ON. In a similar way, configuring PEI monitoring occasion with the offset to the associated PO can be considered.

Proposal 3: Introduce a new compact DCI format that conveys PEI information with smaller DCI bits than paging DCI.

- PEI monitoring occasion can be configured with the offset to the PO

- Conclusion

In this contribution, we discuss and provide our view on potential paging enhancement for idle/inactive mode UE power saving. Proposals and observations in this contribution are summarized as follows.

Observation 1: The UE sub-group indication using PEI outperforms UE sub-group indication within a PO.

Observation 2: Once the SI change indication is transmitted, repetitions of SI change indication may occur within preceding modification period.

Observation 3: Informing short messages over the PEI avoids unnecessary UE wake ups at PO.

Proposal 1: PEI should at least convey the information on UE sub-group indication and short message

– FFS: TRS/CSI-RS availability indication and/or UE group indication via PEI

– FFS: UE sub-group indication at a PO with same slot scheduling

Proposal 2: Conveying PEI information using PDCCH is supported

Proposal 3: Introduce a new compact DCI format that conveys PEI information with smaller DCI bits than paging DCI.

- PEI monitoring occasion can be configured with the offset to the PO

- Reference

[1]	RP-200938, Revised WID UE Power Saving Enhancements for NR, MediaTek Inc., RAN#88e, June 2020