3GPP TSG RAN WG1 Meeting #104bis-e 	R1-2103392

E-meeting, April 12 - 20, 2021

Agenda Item:	8.11.2

Source:	Huawei, HiSilicon

Title:	Physical layer impacts of sidelink DRX

Document for:	Discussion and Decision

# Introduction

As described in the updated NR sidelink enhancement WID [1], it is clarified that RAN1 should consider the impacts sidelink DRX impact, when introducing power saving resource allocation enhancements:

2. Resource allocation enhancement: - Specify resource allocation to reduce power consumption of the UEs [RAN1, RAN2] - Baseline is to introduce the principle of Rel-14 LTE sidelink random resource selection and partial sensing to Rel-16 NR sidelink resource allocation mode 2. - Note: Taking Rel-14 as the baseline does not preclude introducing a new solution to reduce power consumption for the cases where the baseline cannot work properly. - This work should consider the impact of sidelink DRX, if any. - Study the feasibility and benefit of solution(s) on the enhancement(s) in mode 2 for enhanced reliability and reduced latency in consideration of both PRR and PIR defined in TR37.885 (by RAN#91), and specify the identified solution(s) if deemed feasible and beneficial [RAN1, RAN2] - Inter-UE coordination with the following. - A set of resources is determined at UE-A. This set is sent to UE-B in mode 2, and UE-B takes this into account in the resource selection for its own transmission. - Note: The solution should be able to operate in-coverage, partial coverage, and out-of-coverage and to address consecutive packet loss in all coverage scenarios. - Note: RAN2 work will start after RAN#89. 3. Sidelink DRX for broadcast, groupcast, and unicast [RAN2] - Define on- and off-durations in sidelink and specify the corresponding UE procedure - Specify mechanism aiming to align sidelink DRX wake-up time among the UEs communicating with each other - Specify mechanism aiming to align sidelink DRX wake-up time with Uu DRX wake-up time in an in-coverage UE

The reason why the impact of SL DRX on resource allocation should be taken into account is if the UE behavior of monitoring slots would be limited within SL DRX active time, the sensing results within the DRX inactive time may be not available, resulting in the selected resources being less reliable. The detailed analysis can be found in our companion paper [2]. In this contribution, we further discuss the impact no aspects apart from resource allocation, and discuss why it is clear that there are impacts of SL DRX in the physical layer procedures including SL HARQ operation and SL CSI feedback.

# Impacts on physical layer procedure

## SL HARQ operation

SL HARQ operation is specified in NR-V2X to enhance the reliability and efficiency of sidelink communication. With the introduction of SL DRX, the UE behavior of monitoring various sidelink channels is changed. Based on the RAN2#112-e agreement, UE should monitor PSCCH in SL active time, which means the behavior of the UE relying on PSCCH monitoring would be restricted by SL DRX. For the UE reception behavior besides monitoring PSCCH, whether PSFCH reception should be restricted by SL DRX needs to be considered.

In LTE Uu interface, UL HARQ information is received by UE from eNB on PHICH. The reception is not restricted by the DRX state and the UE must keep receiving the HARQ information even during inactive time, as specified in TS 36.321 [3]:

## 5.7 Discontinuous Reception (DRX) … Regardless of whether the MAC entity is monitoring PDCCH or not, the MAC entity receives and transmits HARQ feedback and transmits type-1-triggered SRS, as specified in TS 36.213 [2], when such is expected. The MAC entity monitors PDCCH addressed to CC-RNTI for a PUSCH trigger B, as specified in TS 36.213 [2], on the corresponding SCell even if the MAC entity is not in Active Time. when such is expected.

It is understood that PDCCH monitoring and blind decoding consumes much power. However, the LTE UL HARQ feedback procedure is synchronous, and when PHICH is used, the UE receives HARQ in a deterministically known resource without blindly decoding PDCCH for the purpose, and the additional power consumption is acceptable for UE.

In NR sidelink, the UE monitors PSCCH during DRX active time. Whether the UE can receive PSFCH during DRX inactive time is worth considering. Similar as LTE UL HARQ feedback, PSFCH receptions happen at deterministically known occasions which do not need PSCCH monitoring, i.e., once PSCCH and associated PSSCH are transmitted, the location of PSFCH can be derived deterministically. From the power consumption perspective, according to the agreements in RAN1#103-e and specified UE power consumption model for FR1 in TR 38.840 [4], the minimum value of power consumption level of PSFCH RX is 50 power units, while the power consumption level of “PSCCH/PSSCH RX” is 300 power units (see agreements below). Thus the power consumption of PSFCH RX is substantially lower than data reception. If the UE does not receive PSFCH which locates in SL DRX inactive time, then the UE needs to have a retransmission, which would cost 300 power units. In conclusion, the power consumption of PSFCH reception seems acceptable if PSFCH reception is allowed in DRX inactive time.

Proposal 1:  PSFCH reception during sidelink DRX inactive time is supported.

Agreements: Confirm the following agreement with red changes: - (Working assumption) Scaling of SL BWP size adaptation in RX perspective - X MHz is by (0.4 +0.6*(X-20)/80), where X is in MHz *100 MHz - (Working assumption) For “PSCCH/PSSCH RX”, - In non-PSFCH-slot (i.e., the number of PSCCH/PSSCH symbols is 13), - the power consumption level is the same as that of “PDCCH+PDSCH” - (Working assumption) For power consumption level of “PSFCH RX”, - the power consumption level is power consumption level of “PDCCH-only” for cross-slot scheduling - Its minimum value is 50 Agreements: Remove the square brackets in the following agreements with red-colored clarification. - Agreements made in RAN1#102-e meeting: - For power consumption level of “1st SCI/2nd SCI RX”, - the power consumption level is [0.7]* power consumption level of “PSCCH/PSSCH RX” - For power consumption level of “PSFCH TX”, - the power consumption level is [0.35]*power consumption level of “UL” for long PUCCH or PUSCH - For power consumption level of “S-SSB RX”, - the power consumption level is [1.5]*power consumption level of “Uu SSB-processing”

## SL CSI feedback

In Rel-16, the sidelink CSI feedback mechanism was specified to support high reliability of unicast transmission via fine-grained link adaptation. A UE receives a CSI trigger from another UE, and transmits a CSI report to it within a latency bound.

Differently from the Uu interface, where CSI reports are received by gNB, the sidelink CSI triggering UE may be in the SL DRX inactive state when the sidelink CSI report is to be receivedhen the adaptationso tentative, given teh . The CSI report reception time depends on the CSI triggering time, CSI latency bound, and the reporting UE’s selection of resources. Being in SL DRX inactive state does not preclude that the UE may still perform a SL transmission, because a SL transmission is not triggered by any reception of a grant on the SL. In mode 1, it may be by DCI for DG, and the UE may not be in Uu DRX inactive state. In mode 2, SL transmission occurs without needing to receive any grant. In such cases of SL transmission during SL DRX inactive time, loss of a CSI report would degrade the performance of link adaptation. The CSI report reception impacts under SL DRX should be further considered by RAN1.

Proposal 2: CSI report reception during sidelink DRX inactive time is supported.

# Conclusions

In this contribution, we show that there will be impacts in the physical layer due to the introduction of sidelink DRX in Rel-17. Based on above discussions, the following proposals are given, to kick-off RAN1 discussions.

Proposal 1:  PSFCH reception during sidelink DRX inactive time is supported.

Proposal 2: CSI report reception during sidelink DRX inactive time is supported.

# References

- RP-202846 “WID revision: NR sidelink enhancement”, LG Electronics, RAN#90e, December 7-11, 2020.

- R1-2102323 “Sidelink resource allocation to reduce power consumption”, Huawei, HiSilicon, RAN1#104bis-e, April 12-20, 2020.

- TS 36.321 “EUTRA; Medium Access Control (MAC) protocol specification”.

- TR 38.840 “NR; Study on User Equipment (UE) power saving in NR”.