3GPP TSG RAN WG1 Meeting #104bis-e	R1-2103399

E-meeting, April 12th – 20th, 2021

Agenda Item:	7.2.11

Source:	Huawei, HiSilicon

Title:	Remaining details of Rel-16 NR UE features

Document for:	Discussion and Decision

Introduction

This contribution contains discussion for some remaining issues for Rel-16 NR UE features.

UE features for Rel-16 WI/TEI

eMIMO

The following issues were raised but no agreement/conclusion was reached in the email discussion on FGs 16-1a-1 and 16-1g in RAN1#104-e meeting:

- Whether PL-RS with more than 2-port is supported by NR specification

- Whether UE is mandated to support 2-port PL-RS and CBD-RS

- If UE supports 2-port PL-RS and CBD-RS, should the counting rule in FG 16-1g be relaxed

In this section, our views and proposals towards above questions are provided.

According the definition in TS 38.213, the calculation of pathloss requires ‘higher-layer filtered RSRP’, and as described in TS 38.215, the calculation of CSI-RSRP is based on 1-port or 2-port CSI-RS. Therefore, the current NR specification support at most 2-port CSI-RS for pathloss estimation purpose.

Observation 1: The current NR specs support at most 2-port CSI-RS for pathloss estimation purpose.

In our understanding, UE is not and should not be mandated to support 2-port PL-RS and CBD-RS since the support of 2-port PL-RS is implicitly restricted/represented by UE capability reporting on the support of using 2-port CSI-RS for L1-RSRP measurement. Similarly, candidate beam detection (CBD) is performed based on L1-RSRP measurement, and the support of 2-port CBD-RS is also represented by UE capability reporting on the support of using 2-port CSI-RS for L1-RSRP measurement.

Observation 2: Support of using 2-port CSI-RS for pathloss estimation and candidate beam detection are optional UE features reported by UE feature on the support of using 2-port CSI-RS for L1-RSRP measurement.

Considering that the computational complexity associated with 2-port CSI-RS is twice that with 1-port CSI-RS for L1-RSRP/L1-SINR measurement, we propose that for FG 16-1g, a 2-port CSI-RS is double counted.

Proposal 1: For FG 16-1g, double-counting is applied if one CSI-RS resource is configured with 2 ports.

- Adopt the TP#1 corresponding to the above.

Conclusion

For Rel-16 NR UE feature remaining issues in eMIMO, the following are observed and proposed:

Observation 1: The current NR specs support at most 2-port CSI-RS for pathloss estimation purpose.

Observation 2: Support of using 2-port CSI-RS for pathloss estimation and candidate beam detection are optional UE features reported by UE feature on the support of using 2-port CSI-RS for L1-RSRP measurement.

Proposal 1: For FG 16-1g, double-counting is applied if one CSI-RS resource is configured with 2 ports.

- Adopt the TP#1 corresponding to the above.

- Appendix

- TP#1: Based on R1-2102006

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 16-1g | Resources for beam management, pathloss measurement, BFD, RLM and new beam identification | - The maximum total number of SSB/CSI-RS/CSI-IM resources configured to measure within a slot across all CCs in one frequency range for any of L1-RSRP measurement, L1-SINR measurement, pathloss measurement, BFD, RLM and new beam identification - The maximum total number of SSB/CSI-RS/CSI-IM resources configured across all CCs in one frequency range for any of L1-RSRP measurement, L1-SINR measurement, pathloss measurement, BFD, RLM and new beam identification | 2-24, 2-31 | Yes | N/A |  | Per UE | No | Yes |  | Component-1: candidate value set is {2, 4, 8, 12, 16, 32, 64, 128} Component-2: candidate value set is {2, 4, 8, 12, 16, 32, 40, 48, 64, 72, 80, 96, 128, 256} Note: For RS configured for new beam identification, they are always counted regardless of beam failure event Note: The “configure to measure” RS (component1) only counts those in active BWP but the configured RS (component2) counts all configured including both active and inactive BWP Note: the reference slot duration is the shortest slot duration defined for the reported FR supported by the UE Note: The “configured to measure” RS is counted within the duration of a reference slot in which the corresponding reference signals are transmitted Note: For component-1, a 2-port CSI-RS resource is counted twice. | Optional with capability signalling |