3GPP TSG RAN WG1 Meeting #104-bis-e	R1-2103418

e-Meeting, April 12 – 20, 2021

Source:	Intel Corporation

Title:	Discussion on frequency range determination for NR extension up to 71 GHz

Agenda item:	8.2.7

Document for:	Discussion/Decision

# Introduction

In this contribution, we discuss aspects related to frequency range determination for extending NR up to 71 GHz. In RAN Plenary #91-e, it was concluded that RAN1, RAN2 and RAN4 are asked to provide its analysis or recommendation to RAN#92E (June) on how to introduce the 52.6-71GHz frequency range.

# Discussion on frequency range from RAN1 perspective

The main discussion is whether 3GPP should introduce a new frequency range to cover frequencies from 52.6 GHz to 71 GHz or whether existing frequency ranges could cover frequencies from 52.6 GHz to 71GHz (possibly with minor modifications). The essence of the discussion is not whether which frequency range should apply to 60GHz band, but specification impact that will stem from one approach versus another approach.

Contribution submitted in last RAN Plenary meeting [1] highlights some aspects that would be affected by frequency range definition for NR operating in 52.6 GHz to 71 GHz. The following table is a more comprehensive list features that leverage FR1/FR2 differentiation in the RAN1 specifications.

Table 1. List of RAN1 specification impact from any changes to frequency range definition or creation of new frequency range

| Specification Number | RAN1 Specification dependent on FR1/FR2 definition |
| --- | --- |
| TS38.201 | None |
| TS38.202 | - DL reception type combination |
| TS38.211 | - Random access configuration - Rx-Tx and Tx-Rx transition period - OffsetToPointA unit |
| TS38.212 | None |
| TS38.213 | - cell search (including SSB pattern) - power control (for dual connectivity) - BWP switching in RA procedure - BWP switching (in relation to bwp-InactivityTimer) - Type0-PDCCH configuration in MIB |
| TS38.214 | - PDSCH and SI-PDSCH reception handling (when overlapped) - TRS configuration - PT-RS configuration - PDSCH and PUSCH processing capability - Resource allocation restrictions for UL (“almost” contiguous allocation) |
| TS38.215 | - Measurement reference point for RSRP/RSRQ/SINR/RSSI/RSTD/Rx-Tx Time Difference/RSARP/SSS Transmit Power |
| TS38.306, TS38.331 (PHY UE capability) | - Several UE capabilities have FR1/FR2 differentiation. - Several UE capabilities were intended for only FR1 or FR2. |

As you can see, other than the impact to UE capability, RAN1 specification impact from any changes to frequency range definition or creation of new frequency range is fairly small. Most issues are very localized and does not need correction that span different specification and/or sections within a specification. The number of issues is very limited. The majority of the problem is focused on UE capabilities as many UE capabilities are defined for specific frequency range and mandatory/optionality of some UE capabilities are dependent on frequency range.

To our understanding the any change to existing definition of frequency range or creation of new frequency range, will require RAN1 and RAN2 to comb over all existing UE capabilities and update the specification accordingly. This work needs to be done regardless of which approach is taken for frequency range definition for 60 GHz band.

All other issues that are not UE capability related are limited in scope and number of issues is quite small for RAN1. Therefore, we believe the amount of work that is needed for RAN1 is similar regardless of how frequency range definition is handled for 60GHz band.

Observation 1:

- Overall RAN1 specification impact from changes or creation of frequency range is minimal.

- Work needed in RAN1 to update specification is about the same regardless of approach taken to handle frequency range definition.

Base on this, we propose that RAN1 concludes specification impact for RAN1 is the same regardless of frequency range determination for frequencies between 52.6 GHz and 71 GHz. Furthermore, recommend that RAN4 determine the frequency range for frequencies between 52.6 GHz and 71GHz.

Proposal 1:

- RAN1 concludes specification impact for RAN1 is limited and the amount of effort needed to update the specification is the same regardless of how frequency range is determined for frequencies between 52.6 GHz and 71 GHz.

- RAN1 recommends that RAN4 determine the frequency range for frequencies between 52.6 GHz and 71 GHz.

# Conclusions

In this contribution, we discussed issues related to frequency range determination from RAN1 perspective. The following is a summary of the observation and proposals:

Observation 1:

- Overall RAN1 specification impact from changes or creation of frequency range is minimal.

- Work needed in RAN1 to update specification is about the same regardless of approach taken to handle frequency range definition.

Proposal 1:

- RAN1 concludes specification impact for RAN1 is limited and the amount of effort needed to update the specification is the same regardless of how frequency range is determined for frequencies between 52.6 GHz and 71 GHz.

- RAN1 recommends that RAN4 determine the frequency range for frequencies between 52.6 GHz and 71 GHz.

# References

- RP-210659, “Updates of Rel-17 WI on extending current NR operation to 71GHz,” Huawei, HiSilicon