3GPP TSG RAN WG1 #104bis-e	R1-2103470

e-Meeting, April 12th – 20th, 2021

Source:	Sharp

Title:	Enhancements on Multi-TRP for PUSCH

Agenda Item:	8.1.2.1

Document for:	Discussion and Decision

# Background

At RAN#86, a new work item on “Further enhancements on MIMO for NR” was agreed [1] and the revised WID was agreed at RAN#89-e [2]. In the WID, the following objectives were agreed for multi-TRP enhancement.

- Enhancement on the support for multi-TRP deployment, targeting both FR1 and FR2: - Identify and specify features to improve reliability and robustness for channels other than PDSCH (that is, PDCCH, PUSCH, and PUCCH) using multi-TRP and/or multi-panel, with Rel.16 reliability features as the baseline - Identify and specify QCL/TCI-related enhancements to enable inter-cell multi-TRP operations, assuming multi-DCI based multi-PDSCH reception - Evaluate and, if needed, specify beam-management-related enhancements for simultaneous multi-TRP transmission with multi-panel reception - Enhancement to support HST-SFN deployment scenario: - Identify and specify solution(s) on QCL assumption for DMRS, e.g. multiple QCL assumptions for the same DMRS port(s), targeting DL-only transmission - Evaluate and, if the benefit over Rel.16 HST enhancement baseline is demonstrated, specify QCL/QCL-like relation (including applicable type(s) and the associated requirement) between DL and UL signal by reusing the unified TCI framework

In this contribution, we share our views on PUSCH enhancement for MTRP operation.

# Discussions

In the last meeting, the following agreements were made.

Agreement For single DCI based M-TRP PUSCH Type B repetition schemes, - For maxRank = 2, the number of bits for the indication of PTRS-DMRS association is the same as Rel-15/16, MSB and LSB separately indicating the association between PTRS port and DMRS port for two TRPs. - FFS: the indication of PTRS-DMRS association for maxRank > 2. Agreement For s-DCI based multi-TRP PUSCH repetition Type A and B, if the DCI schedules A-CSI, support multiplexing A-CSI on the first PUSCH repetition corresponding to the first beam and the X-th PUSCH repetition corresponding to the second beam. - For PUSCH repetition Type A, X=1 (the first PUSCH repetition corresponding to the second beam) - For PUSCH repetition Type B, the first actual PUSCH repetition corresponding to the first beam and the X-th actual repetition corresponding to the second beam are considered, - The UE does not expect the first actual repetition corresponding to the first beam and the X-th actual repetition corresponding to the second beam to have a single symbol duration (similar restriction as in Rel-16 NR for the single TRP case). - The first actual repetition corresponding to the first beam and the X-th actual repetition corresponding to the second beam are expected to have the same number of symbols - FFS: X = 1 or X = the first actual repetition corresponding to the second beam that contains the same number of symbols as the first actual repetition with the first beam - FFS: Any further restrictions/enhancements needed on supporting A-CSI multiplexing on PUSCH repetitions - FFS: whether to support multiplexing SP-CSI/P-CSI on PUSCH repetitions towards multiple TRPs. Agreement Further study following aspects related to beam mapping and default behaviors for multi-TRP PUCCH/PUSCH schemes, - Whether enhancements needed on beam mapping in case of PUCCH/PUSCH dropping due to invalid UL symbols - Whether frequency hopping is performed among the repetitions with the same beam - Whether defining default beam for PUSCH is needed when PUSCH scheduled by DCI format 0_0 when two spatial relation info’s are configured for a PUCCH resource Agreement For single DCI based M-TRP PUSCH repetition schemes, in codebook based PUSCH, - Two TPMI fields are indicated in DCI formats 0_1/0_2. - The first TPMI field uses the Rel-15/16 TPMI field design (which includes TPMI index and the number of layers) of DCI format 0_1/0_2. The second TPMI field only containsindicates the second TPMI index. The same number of layers are applied as indicated in the first TPMI field. - FFS: Details of second TPMI field interpretation including changes expected in Tables 7.3.1.1.2-2/2A/2B/3/3A/4/4A/5/5A in 38.212 - FFS: Interpreting TPMI fields when multi-TRP and single-TRP PUSCH repetition is applied. - FFS: whether to support of PUSCH repetitions transmitting towards two TRPs sharing the same TPMI indicated by a TPMI field. - FFS: The size of the second TPMI field can be equal to or smaller than the size of the first TPMI field Agreement For single DCI based M-TRP PUSCH repetition schemes, in non-codebook based PUSCH, - Support two SRI field(s) corresponding to two SRS resource sets are included in DCI formats 0_1/0_2. - Each SRI field indicating SRI per TRP, where the first SRI field based on Rel-15/16 framework, - Support the same number of layers applied over repetitions - FFS: details of second SRI field including the specification change for Table 7.3.1.1.2-28/29/30/31 in 38.212. - Support dynamic switching between multi-TRP and single-TRP operation - FFS: whether/how to use SRI field(s) and additional details of SRI field(s) interpretations - FFS: Minimizing the DCI overhead for PUSCH repetition Type A as a result of number of layers being limited to 1 when more than one repetition is scheduled. - FFS: Support dynamic switching the order of two TRPs - Companies are encouraged to provide total payload size of the two SRI fields and scheduling restriction, if any

Regarding two TPMI field in codebook-based transmission, the remaining issue is how to specify the second TPMI field. Our view is that the simple way is the second TPMI field is added in the DCI based on the number of candidate TPMIs according to the rank of the first TPMI field, the number of antenna ports, and codebookSubset.

Regarding two SRI field in non-codebook-based transmission, because the rank is the same of the first SRI field, the second SRI field is added based on the candidate combinations of selected SRS resources. For example, in Table 7.3.1.1.2-29 in TS38.212, the following interpretation of SRI field is defined.

Table 7.3.1.1.2-29: SRI indication for non-codebook based PUSCH transmission,

| Bit field mapped to index | SRI(s), | Bit field mapped to index | SRI(s), | Bit field mapped to index | SRI(s), |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 1 | 1 | 1 | 1 | 1 |
| 2 | 0,1 | 2 | 2 | 2 | 2 |
| 3 | reserved | 3 | 0,1 | 3 | 3 |
|  |  | 4 | 0,2 | 4 | 0,1 |
|  |  | 5 | 1,2 | 5 | 0,2 |
|  |  | 6-7 | reserved | 6 | 0,3 |
|  |  |  |  | 7 | 1,2 |
|  |  |  |  | 8 | 1,3 |
|  |  |  |  | 9 | 2,3 |
|  |  |  |  | 10-15 | reserved |

For example, the number of SRS field is 4 and the maximum layers is 2, 4 combinations of SRS resource indices can be indicated as rank 1 transmission and 6 combinations of SRS resource indices can be indicated. Therefore, in this case, the second SRI field can be specified as:

- 2 bits (maximum rank = 2, rank 1, the number of SRS resources = 4)

- 3 bits (maximum rank = 2, rank 2, the number of SRS resources = 4)

The same principle can be applied to other combinations of the maximum rank, the rank, and the number of SRS resources

Therefore, we propose

Proposal:

- In codebook-based transmission,

  - The second TPMI field is added to the DCI and the number of bits is determined by on the rank of the first TPMI field, the number of antenna ports, and codebookSubset

- In non codebook based transmission,

  - The second SRI field is added to the DCI and the number of bits is determined by the maximum rank, the rank of the first SRI field, and the number of SRS resources

# Conclusion

In this contribution, we have the following proposal:

Proposal:

- In codebook-based transmission,

  - The second TPMI field is added to the DCI and the number of bits is determined by on the rank of the first TPMI field, the number of antenna ports, and codebookSubset

- In non codebook based transmission,

  - The second SRI field is added to the DCI and the number of bits is determined by the maximum rank, the rank of the first SRI field, and the number of SRS resources

# References

- RP-193133, “New WID: Further enhancements on MIMO for NR”, RAN#86, December 2019

- RP-202024, “Revised WID: Further enhancements on MIMO for NR”, RAN#89-e, September 2020.