3GPP TSG RAN WG1#104-bis-e	R1-2103473

e-Meeting, April 12th – April 20th, 2021

Source:	Sharp

Title:	UE feedback enhancements for HARQ-ACK

Agenda Item:	8.3.1.1

Document for:	Discussion and Decision

# Introduction

In this contribution, we provide discussions on UE feedback enhancements for HARQ-ACK with respect to the following agreed objective in [1]:

1. Study, identify and specify if needed, required Physical Layer feedback enhancements for meeting URLLC requirements covering  UE feedback enhancements for HARQ-ACK [RAN1]

Additionally, following agreements on HARQ-ACK enhancements were achieved in RAN1 104e meeting [2]:

Agreements: - Support deferring SPS HARQ-ACK dropped due to TDD specific collisions until a next available PUCCH in Rel-17 based on semi-static configuration of slot format - FFS: Details (including possible conditions for such a deferring, whether or not to consider semi-statically configured flexible symbols for PUCCH availability, etc.) - Aim for minimal standardization efforts and UE complexity in implementation Agreements: Further down-select between the following two options for SPS HARQ-ACK deferral: - Option 1: Joint RRC configuration of the SPS HARQ-ACK deferral per PUCCH cell group - Note: any SPS HARQ-ACK within a PUCCH cell group in principle is subject to deferral - Option 2: The SPS HARQ-ACK deferral is configured per SPS configuration - Note: part of sps-config, only HARQ-ACK of SPS PDSCH configurations configured for deferral is in principle subject to deferral Agreements: Support sub-slot based PUCCH repetition for HARQ-ACK based on the Rel-16 PUCCH procedure for slot-based PUCCH applied to sub-slot based PUCCH - Note: the intention is to take the Rel-16 slot-based PUCCH by replacing with “sub-slot” appropriately, without further optimization unless necessary - FFS whether or not there is any restriction for the applicability of sub-slot based PUCCH repetition for HARQ-ACK - Dynamic repetition indication is supported also for sub-slot based PUCCH in Rel-17 - FFS: if the method to be specified in Cov. Enh WI for slot-based PUCCH repetition can be directly applied to sub-slot PUCCH or if changes are needed Agreements: Support PUCCH repetition for PUCCH formats 0 and 2 at least for sub-slot based PUCCH repetition. - FFS: Support for slot-based PUCCH repetition Agreement: Rel-16 UCI multiplexing / PUCCH overriding rules are reused for deferred SPS HARQ-ACK in the target slot, if applicable.

# Discussions on SPS HARQ-ACK dropping for TDD

To avoid SPS PDSCH HARQ-ACK dropping for TDD, deferring HARQ-ACK until the first available valid PUCCH resource has been agreed in RAN1 104e meeting. One of the remaining questions is how to configure SPS HARQ-ACK deferral. Comparing to joint RRC configuration per PUCCH cell group, it is more flexible to configure SPS HARQ-ACK deferral per SPS configuration, since each SPS configuration may be used for different services with different requirements and parameters, e.g., periodicity. Thus, we have the following proposal.

Proposal 1:

- The deferring of SPS HARQ-ACK dropped due to TDD specific collisions is configured by RRC per SPS configuration.

# Discussions on SPS HARQ skipping and payload size reduction

In current NR, UE should report HARQ-ACK feedback for each configured SPS PDSCH occasion, even though there is no actual SPS PDSCH transmission. However, due to the sporadic nature of URLLC, it could be a waste of feedback resource, especially when the SPS is configured with shorter periodicities to meet the low latency requirement. Thus, HARQ-ACK transmission should be skipped if SPS PDSCH is not transmitted by the gNB. Comparing to ‘NACK skipping’, dynamic indication of skipped SPS PDSCH occasions will introduce more signalling overhead and reliability issues. Thus, ‘NACK skipping’ is preferred.

Proposal 2:

- ‘NACK skipping’ for (skipped) SPS PDSCH is supported.

In addition, for ‘non-skipped’ SPS PDSCH, HARQ bundling can further reduce the payload size. For example, HARQ-ACK bundling of N bits by using logical ‘AND’ or ‘OR’ can result in M bits (M < N, e.g., M = 1), so that the payload size can be reduced. Hence, we have the following proposal.

Proposal 3:

- ‘HARQ bundling’ for (non-skipped) SPS PDSCH is supported.

# Discussions on PUCCH repetition enhancements

In the last meeting, sub-slot based PUCCH repetition for HARQ-ACK was agreed to be supported. Specifically, we support PUCCH repetition for PUCCH formats 0 and 2 at least for sub-slot based PUCCH repetition. One of the remaining issues is whether to support PUCCH repetition for PUCCH formats 0 and 2 also for slot based PUCCH repetition. Since it has been agreed to support slot based repetition with short PUCCH for MTRP in feMIMO, the following proposal for alignment should be straightforward.

Proposal 4:

- PUCCH repetition for PUCCH formats 0 and 2 is also supported for slot based PUCCH repetition in Rel-17.

Another remaining question is whether to support sub-slot based PUCCH repetition for other UCI types, such as SR and/or CSI. We think sub-slot based PUCCH repetition should not be limited to HARQ-ACK only, since it can also provide flexibility and reliability for other UCI types. On the other hand, slot based PUCCH repetition is configured per PUCCH format. Since sub-slot based PUCCH repetition is agreed to reuse the slot-based PUCCH repetition operation principle, the following proposal should be also applicable.

Proposal 5:

- Sub-slot based PUCCH repetition for other UCI types (than HARQ-ACK) is also supported.

# Discussions on retransmission of cancelled HARQ

It has been agreed to further study retransmission of cancelled HARQ, where we consider Type-3 HARQ-ACK codebook as a potential solution. Although Type-3 HARQ-ACK codebook was originally introduced as a new type of codebook during Rel-16 specification for NR-U, it could be used for general purposes and we observed that exploiting Type-3 HARQ-ACK codebook is beneficial to URLLC operation, especially for HARQ retransmission and/or SPS HARQ-ACK dropping, due to the following advantages:

- Can be dynamically triggered by DCI as required

- One-shot feedback for all configured HARQ processes

- Reusing specification on Type-3 HARQ-ACK codebook in Rel-16 NR-U might cause less specification impact/effort

To adopt Type-3 HARQ-ACK codebook in URLLC operation, one remaining issue is when Type-3 HARQ-ACK codebook meets prioritization of HARQ-ACK codebook. More specifically, the problem is that it is not specified how to handle Type-3 HARQ-ACK codebook with mixed priorities. To solve this problem, there might be several alternatives:

- Alt. 1: Associate Type-3 HARQ-ACK codebook with one priority index

  - 1a: The priority index is predetermined, e.g., priority index 1

  - 1b: The priority index is RRC configured

  - 1c: The priority index is indicated by the Priority field in the triggering DCI format

- Alt. 2: Generate two Type-3 HARQ-ACK codebooks, each corresponding to a priority index

Among the alternatives, we are preferable to Alt. 1c because 1) the flexibility by dynamically indication, 2) integration of Type-3 codebook triggering and prioritization within one DCI format, 3) relatively small specification impact, e.g., no introduction of RRC parameter or modification of pseudo-code. Due to the specification progress, we suggest to firstly achieve consensus on adopting Type-3 HARQ-ACK codebook in URLLC operation.

Proposal 6:

- As a potential solution for retransmission of cancelled HARQ, support Type-3 HARQ-ACK codebook corresponding to mixed priorities.

# Conclusion

In this contribution, we provide the following proposals:

Proposal 1:

- The deferring of SPS HARQ-ACK dropped due to TDD specific collisions is configured by RRC per SPS configuration.

Proposal 2:

- ‘NACK skipping’ for (skipped) SPS PDSCH is supported.

Proposal 3:

- ‘HARQ bundling’ for (non-skipped) SPS PDSCH is supported.

Proposal 4:

- PUCCH repetition for PUCCH formats 0 and 2 is also supported for slot based PUCCH repetition in Rel-17.

Proposal 5:

- Sub-slot based PUCCH repetition for other UCI types (than HARQ-ACK) is also supported.

Proposal 6:

- As a potential solution for retransmission of cancelled HARQ and/or SPS HARQ-ACK dropping, support Type-3 HARQ-ACK codebook corresponding to mixed priorities.

# References

  - 3GPP WID, RP-201310

  - Chairman’s notes of 3GPP TSG RAN WG1 #104-e