3GPP TSG RAN WG1 #104bis-e			R1-2103579

e-Meeting, April 12th – April 20th, 2021

Source:	NTT DOCOMO, INC.

Title:	Discussion on UL time synchronization enhancements for NTN

Agenda Item:	8.4.2

Document for: 	Discussion and Decision

- Introduction

At the RAN1#102-e meeting, the following was agreed [1].

Agreement: In case of GNSS-assisted TA acquisition in RRC idle/inactive mode, the UE calculates its TA based on the following potential contributions: - The User specific TA which is estimated by the UE: - Option 1: The User specific TA is estimated by the UE based on its GNSS acquired position together with the serving satellite ephemeris indicated by the network: - FFS: Details on serving satellite ephemeris indication - Option 2: The User specific TA is estimated by the UE based on the GNSS acquired reference time at UE together with reference time as indicated by the network - The Common TA if indicated by the network: - FFS: The need and details of Common TA indication - FFS: The TA margin, if needed and indicated by the network (in order to account for the TA estimation uncertainty)

Also, at the RAN1#103-e meeting, the following was agreed [2].

Agreement: - In NTN, the network may broadcast - A common timing offset value - FFS details of the common timing offset - FFS: A common timing drift rate - Before Msg1/MsgA transmission, the NR NTN UE in idle/inactive mode calculates its TA as follows: Where: is derived from the User specific TA self-estimation is derived at least from the common timing offset value if broadcasted by the network. The granularity of and whether is indicated as a Timing Advance or as a Timing Offset value [unit] are FFS. Upon resolving the FFS, one of the X in the equation will be removed. - depends on band and LTE/NR coexistence and is specified in TS 38.213 section 4.2. - is specified in TS 38.211 section 4.1. - Note: UE will not assume that the RTT between UE and gNB is equal to the calculated TA for Msg1/Msg A.

At the RAN1#104-e meeting, the following was agreed [3].

Agreement: - RAN1 to support satellite ephemeris broadcast based at least on one of the following format options: - Option 1: Ephemeris format based on satellite position and velocity state vectors - FFS: Details on state vectors formats - FFS: Details on time reference provisioning/format - Option 2: Ephemeris format based on orbital elements - FFS: Details on orbital elements formats - FFS: Details on time reference provisioning/format - FFS: Whether down-selection is needed or both options are supported

In this contribution, we will firstly discuss on the remaining issues on common timing drift rate and TA margin for TA pre-compensation in NR NTN UE as highlighted above, and then discuss on the satellite ephemeris format for NTN.

- Discussion on TA calculation

At the RAN1#103e meeting, TA calculation method was agreed, where UE-specific TA estimated by the NTN UE and a common TA broadcasted by the network are included. During the discussion at RAN1#104e meeting, whether and how to indicate common timing drift rate and TA margin were discussed but without achieving consensus. In this section, we will provide our views on these issues.

  - Discussion on the Common Timing Drift Rate

Satellite/HAPS drift will cause timing drift as shown in Figure 1. For example, if we denote T0 (T0’) and T1 (T1’) as the round trip delay (RTD) from the reference point (RP) to the satellite/HAPS and from the satellite/HAPS to the UE at time t1 (t2), respectively, the common timing drift rate and UE-specific timing drift rate can be calculated as (T0’-T0)/(t2-t1) and (T1’-T1)/(t2-t1), respectively.

Observation 1: Satellite/HAPS drift will cause TA variation for both feeder link and service link. In addition to the common timing drift rate, user-specific timing drift rate also exists.

To improve the TA accuracy, both common and UE-specific timing drift rate should be considered in the TA calculation. UE-specific timing drift rate can be estimated and pre-compensated at the UE side, whereas common timing drift rate should be handled or broadcasted by the network as discussed in the previous meeting.

If RP is located at the gNB, which is a good option to align UL/DL at the gNB, UE can collect multiple common TAs for the feeder link at different times and obtain the gateway’s approximate location before accessing the NTN system [4]. With the estimated gateway location, common timing drift rate for the feeder link can be used for NTN UE to estimate the drift rate of the Satellite/HAPS, which can be further used for the calculation of UE-specific timing drift rate.

In other words, in some cases, e.g., when RP is located at the gNB and UE has the capability on UE-specific timing drift rate calculations, common timing drift rate may be helpful for UE-specific timing drift rate calculation and thus can be indicated to users.

Proposal 1: Common timing drift rate can be indicated to users to cover all the foreseen implementations and scenarios.

Figure 1 Illustration of timing drift rate caused by Satellite/HAPS drift

  - Discussion on the TA margin

As discussed in previous meetings, TA margin denotes the maximum TA estimation error caused by inaccurate location/ephemeris, etc. Based on the agreements in RAN1#102e meeting, the TA margin is still FFS, if needed and indicated by the network (in order to account for the TA estimation uncertainty). In this subsection, we will discuss on this issue.

Common TA (from the RP to the Satellite/HAPS) is estimated by the gNB/Gateway based on the location of Satellite/HAPS. Satellite/HAPS may have different on-board GPS capabilities, which will cause different location accuracy and hence different TA estimation error for the common TA. The maximum TA estimation error for the common TA can be defined as common TA margin. This common TA margin should be known by gNB/Gateway, which is related to the Satellite/HAPS GPS capabilities.

In addition to the inaccurate satellite/HAPS locations, UEs may also have different GNSS capabilities and hence different location accuracies. The inaccurate satellite/HAPS locations and UE locations will cause TA estimation error for the UE-specific TA (from the satellite/HAPS to the UE). The maximum TA estimation error for the UE-specific TA can be defined as UE-specific TA margin. This UE-specific TA margin should be known by the UE, which is related to the Satellite/HAPS GPS-capability and UE GNSS-capability. For each implementation and capability, TA margin (i.e., the maximum estimation error) should be a constant.

Observation 2: For given requirements and capability on location accuracy for both Satellite/HAPS and UE, the maximum TA estimation error would be fixed.

Proposal 2: Common TA margin and user-specific TA margin should be considered separately.

The estimation error of common TA (i.e., common TA margin) can be pre-compensated by the network or user as follows:

- Option 1: Common TA margin is pre-compensated by the network. Due to different satellite/HAPS capabilities, different common TA margins may complicate the network.

- Option 2: Common TA margin is pre-compensated by the user. In this case, the common TA margin can be indicated to the UEs. In addition, the common TA margin can be included in the common TA, i.e., indication of common TA is enough and separate indication of common TA margin is not necessary.

To reduce the network complexity, option 2 is preferred, i.e., common TA margin is pre-compensated by the user.

Proposal 3: Common TA margin can be included in the common TA. The indication of common TA margin is not necessary.

As discussed in previous subsection, UEs may have different capabilities on calculating UE-specific timing drift rate. Besides, UEs may also have different capabilities on obtaining the UE-specific TA margin. In addition to these different capabilities, UEs may also have different GNSS capability on location accuracy, etc. These aspects all result in different capabilities on TA self-estimation accuracy at the UE side. Therefore, we propose to define the UE capability on different TA self-estimation/pre-compensation accuracy. And multiple common TAs can be broadcasted for UEs with different UE capabilities on TA self-estimation/pre-compensation accuracy. Whether to consider overall TA self-estimation accuracy or individual capability on calculating the component for TA can be further discussed.

Proposal 4: UE capability on user-specific TA self-estimation/pre-compensation can be defined to indicate the self-estimation accuracy.

Proposal 5: One or multiple common TAs can be broadcasted for UEs with different UE capabilities on TA self-estimation/pre-compensation accuracy.

Figure 2 Illustration of timing uncertainty caused by the location inaccuracy

- Discussion on Ephemeris Format

In RAN1#104e meeting, two options of ephemeris formats were agreed. But it is still FFS whether down-selection is needed or both options are supported. Based on the analysis and comparisons in the feature lead summary in RAN1#104e meeting [5], the two options both have their advantages and disadvantages.

For example, option 1, i.e., ephemeris format based on satellite position and velocity state vectors, has the advantage of low computation complexity and high compatibility of all implementation systems (e.g., Satellite/HAPS/ATG). However, satellite position and velocity state vectors change fast and should be broadcasted with a short periodicity. Therefore, ephemeris format in option 1 can be used for the scenario requiring accurate estimation.

Option 2, i.e., ephemeris format based on orbital elements, changes slowly and can be broadcasted with a long periodicity. However, the ephemeris format in option 2 should be firstly transformed into the ephemeris format in option 1 before further calculation, which will consume high computation complexity. Therefore, option 2 can be used for some scenarios and objectives, e.g., for initial access or some cell selection functions.

Observation 3: Two options of Satellite ephemeris format have their advantages and disadvantages.

Proposal 6: Two options can be both supported with different indication accuracy and periodicity.

- Option 1: Ephemeris format based on satellite position and velocity state vectors

  - FFS: Details on state vectors formats

  - FFS: Details on time reference provisioning/format

- Option 2: Ephemeris format based on orbital elements

  - FFS: Details on orbital elements formats

  - FFS: Details on time reference provisioning/format

- Conclusion

In this contribution, we firstly discussed the remaining issues on common timing drift rate and TA margin for TA pre-compensation in NR NTN UE, and then discussed the satellite ephemeris format for NTN. Based on the discussion we made following observations.

Observation 1: Satellite/HAPS drift will cause TA variation for both feeder link and service link. In addition to the common timing drift rate, user-specific timing drift rate also exists.

Observation 2: For given requirements and capability on location accuracy for both Satellite/HAPS and UE, the maximum TA estimation error would be fixed.

Observation 3: Two options of Satellite ephemeris format have their advantages and disadvantages.

Based on the discussion we made following proposals.

Proposal 1: Common timing drift rate can be indicated to users to cover all the foreseen implementations and scenarios.

Proposal 2: Common TA margin and user-specific TA margin should be considered separately.

Proposal 3: Common TA margin can be included in the common TA. The indication of common TA margin is not necessary.

Proposal 4: UE capability on user-specific TA self-estimation/pre-compensation can be defined to indicate the self-estimation accuracy.

Proposal 5: One or multiple common TAs can be broadcasted for UEs with different UE capabilities on TA self-estimation/pre-compensation accuracy.

Proposal 6: Two options can be both supported with different indication accuracy and periodicity.

- Option 1: Ephemeris format based on satellite position and velocity state vectors

  - FFS: Details on state vectors formats

  - FFS: Details on time reference provisioning/format

- Option 2: Ephemeris format based on orbital elements

  - FFS: Details on orbital elements formats

  - FFS: Details on time reference provisioning/format

References

- 3GPP, RAN1#102e, RAN1 Chairman’s Notes, Sep. 2020.

- 3GPP, RAN1#103e, RAN1 Chairman’s Notes, Nov. 2020.

- 3GPP, RAN1#104e, RAN1 Chairman’s Notes, Feb. 2021.

- R1-2100223, “Discussion on UL time and frequency synchronization enhancement for NTN”, Huawei, HiSilicon, Feb. 2021.

- R1-2102215, “FL Summary on enhancements on UL time and frequency synchronization for NR NTN”, Thales, Feb. 2021.