3GPP TSG RAN WG1 #104b-e	  R1-2103602

e-Meeting, April 12th – April 20th, 2021

Source: 	ZTE, Sanechips

Title:	Remaining issues of Rel-16 power saving

Agenda Item:	7.2.7

Document for:	Discussion/Decision

- Introduction

In the last meeting, some issues of Rel-16 power saving were solved [1]. However, there is still an ambiguity issue of Rel-16 power saving in TS 38.214[2]. In this contribution, the corresponding text is proposed.

- Applicable PDSCH time domain resource allocation

In RAN1#96bis, the cases that minimum applicable value of K0 is not applicable were agreed as below [3].

Agreements in RAN1#96bis: - The adaptation on the minimum applicable value of K0 does not apply to at least the following cases: RNTI; PDCCH search space. SI-RNTI; Type0 common. SI-RNTI; Type0A common. RA-RNTI, TC-RNTI; Type1 common. P-RNTI; Type2 common.

According to the following yellow part in TS 38.214 [2], the minimum scheduling offset restriction is not applied when PDSCH transmission is scheduled with SI-RNTI, MSGB-RNTI or RA-RNTI. However, the case that PDSCH transmission is scheduled with TC-RNTI is not captured. It is necessary to update the TS38.214 to reflect the agreements.

TS 38.214 V16.5.0 5.1.2.1 Resource allocation in time domain ======skipped text====== When the UE is configured with minimumSchedulingOffsetK0 in an active DL BWP it applies a minimum scheduling offset restriction indicated by the 'Minimum applicable scheduling offset indicator' field in DCI format 1_1 or DCI format 0_1 if the same field is available. When the UE is configured with minimumSchedulingOffsetK0 in an active DL BWP and it has not received 'Minimum applicable scheduling offset indicator' field in DCI format 0_1 or 1_1, the UE shall apply a minimum scheduling offset restriction indicated based on 'Minimum applicable scheduling offset indicator' value '0'. When the minimum scheduling offset restriction is applied the UE is not expected to be scheduled with a DCI in slot n to receive a PDSCH scheduled with C-RNTI, CS-RNTI or MCS-C-RNTI with K0 smaller than K0min⋅2μ'2μ, where K0min and μ are the applied minimum scheduling offset restriction and the numerology of the active DL BWP of the scheduled cell when receiving the DCI in slot n, respectively, and μ' is the numerology of the new active DL BWP in case of active DL BWP change in the scheduled cell and is equal to μ, otherwise. The minimum scheduling offset restriction is not applied when PDSCH transmission is scheduled with C-RNTI, CS-RNTI or MCS-C-RNTI in common search space associated with CORESET0 and default PDSCH time domain resource allocation is used, in the search space set provided by recoverySearchSpaceId when monitoring PDCCH as described in [6, TS 38.213] or when PDSCH transmission is scheduled with SI-RNTI, MSGB-RNTI or RA-RNTI. The application delay of the change of the minimum scheduling offset restriction is determined in Clause 5.3.1.

According to the above analysis, the following TP on TS 38.214 should be adopted.

Proposal 1: Adopt the following TP on TS 38.214.

----------------------------------------------- Start of TP of TS 38.214 -------------------------------------------------------- 5.1.2.1 Resource allocation in time domain *** Unchanged text is omitted *** When the UE is configured with minimumSchedulingOffsetK0 in an active DL BWP it applies a minimum scheduling offset restriction indicated by the 'Minimum applicable scheduling offset indicator' field in DCI format 1_1 or DCI format 0_1 if the same field is available. When the UE is configured with minimumSchedulingOffsetK0 in an active DL BWP and it has not received 'Minimum applicable scheduling offset indicator' field in DCI format 0_1 or 1_1, the UE shall apply a minimum scheduling offset restriction indicated based on 'Minimum applicable scheduling offset indicator' value '0'. When the minimum scheduling offset restriction is applied the UE is not expected to be scheduled with a DCI in slot n to receive a PDSCH scheduled with C-RNTI, CS-RNTI or MCS-C-RNTI with K0 smaller than K0min⋅2μ'2μ, where K0min and μ are the applied minimum scheduling offset restriction and the numerology of the active DL BWP of the scheduled cell when receiving the DCI in slot n, respectively, and μ' is the numerology of the new active DL BWP in case of active DL BWP change in the scheduled cell and is equal to μ, otherwise. The minimum scheduling offset restriction is not applied when PDSCH transmission is scheduled with C-RNTI, CS-RNTI or MCS-C-RNTI in common search space associated with CORESET0 and default PDSCH time domain resource allocation is used, in the search space set provided by recoverySearchSpaceId when monitoring PDCCH as described in [6, TS 38.213] or when PDSCH transmission is scheduled with SI-RNTI, MSGB-RNTI, RA-RNTI or TC-RNTI. The application delay of the change of the minimum scheduling offset restriction is determined in Clause 5.3.1. *** Unchanged text is omitted *** ----------------------------------------------- End of TP of TS 38.214 --------------------------------------------------------

- Conclusion

In this contribution, the text proposal for the remaining issue on minimum applicable scheduling offset indication is as follow:

Proposal 1: Adopt the following TP on TS 38.214.

----------------------------------------------- Start of TP of TS 38.214 -------------------------------------------------------- 5.1.2.1 Resource allocation in time domain *** Unchanged text is omitted *** When the UE is configured with minimumSchedulingOffsetK0 in an active DL BWP it applies a minimum scheduling offset restriction indicated by the 'Minimum applicable scheduling offset indicator' field in DCI format 1_1 or DCI format 0_1 if the same field is available. When the UE is configured with minimumSchedulingOffsetK0 in an active DL BWP and it has not received 'Minimum applicable scheduling offset indicator' field in DCI format 0_1 or 1_1, the UE shall apply a minimum scheduling offset restriction indicated based on 'Minimum applicable scheduling offset indicator' value '0'. When the minimum scheduling offset restriction is applied the UE is not expected to be scheduled with a DCI in slot n to receive a PDSCH scheduled with C-RNTI, CS-RNTI or MCS-C-RNTI with K0 smaller than K0min⋅2μ'2μ, where K0min and μ are the applied minimum scheduling offset restriction and the numerology of the active DL BWP of the scheduled cell when receiving the DCI in slot n, respectively, and μ' is the numerology of the new active DL BWP in case of active DL BWP change in the scheduled cell and is equal to μ, otherwise. The minimum scheduling offset restriction is not applied when PDSCH transmission is scheduled with C-RNTI, CS-RNTI or MCS-C-RNTI in common search space associated with CORESET0 and default PDSCH time domain resource allocation is used, in the search space set provided by recoverySearchSpaceId when monitoring PDCCH as described in [6, TS 38.213] or when PDSCH transmission is scheduled with SI-RNTI, MSGB-RNTI, RA-RNTI or TC-RNTI. The application delay of the change of the minimum scheduling offset restriction is determined in Clause 5.3.1. *** Unchanged text is omitted *** ----------------------------------------------- End of TP of TS 38.214 --------------------------------------------------------

- References

- Chairman's Notes RAN1#104-e, 2021-02.

- 3GPP TS 38.214 V16.5.0, 2021-3.

- Chairman's Notes RAN1#96bis final, 2019-04.