3GPP TSG RAN WG1 #104b-e		 R1-2103638

e-Meeting, April 12th – 20th, 2021

Agenda Item:	7.2.4

Source: 	ASUSTeK

Title:  	Remaining issues on DL pathloss for sidelink power control

Document for:	Discussion and Decision

- Introduction

In this contribution, we discuss about remaining issues on sidelink power control, which is related to DL pathloss in case of Uu-interface L1 link problem.

- Discussion

Agreements:

For open-loop power control based on DL pathloss, the UE calculates DL pathloss using

- The RS resource used for power control for PUSCH transmission scheduled by DCI format 0_0 when the UE monitors DCI format 0_0.

- A RS resource from the SS/PBCH block that the UE uses to obtain MIB when the UE does not monitor DCI format 0_0.

In RAN1 #101-E meeting [1], it is agreed that a UE can be configured to use DL pathloss for sidelink power control. When the UE is configured to monitor PDCCH for detection of DCI format 0_0, the DL pathloss is derived using RS for PUSCH power control scheduled by DCI format 0_0. When the UE is not configured to monitor PDCCH for detection of DCI format 0_0, the DL pathloss is derived using RS resource from the SS/PBCH block that the UE uses to obtain MIB.

RAN2 agrees that mode-1 UE is allowed to continue using the configured SL grant type 1 when beam failure occurs or T310 is running [2][3]. Moreover, mode-2 UE can also perform sidelink transmission when beam failure occurs or T310 is running. In case these Uu-interface L1 problems happens, mode-2 UE and the mode-1 UE with configured SL grant Type 1 may not detect DL RSs normally, especially the DL RS for PUSCH power control scheduled by DCI format 0_0. In other words, DL pathloss derived from DL RS for DCI format 0_0-based PUSCH may be not accurate and be meaningless for sidelink power control. Thus, for a UE configured to monitor PDCCH for detection of DCI format 0_0, one simple way during Uu-interface L1 problems is fall back to use SS/PBCH block that the UE uses to obtain MIB.

Observation 1: In case of Uu-interface beam failure recovery or T310 is running, DL pathloss seems unavailable from the RS resource used for PUSCH power control scheduled by DCI format 0_0.

Proposal 1:  When Uu-interface beam failure recovery procedure or T310 is running, a RS resource from the SS/PBCH block that the UE uses to obtain MIB is used for DL pathloss-based sidelink power control.

Justification

- When Uu-interface beam failure recovery procedure or T310 is running, DL pathloss derived from DL RS for DCI format 0_0-based PUSCH is not accurate and is meaningless for sidelink power control. In this case, a RS resource from the SS/PBCH block that the UE uses to obtain MIB is used for DL pathloss-based sidelink power control.

Text Proposal 1

According text proposal for DL pathloss used for sidelink power control is provided below.

<   Text Proposal 1 for 38.213 >

### 16.2.1 PSSCH A UE determines a power PPSSCH,b,c(i) for a PSSCH transmission on a resource pool in symbols where a corresponding PSCCH is not transmitted in PSCCH-PSSCH transmission occasion i as: PPSSCH(i)=minPCMAX,PMAX,CBR,minPPSSCH,Di,PPSSCH,SL(i) [dBm] where - PCMAX is defined in [8-1, TS 38.101-1] - PMAX,CBR is determined by a value of sl-MaxTransPower based on a priority level of the PSSCH transmission and a CBR range that includes a CBR measured in slot i-N [6, TS 38.214]; if sl-MaxTransPower-r16 is not provided, then PMAX,CBR=PCMAX; - if dl-P0-PSSCH-PSCCH is provided - PPSSCH,D(i)=PO,D+10log102μ⋅MRBPSSCHi+αD⋅PLD [dBm] - elseif sl-P0-PSSCH-PSCCH is provided - PPSSCH,D(i)=minPCMAX,PMAX,CBR,PPSSCH,SL(i) [dBm] - else - PPSSCH,D(i)=minPCMAX,PMAX,CBR [dBm] where - PO,D is a value of dl-P0-PSSCH-PSCCH if provided - αD is a value of dl-Alpha-PSSCH-PSCCH, if provided; else, αD=1 - PLD=PLb,f,c(qd) as described in Clause 7.1.1 except that - the RS resource is the one the UE uses for determining a power of a PUSCH transmission scheduled by a DCI format 0_0 when the UE is configured to monitor PDCCH for detection of DCI format 0_0 and the UE does not perform link recovery procedure on the serving cell c - the RS resource is the one corresponding to the SS/PBCH block the UE uses to obtain MIB when the UE is not configured to monitor PDCCH for detection of DCI format 0_0 or when the UE is performing link recovery procedure on the serving cell c <Omited> ### 16.2.3 PSFCH A UE with Nsch,Tx,PSFCH scheduled PSFCH transmissions, and capable of transmitting a maximum of Nmax,PSFCH PSFCHs, determines a number NTx,PSFCH of simultaneous PSFCH transmissions and a power PPSFCH,k(i) for a PSFCH transmission k, 1≤k≤NTx,PSFCH, on a resource pool in PSFCH transmission occasion i as - if dl-P0-PSFCH is provided, PPSFCH,one=PO,PSFCH+10log102μ+αPSFCH⋅PL [dBm] where - PO,PSFCH is a value of dl-P0-PSFCH - αPFSCH is a value of dl-Alpha-PSFCH, if provided; else, αPFSCH=1 - PL=PLb,f,c(qd) as described in Clause 7.1.1 except that - the RS resource is the one the UE uses for determining a power of a PUSCH transmission scheduled by a DCI format 0_0 when the UE is configured to monitor PDCCH for detection of DCI format 0_0 and the UE does not perform link recovery procedure on the serving cell c - the RS resource is the one corresponding to the SS/PBCH block the UE uses to obtain MIB when the UE is not configured to monitor PDCCH for detection of DCI format 0_0 or when the UE is performing link recovery procedure on the serving cell c

Proposal 2: 	Adopt the text proposal 1 in updating of TS38.213 section 16.2.1 and 16.2.3, for DL pathloss in case of Uu-interface L1 problem.

- Conclusion

In this contribution, we have following proposals:

Observation 1: In case of Uu-interface beam failure recovery or T310 is running, DL pathloss seems unavailable from the RS resource used for PUSCH power control scheduled by DCI format 0_0.

Proposal 1:  When Uu-interface beam failure recovery procedure or T310 is running, a RS resource from the SS/PBCH block that the UE uses to obtain MIB is used for DL pathloss-based sidelink power control.

Proposal 2: 	Adopt the text proposal 1 in updating of TS38.213 section 16.2.1 and 16.2.3, for DL pathloss in case of Uu-interface L1 problem.

- Reference

- 3GPP TSG RAN WG1 #101-E Chairman’s Notes

- Report of 3GPP TSG RAN2#107 meeting

- Report of 3GPP TSG RAN2#108 meeting