3GPP TSG RAN WG1 #104b-e	R1-2103646

e-Meeting, April 12th – 20th, 2021

Source:	Ericsson

Title:	Reduced Latency SCell Activation

Agenda Item:	8.13.3

Document for:	Discussion and Decision

# Introduction

In this contribution we discuss UE procedures/requirements for SCell activation procedure using ‘temporary RS’ including triggering of ‘temporary RS’ and QCL aspects related to ‘temporary RS’.

# Discussion

## Rel 15/16 SCell activation

Figure 1 below shows Rel15/16 SCell activation procedure and Table 1 lists UE and NW steps for SCell activation

Figure 1 – SCell activation in Rel15/16

Table 1 – Rel15/16 SCell activation procedure

|  | Timeline | NW action | UE action | Notes |
| --- | --- | --- | --- | --- |
| 1 | Slot n | Tx PDSCH with activation command MAC CE (on a cell other than SCell) | Rx PDSCH with activation command MAC CE (on a cell other than SCell) |  |
| 2 | Slot n+k1 | Rx HARQ-ACK for PDSCH | Tx HARQ-ACK for PDSCH |  |
| 3 | Slot n+k1+3ms |  | Start CSI reporting for SCell | UE also has to acquire sync/AGC for SCell but exactly how the UE does this is left to UE implementation (there are no UE procedures defined for this in RAN1 spec). RAN4 requirements (sub-clause 8.3.2 of 38.133) allow UE to report OOR CSI value when UE is achieving sync/AGC for the SCell |
| 4 | Slot n+x | Tx CSI-RS on SCell | Rx CSI-RS on SCell | The CSI-RS can be a periodic/SP/aperiodic CSI-RS. Slot n+x can be any slot starting with n+k1+3ms as specified in 38.213 sub-clause 4.3. If aperiodic/SP-CSI RS, the corresponding CSI-RS trigger should be sent on primary cell or another SCell |
| 5 | Slot n+y | Rx valid CSI report for SCell (on a cell other than SCell) | Tx valid CSI for SCell (on a cell other than SCell) | Delay between slot n and slot n+y can be considered as effective SCell activation delay since after n+y, the UE should be ready to apply SCell activation related actions described in 38.213 and 38.321 RAN4 specs have a UE requirement that SCell activation cannot be later than slot n+THARQ+Tactivation_time+TCSI_ReportingNR slot lengthas specified in sub-clause 8.3.2 of 38.133 which is the maximum allowed SCell activation delay Slot n+y can be earlier than maximum allowed SCell activation delay (e.g. if a SSB/P-TRS is available closer to slot n+k1+3ms) |
| 6 | Slots after n+y | Start PDCCH/PDSCH Tx and other activation related actions on SCell | Start PDCCH/PDSCH Rx and other activation related actions on SCell |  |

## SCell activation with temporary RS

In RAN1#104-e following was agreed

Agreement

For efficient activation of SCells, down select at least one option from below:

- Option 1a: MAC CE(s) contained in a single PDSCH to trigger both SCell activation and corresponding temporary RS(s)

  - Details FFS including timeline design for receiving temporary RS

- Option 1b: A single DCI to trigger both SCell activation and corresponding temporary RS(s)

  - Details FFS including potential impact on SCell activation related procedures and, e.g. timeline design for SCell activation and for receiving temporary RS

  - FFS: The same DCI for SCell deactivation

- Option 2: A Rel-15/16 SCell activation MAC-CE to trigger SCell activation and a Rel-15/16 DCI to trigger corresponding temporary RS(s) with enhancement of timeline

  - Details FFS including timeline design for receiving a DCI trigger of temporary RS, and for receiving temporary RS

- Note: Companies are encouraged to provide complete solutions for fast SCell activation.

- Note: the previous agreement on the definitions of Alt 1 and Alt 2 is still effective

Figure 2 below shows expected Rel17 SCell activation procedure and Table 2 lists UE and NW steps with detailed timeline.

Figure 2 – SCell activation with reduced delay for Rel17

Table 2 – Rel17 SCell activation procedure

|  | Timeline | NW action | UE action | Notes |
| --- | --- | --- | --- | --- |
| 1 | Slot n | Tx SCell activation command (on a cell other than SCell) | Rx SCell activation command (on a cell other than SCell) | Options for SCell activation command a) Rel15/16 MAC CE (no spec change needed) b) new Rel17 MAC CE c) new Rel17 DCI based activation command |
| 1a | Slot n+m1 | Tx ‘temporary RS’ trigger (on a cell other than SCell) | Rx ‘temporary RS’ trigger (on a cell other than SCell) | Options for ‘temporary’ RS trigger a) Rel15/16 TRS trigger using DCI 0_1 (no spec change needed) – no NW restriction on slot n+m1 a1) Rel15/16 TRS trigger using DCI 0_1 – slot n+m1 occurs after slot n+k1+3ms b) new Rel17 MAC CE with SCell activation command and also TRS trigger (in this case m1=0, i.e., ‘integrated trigger’) c) new Rel17 DCI based trigger for TRS which also includes SCell activation command (in this case m1=0, i.e., ‘integrated trigger’) d) Rel15/16 SCell activation command serves as implicit trigger for a specific RRC configured TRS configuration (in this case m1=0 .i.e., ‘integrated trigger’) e) new Rel17 TRS trigger via DCI 1_1 – no NW restriction on slot n+m1 f) new Rel17 MAC CE with ‘temporary RS’ trigger (if sent in same PDSCH as Rel15/16 SCell activation MAC CE, m1=0 i.e., ‘integrated trigger’) |
| 2 | Slot n+k1 | Rx HARQ-ACK for SCell activation command | Tx HARQ-ACK for SCell activation command |  |
| 3 | Slot n+k1+3ms |  | Start CSI reporting for SCell | UE also has to acquire sync/AGC for SCell but exactly how the UE does this is left to UE implementation (there are no UE procedures defined for this in RAN1 spec). RAN4 requirements (sub-clause 8.3.2 of 38.133) allow UE to report OOR CSI value when UE is achieving sync/AGC for the SCell Note: If option c is used in step 1a (i.e., L1 based SCell activation, then this slot can be earlier, further RAN4 input will be needed to determine the smaller delay) |
| 3a | Slot n+m2 | Tx TRS on the SCell | Rx TRS on the SCell | Slot n+m2 should be later than slot n+k1+3ms When ‘temporary RS’ is same as Rel15 TRS (i.e., per current WA), no spec change needed for supporting this step If RAN4 identifies cases where new TRS structure is needed (e.g. with more bursts), new Rel17 TRS configuration can be introduced but same triggers as in Rel15/16 can still be used Note: as mentioned is step 3, even when temporary RS is triggered, there is no need to specify exactly how UE acquires sync/AGC for SCell and UE is free to use any approaches implemented for Rel15/16 with potential extra help of temporary RS to speed up the sync/AGC |
| 4 | Slot n+x | Tx CSI-RS on SCell | Rx CSI-RS on SCell | The CSI-RS can be a periodic/SP/aperiodic CSI-RS. Slot n+x can be any slot starting with n+k1+3ms as specified in 38.213 sub-clause 4.3. If aperiodic/SP-CSI RS, the corresponding CSI-RS trigger should be sent on primary cell or another SCell. It should be possible to derive the CSI-RS TCI state from ‘temporary RS’ received by the UE on the SCell. If UE is forced to wait for next SSB/P-TRS, then ‘temporary RS’ will not help in reducing SCell activation delay. |
| 5 | Slot n+y | Rx valid CSI report for SCell (on a cell other than SCell) | Tx valid CSI report for SCell (on a cell other than SCell) | Delay between slot n and slot n+y is can be considered as effective SCell activation delay since after n+y, the UE should be ready to apply SCell activation related actions described in 38.213 and 38.321. RAN4 specs have a UE requirement that SCell activation cannot be later than slot n+THARQ+Tactivation_time+TCSI_ReportingNR slot lengthas specified in sub-clause 8.3.2 of 38.133 which is the maximum allowed SCell activation delay. It is expected that Rel17 RAN4 specs will specify a smaller allowed Tactivation_time for cases where ‘temporary RS’ is available to the UE. Slot n+y can be earlier than maximum allowed SCell activation delay e.g. if a SSB/P-TRS is available closer to slot n+k1+3ms. In such cases, there is no need for NW to trigger an additional ‘temporary RS’. |
| 6 | Slots after n+y | Start PDCCH/PDSCH Tx and other activation related actions on SCell | Start PDCCH/PDSCH Rx and other activation related actions on SCell | It should be possible to derive the PDCCH TCI state from ‘temporary RS’ received by the UE on the SCell. If this is not allowed and UE is forced to wait for next SSB or P-TRS, then ‘temporary RS’ will not help in reducing SCell activation delay. |

From above discussion, it is clear that Rel17 fast SCell activation can be achieved by largely following Rel15/16 UE procedures.  As indicated in steps 4, 6 in Table 2 above (blue highlighted), for receiving initial CSI-RS/PDCCH/PDSCH transmissions on the SCell, the UE procedures should be updated to allow the TRS (used as ‘temporary RS’) to serve as QCL source. In RAN1#104-e, it was also discussed that the TRS trigger should be sent only after slot n+k1+3ms to assist UE implementation. This can be achieved with minimal change to existing procedures.

Observation 1

- Rel15/16 specs already support an SCell activation trigger (i.e., the SCell activation command MAC CE) and a ‘temporary RS’ trigger (i.e., DCI 0_1 trigger for TRS) and Rel17 fast SCell activation can be achieved using these triggers.

Observation 2

- Rel15/16 RAN1 specs do not specify an explicit timeline or the specific RSs used for acquiring sync/AGC on an SCell during SCell activation and Rel17 fast SCell activation can be achieved by retaining similar approach.

Observation 3

- ‘Temporary RS’ can help UE speed up the SCell activation procedure but it is not always needed for fast SCell activation (e.g. when there is an SSB, P-TRS near the SCell activation command MAC CE).

Based on above discussion, we propose the following

Proposal 1

- ‘Rel15/16 SCell activation command MAC CE’ and ‘Rel15/16 DCI 0_1 trigger for TRS’ are supported as the ‘SCell activation trigger’ and ‘temporary RS’ trigger respectively, for Rel17 fast SCell activation.

  - If the Rel15/16 SCell activation MAC CE is received in slot n, and if the UE receives DCI 0_1 in a slot later than n+k1+3ms that triggers an A-TRS transmission on the SCell, the UE requirement for maximum allowed SCell activation latency is set by also taking into account the availability of A-TRS transmission on SCell (Specification of this performance requirement is handled by RAN4).

Also, based on the QCL related discussion above, we make the following observation and proposal.

Observation 4

- UE should be allowed to use ‘temporary RS’ as a QCL source for initial CSI-RS and initial PDCCH/PDSCH DMRS received on the SCell after activation. If UE is forced to wait for next SSB or P-TRS, ‘temporary RS’ will not help in reducing SCell activation delay for Rel17.

Proposal 2

- The TRS used in SCell activation delay reduction should serve as QCL source for the subsequent CSI-RS (based on which the UE reports valid CSI/L1-RSRP upon SCell activation) and other DL RSs on the SCell: CSI-RS, PDCCH/PDSCH DMRS.

- Note: This applies to SCell activation +TRS triggering by Rel15/16 mechanism and any potential new triggering mechanism(s) introduced for Rel17.

Proposal 1 is linked to Option 2 in the RAN1#104-e agreement. This option achieves fast SCell activation by largely reusing the existing signaling and UE procedures. In previous meetings, several other alternatives (corresponding to Alt 1a and Alt 1b in the RAN1#104-e agreement) that require new signaling/UE procedures were discussed. If it is identified (e.g. based on other WG input as discussed in [1]) that for some scenarios, additional flexibility compared to Rel15/16 DCI 0_1 based TRS triggering is needed, one of Alt 1a or Alt 1b can be specified in addition to Alt 2. However, specification of additional signaling options should not come at the cost of precluding the use of signaling that is already specified.

The triggering approaches related to Alt-1a and Alt 1b are listed as options b, c, d, e, f in step 1a of Table 2. In our previous contribution [2], a detailed discussion about these alternatives and their pros and cons compared to existing Rel15/16 triggers is provided.

Considering the input in [1], it may be likely that for some activation scenarios, a temporary RS with multiple bursts of A-TRS is needed fast SCell activation. If a new trigger is being introduced, a new MAC CE is preferable as it allows more flexibility. Given this, we prefer Alt 1a over Alt 1b in case an additional triggering mechanism is specified for Rel17.

Observation 5

- If it is identified (e.g. based on other WG input) that for some scenarios, additional flexibility compared to Rel15/16 DCI 0_1 based TRS triggering is needed, a new triggering mechanism for temporary RS can be specified in Rel17 in addition to reusing the existing Rel15/16 triggers.

  - MAC CE based new trigger is preferable as it provides more flexibility.

- Specification of new triggering mechanism should not come at the cost of precluding the use of already specified Rel15/16 signaling to trigger the Rel15/16 TRS for fast SCell activation.

  - RAN4 input already indicates that Rel15/16 TRS provides reduction in maximum allowed activation delay at least for some SCell activation cases.

## Configuration of ‘temporary RS’

According to the current RAN1 WA, Rel15/16 TRS structure is re-used for ‘temporary RS’. For this case, there is no need to define a separate ‘temporary RS’ configuration in addition to already existing TRS configuration. If a need to extend current TRS structure is identified (e.g. as discussed in [1]), the current TRS RRC structure can be used as starting point for defining the configuration details.

Observation 6

- For cases where Rel15/16 TRS structure is re-used for ‘temporary RS’, there is no need to define a separate ‘temporary RS’ configuration in addition to already existing TRS configuration.

- If a need to extend current TRS structure is identified, the current TRS RRC structure can be used as starting point for defining the associated configuration details.

# Conclusions

In this document we discuss fast SCell activation for Rel17 NR CA and make the following observations and proposals

Observation 1

- Rel15/16 specs already support an SCell activation trigger (i.e., the SCell activation command MAC CE) and a ‘temporary RS’ trigger (i.e., DCI 0_1 trigger for TRS) and Rel17 fast SCell activation can be achieved using these triggers.

Observation 2

- Rel15/16 RAN1 specs do not specify an explicit timeline or the specific RSs used for acquiring sync/AGC on an SCell during SCell activation and Rel17 fast SCell activation can be achieved by retaining similar approach.

Observation 3

- ‘Temporary RS’ can help UE speed up the SCell activation procedure but it is not always needed for fast SCell activation (e.g. when there is an SSB, P-TRS near the SCell activation command MAC CE).

Proposal 1

- ‘Rel15/16 SCell activation command MAC CE’ and ‘Rel15/16 DCI 0_1 trigger for TRS’ are supported as the ‘SCell activation trigger’ and ‘temporary RS’ trigger respectively, for Rel17 fast SCell activation.

  - If the Rel15/16 SCell activation MAC CE is received in slot n, and if the UE receives DCI 0_1 in a slot later than n+k1+3ms that triggers an A-TRS transmission on the SCell, the UE requirement for maximum allowed SCell activation latency is set by also taking into account the availability of A-TRS transmission on SCell (Specification of this performance requirement is handled by RAN4).

Observation 4

- UE should be allowed to use ‘temporary RS’ as a QCL source for initial CSI-RS and initial PDCCH/PDSCH DMRS received on the SCell after activation. If UE is forced to wait for next SSB or P-TRS, ‘temporary RS’ will not help in reducing SCell activation delay for Rel17.

Proposal 2

- The TRS used in SCell activation delay reduction should serve as QCL source for the subsequent CSI-RS (based on which the UE reports valid CSI/L1-RSRP upon SCell activation) and other DL RSs on the SCell: CSI-RS, PDCCH/PDSCH DMRS.

- Note: This applies to SCell activation +TRS triggering by Rel15/16 mechanism and any potential new triggering mechanism(s) introduced for Rel17.

Observation 5

- If it is identified (e.g. based on other WG input) that for some scenarios, additional flexibility compared to Rel15/16 DCI 0_1 based TRS triggering is needed, a new triggering mechanism for temporary RS can be specified in Rel17 in addition to reusing the existing Rel15/16 triggers.

  - MAC CE based new trigger is preferable as it provides more flexibility.

- Specification of new triggering mechanism should not come at the cost of precluding the use of already specified Rel15/16 signaling to trigger the Rel15/16 TRS for fast SCell activation.

  - RAN4 input already indicates that Rel15/16 TRS provides reduction in maximum allowed activation delay at least for some SCell activation cases.

Observation 6

- For cases where Rel15/16 TRS structure is re-used for ‘temporary RS’, there is no need to define a separate ‘temporary RS’ configuration in addition to already existing TRS configuration.

- If a need to extend current TRS structure is identified, the current TRS RRC structure can be used as starting point for defining the associated configuration details.

# References

[1] R1-2102300 “Reply LS on temporary RS for efficient SCell activation in NR CA”, RAN4 -> RAN1 LS, RAN1#104b-e, April 2021.

[2] R1-2101563 “Reduced Latency SCell Activation”, Ericsson, RAN1#104-e, January 2021.