3GPP TSG RAN WG1 #104bis-e	R1-2103676

e-Meeting, April 12 - 20, 2021

Agenda item:	7.2.10

Source:	Moderator (Nokia)

Title:	Moderator summary of MR DC-CA pre-meeting preparation phase

WI:	LTE_NR_DC_CA_enh-Core

Document for:	Discussion and Decision

# 1	Introduction

The following documents have been identified to be addressing topics related to multi-radio dual connectivity and carrier aggregation enhancements work item maintenance.

| TDoc | Title | Source | DC-PC | X-CC CSI |
| --- | --- | --- | --- | --- |
| R1-2102303 | Further Reply LS on power control for NR-DC (AI5) | RAN4 | PC1 |  |
| R1-2102929 | Draft Reply LS on power control for NR-DC (AI5) | vivo | PC1 |  |
| R1-2103757 | UL power control for NR-NR dual connectivity (AI5) | Huawei, HiSilicon | PC1 |  |
| R1-2102937 | Disucssion on CSI request constraint per slot considering xCC scheduling with different numerology (AI7.1) | vivo |  | XCC1 |
| R1-2102489 | Remaining Issues of Power Sharing for NR-DC | ZTE | PC1 |  |
| R1-2102683 | Remaining issues on Rel-16 carrier aggregation | MediaTek Inc. |  | XCC1 |
| R1-2102709 | On removal of p-NR-FR2 from NR-NR DC power control | Nokia, Nokia Shanghai Bell | PC1 |  |
| R1-2102949 | Maintenance on NR-DC power control | vivo | PC1 |  |
| R1-2103147 | Remaining issue on Multi-RAT Dual-Connectivity and Carrier Aggregation | Qualcomm Incorporated |  | XCC1 |
| R1-2103641 | Maintenance for Rel-16 MR-DC and CA enhancements | Ericsson | PC1 |  |
| R1-2103754 | Correction on the MR-DC Uplink Power Control in 38.213 | Huawei, HiSilicon | PC2 |  |

# 2	Summary of issues addressed in the Tdocs

| Tag | TDocs | Issue | Proposal for RAN1#104bis-e handling |
| --- | --- | --- | --- |
| PC1 | R1-2102489 R1-2102709 R1-2102929 R1-2102949 R1-2103641 R1-2103757 | Related to RAN4 LS on applicability of p-NR-FR2 in R1-2102303 Q1: whether there is any concern about p-NR-FR2 to be used in Rel-16 for supporting NR-DC PC on FR2? Yes, RAN4 do have concerns and the reason is similar to p-UE-FR2. RAN4 does not introduce p-NR-FR2 in Rel-16. | To be discussed in RAN1#104bis with a possible reply LS to RAN4 and RAN2 |
| PC2 | R1-2103754 | - In EN-DC, RRC parameter tdm-PatternConfig2 was agreed to replace tdm-PatternConfig-r16 as Rel-16 TDM pattern RRC parameter in CR R2-2006349, while tdm-PatternConfig-r16 under RRCConnectionResume-v1610-IEs is now referring to the Rel-15 TDM pattern instead of the Rel-16 one. In the latest version of TS 38.213, the tdm-PatternConfig-r16 is still used for Rel-16 TDM pattern, not in line with TS 36.331. - Similar issue can be found in the NE-DC. TS 36.331 has defined another RRC configuration tdm-PatternConfigNEDC for NE-DC, which is independent of the TDM pattern RRC configurations for EN-DC. In the latest version of TS 38.213, tdm-PatternConfig-r15 has been used for NE-DC, which is misaligned as well. - Thirdly, the uplinkTxSwitching-OptionSupport UE capability is incorrectly used in the latest version of TS 38.213, as the uplinkTxSwitching-OptionSupport is a UE capability for another Rel-16 feature UL Tx switching. Instead, the right UE capability that should be used here is tdm-restrictionDualTX-FDD-endc-r16 as defined in the latest version of specification TS 38.306: Proposed changes: - Correct the RRC parameter for TDM pattern used in uplink power control of EN-DC - Correct the RRC parameter for TDM pattern used in uplink power control of NE-DC - Correct TDM restriction for dual UL transmission UE capability in uplink power control of EN-DC | To be discussed in RAN1#104bis |
| XCC1 | R1-2102683 R1-2103147 R1-2102937 | A CR with the following was approved in RAN1#104 to TS38.214 (Rel-15 only) (R1-2102178), with Rel-16 left for further discussion: - A UE is not expected to receive more than one DCI with non-zero CSI request field per slot per cell. A UE is not expected to receive DCI with non-zero CSI request field within a cell group in a slot overlapping with any slot receiving DCI with non-zero CSI request field in the same cell group. - A UE is not expected to receive more than one aperiodic CSI report request for transmission in a given slot per cell. A UE is not expected to receive an aperiodic CSI report request for transmission in a slot overlapping with any slot having an aperiodic CSI report transmission in the same cell group. Conclusion [RAN1#104] The text proposal on CSI request constraint per slot and CSI reporting constraint only considers Rel-15 where PDCCH with which the CSI request is transmitted, the PUSCH on which the reports are to be transmitted, and the CSI-RS associated in the CSI reports triggered by the DCI have same numerology. The scenario where at least two of them have different numerology should be further discussed in Rel-16. | Develop a Rel-16 38.214 CR in RAN1#104bis |

Moderator proposal: Discuss the three issues identified in the submitted Tdocs in RAN1#104bis-e:

- PC1: Discuss the issue raised in R1-2102489, R1-2102709, R1-2102949, R1-2103641, R1-2103641 and R1-2103757 related to RAN4 LS in R1-2102303 and potentially develop and agree on a CR to 38.213 and/or a response LS to RAN4/RAN2

- PC2: Discuss and potentially agree the draft CR in R1-2103754

- XCC1: Discuss and potentially develop a Rel-16 38.214 CR in RAN1#104bis based on the proposals in R1-2102683 and R1-2103147