3GPP TSG-RAN WG1 Meeting #104-bis-e	R1-2103721

e-Meeting, April 12th – 20th, 2021

Agenda Item:	6.2.1

Source:	Ericsson

Title:	PUR maintenance issues for Rel-16 LTE-MTC

Document for:	Discussion and Decision

# 1	Introduction

In Release-16 the Work Item (WI) on “Additional MTC enhancements for LTE” [1], had as one of its objectives to specify the following improvement for machine-type communications for BL/CE UEs:

Improved UL transmission efficiency and/or UE power consumption: - Specify support for transmission in preconfigured resources in idle mode based on SC-FDMA waveform for UEs with a valid timing advance [RAN1, RAN2, RAN4] - Both shared resources and dedicated resources - Note: This is limited to orthogonal (multi) access schemes

In RAN1 #103-e, the following e-mail discussions were held:

- [104-e-LTE-eMTC5-01] PUR issues

The agreements from the e-mail discussions above can be found in [2]. This contribution clarifies for PUR the obtention of the “PRB resources for PUSCH transmissions” described in [3].

# 2	Discussion

The subsections below address per Technical Specification what has been identified to be still open or to be clarified.

## 2.1 	PRB resources for PUSCH transmission

In TS 36.211 [3], the following statement can be found “For BL/CE UEs, the PRB resources for PUSCH transmission in the first subframe are obtained from the DCI as described in clauses 5.3.3.1.10 and 5.3.3.1.11 in [3]”. The cited statement does not apply for PUR, since in this case the “PRB resources for PUSCH transmission” are obtained via higher layers from PUR-Config.

Accounting for the above, the proposed TP for clause 5.3.4 in TS 36.211 is as follows:

------------------------------------------------------- Text start (TS 36.211 Clause 5.3.4)-----------------------------------

For BL/CE UEs, the PRB resources for PUSCH transmission in the first subframe are obtained from the DCI as described in clauses 5.3.3.1.10 and 5.3.3.1.11 in [3], or from higher layers in PUR-Config when PUSCH is transmitted using preconfigured uplink resources. Each of the NTB≥1 PUSCH codewords is transmitted with repetitions, where NTB is the number of transport blocks defined in clause 8.0 of 3GPP TS 36.213 [4]. The PUSCH transmission spans NabsPUSCH≥NTBNrepPUSCH consecutive subframes, including subframes that are not BL/CE UL subframes where the UE postpones the PUSCH transmission if .

------------------------------------------------- Text end (TS 36.211 Clause 5.3.4)------------------------------------------

Agree on the text proposal presented in this document and capture its content in TS 36.211.

# 4	Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	Agree on the text proposals presented in this document and capture its content in TS 36.211.

# 5	References

RP-192875, Revised WID: Additional MTC enhancements for LTE, RAN #86, Sitges, Spain, December 9th-12th, 2019.

Session notes for 8.9 (Rel-17 enhancements for NB-IoT and LTE-MTC), Ad-hoc chair (Samsung), 3GPP TSG RAN WG1 Meeting #104-e, e-Meeting, January 25th – February 5th, 2021.

TS 36.211, v16.4.0, “Evolved Universal Terrestrial Radio Access (E-UTRA); Physical channels and modulation”.