3GPP TSG RAN WG1 Meeting #104-bis-e	R1- 2103729

# E-meeting,  April 12th – 20th, 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.213 | CR | 1385 | rev | - | Current version: | v15.13.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on the UE transmit power for the 2-of-3 subcarrier allocation in sub-PRB |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN1 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_eMTC4-Core |  |  |  |  |  | Date: |  |  | 2021-04-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In the UE’s transmit power equation, the term “” scales-down the power for a full-PRB allocation and sub-PRB allocations respectively as follows: - When 12 subcarriers are allocated MPUSCH,c = 0 dB, whereas when sub-PRB is used, for 6 allocated subcarriers MPUSCH,c = -3.0103 dB, for 3 allocated subcarriers MPUSCH,c = -6.0206 dB and for 2-of-3 allocated subcarriers MPUSCH,c = -7.7815 dB. The current “MPUSCH,c” calculation for the 2-of-3 allocation does not account for the fact that only 1 of the 2 utilized subcarriers is active at a time. That is, the two subcarriers are not simultaneously used, since its usage stochastically commutes one at a time among the two usable tones. Thus, the current specification provides a power scaling given by MPUSCH,c = (3+1-2)/12 = 2/12 (i.e., -7.7815 dB), instead of MPUSCH,c = 1/12 (i.e., -10.7918 dB). This means, that for the 2-of-3 subcarrier allocation currently there is an excess of transmit power of ⁓3dB. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To avoid an excess of transmit power of ⁓3dB when sub-PRB uses a 2-of-3 subcarrier allocation, and to keep unmodified “”, “Qm”, and “”, which are the legacy variables used by “MPUSCH,c”, in clause 5.1.1.1 the following is appended to the legacy “MPUSCH,c” equation as to obtain MPUSCH,c = 1/12 for the 2-of-3 subcarrier allocation case: “if ≠ 1 or MPUSCH,c(i)=MscRU-2NscRB if = 1”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The power scaling produced from a 2-of-3 subcarrier allocation would result in an excess of power of ⁓3dB. This excess of transmit power goes against the UE’s battery life. This also might cause additional interference to the eNB. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

----------------------------------------------------------------- Text Starts -----------------------------------------------------------------

#### 5.1.1.1	UE behaviour

The setting of the UE Transmit power for a Physical Uplink Shared Channel (PUSCH) transmission is defined as follows.

If the UE transmits PUSCH without a simultaneous PUCCH for the serving cell , then the UE transmit power  for PUSCH transmission in subframe/slot/subslot i for the serving cell is given by

[dBm]

If the UE transmits PUSCH simultaneous with PUCCH for the serving cell , then the UE transmit power  for the PUSCH transmission in subframe/slot/subslot i for the serving cell  is given by

[dBm]

----------------------------------------------------------------- Text Omitted --------------------------------------------------------------

-	If the UE is a BL/CE UE configured with higher layer parameter ce-PUSCH-SubPRB-Config-r15, and the PUSCH resource assignment valid for subframe i and serving cell is using uplink resource allocation type 5, is the bandwidth of the PUSCH resource assignment expressed in fraction of a resource block and is given by  if ≠ 1 or MPUSCH,c(i)=MscRU-2NscRB if = 1 where  are defined in [3] and is defined in Subclause 8.6.1 for subframe i, is the bandwidth of the PUSCH resource assignment expressed in number of resource blocks valid for subframe/slot/subslot i and serving cell  otherwise.

----------------------------------------------------------------- Text Ends -----------------------------------------------------------------