3GPP TSG RAN WG1 #104b-e		R1-2103731

E-meeting, April 12th – April 20th, 2021

Agenda Item:	8.4.2

Source: 	CEWiT, IITH, IITM, Tejas Networks, Reliance Jio

Title:	Further discussion on UL time synchronization methods for NTN systems

Document for:	Discussion and Decision

- Introduction:

In RAN plenary #86 a work item on non-terrestrial network for Rel 17 is agreed after the two phased study items during release 16 phase (RP-193234). Based on that the work item was initiated in RAN1 from meeting RAN1#102-e onwards. In the last Meeting RAN1#104e following agreement are drawn on UL time synchronization for NTN,

Agreement:

An NTN UE in RRC_CONNECTED state is required to support UE specific TA calculation based at least on its GNSS-acquired position and the serving satellite ephemeris.

FFS: Operation of closed loop and open loop TA control

Agreement:

For TA update in RRC_CONNECTED state, combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops shall be supported for NTN.

FFS: Details of the combination of open and closed loop TA control

Conclusion:

It is up to RAN4 to decide whether interruptions or measurement gaps are required for GNSS measurements during NTN operation

Agreement:

RAN1 should send an LS to RAN4 with the following questions:

Question 1: RAN1 would like to ask RAN4, to indicate what are the NTN UL time synchronization requirements?

- For initial access (i.e. PRACH transmission)

- For UL transmissions in RRC Connected State

Question 2: RAN1 would like to ask RAN4, to indicate what are the NTN UL frequency synchronization requirements?

- For initial access (i.e. PRACH transmission)

- For UL transmissions in RRC Connected State

Conclusion:

If DL frequency compensation for the service link Doppler is applied, indication of the amount of frequency compensation is necessary.

- FFS: support of DL frequency compensation for the service link Doppler.

Agreement:

- RAN1 to support satellite ephemeris broadcast based at least on one of the following format options:

  - Option 1: Ephemeris format based on satellite position and velocity state vectors

    - FFS: Details on state vectors formats

    - FFS: Details on time reference provisioning/format

  - Option 2: Ephemeris format based on orbital elements

    - FFS: Details on orbital elements formats

    - FFS: Details on time reference provisioning/format

- FFS: Whether down-selection is needed or both options are supported

This contribution tries to provide further details on UL time synchronization techniques that can be adopted in NTN.

- Initial TA acquisition for RACH process

During the RAN1#103-e meetings initial proposals on the estimation of the full TA acquisition was discussed. Full TA estimation can be divided as UE specific TA and Feeder link + gNB-Gateway delay.  Feeder link + gNB-Gateway delay may be provided to the UE as a common timing offset value.

Following equation is agreed to be used at the UE, where X will be common timing offset value.

  - Common TA and common timing drift rate

In the last meeting, extensive deliberation took place but with no agreement. Important point of discussion was the characteristic of common TA that will signalled by the gNB/network and the granularity of it. According to us the Common TA value with same granularity as the Rel 16 TA. In last meeting further modification is discussed to avoid confusing between close loop TA and UE specific TA. The corresponding proposal is,

TTA=NTA+NTA,UE-specific+NTA,common+NTA,offset×Tc

Where:

  NTA and NTA,offset are defined as in Release-16.

  NTA,UE-specific  is UE self-estimated TA

  NTA,common is network-controlled common TA and may include any timing offset considered necessary by the network.

We support this proposal. In general, it is common understanding that the common TA is function of reference point that network decides for the purpose of aligning the DL and UL frame at gNB. Therefore, it will define the round-trip delay (RTT) between reference point and satellite. The reference point could be gateway or gNB itself which UE need not to know. The granularity of the common TA should be same as granularity use in Rel 16 TA.

gNB can send this common TA value over DL broadcast message in the NTN specific SIB message along with other NTN specific broadcast messages.

Proposal 1: The final equation for the full TA at UE should be,

TTA=NTA+NTA,UE-specific+NTA,common+NTA,offset×Tc

Where:

  NTA and NTA,offset are defined as in Release-16.

  NTA,UE-specific  is UE self-estimated TA

  NTA,common is network-controlled common TA and may include any timing offset considered necessary by the network.

Proposal 2: gNB broadcasts the common TA value in the NTN specific SIB message along with other NTN specific broadcast messages.

In case of LEO based deployment, the feeder link delay will be changing due to the relative speed of the satellite. This will make the common TA obsolete after certain time. This topic is also discussed extensive in last meeting. To maintain the common TA accuracy, it is necessary to signal the common TA frequently which will over burden of UE as well as the network from resource perspective. One possible way could be, as discussed during last meeting, the indication of the common timing drift rate. This will enable the UE to track the change in the common TA and correct it appropriately after a specific interval. Therefore, indication of the common timing drift rate is necessary. Similarly, this value can be broadcasted along with the common TA value. Let us assume δcomm_drift is common TA drift rate which is received at timestamp t0 and  Tperiod is the duration after which UE should update the common TA then  NTA,common,new is given as,

NTA,common,new= NTA,common  + δcomm_drift  Tperiod

Proposal 3: In NTN, the network may broadcast a common timing drift rate and update duration to update the common TA by UE in periodic interval. It can be broadcasted in the NTN specific SIB.

  - TA marginal

Other aspect is accuracy of the acquisition of satellite position using ephemeris and rate of change of TA by the UE. In general, due to different UE classes, it will be difficult to assume the similar performance across the UEs with respect to the TA estimation. The UE may overestimate or underestimate the TA due many reasons like different UE classes, operating condition like environmental conditions, multipath scenarios, tropospheric and ionospheric losses etc. Therefore, this particular scenario should be considered while designing the signalling for the UE specific TA estimation. Similarly, it is necessary for gNB to have the control over the TA that the UE applies finally for UL transmission for better gNB resource utilization as well as for security perspective. A TA margin value is being discussed for this purpose. The TA margin will account for the any uncertainty in the full TA that UE measures. Further, it will maintain the TA unipolar and will save required changes in the RAR TAC bit field. Multiple options on TA margin were discussed in the last meeting.

We believe, there should be provision to configure the TA margin directly or indirectly to the UE. gNB can directly configure the TA margin through the SIB. Other method could be to interpret the TA margin as fraction or multiple of the CP of the configured PRACH. In both cases there should be TA margin related indication from the gNB. But TA margin should not be part of the common TA as the common TA will be same across the UEs, whereas TA margin needs to be configured UE specifically. Common TA will counter the common delays e.g. the feeder link delay, but TA margin can be different across UE classes.

Observation 1: TA margin is necessary to control any uncertainty in the full TA estimation at UE. It will be UE dependent.

Proposal 4: TA margin should be configured to the UE directly or indirectly to control the uncertainty in the full TA estimation at the UE. Full TA equation including TA margin will be TA=NTA+NTA,UE-specific+NTA,common+NTA,offset-NTA, margin ×Tc.

Proposal 5: TA margin can be configured indirectly as fraction or multiple of the CP of the configured PRACH. NTA, margin=Y* CPPRACH. FFS Y value.

Further, as TA margin will eliminate the need to have bipolar TA update command in RAR. Therefore, there is no need to define bipolar TA command for NTN. Similarly, 12 bits TAC field in the RAR message will be sufficient for the NTN purpose in Rel 17 and there is no need to extend it in Rel 17. In meeting 103-e a working assumption is defined as below,

Working assumption:

It is assumed that the requirement on UL time pre-compensation for Msg1/MsgA transmission of an NR NTN UE in idle/inactive mode will be defined such that the existing TAC 12-bit field in msg2 (or msgB) can be reused without any extension.

We propose to agree the working assumption on TAC 12-bit field reuse in this meeting.

Proposal 6: Agree the working assumption on TAC 12-bit field reuse.

  - UE specific TA update under gNB/network control

In NTN systems, it is necessary to specify the accuracy and availability of the GNSS for TA estimation for GNSS based TA estimation. Similarly, the possibility of GNSS position degradation also should be considered while designing the TA pre-compensation solution. As mentioned previously, due to different UE classes, it will be difficult to assume similar performance across the UEs for the TA estimation. This may lead to asymmetric behaviour of the UEs w.r.t. TA estimation over which network or gNB cannot have control. Therefore, it is necessary for gNB to have the control over TA that the UE applies for the UL transmission for better gNB resource utilization as well as for security purposes.

Observation 2: gNB should have control over the TA that the UE applies for its UL transmission for better gNB resource utilization as well as for security purposes.

Thus, the UE can estimate its TA using the GNSS capability and the assistance information received from the gNB, but it should also refine the estimated UE specific TA using the gNB instruction. As an example, gNB can provide method to quantize the estimated TA by predefined step size. The step size can be the TA margin which can be configured to the UE in UE specific manner. UE should report back the applied TA to the gNB. This will give the gNB better control over the UE’s behaviour.

gNB can use this for better scheduling decisions, better resource management and, to maintain the connected mode timing advance.

Proposal 7: gNB should provide the set of instructions to refine the TA estimated by the UE for better control of the gNB over UE specific TA estimation.

Proposal 8: UE should report the applied TA to the gNB for better control over UE behaviour.

Proposal 9:  In UE specific TA estimation, TA value estimated by UE is quantized using predefined step size to get the final TA. The step size can be the TA margin configured to the UE.

  - Reporting UE specific TA to the gNB:

In terrestrial network, gNB has the knowledge of the UE’s TA, therefore, it can manage the UE’s UL transmissions and resources better. But in NTN, as UE estimate its own TA (e.g., UE specific TA part), it will be advantageous to report back the estimated TA to the gNB. This is necessary to avoid undesirable behaviour of UE. As mentioned in the last sub-section, UE can report back the TA value applied in terms of the number of steps. This will reduce the required bits for TA reporting compared with reporting actual TA estimated at UE.

Proposal 10: UE will report the applied TA to the gNB in terms of number of steps used in the quantization of TA.

During the study phase, a solution shown in the figure 1(a) was discussed and captured in TR 38.821 for TA acquisition for UL in NTN. It shows the reference point on the ground which will be provided by gNB as offset into the final UE specific TA value. This reference TA can be minimum TA (TAmin) value which can be provided by gNB per beam and/or cell specific as gNB/satellite can know the minimum and maximum point on the footprint.

Figure 1: a) Agreed mechanism in TR 38.821 b) Shows that the reference point in (a) can be minimum TA (TAmin) than can be experience in the beam.

TAminvalue will be useful for UEs with limited or no GNSS capability to acquire the UE specific TA. It can act as lower threshold for the UE estimating its own TA value. This will make the NTN TA mechanism forward compatible with respect to the fact that in future non GNSS capable UE also can be brought under NTN deployment.

Proposal 11: gNB can provide the TAmin value to the UE per beam/cell where TAmin is the minimum timing advance UE can experience in the beam/cell.

- TA updating in connected mode

Maintaining the TA value in connected mode is also challenging especially in LEO scenario compared with GEO as the LEO satellite will be in motion, and its speed will be around 8 Km/s. This makes the TA estimated to go invalid over time. Therefore, there is need for TA update over the time. As mentioned in above section, gNB can convey the satellite velocity information including velocity and velocity vector along with the ephemeris. This can be used for calculating the rate of change in the velocity.

In TN system, the TA will be updated using MAC-CE based TA command. The present range of TA is not sufficient for the NTN system and moreover due to satellite motion if MAC-CE command-based TA correction is used in TA then required overhead will be very high. Therefore, autonomous TA correction in connected mode is beneficial.

Proposal 12: UE will correct the TA in connected mode using velocity information of satellite apart from the MAC-CE TA based update.

In the previous meeting, above concept was discussed considering three solutions 1) open loop 2) closed loop and 3) combined open loop and closed loop TA update in connected mode. Open loop TA maintenance is based on the estimation of change in TA at UE and updating the TA. This can be given as, NTA,new= NTA,old+∆NTA ,where ∆NTA is the change in the TA which the UE will estimate using the knowledge of the satellite motion and its own velocity. So, the new TA equation will be TA=NTA,old ± ∆NTA ×Tc. In case of closed loop TA update the gNB will provide change in the TA i.e., using MAC-CE based TA command. This case TA update equation will be TA=(NTA.old+Ta-31.16×642μ )×Tc. In closed loop gNB can additionally indicate the additional time drift apart from the MAC-CE TAC which will be common in the cell. This value can be derived based on the reported UE specific TA to gNB and any other known error in the estimation. For better control over TA update, it is better to adopt the combination of open loop and closed loop TA update in connected mode.

Proposal 13: In connected mode, combination of open and closed loop TA update should be adopted. New TA value update equation will be, TA=NTA,old ± ∆NTA+Ta-31.16.642μ )×Tc ×Tc .

Proposal 14: The ∆NTA will be determined by UE using estimated drift value and additional drift provided by gNB.

- Conclusions

Following are the observations and proposals for NTN presented in this contribution.

Observation 1: TA margin is necessary to control any uncertainty in the full TA estimation at UE. It will be UE dependent.

Observation 2: gNB should have control over the TA that UE applies for its UL transmission for better gNB resource utilization as well as for security specific.

Proposal 1: The final equation for the full TA at UE should be,

TTA=NTA+NTA,UE-specific+NTA,common+NTA,offset×Tc

Where:

  NTA and NTA,offset are defined as in Release-16.

  NTA,UE-specific  is UE self-estimated TA

  NTA,common is network-controlled common TA and may include any timing offset considered necessary by the network.

Proposal 2: gNB broadcast the common TA value in the NTN specific SIB message along with other NTN specific broadcast messages.

Proposal 3: In NTN, the network may broadcast a common timing drift rate and update duration to update the common TA by UE in periodic interval. It can be broadcasted in the NTN specific SIB.

Proposal 4: TA margin should be configured to the UE directly or indirectly to control the uncertainty in the full TA estimation at the UE. Full TA equation including TA margin will be TA=NTA+NTA,UE-specific+NTA,common+NTA,offset-NTA, margin ×Tc.

Proposal 5: TA margin can be configured indirectly as faction or multiple of the CP of the configured PRACH. NTA, margin=Y* CPPRACH. FFS Y value.

Proposal 6: Agree the working assumption on TAC 12-bit field reuse.

Proposal 7: gNB should provide the set of instructions to refine the TA estimated by the UE for better control of the gNB over UE specific TA estimation.

Proposal 8: UE should report the applied TA to the gNB for better control over UE’s behaviour.

Proposal 9:  In UE specific TA estimation, TA value estimated by UE is quantized using predefined step size to get the final TA. The step size can be the TA margin configured to the UE. Proposal 10: UE will report the applied TA to the gNB in terms of number of steps used in the quantization of TA.

Proposal 11: gNB can provide the TAmin value to the UE per beam/cell where TAmin is the minimum timing advance UE can experience in the beam/cell.

Proposal 12: UE will correct the TA in connected mode using velocity information of satellite apart from the MAC-CE TA based update.

Proposal 13: In connected mode, combination of open and close loop TA update should be adopted. New TA value update equation will be, TA=NTA,old ± ∆NTA+Ta-31.16.642μ )×Tc ×Tc .

Proposal 14: The ∆NTA will be determined by UE using estimated drift value and additional drift provided by gNB.

- References

[1]   RP-201256 - Solutions for NR to support non-terrestrial networks (NTN) (WID)

[2]  R1-2102215 FL Summary on enhancements on UL time and frequency synchronization for NR	Moderator (Thales)