3GPP TSG RAN WG1 Meeting #104bis-e 	R1-2103746

E-meeting, April 12th – 20th, 2021

Agenda Item:	7.2.12

Source:	Huawei, HiSilicon

Title:	Discussion on the remaining problems of supporting Tx switching between two uplink

Document for:	Discussion and Decision

Introduction

In RAN1#104-e, discussions regarding remaining issues of supporting Tx switching between two UL carriers have achieved some agreements alone with some endorsed CRs as summarized in [1]. In this contribution, our view on the remaining issues for the CA based SRS switching are provided.

Discussion

During last meeting, concern has been raised for the following scenario:

The UE is configured with CA, SUL or EN-DC, and this UE is configured with three carriers (CC1, CC2 and CC3):

- Tx switching is configured between CC1 and CC2,

- SRS carrier switching is configured between CC2 and CC3, where the CC2 is the “switching from” carrier, and the CC3 is the “switching to” carrier,

- CC1 supports 1Tx,

- CC2 supports 2Tx,

- CC3 is for downlink transmission only (no PUSCH is configured in CC3).

Issue #1: whether the CC1 should be suspended during the RF retuning time due to SRS carrier switching on CC3

When the UE receives a trigger for aperiodic carrier switching SRS transmission on CC3, all uplink transmission CC2 will be temporarily suspended during the RF retuning time according to the related specification. Obviously, there is no enough Tx chains for any uplink transmission on CC1 since all the carriers are sharing the two Tx chains. Hence, CC1 should be suspended as well, and we suggest to adopt TP1 in Appendix.

Proposal 1: Adopt TP1 for TS 38.214 clause 6.2.1.3 in the Appendix for uplink suspension.

Issue #2: whether the dropping rules defined in the TS 38.213 subclause 6.2.1.3 needs clarification

Another discussion point in the last meeting is about the dropping rules between SRS transmission and another uplink transmission on other carriers. Obviously, both the carrier indicated by “srs-SwitchFromCarrier” and any other carrier which is sharing the Tx chain with this carrier should be taken into account for the dropping rules (e.g., CC1 and CC2 as in the scenario). Since it was agreed in RAN1#90 that NR SRS switching among CCs similar to Rel-14 LTE SRS carrier-based switching, after reviewing the discussion history of LTE SRS carrier-based switching [2], we feel the clarification will be clearer if we follow the same structure as in the LTE CR [2].

RAN1#90bis: Agreement: Specify NR SRS switching among CCs similar to Rel-14 LTE SRS carrier-based switching design including Periodic/aperiodic/semi-persistent SRS on a CC without PUCCH/PUSCH configured TA (through PRACH) on TAG without PUSCH/PUCCH configured Power control separated from that of PUSCH Group common DCI for aperiodic SRS triggering and TPC DL/UL interruptions and collision handling due to SRS switching

In conclusion, we propose TP2 as in the Appendix to clarify the dropping rules between uplink carriers.

Proposal 2: Adopt TP2 in the Appendix for the prioritization/dropping rules defined in TS 38.214 clause 6.2.1.3.

Conclusions

In this contribution, two remaining issues of uplink Tx switching are discussed. The following proposals are provided:

Proposal 1: Adopt TP1 for TS 38.214 clause 6.2.1.3 in the Appendix for uplink suspension.

Proposal 2: Adopt TP2 in the Appendix for the prioritization/dropping rules defined in TS 38.214 clause 6.2.1.3.

- References

R1-2102149, “[104-e-NR-TxSwitching-01] Summary of email discussion on maintenance of Rel-16 uplink Tx switching”, Moderator (China Telecom), 3GPP RAN1 #104-e Electronic Meeting, January 25th – February 5th.

R1-1721095, “Correction for dropping rules in intra-band SRS carrier switching”, Qualcomm, 3GPP RAN1 #91,  Reno, USA, November 27th – December 1st.

- Appendix

- TP1 for the clarification of suspended carrier due to the RF retuning time of SRS carrier switching in 38.214 clause 6.2.1.3

<Unchanged parts are omitted – 38.214> #### 6.2.1.3 UE sounding procedure between component carriers For a carrier of a serving cell with slot formats comprised of DL and UL symbols, not configured for PUSCH/PUCCH transmission, the UE shall not transmit SRS whenever SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR) on the carrier of the serving cell and PUSCH/PUCCH transmission carrying HARQ-ACK/positive SR/RI/CRI/SSBRI and/or PRACH happen to overlap in the same symbol and that can result in uplink transmissions beyond the UE's indicated uplink carrier aggregation capability included in [13, TS 38.306]. For a carrier of a serving cell with slot formats comprised of DL and UL symbols, not configured for PUSCH/PUCCH transmission, the UE shall not transmit a periodic/semi-persistent SRS whenever periodic/semi-persistent SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR) on the carrier of the serving cell and PUSCH transmission carrying aperiodic CSI happen to overlap in the same symbol and that can result in uplink transmissions beyond the UE's indicated uplink carrier aggregation capability included in [13, TS 38.306]. For a carrier of a serving cell with slot formats comprised of DL and UL symbols, not configured for PUSCH/PUCCH transmission, the UE shall drop PUCCH/PUSCH transmission carrying periodic CSI comprising only CQI/PMI/L1-RSRP/L1-SINR, and/or SRS transmission on another serving cell configured for PUSCH/PUCCH transmission whenever the transmission and SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR) on the serving cell happen to overlap in the same symbol and that can result in uplink transmissions beyond the UE's indicated uplink carrier aggregation capability included in [13, TS 38.306]. For a carrier of a serving cell with slot formats comprised of DL and UL symbols, not configured for PUSCH/PUCCH transmission, the UE shall drop PUSCH transmission carrying aperiodic CSI comprising only CQI/PMI/L1-RSRP/L1-SINR whenever the transmission and aperiodic SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133]) as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR) on the carrier of the serving cell happen to overlap in the same symbol and that can result in uplink transmissions beyond the UE's indicated uplink carrier aggregation capability included in [13, TS 38.306]. For an aperiodic SRS triggered in DCI format 2_3 and if the UE is configured with higher layer parameter srs-TPC-PDCCH-Group set to 'typeA', and given by SRS-CarrierSwitching, without PUSCH/PUCCH transmission, the order of the triggered SRS transmission on the serving cells follow the order of the serving cells in the indicated set of serving cells configured by higher layers, where the UE in each serving cell transmits the configured one or two SRS resource set(s) with higher layer parameter usage set to 'antennaSwitching' and higher layer parameter resourceType in SRS-ResourceSet set to 'aperiodic'. For an aperiodic SRS triggered in DCI format 2_3 and if the UE is configured with higher layer parameter srs-TPC-PDCCH-Group set to 'typeB' without PUSCH/PUCCH transmission, the order of the triggered SRS transmission on the serving cells follow the order of the serving cells with aperiodic SRS triggered in the DCI, and the UE in each serving cell transmits the configured one or two SRS resource set(s) with higher layer parameter usage set to 'antennaSwitching' and higher layer parameter resourceType in SRS-ResourceSet set to 'aperiodic'. A UE can be configured with SRS resource(s) on a carrier c1 with slot formats comprised of DL and UL symbols and not configured for PUSCH/PUCCH transmission. For carrier c1, the UE is configured with higher layer parameter srs-SwitchFromServCellIndex and srs-SwitchFromCarrier the switching from carrier c2 which is configured for PUSCH/PUCCH transmission. During SRS transmission on carrier c1 (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR), the UE temporarily suspends the uplink transmission on carrier c2, and also the uplink transmission on carrier c3 if the UE is configured with uplinkTxSwitching-r16 for uplink switching between uplink carrier c2 and c3. If the UE is not configured for PUSCH/PUCCH transmission on carrier c1 with slot formats comprised of DL and UL symbols, and if the UE is not capable of simultaneous reception and transmission on carrier c1 and serving cell c2, the UE is not expected to be configured or indicated with SRS resource(s) such that SRS transmission on carrier c1 (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR) would collide with the REs corresponding to the SS/PBCH blocks configured for the UE or the slots belonging to a control resource set indicated by MIB or SIB1 on serving cell c2. <Unchanged parts are omitted – 38.214>

- TP2 for the prioritization/dropping rules in 38.214 clause 6.2.1.3

<Unchanged parts are omitted – 38.214> #### 6.2.1.3 UE sounding procedure between component carriers For a carrier of a serving cell d with slot formats comprised of DL and UL symbols, not configured for PUSCH/PUCCH transmission, denote as s0(d) the corresponding carrier of a serving cell whose UL transmissions are temporarily suspended as signalled by higher layer parameter srs-SwitchFromServCellIndex and srs-SwitchFromCarrier. Define the set Sd={s0d, s1(d)} as the set of carriers of serving cells that meet all the following conditions: - {s0d,s1(d)} are in the same TAG as s0(d). - {s0d,s1(d)} are in the different band as s0(d), s0(d) and s1(d) are both configured with uplinkTxSwitching-r16. The following prioritization rules shall be applied in case of collision between a transmission of SRS over carrier and transmission of a physical signal/channel over a carrier of a serving cell in set Sd: - the UE shall not transmit SRS whenever SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR) on the carrier of the serving cell and PUSCH/PUCCH transmission carrying HARQ-ACK/positive SR/RI/CRI/SSBRI and/or PRACH on a carrier of a serving cell in set Sd happens to overlap in the same symbol. - the UE shall not transmit a periodic/semi-persistent SRS whenever periodic/semi-persistent SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR) on the carrier of the serving cell and PUSCH transmission carrying aperiodic CSI on a carrier of a serving cell in set Sd happens to overlap in the same symbol. - the UE shall drop PUCCH/PUSCH transmission carrying periodic CSI comprising only CQI/PMI/L1-RSRP/L1-SINR, and/or SRS transmission on a carrier of a serving cell in set Sd configured for PUSCH/PUCCH transmission whenever the transmission and SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR) on the carrier of the serving cell d happen to overlap in the same symbol. - the UE shall drop PUSCH transmission carrying aperiodic CSI comprising only CQI/PMI/L1-RSRP/L1-SINR on a carrier of a serving cell in set Sd whenever the transmission and aperiodic SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR) on the carrier of the serving cell d happen to overlap in the same symbol. For an aperiodic SRS triggered in DCI format 2_3 and if the UE is configured with higher layer parameter srs-TPC-PDCCH-Group set to 'typeA', and given by SRS-CarrierSwitching, without PUSCH/PUCCH transmission, the order of the triggered SRS transmission on the serving cells follow the order of the serving cells in the indicated set of serving cells configured by higher layers, where the UE in each serving cell transmits the configured one or two SRS resource set(s) with higher layer parameter usage set to 'antennaSwitching' and higher layer parameter resourceType in SRS-ResourceSet set to 'aperiodic'. For an aperiodic SRS triggered in DCI format 2_3 and if the UE is configured with higher layer parameter srs-TPC-PDCCH-Group set to 'typeB' without PUSCH/PUCCH transmission, the order of the triggered SRS transmission on the serving cells follow the order of the serving cells with aperiodic SRS triggered in the DCI, and the UE in each serving cell transmits the configured one or two SRS resource set(s) with higher layer parameter usage set to 'antennaSwitching' and higher layer parameter resourceType in SRS-ResourceSet set to 'aperiodic'. A UE can be configured with SRS resource(s) on a carrier c1 with slot formats comprised of DL and UL symbols and not configured for PUSCH/PUCCH transmission. For carrier c1, the UE is configured with higher layer parameter srs-SwitchFromServCellIndex and srs-SwitchFromCarrier the switching from carrier c2 which is configured for PUSCH/PUCCH transmission. During SRS transmission on carrier c1 (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR), the UE temporarily suspends the uplink transmission on carrier c2. If the UE is not configured for PUSCH/PUCCH transmission on carrier c1 with slot formats comprised of DL and UL symbols, and if the UE is not capable of simultaneous reception and transmission on carrier c1 and serving cell c2, the UE is not expected to be configured or indicated with SRS resource(s) such that SRS transmission on carrier c1 (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR) would collide with the REs corresponding to the SS/PBCH blocks configured for the UE or the slots belonging to a control resource set indicated by MIB or SIB1 on serving cell c2. For n-th (n ≥ 1) aperiodic SRS transmission on a cell c, upon detection of a positive SRS request on a grant, the UE shall commence this SRS transmission on the configured symbol and slot provided - it is no earlier than the summation of - the maximum time duration between the two durations spanned by N OFDM symbols of the numerology of cell c and the cell carrying the grant respectively, and - the UL or DL RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR, - it does not collide with any previous SRS transmissions, or interruption due to UL or DL RF retuning time. otherwise, n-th SRS transmission is dropped, where N is the reported capability as the minimum time interval in unit of symbols, between the DCI triggering and aperiodic SRS transmission. In case of inter-band carrier aggregation, a UE can simultaneously transmit SRS and PUCCH/PUSCH across component carriers in different bands subject to the UE's capability. ### In case of inter-band carrier aggregation, a UE can simultaneously transmit PRACH and SRS across component carriers in different bands subject to UE's capability. <Unchanged parts are omitted>