3GPP TSG-RAN WG1 Meeting #104bis-e	R1-2103751

# E-meeting, April 12 – April 20, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [DRAFT] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | XXX | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on step 5 of mode 2 resource allcoation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Core |  |  |  |  |  | Date: |  |  | 2021-04-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | During the resource determination procedure for mode 2 in TS 38.214, the sensing UE excludes the resources from SA by step 5) and step 6). Since the RSRP measurement is only associated with step 6), the number of the remaining resources in SA after performing step 5) may be already smaller than X⋅Mtotal, thus the operation of increasing RSRP threshold cannot ensure the final resources set satisfies the requirement. Especially when the periodicity value is very small, many slots have be excluded in step 5), resulting in the number of remaining resources being still smaller than X⋅Mtotal after step 5). |  |  |  |  |  |  |  |  |
| Summary of change: |  | If the number of the excluded resources in step 5) is larger than (1-X)· Mtotal , the UE randomly selects some excluded resources in step 5) and adds them back in the set SA until the number of the candidate single-slot resources remaining in the set SA is not smaller than X⋅Mtotal. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The remaining number of candidate single-slot resources in SA is always smaller than X⋅Mtotal in some cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

--------------------------- Start of Text Proposal for TS 38.214 ------------------------

<Unchanged parts omitted>

8.1.4    UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2

In resource allocation mode 2, the higher layer can request the UE to determine a subset of resources from which the higher layer will select resources for PSSCH/PSCCH transmission. To trigger this procedure, in slot n, the higher layer provides the following parameters for this PSSCH/PSCCH transmission:

<Unchanged parts omitted>

The following steps are used:

<Unchanged parts omitted>

4)	The set SA is initialized to the set of all the candidate single-slot resources.

5)	The UE shall exclude any candidate single-slot resource Rx,y from the set SA if it meets all the following conditions:

-	the UE has not monitored slot t'mSL in Step 2.

-	for any periodicity value allowed by the higher layer parameter sl-ResourceReservePeriodList-r16 and a hypothetical SCI format 1-A received in slot tmSL with "Resource reservation period" field set to that periodicity value and indicating all subchannels of the resource pool in this slot, condition c in step 6 would be met.

5-1)	If the number of candidate single-slot resources excluded from the set SA is larger than (1- X)⋅ Mtotal , randomly selected resources from those excluded in step 5) are added to set SA until the number of the candidate single-slot resources remaining in the set SA is not smaller than  X⋅Mtotal

6)	The UE shall exclude any candidate single-slot resource Rx,y from the set SA if it meets all the following conditions:

<Unchanged parts omitted>

------------------------------------End of Text Proposal ------------------------------