3GPP TSG RAN WG1#104b-e					R1-2103764

e-Meeting, April 12th – 20th, 2021

Source:	Nokia, Nokia Shanghai Bell

Title:	Maintenance for Resource allocation for sidelink - Mode 1

Agenda item:	7.2.4

Document for:	Discussion and Decision

# Introduction

In this contribution we discuss remaining editorial issues relevant for mode 1.

# Discussion

## PUCCH resource for sidelink configured grant type 2

At RAN1#104-e we discussed PUCCH slot/resource determination and reached the following agreement:

Agreements:  Send an LS to RAN2 describing that:  Per RAN1 agreements, the parameter sl-N1PUCCH-AN-r16 should be used for SL CG Type 2 (only for PSCCH/PSSCH transmissions without a corresponding PDCCH).  In RAN1’s understanding, the parameter cannot be configured for SL CG type-2 and asking RAN2 whether they can provide a solution that would be compatible with the existing RAN1 agreements.

The LS was sent in R1-2102176. RAN2 had an email discussion on this topic, which is summarized in R2-2104109. Based on company input, the rapporteur of the email discussion made the following recommendation:

Recommendation 1: RAN2 agree to clarify in the field description of sl-N1PUCCH-AN that this field indicates the HARQ resource for PUCCH for sidelink configured grant type 1 and PSCCH/PSSCH transmissions without a corresponding PDCCH on sidelink configured grant type 2.

Proposal 1: Discuss text proposal regarding PUCCH resource for sidelink configured grant type 2 once the reply LS from RAN2 is received.

## UE-specific Search space configuration

In TS 38.331, dci-FormatsSL configures whether a UE monitors for the SL-related DCI formats in a UE-specific search space:

SearchSpace ::=                         SEQUENCE {

-- irrelevant stuff omitted

searchSpaceType                         CHOICE {

ue-Specific                                 SEQUENCE {

dci-Formats                                 ENUMERATED {formats0-0-And-1-0, formats0-1-And-1-1},

...,

[[

dci-Formats-MT-r16                   ENUMERATED {formats2-5}                                OPTIONAL,    -- Need R

dci-FormatsSL-r16                    ENUMERATED {formats0-0-And-1-0, formats0-1-And-1-1, formats3-0, formats3-1,

formats3-0-And-3-1}                        OPTIONAL,    -- Need R

dci-FormatsExt-r16                   ENUMERATED {formats0-2-And-1-2, formats0-1-And-1-1And-0-2-And-1-2}

OPTIONAL     -- Need R

]]

}

}                                                                                                   OPTIONAL    -- Cond Setup2

}

SearchSpace field descriptions

dci-Formats Indicates whether the UE monitors in this USS for DCI formats 0-0 and 1-0 or for formats 0-1 and 1-1.

dci-FormatsExt If this field is present, the field dci-Formats is ignored and dci-FormatsExt is used instead to indicate whether the UE monitors in this USS for DCI format 0_2 and 1_2 or formats 0_1 and 1_1 and 0_2 and 1_2 (see TS 38.212 [17], clause 7.3.1 and TS 38.213 [13], clause 10.1).

dci-Formats-MT Indicates whether the IAB-MT monitors the DCI formats 2-5 according to TS 38.213 [13], clause 14.

dci-FormatsSL Indicates whether the UE monitors in this USS for DCI formats 0-0 and 1-0 or for formats 0-1 and 1-1 or for format 3-0 or for format 3-1 or for formats 3-0 and 3-1.

In TS 38.213 on the other hand, there is no mention of dci-FormatsSL; dci-FormatsExt configures whether the UE monitors for the SL-related DCI formats:

- if search space set s is a USS set, an indication by dci-Formats to monitor PDCCH candidates either for DCI format 0_0 and DCI format 1_0, or for DCI format 0_1 and DCI format 1_1, or an indication by dci-FormatsExt to monitor PDCCH candidates for DCI format 0_2 and DCI format 1_2, or for DCI format 0_1, DCI format 1_1, DCI format 0_2, and DCI format 1_2, or for DCI format 3_0, or for DCI format 3_1, or for DCI format 3_0 and DCI format 3_1

However, as the definitions in TS 38.331 show, dci-FormatsExt cannot be used to configure monitoring of DCI format 3_0 and 3_1.

Proposal 2: Discuss configuration of the UE-specific search space and resolve the inconsistency between TS 38.331 and TS 38.213.

Proposal 3: Adopt the following text proposal

-------------------------- Start of Text Proposal for TS 38.213 --------------------------

10.1	UE procedure for determining physical downlink control channel assignment

------------------- < Unchanged parts are omitted > --------------------------

-	if search space set s is a USS set, an indication by dci-Formats to monitor PDCCH candidates either for DCI format 0_0 and DCI format 1_0, or for DCI format 0_1 and DCI format 1_1, or an indication by dci-FormatsExt to monitor PDCCH candidates for DCI format 0_2 and DCI format 1_2, or for DCI format 0_1, DCI format 1_1, DCI format 0_2, and DCI format 1_2, or an indication by dci-FormatsSL to monitor PDCCH candidates for DCI format 0_0 and DCI format 1_0, or for DCI format 0_1 and DCI format 1_1, or for DCI format 3_0, or for DCI format 3_1, or for DCI format 3_0 and DCI format 3_1

------------------- < Unchanged parts are omitted > --------------------------

-------------------------- End of Text Proposal for TS 38.213 --------------------------

# Conclusions

In this contribution we discussed remaining editorial issues in sidelink resource allocation mode 1 and made the following proposals:

Proposal 1: Discuss text proposal regarding PUCCH resource for sidelink configured grant type 2 once the reply LS from RAN2 is received.

Proposal 2: Discuss configuration of the UE-specific search space and resolve the inconsistency between TS 38.331 and TS 38.213.

Proposal 3: Adopt the following text proposal

-------------------------- Start of Text Proposal for TS 38.213 --------------------------

10.1	UE procedure for determining physical downlink control channel assignment

------------------- < Unchanged parts are omitted > --------------------------

-	if search space set s is a USS set, an indication by dci-Formats to monitor PDCCH candidates either for DCI format 0_0 and DCI format 1_0, or for DCI format 0_1 and DCI format 1_1, or an indication by dci-FormatsExt to monitor PDCCH candidates for DCI format 0_2 and DCI format 1_2, or for DCI format 0_1, DCI format 1_1, DCI format 0_2, and DCI format 1_2, or an indication by dci-FormatsSL to monitor PDCCH candidates for DCI format 0_0 and DCI format 1_0, or for DCI format 0_1 and DCI format 1_1, or for DCI format 3_0, or for DCI format 3_1, or for DCI format 3_0 and DCI format 3_1

------------------- < Unchanged parts are omitted > --------------------------

-------------------------- End of Text Proposal for TS 38.213 --------------------------

# References

- RP-200129, “Revised WID on 5G V2X with NR sidelink”, 3GPP RAN #87e