3GPP TSG RAN WG1 #104b-e		                                    R1-2103787

e-Meeting, April 12th – 20th, 2021

Agenda Item:	8.3.1.2

Source:	Moderator (InterDigital, Inc.)

Title:	Feature lead summary #2 on CSI feedback enhancements for enhanced URLLC/IIoT

Document for:	Discussion and Decision

# Introduction

This contribution is a summary of contributions [2]-[22] submitted under AI 8.3.1.2 (CSI feedback enhancements) The AI is related to the following objective of the revised work item on Enhanced IIoT and URLLC support for NR [1]:

- Study, identify and specify if needed, required Physical Layer feedback enhancements for meeting URLLC requirements covering - UE feedback enhancements for HARQ-ACK [RAN1] - CSI feedback enhancements to allow for more accurate MCS selection [RAN1] Note: DMRS-based CSI feedback is not in scope of this WI

In RAN1#102-bis-e, RAN1 agreed to study/evaluate a set of CSI enhancement schemes in terms of technical benefits, specification and implementation impacts. The candidate enhancement schemes include at least new triggering methods for A-CSI and/or SRS, new reporting based on channel/interference measurement (Case 1), and new reporting based on other measurement (Case 2). RAN1 also agreed on a set of baseline assumptions for system-level simulations.

In RAN1#103-bis-e, RAN1 agreed to continue evaluation for a set of identified candidate schemes for Case 1 to address the fast interference change over time. RAN1 also agreed to continue studying and focus on Case 2 new reporting based on PDSCH decoding for OLLA performance enhancement for initial and re-transmissions of PDSCH.

In RAN1#104-e, a detailed set of Case 1 and Case 2 schemes was identified for continued evaluation ([23], Appendix B) and additional discussions took place after RAN1#104-e to better understand each scheme and associated aspects such as implementation complexity, specification impact and testability [24].

Here is the color code used in this summary:

- FL’s proposals

- Questions for the inputs from companies

- FL summary based on the companies’ input

- RAN1 agreements

# Collection of agreements/conclusion in RAN1 #104b-e

To be captured once agreement is made during this meeting

# Proposals for 1st GTW

New reporting Case 1:

FL proposal 8.1-1: Support new reporting of CQI only, where CQI is conditioned on the latest reporting instance containing RI/PMI (Case 1-11).

New reporting Case 2:

FL proposal 9.1-1: For new reporting Case 2, continue study focusing on reporting of (delta) CQI/MCS/SINR (Case 2-3).

# Proposals for 2nd GTW

For Case 1, FL proposal 8.1-2 was proposed in an attempt to address concerns with 8.1-1.

FL proposal 8.1-2:

Support new reporting of CQI only, where CQI is conditioned on a previous reporting instance containing RI/PMI (Case 1-11).

- One IMR occasion is associated with a CQI-only reporting instance.

  - FFS association of more than one IMR occasion to a CQI-only reporting instance. (Case 1-5, Case 1-1)

- Rel-15/16 CQI reporting formats are at least supported for CQI-only reporting instance. FFS if one or more of the following new formats are also supported:

  - Mean and stdev CQI/SINR (Case 1-1)

  - Worst-M CQI (Case 1-6)

  - 3-bits differential or 4-bits subband CQI (Case 1-8)

The proposal did not achieve consensus. A proposal for down-selection was provided and the following was concluded:

Conclusion:

For new reporting Case 1, do not consider further the following schemes:

- Case 1-2: CSI prediction

- Case 1-4: Interference covariance matrix

- Case 1-9: Reference wideband CQI excludes worst sub-bands

- Case 1-10: CSI expiration time

For Case 2, the following proposal was discussed:

FL proposal 9.1-1: For new reporting Case 2, continue study focusing on reporting of (delta) CQI/MCS/SINR (Case 2-3).

# Proposals for 3rd GTW

TBD

# Proposals for 4th GTW

TBD

# Topic #1: New triggering methods for A-CSI and/or SRS

In this section, we provide summary of contributions discussing candidate enhancement schemes for new triggering methods.

## Summary of issues for Topic #1

Several contributions discuss potential benefits and drawbacks of supporting triggering of a A-CSI report by DCI:

Issue #1-1: Support A-CSI triggering on PUCCH by DL assignment

- Yes: Huawei [2], ZTE [5], vivo [6], Ericsson [10], CMCC [13], NTT DOCOMO [21]

  - The development of new feedback reporting schemes (especially case 2 schemes) is impacted by whether aperiodic CSI can be reported on PUCCH or not [2].

  - Better performance than A-CSI on PUSCH and P/SP-CSI on PUCCH due to more flexible feedback [5], because wideband P-CSI may not be accurate enough [13]

  - Trigger reporting based on traffic needs for sporadic traffic [13][21]

  - No latency increase for CSI reporting (e.g. due to waiting for UL grant for triggering) [2]

  - Less uplink overhead than A-CSI on PUSCH in DL-heavy scenarios, or SP-CSI/P-CSI with low periodicity [21]

  - More flexible triggering mechanism of A-CSI [10]

  - Lower PUCCH resource utilization than P/SP-CSI on PUCCH [10]

  - Transmission of single PDCCH transmission instead of two PDCCH with A-CSI on PUSCH [2][5][6][13]:

    - Less interference and resource utilization than A-CSI on PUSCH

    - Avoid blocking/increased latency from exceeding blind decoding limit per span or lack of coreset capacity

    - Better spectral efficiency

    - Avoid reduction of reliability due to CCE channel estimation limit

    - Avoid reduction of reliability from having to successfully receive two PDCCHs

- Some concerns: Apple [15], Lenovo [22]

  - Need to decide whether PUCCH resource is same or different than HARQ-ACK [22]

  - Total number of activated trigger states needs to be limited [15]

- No: Mediatek [8], Intel [14], LG [19]

  - P/SP-CSI reporting more suitable for factory scenario with periodic traffic [8]

  - For factory scenario, coherence time is larger than latency requirement, therefore no need to update the CSI report for re-transmission [8]

  - P/SP-CSI reporting every 10 ms sufficient for AR/VR scenario with 22 ms coherence time [8][19]

  - No clear enhancement compared to A-CSI on PUSCH [8][14]

  - Does not address the problem of bursty interference which is the main performance issue [14]

  - If CSI and HARQ-ACK are combined in same resource, need to delay HARQ-ACK compared to processing capability 2 and increased probability of error with larger payload [8]

  - Non-negligible specification efforts [14], e.g. complicated timeline [19]

  - Added overhead in DL assignments if new fields are required [14], wasting resource since no retransmission is needed ~99% of the time [8][14]

  - Resources for CSI in the UL may be limited by other URLLC transmissions [19]

Several contributions discussed potential benefits and drawbacks of supporting triggering of a CSI-RS/SRS and/or A-CSI report by NACK:

Issue #1-2: Support CSI-RS/SRS/A-CSI report triggering by NACK

- Yes: Huawei [2], ZTE [5], Qualcomm [16]

  - No extra demands on PDCCH blind decoding

  - Avoid reduction of reliability due to CCE channel estimation limit

  - Good performance in terms of percentage of satisfied UEs [5]

  - Avoids excessive overhead of low CSI-RS periodicity/CSI report [16]

  - Can be used with semi-persistently scheduled PDSCH [16]

- No: Mediatek [8]

  - May increase power consumption by requiring unnecessary A-CSI computation 99% of the time [8]

Issue #1-3: Support A-CSI triggering on PUCCH by group DCI

Two contributions [5][8] discuss potential support of triggering a A-CSI report by group DCI. However, neither contributions support this option. The main reason is the inefficient use of group DCI resources since packet arrivals are not synchronous between UEs.

One company proposed to trigger CSI-RS or SRS when PDSCH is successfully received but with a low margin:

Issue #1-4: Support CSI-RS/SRS triggering by low-margin ACK

- Yes: Qualcomm [16]

  - To provide new report quickly when conditions start degrading [16]

  - One extra bit in HARQ-ACK codebook can be used to indicate preconfigured whether RS is triggered [16]

One company proposed to support UE requesting CSI measurement to update CSI

Issue #1-5: Support UE request for CSI measurement to update CSI for a new Tx-Rx beam pair

- Yes: Qualcomm [16]

  - To provide new CSI measurements when UE autonomously updates its Rx beam [16]

Observations on new triggering methods.

For A-CSI on PUCCH triggered by DL DCI:

- 6 companies support this, 3 companies do not support it and 2 do not provide a definitive view.

- Following evaluation results are available:

  - ZTE [5] provided additional evaluation results and observes the following gains:

    - 67% satisfied UEs vs 53% (if using A-CSI on PUSCH), or 50% (if using SP-CSI)

    - 2.9% resource utilization vs 3.1% (if using A-CSI on PUSCH) or 1.9% (if using SP-CSI)

For A-CSI on PUCCH triggered by NACK

- 3 companies support this, 1 company does not support it.

Compared to RAN1#104-e, there does not seem to be any change of view or any additional data. Several companies do not discuss the topic any more in their contribution submitted to RAN1#104-be. For this reason, it is suggested to focus on Topic #2 and Topic #3 in RAN1#104-be.

## E-mail discussion (1st round) for Topic #1

TBD

# Topic #2: New reporting (Case 1)

In this section, we provide summary of contributions discussing candidate enhancement schemes for new reporting based on channel/interference measurement (Case 1).

## Summary of issues for Topic #2

At RAN1#104-e, 11 schemes were identified for Case 1 new reporting. Subsequent detailed discussions after RAN1#104-e allowed further understanding of the potential benefits of each proposed scheme and issues. The main design objectives can be identified as follows:

- Enabling more accurate MCS selection without very frequent and detailed CSI reporting by the UE:

  - Presumed scheduler behaviour is to pick a MCS based on the tail of a distribution of CQI samples recently measured by UE (but not necessarily latest CQI), assuming that interference at scheduling time may not be correlated with interference at the latest CQI measurement.

  - UE reports statistics from multiple measurements: Case 1-1, Case 1-3

  - UE reports from worst-case measurement: Case 1-5, Case 1-6, Case 1-7, Case 1-9

  - UE frequently reports only CQI conditioned previously reported full CSI: Case 1-11

- Enabling more accurate MCS selection by decreasing CQI quantization error and/or reducing CQI processing latency:

  - Presumed scheduler behaviour is to pick a MCS based on most recent CQI report, assuming that interference at scheduling time may be correlated with interference in most recent CQI report.

  - Decreasing CQI quantization error: Case 1-8, Case 1-9

  - Reducing CQI processing latency: Case 1-11

- Enabling more accurate MCS selection in other scenarios:

  - Predicting future CSI: Case 1-2

  - MU-MIMO, TDD: Case 1-4

  - Change of UE speed: Case 1-10

In the following sub-sections, performance results and detailed views on benefits and complexity are presented for each scheme.

### Case 1-1: Statistical CSI/SINR

Evaluation results

| ZTE [5] | Case 1-1 Mean + stdev CQI | AR/VR | 31% satisfied UEs [50%] 2.9% RU [1.9%] (gNB sets MCS based on MeanCQI – StdevCQI) |
| --- | --- | --- | --- |
| InterDigital [9] | Case 1-1 Mean+stdev SINR | AR/VR | 100% satisfied UEs [100%] 6.43 RU [6.47 RU] Report periodicity 2 ms |
| Ericsson [10] | Case 1-1 Mean and stdev CQI (wideband) | AR/VR (mixed traffic) | 97.5% satisfied UEs [78.5%] 76% median RU [77%] Baseline uses fixed backoff of 20 dB |
| Ericsson [10] | Case 1-1 Mean and stdev CQI (subband) | AR/VR (mixed traffic) | 97.2% satisfied UEs [78.5%] 60% median RU [77%] Baseline uses fixed backoff of 20 dB |
| Futurewei [11] | Case 1-1 Mean and stdev SINR | AR/VR | 81% satisfied UEs [68%] 6.5% RU [9.5%] |
| Intel [14] | Case 1-1 Mean+stdev SINR | Factory | ~5% more satisfied UEs than baseline (1e-5 target) 6.3% RU [23.6%] |
| Nokia [20] | Case 1-1 Mean + stdev SINR | AR/VR | 1 ms 99.9999%-pct latency [2 ms] 5% RU [3%] |
| Nokia [20] | Case 1-1 Mean + stdev SINR | Factory | ~1 ms 99.999%-pct latency [1 ms] |

Observations

- Ericsson [10], Futurewei [11], Intel [14], Nokia [20] observe increase of % of satisfied UEs and/or decrease of RU compared to baseline. InterDigital [9] observes similar performance between the scheme and the baseline. ZTE [5] observes degradation in the % of satisfied UEs and increase of RU.

- The UL overhead of the scheme is 15 smaller than the baseline in [10] due to lower reporting frequency. In [5], [9], [20], the report periodicity is the same but the payload of each report is smaller (mean + stdev of CQI or SINR instead of subband CQI report). In [11], UL overhead of the scheme is slightly higher than the baseline (mean + stdev of SINR reported for in addition to periodic report for every 20 reports).

Benefits/concerns

+Reduction of UL overhead: Huawei [2], Vivo [6], InterDigital [9], Ericsson [10], Nokia [20]

-~7 bits for mean, variance, scaling factor: Ericsson [10]

+Avoid frequent reporting of CQI: InterDigital [9], NTT DoCoMo [21]

+Variance report allows estimation of very low probability value (tail): Ericsson [10], Nokia [20]

+Simpler for gNB if UE reports single RI/PMI for all reports: Ericsson [10]

+Overcomes limited dynamic range/quantization of CQI table: InterDigital [9], Intel [14]

-May use pre-scheduling as R16 solution: Huawei [2]

-gNB can calculate statistics instead if there are sufficient CQI reports: Vivo [6]

-Performance may be sensitive to actual PDF of interference: Huawei [2], InterDigital [9], Apple [15]

-Need to ensure sufficient samples of CQI/SINR: Vivo [6]

-No clear benefit compared to baseline (frequent sub-band reporting): Vivo [6], LG [19]

-Different schedulers may need different statistics: Huawei [2], Vivo [6], CATT [7]

-Variance CQI may be enlarged due to potential frequency selection: Vivo [6]

-Backoff can be determined by other means, instantaneous CQI, RSRP/OLLA: Samsung [17]

-Many issues to solve: number of samples, window size, outliers, quantization, reliability: Lenovo [22]

[If using SINR instead of CQI]

+SINR is not subject to limitation of just two possible BLER targets, allows more accurate performance estimation for various RB allocations: Nokia [20]

-Different UEs have different performance for same SINR. Mapping to CQI is proprietary: Quectel [12], Apple [15], Samsung [17], LG [19]

Implementation impact

+UE computation complexity is reduced compared to frequent CSI: Ericsson [10]

-Potential increased UE complexity: Huawei [2]

-UE computational complexity not improved: Samsung [17]

Specification/testing impact

-Medium: Huawei [2]

-High: Spreadtrum [4], ZTE [5]

-High effort to define new feedback quantity/quantization/processing latency: ZTE [5]

-Difficult to test performance of mean/variance CQI reporting, should clarify how it is used: Vivo [6], Apple [15]

### Case 1-2: CSI prediction

Evaluation results

None available

Benefits/concerns

+Reduction of UL overhead: Huawei [2]

+UE can directly measure interference: InterDigital [9], Qualcomm [16]

-Assumes a-priori knowledge of interference PDF: Huawei [2]

-Potential increased UE complexity: Huawei [2]

-Unclear how UE can predict future CSI with bursty interference: Spreadtrum [4], CATT [7], InterDigital [9], Quectel [12], Samsung [17], NTT DoCoMo [21]

-Channel prediction possible at gNB based on SRS: Samsung [17]

-Not aligned with goal for accurate CSI: LG [19]

-UE does not know when resource is scheduled by gNB: InterDigital [9], NTT DoCoMo [21]

-If predicted interference is during burst, results won’t be valid later: InterDigital [9]

-Unclear how to specify or test: InterDigital [9]

Implementation impact

-Hard to judge: Huawei [2]

Specification/testing impact

-Hard to judge: Huawei [2]

### Case 1-3: Interference statistics

Evaluation results

| Futurewei [11] | Case 1-3 Interference stdev | AR/VR | 92% satisfied UEs [68%] 7.2% RU [9.5%] |
| --- | --- | --- | --- |

Observations

- Futurewei [11] observes increase of % of satisfied UEs and decrease of RU compared to baseline. The improvement is larger than what [11] observes for Case 1-1.

- The UL overhead of the scheme is slightly higher than the baseline (mean + stdev of interference reported for in addition to periodic report for every 20 reports).

Benefits/concerns

+Reduction of UL overhead: Huawei [2], Nokia [20]

+Isolate variations of interference only, better if time window for statistics is larger than interval between reports: Futurewei [11]

+Overcomes limited dynamic range of CQI table: Intel [14]

-May use pre-scheduling as R16 solution: Huawei [2]

-Can use RSRP/RSSI more reliably than short term interference reports: Samsung [17]

-Performance may be sensitive to actual PDF of interference: Huawei [2]

-Different schedulers may need different statistics: Huawei [2], CATT [7]

-No clear benefit compared to sub-band reporting: LG [19]

-No clear benefit over 1-1: Lenovo [22]

-May be less efficient than 1-1 (need average and variance on interference on top of channel part): InterDigital [9]

-Sensitive to different SINR-BLER performance: Quectel [12]

-Does not capture variations of the desired signal: Nokia [20]

Implementation impact

-Low: Futurewei [11]

-Medium: Huawei [2]

-High: Spreadtrum [4]

Specification/testing impact

-Low specification effort, testable: Futurewei [11]

-Hard to judge: Huawei [2]

-High effort to define new feedback quantity/quantization/processing latency: ZTE [5], InterDigital [9]

-Difficult to test: CATT [7]

### Case 1-4: Interference covariance matrix

Evaluation results

| Huawei [2] | Case 1-4 | Factory (non-baseline) | 160 supported UEs [100] 38% RU [100%] |
| --- | --- | --- | --- |

Observations

- Huawei [2] observes increase of number of UEs that can be supported and lower RU% compared to baseline. However, the scenario is not according to agreed baseline.

Benefits/concerns

+Allows fast CSI reports: Huawei [2]

+Enables gNB to select MCS matching SINR with MMSE-IRC: Huawei [2]

-Large payload: Spreadtrum [4], Quectel [12], Nokia [20], InterDigital [9]

-Unclear benefit for scheduler: Spreadtrum [4]

-Not suitable for URLLC traffic (requires delay-tolerant and non-bursty traffic): Samsung [17], InterDigital [9]

-Restrictive use case (TDD, single dominant interferer): InterDigital [9], LG [19], Nokia [20]

-Mainly targets MU-MIMO [21], not typical for URLLC: InterDigital [9], NTT DoCoMo [21]

-No benefit in agreed scenario: InterDigital [9]

-Not useful if gNB can control interferers: LG [19]

Implementation impact

-Medium: Huawei [2]

-High at UE, gNB: InterDigital [9], NTT DoCoMo [21]

Specification/testing impact

-Medium: Huawei [2]

-More difficult than 1-3: Spreadtrum [4]

-High (e.g. define new feedback quantity/quantization/processing req): ZTE [5], InterDigital [9], NTT DoCoMo [21]

-Difficult to test: CATT [7]

### Case 1-5: CSI based on worst IMR occasion

Evaluation results

| ZTE [5] | Case 1-5 | AR/VR | 58% satisfied UEs [50%] 2.3% RU [1.9%] |
| --- | --- | --- | --- |
| InterDigital [9] | Case 1-5 | AR/VR | 99.5% satisfied UEs [99.5%] 6.65 RU [6.59 RU] Report periodicity 20 ms |
| Intel [14] | Case 1-5 | Factory | ~20% more satisfied UEs than baseline (1e-5 target) 35.9% RU [23.6%] |

Observations

- ZTE [5] and Intel [14] observe gain in % of satisfied UEs at the expense of increasing RU. InterDigital [9] observe similar performance as baseline.

Benefits/concerns

+Reduction of UL overhead: Huawei [2], Nokia [20]

+Enables less frequent reporting: InterDigital [9]

+Network avoids too conservative MCS, reflects interference jitter: Spreadtrum [4], ZTE [5]

+Does not require additional report quantity: ZTE [5]

-Assumes a priori knowledge of interference PDF: Huawei [2]

-Worst IMR not correlated to interference at scheduling: Samsung [17]

-Backoff can be obtained from CQI statistics, OLLA, RSRP/RSSI: Samsung [17]

-No clear benefit compared to sub-band reporting: LG [19]

-Subject to limitations of CQI-based reporting: Nokia [20]

-Unclear whether time window is feasible or not, instantaneous reports not meaningful for URLLC: NTT DoCoMo [21]

-gNB capable by implementation: NTT DoCoMo [21]

-May lead to too conservative MCS selection: Lenovo [22]

Implementation impact

-Low: InterDigital [9]

-Medium: Huawei [2]

Specification/testing impact

-Low: Huawei [2], Spreadtrum [4], ZTE [5], InterDigital [9]

-More than 1-6: need to define filtering window, storing multiple IMR measurements: Nokia [20]

### Case 1-6: Worst-M CQI

Evaluation results

| Mediatek [8] | Case 1-6 Worst-2 CQI | Factory | 35.6% RU (25.1%) |
| --- | --- | --- | --- |
| InterDigital [9] | Case 1-6 Worst-1 CQI | AR/VR | 100% satisfied UEs [100%] 6.78 RU [6.47 RU] Report periodicity 2 ms |
| Nokia [20] | Case 1-6 Worst-2 CQI | AR/VR | 1 ms 99.9999%-pct latency [2 ms] 5% RU [3%] |
| Nokia [20] | Case 1-6 Worst-2 CQI | Factory | ~1 ms 99.999%-pct latency [1 ms] |

Observations

- Mediatek [8] and Interdigital [9] observe increase of %RU for same number of satisfied UEs. Nokia [20] observe improvement in 1e-5 percentile latency from 2 to 1 ms in AR/VR environment at the expense of increase of RU%.

- The UL overhead of the scheme is smaller than the baseline (subband D-CQI with small periodicity)

Benefits/concerns

+Reduction of UL overhead: Huawei [2], InterDigital [9], LG [19], Nokia [20]

+Allows scheduler to know worst-case conditions: Spreadtrum [4], InterDigital [9]

-Assume a-priori knowledge of interference PDF: Huawei [2]

-Observed gain may be due to random allocation: Vivo [6]

-Interference may change from CSI feedback to PDSCH scheduling: Apple [14]

-No additional information compared to subband CQI: Samsung [17], LG [19]

-Subject to limitations of CQI-based reporting: Nokia [20]

Implementation impact

-Low, simple extension from R16: InterDigital [9], Quectel [12], Nokia [20], NTT DoCoMo [21], Lenovo [22]

-Low/medium: Huawei [2]

Specification/testing impact

-Low: Spreadtrum [4], InterDigital [9], Nokia [20], NTT DoCoMo [21]

-No change to CMR/IMR, CRI, RI, PMI. Could work with CQI-only reporting. M is RRC configured: Nokia [20]

-Easy to test: Spreadtrum [4]

-e.g. index difference with wideband CQI to be reported between a% and b% of the time: Nokia [20]

-Medium: Huawei [2]

### Case 1-7: Worst-best criteria for subband CQI report

Evaluation results

None available

Benefits/concerns

Note: Configurable variant can be worst-worst-M: Qualcomm [25]

+/-Same as Case 1-6, plus:

-Unclear how the scheme works, performance: Nokia [20]

-Unclear why subband-specific CRI needs to be reported unless for multi-TRP: InterDigital [9]

-Unclear benefit over 1-6, requires more payload: NTT DoCoMo [21]

Implementation impact

-Low/medium: Huawei [2]

Specification/testing impact

-Low: Spreadtrum [4]

-Medium: Huawei [2]

### Case 1-8: 3-bits differential subband CQI or 4-bit full subband CQI

Evaluation results

| Mediatek [8] | Case 1-8 3-bit Diff-CQI | Factory | 0.4% of incorrect MCS [22%] Baseline uses 2-bit D-CQI Incorrect MCS defined as scheduled MCS using scheme minus scheduled MCS using 4-bits SB-CQI |
| --- | --- | --- | --- |
| Mediatek [8] | Case 1-8 3-bit Diff-CQI | Factory | 21.2% RU (25.1%) |
| Mediatek [8] | Case 1-8 4-bits full CQI | Factory | 21.2% RU (25.1%) |
| InterDigital [9] | Case 1-8 3-bit Diff-CQI | AR/VR | 100% satisfied UEs [100%] 6.48 RU [6.47 RU] |
| InterDigital [9] | Case 1-8 4-bits full CQI | AR/VR | 100% satisfied UEs [100%] 6.48 RU [6.47 RU] |
| Intel [14] | Case 1-8 4-bis full CQI | Factory | ~5% less satisfied UEs than baseline (1e-5 target) 23.9% RU [23.6%] |
| Samsung [17] | Case 1-8 3-bit Diff-CQI | ??? | 0.2%, 1.9%, 1.0% gain for average/median/5 pctile throughput respectively. |
| Samsung [17] | Case 1-8 4-bits full CQI | ??? | 0.5%, 0.7%, 15.6% gain for average/median/5 pctile throughput respectively |
| Nokia [20] | Case 1-8 4-bits full CQI | AR/VR | 1 ms 99.9999%-pct latency [2 ms] 6% RU [3%] |

Observations

- Mediatek [8] observes some decrease of RU% for same number of satisfied UEs. InterDigital [9] observes slight increase of RU% for same number of satisfied UEs. Intel [14] observes decrease of number of satisfied UEs. Nokia [20] observe improvement in 1e-5 percentile latency from 2 to 1 ms in AR/VR environment at the expense of doubling of RU%.

- Samsung observes a 15% gain in 5th percentile throughput for 4-bits full CQI. However, scenario/assumptions are unknown and metrics does not seem to correspond to agreed baseline for URLLC.

- The UL overhead of the scheme is higher than the baseline (subband 2-bits D-CQI with small periodicity)

Benefits/concerns

+Improved accuracy: Huawei [2], Nokia [20]

+Avoid lost information in the negative CQI offset values: Mediatek [8]

-Interference may change from CSI feedback to PDSCH scheduling: Apple [14], InterDigital [9]

-Larger CQI payload and CQI BLER: Samsung [17], Nokia [20]

-Frequency-selective CQI has limited benefit because of fast and random interference variations: Nokia [20]

-Subject to limitations of CQI-based reporting: Nokia [20]

Implementation impact

-Mature and low complexity: Huawei [2], Spreadtrum [4], Vivo [6], InterDigital [9], NTT DoCoMo [21]

Specification/testing impact

-Low: Huawei [2], Spreadtrum [4], Vivo [6], InterDigital [9]

### Case 1-9: Reference wideband CQI excludes worst subbands

Evaluation results

| Mediatek [8] | Case 1-9 (excludes 5 SB) | Factory | Reported enhanced wideband CQI better than baseline wideband CQI 62% of time |
| --- | --- | --- | --- |
| InterDigital [9] | Case 1-9 (excludes 5 SB) | AR/VR | 100% satisfied UEs [100%] 6.48 RU [6.47 RU] |
| InterDigital [9] | Case 1-8 (3-bits) + Case 1-9 (excludes 5 SB) | AR/VR | 100% satisfied UEs [100%] 6.48 RU [6.47 RU] |

Observations

- InterDigital [9] observes slight increase of RU% for same number of satisfied UEs.

- The UL overhead of the scheme is either same as baseline (Case 1-9 alone) or higher than baseline (Case 1-8 combined with Case 1-9)

Benefits/concerns

+Leads to smaller range of CQI offset values: Mediatek [8]

+Better represent CSI for subbands to be allocated to the UE: InterDigital [9]

-Interference may change from CSI feedback to PDSCH scheduling: Apple [14]

-Not expected to have impact in improving PDSCH reception reliability: Samsung [17]

-Reporting of subbands increases overhead: Nokia [20]

-Allocation of interference is unpredictable: Nokia [20]

-Subject to limitations of CQI-based reporting: Nokia [20]

Implementation impact

-Low: Huawei [2], InterDigital [9]

Specification/testing impact

-Low: Huawei [2], Spreadtrum [4], InterDigital [9]

### Case 1-10: CSI expiration time

Evaluation results

| Qualcomm [16] | Case 1-10 | AR/VR | 60 km/h: CQI periodicity of 4 slots saves 16.7% RB compared to CQI periodicity of 80 slots (40 UEs). 3 km/h: CQI periodicity of 80 slots has about same RB usage compared to CQI periodicity of 4 slots. |
| --- | --- | --- | --- |

Observations

- Qualcomm observes a benefit in adapting CQI periodicity to UE speed so that UL overhead is only increased when there is a gain in PDSCH RU.

Benefits/concerns

[Note: UE can provide CSI correlation coefficient instead of expiration time: Qualcomm [25]]

+Provide right CSI sampling time to gNB: Qualcomm [16]

-Can use a short periodicity instead: Spreadtrum [4]

-Could be derived by gNB from observation: Intel [14], LG [19]

-Existing gNB implementation use SRS or DM-RS or channel variations from successive CSI reports: Samsung [17]

-May be difficult to make accurate prediction with fast changing interference, not suitable to URLLC: InterDigital [9], Quectel [12], NTT DoCoMo [21]

Implementation impact

-Hard to judge: Huawei [2]

Specification/testing impact

-Hard to judge: Huawei [2]

-Large: Quectel [12]

-Can define three tests (channel variations, interference variations, both): Qualcomm [25]

### Case 1-11: Partial information update

Evaluation results

| Huawei [2] | Case 1-11 (?) 1 ms CQI computation time | Factory (non-baseline) | 100 supported UEs for 100% availability (70) |
| --- | --- | --- | --- |
| Vivo [6] | Case 1-11 Full CSI every 40 ms Update CQI (only) based on IMR every 10 ms | AR/VR | 71% satisfied UEs [67%, period 40 ms]/[98%, period 10 ms] 56% RU [77%, period 40 ms]/[48%, period 10 ms] Baseline uses full CSI recalculation |
| Vivo [6] | Case 1-11 Full CSI every 40 ms Update CQI based on CSI-RS and IMR every 10 ms | AR/VR | 89% satisfied UEs [67%, period 40 ms]/[98%, period 10 ms] 52% RU [77%, period 40 ms]/[48%, period 10 ms] Baseline uses full CSI recalculation |
| InterDigital [9] | Case 1-11 (Update interference only) | AR/VR | 99% satisfied UEs [100%, period 2 ms]/[99.5%, period 20 ms] 6.60 RU [6.47 RU, period 2 ms]/[6.59 RU, period 20 ms] Report periodicity 2 ms (CQI update), 20 ms (full CSI) |

Note: Ericsson results for Case 1-1 may also be applicable here, since reporting is based on CQI:

| Ericsson [10] | Case 1-1 Mean and stdev CQI (wideband) | AR/VR (mixed traffic) | 97.5% satisfied UEs [78.5%] 76% median RU [77%] Baseline uses fixed backoff of 20 dB |
| --- | --- | --- | --- |
| Ericsson [10] | Case 1-1 Mean and stdev CQI (subband) | AR/VR (mixed traffic) | 97.2% satisfied UEs [78.5%] 60% median RU [77%] Baseline uses fixed backoff of 20 dB |

Observations

- Vivo [6] observes increase of % of satisfied UEs and decrease of %RU compared to a baseline with same frequency of full CSI reports but without partial CQI updates. Vivo [6] also observes some decrease of % of satisfied UEs and slight increase of % RU compared to a baseline where full CSI reports would be transmitted as frequently as the partial CQI updates. InterDigital [9] observes similar performance versus baseline.

- Huawei [2] observes significant increase of number of UEs that can be supported. However, the scenario is not according to agreed baseline.

- The results from Ericsson [10], Case 1-1 may be applicable assuming that the gNB can perform same calculations of statistics that the UE would perform under Case 1-1, based on the partial CQI reports.

- The UL overhead is smaller than a baseline where (full) CSI reports would be transmitted as frequently as partial CQI reports, and larger than a baseline where full CSI reports would be transmitted as frequently as full CSI reports in the enhanced scheme.

Benefits/concerns

+Better accuracy due to reduced CQI delay without increase of UE complexity: Huawei [2], Oppo [3], Spreadtrum [4], Vivo [6], (LG [19]), NTT DoCoMo [21], InterDigital [9]

+No change to reporting: Huawei [2], Vivo [6]

+Can work w/wo A-CSI on PUCCH: Huawei [2]

+Reduced UL overhead: Spreadtrum [4], NTT DoCoMo [21], InterDigital [9]

+Mature and low complexity: Spreadtrum [4], NTT DoCoMo [21]

+Suited to typically URLLC use case where UE mobility is low RI/PMI does not change frequently: Vivo [6]

+Timely captures interference variations: Vivo [6]

-How much gain can be achieved is not clear: CATT [7]

-May result in error propagation if RI/PMI is separately reported: Samsung [17], (Lenovo [22])

-URLLC UE typically do not require RI reporting, and PMI can be obtained by SRS in TDD: Samsung [17]

-Overhead savings would be minimal: Samsung [17]

-UE processing requirements need to be defined for full CSI report: Samsung [17]

-Reducing CSI timeline does not improve accuracy based on our evaluations because of small interference coherence time: Nokia [20]

-Subject to limitations of CQI-based reporting: Nokia [20]

Implementation impact

-Low: Huawei [2], Spreadtrum [4], NTT DoCoMo [21]

Specification/testing impact

-Low: Huawei [2], Spreadtrum [4], NTT DoCoMo [21]

### Other proposals related to Case 1 new reporting

-Downselect from 1-5/1-6/1-7/1-9: Spreadtrum [4]

-If overhead reduction is needed for URLLC CSI, focus on reducing CSI computation complexity and overhead based on existing CSI measurement/reporting mechanisms: Vivo [6]

-Group Case 1-1/1-2/1-3/1-11 for further study since they target providing statistics: Ericsson [10]

-Group Case 1-5/1-6/1-7/1-9 for further study since they target providing worst channel conditions: Ericsson [10]

-If new reporting is supported it should be possible to transmit simultaneously with legacy CQI: Quectel [12]

-Enable configuration of measurement time intervals or measurement occasions counter for IMR and CMR (Many Case 1 schemes (1-1, 1-3, 1-4, 1-5, 1-11) benefit from multiple IMR measurements per CSI report): Nokia [20]

## Observations for Topic #2

Observations on new reporting Case 1.

Based on the submitted contributions, the support for each scheme can be summarized as follows (for the companies who explicitly stated it). The Table also includes a recommended way forward related to each scheme.

| Scheme | Company views | Recom. |
| --- | --- | --- |
| Case 1-1 Statistical CSI/SINR | Supportive (6): InterDigital [9], Ericsson [10], Intel [14], Nokia [20], Lenovo [22] Not supportive (2): CATT [7], Samsung [17] Open to study: Huawei [2], Spreadtrum [4], Quectel [12], LG [19], NTT DoCoMo [21] | Study further |
| Case 1-2 Predicted CSI | Supportive (1): Qualcomm [16] Not supportive (5): CATT [7], InterDigital [9], Samsung [17], Nokia [20], NTT DoCoMo [21] Open to study: Huawei [2], Spreadtrum [4], Ericsson [10], LG [19] | Not study further |
| Case 1-3 Interference statistics | Supportive (3): Futurewei [11], Intel [14], Qualcomm [16], Not supportive (3): CATT [7], InterDigital [9], Samsung [17] Open to study: Huawei [2], Spreadtrum [4], Ericsson [10], LG [19], Lenovo [22] | Study further |
| Case 1-4 Interference covariance matrix | Supportive (1): Huawei [2] Not supportive (6): Spreadtrum [4], CATT [7], InterDigital [9], Samsung [17], LG [19], NTT DoCoMo [21] | Not study further |
| Case 1-5 CSI based on worst IMR occasion | Supportive (2): ZTE [5], InterDigital [9] Not supportive (3): CATT [7], Samsung [17], NTT DoCoMo [21] Open to study: Huawei [2], Spreadtrum [4], Ericsson [10], LG [19], Lenovo [22] | Study further |
| Case 1-6 Worst-M CQI | Supportive (2): InterDigital [9], Nokia [20] Not supportive (2): CATT [7], Samsung [17] Open to study: Huawei [2], Spreadtrum [4], Ericsson [10], Quectel [12], LG [19], NTT DoCoMo [21], Lenovo [22] | Study further |
| Case 1-7 Worst-best criteria for subband CQI report | Supportive (1): Qualcomm [16] Not supportive (3): CATT [7], InterDigital [9], Samsung [17] Open to study: Huawei [2], Spreadtrum [4], Ericsson [10], Quectel [12], LG [19], NTT DoCoMo [21] | Study further |
| Case 1-8 3-bits D-CQI or full SB-CQI | Supportive (3): Huawei [2], Spreadtrum [4], Mediatek [8] Not supportive (3): CATT [7], InterDigital [9], Ericsson [10], Intel [14] Open to study: Samsung [17], LG [19], NTT DoCoMo [21] | Study further |
| Case 1-9 Wideband CQI excluding worst SB | Supportive (1): Mediatek [8] Not supportive (4): Huawei [2], CATT [7], InterDigital [9], Samsung [17] Open to study: Spreadtrum [4], Ericsson [10], LG [19], NTT DoCoMo [21] | Not study further |
| Case 1-10 CSI expiration time | Supportive (1): Qualcomm [16] Not supportive (6): Huawei [2], CATT [7], InterDigital [9], Intel [14], Samsung [17], Lenovo [22] Open to study: Spreadtrum [4], LG [19], NTT DoCoMo [21] (not in Case 1) | Not study further |
| Case 1-11 Partial information update | Supportive (9): Huawei [2], Oppo [3], Spreadtrum [4], Vivo [6], InterDigital [9], Ericsson [10], Intel [14], LG [19], Lenovo [22] Not supportive (2): CATT [7], Samsung [17] Open to study: NTT DoCoMo [21] | Agree to support |

From the above schemes, Case 1-11 (Partial information update) appears to gather the most support. Based on discussions, the implementation and standardization impact is considered low. The scheme enables provision of relevant channel information at lower overhead cost than the baseline (since most reports would contain only CQI) for schedulers that employ a statistical strategy (similar to Case 1-1). The scheme may also enable reduction of processing latency for the CQI updates, or at least reduction of UE processing compared to frequent reporting of full CSI reports. For these reasons, moderator proposes to agree on the following:

FL proposal 8.1-1: Support new reporting of CQI only, where CQI is conditioned on the latest reporting instance containing RI/PMI (Case 1-11).

Among remaining schemes, Case 1-1 (Statistical CSI) gathers the most support and has also been evaluated by the most companies. In 5 out of 6 evaluations, the evaluated scheme had lower UL overhead than the baseline. In 5 out of 6 evaluations, the performance in terms of percentage of satisfied UEs and/or resource utilization was as good as or better than the baseline. Performance-wise, this scheme appears promising. However, there are concerns related to testability if SINR is utilized as a reporting metric for the new report. There is also higher specification impact and effort for defining the new statistical quantities, quantization, time window, etc. The situation is similar for Case 1-3 (Interference statistics) except that there are less supporting companies.

A subset of schemes, Case 1-5 (CSI based on worst IMR occasions) and Case 1-6 (Worst-M CQI) have also been evaluated by several companies. These schemes may provide gains in UL overhead and sometimes in % of satisfied UEs, at the expense of higher resource utilization. Most companies are interested in studying further these schemes as they have low implementation complexity and standardization impact.

Another subset of schemes, Case 1-8 (3-bits differential subband CQI or 4-bits subband CQI), has been evaluated by several companies. The results so far are mixed, with some showing gain in % of satisfied UEs and others showing no effect or degradation. These schemes have low implementation complexity and specification impact. However, several companies have the concern that these schemes increase overhead compared to R16 and should therefore prove significant benefit.

For the remaining subset of schemes, Case 1-2 (CSI prediction), Case 1-4 (Interference covariance matrix), Case 1-9 (Reference wideband CQI excluding worst subbands), Case 1-10 (CSI expiration time), they each have a single supporting company and 4 or more non supportive companies. It is therefore suggested to not continue further study of these schemes for R17.

## E-mail discussion (1st/2nd round) for Topic #2

Question 2-1: Please provide feedback if you would like to either (a) make correction in this moderator summary (such as evaluation results or company position) or (b) add your company position relative to the schemes listed in the above.

| Company | Yes/No | Comments |
| --- | --- | --- |

Question 2-2: Do you have any question for clarification, or any comment, on the available evaluation results?

| Company | Yes/No | Comments |
| --- | --- | --- |
| vivo |  | We would like to further clarify the evaluation results for Case 1-11. We have provided and compared the simulation results for the following cases. Whether or not the performance gain can be observed depends on the baseline assumption. - Scheme 1: full subband CSI report including CQI/PMI/RI with 10 ms or 40 ms periodicity - Scheme 2-1 (i.e. Case 1-11): full subband CSI including CQI/PMI/RI is reported with 40 ms periodicity. CQI-only is reportred with 10 ms periodicity conditioned on a previous reporting instance containing RI/PMI. It should be noted that scheme 1 with the shortest periodicity, i.e. 10 ms can achieve the best performance among all these cases since gNB can obtain the full CSI every 10 ms. When comparing to the scheme 1 with 10 ms, performance of scheme 2-1 is lower than that of scheme 1. This is because CQI-only is reported with 10 ms assuming previous PMI/RI within the 40 ms, while CQI/PMI/RI is reported with 10 ms for scheme 1 (10ms). However, scheme 1 with short periodicity requires large overhead and UE computation effort to obtain the CQI/PMI/RI. When compare to the scheme 1 with 40 ms, it can be seen that scheme 2-1 can achieve better performance than that of scheme 1 (40ms periodicity). Hence, CQI-only reporting conditioned on prevous PMI/RI can be beneficial in terms of the performance, with relative low computation effort. |

Following first GTW session, moderator updated FL proposal:

FL proposal 8.1-2:

Support new reporting of CQI only, where CQI is conditioned on a previous reporting instance containing RI/PMI (Case 1-11).

- One IMR occasion is associated with a CQI-only reporting instance.

  - FFS association of more than one IMR occasion to a CQI-only reporting instance. (Case 1-5, Case 1-1)

- Rel-15/16 CQI reporting formats are at least supported for CQI-only reporting instance. FFS if one or more of the following new formats are also supported:

  - Mean and stdev CQI/SINR (Case 1-1)

  - Worst-M CQI (Case 1-6)

  - 3-bits differential or 4-bits subband CQI (Case 1-8)

In this updated proposal, Case 1-11 is supported. It is clarified that there is association between IMR occasion and CQI-only reporting instance to support this functionality. The FFS sub-bullet would address possibility to support statistical CSI and CSI based on worst-IMR. Furthermore, as a baseline the CQI-only reporting instance support at least the R16 reporting formats, but the proposal leaves open the possibility of supporting additional reporting formats that are currently under study.

Question 2-3: Please indicate whether FL proposal 8.1-2 is acceptable:

| Company | Yes/No | Comments |
| --- | --- | --- |
| Vivo |  | For the updated proposal 8.1-2 by FL, we agree with the main bullet and the second sub-bullet without the FFS part. For the other sub-bullet, including the FFS parts, we think they can be decoupled with the main bullet because it is not necessary to combine the CQI-only reporting with the new reporting formats e.g. mean/STD/worst-M CQI. So they can be discussed separately whether or to support. Hence, we propose the following update for proposal 8.1-2. FL proposal 8.1-2: Support new reporting of CQI only, where CQI is conditioned on a previous reporting instance containing RI/PMI (Case 1-11). - One IMR occasion is associated with a CQI-only reporting instance. - FFS association of more than one IMR occasion to a CQI-only reporting instance. (Case 1-5, Case 1-1) - Rel-15/16 CQI reporting formats are at least supported for CQI-only reporting instance. FFS if one or more of the following new formats are also supported: - Mean and stdev CQI/SINR (Case 1-1) - Worst-M CQI (Case 1-6) - 3-bits differential or 4-bits subband CQI (Case 1-8) FFS the following - One IMR occasion is associated with a CQI-only reporting instance. - FFS association of more than one IMR occasion to a CQI-only reporting instance. (Case 1-5, Case 1-1) - if one or more of the following new formats are also supported: - Mean and stdev CQI/SINR (Case 1-1) - Worst-M CQI (Case 1-6) - 3-bits differential or 4-bits subband CQI (Case 1-8) |
| Mediatek | No | Regarding FL proposal 8.1-2, the reported evaluation results (by two companies as copied below) for Case 1-11 shows performance degradation compared to R16 baseline. The result in R1-2102352, which mentioned in the FL summary, is actually evaluates “fast CSI feedback”. So, given that all the results show that partial information update leads to performance degradation, we can’t accept proposal 8.1-2. R1-2102739 Scheme/Scenario; Report periodicity (ms); Rel-15 enabled AR/VR (OLLA OFF); Rel-15 enabled AR/VR (OLLA ON). Baseline; 2; 98.6; 100. Baseline; 20; 91.4; 99.5. Case 1-1; 2; 100; 100. Case 1-5; 20; 85.7; 99.5. Case 1-6; 2; 98.6; 100. Case 1-8 (3-bits); 2; 97.1; 100. Case 1-8 (4-bits); 2; 97.1; 100. Case 1-9; 2; 97.1; 100. Case 1-8 (3-bits) +Case 1-9; 2; 97.1; 100. Case 1-11 Int. update only; 2 (CQI) 20 (Full); 90.5; 99.0. R1-2102522 Schemes; Percentage of UEs satisfying BLER reliability requirement; Resource utilization. Scheme 1(10ms); 98.25%; 48.19%. Scheme 1(40ms); 67.22%; 77.45%. Scheme 2-0; 70.87%; 56.34%. Scheme 2-1; 89.13%; 52.43%. |
| Vivo |  | Regarding the evaluation results for Case 1-11 in R1-2102739, it should be noted that only interference is updated for the CQI-only reporting instance, which is different from the definition of Case 1-11. Regarding the evaluation results for Case 1-11 in R1-2102522, we would like to further clarify the evaluation results for Case 1-11. We have provided and compared the simulation results for the following cases. - Scheme 1 (10/40 ms for full CSI): full subband CSI report including CQI/PMI/RI with 10 ms or 40 ms periodicity - Scheme 2-1 (10 ms for CQI-only and 40 ms for full CSI): full subband CSI including CQI/PMI/RI is reported with 40 ms periodicity. CQI-only is reported with 10 ms periodicity conditioned on a previous reporting instance containing RI/PMI. It should be noted that scheme 1 with the shortest periodicity, i.e. 10 ms can achieve the best performance among all the schemes since gNB can obtain the full CSI every 10 ms. - When baseline is scheme 1 with 10 ms full CSI report, performance of scheme 2-1 is lower than that of scheme 1. This is because CQI-only is reported with 10 ms assuming previous PMI/RI within the 40 ms, while CQI/PMI/RI is reported with 10 ms for scheme 1 (10ms for full CSI). However, scheme 1 with short periodicity requires large overhead and UE computation effort to obtain the CQI/PMI/RI. - When baseline is scheme 1 with 40 ms full CSI report, it can be seen that scheme 2-1 can achieve better performance than that of scheme 1 (40ms for full CSI). Hence, CQI-only reporting conditioned on previous PMI/RI can be beneficial in terms of the performance, with relative low computation effort. Whether or not the performance gain can be observed depends on the baseline assumption. We believe this is also true for other CSI enhancement schemes. Schemes; Percentage of UEs satisfying BLER reliability requirement; Resource utilization. Scheme 1(10ms); 98.25%; 48.19%. Scheme 1(40ms) (40 ms for full CSI); 67.22%; 77.45%. Scheme 2-0; 70.87%; 56.34%. Scheme 2-1 (10 ms for CQI-only and 40 ms for full CSI); 89.13%; 52.43%. |
| Huawei |  | We are supportive to case 1-11 but if the group wants to specify additional schemes then we are also open for that. At this stage we agree with vivo that we should not try mix schemes under one framework. This is very difficult and could result into side effects that are difficult to anticipate. We think it is a cleaner approach to take a high level agreement which schemes to support. This is especially valid for case 1, where the different schemes intend to solve different issues. Yesterday in the GTW Paul proposed to agree on case 1-11. One benefit of this scheme is that partial CQI update can reduce the UE processing time and therefore allows to shorten the time gap between channel measurement and the instance when it can be applied. The schemes under case 1 are not mutual exclusive. We think that rather than debating between them, we could spent our time to specify multiple schemes. We think this is feasible and will also result into a better outcome of this WI. Regarding the proposal 9.1-1, we suggest to merge it with case 1 and have one common proposal for case 1 and case 2. So our suggestion would be the following proposal. Potential proposal: For CSI enhancements in Rel-17 - Support new reporting of CQI only, where CQI is conditioned on a previous reporting instance containing RI/PMI (Case 1-11). - FFS: The CQI processing time is reduced compared to Rel-16 CSI processing delay - Support at least one additional case 1 scheme for CSI enhancement, e.g. - Mean and stdev CQI/SINR (Case 1-1) - Worst-M CQI (Case 1-6) - 3-bits differential or 4-bits subband CQI (Case 1-8) - FFS further scheme(s) - For new reporting Case 2, continue study focusing on reporting of (delta) CQI/MCS/SINR (Case 2-3). It would be great to hear the views from others about the proposal above. Would this be an acceptable way forward? |
| Mediatek |  | Regarding your comment “Whether or not the performance gain can be observed depends on the baseline assumption”, the baseline should be what can be achieved with R16. We don’t see it as meaningful to use bad configuration to show some gains. In the results you provided, R16 (with 10ms periodicity) is outperforming Scheme 2-0 & Scheme 2-1 in terms of percentage of satisfied UEs and resource utilization. In addition, if you compare Scheme 1(10ms) with Scheme 2-1, the results clearly show that partial information update degrade the system’s performance. Both schemes report every 10 ms, but Scheme 2-1 provides partial CSI update. Hence, there was negative performance impact. Schemes; Percentage of UEs satisfying BLER reliability requirement; Resource utilization. Scheme 1(10ms); 98.25%; 48.19%. Scheme 1(40ms) (40 ms for full CSI); 67.22%; 77.45%. Scheme 2-0; 70.87%; 56.34%. Scheme 2-1 (10 ms for CQI-only and 40 ms for full CSI); 89.13%; 52.43%. |
| Vivo |  | We acknowledge that in our evaluation scheme 1 (with 10 ms periodicity) outperforms the partial update scheme 2-1/2-0. In fact, we assume 4-bits subband CQI (not really Rel-16 method) for scheme 1, which will have the best performance when the report interval is not smaller than that for scheme 1. If we don’t care about the overhead and complexity, scheme 1 with short periodicity i.e. full 4-bit subband CSI reporting, is definitely desirable. But we do need to consider the overhead and complexity when designing the enhancement for CSI. As Thorsten pointed out, one benefit of this scheme is that partial CQI update can reduce the UE processing time and therefore allows to shorten the time gap between channel measurement and the instance when it can be applied. We think that is the main motivation for Case 1-11. Thanks again for the focusing on the evaluation results and the discussion is very helpful for better understanding. For the proposal, we think Thorsten’s suggestion is a good point and the update from Thorsten can be good way forward for further discussion. Let’s hear more views from other companies. |
| ZTE |  | Regarding the case 1-11 in the main bullet, we think maybe we should first clarify the reduced CSI processing timeline. Since this scheme is beneficial in terms of the more accurate MCS selection only if the CSI timeline is reduced for this CSI report in our understanding. But we are not sure if we can make progress on the timeline considering that it is really a complicated issue. Regarding the remaining parts, we are fine with either the suggested proposal from FL or the updates from Xiaohang. Below is the updates on top of the Xiaohang's version with incorporating the updates from Thorsten. FL proposal 8.1-2: Support new reporting of CQI only if the CQI processing timeline is reduced, where CQI is conditioned on a previous reporting instance containing RI/PMI (Case 1-11). - One IMR occasion is associated with a CQI-only reporting instance. FFS: The CQI processing time is reduced compared to Rel-16 CSI processing delay - FFS association of more than one IMR occasion to a CQI-only reporting instance. (Case 1-5, Case 1-1) - Rel-15/16 CQI reporting formats are at least supported for CQI-only reporting instance. FFS if one or more of the following new formats are also supported: - Mean and stdev CQI/SINR (Case 1-1) - Worst-M CQI (Case 1-6) - 3-bits differential or 4-bits subband CQI (Case 1-8) FFS the following - One IMR occasion is associated with a CQI-only reporting instance. - FFS association of more than one IMR occasion to a CQI-only reporting instance. (Case 1-5, Case 1-1) - if one or more of the following new formats are also supported: - Mean and stdev CQI/SINR (Case 1-1) - Worst-M CQI (Case 1-6) - 3-bits differential or 4-bits subband CQI (Case 1-8) |
| Qualcomm |  | We are in general fine with the spirit of FL’s proposal showed in GTW, which is a good direction for further study. We just have one comment, we think the maturity level of case 1 and case 2 are similar. We don’t see that in case 1, we are ready to go ahead to decide “support” case 1-11, while in case 2, we just continue to study case 2-3, as we don’t see the study of case 1-11 is more mature than case 2-3. Actually, as MTK pointed out, the simulation results of case 1-11 needs some discussion. We are OK to continue study focusing on these two cases. A side comment regarding the proposals from companies, in general it looks a little ad hoc. Particularly, the proposal from Hisilicon is not acceptable to us. We don’t see the motivation to agree at this point supporting 2+ schemes in case 1 while just continue to study case 2. |
| LG |  | We are fine with the FL proposal. It should be common understanding that the smaller gap between measurement and report would bring better performance. For supporting additional scheme, it would be difficult to decided on “how many scheme can be supported additionally” and “which scheme needs to be supported”. It would be repetition of RAN1#104-e. On the other hands, current proposal takes case 1-11 as a baseline, which give us criteria for upcoming decision. We think it is more better way. |
| Samsung |  | Agree with the comment from Qualcomm. The proposal from the GTW session is generally acceptable – the update proposal above is not. Also fine to follow Qualcomm’s suggestion and revise the proposal as follows. FL proposal 8.1-2: Study support of new reporting of CQI only, where CQI is conditioned on a previous reporting instance containing RI/PMI (Case 1-11). |
| HW/HiSI |  | The original GTW proposal is also acceptable for us. But if we now switch “support” to “study”, what progress have we then made? We have already agreed to study these schemes that are now under discussion. |
| Intel |  | We think the main bullet to support CQI-only reporting does not capture the commonalities between schemes which were showing benefits by simulations. To us, calling the mean/std reporting to be “CQI-only” reporting looks artificial. We are fine to formulate the proposal in an “umbrella” manner to cover more than one Case 1-x scheme. In that sense, something like: - Support explicit enabling/disabling of filtering of measurements on CSI-RS/IM occasions in time and/or frequency domain - FFS the filtering scheme(s): Mean, StdEv, Worst sub-band(s), Selective reporting, etc. FFS other details |
| Hw/HiSi |  | It get the impression that the proposals are now diverging a lot and my feeling is that it will be difficult to achieve consensus during the next GTW, maybe a small step forward would be to agree on which schemes to support (if any at this stage), study further or not support. At least for the schemes not to support, we could for example follow the FL guidance in section 8.2. and 9.2 of the summary. Then, we have more time for more discussion and converge further on the next checkpoint. |
| Lenovo, Motorola Mobility |  | In our view, it would be good to check how much reporting CQI only can reduce CSI computation delay (e.g., from CSI computation delay requirement 2 to CSI computation delay requirement 1?) before deciding to support Case 1-11. |
| Futurewei |  | Not support FL proposal 8.1-2 in its current form. It has been pointed out by companies that simulation results show that performance of Case 1-11 is actually worse than baseline. On the other hand, we have shown in our contribution R1-2102759 that Case 1-3 (interference statistics) achieves a 35% performance gain over the baseline. Furthermore, we also showed that Case 1-3 achieves a 14% performance gain over Case 1-1 (SINR statistics). However, in Proposal 8.1-2, Case 1-3 with better performance was excluded from the list of support/FFS while other schemes with worse performance were selected. In our opinion, this kind of selection process is unfair, not technical-based, and should be avoided. We suggest the group should spend more effort on comparing performance of different schemes based on available simulation results. The group can then make decision on schemes for supporting and/or FFS based on these performance comparisons. |
| Apple |  | It seems the simulation results don’t lead to the conclusion of supporting 1-11. |
| Spreadtrum |  | We support the main bullet and first sub-bullet. It is a good achievement. Case 1-11 is the one has the most supporters considering this specification impact and complexity. For second sub-bullet FFS point, it would be better to separate them a part from Case 1-11. Since we don't achieve a common understanding wether or not to support them. It is even harder to combine them with Case 1-11 together. So our suggestion is the other Cases can be studied in another proposal. |
| Quectel |  | We are in principle fine with FL’s proposal. We also tend to agree with Huawei that at this stage it may be difficult to converge on the support of certain schemes.To narrow down the checklist could be a way forward at this point. |
| CATT |  | We agree with the comments from other companies that it is difficult to agree to support a scheme at this point. |
| OPPO |  | We agree with FL’s proposal 8.1-2, except the first bullet. To our understanding, Rel-16 already supports the association of one single IMR and multiple IMRs to a CQI report. There is no sufficient technical motivation to promote one over the other at this stage. The whole bullet relating to IMR can be FFS or simply removed. |

Following initial comments, moderator proposes to check if following FL can be agreeable as a first step:

FL proposal 8.1-3:

For new reporting Case 1, do not consider further the following schemes:

- Case 1-2: CSI prediction

- Case 1-4: Interference covariance matrix

- Case 1-9: Reference wideband CQI excludes worst sub-bands

- Case 1-10: CSI expiration time

Question 2-3: Please indicate whether FL proposal 8.1-3 is acceptable:

| Company | Yes/No | Comments |
| --- | --- | --- |
| QC | No | We still prefer the original FL proposal with minor modification to use the similar wording as for case 2. In our view, the original FL proposal can achieve more progress than the current proposal, because it narrow down to prioritize future study to a single scheme. For case 1, continue study focusing on new reporting of CQI only, where CQI is conditioned on a previous reporting instance containing RI/PMI (Case 1-11). |
| Apple | Yes |  |
| DOCOMO | Yes |  |
| ZTE | Yes |  |
| Quectel | Yes |  |
| CATT | Yes |  |
| OPPO | Yes |  |

# Topic #3: New reporting (Case 2)

## Summary of issues for Topic #3

For Case 2 new reporting, RAN1 agreed to continue studying with focus on new reporting type based on PDSCH decoding for OLLA performance enhancement. Many companies discuss and evaluate potential benefits of such schemes. The main design objectives can be identified as follows:

- Better support of OLLA for very low target BLER:

  - Indicate to scheduler to be more conservative even in case of ACK

  - Allow OLLA converge faster to proper operating point

  - Case 2-1, Case 2-2, Case 2-3

- Help gNB schedule a retransmission:

  - Meet target latency while avoiding spending too much resources on retransmissions

  - Case 2-1, Case 2-3, Case 2-4

- Address other potential issues:

  - Differentiating DTX from NACK: Case 2-6

  - Differentiating fading/blockage/interference: Case 2-5

In the following sub-sections, performance results and detailed views on benefits and complexity are presented for each scheme.

### Case 2-1: Decoding margin

Evaluation results

| InterDigital [9] | Case 2-1 (LDPC iters) | AR/VR | 99.5% satisfied UEs [99.5%] 6.59 PRBs [6.59] Report periodicity 20 ms |
| --- | --- | --- | --- |
| InterDigital [9] | Case 2-1 (dB margin, slow) | AR/VR | 100% satisfied UEs [99.5%] 6.75 PRBs [6.59] Report periodicity 20 ms |

Observations

- InterDigital [9] observes same performance as baseline for scheme based on number of LDPC iterations, and slight increase of % of satisfied UEs for scheme based on dB margin, at the expense of slight increase of RUs.

Benefits/concerns

+May be useful for specific scenarios (rapid channel changes or inter-cell interference): Mediatek [8]

+Helps OLLA work for low BLER targets: Ericsson [10], Sony [18], NTT DoCoMo [21], InterDigital [9]

-Does not address bursty interference: Huawei [2], Vivo [6], Futurewei [11], Samsung [17]

-Results so far inconclusive due to improper baseline, no obvious gain: Huawei [2], Vivo [6]

-May not work if retransmission uses different frequency/TBS/precoder: Vivo [6]

-Existing OLLA may be sufficient if CQI reporting is as frequent as packet transmission: Mediatek [8]

-Requires twice overhead of R16: Samsung [17]

-Large impact of quantization and measurement errors: Samsung [17]

-Need common metric not UE-specific: LG [19]

-Unclear what criteria to be fulfilled: Nokia [20]

-Soft margin based approach does not converge towards any BLER level

Implementation impact

-Medium to high: Huawei [2]

-Can be done by attempting to decode PDSCH with partial resource: Sony [18]

-Requires new UE implementation and complex gNB implementation: Samsung [17]

Specification/testing impact

-Medium to high: Huawei [2]

-LDPC iteration is vendor-specific: Nokia [20], NTT DoCoMo [21]

-Issues to discuss: whether additional feedback is always sent, whether it is jointly encoded with HARQ-ACK or not, impact on timing /processing, which PDSCH is selected: Lenovo [22], LG [19]

-Test requirement that reported values from UE e.g. varies monotically with varied SINR at fixed MCS: Ericsson [10]

-Not testable: Samsung [17]

### Case 2-2: Block error probability

Evaluation results

| InterDigital [9] | Case 2-2 | AR/VR | 100% satisfied UEs [99.5%] 6.80 PRBs [6.59] Report periodicity 20 ms |
| --- | --- | --- | --- |
| Intel [14] | Case 2-2 | Factory | ~3% less satisfied UEs than baseline (1e-5 target) 27.2% RU [23.6%] |
| Nokia [20] | Case 2-2 | AR/VR | 5 ms 99.9999%-pct latency [2 ms] 20% RU [3%] |
| Nokia [20] | Case 2-2 + Case 1-1: Mean + stdev SINR | AR/VR | 1 ms 99.9999%-pct latency [2 ms] 6% RU [3%] |
| Nokia [20] | Case 2-2 | Factory | ~1 ms 99.999%-pct latency [1 ms] |
| Nokia [20] | Case 2-2 Case 1-1: Mean + stdev SINR | Factory | ~1 ms 99.999%-pct latency [1 ms] |

Observations

- InterDigital [9] observes slight increase of % of satisfied UEs at the expense of slight increase of RUs. Intel [14] observes slight decrease of % of satisfied UEs with slight increase of RUs. Nokia [20] observes degradation in 1e-5 percentile latency from 2 to 5 ms in AR/VR environment if used alone, and improvement in 1e-5 percentile latency from 2 to 1 ms in AR/VR environment is used in combination with Case 1-1. Nokia [20] observes same performance as baseline in Factory environment.

Benefits/concerns

+Allow fast convergence of OLLA by adjusting every TB: Nokia [20]

+May allow OLLA to converge toward wanted EP level without NACK: InterDigital [9]

-Does not address bursty interference: Huawei [2], Vivo [6], Futurewei [11], Samsung [17]

-Results so far inconclusive due to improper baseline, no obvious gain: Huawei [2], Vivo [6]

-May not work if retransmission uses different frequency/TBS/precoder: Vivo [6]

-Large impact of quantization and measurement errors: Samsung [17]

-Large overhead (larger than Case 2-1): Samsung [17], InterDigital [9]

-Need common metric not UE-specific: LG [19]

Implementation impact

-Medium to high: Huawei [2]

-Requires new UE implementation and complex gNB implementation: Samsung [17]

-Can be derived from LLR: Nokia [20]

Specification/testing impact

-Medium to high: Huawei [2]

-Issues to discuss: whether additional feedback is always sent, whether it is jointly encoded with HARQ-ACK or not, impact on timing /processing, which PDSCH is selected: Lenovo [22], LG [19]

-Can be tested by measuring long-term BLER in non-fading channel scenarios: Mediatek [8]

### Case 2-3: (Delta) CQI/MCS/SINR

Evaluation results

| ZTE [5] | Case 2-3 (Delta SINR) Initial transmission | AR/VR | 61% satisfied UEs [50%] 2.3% RU [1.9%] |
| --- | --- | --- | --- |
| ZTE [5] | Case 2-3 Retransmission: Delta SINR (3-bit) | AR/VR | 94% satisfied UEs [50%] 33% RU [1.9%] |
| ZTE [5] | Case 2-3 Retransmission: Delta MCS (3-bit) | AR/VR | 60% satisfied UEs [50%] 1.9% RU [1.9%] |
| InterDigital [9] | Case 2-3 Retransmission: MCS | AR/VR | 99.5% satisfied UEs [99.5%] 6.59 PRBs [6.59] Report periodicity 20 ms |
| Qualcomm [16] | Case 2-3 Retransmission: Report CQI/MCS | AR/VR (mixed traffic, 20 URLLC UEs) | 100% satisfied UEs [100%] 3471 RBs for 2nd Tx [5255] |
| Qualcomm [16] | Case 2-3 Retransmission: Report CQI/MCS | AR/VR (mixed traffic, 100 URLLC UEs) | 100% satisfied UEs [100%] 5878 RBs for 2nd Tx [7545] |

Observations

- When using the delta-SINR scheme for initial transmission, ZTE [5] observes increase of % of satisfied UEs at the expense of some increase of RU.

- When using delta-SINR scheme for retransmission, ZTE [5] observes significant increase of % of satisfied UEs at the expense of much higher RU. When using delta-MCS scheme for retransmission, ZTE [5] observes some increase of % of satisfied UEs with similar RU as baseline. When using MCS scheme for retransmission, InterDigital [9] observes same performance as baseline. When using MCS scheme for retransmission, Qualcomm [16] observes between 22% and 36% decrease of RU for the retransmission depending on the scenario.

Benefits/concerns

+Helps OLLA work for low BLER targets: Ericsson [10], Sony [18]

+Improves convergence performance: Oppo [3]

+No additional overhead of measurement resource or computation time budget: Oppo [3]

+Small feedback overhead compared to Case 1: ZTE [5]

+Enables gNB to know proper MCS for retransmission, saving number of RBs: Qualcomm [16]

-Does not address bursty interference: Huawei [2], Vivo [6], Futurewei [11], Samsung [17]

-Results so far inconclusive due to improper baseline, no obvious gain: Huawei [2], Vivo [6]

-May not work if retransmission uses different frequency/TBS/precoder: Vivo [6], NTT DoCoMo [21]

-Low probability of retransmission in URLLC limits gains: Intel [14]

-Requires larger overhead: Samsung [17]

-Large impact of quantization and measurement errors: Samsung [17]

-Need common metric not UE-specific: LG [19]

Implementation impact

-Medium to high: Huawei [2]

-Can use number of failed parity checks: Qualcomm [16]

-Requires new UE implementation and complex gNB implementation: Samsung [17]

Specification/testing impact

-Medium to high: Huawei [2]

-More mature than Case 2-1/Case 2-2: Spreadtrum [4]

-Issues to discuss: whether additional feedback is always sent, whether it is jointly encoded with HARQ-ACK or not, impact on timing /processing, which PDSCH is selected: Lenovo [22], LG [19]

-Throughput test or BLER test: Qualcomm [16]

-May be easier to specify/test than Case 2-1/Case 2-2: InterDigital [9]

### Case 2-4: HARQ redundancy version sequence

Evaluation results

None available

Benefits/concerns

+UE has a clear view of the status of UE’s soft bits: Apple [15]

-Impact of RV may be negligible: InterDigital [9]

-Only provides information about scheduled PRBs: Huawei [2]

-Questionable if any gain can be observed: Huawei [2]

-Unclear how the UE gets the information: Spreadtrum [4]

-Low benefit due to low BLER of URLLC and small TB size: Samsung [17], NTT DoCoMo [21]

-Need common metric not UE-specific: LG [19]

-Which PDSCH is selected?: LG [19]

Implementation impact

-Hard to judge: Huawei [2]

-Unclear feasibility: Samsung [17]

Specification/testing impact

-Hard to judge: Huawei [2], InterDigital [9]

-Unclear how to test: Samsung [17]

### Case 2-5: Reason for NACK

Evaluation results

None available

Benefits/concerns

+Helps gNB determine if it should change beam, RB, cell: Qualcomm [17]

+Allows OLLA to not overreact to interference spike: Sony [18]

-Unclear how UE derives information: Spreadtrum [4], NTT DoCoMo [21]

-High reporting overhead: Samsung [17]

-UE rarely reports NACK, no actual benefit expected for URLLC: Samsung [17]

Implementation impact

-Medium to high: Huawei [2]

-Can be based on difference between interference+noise in PDSCH and long term interference being above or below threshold: Sony [18]

Specification/testing impact

-Medium to high: Huawei [2]

-Unclear how to specify/test: InterDigital [9]

-Can set up a test scenario for each reason for failure: Qualcomm [16]

### Case 2-6: Number of NACK values

Evaluation results

None available

Benefits/concerns

+Enables conventional OLLA by differentiating NACK from DTX: Samsung [17]

+No additional UE complexity, testability issue, RAN4 performance requirements: Samsung [17]

+Allows the network to detect missed PDCCH: InterDigital [9]

-Low probability of NACK in URLLC: Spreadtrum [4], NTT DoCoMo [21]

-PDCCH misdetection probability is low for URLLC: NTT DoCoMo [21]

Implementation impact

-Hard to judge: Huawei [2]

Specification/testing impact

-Hard to judge: Huawei [2]

-Small: Samsung [17]

### Other proposals related to Case 2 reporting

-A-CSI on PUCCH should be pre-requisite to further study Case 2: Huawei [2]

-Simulation results should be compared to normal OLLA and full R16 CSI report as baseline: Huawei [2]

-Discuss together Cases 2-1/2-2/2-3: Spreadtrum [4], Ericsson [10]

-Discuss CSI priority between case 1/case 2/legacy: LG [19]

## Observations for Topic #3

Observations on new report types (Case 2)

Based on the submitted contributions, the support for each scheme can be summarized as follows (for the companies who explicitly stated it):

| Scheme | Company views | Recom. |
| --- | --- | --- |
| Case 2-1 Decoding margin | Supportive (2): Spreadtrum [4], (Ericsson [10]) Not supportive (1): Samsung [17] Open to study: Huawei [2], Futurewei [11], Quectel [12], NTT DoCoMo [21], Lenovo [22] | Not study further |
| Case 2-2 Block error probability | Supportive (3): Spreadtrum [4], Mediatek [8], Nokia [20] Not supportive (1): Samsung [17] Open to study: Huawei [2], Futurewei [11], Quectel [12], NTT DoCoMo [21], Lenovo [22] | Not study further |
| Case 2-3 (Delta) CQI/MCS/SINR | Supportive (7): Oppo [3], Spreadtrum [4], ZTE [5], CATT [7], InterDigital [9], Ericsson [10], Qualcomm [16] Not supportive (1): Samsung [17] Open to study: Huawei [2], Futurewei [11], Quectel [12], Lenovo [22], NTT DoCoMo [21] | Study further |
| Case 2-4 HARQ RV sequence | Supportive (1): Apple [15] Not supportive (5): Huawei [2], Quectel [12], InterDigital [9], Samsung [17], NTT DoCoMo [21] Open to study: Spreadtrum [4] | Not study further |
| Case 2-5 Reason for NACK | Supportive (2): Qualcomm [16], Sony [18] Not supportive (4): Huawei [2], InterDigital [9], Quectel [12], Samsung [17] Open to study: Spreadtrum [4], LG [19], NTT DoCoMo [21] | Not study further |
| Case 2-6 Number of NACK values | Supportive (1): Samsung [17] Not supportive (5): Huawei [2], InterDigital [9], Ericsson [10], Quectel [12], NTT DoCoMo [21] Open to study: Spreadtrum [4], LG [19] | Not study further |

From the above schemes, Case 2-3 ((Delta) CQI/MCS/SINR) appears to gather the most support. These schemes have been evaluated by 3 companies, and benefits are observed by 2 companies. Since there are still not a lot of evaluations showing significant gain and given that the specification and implementation impact of the scheme is significant, it is difficult to recommend to agree on supporting this scheme for this meeting. For Case 2-2 (Block error probability), it seems that no evaluation result shows a gain except [20] when combined with a statistical CSI scheme. For Case 2-1 (Decoding margin), the only evaluation result does not show a gain. Case 2-1 may also be considered as a special case of Case 2-3 where the number of reported levels is 2. The remaining Case 2 schemes have no evaluation result and substantial number of not supportive companies. For these reasons, it is recommended to focus Case 2 efforts on Case 2-3 for further studies.

FL proposal 9.1-1: For new reporting Case 2, continue study focusing on reporting of (delta) CQI/MCS/SINR (Case 2-3).

## E-mail discussion (1st/2nd rounds) for Topic #3

Question 3-1: Please provide feedback if you would like to either (a) make correction in this moderator summary (such as evaluation results or company position) or (b) add your company position relative to the schemes listed in the above.

| Company | Yes/No | Comments |
| --- | --- | --- |

Question 3-2: Do you have any question for clarification, or any comment, on the available evaluation results?

| Company | Yes/No | Comments |
| --- | --- | --- |

Question 3-3: Please indicate whether FL proposal 9.1-1 is acceptable:

| Company | Yes/No | Comments |
| --- | --- | --- |
| Vivo | Yes |  |
| Mediatek |  | We prefer to keep the option of reporting a form of BLEP (e.g. flipped bits before and after LDPC decoding) for further study. Our LLS results show that flipped-bits/raw-BER is good indicator of the BLER, which could help with OLLA/link adaptation. |
| Huawei | (Yes) | Regarding the proposal 9.1-1, we suggest to merge it with case 1 and have one common proposal for case 1 and case 2. |
| Nokia |  | We also have some concerns about focusing on 2-3 only when Case 2-2 also showed good performance. When I checked details, it seems more or less all reporting of case 2-2, 2-3 uses similar measurements of SINR of REs. We should be able to have a joint way forward there. |
| Moderator |  | @Mohammed, you said: Regarding FL proposal 9.1-1, we prefer to keep the option of reporting a form of BLEP (e.g. flipped bits before and after LDPC decoding) for further study. Our LLS results show that flipped-bits/raw-BER is good indicator of the BLER, which could help with OLLA/link adaptation. I think utilizing flipped bits is one of the ways (just like LDPC iterations, LLR, etc.) that a UE implementation can derive the report. It could be used to derive e.g. a “delta-MCS” as well? At some level, I suspect that all these types of reporting would be somewhat equivalent for a realistic design that would involve just 1 additional bit? If the report is “log-BLEP” but there are 2 levels, then it is either above or below a certain BLEP and the scheduler either steps up or down the OLLA target (in case of ACK). If the report is 1-bit “delta-MCS” for a certain reference BLEP, then the scheduler would behave in the same way. |
| LG |  | We are fine with the proposal. |
| Samsung |  | FL proposal 9.1-1 is acceptable. |
| Intel |  | Up to this point we did not obserce SLS gains from Case-2 schemes, including delta MCS reporting. Note, in previous meetings we evaluated CSI triggering by NACK which delivers up-to-date CSI to the scheduler which can be used for ReTX scheduling, which did not show gains. However, we are happy to narrow-down to Case 2-3 only for further debates. |
| Lenovo, Motorola Mobility |  | Ok with the proposal |
| Futurewei |  | We are ok to narrow-down the scope of Case 2 schemes to Case 2-3. So we are ok with FL proposal 9.1-1. |
| QC | Yes | To MTK and Nokia: we share the same view as moderator that either using BLEP or # flip bits, # LDPC iteration, or # failed parity checks (QC proposal) to reflect SNR are all UE internal implementation. Maybe we don’t need to debate which one of the above is better. At the end of the day, what we need is use 1 additional bit to indicate multiple ACK or NACK levels, which can be captured as a delta MCS. I think delta MCS report should be a good WF, which is easier to specify in RAN1 and easier to test in RAN4. To Intel: we have provided both SLS and LLS in our contribution for case 2-3. Both simulations showed significant gain in RB usage. Please notice that RB usage is an important factor to consider for URLLC. More efficient RB usage will translate into gain in percentage of URLLC UE that can meet URLLC requirements (if that is the gain your are looking for), when there are more and more URLLC UEs in the system. It is just due to extremely long simulation time, it is impractical to run long URLLC simulations with large number of URLLC UEs. |
| DOCOMO |  | We are fine with the proposal. |
| ZTE |  | Support |
| Spreadtrum |  | We are fine with the proposal. |
| Quectel | Yes |  |
| CATT | Yes |  |
| OPPO | Yes |  |

# Topic #4: Other enhancements

Contributions discuss enhancements that do not fall in one of the above categories.

## Summary of issues for Topic #4

Issue #4-1: Support CSI feedback for PDCCH

- Support: Qualcomm [16]

  - Motivations

    - PDCCH needs to be at least as reliable as PDSCH [19][21]

    - OLLA not possible for PDCCH because gNB cannot distinguish between NACK and DTX for multi-bit HARQ-ACK [19]

    - CSI for PDCCH cannot be derived from CSI for PDSCH as coding scheme, resource (coreset), TCI state, DMRS configuration are different [21]

    - Increased PDCCH blocking/overhead if PDCCH is scheduled too conservatively [21]

  - Candidate solution

    - Tri-state HARQ-ACK [16]

- No support: Ericsson [10], Intel [14]

  - Can use rank1 restriction which is anyway useful for URLLC [10]

  - Does not need to be more accurate than PDSCH link adaptation for small allocation [10]

  - Main challenge is bursty interference which can be addressed by statistical CSI [10]

  - Code rate / resource adaptation for PDCCH is very coarse [10][14]

  - RSRP, L1-SINR, DTX of HARQ-ACK can be used [14]

Issue #4-2: Support priority index 1 for P-CSI/SP-CSI/A-CSI on PUCCH

- Support for P-CSI/SP-CSI:

  - Yes: Intel [14]

  - No: ZTE [5]

- Support for A-CSI (if supported):

  - Yes: ZTE [5], NTT DOCOMO [21]

The following miscellaneous proposed enhancements do not neatly fall in one of the above categories:

- Specify CSI enhancements to better fit the needs of SPS PDSCH(s) [10]

- Reconfigure definition of CSI reference resource to better align with typical URLLC payload sizes: Nokia [20]

- Split CSI report in multiple parts and multiplex as they become available: Lenovo [22]

- Improve CSI framework to minimize delaying/dropping CSI reports for URLLC/IIoT operation: Lenovo [22]

- Link MCS table to priority indicator: Samsung [17]

- UE request for CSI measurement to update CSI for a new Tx-Rx beam pair: Qualcomm [16]

- A-CSI on PUCCH multiplexed on PUSCH repetition type B: NTT DOCOMO [21]

## E-mail discussion (1st round) for Topic #4

TBD

# References

RP-201310	Revised WID: Enhanced IIoT and URLLC support for NR, Nokia, Nokia Shanghai Bell.

R1-2102352	CSI feedback enhancements	Huawei, HiSilicon, SIA

R1-2102393	CSI feedback enhancements for URLLC	OPPO

R1-2102455	Discussion on CSI feedback enhancements	Spreadtrum Communications

R1-2102494	Discussion on CSI feedback enhancements for eURLLC	ZTE

R1-2102522	CSI feedback enhancements for Rel-17 URLLC	vivo

R1-2102629	CSI feedback enhancements	CATT

R1-2102695	CSI feedback enhancements for URLLC	MediaTek Inc.

R1-2102739	CSI feedback enhancements	InterDigital, Inc.

R1-2102745	CSI Feedback Enhancements for IIoT/URLLC	Ericsson

R1-2102759	CSI feedback enhancements for URLLC	FUTUREWEI

R1-2102872	Discussion on CSI Feedback Enhancements	Quectel, Langbo

R1-2102911	Discussion on CSI feedback enhancements for URLLC	CMCC

R1-2103028	Analysis of enhanced CSI feedback schemes	Intel Corporation

R1-2103104	Views on URLLC CSI feedback enhancements	Apple

R1-2103164	CSI enhancement for IOT and URLLC	Qualcomm Incorporated

R1-2103237	On Enahncements of CSI Reports for URLLC	Samsung

R1-2103301	Considerations on CSI feedback enhancements	Sony

R1-2103348	Discussion on CSI feedback enhancements for URLLC	LG Electronics

R1-2103434	CSI feedback enhancements for URLLC/IIoT use cases	Nokia, Nokia Shanghai Bell

R1-2103575	Discussion on CSI feedback enhancements for Rel.17 URLLC	NTT DOCOMO, INC.

R1-2103611	CSI feedback enhancements for URLLC/IIoT	Lenovo, Motorola Mobility

R1-2102131, “Feature lead summary#4 on CSI feedback enhancements for enhanced URLLC/IIoT”, Moderator (InterDigital).

R1-2102749	Summary of additional discussions on CSI feedback enhancements for enhanced URLLC/IIoT after RAN1#104-e	Moderator (InterDigital, Inc.)

R1-2103800	CSI enhancement for IOT and URLLC	Qualcomm Incorporated

# Appendix: Previous agreements

Agreements from RAN1#104-e

R1-2101811

Conclusion: Continue evaluation of new reporting Case 1 and Case 2 for the schemes identified in Appendix B of R1-2102131.

- Companies are encouraged to provide their views on each scheme against each criterion in respective Tables in Appendix B.

- Companies are encouraged to provide additional evaluation results for as many schemes as possible, based on assumptions agreed in RAN1#102-e.

- Aim for down-selection at RAN1#104-b-e by taking into account evaluation results and assessment against criteria from Appendix B.

- Agreements from RAN1#103-e:

Agreements

- No change of CSI processing time relative to Rel-16 CSI in this WI

- CSI processing time specific to a new CSI reporting quantity/type (if supported) can be studied

Agreement:

- For Case-2 new reporting, continue studying with focus on the new reporting type based on PDSCH decoding for OLLA performance enhancement for initial and re-transmissions of PDSCH.

Agreements:

For Case-1 New reporting, the following candidate schemes have been identified to address the fast interference change over time. Continue studying with focus on the identified schemes below for further study and evaluation.

- Scheme 1a: New reporting quantity based on CQI/SINR statistics, e.g.,

  - CQI/SINR statistics (e.g., mean, variance, etc.)

  - CSI prediction

- Scheme 1b: New reporting quantity of interference statistics (e.g., mean, variance, interference covariance matrix, etc.)

- Scheme 1c: New reporting quantity based on modifying existing reporting format, e.g.,

  - CQI reporting considering the worst subbands

  - Subband CQI granularity enhancement

- Scheme 1d: New reporting quantity related to CSI expiration time

- Scheme 1e: New reporting quantity with partial information update, e.g.,

  - CSI reporting with interference update only

Companies are encouraged to investigate the above schemes, aiming for down-selection in RAN1#104-e

- Agreements from RAN1#102-e:

Agreement:

- CSI feedback enhancement for Multi-TRP transmission is not to be discussed further under IIoT/URLLC enhancement WI

Agreements:

- Baseline assumptions are used as the required minimum to be simulated for the evaluation of candidate CSI enhancement schemes

  - Reuse the assumptions in TR 38.824 and TR 38.901 as a starting point

  - Companies shall report additional parameters (e.g., CSI measurement settings, CSI reporting schemes) used in their evaluation

  - FFS details of baseline assumptions

- Companies can bring additional simulation results with other set(s) of assumptions

Agreements:

- Study/evaluate further on following CSI enhancement schemes in terms of technical benefit, specification and implementation impacts.

  - New triggering methods for A-CSI and/or SRS

  - New reporting based on one or more of the following:

    - Case 1: channel/interference measurement for new CSI reporting, considering aspects such as one or more of the following:

      - Reporting more accurate interference characteristics

      - Reduced CSI feedback overhead (e.g., reporting interference measurement only)

      - Enhanced CSI reporting such as WB/SB CQI

    - Case 2: other measurement (other than channel/interference) for additional information

      - E.g., PDCCH/PDSCH decoding, recommended HARQ RV sequence, etc.

    - It targets to help gNB scheduler for better link adaptation of (re)transmission

  - [Reduced CSI computation time/complexity]

  - [CSI feedback for PDCCH]

  - Other CSI enhancement schemes that enable accurate MCS selection are not precluded

- Detailed assumptions of the proposed CSI enhancement schemes should be provided by the proponent, such as

  - Reporting values

  - Triggering conditions for the reporting

  - Associated measurement resource

  - Uplink resource to be used for the reporting

  - How to use the reported information at the gNB scheduler

  - CSI-RS overhead and CSI reporting frequency

  - CSI reporting latency/timeline

  - Etc.

Agreements:

- Consider Table 1 as baseline assumption for system level simulation for evaluating CSI enhancement schemes

  - The uses cases in Table 1 is for simulation purposes and it does not preclude a CSI enhancement scheme which is beneficial for the other URLLC use cases

- No baseline assumption is used for link level simulation

  - Companies are encouraged to use one of LLS assumption tables in Section A.3 in TR38.824 for any link level simulation

Table 1. Baseline SLS assumption for CSI enhancement schemes in URLLC/IIoT

| Parameters | Values |
| --- | --- |
| Performance metric | Option-1 (section 5.1 of TR 38.824) Additional metrics (it is up to company to bring results with additional metric): - MCS prediction error (e.g., difference of a scheduled MCS and an ideal MCS) - DL/UL signaling overhead - CCDF of latency samples from all UEs - BLER of 1st transmission - Resource utilization - Spectral efficiency |
| Use cases | Following two use cases can be considered for new triggering method and new reporting. Companies are encouraged to evaluate the following cases in descending priority: - Rel-15 enabled use case (e.g. AR/VR) in TR 38.824 - Reliability: 99.999 - Latency: 4ms (200bytes) - Traffic mode: FTP model 3 (100p/s) - Factory automation in TR 38.824 - Reliability: 99.9999 - Latency: 1ms (32bytes) - Traffic mode: Periodic deterministic traffic model with arrival interval 2ms - Rel-15 enabled use case (e.g. AR/VR) in TR 38.824 - Reliability: 99.999 - Latency: 1ms (32bytes) - Traffic mode: FTP model 3 (100p/s) - Assumptions for eMBB and URLLC UEs sharing the same carrier is used (as in A2.5 of TR 38.824) |
| Simulation assumptions | Following simulation assumption is used based on the use case selected: - Rel-15 enabled use case with UMa (Table A.2.4-1 in TR 38.824) - Factory automation at 4GHz (Table A.2.2-1 in TR38.824) with following update: - Channel model is replaced with InF (InF-DH) in TR 38.901 - Companies can bring results with other InF scenarios additionally - Layout is replaced with BS deployment in Table 7.8-7 in TR 38.901 |
| Transmission scheme | Multiple antenna ports Tx scheme - Companies report the details of Tx scheme used |