3GPP TSG-RAN WG1 Meeting #104bis-e	R1-2103799

e-Meeting, 12th – 20th April, 2021

Agenda Item:	8.6.1.2

Title:	FL summary #1 for reduced number of Rx branches for RedCap

Source:	Moderator (Apple)

Document for:	Discussion, Decision

# Introduction

This document summarizes the contributions [3] – [31] made under the “Aspects related to reduced number of Rx branches” agenda item of the Rel-17 work item on support of reduced capability NR devices [1].

Earlier RAN1 agreements for this work item are summarized in [2].

The revised Redcap WID [1] contains the following objectives related to this agenda item:

- Specify support for the following UE complexity reduction features [RAN1, RAN2, RAN4]: […] - Reduced minimum number of Rx branches: - For frequency bands where a legacy NR UE is required to be equipped with a minimum of 2 Rx antenna ports, the minimum number of Rx branches supported by specification for a RedCap UE is 1. The specification also supports 2 Rx branches for a RedCap UE in these bands. - For frequency bands where a legacy NR UE (other than 2-Rx vehicular UE) is required to be equipped with a minimum of 4 Rx antenna ports, the minimum number of Rx branches supported by specification for a RedCap UE is 1. The specification also supports 2 Rx branches for a RedCap UE in these bands. - A means shall be specified by which the gNB can know the number of Rx branches of the UE. - Maximum number of DL MIMO layers: - For a RedCap UE with 1 Rx branch, 1 DL MIMO layer is supported. - For a RedCap UE with 2 Rx branches, 2 DL MIMO layers are supported. […] - Specify a system information indication to indicate whether a RedCap UE can camp on the cell/frequency or not; it shall be possible for the indication to be specific to the number of Rx branches of the UE. [RAN2, RAN1]

Follow the naming convention in this example:

- RedCapRxFLS1-v000.docx

- RedCapRxFLS1-v001-CompanyA.docx

- RedCapRxFLS1-v002-CompanyA-CompanyB.docx

- RedCapRxFLS1-v003-CompanyB-CompanyC.docx

# Reporting of Number of Rx branches

RAN1#104e made the following agreements related to initial DL BWP:

Agreements: - For reduced minimum number of Rx branches in FR1 and FR2 frequency bands where a legacy NR UE is required to be equipped with a minimum of 2 Rx antenna ports: - FFS: need for solutions to reduced PDCCH blocking - FFS: need for reporting of UE antenna related information to gNB (e.g., # of panels, polarization, etc.) - Information related to the reduction of the number of antenna branches is assumed to be known at the gNB (either implicitly or explicitly, to be FFS)

In addition, a new objective was added in the revised WID [1] to specify a means by which the gNB can know the number of Rx branches of the Redcap UE. This issue was widely discussed in contributions [3][4][6][7][8][9] [10][11][12][13][14][15][16][18][19][21][23][24][27]

Table 1 summarized the proposed options to indicate the number of Rx branches of the Redcap UE:

Table 1: Options to indicate the number of Rx branches at least for FR1

| Index | Description | Companies | Motivations | Num. of Companies |
| --- | --- | --- | --- | --- |
| Opt.1 | Using UE capability report explicitly or implicitly | Huawei [3], Vivo [6], CATT [7], MediaTek [9], Futurewei [11], Intel [15], Apple [16], Sharp [23], Ericsson [10]/ Samsung [18]: (Using the capability parameter maxNumberMIMO-LayersPDSCH) | - Msg2/Msg4 coverage is not an essential issue for 1 Rx/2 Rx with TB scaling [3][6][7]. - Coverage for 1Rx/2Rx for wearable maybe simliar [3]. - No DL coverage enhancements are agreed for RedCap with minimum 1 Rx branch. For initial access, conservative scheduling, power boosting, TB scaling and/or HARQ-based retransmission for Msg.4 are all available to enhance the DL channel performance [6]. - The cost for supporting early identification during Msg1 transmission is in terms of increased PRACH OH in the cell due to partitioning of ROs, RACH resources, [15] - To reduce reporting overhead and to avoid unnecessary specification work [10] [16] | 10 |
| Opt.2 | Using Msg1 and/or Msg3, and MsgA | OPPO [4], ZTE [12] Nordic Semiconductor ASA [27] | - Improve the performane of Msg2/4 [4][12]. - If the number of Rx branches is not part of this early indication, the network would have to assume the UE has 1 Rx branch. In addition, the network would not be able reject connection from 1Rx UE during initial access [8]. - Support load balancing of RACH resources between REDCAP and non-REDCAP devices [8]. | 3 |
| Opt.3 | Configuration between Opt.1 and Opt.2 via SIB1 | CMCC [13], LGe [19], Nokia [8] (optionally configured) | - The need of differentiate 1 Rx/2 Rx UE maybe frequency and deployment scenario dependent [8] | 3 |

Based on the Table 1 above, clearly Opt.1 is the preferred approach by major companies. Hence, Question 1 was asked to address this open issue:

Question 2-1: Can we agree the following proposal? If not, which modifications are needed?

- Using UE capability report (i.e., Opt.1) as baseline to indicate the number of Rx branches?

  - FFS: Using earlier indication by Msg1 and/or Msg3, and MsgA (Opt.2)

  - FFS: The need of selection by SIB1 between earlier indication and UE capability report (Opt.3)

| Company | Y/N | Comments |
| --- | --- | --- |

# Potential PDCCH Enhancement

Reducing the number of Rx branches degrades the link performance and coverage. Therefore, for a given PDCCH BLER-performance target, higher ALs may be needed for RedCap UEs to compensate for the coverage loss. Generally, the PDCCH blocking rate increases when higher ALs are used. Hence, reducing the number of Rx branches may result in a higher PDCCH blocking rate. In general, the impact on PDCCH blocking performance from RedCap UEs would depend on various factors such as the number of UEs which need to be scheduled (may depend on the traffic), CORESET size (i.e., number of CCEs), number of PDCCH candidates, and PDCCH link performance/coverage (which affects the required aggregation level, AL), and relative fraction of RedCap UEs with reduced capability on number of Rx branches.

In contributions [3] [4] [5] [6] [7] [8] [10] [11] [12] [13] [14] [15] [16] [17] [18] [20] [23] [25] [26] [28], views on the necessity of PDCCH enhancement have been presented to enhance the PDCCH blocking rate. A few alternatives were proposed as listed in Table 2, mainly motivated by the use case where has a relatively larger fraction of UEs that are RedCap UEs with reduced capability for number of Rx branches or PDCCH capacity shortage due to the reduced BW and demanding CCE ALs for Redcap devices.

On the other hand, athough SNR gap could be as large as 5~6 dB, e.g., between a 1Rx RedCap UE and a 4Rx non-RedCap UE, whether the overall PDCCH user blocking performance is impacted would be a function of the deployment and relative number for such RedCap UEs within all UEs in the cell. It was observed in [10] [8] that the number of simultaneously scheduled UEs is expected to be between 1 and 5 and the impact of reducing the number of Rx branches on PDCCH blocking probability is small. Hence, no solution of reducing PDCCH blocking rate enhance was proposed in [4] [6] [7] [8] [10] [15] [18] and [25].

Table 2: View on PDCCH enhancement for Redcap

| Index | Description | Companies | # of Companies |
| --- | --- | --- | --- |
| Alt.1 | Reuse the existing DCI format, including Rel-16 DCI format 0_2/1_2 | OPPO [4], Vivo [6], CATT [7], Nokia [8], Ericsson [10], Intel [15], Samsung [18], Panasonic [25] | 8 |
| Alt.2 | Introducing new Compact DCI(s) | Huawei [3], Futurewei [11], Qualcomm [17], Samsung [18], CEWiT [20], | 5 |
| Alt.3 | Introducing a group-wise DCI that can be used to schedule multiple UEs. | Huawei [3], CMCC [13], CEWiT [20], | 3 |
| Alt.4 | Support PDCCH link adaptation (e.g., RS resource for CSI measurement associated with CORESET) | Samsung [18] | 1 |
| Alt.5 | Multi-TB scheduling | Samsung [18], Intel [15], CEWiT [20], | 3 |
| Alt.6 | Configuring seperate CORESETs or Initial DL BWP for Redcap UEs | Spreadtrum [5], ZTE [12], Intel [15], ASUSTeK [28], Sharp [23] | 5 |
| Alt.7 | Joint optimization of RV cycling order and number of repetitions | Qualcomm [17], | 1 |
| Alt.8 | SPS-based and CG-based transmission in RRC connected state | Qualcomm [17], | 1 |
| Alt.9 | RACH-based or CG-based SDT in RRC inactive state | Qualcomm [17] | 1 |

Question 3-1: Which alterative(s) among these listed in Table 1 are preferred and Why? Please share your views including any further modification on the listed options.

- Note that the feedback is intended to be used to down select Alternatives (e.g., excluding alternative(s) that are only interested by one or two companies), such that in a next step during RAN1 104-bis e-meeting, RAN1 to focus on selected alternatives including establishing feasibility and identifying pros and cons to make progress:

| Company | Comments/Reasoning |
| --- | --- |

# MCS and CQI Tables

RAN1#104e made the following agreements related to MCS and CQI table for Redcap devices:

Agreements: - The MCS tables currently defined are re-used for RedCap UEs - FFS which MCS table is the default one for RedCap (i.e., the default one for non-RedCap UEs or the one with low SE entries) - FFS mandatory/optional of the MCS tables - Note: there is no new MCS table to be introduced for RedCap UEs Agreements: - The CQI tables currently defined are re-used for RedCap UEs. - FFS mandatory/optional of the CQI tables - There is no new CQI table to be introduced for RedCap UEs

Figure 1: MCS Tables

In contribution [5] [8] [11] [18], views on MCS and CQI tables support for Redcap devices have been presented. Companies’ positions were summarized in Table 3:

Table 3: View on MCS table support

| Index | Description | Companies | # of Companies |
| --- | --- | --- | --- |
| Opt.1 | Low-SE MCS table is mandatory for Redcap UE | Spreadtrum [5], Futurewei [11] (at least for 1 Rx capable UE), | 2 |
| Opt.2 | Keep same as normal UE (i.e., MCS table 1 is the default Table and MCS Table 3 is optionally supported.) | Nokia [8], Samsung [18] | 2 |

In [11], differences between low-SE MCS table (i.e., MCS Table 3) and MCS table 1 were analyzed. It was observed that there are six more indices available below 0.12 (lowest code rate of the normal MCS table). The ratio between the lowest code rates in each table is 4. While TB scaling and the lower SE table each provide a maximum factor of 4 decrease in code rate, the lower SE table provides four additional levels; allowing the network more control to fine tune the coding rate. In addition, TB scaling and the lower SE table can be coupled; providing up to a factor of 16 decrease in code rate. This benefit in network flexibility was further motivates to make the lower SE table the default MCS table for RedCap UEs to address performance for initial access. It also pointed out in [11] that for certain network configurations, a RedCap UE and legacy UE can be scheduled with the same DCI for Msg2. As one consequence, a RedCap UE is required to translate the signaled MCS index to an index to the lower SE table.

On the other hand, contribution [8] emphasized that from the coverage results in TR 38.875, it is seen that MCS Table 1 is sufficient. In addition, during initial access, legacy UEs would only use MCS Table 1. Therefore, if MCS Table 3 is the default table for RedCap UE, it would be necessary to differentiate RedCap UE starting from Msg1. This is against the WI objective that RedCap UE can be identified in Msg1 and/or Msg3, and Msg A if supported, including the ability for the early indication to be configurable by the network. In [8], it was additionally acknowledged that for some services (e.g., industrial sensor requires commercial service availability of 99.99% and video surveillance requires reliability of up to 99.9%), it would be beneficial to use MCS Table 3 to improve coverage. Therefore, support for MCS Table 3 would be beneficial for some RedCap use cases or UEs. However, there is no need to make Table 3 support mandatory for all RedCap UEs.

Question 4-1: Which one between Opt.1 and opt.2 in Table 3 is preferred? If none of them, please describe the preferred option in ‘comment’ column. As usual, please provide brief justification for your preference.

- Opt.1: Low-SE MCS table is mandatory for Redcap UE

- Opt.2: Keep same as normal UE (i.e., MCS table 1 is the default Table and MCS Table 3 is optionally supported.)

| Company | Preferred Option | Comments |
| --- | --- | --- |

In NR, there are 3 CQI tables defined with one-to-one correspondence to the configured MCS table. Similarly, Companies were invited to provide inputs regarding the CQI table support for Redcap UEs in the following question.

Question 4-2: Which option is preferred regarding CQI table support for Redcap devices? If none of them, please describe the preferred option in ‘comment’ column. As usual, please provide brief justification for your preference.

- Opt.1: CQI table corresponding to low-SE MCS table is mandatory for Redcap UE

- Opt.2: Keep the same as normal UE (i.e., CQI table one corresponding to MCS table 1 is the default Table and CQI table one corresponding to MCS Table 3 is optionally supported.)

| Company | Preferred Option | Comments |
| --- | --- | --- |

# Need of DL Coverage Recovery

Based on the revised WID [1], the minimum number of Rx branches is 1 for Redcap device on all of the FR1 bands. In TR 38.875, the observation of coverage evaluation can be summarized in Table below [29].

- DL coverage recovery for RedCap UE is needed for FR1 only - For RedCap UE with 1 Rx branch and reduced antenna efficiency, the need for coverage recovery depends on the frequency bands and DL PSD: - For carrier frequency of 4 GHz with DL PSD 24 dBm/MHz, coverage recovery may be needed for the downlink channels of Msg2, Msg4 and PDCCH CSS. A small or moderate compensation can be considered, where the square brackets indicate that the exact amount will depend on the techniques, scenarios, etc.: - [1 dB] for PDCCH CSS - [2-3 dB] for Msg4 - [6 dB] for Msg2 without TBS scaling. It is noted that coverage loss for Msg2 can be compensated by using the existing TBS scaling technique. - For other carrier frequencies or DL PSD of 33 dBm/MHz, coverage recovery is not needed for the downlink channels if the target for coverage recovery is based on the MIL of the bottleneck channel for the reference NR UE. - For RedCap UE with 2 Rx branches and reduced antenna efficiency, the need for coverage recovery also depends on the frequency bands and DL PSD: - For carrier frequency of 4 GHz with DL PSD 24 dBm/MHz, coverage recovery may be needed for the downlink channels of Msg2. A small or moderate compensation can be considered, where the square brackets indicate that the exact amount will depend on the techniques, scenarios, etc.: - [1 dB] for Msg2 without TBS scaling. It is noted that coverage loss for Msg2 can be compensated by using the existing TBS scaling technique. - For other carrier frequencies or DL PSD of 33 dBm/MHz, coverage recovery is not needed for the downlink channels if the target for coverage recovery is based on the MIL of the bottleneck channel for the reference NR UE.

This was further summarized in Table below [11]

Table 4: Coverage recovery observations in [29]

|  | 2 Rx branches |  | 1 Rx branch |  |
| --- | --- | --- | --- | --- |
| Channel | 4GHz, and DL PSD 24 dBm/MHz | not at 4GHz or using DL PSD 33 dBm/MHz | 4GHz, and DL PSD 24 dBm/MHz | not at 4GHz or using DL PSD 33 dBm/MHz |
| PDCCH |  |  | [1 dB] needed |  |
| PDSCH | [1 dB] needed |  | [2-3 dB] Msg4, [6 dB] Msg2 |  |
| PUSCH | [~3 dB] needed | [~3 dB] needed | [~3 dB] needed | [~3 dB] needed |

A few contributions [11] [17] [21] [22] [24] [26] [28 discussed the necessity of DL coverage enhancements especially for 1 Rx capable Redcap devices with the following proposals:

- One contribution [11] proposed to make two optional UE features to be mandatory to improve the PDSCH coverage especially in initial access phase, one is Low-SE MCS table and the other is PDSCH repetition feature.

- In [17], it was observed that a sufficient number of solutions exist commonly for Redcap and non-Redcap UEs, including TB scaling for msg2 PDSCH or msgB PDSCH, Low MCS, PDSCH repetition, Power boosting of gNB, VRB-to-PRB mapping, large AL for PDCCH. DL coverage recovery can be triggered by earlier identification.

- One contribution [21] proposed to investigate the msg2, msg4, and PDCCH coverage enhancement. BLER results for Msg2 with and without TBS scaling were provided in [21] and it was observed that TBS scaling only may not be sufficient to compensate the observed 6 dB coverage gap.

- In [22], it was proposed that RAN1 to confirm the following:

  - Whether or not DL PSD = 24dBm/MHz should be considered in normative work.

  - Confirm if antenna loss up to 3dB in both DL and UL due to compact form factor should be considered and if DL coverage recovery for Msg2 and PDCCH CSS and UL coverage recovery for Msg3/PUSCH should be supported by the spec.

  - Discuss whether to adopt different level of coverage recovery (including no coverage recovery) for 1Rx and 2Rx RedCap UE, if DL coverage recovery is supported

- One contribution [26] proposed to clarify whether DL coverage recovery is in the scope of RedCap WI.

- One contribution [28] proposed to support repetition of CORESET#0/CommonCORESET in RB-sets of a single wide carrier/BWP was proposed.

Question 5-1:  Whether or not DL coverage enhancement should be considered based on current WID scope?  If yes, please also explain the justification and list the preferred solution in Comment column.

| Company | Yes/No | Comments |
| --- | --- | --- |

# Access Control for Redcap

In the updated WID, one objective was added to allow system information indicating whether a RedCap UE can camp on the cell/frequency, and the indication can specific to 1Rx or 2Rx.

- Specify a system information indication to indicate whether a RedCap UE can camp on the cell/frequency or not; it shall be possible for the indication to be specific to the number of Rx branches of the UE. [RAN2, RAN1]

Contribution [3][13][19] discussed aspects realizing this objective of access control for Redcap UEs. In [3], different mechanisms were mentioned for the network to indicate whether the network allows the UE’s access, including indications in MIB or SIB1 or load balancing mechanism during the initial access procedure, or during paging procedure, use of 2 more spare bits in PBCH payload or using sparse bits in DCI that schedules SIB1. In [19], different alternatives were discussed depending on the bits number of signalling for access control, such as Bar RedCap UEs (regardless of the number of Rx branches), 1-Rx only, or 1-Rx only for bands requiring 4 Rx branches. Using sparse bits in DCI that schedules SIB1 is preferred by [3] [19] due to potential power saving benefit. It was proposed in [19] that access control signalling in SIB should provide the flexibility to indicate per band whether a RedCap UE with specific number of Rx branches of the UE can camp on the cell/frequency. In [13], it proposed that System information can indicate the conditions that RedCap UEs or RedCap UEs with specific number of Rx branches are allowed to camp on the cell/frequency.

On the other hand, it should be noted that the issue of access control is being discussed in RAN WG2 as leading work group on this objective. To avoid duplicate efforts and considering the fact that the objective is tasked to RAN2 as leading WG, it may be prudent to wait for RAN WG2 to make further progress first. In addition, the access and earlier identification were typically managed to be handled in dedicated Redcap agendas.

Conclusion 6-1: On the issue of access control for Redcap UEs, RAN1 waits for RAN2 further progress and continue discussion in other Redcap agendas starting from RAN1 105 meeting.

- If ’no’, please kindly explain which aspects you think RAN1 need to work on in this meeting and why?

| Company | Y/N | Comments |
| --- | --- | --- |

# Earlier Identification of Redcap Devices

The revised WID lists the following objectives

- Specify functionality that will enable RedCap UEs to be explicitly identifiable to networks through an early indication in Msg1 and/or Msg3, and Msg A if supported, including the ability for the early indication to be configurable by the network. [RAN2, RAN1]

In contributions [3] [4] [6] [15] [19] [24] [27], views on redcap device type defintion and realizing the earlier identification of RedCap UEs have been presented, as summarized in Table below:

Table

| Companies | Views |
| --- | --- |
| Huawei [3] | The one RedCap UE type has only maximum UE channel bandwidth including in the minimized set of basic capabilities |
| OPPO [4] | Using Msg1 and FFS on Msg3 |
| Intel [15] | Configurable between Msg1/Msg3 |
| LGe [19] | - Separate RO in time/frequency for Redcap devices, - Configurable by SIB1 on Msg1 or Msg3 or both and even inclusion of number of Rx branches |
| Nordic Semiconductor ASA [27] | - Using Msg1. - Support replicating Ros to multiple RB-sets based on configuration within one RB set. - Using Msg3 to indicate number of Rx branches if both 1 Rx and 2 Rx Redcap devices are allowed by network. |

Similar as ‘Access control’ topic, the ‘earlier identification’ was handled in another dedicated agenda (i.e., AI 8.6.2), which was void in this meeting. In addition, the issue of earlier identification control is being discussed in RAN WG2 as leading work group on this objective.

Conclusion 7-1: No further discussion on ‘earlier identification’ of Redcap device in RAN1 104 bis e-meeting.

- If the answer is ’no’, please kindly explain which aspects you think RAN1 need to discuss in this meeting and why?

| Company | Y/N | Comments |
| --- | --- | --- |

# Other aspects

Cell selection

- P1: One contribution proposed to introduce specific RSRP thresholds for Redcap device which are configured by gNB for SSB and UL carrier selection for performing random access

DCI size reduction

- P2:  One contribution [16] proposed to consider capturing in physical specification TS 38.212 that Redcap UE always assumes MCS/NDI/RV of TB2 is not presence to avoid the need of RRC signaling of ‘maxNrofCodeWordsScheduledByDCI’.

Access barring

- P3: It was proposed in [24], that barring of RedCap UEs could be based on DL channel status, instead of simply based on number of Rx branches

Question 8-1: Which of the proposals in the list above (P1, P2, P3) are important and need to be discussed in the RAN1 104-bis e-meeting to make progress? What other aspects/proposal need to be added for discussion?

| Company | Comments |
| --- | --- |

# References

RP-210918 	“Revised WID on support of reduced capability NR devices ”, RAN#91e, Nokia, Ericsson.

R1-2102220	RAN1 agreements for Rel-17 NR RedCap 		Rapporteur (Ericsson)

R1-2102355	Discussion on reduced number of Rx branches for RedCap	Huawei, HiSilicon

R1-2102403	Discussion on reduced number of UE Rx branches	OPPO

R1-2102461	Discussion on aspects related to reduced number of Rx branches	Spreadtrum Communications

R1-2102530	Discussion on reduced number of Rx branches	vivo, Guangdong Genius

R1-2102639	Discussion on reduced number of Rx branches	CATT

R1-2102650	UE complexity reduction aspects related to reduced number of Rx branches	Nokia, Nokia Shanghai Bell

R1-2102700	On reduced number of Rx branches for RedCap UEs	MediaTek Inc.

R1-2102723	Reduced number of Rx branches for RedCap	Ericsson

R1-2102779	RX branch reduction for RedCap UEs	FUTUREWEI

R1-2102855	Discussion on reduced number of UE Rx branches	ZTE

R1-2102890	Discussion on reduced number of Rx branches	CMCC

R1-2102989	Aspects on reduced number of Rx branches	Xiaomi

R1-2103039	On reduced number of Rx branches for RedCap devices	Intel Corporation

R1-2103113	On reduced number of Rx branches for Redcap	Apple

R1-2103175	RX Branch Reduction for RedCap UE	Qualcomm Incorporated

R1-2103247	Discussion on reduced number of RX branches for RedCap UEs	Samsung

R1-2103353	Aspects related to the reduced number of Rx branches of RedCap	LG Electronics

R1-2103404	Discussion on solutions for reducing PDCCH blocking	CEWiT

R1-2103422	Reduced number of Rx branches for RedCap UEs	InterDigital, Inc.

R1-2103456	Discussion on aspects of coverage recovery	NEC

R1-2103477	Discussion on reduced minimum number of Rx branches	Sharp

R1-2103535	Reduced number or Rx branches for RedCap	Lenovo, Motorola Mobility

R1-2103541	Aspects related to reduced number of Rx branches	Panasonic Corporation

R1-2103584	Discussion on reduced minimum number of Rx branches for RedCap	NTT DOCOMO, INC.

R1-2103651	On aspects related to reduced number of Rx branches	Nordic Semiconductor ASA

R1-2103665	Discussion on aspects related to reduced number of Rx branches	ASUSTeK

TR 38.875 	    Study on Support of Reduced Capability NR Devices,  December 2020.