3GPP TSG-RAN WG1 #104b-e	R1-2103838

e-Meeting, April 12th – 20th, 2021

Agenda Item:	8.10.1

Source:	Moderator (AT&T)

Title:	Summary of [104b-e-NR-eIAB-01] – 1st Checkpoint

Document for:	Discussion/Approval

# Introduction

This contribution provides a summary of the discussion in RAN1#104bis-e for the following email discussion:

/This one is to use NWM – please use RAN1-104b-e-NWM-NR-eIAB-01 as the document name

[104b-e-NR-eIAB-01] Email discussion on enhancements to resource multiplexing between child and parent links of an IAB node – Thomas (AT&T)

- 1st check point: April 15

- 2nd check point: April 20

# Simultaneous Operation of Access and Backhaul Links

From the eIAB WID:

- Specification of enhancements to the resource multiplexing between child and parent links of an IAB node, including:

  - Support of simultaneous operation (transmission and/or reception) of IAB-node’s child and parent links (i.e., MT Tx/DU Tx, MT Tx/DU Rx, MT Rx/DU Tx, MT Rx/DU Rx)

  - Support for dual-connectivity scenarios defined by RAN2/RAN3 in the context of topology redundancy for improved robustness and load balancing.

ISSUE 2.1: FREQUENCY DOMAIN MULTIPLEXING RESOURCE GRANULARITY

Agreement

The extension of the semi-static DU resource type indication to frequency-domain resources within a carrier (in addition to existing Rel-16 per-carrier granularity) for H/S/NA resource types is supported

Proposal 2.1.3

In case of FDM within a carrier, the frequency-domain granularity is configurable

- FFS: resource size e.g. N PRBs/N PRGs

- FFS: Separate or joint TDM and FDM semi-static DU resource configurations

Proposal 2.1.4

Soft resource availability indications for frequency-domain resources are supported

- FFS enhancements to DCI Format 2_5

ISSUE 2.2: SPATIAL DOMAIN MULTIPLEXING RESOURCE GRANULARITY

Proposal 2.2.2 Signalling indications/restrictions of supported beams for a given child/parent backhaul link is supported

- The existing beam management framework is taken as the baseline for necessary IAB-specific enhancements

- FFS: Impact on semi-static resource configuration (e.g. H/S/NA resource attributes)

- FFS: whether panel-based granularity is additionally supported

ISSUE 2.3: MULTIPLEXING CAPABILITY INDICATION ENHANCEMENTS

Proposal 2.3.2.

Dynamic switching of an IAB-node’s multiplexing-capability is supported. The switching may be based on multiple factors, including:

- Resource type (D/U/F) at the IAB-DU and IAB-MT

- Specific sets of time/frequency resources

- Certain conditions being met (e.g. supported timing modes, power control enhancements (if supported), etc.)

FFS:  Mechanisms for informing/coordination the change in multiplexing capability(ies) between child and parent nodes

FFS: Need for explicit linkage between indicated multiplexing-capabilities and other features/enhancements – e.g. number of required guard symbols, supported timing modes, and power control enhancements (if supported)

# Resource allocation for dual-connectivity scenarios (i.e. IAB-MT with concurrent BH links with two parent nodes)

From the eIAB WID:

- Specification of enhancements to the resource multiplexing between child and parent links of an IAB node, including:

  - Support of simultaneous operation (transmission and/or reception) of IAB-node’s child and parent links (i.e., MT Tx/DU Tx, MT Tx/DU Rx, MT Rx/DU Tx, MT Rx/DU Rx)

  - Support for dual-connectivity scenarios defined by RAN2/RAN3 in the context of topology redundancy for improved robustness and load balancing.

ISSUE 3.1: INTRA-BAND, INTER-CARRIER DC SUPPORT

Proposal 3.1.2:  The following enhancements to support intra-band inter-carrier dual connectivity for both inter-donor and intra-donor scenarios are considered (in addition to reusing solutions for inter-band dual connectivity):

- Extending the Rel-16 CA TDD confliction resolution framework for at least synchronous intra-band NR-DC operation

- Exchanging TDD configurations for the parent nodes (intra-donor and inter-donor) and exchanging/coordinating H/S/NA configurations for the child node (at least for inter-donor operation)

- FFS: Need to support either simultaneous Tx or simultaneous Rx at the child IAB-MT to/from both parent links

ISSUE 3.2: MULTI-PARENT SOFT RESOURCE AVAILABILITY INDICATION

Proposal 3.2.2: Explicit conflict resolution rules in case of reception of DCI Format 2_5 from multiple parents/cells are supported in Rel-17.

- FFS: Enhancements to support coordination between parent nodes (e.g. indication of soft resource availability from child node to parent node(s))

ISSUE 3.3: RESOURCE COORDINATION

Proposal 3.3.2: In case of DC operation, per-backhaul link (e.g. per child IAB-MT link) resource configurations at the parent nodes are supported in addition to per-DU resource configurations

# Summary

TBD