# 3GPP TSG RAN WG1 #104b-e																	R1- 2103910

# e-Meeting, April 12th – 20th, 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.212 | CR | 0364 | rev |  | Current version: | 15.13.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for DCI size alignment in NBIOT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated, ZTE, Sanechips, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOTenh-Core |  |  |  |  |  | Date: |  |  | 2021-04-06 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | DCI size alignment shall only be performed among DCI in the same search space. This issue happens in Rel-14, when the UE may use a different DCI size for random access procedure in connected mode (Type-2 Common Search Space) and unicast transmission (UE-specific Search Space by C-RNTI). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that DCI size alignment only occurs among DCIs in the same search space |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE may perform DCI size alignment across Type-2 and UE-specific search spaces, leading to errors. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 6.4.3.2	DCI Format N1

DCI format N1 is used for the scheduling of one NPDSCH codeword in one cell, random access procedure initiated by a NPDCCH order, and notifying SC-MCCH change. The DCI corresponding to a NPDCCH order is carried by NPDCCH.

The following information is transmitted by means of the DCI format N1:

-	If the format N1 CRC is scrambled by C-RNTI or RA-RNTI:

-	Flag for format N0/format N1 differentiation – 1 bit, where value 0 indicates format N0 and value 1 indicates format N1

-	NPDCCH order indicator – 1 bit

-	Else if the format N1 CRC is scrambled by a G-RNTI:

-	Information for SC-MCCH change notification – 2 bits as defined in clause 5.8a of [6]

Format N1 is used for random access procedure initiated by a NPDCCH order only if NPDCCH order indicator is set to '1', format N1 CRC is scrambled with C-RNTI, and all the remaining fields are set as follows:

-	Preamble format indicator – 1 bit, where value 0 indicates preamble format 0/1 and value 1 indicates preamble format 2. This field is only present if nprach-ParametersListFmt2 is configured and the UE indicates the nprach-Format2 capability.

-	Starting number of NPRACH repetitions – 2 bits as defined in clause 16.3.2 of [3]

-	Subcarrier indication of NPRACH – 6 or 8 bits, this field is 8 bits only if Preamble format indicator is present and set to 1, as defined in clause 16.3.2 of [3]

-	Carrier indication of NPRACH – 4 bits as defined in clause 16.3.2 of [3]. This field is only present if ul-ConfigList is configured and the UE indicates the multiCarrier-NPRACH capability.

-	All the remaining bits in format N1 are set to one

Otherwise,

-	Scheduling delay – 3 bits as defined in clause 16.4.1 of [3]

-	Resource assignment – 3 bits as defined in clause 16.4.1.3 of [3]

-	Modulation and coding scheme – 4 bits as defined in clause 16.4.1.5 of [3]

-	Repetition number – 4 bits as defined in clause 16.4.1.3 of [3]

-	New data indicator – 1 bit

-	HARQ-ACK resource – 4 bits as defined in clause 16.4.2 of [3].

-	DCI subframe repetition number – 2 bits as defined in clause 16.6 in [3]

-	HARQ process number – 1 bit. This field can only be present if 2 HARQ processes are configured and the corresponding DCI format is mapped onto the UE specific search space given by the C-RNTI as defined in [3].

When the format N1 CRC is scrambled with a RA-RNTI or a G-RNTI, then the following fields among the fields above are reserved for RA-RNTI and not present for G-RNTI:

-	New data indicator

-	HARQ-ACK resource

If the number of information bits in format N1 mapped onto the same search space is less than that of format N0 and the format N1 CRC is not scrambled by G-RNTI, zeros shall be appended to format N1 until the payload size equals that of format N0.