3GPP TSG RAN WG1 Meeting #104b-e	R1-2103954

E-meeting, April 12th – April 20th, 2021

Agenda Item:	8.9.1

Source:	Moderator (Huawei)

Title:	Feature lead summary #2 on 104b-e-LTE-Rel17_NB_IoT_eMTC-01

Document for:	Discussion and Decision

Introduction

The WID for Rel-17 enhancements for NB-IoT and LTE-MTC [1] includes an objective to support 16-QAM for unicast in UL and DL in NB-IoT.

- Specify 16-QAM for unicast in UL and DL, including necessary changes to DL power allocation for NPDSCH and DL TBS. This is to be specified without a new NB-IoT UE category. For DL, increase in maximum TBS of e.g. 2x the Rel-16 maximum, and soft buffer size will be specified by modifying at least existing Category NB2. For UL, the maximum TBS is not increased. [NB-IoT] [RAN1, RAN4]

  - Extend the NB-IoT channel quality reporting based on the framework of Rel-14—16, to support 16-QAM in DL. [NB-IoT] [RAN2, RAN1, RAN4]

This documents provides the proposals and summary of discussions of the following email discussion according to the inputs [2-8]

[104b-e-LTE-Rel17_NB_IoT_eMTC-01] Email discussion on support of 16-QAM for unicast in UL and DL for NB-IoT – Yubo (Huawei)

- 1st check point: April 15

- 2nd check point: April 20

Issues

Support of 16-QAM for NB-IoT downlink

Issue 1: The TBS table.

The following are proposed:

| Sourcing | Proposals |
| --- | --- |
| [2] | Proposal 1: Confirm the following working assumption The following TBS indices are introduced for downlink 0; 1; 2; 3; 4; 5; 6; 7. 14; 256; 552; 840; 1128; 1416; 1736; 2280; 2856. 15; 280; 600; 904; 1224; 1544; 1800; 2472; 3112. 16; 328; 632; 968; 1288; 1608; 1928; 2600; 3240. 17; 336; 696; 1064; 1416; 1800; 2152; 2856; 3624. 18; 376; 776; 1160; 1544; 1992; 2344; 3112; 4008. 19; 408; 840; 1288; 1736; 2152; 2600; 3496; 4264. 20; 440; 904; 1384; 1864; 2344; 2792; 3752; 4584. 21; 488; 1000; 1480; 1992; 2472; 2984; 4008; 4968. |
| [3] | Proposal 1: Confirm the working assumption on downlink TBS table from RAN1#104-e. |
| [5] | Proposal 1: Confirm the working assumption for DL TBS table for NB-IoT 16QAM. - 328 bits for {I_TBS=16, I_SF=0} - 2472 bits for { I_TBS=21, I_SF=4} |
| [6] | Proposal 1: Confirm the working assumption for the DL TBS table from RAN1#104-e. |
| [8] | - Observation 1 For ITBS = 16, ISF = 0 → [328, 296]: If a TBS = 328 bits is selected, then a performance crossing issue is observed with respect to the TBS = 336 bits. - Observation 2 Apparently, when there are coding rates that are nearly close to each other (TBS = 328 bits → 0.58 , TBS = 336 bits → 0.59), the “soft-demapper module” and Viterbi decoding algorithms sometimes produce a non-linear response that produces an unusual performance. - Observation 3 The performance crossing issue is not observed when instead of a TBS = 328 bits a TBS = 296 bits (code rate → 0.53) is utilized. - Observation 4 16-QAM in principle makes possible to double the throughput with respect to QPSK. However, for 16-QAM in DL the largest TBS to be used has been agreed to be 4968 bits which does not allow to double throughput since is not exactly twice the max TBS available in Rel-16. - Observation 5 If in DL the introduction of 16-QAM is meant to double the throughput, then the max TBS available in Rel-16 can be transmitted using half of the time-domain resource by selecting 2536 among the two choices for ITBS = 21, ISF = 4 → [2472, 2536], otherwise only 16-QAM in UL will be able to effectively double the throughput. - Observation 6 A TBS = 2536 bits can be used for ITBS = 21, ISF = 4 without incurring in any performance issue. - Proposal 1 Confirm the Working Assumption referring to the TBS indices to be introduced for downlink, including the following resolution on the TBS entries surrounded by brackets: - To avoid a performance crossing issue, between [328, 296] the TBS = 296 bits is selected. - To transmit the max Rel-16 TBS with half of the time domain resources, between [2472, 2536] the TBS = 2536 bits is selected. Note: The TBS/MCS Table for UL accounts for this exact change, which is an ability that DL should also have. |

For working assumption on DL TBS, there are following proposals:

- Confirm the working assumption

  - Huawei, HiSilicon, Nokia, NSB, ZTE, QC

- Confirm the working assumption with changes: TBS=296bits for I_TBS=16, I_SF=0; TBS=2536bits for I_TBS=21, I_SF=4

  - Ericsson

Based on the inputs, the following is proposed:

Proposal 1: Confirm the working assumption that the following TBS indices are introduced for downlink:

|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 14 | 256 | 552 | 840 | 1128 | 1416 | 1736 | 2280 | 2856 |
| 15 | 280 | 600 | 904 | 1224 | 1544 | 1800 | 2472 | 3112 |
| 16 | 328 | 632 | 968 | 1288 | 1608 | 1928 | 2600 | 3240 |
| 17 | 336 | 696 | 1064 | 1416 | 1800 | 2152 | 2856 | 3624 |
| 18 | 376 | 776 | 1160 | 1544 | 1992 | 2344 | 3112 | 4008 |
| 19 | 408 | 840 | 1288 | 1736 | 2152 | 2600 | 3496 | 4264 |
| 20 | 440 | 904 | 1384 | 1864 | 2344 | 2792 | 3752 | 4584 |
| 21 | 488 | 1000 | 1480 | 1992 | 2472 | 2984 | 4008 | 4968 |

Please input your comments for the above proposal:

| Companies | Comments |
| --- | --- |
| Ericsson | We support the WA with the following updates: TBS=296bits for I_TBS=16, I_SF=0; TBS=2536bits for I_TBS=21, I_SF=4. |
| Huawei, HiSilicon | We agree with proposal 1. |
| MTK | We are ok with proposal1. |

The following agreement has been achieved:

Agreement

Confirm the working assumption that the following TBS indices are introduced for downlink with modification in RED:

|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 14 | 256 | 552 | 840 | 1128 | 1416 | 1736 | 2280 | 2856 |
| 15 | 280 | 600 | 904 | 1224 | 1544 | 1800 | 2472 | 3112 |
| 16 | 328296 | 632 | 968 | 1288 | 1608 | 1928 | 2600 | 3240 |
| 17 | 336 | 696 | 1064 | 1416 | 1800 | 2152 | 2856 | 3624 |
| 18 | 376 | 776 | 1160 | 1544 | 1992 | 2344 | 3112 | 4008 |
| 19 | 408 | 840 | 1288 | 1736 | 2152 | 2600 | 3496 | 4264 |
| 20 | 440 | 904 | 1384 | 1864 | 2344 | 2792 | 3752 | 4584 |
| 21 | 488 | 1000 | 1480 | 1992 | 2472 | 2984 | 4008 | 4968 |

Issue 2: The breaking point from QPSK to 16QAM.

The following are proposed:

| Sourcing | Proposals |
| --- | --- |
| [2] | Proposal 2: Confirm the following working assumption - For standalone and guardband deployments, the downlink TBS entries between 14 (TBS of 2856 for I_SF=7) and 21 are used for 16-QAM. |
| [3] | Proposal 2: Confirm the following working assumptions from RAN1#103-e – - For standalone and guardband deployments, the downlink TBS entries between 14 (TBS of 2856 for I_SF=7) and 21 are used for 16-QAM. |
| [4] | Proposal1：ITBS=14 and ITBS=11 are switching point for SB/GB and IB, respectively. |
| [5] | Observation 1: For NPDSCH with 1T1R antenna, 16QAM performance is about 1.8 dB better than QPSK performance at BLER =0.1 for standalone deployment when TBS 12 (2280 bits with ISF=7) is used. And the performance gap further expands when TBS 13 is used. Observation 2: For NPDSCH with 2T1R antenna, 16QAM performance is about 0.6 dB better than QPSK performance at BLER =0.1 for standalone deployment when TBS 13 (2536 bits with ISF=7) is used. Proposal 2: On DL breaking point, 16QAM can be used for TBS indexes from 12 to 21 for standalone and guard-band deployment. |
| [8] | - Observation 7 For the evaluations in which 16-QAM and QPSK shared the same row (i.e., ITBS =13), 16-QAM did not outperform QPSK according with the average required SINR at 10% BLER. The same trend was observed for AWGN, ETU1, and ETU5. - Observation 8 For stand-alone and guard-band deployments, the breaking-point evaluations using AWGN, ETU1, and ETU5 channels resulted in the following observations: - Regardless of whether ITBS = 13 or ITBS = 14 is used as initial index for 16-QAM, in both cases the average SINR difference for the breaking point is less than 1dB and the same trend holds for all channels (AWGN, ETU1, and ETU5). - That is, the fading channels did not cause the average SINR difference for neither of the breaking points to go beyond 1dB. - Although using either of the breaking points seems to be suitable, the range of TBS indices spanning from “I_TBS = 14 to I_TBS = 21” is preferred because: - The range is composed by 8 indices (not 9 as the other range) which will also play a role when we begin to discuss DCI implementation. - The range of ITBS indices from “I_TBS = 14 to I_TBS = 21” will be symmetric with respect to the range already agreed for UL. - Proposal 2 The TBS/MCS Table for stand-alone and guard-band deployments uses as breaking point ITBS = 13 as last ITBS index for QPSK and ITBS = 14 as first ITBS index for 16-QAM. - Proposal 3 Confirm the Working Assumption referring to the use of 16-QAM for the TBS entries encompassed by ITBS indices between 14 and 21 for stand-alone and guard-band deployments. - Proposal 4 Upon resolving the FFS on the breaking-point and the TBS entries surrounded by brackets, the TBS/MCS Table to support 16-QAM in DL for stand-alone and guard-band deployments is as follows: Modulation Scheme; Number of NPDSCH Subframes (NSF). 1; 2; 3; 4; 5; 6; 8; 10. QPSK; 0; 16; 32; 56; 88; 120; 152; 208; 256. 1; 24; 56; 88; 144; 176; 208; 256; 344. 2; 32; 72; 144; 176; 208; 256; 328; 424. 3; 40; 104; 176; 208; 256; 328; 440; 568. 4; 56; 120; 208; 256; 328; 408; 552; 680. 5; 72; 144; 224; 328; 424; 504; 680; 872. 6; 88; 176; 256; 392; 504; 600; 808; 1032. 7; 104; 224; 328; 472; 584; 680; 968; 1224. 8; 120; 256; 392; 536; 680; 808; 1096; 1352. 9; 136; 296; 456; 616; 776; 936; 1256; 1544. 10; 144; 328; 504; 680; 872; 1032; 1384; 1736. 11; 176; 376; 584; 776; 1000; 1192; 1608; 2024. 12; 208; 440; 680; 904; 1128; 1352; 1800; 2280. 13; 224; 488; 744; 1032; 1256; 1544; 2024; 2536. 16-QAM; 14; 256; 552; 840; 1128; 1416; 1736; 2280; 2856. 15; 280; 600; 904; 1224; 1544; 1800; 2472; 3112. 16; 296; 632; 968; 1288; 1608; 1928; 2600; 3240. 17; 336; 696; 1064; 1416; 1800; 2152; 2856; 3624. 18; 376; 776; 1160; 1544; 1992; 2344; 3112; 4008. 19; 408; 840; 1288; 1736; 2152; 2600; 3496; 4264. 20; 440; 904; 1384; 1864; 2344; 2792; 3752; 4584. 21; 488; 1000; 1480; 1992; 2536; 2984; 4008; 4968. - Observation 9 For in-band deployments, it has been agreed to use ITBS indices spanning from index 11 to 17, which includes one TBS entry that is surrounded by brackets (ITBS = 16, ISF = 0 → [328, 296]) according with the WA on the TBS/MCS table for DL. - Proposal 5 Upon resolving the FFS on the TBS entry surrounded by brackets, the TBS/MCS Table to support 16-QAM in DL for in-band deployments is as follows: .Modulation Scheme; Number of NPDSCH Subframes (NSF). 1; 2; 3; 4; 5; 6; 8; 10. QPSK only; 0; 16; 32; 56; 88; 120; 152; 208; 256. 1; 24; 56; 88; 144; 176; 208; 256; 344. 2; 32; 72; 144; 176; 208; 256; 328; 424. 3; 40; 104; 176; 208; 256; 328; 440; 568. 4; 56; 120; 208; 256; 328; 408; 552; 680. 5; 72; 144; 224; 328; 424; 504; 680; 872. 6; 88; 176; 256; 392; 504; 600; 808; 1032. 7; 104; 224; 328; 472; 584; 680; 968; 1224. 8; 120; 256; 392; 536; 680; 808; 1096; 1352. 9; 136; 296; 456; 616; 776; 936; 1256; 1544. 10; 144; 328; 504; 680; 872; 1032; 1384; 1736. 16-QAM only; 11; 176; 376; 584; 776; 1000; 1192; 1608; 2024. 12; 208; 440; 680; 904; 1128; 1352; 1800; 2280. 13; 224; 488; 744; 1032; 1256; 1544; 2024; 2536. 14; 256; 552; 840; 1128; 1416; 1736; 2280; 2856. 15; 280; 600; 904; 1224; 1544; 1800; 2472; 3112. 16; 296; 632; 968; 1288; 1608; 1928; 2600; 3240. 17; 336; 696; 1064; 1416; 1800; 2152; 2856; 3624. 18; -; -; -; -; -; -; -; -. 19; -; -; -; -; -; -; -; -. 20; -; -; -; -; -; -; -; -. |

For the working assumptions on breaking points of standalone and guardband deployments, there are following proposals:

- Confirm the working assumption

  - Huawei, HiSilicon, Nokia, NSB, MTK, Ericsson

- For standalone and guardband deployments, downlink TBS entries between 12 and 21 are used for 16QAM.

  - ZTE

Based on the input, the following is proposed

Proposal 2: Confirm the working assumption:

- For standalone and guardband deployments, the downlink TBS entries between 14 (TBS of 2856 for I_SF=7) and 21 are used for 16QAM.

Please input your comments for the above proposal:

| Companies | Comments |
| --- | --- |
| Ericsson | Ok with Proposal 2 |
| Huawei, HiSilicon | We agree with proposal 2. |
| MTK | We are ok with proposal2. |

The following has been achieved:

Agreement

Confirm the working assumption:

- For standalone and guardband deployments, the downlink TBS entries between 14 (TBS of 2856 for I_SF=7) and 21 are used for 16QAM.

Applicability

Issue 3: Applicability

The following are proposed:

| Sourcing | proposals |
| --- | --- |
| [2] | Proposal 5: 16-QAM is not supported for EDT. Proposal 6: 16-QAM is supported for PUR. Proposal 7: 16-QAM is supported for multi-TB scheduling. |
| [3] | Proposal 9: 16-QAM can be supported for PUR. Proposal 10: 16-QAM is not supported for EDT. |
| [5] | Proposal 10:16QAM could not be supported for NPDSCH and NPUSCH associated with C-RNTI from CSS. Proposal 11: 16QAM are not supported for UL and DL transmission in PUR procedure. Proposal 12: 16QAM is not supported in EDT for NB-IoT. |
| [6] | Proposal 2: DL 16-QAM is not applicable to C-RNTI scheduled from CSS. Proposal 3: Do not support DL 16-QAM during EDT. Proposal 4: Do not support DL 16-QAM during PUR procedure. Proposal 8: UL 16-QAM is not applicable to C-RNTI scheduled from CSS. Proposal 9: Do not support UL 16-QAM during EDT. Proposal 10: Support UL 16-QAM during PUR procedure. Include the corresponding configuration flag in PUR-Config. |
| [8] | - Proposal 11 Unless a concrete scenario be proposed to be investigated and proven to be beneficial, for both UL and DL 16-QAM is not applied to C-RNTI from CSS. - Observation 15 In downlink, making 16-QAM applicable for EDT and PUR will imply for example having to design a channel quality reporting for idle-mode. - Observation 16 Applying 16-QAM in UL for EDT and/or PUR seems to be an overdesign of the usable modulation schemes since EDT is meant to be a mechanism for transmitting small data packets during the RA procedure, whereas one of the main use-cases for PUR is related to periodic transmissions of small data from e.g., metering devices (temperature sensor). - Proposal 12 Unless a concrete scenario be proposed to be investigated and evaluated, 16-QAM is not applicable in downlink nor in uplink for PUR and EDT. |

Three companies (ZTE, QC, Ericsson) propose to not support 16-QAM for C-RNTI from CSS, therefore, the following is proposed:

Proposal 3: 16-QAM is not applied to C-RNTI from CSS.

Seven companies (Huawei, HiSilicon, Nokia, NSB, ZTE, QC, Ericsson) propose to not support 16-QAM for EDT. Therefore, the following is proposed:

Proposal 4: 16-QAM is not applied to EDT.

Five companies (Huawei, HiSilicon, Nokia, NSB, QC (for UL)) propose to support PUR at least for uplink, and three companies (ZTE, QC (for DL), Ericsson) propose to not support PUR at least for downlink. In addition, Huawei and HiSilicon propose to support multi-TB scheduling for 16QAM. Therefore, the following is proposed:

Proposal 5: FFS for PUR and multi-TB scheduling.

Please input your comments for the above proposals, and also the reasons to support or not support PUR and multi-TB scheduling:

| Companies | Comments |
| --- | --- |
| Ericsson | Ok with Proposal 3 and 4. For PUR our view was misinterpreted since it was the same view for both UL and DL, “16-QAM is not applicable in downlink nor in uplink for PUR”. Proposal 5, is ok in our view to have an FFS on “multi-TB scheduling”. |
| Huawei, HiSilicon | We agree with proposal 3-5. Due to the benefit of improving peak data rate, 16QAM for multi-TB scheduling can be supported. And we also support 16QAM for PUR to improve the PUR transmission data rate. |
| MTK | We are ok with proposal 3~5. |

For C-RNTI from CSS and EDT, the following has achieved:

Agreement

For both uplink and downlink

- 16-QAM is not applied to C-RNTI from CSS.

- 16-QAM is not applied to EDT.

For PUR and multi-TB scheduling, more discussion would be needed.

Proposal 5: FFS for PUR and multi-TB scheduling.

Please input your comments on support of 16-QAM for PUR and multi-TB scheduling:

| Companies | Comments |
| --- | --- |
| Ericsson | Ok |
| Nokia, NSB | Ok |
| ZTE,Sanechips | OK |
| Huawei, HiSilicon | OK, and we support 16-QAM for PUR and multi-TB scheduling as commented before. |
| Lenovo, MotoM | OK |

DCI

Issue 4: DCI design.

There are following proposals on power allocation

| Sourcing | proposals |
| --- | --- |
| [2] | Proposal 3: The introduction of 16-QAM should avoid increase of DCI size. Proposal 4: Considering variations of channel quality, the flexibility of scheduling Rel-16 QPSK TBS and repetition numbers should be supported if 16-QAM is enabled. |
| [3] | Proposal 7: The DCI size is not increased for 16-QAM support. Proposal 8: When UE is configured with 16-QAM, the DCI should support all existing MCS and repetition combinations. FFS details of DCI design. |
| [5] | Proposal 3: If 4-bit MCS table is adopted for DL 16QAM, it can be considered to - Remove the existing 6 MCS entries and add new 8 MCS entries for guard-band and standalone deployments. - Utilize 1 bit of ‘Repetition number’ in DCI to dynamically indicate the fallback from 16QAM MCS table to legacy QPSK MCS table Proposal 4: If 5-bit MCS table is adopted for DL 16QAM, - Overlapped TBS 12 and 13 could be set for MCS entries with QPSK and 16QAM. - The most significant bit of ‘repetition number’ field can be repurposed to indicate MCS. Proposal 6: For UL 16QAM, the most significant bit of ‘subcarrier indication’ field can be used for 5-bit MCS indication. |
| [6] | Proposal 5: Enabling DL 16-QAM does not change the DCI size. Proposal 6: If the “repetition number” field in DCI indicates 1 repetition, the MCS field indicates an entry in the DL 16-QAM MCS table. If the “repetition number” field indicates more than 1 repetition, the MCS field indicates an entry of the legacy QPSK table. - FFS: Details of the 16-QAM MCS table Proposal 11: Enabling UL 16-QAM does not change the DCI size. Proposal 12: If the “repetition number” field in DCI indicates 1 repetition and the “subcarrier indication” field indicates more than one subcarrier, the MCS field indicates an entry in the UL 16-QAM MCS table. If the “repetition number” field indicates more than 1 repetition, the MCS field indicates an entry of the legacy UL MCS table. - FFS: Details of the UL 16-QAM MCS table |
| [7] | Proposal 1: For the DCI format N1 optimization, the joint coding of MCS and repetition number field can be considered. Proposal 4: For the DCI format N0 optimization, the joint coding of MCS, repetition number and/or resource assignment field can be considered. |

The proposed schemes are diverse, and all proposed schemes seem to be aligned with the principle that the DCI size is not increased to support 16-QAM. Therefore, the following is proposed:

Proposal 6: The DCI size is not increased to support 16-QAM in uplink and downlink.

On the indication of downlink 16-QAM, there are following proposals from companies:

- MCS is increased to 5 bits, and repetition field is reduced by 1 bit;

- MCS is 4 bits, and 1 bit in repetition field is used to indicate legacy QPSK or 16QAM;

- MCS is 4 bits, and a reserved states of MCS indicates 16QAM. Repetition field indicates 16QAM MCS.

- MCS is 4 bits, 16QAM is used if repetition is indicated as one, and legacy QPSK otherwise.

Therefore, the following is proposed:

Proposal 7: It is down-selected from following options on the indication of downlink 16-QAM:

- Option 1: MCS is increased to 5 bits, and repetition field is reduced by 1 bit;

- Option 2: MCS is 4 bits, and 1 bit in repetition field is used to indicate legacy QPSK or 16QAM;

- Option 3: MCS is 4 bits, and a reserved states of MCS indicates 16QAM. Repetition field indicates 16QAM MCS.

- Option 4: MCS is 4 bits, 16QAM is used if repetition is indicated as one, and legacy QPSK otherwise.

Please input your comments for the above proposals:

| Companies | Comments |
| --- | --- |
| Ericsson | It is better to start discussing DCI designs once the WAs (MCS/TBS table, breaking-point) have been settled. Even the FSS on PUR should be settled since some designs make use of fields utilized by PUR. |
| Huawei, HiSilicon | We agree with proposal 6 and support option 3 of proposal 7. Considering possible channel condition variations, the flexibility of scheduling Rel-16 QPSK TBS and repetition numbers should be supported even if 16QAM is enabled. |
| MTK | We are ok with proposal6 and prefer opition3 in proposal7.Similarly, a reserved state also could be used for CQI reporting request. |
| ZTE,Sanechips | Regarding the proposal7, at present, the MCS table design is not sufficient, it is suggested to not preclude the other options. So we need to add a note. Regarding option4, from our understanding, one repetition should also be supported for QPSK. So ZTE suggest to modify the sentence as following or just add the option5. Proposal 7: It is down-selected from following options on the indication of downlink 16-QAM: - Option 1: MCS is increased to 5 bits, and repetition field is reduced by 1 bit; - Option 2: MCS is 4 bits, and 1 bit in repetition field is used to indicate legacy QPSK or 16QAM; - Option 3: MCS is 4 bits, and a reserved states of MCS indicates 16QAM. Repetition field indicates 16QAM MCS. - Option 4: MCS is 4 bits, 16QAM and QPSK can be used if repetition is indicated as one, and legacy QPSK otherwise. Note: other options are not precluded. |
| Nokia, NSB | We are OK with Proposal 7 and fine with ZTE’s suggested changes. Our preference is Option 3. |
| Qualcomm | We are OK with the proposal as modified by ZTE, but we thought it would be good to agree that the repetition / MCS is jointly indicated by the MCS / repetition fields: - MCS field is 4 bits and {repetition, MCS} are indicated by a combination of the MCS field and repetition field |
| Lenovo, MotoM | We are OK with the current proposal 7 or updated proposal of ZTE. It seems current option 4 of proposal 7 is from QC and ZTE proposes another option. Based on the discussion, we prefer option 3 of proposal 7 since option 3 covers all legacy MCS and repetition number of QPSK and supports new MCS of 16QAM. This option is most straightforward. |

The following has been achieved:

Working Assumption

The DCI size is not increased to support 16-QAM in uplink and downlink.

For the options of DCI indication, the proposal is updated as below:

Proposal 7: It is down-selected from following options on the indication of downlink 16-QAM:

- Option 1: MCS is increased to 5 bits, and repetition field is reduced by 1 bit;

- Option 2: MCS is 4 bits, and 1 bit in repetition field is used to indicate legacy QPSK or 16QAM;

- Option 3: MCS is 4 bits, and a reserved states of MCS indicates 16QAM. Repetition field indicates 16QAM MCS.

- Option 4: MCS is 4 bits, 16QAM and QPSK can be used if repetition is indicated as one, and legacy QPSK otherwise.

- Option 5: MCS field is 4 bits and {repetition, MCS} are indicated by a combination of the MCS field and repetition field

- Note: other options are not precluded.

Please input your preference on the options and reasons for further down-selection:

| Companies | Comments |
| --- | --- |
| Ericsson | OK, although the “Note” seems to override the intended down-selection towards focusing on only 5 options. |
| Nokia, NSB | Ok |
| ZTE,Sanechips | OK with the proposal. The details of each option is not clear and better solutions may be provided in next meeting. Therefore, other options we think still shall have a chance to be discussed. Therefore, we are OK with the proposal and the note should be kept. Additionally, regarding the overriding issue raised by Ericsson, if we need to address it, we can modify the main bullet as following: The following options on the indication of downlink 16-QAM can be considered: |
| Qualcomm | OK with ZTE’s proposed main bullet. |
| Huawei, HiSilicon | Ok with the proposal, and also fine with updates from ZTE |
| Lenovo, MotoM | We share similar view as ZTE, some of the options are not clear. I am not the following is the right understanding or not of the above options. Option 1: 5 bit MCS field indicates modulation and TBS, 2 bit Repetition field indicates the repetition number Option 2: 1 bit indicates modulation, 4 bit MCS field indicates TBS, 2 bit Repetition field indicates the repetition number (?) Option 3: 4 bit MCS filed indicates QPSK+TBS or 16QAM, 3 bit Repetition field indicate the repetition number of QPSK or TBS for 16QAM Option 4: 3 bit Repetition field indicate the 16QAM or QPSK+Repetition number, 4bit MCS fields indicates TBS (?) Option 5: Other joint 7 bit to indicate MCS and Repetition fields to indicate the modulation, TBS and repetition number. |

Based on the comments, the proposal is updated as below to make the options more clear:

Proposal 7: The following options on the indication of downlink 16-QAM can be considered:

- Option 1: MCS field is increased to 5 bits to indicate modulation and TBS, and repetition field is reduced to 3 bits to indicate the repetition number;

- Option 2: MCS field is 4 bits to indicate modulation and TBS, and repetition field is reduced to 3 bits to indicate the repetition number;

  - 1 bit is used to indicate legacy QPSK or 16QAM

- Option 3: MCS field is 4 bits to indicate modulation and TBS

  - A reserved states of MCS field indicates use of 16QAM,

  - Repetition field indicates 16QAM MCS if 16QAM is indicated to be used.

- Option 4: MCS is 4 bits,

  - If repetition is indicated as one, 16QAM and QPSK can be indicated by MCS field;

  - If repetition is indicated larger than one, the legacy QPSK MCS can be indicated by MCS field.

- Option 5: {repetition, MCS} are indicated by 8 bits (a combination of the MCS field and repetition field)

- Note: other options are not precluded.

Downlink power allocation to support 16QAM

Issue 5: Power allocation.

There are following proposals on power allocation

| Sourcing | proposals |
| --- | --- |
| [2] | Proposal 8: The NPDSCH EPRE in symbols with CRS can be different and can be the same with the NPDSCH EPRE in symbols with NRS and the NPDSCH EPRE in symbols without NRS and CRS. Proposal 9: For standalone/guard-band deployment, if the NPDSCH EPRE in symbols with NRS is the same with the NPDSCH EPRE in symbols without NRS, the NPDSCH EPRE w/o NRS and NRS EPRE are the same. For in-band deployment, if the NPDSCH EPRE in symbols with NRS is same with the NPDSCH EPRE in symbols without CRS and NRS, the NPDSCH EPRE with NRS, NPDSCH EPRE without NRS and CRS, and NRS EPRE are the same. FFS power ratio of NPDSCH EPRE with CRS and NPDSCH EPRE w/o NRS. |
| [3] | Proposal 5: The total transmit power across OFDM symbols should be constant. Proposal 6: The power ratio of NPDSCH EPRE to NRS EPRE in symbols with NRS (ρC) is explicitly signaled. The other two power ratio values can be determined by the UE. |
| [5] | Observation 3: In practice, different transmit power configured between OFDM symbols may cause phase diversion. Observation 4: The transmit power between different OFDM symbols is constant in R16 NB-IoT when a given NRS power is configured. Proposal 7: The same total transmit power is set for each OFDM symbol for Rel-17 NB-IoT DL power allocation. Proposal 8: A new UE-specific higher layer parameter is introduced to indicate the ratio of NPDSCH EPRE and NRS EPRE in symbols with NRS. Proposal 9: For in-band deployment, the ratio of NRS EPRE to CRS EPRE is indicated for both same PCI and different PCI. |
| [6] | Proposal 7: The UE derives the values of ρA, ρB, ρC implicitly based on - Power boost value for NRS - NRS and CRS relative power level. - Number of NRS and CRS ports. |
| [7] | Proposal 3: Network should semi-statically configure three types of NPDSCH EPRE with assuming all the OFDM symbols with same transmission power. |
| [8] | - Proposal 8 For the downlink power allocation of 16-QAM, the data-to-pilot power ratios are as in LTE defined in terms of the following types: - Stand-alone and Guard-band: - Type A, NPDSCH in symbols without NRS: NPDSCH EPRE = NRS EPRE + ρ_a [dB] - Type B, NPDSCH in symbols with NRS: NPDSCH EPRE = NRS EPRE + ρ_b [dB] - In-band: - Type A and Type B as defined for Stand-alone and Guard-band deployments. - Type C, NPDSCH in symbols with CRS: NPDSCH EPRE = NRS EPRE + ρ_c [dB] - Where: - ρ_a = PA [dB] - PB is an index that refers to the linear ratio between ρ_b and ρ_a (i.e.,10ρ_b1010ρ_a10) - PC is an index that refers to the linear ratio between ρ_c and ρ_a (i.e.,10ρ_c1010ρ_a10) - Proposal 9 For the downlink power allocation of 16-QAM, down-select among the following two alternatives as to resolve the “FFS on signaling details” and the “FFS for the handling on whether the PCI is different or the same”: - Alt-1: “ρ_a” is UE-specific signaled and the index “PB“ referring to a linear power ratio between “ρ_b” and “ρ_a” is cell-specific signaled as to obtain “ρ_b”. - In addition, for in-band deployments in the case of a same PCI “nrs-CRS-PowerOffset” is used to implicitly obtain “ρ_c”, otherwise (i.e., when different PCI) the index “PC“ referring to a linear power ratio between “ρ_c and “ρ_a” is cell-specific signaled as to obtain “ρ_c”. - Alt-2: “ρ_a” is UE-specific signaled and the index “PB“ referring to a linear power ratio between “ρ_b” and “ρ_a” is cell-specific signaled as to obtain “ρ_b”. - In addition, for in-band deployments regardless of whether the PCI is the same or different the index “PC“ referring to a linear power ratio between “ρ_c” and “ρ_a” is cell-specific signaled as to obtain “ρ_c”. - Observation 13 For the downlink power allocation of 16-QAM, it can be pondered between a solution offering commonality for handling both “different and same PCI” (Alt-2) versus a solution that can partially re-use the existing toolbox (i.e., “nrs-CRS-PowerOffset” for same PCI) to save signaling for one out of the two PCI cases (Alt-1). |

For power allocation, several companies assumes constant transmit power between symbols. Based on this, for standalone and guard-band deployments:

- If there’s no NRS power boost, the ratio of NPDSCH EPRE to NRS EPRE is 0dB for one NRS antenna port, and -3dB for two NRS antenna ports;

- If there’s NRS power boost,

  - Option 1: the power ratios of NPDSCH EPRE to NRS EPRE in symbols with and without NRS are signaled;

  - Option 2: the power ratio of NPDSCH EPRE to NRS EPRE in symbols with NRS is signaled.

And for inband deployments, the power ratio of NRS EPRE to CRS EPRE is further indicated, which can reuse the existing parameter nrs-CRS-PowerOffset.

Based on the inputs, the following is proposed to move forward:

Proposal 8: For downlink power allocation to support 16QAM:

- For standalone and guard-band deployments:

  - If there’s no NRS power boost, the ratio of NPDSCH EPRE to NRS EPRE is 0dB for one NRS antenna port, and -3dB for two NRS antenna ports;

  - If there’s NRS power boost,

    - Option 1: the power ratios of NPDSCH EPRE to NRS EPRE in symbols with and without NRS are signaled;

    - Option 2: the power ratio of NPDSCH EPRE to NRS EPRE in symbols with NRS is signaled.

- For inband deployments, the power ratio of NRS EPRE to CRS EPRE is signaled in addition to the signaling for standalone and guard-band deployments.

  - FFS to reuse the existing parameter nrs-CRS-PowerOffset.

Please input whether you are fine with the proposal, and the preferred option if not fine with it:

| Companies | Comments |
| --- | --- |
| Ericsson | We believe more discussion is needed. What is meant by “no NRS power boost”? Is it meant to be when the power of the NPDSCH symbols without NRS is equal to the one of the NPDSCH symbols with NRS? |
| Huawei, HiSilicon | We agree with proposal 8. |
| ZTE,Sanechips | For option2, from our understanding, there is an implicit assumption that the transmit power is the same for each OFDM symbol. If not, “FFS on the same transmit power for each OFDM symbol” should be added for option2 to make it more clear. |
| Nokia, NSB | We are ok with the proposal. |
| Qualcomm | The first bullet describes legacy behavior, we are OK with it. For the case of NRS power boost, we do not understand how Option 1 would work. Option 1 would mean that NPDSCH has the same power in all symbols, which would also mean that different symbols have different total power. |
| Lenovo, MotoM | For the case of NRS power boost, Option 1 means ρA and ρB are signaled. Option 2 means only ρB is signaled. Would you please add option 3, that only ρA is signaled (Option 3: the power ratio of NPDSCH EPRE to NRS EPRE in symbols without NRS is signaled). Because in legacy LTE power allocation, PA(e.g., ρA) and PB (not simple ρB) are signaled if we assume NRS in standalone/ guradband is equivalent to CRS in LTE. So we hope to follow LTE legacy behavior/signalling. ρA =E_A/E_NRS, ρB=E_B/E_NRS, ρC =E_C/E_NRS |

On comments of no NRS power boost, this is the legacy behavior, which means the NRS EPRE is not power boosted compared with PDSCH EPRE.

The option 1 is just listed as there was proposal regarding this, we can down-select in following discussion.

Based on the comments, the proposal is updated as below:

Proposal 8: For downlink power allocation to support 16QAM:

- For standalone and guard-band deployments:

  - If there’s no NRS power boost, the ratio of NPDSCH EPRE to NRS EPRE is 0dB for one NRS antenna port, and -3dB for two NRS antenna ports;

  - If there’s NRS power boost,

    - Option 1: the power ratios of NPDSCH EPRE to NRS EPRE in symbols with and without NRS are signaled;

    - Option 2: the power ratio of NPDSCH EPRE to NRS EPRE in symbols with NRS is signaled, assuming the same transmit power of different symbols.

    - Option 3: the power ratio of NPDSCH EPRE to NRS EPRE in symbols without NRS is signaled, assuming the same transmit power of different symbols.

- For inband deployments, the power ratio of NRS EPRE to CRS EPRE is signaled in addition to the signaling for standalone and guard-band deployments.

  - FFS to reuse the existing parameter nrs-CRS-PowerOffset.

Please input your comments on the above proposal:

| Companies | Comments |
| --- | --- |
| Ericsson | For the three options under the second main bullet (i.e., “If there’s NRS power boost”), we need to add an FFS on whether the signaled power ratio(s) will be “UE specific or Cell specific”. One question, for “Option 2” and “Option 3”, do they mean that “the power ratios of NPDSCH EPRE to NRS EPRE in symbols with and without NRS” would be the same, being the only difference between the options 2 and 3 the reference case used to signal the power ratio? |
| Nokia, NSB | Ok |
| ZTE,Sanechips | The distance between UE and eNB is different, which requires different transmit power for different UE configured with 16-QAM. Therefore, the parameter of power ratio shall be UE specific. And the FFS on whether the signaled power ratio(s) will be “UE specific or Cell specific”is not needed. We are general OK with the proposal just with some modifications. Actually option 1, option2 and option3 include the cases with or without NRS power boost. Therefore, we think it is not needed to redundantly state “If there’s no NRS power boost” or “If there’s NRS power boost”. Therefore, it is suggest to remove the related sentences as following: Proposal 8: For downlink power allocation to support 16QAM: - For standalone and guard-band deployments: - Option 1: the power ratios of NPDSCH EPRE to NRS EPRE in symbols with and without NRS are signaled; - Option 2: the power ratio of NPDSCH EPRE to NRS EPRE in symbols with NRS is signaled, assuming the same transmit power of different symbols. - Option 3: the power ratio of NPDSCH EPRE to NRS EPRE in symbols without NRS is signaled, assuming the same transmit power of different symbols. - For inband deployments, the power ratio of NRS EPRE to CRS EPRE is signaled in addition to the signaling for standalone and guard-band deployments. - FFS to reuse the existing parameter nrs-CRS-PowerOffset. |
| Qualcomm | The proposal from ZTE seems more clear. |
| Huawei, HiSilicon | For the updates from ZTE, we are generally fine, but we think the signaling of option1/2/3 is optional. That is, if UE does not receive this signaling, then UE assumes the ratio of NPDSCH EPRE to NRS EPRE is 0dB for one NRS antenna port, and -3dB for two NRS antenna ports which is the same as legacy. |
| Lenovo, MotoM | The proposal from ZTE is fine to us |

Based on the above comments, the following is proposed:

Proposal 8: For downlink power allocation to support 16QAM:

- For standalone and guard-band deployments:

    - Option 1: the power ratios of NPDSCH EPRE to NRS EPRE in symbols with and without NRS are signaled;

    - Option 2: the power ratio of NPDSCH EPRE to NRS EPRE in symbols with NRS is signaled, assuming the same transmit power of different symbols.

    - Option 3: the power ratio of NPDSCH EPRE to NRS EPRE in symbols without NRS is signaled, assuming the same transmit power of different symbols.

    - If the signaling(s) is(are) not indicated, the legacy power allocation is used.

      - i.e., the ratio of NPDSCH EPRE to NRS EPRE is 0dB for one NRS antenna port, and -3dB for two NRS antenna ports

- For inband deployments, the power ratio of NRS EPRE to CRS EPRE is signaled in addition to the signaling for standalone and guard-band deployments.

  - FFS to reuse the existing parameter nrs-CRS-PowerOffset.

Channel quality reporting

Issue 6: Channel quality reporting

There are following proposals on power allocation

| Sourcing | proposals |
| --- | --- |
| [2] | Observation 1: In current NB-IoT, the channel quality reporting in Msg3 and connected mode are reports of NPDCCH repetition level. Observation 2: For UE scheduled with 16-QAM, the NPDCCH repetition-level based channel quality reporting does not convey sufficient channel quality information. Proposal 10: For 16-QAM, the channel quality report is the NPDSCH MCS, when NPDCCH repetition number is 1. FFS the set of NPDSCH MCS for channel quality reporting. Proposal 11: Re-purpose the channel quality reporting field in Msg3 and MAC CE to support CQI reporting for 16-QAM. |
| [3] | Proposal 3: Introduce CQI reporting for 16-QAM in the DL using the same definition for CQI as eMTC. FFS how to define CSI reference resource. Proposal 4: Reuse Msg3 and MAC CE downlink channel quality measurement report defined in 36.133 for 16-QAM CQI reporting. FFS details. |
| [4] | Observation1: Aperiodic CQI reporting should be supported in NB-IOT R17. Observation2: There is no need to define extra measurement reference resource for CQI reporting. Observation3: The CQI table should cover both QPSK and 16QAM. Proposal2: Reuse 1 bit from the spare bits to reach 5 bits MCS field both in DCI N0 and DCI N1. Proposal3: Reuse 1 bit from the spare bits to indicate CQI reporting request in DCI N0. |
| [7] | Proposal 2: The three unused states of CQI reporting can consider metric or values based on the same target of hypothetical NPDCCH BLER of 1%. |
| [8] | - Proposal 6 The CQI reporting definition to support 16-QAM in DL is as in clause 7.2.3 of TS 36.213 for LTE-MTC with the corresponding updates to adapt it to NB-IoT. - Observation 10 The legacy CQI mapping table in TS 36.133 clause 9.1.22.15 currently uses 13 out of 16 entries, hence the three unused fields could be utilized to incorporate the channel quality reporting for 16-QAM in DL. - Observation 11 For the TBS/MCS table for DL, the step-size between ITBS indices is in most cases smaller than 1dB, which is a level of granularity that might be unfeasible in terms of NRS. Today the channel quality reporting is specified for each repetition level 1, 2, 4, 8, …, which means that in legacy the step size is 3dB. - Observation 12 In Rel-17, the full range of ITBS indices (14 to 21 and 11 to 17 depending on the deployment mode) can be covered using only three candidate reports (i.e., candidateRep-M, candidateRep-N, or candidateRep-O) as to have a feasible level of granularity with step-sizes larger than 1dB. - Proposal 7 The three unused entries in the legacy CQI mapping Table in clause 9.1.22.15 of TS 36.133 (i.e., Table 9.1.22.15-1) are used for the CQI reporting of 16-QAM in DL. Reported value; NPDCCH repetition level; 16-QAM CQI index with NPDSCH transport block error probability not exceeding 0.1. candidateRep-M; 1; 0. candidateRep-N; 1; 1. candidateRep-O; 1; 2. CQI Index; ITBS index. Guard-band and Stand-alone deployments; In-band deployments. 0; [17]; [13]. 1; [20]; [16]. 2; [21]; [17]. |

On the content of channel quality report, several companies proposed to reuse LTE-MTC, but the NB-IoT channel quality report is the estimation of NPDCCH instead of data channel. Therefore, the first step should be the metric for channel quality report. The following is proposed:

Proposal 9: If 16-QAM is configured for NPDSCH, the channel quality report for 16-QAM is the MCS to achieve 10% BLER of NPDSCH.

Please input your comments for the above proposal:

| Companies | Comments |
| --- | --- |
| Ericsson | The statement “… is the MCS to achieve 10% BLER of NPDSCH”, we believe it should be written as “… is the MCS index associated to an NPDSCH transport block that achieves an error probability not exceeding 10% BLER”. |
| Huawei, HiSilicon | OK with proposal 9. |
| MTK | We are ok with proposal9. |
| ZTE,Sanechips | The size of MCS index table(4bits or 5bits) is not certain. It is not suggested to say the channel quality report is the MCS index. Based on the Ericsson reply, the following can be considered “If 16-QAM is configured for NPDSCH, the channel quality report for 16-QAM is based on NPDSCH transport block that achieves an error probability not exceeding 10% BLER” |
| Nokia, NSB | We would like clarification here. In our view, we would like to support both legacy values (NPDCCH-based) and new values (NPDSCH-based as proposed above). It seems Proposal 9 doesn’t support NPDCCH-based reports if 16-QAM is configured but would like to check whether that is the intention. |
| Qualcomm | We would like some discussion on how the whole framework would work before agreeing to the proposal above. First, we would like to remind the group that the “channel quality reporting” in previous NBIOT releases had no RAN1 specification impact. Do companies think that something needs to be specified in RAN1 for this particular case? Note that this objective is RAN2-led. If we need to specify something in RAN1, it would be good to discuss what framework we will use. Is the assumption that we will define a “CSI reference resource” over which a virtual NPDCCH will be mapped? |
| Lenovo, MotoM | Although we don’t have strong view here, I am confused to the WID - Specify 16-QAM for unicast in UL and DL, including necessary changes to DL power allocation for NPDSCH and DL TBS. This is to be specified without a new NB-IoT UE category. For DL, increase in maximum TBS of e.g. 2x the Rel-16 maximum, and soft buffer size will be specified by modifying at least existing Category NB2. For UL, the maximum TBS is not increased. [NB-IoT] [RAN1, RAN4] - Extend the NB-IoT channel quality reporting based on the framework of Rel-14—16, to support 16-QAM in DL. [NB-IoT] [RAN2, RAN1, RAN4] NBIoT CQI reporting metric is based on the hypothetical NPDCCH BLER of 1%. If we use LTE metric of PDSCH TB BLER metric, does it still align with the WID? If so, we prefer the update from ZTE. |

Regarding Nokia’s comments, the proposal just gives the report for 16-QAM and doesn’t mention the part for legacy QPSK/BPSK.

Regarding QC’s comments, although the part of channel quality report is not captured in RAN1 spec, it may be more proper for RAN1 to decide the metric for reporting etc., as the TBS, breaking points are determined by RAN1.

Regarding Lenovo’s comments, my understanding is that using the PDSCH TB BLER metric would also be within the framework, it’s just an update to support 16-QAM, as the repetition number has reached 1 in legacy.

Based on the comments, the proposal is updated as below:

Proposal 9: If 16-QAM is configured for NPDSCH, the channel quality report for 16-QAM is based on NPDSCH transport block that achieves an error probability not exceeding 10% BLER.

Please input your comments on the above proposal:

| Companies | Comments |
| --- | --- |
| Ericsson | Ok |
| Nokia, NSB | Ok |
| ZTE,Sanechips | OK |
| Huawei/HiSilicon | OK |
| Lenovo, MotoM | OK |

Others

### Issue 7: Others

There are also other proposals as below:

| Sourcing | proposals |
| --- | --- |
| [5] | Proposal 5: For NB-IoT UE with 16QAM, double RU number should be considered to improve the performance for some QPSK entries with the repetition number reduced half. |
| [6] | Proposal 13: RAN1 to consider adding an additional power control parameter to allow for increased power with 16-QAM (e.g. similar to ΔTF) |
| [8] | - Observation 14 In LTE, the term ΔTF in the power control equation increases the power when the number of bits per RE is increased by a higher order modulation scheme, a similar element can be incorporated into the NB-IoT’s equation for 16-QAM in UL. - Proposal 10 Incorporate into the UE’s transmit power control equation, a new term to boost the power when the number of bits per RE is increased due to the use of 16-QAM in UL. |

Two companies proposed to add a new term for power offset between different modulations, therefore, the following is proposed for discussion:

Proposal 10: FFS on an additional power control parameter for 16-QAM (e.g. similar to ΔTF).

Please input your comments for the above proposal, or if you think any proposal listed in this section or any other issue can be discussed in this meeting:

| Companies | Comments |
| --- | --- |
| Ericsson | We are ok in having an FFS on Proposal 10 of the FLS. |
| ZTE,Sanechips | For NB-IoT UE with 16QAM, the performance for some QPSK entries with high code rate has serious degradation, which should be addressed also. So it is suggested to add the FFS in proposal 10 or add the proposal 11, like: Proposal 11: FFS on double RU number with repetition reduced half for some QPSK entries for NB-IoT UE with 16-QAM. |
| Nokia, NSB | We are ok with Proposal 10. We do not think the proposal on doubling RU is needed. |

As several companies agreed to further study the additional power control parameter for 16-QAM, the following is proposed:

Proposal 10: FFS on an additional power control parameter for 16-QAM (e.g. similar to ΔTF).

Please input your reason to support or not support the additional power control parameter:

| Companies | Comments |
| --- | --- |
| Ericsson | Ok |
| Nokia, NSB | Ok |
| ZTE,Sanechips | Double RU number with repetition reduced half for some QPSK entries for NB-IoT UE with 16-QAM also should be discussed since it can avoid the obvious performance loss. |
| Qualcomm | OK |
| Huawei, HiSilicon | OK |
| Lenovo, MotoM | OK |

As RAN2 and RAN4 would start their work related to 16-QAM (channel quality report, RF and etc.), it may be needed to inform them the situation of the TBS and breaking points for their future work.

Proposal 11: Send LS to RAN2 and RAN4 to inform the RAN1 agreements achieved for TBS and breaking points to support 16-QAM.

Please input your comments to the above proposal:

| Companies | Comments |
| --- | --- |
| Ericsson | In the past similar questions have been raised and I remember that the guidance was that unless we have a specific question/action for other WGs, other WGs can always read the agreements. We just wonder if the LS is really needed at this point. |
| ZTE,Sanechips | Agree with Ericsson. It seems to be unnecessary. |
| Qualcomm | RAN4/RAN2 can check the agreements, no need to send out LS. |

Summary

- References

- RP-201306, “WID revision: Additional enhancements for NB-IoT and LTE-MTC”, Huawei, HiSilicon, RAN#88e, E-meeting, June 2020.

- R1-2102357	Support of 16QAM for unicast in UL and DL in NB-IoT	Huawei, HiSilicon

- R1-2102652	Support of 16-QAM for NB-IoT	Nokia, Nokia Shanghai Bell

- R1-2102680	Switching point between QPSK and 16QAM and DCI design for 16QAM in R17	MediaTek Inc.

- R1-2102857	Discussion on UL and DL 16QAM for NB-IoT	ZTE

- R1-2103067	Support of 16-QAM for NB-IoT	Qualcomm Incorporated

- R1-2103531	Support 16QAM for NBIoT	Lenovo, Motorola Mobility

- R1-2103723	Support of 16-QAM for unicast in UL and DL in NB-IoT	Ericsson