3GPP TSG RAN WG1 #104b-e-	R1-2104025

e-Meeting, April 12th – 20th, 2021

Source:	moderator (vivo)

Title:	Feature lead summary#3 on Enhancements on Multi-TRP inter-cell operation

Agenda Item:	8.1.2.2

Document for:	Discussion and Decision

Proposals

Proposal 0:

- For intercell MTRP operation, at most 1 additional PCI, which is different from the serving cell PCI , can be [indirectly] associated with one or more TCI states that are activated for CSI -RS for CSI /PDSCH /PDCCH, per CC.

- RAN1 will further discuss and decide on the maximum number of additional PCIs (different from serving cell PCI ) that can be RRC -configured for intercell MTRP operation (per CC)

- [Here “indirectly associated” means the CSI -RS for CSI /PDSCH DM-RS/PDCCH DM-RS from a serving cell is QCLed with a CSI -RS for tracking/CSI /BM, which is then further QCLed with an SSB that is associated with a PCI different than the serving cell PCI. This is following the conclusion on reusing R15 QCL rules in RAN1#104]

Alternative proposal 0

- For intercell MTRP operation, 1 PCI different from the serving cell PCI is supported per CC

  - RAN1 to decide on the maximum number of non-serving PCIs per CC and/or across all CCs that can be RRC -configured for multi-DCI based inter-cell multi-TRP

Proposal 4(Conclusion):

- Do not support additional spec impact to configure CSI-RS for mobility as QCL source for intercell MTRP operation

Proposed Conclusion 1-1

For discussion purposes only in Rel-17 FeMIMO on inter-cell multi-TRP (AI 8.1.2.2), the term “non-serving cell” is used in the following context:

- A UE  receiving [a DL channel and/or] a DL RS which are QCL-ed to an SSB with a PCI different from a serving cell PCI

  - [The DL channel can be PDSCH or PDCCH  in a CC]

Alternative proposal 1-1

For discussion purposes only in Rel-17 FeMIMO on inter-cell multi-TRP (AI 8.1.2.2), the term “non-serving cell” is used in the following context:

- A UE receiving [a DL channel and/or] a PDSCH/PDCCH/DL RS which are QCL-ed  to an SSB with a PCI different from a serving cell PCI

  - [The DL channel can be PDSCH or PDCCH in a CC]

- A UE receiving PDSCH/PDCCH that is indirectly QCL-ed to an SSB with a PCI different from a serving cell PCI

Reference

Previous agreements

In RAN1 #102e meeting, the following agreements were made:

Agreement

Study the following aspects of QCL /TCI-related enhancement to enable inter-cell multi-DCI based multi-TRP operation.

- Details on configuration of non-serving cell RS;

- Allowed source and target RS types for RS transmitted from the non-serving cell TRP ;

- Allowed QCL types for RS transmitted from the non-serving cell TRP ;

- Measurement and reporting related to QCL /TCI enhancement except for that in 8.1.1, if any;

- Clarification on potential UE behavior for associating/multiplexing non-serving cell RS with other RS/channels;

- Other details not precluded.

In RAN1#103e meeting, further agreements were made as below:

Agreement

For QCL /TCI related enhancement for enhanced inter-cell multi-TRP operations, support RRC configuration of non-serving cell information

- Non-serving cell information can be associated with the TCI state and/or QCL -info at least when “neighbor cell SSB” is used as “QCL referenceSignal ”

  - FFS : Whether beam indication enhancement is needed in addition to QCL -info enhancement

  - FFS : Whether the association is explicit or implicit

Agreement

The information provided by SSB-Configuration-r16/ssb-InfoNcell-r16 and/or MeasObject can be starting point for providing non-serving cell information

For future meetings

Consider rate matching behavior related to non-serving cell SSB.

In RAN1#104e meeting, further agreements were made as below:

Agreement

Non-serving cell information at least includes non-serving cell PCI to support inter-cell multi-DCI multi-TRP operation

- FFS: Whether the indication of PCI is implicit or explicit

Conclusion

Reuse Rel-15/16 QCL rule between the source and target RS/channel for non-serving cell RS/channel.

Agreement

At least following non-serving cell SSB information are needed in inter-cell MTRP operation

- SSB time domain position

- SSB transmission periodicity

- SSB transmission power

FFS: Other non-serving cell information

FFS: Whether indication of these information is implicit or explicit

Agreement

For inter-cell MTRP operation, further discuss following options and down select in RAN1#104bis-e

- Option1: Indicate/associate non-serving cell PCI in the TCI state

  - FFS other non-serving cell information

- Option2: Introduce a flag to indicate whether a TCI state/QCL information is associated with non-serving cell information or serving cell

  - FFS: how the flag is linked to non-serving cell

- Option3: Explicit or implicit grouping of TCI states associated with non-serving cell information corresponding to the serving cell and the non-serving cell respectively.

  - FFS: Each group is associated with a CORESETPoolIndex value.

  - FFS: how to link the group of TCI states to non-serving cell.

- Option4: Re-index the non-serving cell RS, e.g., in the TCI state/QCL-Info, so that the UE can differentiate between a serving cell RS and a non-serving cell RS

  - Example: serving cell RSs are indexed from #0, #1, …, #N-1, while non-serving cell RSs are re-indexed from #N, #N+1, …

  - FFS: detailed re-indexing rule(s) of non-serving cell RSs

- Option5: Introduce a new indicator (e.g., re-index the non-serving cell) to indicate the non-serving cell information that a TCI state/QCL information is associated with

  - FFS: how the indicator is linked to non-serving cell

  - Note: when there is only one non-serving cell, it means the same as Option2.

Agreement

Agree on scheme1

- Scheme1: PDSCH/PDCCH from non-serving cell (PCI) associated with TCI state and/or QCL-info is rate matched around non-serving cell SSB with the same PCI

- FFS: whether PDSCH /PDCCH from serving cell (PCI) is rate matched around non-serving cell SSB

- FFS: whether PDSCH/PDCCH from non-serving cell (PCI) associated with TCI state and/or QCL-info is rate matched around serving cell SSB

Conclusion

The UE may assume received DL transmission from multiple TRP within a CP in FR1 and FR2.

- Note: This does not imply that RAN1 intends to ask RAN4 to tighten network synchronization requirements.