3GPP TSG RAN WG1 #104b-e																			R1-2104068

e-Meeting, April 12th – 20th, 2021

Source:	Moderator (Intel Corporation)

Title:	Summary#4 of AI: 8.1.2.4 Enhancements on HST-SFN deployment

Agenda item:	8.1.2.4

Document for:	Discussion and Decision

# Introduction

In RAN#86 meeting the work item on enhanced MIMO support was agreed for Rel-17 [1]. The objectives of WID include enhancements to multi-TRP transmission scheme in HST-SFN scenario.

2. Enhancement on the support for multi-TRP deployment, targeting both FR1 and FR2: … d. Enhancement to support HST-SFN deployment scenario: i. Identify and specify solution(s) on QCL assumption for DMRS, e.g. multiple QCL assumptions for the same DMRS port(s), targeting DL-only transmission ii. Evaluate and, if the benefit over Rel.16 HST enhancement baseline is demonstrated, specify QCL/QCL-like relation (including applicable type(s) and the associated requirement) between DL and UL signal by reusing the unified TCI framework

The document contains summary of the company’s proposal and Moderator’s proposals.

# Possible enhancements for HST-SFN deployment

The section summarizes company proposals regarding enhancements that can be supported for HST-SFN deployment. The proposals are based on the contributions [2]-[23] submitted to RAN1#104b-e meeting.

## UE-based solutions

### Issue #1-1 (Dynamic switching of scheme 1 and single-TRP)

Regarding support of dynamic switching of scheme 1 and single TRP. In RAN1#104-e meeting there was extensive discussion on this issue, but no agreement was reached. Several companies provided their preference regarding this issue in this meeting and summary of the company’s preferences are provided below:

Issue#1-1: Whether or not to support dynamic switching of scheme 1 and single TRP?

- Alt 1a: Dynamic (DCI-based) is supported

  - Supported by: Huawei/HiSilicon, vivo, ZTE, Ericsson, Samsung, Nokia/NSB, LGE, NTT DOCOMO, Intel

- Alt 1b: Dynamic (DCI-based) is supported with UE capability

  - Supported by: OPPO, Apple, Sony

- Alt 2: Dynamic switching is not supported

  - Supported by: Qualcomm, InterDigital, Spreadtrum, OPPO, NEC, Lenovo/Motorola Mobility (further study), Sony (2nd preference)

Based on the above preferences and considering that dynamic switching to single TRP should be supported as part fallback mode PDSCH reception, it is proposed to agree on the following proposal to address Issue #1-1:

#### Round-1

Proposal #1-1:

- Support dynamic (DCI-based) switching of scheme 1 (PDSCH) with single-TRP scheme by TCI state field in DCI format 1_1/1_2

  - Note: whether or not it should have UE capability to be discussed as part of FG discussion for Rel-17

- FFS all other details including RRC signalling, possible RAN4 impact (if any), etc.

| Company | Comment |
| --- | --- |
| OPPO | We don’t support the proposal without UE capability. Both Alt 1b and 2 are fine to us. Dynamic switching is challenging for UE adopting different channel estimators for scheme 1 and single TRP transmission. A corresponding UE capability is needed. |
| DOCOMO | Support. Even if UE is configured with Scheme 1, UE can receive S-TRP PDSCH scheduled by DCI format 1_0. It means UE must supports dynamic switching between PDSCH scheduled by Scheme1 and S-TRP PDSCH scheduled by DCI format 1_0. Hence, UE should be able to support dynamic switching between Scheme 1 and S-TRP within a DCI format (e.g. by TCI state field) as well. The relationship between Scheme 1 and RRC parameter of highSpeedDemodFlag-r16 should be discussed in RAN4. |
| vivo | Support in principle. But as some companies have concerns about the UE complexity, we wonder if UE doesn’t have the capability for dynamic switching between STRP scheme and SFN scheme, does it also mean that SFN-based PDSCH can’t be scheduled by STRP-based PDCCH? Because STRP-based PDCCH scheduling SFN-based PDSCH is also a dynamic switching case that requires UE to switch its receiving algorithm. Therefore, the UE capability might affect the flexibility of scheduling. |
| ZTE | Support. And have the same view as vivo and DOCOMO. Further, it should not spend time on UE capability discussion at this stage since we have UE capability discussion in the end of Rel-17 anyway. |
| Huawei, HiSilicon | Support. Single TRP transmission is a fall back transmission scheme. Each UE worked on Scheme-1 for HST can fall back to single TRP transmission. With an additional UE capability, then the UE would always work on Scheme-1 (multiple TRP based HST transmission) when UE is not reporting the capability. So, there’s no need to add a UE capability. When Scheme 1 is configured, anyway UE needs to support both Scheme 1 and signle-TRP transmission in a slot, when receiving the common search space. We don’t see any impact to UE complexity. |
| Samsung | Support without additional UE capability. |
| Apple | We do not support this without UE capability. |
| Sony | We agree that the dynamic fallback scheme (S-TRP) from scheme 1 is a useful feature. But on the other hand, as RAN1 discussed in previous meeting(s), there are additional UE implementation complexity identified. Therefore, we used to think Alt.1b can serve as a compromised solution. As for UE capability, perhaps we got it wrong, but it might be possible that scheme 1 itself ends up as an optional feature in Rel.17. If so, other feature depending on scheme 1 might be optional as well. In summary, we would like to share similar view as OPPO that Alt.2 can be out second choice. |
| CATT | Support FL’s proposal. In our opinion, there is no need to have a UE capability for switching of scheme 1 with S-TRP. |
| Nokia/NSB | Support FL’s proposal. No need for separate UE capability. When the difference of the received power from two TRPs is large, only single TRP operation is required both for performance and overhead. |
| Lenovo/MotM | We support Alt 2, and also we are OK with Alt 1b |
| QC | We don’t support the FL proposal. UE doesn’t need to increase its complexity to give network extra flexibility. |
| Ericsson | Support FL’s proposal. |
| LG | Support the proposal. |
| Futurewei | Support |
| NEC | We are OK with Alt 1b or Alt 2. |

#### Round-2

| Company | Comment |
| --- | --- |
| Moderator | Re: companies with concerns on Moderator’s proposal in Round-1. Please follow up on NTT DOCOMO comment for PDSCH scheduling using DCI Format 1_0. Please clarify how semi-static switching helps to save UE complexity in this case. |
| Apple | Fall-back DCI has limited scheduling flexibility so it does not define the UE processing envelope. This is very basic. Otherwise, what is the price to make fall-back DCI size so small? The fundenmetnal question is, we already allow MAC-CE to activate TCI codepoint for PDSCH which can achieve latency on the order of 3-10ms. How can NW even change its sTRP vs mTRP every 0.5ms. Based on what, AI/ML or constant UE CSI reporting? Even for CSI reporting, if you look at tyical timeline, it takes more than 0.5ms for the round trip. The issue is that, there is no evident or concrete deployment plan for the DCI based dynamic switching, there is no complete system design in terms of CSI feedback, etc. It is just empty statement with missing pieces if you even take little time to think about it. Within 0.5ms, even at 500km/hr, UE moves less than 10cm. how can a system change its operation within ms? There is no need to grille UE on why UE cannot support something. UE does not support something because it makes no sense. The moe fundamental question we need proponent of dynamic switching to comment is based on what gNB can toggle its sTRP vs mTRP operation every 0.5ms. We need a clear deployment plan for this to impact our UE design. There is no deployment plan and complete design at all at this moment. We do not support dynamic switching regardless of UE capability or not. We need more techinial discussion on how gNB can swtich its operation mode within 0.5ms, and we need this to be first mandatory feature for the infra-vendor with clear IoDT plan with us before we can even consider. We do not like empty statement. |
| QC | Share similar views with Apple. Scheme-1 should be purely RRC configured and semi-static switched with any other tranmissiom scheme. All other arguments about switching scenarios when train slow down or when the UE is close to one of the TPP does NOT require 0.5ms switching delay. Also, S-TRP operation can be accommodated by network implementation where one TRP is only transmitting. This is similar scenario when there is blockage of one the TRP. Scheme-1 is a special mode that requires advanced tracking, CE and demod processing which is completely different processing mode for S-TRP. We are not okay to double complexity and increase our processing timeline to be ready to receive either scheme-1 or S-TRP. If NW wants to fallback to s-TRP, this can be done semi-statically. |
| ZTE | It seems we are stuck here. In our view, we will have UE feature discussion later, companies can insist that it should be optional feature later on. |
| Huawei, HiSilicon | We share similar view as DOCOMO. Anyway dynamic switching will be supported by using DCI format 1_0 and 1_1/1_2, so supporting dynamic switching by using TCI state indication in DCI format 1_1/1_2 won’t cause additional complexity. |
| Nokia/NSB | First, UE should receive common PDCCH/PDSCH regardless of scheme 1. We don’t expect the network supports only HST-SFN UE. Then, supporting single TRP operation is mandatory regardless of support of HST for PDSCH. Also, when UE is only hearing from a single TRP (no beam is available from other TRP), what is the network assumption for scheme 1. gNB will include MAC-CE with TCI codepoints with one or two TCI states. TCI indication may be the way to switch. The reason for 3ms delay for MAC-CE is not directly related to beam switching. MAC-CE is used for update full set of active TCI states, and if UE receives MAC-CE, UE shall be ready to switch from a TCI to another TCI. |
| Sony | We share the same view with QC when a UE is semi-statically configured with SFN, it is up to NW to transmit with either two TRP SFN or single TRP. If S-TRP transmission applied by NW, it’s also up to NW to adjust DL assignment parameters, e.g. MCS, PMI, accordingly. This can be done transparently to UE. With above-mentioned reasons, we think the fall back DCI format 1_0 can be accommodated. Any more reason to support dynamic switch of scheme 1 and S-TRP with specification impact? |
| vivo | Share similar view with Huawei and DOCOMO. In our view, if SFN-based PDSCH is scheduled by STRP-based PDCCH, UE also should keep two different receive processing for PDCCH and PDSCH. The double complexity might not be avoided, unless we restrict that SFN-based PDSCH can only be scheduled by SFN-based PDCCH. Besides, it’s suboptimal for UE to use the advanced receiving algorithm for scheme 1 to demodulate STRP transmission, since the receiving algorithm for scheme 1 assumes to using the estimation results of two TRSs to construct the frequency-domain and time-domain filters for DMRS estimation, which would mismatch with the channel of STRP transmission. |
| Futurewei | Support the FL proposal, and share similar views as Huawei, DOCOMO, and vivo. Also agree with ZTE that optional UE feature can be discussed later. |
| CATT | Support this proposal. |

#### Round-3

| Company | Comment |
| --- | --- |
| Moderator | Based on the discussion during GTW, one operator believes that dynamic switching of scheme 1 and single TRP is important to support from signalling overhead perspective. Considering that there is no significant impact on UE complexity (UE needs to support dynamic fallback to single TRP mode for PDSCH scheduled by DCI Format 1_0), I kindly request companies with concerns to accept original Proposal #1-1 and discuss UE capability issues by end of Release 17. |
| Apple | We support CORESET with two TCI states, so it is not correct to say that UE need to fall back to sTRP for fallback DCI. The PDSCH scheduled by fallback DCI is linked to the TCI for CORESET since there is no TCI field in fallback DCI Secondly, we do not even support DCI based TCI switching for fallback DCI. It is very contradictory to what operator planned, i.e., dynamic DCI based sTRP, mTRP switching. Last but the most important, we need explanation from the proposal of dynamic switching to explain - Within 0.5ms, UE barely move less than 10cm, why we need to switch between sTRP and mTRP - Why CORESET (fallback DCI scheduled PDSCH) needs to have different sTRP/mTRP mode compared to non-fallback scheduled PDSCH - In real deployment, how can NW determine, every slot, to switch between sTRP or mTRP. Is it based on CSI feedback, or AI/ML In the end, the solution needs to be technically reasonable before we can accept, not because operator is very special. It could be that real deployment is very different from what people claim during the 3GPP. The other solution is - We need to be confirmed with some infra-vendor, which is promoting this to commit to deploy this with the confirmed time frame - We need to check internally whether it matches our product information This is to avoid people use false statement to influence 3GPP decision . For this topic, we do not need to first discussion capability. People has the wrong impression that 3GPP can specify any useless feature as long as no-one implements it. This is out initial intention. But it is clear that people are abusing this compromise. Secondly, you cannot push a design that we do not feel comfortable and tell us to wait for feature for UE capability discussion. This is not how everyone would like to be treated in real life. |
| QC | Share same views with Apple. We need to get clarification first on the use-case and the scenario in HST deployment where dynamic switching to sTRP is mandated. Also, why sTRP mode can’t be done by gNB implementation? From UE perspective, it will be similar to blockage scenario. |
| vivo | To Apple: If PDSCH is SFN-based, you mean PDCCH (whether non-fallback or fallback DCI) should also be SFN-based in the practical deployment, right? If yes, it seems we have to restrict something in spec for the combination of transmission mode between PDSCH and PDSCH. Meanwhile, STRP-based PDCCH scheduling SFN-based PDSCH, or SFN-based PDCCH scheduling STRP-based PDSCH would lack the use case, and the default beam discussion of these two cases would make no sense. If not, as we have mentioned many times, in the case of STRP-based PDCCH scheduling SFN-based PDSCH, UE also needs two different processing for receiving STRP-based PDCCH and SFN-based PDSCH separately in a short time. From this view, it can also be regarded as a dynamic switching between STRP-based PDCCH and SFN-based PDSCH. To QC: In our understanding, STRP mode can’t be done by gNB implementation, please find our analysis in Round 2. |
| Nokia/NSB | Support Moderator’s proposal. |
| ZTE | Support Moderator’s proposal To QC, it is not a good to implement dynamic switching between STRP and SFN via transparent way. If gNB only transmits PDSCH in STRP, but UE assumes SFN scheme 1, then QCL estimation especially for Doppler parameters will be mismatched. |
| Sony | Share same view with QC. Assuming a UE operating in SFN mode, it needs to prepare reception from two TRPs. But NW can choose to use only one TRP for such transmission (up to NW). From UE’s perspective, though there are some resource waste or misalignment, e.g. UE prepares 2 beams for CORESETs reception, it may assume that one TRP transmission is totally blocked. Since NW knows S-TRP transmission, it could and should try even more suitable S-TRP based DL assignment for PDSCH. |
| DOCOMO | Support Moderator’s proposal. |
| Lenovo/MotM | Share similar views as Apple, QC and Sony, however the situation is very unfortunate, and it doesn’t seem companies are ready to make compromises. We suggest reconsidering dynamic switching as a UE-optional capability as a compromise for both sides. |
| Futurewei | Support Moderator’s proposal. |
| CATT | Support Moderator’s proposal. |
| Huawei, HiSilicon | Support Moderator’s proposal. Please note also that with Alt 1a, there are also benefits in terms of reduced configuration delay and signal overhead by RRC configuration. |

### Issue #1-2 (Dynamic switching of scheme 1 and scheme 1a)

Regarding support of dynamic switching of scheme 1 and Rel-16 scheme-1a. In RAN1#104-e meeting there was discussion on this issue, but no agreement was reached. Several companies provided their preference regarding this issue in this meeting and summary of the company’s preferences are provided below:

Issue#1-2: Whether or not to support dynamic switching of scheme 1 and Rel-16 scheme-1a

- Alt 1a: Dynamic (DCI-based) is supported

  - Supported by: vivo, CATT, ZTE, [Samsung], LGE, Huawei, HiSilicon, …

- Alt 1b: Dynamic (DCI-based) is supported with UE capability

  - Supported by: Huawei/HiSilicon, [Samsung], …

- Alt 2: Dynamic (DCI-based) switching is not supported

  - Supported by: OPPO, Qualcomm, InterDigital, Spreadtrum, Apple, Sony, Nokia/NSB,  NEC, Lenovo/Motorola Mobility, Intel …

Based on the received inputs, Alt 2 has more support and the following proposal to address Issue #1-2 is made.

#### Round-1

Proposal #1-2:

- Support semi-static (RRC-based) switching of scheme 1 (PDSCH) with Rel-16 scheme 1a

| Company | Comment |
| --- | --- |
| OPPO | Support the proposal. |
| DOCOMO | Support the proposal#1-2. (Isn’t it “Rel-16 scheme 1a”?) As long as RRC based switching between scheme 1 (PDSCH) with Rel-16 scheme 1a, we are fine with supporting additionally DCI based switching between scheme 1 (PDSCH) with Rel-16 scheme 1. |
| vivo | If some companies still have concerns about the UE complexity, we are ok to have a UE capability for dynamic switching of scheme 1 (PDSCH) with Rel-16 scheme 1a. But we don’t support semi-static (RRC-based) switching of scheme 1 (PDSCH) with Rel-16 scheme 1a, since RRC-based switching would cause large RRC overhead when many UEs in the compartment would perform the switching. Thus we are ok to modify the proposal as follows. • Support to have a UE capability for dynamic switching of scheme 1 (PDSCH) with Rel-16 scheme 1a |
| ZTE | Do not support this proposal. Please noted that there is no RRC signaling to turn off Rel-16 scheme 1a, and dynamic switching between 1a and other schemes are always supported. Thus, Rel-17 should follow Rel-16 design for flexibility. Otherwise, Rel-17 seems a backward design. |
| Spreadtrum | Support FL’s proposal |
| Huawei, HiSilicon | Do not support the proposal. Both 1a and 1b are fine for us. The Scheme 1 and Scheme 1a are more suitable under different channels, it can provide flexibity and benefits for scheduling. In addition, we have similar view as vivo that with RRC reconfiguration, a large delay and RRC signaling storm would be an issue for network. |
| Samsung | Support FL’s proposal. |
| Apple | Support |
| Sony | Support the FL proposal. |
| CATT | Not support. Considering the fact that more than 4 layers are not likely to be supported in high speed train deployment scenario with L-o-S propagation, one CDM group restriction for DM-RS is reasonable. And if such restriction is supported, dynamic switching between scheme 1 and SDM 1a can be achieved. So Alt 1a or 1b is preferred for Rel-17 HST-SFN deployment scenario. |
| Nokia/NSB | Support FL’s proposal. We don’t see any use case of switching of two schemes. |
| Lenovo/MotM | Support |
| QC | Support FL’s proposal. |
| Ericsson | Support FL’s proposal. |
| LG | We are fine with vivo’s suggestion. |
| Futurewei | Fine with vivo’s suggestion |
| NEC | Support the proposal. |

#### Round-2

| Company | Comment |
| --- | --- |
| Moderator | Re: companies with concerns on Moderator’s proposal. Please clarify the use case of dynamic switching of scheme 1 and Rel-16 scheme 1a in the context of HST-SFN scenario. |
| ZTE | Rel-16 SDM can provide more capacity than SFN because of large rank transmission even in Los scenario. Further, only RRC enabled switching is not preferred because of RRC ambiguity duration. RRC also casues large delay. |
| OPPO | We don’t think UE can be scheduled with high rank in LOS of HST-SFN scenario. For HST-SFN, even close loop CSI feedback is not available in most cases. How can gNB determine when to perform switching between scheme 1 and scheme 1a dynamically? |
| Huawei, HiSilicon | Dynamic switching is beneficial for scheduling flexibility under different channel environments. For example, for UEs in LoS scenario when near the window, SFN with low rank may be preferred, and if in LoS+NLoS scenario when moving away from the window, higher rank may be achieved which is more suitable for NCJT scheduling. Also, SNR level may affect the scheduling to switch between Scheme 1 and Scheme 1-a. In addition, to avoid the large delay and RRC signaling storm in RRC reconfiguration is also the motivation to support dynamic switching. |
| LG | Based on CSI enhancement for Rel-16 SDM scheme, a UE may be able to report STRP CSI and/or NCJT CSI. When we consider HST-SFN CSI can be reported by STRP CSI based on SFNed CSI-RS, a UE can report both HST-SFN CSI and/or Rel-16 SDM CSI. In this case, if dynamic switching of scheme 1 and Rel-16 is supported, the UE can be scheduled based on the perffered CSI. So, we prefer to support dynamic switching. |
| Nokia/NSB | To ZTE and HW, the scenario we are considering is , SDM 1a is supported in city area or near station with dense TRP deployment, while HST schemes can be applied between cities with high-speed where TRPs are deployed in a certain distance. So, they are different deployment and I don’t see any reason to support both schemes in a certain area. Regarding to RRC signaling storm, it may come whenever handover is occurred. This is not only the matter of LoS/NLoS. |
| Lenovo/MotM | Support moderator’s proposal. The fact that Scheme 1a can be useful for UEs in a train under certain channel environments verifies switching between SFN Scheme 1 and SDM Scheme 1a, but does not verify the need of dynamic switching. |
| Futurewei | Support the FL proposal |
| CATT | Dynamic switching should be supported. Rel-16 SDM 1a can improve transmission efficiency and Rel-17 SFN can improve robustness. In addition, scheme 1(SFN) can also be used in scenario other than HST. Thus, dynamic switching between these schemes should be supported in Rel-17. |
| ZTE2 | @QC and Sony, it is not a good to implement dynamic switching between STRP and SFN via transparent way. If gNB only transmits PDSCH in STRP, but UE assumes SFN scheme 1, then QCL estimation especially for Doppler parameters will be mismatched. |
| Moderator | No more discussion |

### Issue #1-3 (Identification/Configuration of scheme 1 for PDSCH)

Regarding identification of scheme 1 for PDSCH. Several companies provided their preference regarding configuration of scheme 1. Summary of the company’s preferences on this issue are provided below:

Issue#1-3: How to indicate PDSCH scheme 1 for the UE?

- Alt-1: Explicit indication using higher layer parameter

  - Supported by: Qualcomm, Nokia/NSB, Intel, Sony

- Alt-2: Implicit indication by restricting the number of CDM groups for DM-RS equal to one

  - Supported by: vivo, CATT, ZTE, Huawei/HiSilicon, LGE, …

Issue #1-3 has some dependency from resolution of Issue# 1-2. The proposal will be formulated based on the outcome of that discussion.

#### Round-1

Proposal #1-3:

- TBD, pending resolution for issue #1-2

| Company | Comment |
| --- | --- |
| OPPO | We think both restriction of one CDM group and explicit higher layer signaling are needed. When one CDM group and two TCI states are indicated, RRC signaling is needed to differentiate Rel-17 scheme 1 from Rel-16 scheme 2a/2b/3/4. |
| vivo | We prefer Alt-2. [one CDM group + 2 TCI states] is enough for scheme 1 to differentiate with other R16 MTRP schemes as shown in the table below. Transmission schemes; CDM groups; TCI state; repetitionNumber; RRC parameter. Scheme 1a; 2; 2; Condition 1; \. Scheme 1; 1; 2; Condition 1; \. Scheme 2a; 1; 2; \; 'fdmSchemeA'. Scheme 2b; 1; 2; \; 'fdmSchemeB'. Scheme 3; 1; 2; \; 'tdmSchemeA'. Scheme 4; 1; 2; Condition 2; \. Condition 1: DCI field 'Time domain resource assignment' not indicating an entry which contains repetitionNumber in PDSCH-TimeDomainResourceAllocation. Condition 2: DCI field "Time domain resource assignment' indicating an entry which contains repetitionNumber in PDSCH-TimeDomainResourceAllocation. |
| ZTE | Both Alt-1 and Alt-2 should be supported. All Rel-17 features need new RRC signaling. Further, it is usually lower rank transmission in HST, so one DMRS CDM group is sufficient. |
| Huawei, HiSilicon | Alt.2 is preferred, similar as Rel-16 design. |
| Samsung | Both Alt1 and Alt2 should be supported. |
| Apple | Alt-1 The CDM group, or, maximum DL layers can be further discussed |
| Sony | Agree with FL’s obvervation that this also depends on the outcome of Issue 1-2. Since RRC-based semi-static switch between scheme 1 and Rel.16 M-TRP scheme 2a, 2b, 3 and 4 was supported in RAN1#104e, it seems nature design to configure scheme 1 to UE at least via a RRC parameters. By the way, we also think one CDM group + two TCI states could convince UE that this is scheme 1 SFN PDSCH transmission. |
| CATT | Support Alt-2. If all the DM-RS for PDSCH scheme 1 can be restricted one CDM group, dynamic switching between scheme 1 and SDM 1a can be achieved. |
| Nokia/NSB | Support Alt-1, Supporting a scheme is more than how to signal it. UE should know the supported schemes in the network regardless of switching method. Detail scheduling indication can be discussed later if we agree to support dynamic switching with SDM 1a. |
| Lenovo/MotM | Support Alt-1 |
| QC | Support Alt-1. Scheme-1 should be RRC configured for UE demod and tracking purposes. |
| Ericsson | Support Alt-1 |
| LG | We also think both Alt1 and Alt2 are needed, and this is related to the decision of issue #1-2. So, it is better to discuss this issue after decision of issue #1-2. |
| Futurewei | Support Alt-2 |
| NEC | Support both Atl-1 and Alt-2. |

#### Round-2

| Company | Comment |
| --- | --- |
| Moderator | TBD, pending resolution for issue #1-2 |
| Nokia/NSB | Before UE receives DCI, UE should know about what transmission schemes can be scheduled, which is activation of schemes. NW activates SDM 1a for a UE by sending MAC-CE with codepoints with two TCI states. The, UE can assume either SDM 1a or S-TRP transmission will happen. Without any signaling of HST scheme 1, how UE knows support of HST scheme 1. In order to distinguish HST scheme 1 from SDM 1a, at least separate signaling is required, which can be an explicit RRC configuration. Now, when UE receives MAC-CE with codepoints with two TCI states, UE can understand what scheme is intended for by RRC configuration. Regarding to Issue #1-2, if RRC is supported for activation of HST scheme 1, dynamic switching is not easy to be implemented. In this case, we have to have another signaling to indicate support both schemes by RRC signaling. |
| QC | Given recent progress on issue 1-2, we believe Alt-1 (RRC signalling) should be agreed. |
| LG | For the clarification, does Alt1 mean supporting of semi-static switching of scheme 1 with single-TRP scheme? If it is correct understanding, it should be decided after decision of Issue#1-1. |

#### Round-3

N/A

#### Round-4

Proposal #1-3:

- Scheme 1 for PDSCH is identified by

  - New RRC parameter

    - FFS configuration details, e.g., per BPW or per CC

  - FFS whether or not restriction to a single CDM group for DM-RS is also supported

| Company | Comment |
| --- | --- |
| Moderator | According to the comments above, RRC parameter is required to enable scheme 1 at the UE. From UE implementation perspective scheme 1 may require special configuration of hardware resources comparing to the case when PDSCH transmission is performed according to Rel-15/Rel-16 case. It would be also the benefits of saving one bit in RRC signalling comparing to possible increase in UE complexity. |
| QC | Support |
| ZTE | Based the agreement yesterday, RRC parameter is required to enable scheme 1. If scheme 1 is enabled by RRC, and two TCI states are indicated by DCI, then the PDSCH transmission is identified as scheme 1. Proposal #1-3: - Scheme 1 for PDSCH is identified by - New RRC parameter and the number of TCI states indicated by DCI - FFS RRC configuration details, e.g., per BPW or per CC - FFS whether or not restriction to a single CDM group for DM-RS is also supported |
| LG | Share the same view with ZTE. Even when the RRC parameter is configured, the condition for the number of TCI states should be satisfied as commented by ZTE in order to differentiate between scheme 1 and single TRP. |
| OPPO | Agree with the version from ZTE. |
| Huawei, HiSilicon | We are fine with the change from ZTE. |
| vivo | Fine with ZTE’s modifying. |
| Sony | Supportive in principle. Since dynamic switch between S-TRP and SFN was supported at last GTW session, we believe Scheme 1 for PDSCH can be identified by UE - New RRC parameter and 2 TCI states indicated by DCI As for single CDM group for DMRS, we understand that typically at HST scenario, low rank (Rank1 or Rank2) would be applied by NW for DL PDSCH transmission. But it could be up to CSI reporting and NW scheduling. We fail to see any benefit for such constraint. Btw, such constraint also gives us an impression that we are going to additional support dynamic switch between scheme 1 and scheme 1a. So we think it’s not necessary to have this FFS. - FFS whether or not restriction to a single CDM group for DM-RS is also supported |
| Nokia/NSB | Support |
| Spreadtrum | Support |
| Samsung | Support in principle and also fine with ZTE’s revision since at least a new RRC parameter and two TCI states are mandated for identification/configuration of SFN scheme 1. |
| CATT | We are fine with ZTE’s revision. |

### Issue #1-4 (Additional source RS in TCI for scheme 1)

Several companies have mentioned that in Rel-15 for PDSCH a TCI state may be configured not only with TRS as source RS, but also with other reference signals (i.e., CSI-RS for CSI acquisition) as illustrated below. Therefore, it should be decided whether to restrict supported source RS configurations in TCI state to TRS only or allow all Rel-15/Rel-16 source RS configurations for HST-SFN scenario.

- ‘QCL-TypeA’ with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info and, when applicable, ‘QCL-TypeD’ with the same CSI-RS resource, or - ‘QCL-TypeA’ with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info and, when applicable, ‘QCL-TypeD’ with a CSI-RS resource in an NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition,or - QCL-TypeA’ with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured without higher layer parameter trs-Info and without higher layer parameter repetition and, when applicable, ‘QCL-TypeD’ with the same CSI-RS resource.

Issue#1-4: Whether to restrict source RS for scheme 1 to TRS only or allow the same QCL and source RS combination as currently supported for PDSCH in Rel-15?

- Alt-1: All QCL source RS resource types as defined in TCI state of Rel-16 multi-TRP are supported for scheme 1

  - Supported by: CATT, Samsung, Nokia/NSB, Intel, …

- Alt-2: Only TRS is supported as QCL source for QCL-TypeA in TCI

  - Supported by: …

- Note: It was already agreed that each TCI state may be additionally associated with {Spatial Rx parameter} (i.e., QCL-TypeD)

Based on the inputs above, it is proposed not to have QCL source RS restrictions for PDSCH scheme 1.

#### Round-1

Proposal #1-4:

- All QCL source RS resource types as defined in TCI state for Rel-16 multi-TRP are supported for scheme 1

| Company | Comment |
| --- | --- |
| OPPO | Support the proposal. |
| DOCOMO | Support the proposal#1-4 (i.e. Alt-1). |
| vivo | Support the proposal. |
| ZTE | Support |
| Spreadtrum | Support |
| Huawei, HiSilicon | We are fine with the proposal. |
| Samsung | Support |
| Apple | Okay |
| Sony | Support. |
| CATT | Support FL’s proposal. |
| Nokia/NSB | Support the proposal. |
| Lenovo/MotM | Support |
| QC | Don’t support. We support only Alt-2 (TRS only). Concerns on tracking accuracy with lower density CSI-RS. |
| Ericsson | Support FL’s proposal. |
| LG | We are fine with the proposal |
| Futurewei | Support |
| NEC | Support the proposal. |

#### Round-2

| Company | Comment |
| --- | --- |
| Moderator | Re: Qualcomm I agree that CSI-RS for CSI acquisition is unlikely to be used in HST-SFN scenario. Could you please clarify how the proposed restriction can be leveraged, e.g., in UE implementations? Please also indicate whether you have strong concern on Proposal #1-4 as such. |
| QC | Yes, we have strong concerns on proposal 1-4. Re: Moderator: The question is not clear to me. What do you mean UE implementation restrictions? We don’t want the spec to allow scheme-1 to have non TRS as QCL source RS. On the other hand, can proponent clarify what are the advantages, use-case and deployment scenario for configuring non-TRS as QCL source RS resource? Flexbility or spec completelness are not justifiable. Also, how are the CSI-RS for CSI acquisition is QCLed with TRS? It is it for both TCI states? If so, why don’t use TRS directly? |
| ZTE | We don’t see any issue to support CSI-RS for CSI in SFN scenario. If Rel-16 supports it for PDSCH, why not in Rel-17. |
| Moderator | Re: QC Your concern on CSI-RS density is not very clear given that CSI-RS for CSI acquisition has to be QCL-ed with TRS according to current spec, i.e., Doppler related parameters are estimated from TRS not from CSI-RS for CSI. For a CSI-RS resource in an NZP-CSI-RS-ResourceSet configured without higher layer parameter trs-Info and without the higher layer parameter repetition, the UE shall expect that a TCI-State indicates one of the following quasi co-location type(s): - 'typeA' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info and, when applicable, 'typeD' with the same CSI-RS resource, or - 'typeA' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info and, when applicable, 'typeD' with an SS/PBCH block, or - 'typeA' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info and, when applicable, 'typeD' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition, or - 'typeB' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info when 'typeD' is not applicable. There was motivation in Rel-15 to define such QCL relation and there should be sufficient justificiation to remove it from the current spec just for HST. |
| Huawei, HiSilicon | Support. We don’t see the necessity to remove CSI-RS from the legacy QCL resource RSs, whether to use CSI-RS for CSI can be up to gNB configuration. |
| Samsung | Agree with Moderator and ZTE. |
| Nokia/NSB | Agree with Moderator’s comment, CSI-RS for CSI is QCLed with type A/B only with TRS, so it is almost equivalent to use TRS as QCL reference. For simple implementation of the specification, it is better to keep the current principles. |
| Sony | Let’s consider real-life deployment. A UE can be either located in a HST or served in a non-HST scenario in which the legacy QCL rule in Rel.15/16 can be applied in latter case. But if a UE steps into a HST, should the QCL rule be changed to TRS as QCL source only? Of course, by then NW can only activate TCI states with TRS inside as QCL-TypeA and QCL-TypeD, but it seems we don’t need to put artificial constraint on it. Not to mention that CSI-RS for CSI can help UE to obtain CSI, which cannot be fulfilled by TRS at any condition. Therefore, we are supportive to moderator’s proposal. |
| Lenovo/MotM | Support the proposal. No need to remove CSI-RS for CSI from the Rel. 16 QCL source RSs given the underlying QCL assumption with TRS |
| Futurewei | Support the proposal. Removing CSI-RS requires spec changes and restricts gNB flexibility. |
| CATT | Agree with Moderator’s comment. |

#### Round-3

| Company | Comment |
| --- | --- |
| Moderator | I propose we take the Proposal #1-4 as offline agreement |
| QC | Still have concerns on the proposal and not ready yet to agree on the proposal. Also, to clarify, we are not arguing to remove something from specification rather saying for HST-SFN, only TRS should be used as QCL RS source. It is understood that DMRS at the end will be linked to TRS, however, it is not the same as DMRS associated directly to TRS. For example, CSIRS could be narrowband precoded compared to TRS, which will affect delay spread and average delay compared what is estimated from TRS. This will create mismatch on the DMRS channel assumption. |
| Nokia/NSB | Support Moderator’s proposal. @QC: Current 38.214 specifies QCL type A with TRS and CSI-RS. |
| ZTE | Support Moderator. As companies explained, the final QCL source is still TRS. Although CSI-RS for CSI may be configured in the indicated TCI state(s), UE finally estimates QCL parameters still based on TRS which is the QCL source of CSI-RS for CSI. So the concern from QC is not an issue from our view. |
| Sony | From QC’s example, we see a bad case that CSI-RS for CSI is widely different from TRS either in frequency domain or precoding. For surely, for such CSI-RS for CSI, it may not be suitable to serve as source RS in TCI in parallel with TRS or QCL chained with TRS. But should NW find and configure a suitable CSI-RS for CSI to do so? Nevertheless, TRS and CSI-RS for CSI are CSI-RS, and they share the same root for configuration. |
| DOCOMO | Support the proposal. |
| Lenovo/MotM | Support the proposal |
| Futurewei | Support Moderator’s proposal. |
| CATT | Support Moderator’s proposal. |
| Huawei, HiSilicon | Support Moderator’s proposal. We have similar view with ZTE and Sony that with QCL source of TRS, CSI-RS can also provide correct QCL estimations. And network has the flexibility to configure the proper CSI-RS. There’s no need to repeat the discussion since Rel-15. |
| Mod | Re QC. It seems your concern on different precoding on CSI-RS and TRS is rather generic that are relevant to other deployment scenarios. It is still unclear why this configuration is specifically problematic for HST-SFN? |

### Issue #1-5 (Support of scheme 2)

Regarding support of scheme 2. Several companies expressed their preference regarding support of scheme 2 in Rel-17. Some companies have also provided LLS evaluation results comparing performance of scheme 2 with scheme 1 and the baseline scheme. Summary of the company’s views are provided below:

Issue#1-5: Whether to support scheme 2 in Rel-17?

- Scheme 2 is supported

  - Supported by: InterDigital, Intel, Lenovo / Motorola Mobility, …

- Scheme 2 is not supported / low priority

  - Supported by: vivo, OPPO, Samsung, Nokia/NSN, Qualcomm, …

Since there is no clear majority to support scheme 2 in Rel-17, it is recommended to make the following conclusion on Issue #1-5.

#### Round-1

Proposal #1-5:

- Conclusion:

  - Scheme 2 is not supported in Rel-17

| Company | Comment |
| --- | --- |
| OPPO | Support the conclusion. |
| DOCOMO | Support proposal#1-5 (i.e. Scheme 2 is not supported). |
| vivo | Support the proposal |
| ZTE | Support the proposal |
| Spreadtrum | Support |
| Huawei, HiSilicon | We are fine with the conclusion. |
| Samsung | Support |
| Apple | Support |
| Sony | Support the conclusion. |
| CATT | Support this conclusion. |
| Nokia/NSB | Support the proposal. |
| Lenovo/MotM | We prefer prioritizing a decision on whether a network-based solution is adopted for HST-SFN before making a decision on Scheme 2 |
| QC | Support the conclusion |
| Ericsson | We are fine with the conclusion. |
| Futurewei | Fine with the conclusion |
| NEC | We are fine with the conclusion. |
| Moderator | No more discussion on this issue in the next rounds |

### Other issues

This section contains other issues that companies want to highlight for discussion regarding support of UE-based schemes.

| Company | Comment |
| --- | --- |

## TRP-based solutions

### Issue #2-1 (Support of TRP-based pre-compensation)

Regarding support of TRP-based pre-compensation scheme in Rel-17. In RAN1#104-e meeting it was agreed that decision on support will take place in RAN1#104b-e meeting. Below is summary of company’s preferences on this issue.

Issue#2-1: Whether to support specification-based TRP pre-compensations?

- TRP-based frequency offset pre-compensation is supported in Rel-17

  - Supported by: Huawei / HiSilicon, Vivo, ZTE, CATT, Lenovo/Motorola Mobility, Qualcomm, CMCC, Futurewei, Samsung, OPPO, Apple, NEC, Spreadtrum, Sony, NTT Docomo

- TRP-based frequency offset pre-compensation is not supported in Rel-17

  - Supported by: LGE, Nokia / NSN, Ericsson?

Based on the inputs above, there is majority that prefers specification of TRP-based frequency offset compensation scheme in Rel-17 for HST-SFN scenario. Moderator notes that situation is the same to RAN1#103-e and RAN1#104-e meetings and recommends to agree on the following proposal:

#### Round-1

Proposal #2-1:

- Specification-based TRP pre-compensation is supported in Rel-17

  - FFS other details

| Company | Comment |
| --- | --- |
| OPPO | Support the proposal |
| DOCOMO | Support. |
| vivo | Support the proposal |
| ZTE | Support |
| Spreadtrum | Support |
| Huawei, HiSiliocn | Support the proposal. There have been many companies show the performance gain for TRP based pre-compensation. |
| Samsung | Support |
| Sony | Support the FL proposal. |
| CATT | Support FL’s proposal. According to our simulation, even with CFO and Doppler adjustment delay due to SRS, compared with Rel-15 SFN transmission, obvious performance gain can still be observed for the schemes with pre-compenastion. Therefore, from performance perspective, pre-compensation should be supported. |
| Nokia/NSB | Do not support the proposal. In our evaluation with practical gNB implementation (inter-OFDM symbol pre-compensation), we don’t see any gain of pre-compensation scheme over scheme 1. And, specification-based pre-compensation requires UE capability, and no big difference from scheme 1 in terms of deployment, and additional problem of co-existence with legacy UE happens. Also, regarding to two key issues, - Issue 1: how to signal QCL relation - Issue 2: how to measure frequency offset. For both options, there are specification-transparent schemes. - For Issue 1, if pre-compensated TRS/CSI-RS is supported, it is a spec transparent scheme. - For Issue 2, if normal SRS is used for measuring frequency offset, it is also a spec transparent scheme. There is a possible option only using specification transparent schemes, which is implementation-based scheme. |
| Lenovo/MotM | Support |
| Ericsson | According to our observation, the gain of pre-compensaton over DPS is only shown at very low SNR(SNR < 0) and the middle point of 2 TRPs, UE in this condition are most likely to be power limited, the estimation based on UL measurement become unreliable. As the performance gain relies on accurate doppler shift information received/estimated at gNB and reported CSI from the UE, explicit dopper shift report from the UE shall be supported. We understand many companies are willing to support the pre-compensation to reduce the implementation complexity from UE side. In order to proceed the pre-compensation discussion on this meeting, may we suggest to add “support explicit dopper shift indication from UE” in the subbullet to the main proposal? |
| LG | If majority supports the proposal, we are generally ok with the proposal. But, ‘Specification-based’ is not clear yet. So, the proposal can be modified as follows. Proposal #2-1: - Specification-based TRP-based pre-compensation scheme is supported in Rel-17 - FFS other details |
| Futurewei | Support. Also ok with LG’s version |
| NEC | Fine with the proposal. |

#### Round-2

| Company | Comment |
| --- | --- |
| Moderator | Let’s continue technical discussion focusing on the concerns from two objecting companies Below are replies to Nokia/NSB and Ericsson concerns mentioned in the GTW session: - Doppler shift estimation at TRP concern raised by E///. In current HST-SFN deployment, Doppler shift estimation from the UE is already supported. PUCCH / PUSCH are typically used for that purpose. It was shown in RAN4 that the accuracy is reasonable and provides good uplink performance. - Performance concern raised by Nokia relative to scheme 1. Scheme 1 still requires advanced receiver (e.g. special LMMSE channel estimation filter for HST-SFN), which adds implementation complexity to UE. On the other hand, TRP-based pre-compensation scheme, advanced receivers at the UE are not needed and UE can re-use conventional way to process the received signals. Ericsson and Nokia please follow up on the above comments and provide additional concerns, if any. |
| Apple | If we report doppler shift. That means we need to handle at least 3 TRS. Two without pre-compensation, and, one with pre-compensation. TRS is mandatory feature without UE capability. What we do not understand is the following - Why the same infra-vendor kept arguing that things needs to be mandaorty for UE even though there are already a lot of UE mandatory feature enough for NR operation - Why the same infra-vendor cannot even do the doppler shift estimation Normally we can be flexibile as long as we can resolve it during deployment or product alignement. But based on the countless aggressive comment from infra-vendor in this agenda, we cannot accept that UE is required to feedback doppler shift for pre-compensation. gNB inability cannot be the excuse for endless discusson of UE mandatory feature. The ironic part is that everything for infra-vendor is optional. In this agenda, it comes to the point that fairness becomes more important even than any enhancemnt or techinal discussion. |
| ZTE | I sincerely request Nokia and Ericsson consider operaters’ clear need. Newwork vendors can freely decide whether to deploys this feature anyway. |
| Huawei, HiSilicon | We agree with FL’s analysis. TRP based frequency compensation is beneficial for HST scenarios, where the Doppler impact can be pre-compensated by gNB side. The same as many companies, we have shown the obvious benefits with evaluation results for frequency pre-compensation. By the way, a clarification for online comment from Nokia, our evaluations are not only based on high SNR, we provided 4/8/20 dB, selected from the agreed values. For the concern on gNB complexity and proposal to use UE Doppler reporting, we share the view with FL, there is no additional complexity on gNB side and it does not make sense to bundle UE reporting scheme with TRP frequency precompensation. Frequency estimation is anyway need to do when receiving PUSCH/PUCCH in gNB side, we do not see any problem gNB do the frequency estimation. Then, for the proposed UE reporting on Doppler, there are some problems need to be discussed further, one is the UE side complexity as many UE vendors raised during GTW, another one is the accuracy on the Doppler quantization, and there are also some issues on feedback latency and overhead. Then, for the concern on performance, PDSCHs from two TRPs are not frequency pre-compensated in Scheme-1, the interference is totally left to UE to handle. On one hand, it is much high complexity on UE side. On the other hand, the mixed interference will impact the performance. Many companies have already shown the performance gain for frequency pre-compensation. Honestly, we have provided evaluation results for several rounds and addressed different concerns for different meetings from the objecting company. In previous meeting, we provided the performance comparison between DPS and Pre-compensation due to the comment, and then we again provided further evaluation results with UL estimation errors in this meeting considering the requirement and comment from objecting company. Till now, majority companies have shown the performance gain with TRP frequency precompensation. It is time to progress in RAN1. The discussion on whether UE reporting or not for Doppler estimation can be for further discussion. |
| Ericsson | Thanks FL for the comment. The RAN4 results are not evaluating very low SNR, we would appreciate if FL could provide relevant references. We’ve observed limited performance gain with pre-compensation with idea frequency estimiation and concerned about reliability/accuracy with UL based Doppler estimation at low SNR. However, for sake of progress, we can accept the proposal with the following modification as a compromise: Proposal: TRP-based Doppler pre-compensation scheme is supported in Rel-17 with one or both: - UL RS based Doppler estimation by gNB - Further study on UL RS enhancements - DL RS based Doppler feedback by UE - Further study on reporting details To Apple: We are asking the report from UE in order to utilizing the network resources in a more efficient way. The UEs with similar doppler shift can also easily get grouped and scheduled on same symbols. Instead of consuming a lot overhead in the uplink, as UE already keep track of the doppler shift in the algorithems, the UE is encouraged to report that to the network. It would beneficial for both sides, and give better spectrum efficiency. |
| Huawei, HiSilicon2 | Thanks Ericsson’s consideration. For the proposal, as we clarified in the previous reply, frequency can be estimated by gNB, which has already been possible via PUSCH/PUCCH and also can be through SRS based on implementation. Numerous results also show significant performance gain compared with Scheme 1. It may not be necessary to enhance UL RS or UE reporting for frequency estimation. So, the wording needs some updating to reflect the situation. Then, implementation based compensation can be done since R15, and in Rel-17, the enhancement is for spec-based from view of super majority companies. We prefer the version in Chairman’s notes, but for sake of progress also can accept the following: Specification-based TRP frequency pre-compensation is supported in Rel-17 - FFS whether or not to support: - Enhancements on UL RS for Doppler estimation by gNB DL RS based Doppler feedback by UE |
| Nokia/NSB | Thanks all for your effort. If we define two schemes with the same functionality, it is hard for NW vendors handling of different requirement from the operators. Regarding to UE complexity issue, understand the point. However, gNB complexity is much higher than UE complexity increase. In general UE frequency offset in UL is only measured by DMRS for PUSCH/PUCCH decoding, no loop for controlling UE’s frequency offset. This is almost the same as having a big functionality such as PC, of course, frequency pre-compensation. For UE complexity, both for Variant A and Variant B, when train is moving from TRP1 (TRS1) to TRP2(TRS2), we need switch QCL RS one to the other. E.g. (TRS1, TRS2) (TRS2, TRS1). Then, frequency reference shall be TRS1 TRS2. Then, how to adjust UE’s carrier frequency from f1 to f2 in UE implementation. Since with implicit frequency offset measure is requiring UE carrier frequency to synchronize to reference TRS, it should require very high UE complexity. (RF chipset level management is required.) Also, in case of using existing UL signals for measuring frequency offset, we only need to define new QCL variants. But, still there is a way to use pre-compensated TRS as QCL source for PDSCH. For CSI acquisition, pre-compensated CSI-RS can be used without specification impact. The overhead of pre-compensated TRS can be controlled by using larger periodicity. At least this should be the same as Variant B, because at least three TRS shall be sent, TRS 1, TRS 2, and SFN TRS assuming switch from (TRS1, SFN TRS) (TRS2, SFN TRS) when approaching to TRP2. UE cannot switch its reference from TRS 1 to TRS 2 by one shot, time is required for adjusting such update, and UE shall receive TRS1 and TRS2 as well as SFN TRS, which are three TRSs. We think UE should determine its carrier frequency by itself, and explicit reporting or pre-compensated TRS (pre-compensation in both TRPs toward UE’s carrier frequency) should be the best option at least for pre-compensation scheme. |
| Ericsson | To Huawei, HiSilicon 2: The UE reporting of its doppler shift is especially beneficial to provide large number of UEs in the network with pre-compensation service. The UL RS, as pointed out by other companies, shall be studied for better supporting HST scenario. From network perspective we hope you could support to further study both alternatives. About the wording on “specification-based”, our understanding is for non-specification based solution, we won’t need agreement in RAN1 at all. Another issue we would like to raise is, if the FR2 can be considered later. As currently RAN4 is working on HST FR2 evaluation. For the 700m TRP distance there’s severe inter symbol interference between the TRPs for bidirectional transmission. Hence, we suggest limiting the proposal on FR1 only, treat FR2 later in case there’s no such requirement for real deployment in FR2. Proposal: TRP-based Doppler pre-compensation scheme is supported in Rel-17 for FR1 with one or both: - UL RS based Doppler estimation by gNB - Further study on UL RS enhancements - DL RS based Doppler feedback by UE - Further study on reporting details |
| CMCC | Thanks for Ericsson’s step forward. We can consider following update. We would like to explicitly say “specification-based” here to prevent some companies raise that it can be based on implementation without spec impact again and again. Whether UL RS as well as other aspects need to be enhanced can discussed later. Proposal: Specification-based TRP Doppler pre-compensation scheme is supported in Rel-17 for FR1 with one or both: - UL RS based Doppler estimation by gNB - FFS details - DL RS based Doppler feedback by UE - FFS details Regarding Nokia’s comments, the two schemes (UE-based scheme 1 and TRP-based pre-compensation) have different requirements on the complexity of gNB and UE. It’s hard to which scheme is more easier to be applied at this moment. Regarding the UE complexity issue in the TRP-based scheme mentioned by you, i.e., how to adjust UE’s carrier frequency from f1 to f2 in UE implementation, it is the same as for DPS. I’m not sure what is the additional complexity. |
| Sony | We observed that companies fancy the specification-based TRP frequency pre-compensation in Rel.17, but what part of specification should be enhanced remains unclear yet. Surely, the reporting of Doppler shift from UE to NW has standard impact and some benefits. To address the dilemma, we would like to try the following compromise based on HW’s latest proposal Specification-based TRP frequency pre-compensation is supported in Rel-17 - FFS whether or not to support: - Enhancements on UL RS for Doppler estimation by gNB DL RS based Doppler feedback by UE is supported as a working assumption. |
| vivo | Considering some companies have concerns about the effect of frequency errors on pre-compensation in the #104 e-meeting, we have given the LLS simulation results in our contribution(R1-2102510). The results show that pre-compensation still outperforms scheme 1 with frequency errors including CFO error, latency error, and estimation error. From the perspective of performance and UE complexity, scheme 1 is suboptimal for SFN-HST. Thus, pre-compensation should be supported, especially when some UEs don’t support the advanced algorithm for scheme 1. To Ericsson, I remember that your simulation results in the previous meetings showed that a small CDD can improve the performance of SFN transmission, which is similar to our simulation results. Thus, SFN with a small CDD would bring more gain than DPS. |
| Lenovo/MotM | We support pre-compensation scheme. We do not have an issue in principle with supporting Doppler reporting, however our concern is that the specification impact would be large, and more importantly needs to be coordinated with AI 8.1.4. Even if we end up supporting pre-compensation scheme but did not have the time to finalize the corresponding CSI framework (which would then depend fundamentally on the reported Doppler), the entire scheme would be useless and cannot be supported. |
| Futurewei | Support pre-compensation, and support CMCC’s version. |
| Moderator | Based on the discussion, it seems we can take the latest proposal based on Ericsson & CMCC versions as possible agreement? Proposal 2-1a: Specification-based TRP Doppler pre-compensation scheme is supported in Rel-17 for FR1 with one or both: - UL RS based Doppler estimation by gNB - FFS details - DL RS based Doppler feedback by UE - FFS details |
|  | Re: Ericsson: TS 38.141-1 clause 8.2.4. Test case with FRCs G-FR1-A3-… assume QPSK MCS 2. |
| QC | Support and suggest adding the enhancement on UL RS as part of the agreement. Proposal 2-1a: Specification-based TRP Doppler pre-compensation scheme is supported in Rel-17 for FR1 with one or both: - UL RS based Doppler estimation by gNB - Further study on UL RS enhancement - DL RS based Doppler feedback by UE - FFS details Also, regarding Nokia’s comment on TRS switching from TRS1 to TRS2: That is already happening in HST deployment with DPS where the UE switches roughly at mid track point after it receives MAC-CE activating a new TCI state. There is some transient period for the FTL loop to settle to the new Doppler shift. We did not observe any field issues with such switch. |
| CATT | Support the latest proposal. Regarding the two options for Doppler compensation: For UL RS based Doppler estimation by gNB, we have already spent a lot of time discussing this issue and we need to move forward as soon as possible. To address the concerns on TRP-based pre-compensation approach from some companies, many companies showed significant gain of TRP-based pre-compensation schemes even based on non-ideal assumptions such as CFO, UL estimation error and Doppler adjustment delay in this meeting. According our simulation results, it can be seen that obvious performance gain can be achieved by using uplink signal(s) transmitted on the carrier frequency acquired in the 1st step. Moreover, the overhead for CSI reporting and overhead of TRS of implicit Doppler shift reporting may be less than explicit Doppler shift reporting. To summarize, TRP based pre-compensation schemes should be supported and the implicit Doppler shift reporting is enough to enusure the estimation accuracy. |
| ZTE | Thanks Ericsson’s compromise and Moderator’s great effort. We actually also provided numerous results in which obvious performance gain can be observed. We believe this feature is very beneficial especially in FR1 for HST scenario. Regarding the proposal, we are supportive of it. But the main bullet is not very clear for me. I guess the intention is to further decide whether one or both are being supported. Perhaps we should make it clearer Specification-based TRP Doppler pre-compensation scheme is supported in Rel-17 for FR1 with one or both: - UL RS based Doppler estimation by gNB - Further study on UL RS enhancement - DL RS based Doppler feedback by UE - FFS details - Whether to support one or both will be decided later |
| Nokia/NSB | @vivo, when inter-symbol pre-compensation is applied, scheme 1 is almost the same or even better. Inter-intra (ideal) is hard to be implemented. @CMCC, QC, in current formulation, QCL relation is defining a RS a frequency source, pre-compensation is applied for only the other TRP. Since UL frequency is the reference for the frequency offset estimation, UE strives for being locked to the reference TRP. But, for DPS, UE’s carrier frequency is upto UE implementation, there is no ambiguity. In order to reduce such dependency, we prefer to allow UE’s freedom to determine its carrier frequency. Type A or B may have such problem. We believe pre-compensated TRS or using Variant E with UE’s freedom to implementation should be supported. |
| Moderator | No more discussion on this topic. |

### Issue #2-2 (QCL types/assumptions when TRS is source)

Regarding new QCL types/assumption for TRS, when TRS resource(s) is used as source RS in the TCI state. The following preference on the QCL Variants (agreed in RAN1#103-e meeting) were provided by companies for TRP-based pre-compensation schemes.

Issue#2-2: For TRP-based pre-compensation, when the same DMRS port(s) are associated with two TCI states containing TRS as source reference signal, at least one variant from RAN1#103-e meeting agreement is supported for Rel-17 HST-SFN scenario

- Variant A

  - Supported by: Huawei / HiSilicon, OPPO, Spreadtrum, CATT, Futurewei, ZTE, CMCC, Apple, Ericsson (2nd preference), Samsung,  Lenovo/Motorola Mobility, Nokia/NSB(if supported),……

- Variant B

  - Supported by: CATT, Intel, CMCC, Ericsson (1st preference), Qualcomm,

- Variant C

  - Supported by: vivo, CMCC, …

- Variant E

  - Supported by: InterDigital, Futurewei, Samsung,  Sony, Nokia/NSB(if supported)…

Several companies mentioned that Variant A/C or Variant B for QCL assumptions depends on TRS transmission, i.e., TRP-specific TRS for Variants A/C and SFN TRS for Variant B. Considering that Variant A has higher support than Variant C, it is proposed to down-select Variant A and also support Variant B as QCL types/assumptions for TRP-based pre-compensation.

#### Round-1

Proposal #2-2:

- Variant A or Variant B can be used as QCL types/assumption, when the same DMRS port(s) are associated with two TCI states containing TRS as source reference signal.

| Company | Comment |
| --- | --- |
| OPPO | We don’t support to agree on both Variant A and B. One variant is sufficient to support TRP based pre-compensation. Variant A and B needs different UE implementation and different DMRS estimators. If both are supported, UE may need to dynamically switch between different estimators since TCI state is dynamically indicated. Hence, we sugget the updated proposal below: Proposal #2-2: - Down select from Variant A and Variant B to be used as QCL types/assumption, when the same DMRS port(s) are associated with two TCI states containing TRS as source reference signal. |
| vivo | Don’t support the proposal. - For Variant B, SFN-based TRS for {average delay, delay spread} should use the same Tx beams with TRP-specific TRSs for {Doppler shift, Doppler spread} in FR2. However, for Variant A/C, SFN-based TRS deployed for legacy UE can use different Tx beams with TRP-specific TRS for R17 UE in FR2. Therefore, Variant A/C is more flexible than Variant B in FR2 deployment. - In the existing spec, {average delay} is mainly used for downlink timing adjustment by TRS or SSB. For Variant A/C, since downlink frequency shift is only adjusted based on one TCI state(e.g. the first TCI state associated with TRS1) in the pre-compensation scheme, it is natural that adjusting downlink timing shift based on the same TCI state is enough. We have analyzed in our contribution that {average delay} in the second TCI state is redundant, and we don’t see the usage of {average delay} in the second TCI state. |
| ZTE | We support this proposal in principle. However, as we analysed in our tdoc, if new QCL type is introduced, the spec/implementation impact will be huge. For example, the new TCI states with new QCL type will not be shared for other target signals, like CSI-RS, PDSCH/PDCCH for single TRP. Thus, we prefer using the current TCI structure. Once UE identifies the transmission scheme is precompensaton based SFN, UE just ignore some QCL parameters. Suggested wording as follows Proposal #2-2: - Variant A or Variant B can be used as QCL types/assumption, when the same DMRS port(s) are associated with two TCI states containing TRS as source reference signal. - FFS whether new QCL type is supported or not |
| Spreadtrum | Fine with the revised proposal from OPPO |
| Huawei, HiSilicon | Support Variant-A for QCL assumption. In the SFN-HST transmission, the delay information. i.e., delay spread and average delay, should be considered in receiving filter. Since there is precompensation on Doppler, only one Dopple information is needed. So, Variant-A is a proper solution for QCL assumptions. For Variant-B, we do not see how Variant-B can work. |
| Samsung | Support the revised proposal from OPPO and prefer to support Variant A |
| Apple | We are fine with the modified proposal from OPPO |
| Sony | The selection of variants also depends on the results of Issue 2-3. If Alt-2 in Issue 2-3 can be supported, then it seems possible we don’t need to additional support variant. In other words, previously supported Variant E for non-precompensated TRP transmission can be reused for TRP-based pre-compensation. |
| CATT | Support FL’s proposal. |
| Nokia/NSB | We support for using pre-compensated TRS instead of having new QCL relation. If UE is anchored by a TRP (TRS), then when anchor is changed, UE’s carrier frequency should be jumped a lot (2xf_offset). In general UE implementation, UE’s frequency shifting is only supported by unit of SCS/2 used for frequency hopping or carrier switching. Fractional shift is only suppored by AFC. So, RAN4 should define new UE requirement for fractional frequency shift in time. For smooth changing of UE’s carrier frequency between two TRPs, UE shall select its carrier frequency by UE’s choice. However, if specification support is required for this, we are preferring Variant A or Variant E. Also, as ZTE’s comment, UE’s can select the QCL parameter to refer from the two QCL sources. |
| Lenovo/MotM | There are multiple variants of the network-based solution (e.g., with 1 or 2 TRSs). We should finalize the outline of the TRS transmission for pre-compensation scheme first before discussing the QCL assumptions |
| QC | Support FL proposal There will be different network deployments where one of the varriants is utilized. |
| Ericsson | Both Variant A and B would work. In case of separate TRS per TRP and if a same Doppler spread is assumed from two TRPs, Variant A can be used. In case that one TRS is transmitted with SFN and if a same Doppler spread is assumed from two TRPs, Variant B can be used. |
| LG | We support the revised proposal from OPPO/ZTE. |
| Futurewei | Support Variant A but not B |

#### Round-2

| Company | Comment |
| --- | --- |
| Moderator | Let’s continue discussion to better understand pros and cons of different Variants for QCL. Re Huawei, OPPO: The use case of Variant B is 1st TRS transmitted in SFN way (used to convey ‘average delay’ and ‘delay spread’) and another TRS transmitted in TRP-specific manner (used to convey ‘Doppler shift’ and ‘Doppler spread’ of the reference TRP). Note SFN TRS may need to be transmitted anyway by TRP for legacy Rel-15/Rel-16 UEs not supporting scheme 1, i.e., there is no extra overhead. Hence, Variant A or Variant B are targeting different scenarios and should not be down-selected. At the same time Variant A and Variant C are targeting the same TRS transmission scenario and should be down-selected. Re ZTE: New QCL to be discussed as part of Issue #2-3 |
| OPPO | We can’t understand the logic why both Variant A and Variant B should be supported for TRP pre-compensation. Does it means that at least three TRS, TRP-specific TRS for each TRP and SFN TRS from both TRPs, are needed to be configured by gNB, and UE needs to detect these three TRS to support TRP pre-compensation? In our understanding, the same large scale parameters are acquired from Varinat A and variant B. They are different methods to acquire the same DMRS channel estimation. The additional TRS overhead and UE complexity to support both of them is unnecessary. On the other hand, Variant A and B needs different UE implementation and different DMRS estimators. Network vonder wants dynmiac switching between S-TRP and HST-SFN transmission and dynmiac switching between scheme 1a and HST-SFN transmission. Now some companies want dynamic switching between Variant A and B for HST-SFN based on pre-compensation. If support of TRP pre-compensation means supporting dynamic switching among four different DMRS channel estimators, I doubt whether there is UE able to report this capability. Then what is the motivation to support this feature. |
| Huawei, HiSilicon | To FL, thanks for the elaboration of the scenario. The problem for variant B is that the gNB would need to configure three TRS resource sets: TRS1 transmitted in SFN way, and TRS2 and TRS3 transmitted from two TRPs respectively. Then when the UE moves from one TRP to another, the QCL indication becomes {TCI1, TCI2} to {TCI1, TCI3} with Variant B. With Variant A, only the two TRP-specitic TRS resource sets are needed. Besides TRS overhead, a more critical issue is the UE capability for variant B. For now it’s challenging for UE to support more than two TRS for simultaneous tracking and that’s why cooperating TRPs are limited to two. Therefore, Variant A can have less TRS resource overhead, and less UE complexity for UE tracking. Then with support Variant A, there’s no necessary to support Variant B anymore. |
| LG | In our perspective, if there are TRSs transmitted in TRP-specific manner in addition to SFN TRS, then it seems that Variant A is sufficient because there are already TRSs from different TRPs. So, the motivation of supporting both variants is not clear to us. |
| Nokia/NSB | Question to proponents, please clarify my question above (copied below). For Variant A and Variant B, when a train is moving from TRP1 (TRS1) to TRP2(TRS2), we need switch QCL RS one to the other. E.g. (TRS1, TRS2) (TRS2, TRS1). Then, frequency reference shall be TRS1 TRS2. Then, how to adjust UE’s carrier frequency from f1 to f2 in UE implementation. Since with implicit frequency offset measure is requiring UE carrier frequency to synchronize to reference TRS, it should require very high UE complexity. (RF chipset level management is required.). Such fractional shift is only part of AFC. Frequency shift is supported by half SCS granularity. |
| Sony | For variant B, it serves the case that one TRS is transmitted in TRP-specific (Doppler shift and Doppler spread) with another TRP frequency pre-compensation to it and the other TRS is transmitted in SFN manner (providing delay spread and avg. delay). Even though SFN TRS is not included in either scheme 1 or scheme 2, we see it is important use cases as mentioned by moderator and others for legacy UEs. And SFN TRS can be implemented without additional standard impact. From this point of view, we think it’s not necessary to down select Variant A or Variant B. Both are valuable. In addition, we would like to emphasize that both variants can be implemented from already-supported Variant E (TypeA + TypeA), if UE can drop some large-scale parameters. For example, Variant B can be implemented by dropping Doppler shift and Doppler spread from TypeA + Delay spread and avg. delay from another TypeA. From this point of view, new variants are not necessary, but only need to define new UE behavior for SFN TRS or TRP-specific TRS. |
| vivo | Share similar views with OPPO, Variant A/C and B require different receive processing for UE. It’s unnecessary for UE to support both cases considering the UE complexity. In the last meeting, we have the agreement that supporting Variant E(Two TCI states both are associated with QCL-typeA) for scheme 1, which implies that UE would use both TRS1 and TRS2 to estimate {average delay, delay spread} rather than only SFN-based TRS0. Therefore, if an SFN-based TRS0 and a TRP-specific TRS1 are deployed in the HST network, UEs using scheme 1 couldn’t work well. |
| Futurewei | Support Variant A. For Variant B, the schemes described by the FL (first SFN and second from one TRP) and by Sony (first TRP specific from both TRPs with pre-compensation and second SFN) seem a bit different. To us, both may be feasible but require different implementations. For the FL’s first SFN TRS, if it is only to provide ‘average delay’ and ‘delay spread’, can it be just from the non-reference TRP? Is there any advantage of have a SFN that can only provide some of the channel properties? For Sony’s 1st TRS, is it also a SFN? If yes, it can provide all 4 properties. Please clarify. |
| Moderator | Proposal #2-2 is modified as follows based on the comments above: Proposal #2-2a: - Variant A is used as QCL types/assumption, when the same DMRS port(s) are associated with two TCI states containing TRS as source reference signal. - FFS whether Variant B, C, or E should be additionally supported |
| QC | Re OPPO: We don’t believe that there will dynamic switching between variant A and variant B. One of these variants will be used based on network deployment whether SFN TRS is used or not. Also, scheme-1 and variant A share similar TRS processing timeline where UE needs to combine CIR of both TRS and then compute average delay and delay spread. Figure 2-1: scheme-1 (variant E) Figure 2-2: variant A Also, we would like to hear from DOCOMO on their views on support of variant B. As per their comments from last meeting, they plan to support backward compatible SFN transmission for Rel-15/16 UEs. |
| CATT | Support the updated Proposal #2-2a from moderator. |
| Nokia/NSB | When using variant A or B, I think there should be a problem of mismatching when type A RS switched. |
| Huawei, HiSilicon2 | Support FL’s proposal. For Variant B, as we have raised issues on TRS overhead and UE complexity, further study is needed. To reply Nokia’s question, it is implementation dependent and we do not think there is any problem. For example, UE can always assume TRS1 as reference frequency (showing as following) in the TRP switching, then there is no problem on carrier frequency mismatching. |

#### Round-3

| Company | Comment |
| --- | --- |
| Moderator | Please continue discussion: Based on the GTW session and NTT DOCOMO comments it seems that SFN TRS will be deployed first and TRP-specific TRS will be deployed on top of this. Question from Moderator regarding required TRS processing for Variant A/C and Variant B: Then for Variant A, UE would be required to track - 1st TRS from the 1st TRP - 2nd TRS from the 2nd TRP -3rd TRS for fall back based on SFN-ed TRS UE should also perform combining of the delay related parameters based on 1st and 2nd TRS measurements For Variant B UE would be required to track - 1st TRS from 1st TRP for Doppler related parameters - 2nd TRS based on SFN transmission for fallback and delay related parameters Is it correct understanding of the requirements for TRS processing for Variants A and B? |
| OPPO | In our understanding, if dynamic switching between TRP pre-compensation scheme based on Variant A and other schemes (e.g. Variant B or transmission based on SFNed TRS) is not supported, UE doesn’t need to track SFN-ed TRS to support Variant A. But if dynamic switching is supported, UE always needs to track 3 TRS regardless of Variant A or B. The UE complexity and estimated large scale parameters are the same for Variant A and B. |
| QC | Re Moderator: Yes, that is correct understanding. For variant B, first TCI state to get Freq. related parameters and 2nd TCI state to get average time related parameters (shown below). While for variant A, the UE needs to combine the channels from both TRS1 and TRS2 to get time related parameters. |
| vivo | We have some different understanding, in our view: For Variant A/C, a Rel-17 UE would be required to track - 1st TRS from the 1st TRP - 2nd TRS from the 2nd TRP - 3rd TRS for fall back based on SFN-ed TRS A Rel-17 UE doesn’t need to track the SFN-TRS, only tracks two TRSs. A legacy UE tracks the 3rd TRSs which is SFN-based. For Variant B, a Rel-17 UE would be required to track - 1st TRS from 1st TRP for Doppler related parameters - 2st TRS from 2st TRP for Doppler related parameters used for the frequency anchor switching when UE passes the middle point of two TRPs. - 3nd TRS based on SFN transmission for delay related parameters A Rel-17 UE should track the above three TRSs. A legacy UE tracks the 3rd TRSs which is SFN-based. Therefore, we support Variant A/C. |
| Nokia/NSB | Could you clarify if 3rd TRS is only SFN or pre-compensated? Also, we need clarification if TRP with QCL-type A is also applying pre-compensation both for TRS and PDSCH. Otherwise, UE’s frequency tracking to fixed TRS may cause problem as listed in my previous comment. |
| Moderator | Ok, thanks OPPO for clarification. Let me reformulate my observation for Variant A and Variant B: Variant B allows support of dynamic switching to single TRP with smaller UE complexity comparing to Variant A which would require processing of the additional parameters from SFN-ed TRS. Re Nokia: My assumption that TRP-specific TRSs and SFN-ed TRS are not pre-compensated for Variant A and Variant B. Only PDSCH / PDCCH are pre-compensated. |
| ZTE | We still support Variant A because of lowest TRS overhead. It is noted that TRS overhead is huge in HST because of small period. Also, not all operators plan to deploy legacy SFNed TRS. For Variant B, we share the same view with vivo, three TRS are needed. It is noted that 2 TRS should be transmitted for other UEs. To Nokia, we think UL center frequency may not be key point since two TRPs just needs to measure frequency offset between two TRPs. It does not matter UL TX center frequency. |
| OPPO | Re Moderator: Maybe we have different understanding on “Single TRP transmission”. In our understanding, single TRP means PDSCH and TRS are transmitted from one TRP (including DPS). In this case, even if dynamic switching between Variant A and S-TRP is supported, UE doesn’t need to further detect additional TRS. In the deployment where PDSCH and TRS are both transmitted via SFN, and they are QCLed via QCL type A, we doubt the performance may be even worse than S-TRP/DPS, since the Doppler estimation would be inaccurate especially when the UE is close to the midpoint of two TRPs. Maybe we can provide some results on this in next meeting. Also, dynamic switching is being discussed in another issue. We expect that at least a UE capability is introduced for it. Furthermore, if both variants are supported and reported by UE capability, different UEs may report different capability, and gNB should always transmit 3 TRS then. In our opinion, “agree on a scheme in 3GPP-> deploy the network accordingly” is a better way than “(plan to) deploy the network->agree on a scheme in 3GPP accordingly” for deployment. |
| DOCOMO | As we commented in GTW, we believe operator should first deploy SFNed TRS to support Rel.15/16 UEs. Operators pays large cost to deploy 5G NR network in HST scenario. We don’t deploy network which is only usable for a UE supports Rel.17 HST. Our network also plan to deploy SFNed TRS in HST scenario. Rel.17 HST will be deployed on top of it. |
| Lenovo/MotM | Share similar views as VIVO. Our preference is Variant A |
| Futurewei | Support Variant A, and suggest to further study Variant B |
| CATT | In our understanding, For Variant A, a Rel-17 UE would be required to track - 1st TRS from the 1st TRP for Doppler and delay related parameters without pre-compensation - 2nd TRS from the 2nd TRP for Delay related parameters without pre-compensation Similar view as OPPO and vivo, a Rel-17 UE doesn’t need to track a SFN-ed TRS for Variant A. For Variant B, a Rel-17 UE would be required to track - 1st TRS from the 1st TRP for Doppler related parameters without pre-compensation - 2nd TRS based on SFN transmission from the 2nd TRP for delay related parameters without pre-compensation Based on our understanding above, both Variant A and B use the same TRS from a reference TRP for Doppler shift estimation. The difference between Variant A and B is that the TRS(s) for delay estimation is transmitted in SFN or distributed manner. From our simulations results, the performance of Variant A and B is very similar. So both Variant A and B are fine to us. |
| Huawei, HiSilicon | Support Variant A, and also fine with FL’s proposal in Round 2. Similar view as vivo and other companies, a Rel-17 UE does not need to track the 3rd TRS via SFN transmission in Variant A, and the TRS resources are not necessary with pre-compensation (only PDSCH transmission is with pre-compensation). While for Variant B, there will be three TRS resources. Even for dynamic switching for single TRP transmission, Rel-17 UE can fall back to TRS 1 or TRS 2 by implementation. It’s not a problem for Variant A. |
| QC2 | It is not true that that variant B requires three TRS. I believe this is miss-understanding of variant B. Regardless which variant is used, the UE derives its FTL using only single TRS and utilize the corrected frequency offset to change the UL frequency. For variant A and similarly for variant B, once the UE crosses the middle point, the FTL will be driven by the TRS of other TRP which will incur some transient time to settle to the new frequency. |

#### Round-4

| Company | Comment |
| --- | --- |
| Moderator | Many thanks for clarifications on Variant A vs Variant B. It would be great if companies will also share some technical comparison of Variant A vs Variant C and Variant E for the next meeting discussion. |
| ZTE | Variant E should be precluded as SFNed TRS will cause very large overhead. |
| OPPO | According to the evaluation result in our contribution (R1-2102382), the performance of Variant C is worse than A/B since only delay from one TRP can be used for channel estimation. For variant E, if PDSCH from one TRP is pre-compensated but TRS is not pre-compensated, we don’t think the PDSCH can QCL to the TRS w.r.t Doppler shift. |
| vivo | To OPPO, thanks for the evaluation results of Variant A/B/C. But you may misunderstand Variant C which doesn’t mean that UE only use delay from one TRP. With Variant C, UE would also use both {delay spread} from two TRPs, but the average delay used for downlink timing would just refer to one TRP. Therefore, in theory, the performance of Variant C and Variant A/B would be similar. The only difference between Variant A and Variant C is the additional {average delay} from another TRP. As we mentioned, in the pre-compensation scheme, since downlink frequency shift is only adjusted based on one TCI state(e.g. the first TCI state associated with TRS1), adjusting downlink timing shift based on the same TCI state (i.e. the first TCI state associated with TRS1) is enough. Thus, {average delay} in the second TCI state is redundant, and we don’t see the usage of {average delay} in the second TCI state. |
| Nokia/NSB | We need more study on UE’s operation such as determining UL frequency when anchor TRP is switched. |
| Samsung | Support Variant A. We also think that Rel-17 UE does not need to receive SFN TRS. |
| CATT | We support either A or B. |

### Issue #2-3 (New QCL types/assumption)

Regarding signalling of QCL type/assumptions for TRP-based pre-compensation scheme. The following two approaches were identified by companies for TRP-based pre-compensation scheme as captured in Alt 1 and Alt 2:

Issue#2-3: For TRP-based pre-compensation QCL assumptions is provided to the UE by using

- Alt-1: New QCL type

  - Supported by: Huawei / HiSilicon, [Lenovo/MotMobility], [Spreadtrum], [Intel], Vivo, Futurewei, Qualcomm, Ericsson, [CATT]

- Alt-2: The existing QCL type(s) with certain QCL parameters dropped from the indicted QCL type

  - FFS rule to determine which TCI state with dropped QCL parameters

  - Supported by: ZTE, Sony, Nokia/NSB (if supported), OPPO, LGE, NEC, Samsung, Apple, …

Companies are invited to share their preference on ignaling option of QCL types/assumptions for TRP-based pre-compensation scheme.

#### Round-1

Proposal #2-3:

- TBD

| Company | Comment |
| --- | --- |
| OPPO | We support Alt-2. |
| DOCOMO | Slightly prefer Alt.2. |
| vivo | Support Alt-1. |
| ZTE | Support Alt 2. As we analysed in our tdoc, if new QCL type is introduced, the spec/implementation impact will be huge. For example, the new TCI states with new QCL type will not be shared for other target signals, like CSI-RS, PDSCH/PDCCH for single TRP. Thus, we prefer using the current TCI structure. Otherwise, to support the same number of beams for all of PDSCH, PDCCH and CSI-RS as Rel-16, the maximum number of configured/activated TCI states should be increased. |
| Huawei, HiSilicon | Support Alt.1. For the contents of QCL assumptions, the two solutions are ignaling. However, from specification point of view, Alt.1 is much more clear in the configuration. |
| Samsung | Support Alt2. |
| Apple | Alt-2 |
| Sony | Support Alt-2. We share similar view with ZTE that if new QCL-Type is introduced, then the combinations of QCL-Info1 and QCL-Info2 (forming TCI state) should be increased accordingly to cover both scheme 1 and other scheme, e.g. S-TRP Tx. As for Alt-2, we only need to specify UE’s behavior in Spec. |
| CATT | Alt-1 is slightly preferred. |
| Nokia/NSB | Alt-2 if supported. |
| Lenovo/MotM | Similar to our previous comment, the discussion on QCL assumptions should be deferred until after the scheme details are finalized. Our proposal in the tdoc was based on one variant of the pre-compensation scheme, which is not clear yet whether it would be the supported version |
| QC | Support Alt-1 From UE prespective, Alt-1 is explicitly clear how to obtain the large-scale channel properties from each TRS. |
| Ericsson | Alt-1 in our understanding is a cleaner solution. |
| LG | Support Alt2. |
| Futurewei | Support Alt-1 |
| NEC | Support Alt-2. |

#### Round-2

| Company | Comment |
| --- | --- |
| Moderator | There is no clear majority for Alt 1 or Alt 2. Proponents of Alt 1 could you please provide reply to ZTE claim that Alt 2 would require less configurations comparing to Alt 1? Are there any benefits of Alt 1 over Alt 2? |
| QC | Both alternatives work, they convey same message in different ways. Alt 1 is preferred as it reduces ignaling overhead (no need to indicate to the UE which QCL parameters should be ignored and for which TCI state). Then, switching between scheme-1 and TPR pre-compensation can be implicity achieved by the QCL types. |
| ZTE | @QC Do you want to support dynamic switching between SFN scheme 1 and SFN pre-compensation ? I don’t think it will be supported, the usecase is quite unclear. A few companies even don’t support dynamic switching with STRP. For Alt. 2, let’s assume variant A is adopted, if SFN pre-compensation is enabled, then we can predefine that UE only drop {Doppler shift and Doppler spread} from the first one of two indicated TCI states. That’s all the spec impact for Alt. 2. There is not any overhead issue. However, for Alt 1, as we mentioned many times, the key issue is TCI sharing. So far, I haven’t got any explanation. Moreover, RRC for TCI strucurue should be changed. More RRC overhead is needed for each TCI state. |
| OPPO | We agree with ZTE that Alt 2 needs less signaling overhead and specification impact, and can achieve the same function as Alt 1. |
| Huawei, HiSilicon | In Rel-15, when reporting TCI state number, there’s no clear definition whether these TCI states will be configured with same or different parameters. For example, TCI states with QCL typeA and B/C respectively are regarded as two TCI states, although QCL type B/C is subset of type A. So even it goes to Alt2, it won’t save TCI counting number because following Rel-15 design, the behavior that the UE drops some parameter will be regarded as two different behavior for TCI state counting still. Therefore, we support Alt 1 for a clear spec. |
| Samsung | Support Alt2. |
| Vivo | Alt 1 is a clear way, just as QCL-type A/B/C are specified individually, though the parameters of QCL-type A contain the parameters in QCL-type B and QCL-type C. Therefore, it’s natural to specify a new QCL-type for Rel-17 HST. |
| Futurewei | Support Alt-1 for clarity. It may need to appear in many places and it is cumbersome to repeat “QCL with a property ignored”. |
| Moderator | My recommendation is Alt-1 to be consistent with Rel-15 signalling framework. Other possible option is let RAN2 to decide the ignaling design between indication of QCL dropping and indication of a new QCL type |
| QC | @ZTE, we are fine with either switching approach (semi-static or dynamic) between scheme-1 and pre-compensation. Motivation for dynamic switching as they share similar processing. You can consider pre-compensation scheme is sub-set of scheme-1. |
| CATT | Alt 1 is more straightforward extension, and it can have less spec impact. Therefore, Alt 1 is supported. |
| ZTE2 | We don’t agree to introduce new QCL type as we mentioned above many times. No body answer my questions. Regarding Rel-15 design, actually QCL-Type C is only defined when SSB is as QCL source, it will not be used QCL reference of PDSCH/PDCCH after RRC connection. Then, there is no need to activate/indicate QCL type C. For QCL-TypeB, it is also not used for PDSCH and PDCCH. Please check 38.214 as below. It even rarely used in practice. Once we introduce new QCL type for PDSCH corresponding to new TCI state, that means the maximum X configured/activated TCI states by RRC or MACCE will be split to two groups, where one group is for e.g. STRP, the other is for SFN. Even a legacy TCI and a new TCI correspond to the same TRS and the same beam, they have to be configured twice and cannot be shared for SFN PDSCH and STRP PDSCH. The flexibility will be sacrificed since the maximum number of TCI states are limited. Further, new TCI states cannot be used for CSI-RS also. -------------------------38.214--------------- For the DM-RS of PDCCH, the UE shall expect that a TCI-State indicates one of the following quasi co-location type(s): - ‘typeA’ with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info and, when applicable, ‘typeD’ with the same CSI-RS resource, or - ‘typeA’ with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info and, when applicable, ‘typeD’ with a CSI-RS resource in an NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition, or - ‘typeA’ with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured without higher layer parameter trs-Info and without higher layer parameter repetition and, when applicable, ‘typeD’ with the same CSI-RS resource. For the DM-RS of PDSCH, the UE shall expect that a TCI-State indicates one of the following quasi co-location type(s): - ‘typeA’ with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info and, when applicable, ‘typeD’ with the same CSI-RS resource, or - ‘typeA’ with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info and, when applicable, ‘typeD’ with a CSI-RS resource in an NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition,or - typeA’ with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured without higher layer parameter trs-Info and without higher layer parameter repetition and, when applicable, ‘typeD’ with the same CSI-RS resource. |
| Nokia/NSB | This can be up to editor. No critical difference. But, how much UE’s freedom to take part of QCL parameters is allowed is discussion point. |

#### Round-3

| Company | Comment |
| --- | --- |
| Moderator | Please continue discussion and address ZTE concern on new QCL type. |
| Nokia/NSB | In some sense, this can be up to editor. No critical difference. But, if we allow UE’s freedom to take part of QCL parameters is allowed is discussion point. |
| ZTE | Two solutions have essential difference, should not be up to editor. We can further discuss it in next meeting. |
| Sony | Thank you Moderator for prompting Round 3 discussion. We are not here to address ZTE’s concern over new QCL type. We are sharing the same concern. Any new QCL-Type we have seen so far is a subset of QCL-TypeA. In our understanding, if QCL parameters dropping rule is specified or semi-statically configured, both standard impact and signaling overhead would be minimum. |
| Lenovo/MotM | Although Alt 2 has less spec impact, introducing a dropping rule for QCL parameter seems odd. QCL Type B, C can in fact be defined with a dropping rule w.r.t. QCL Typa A. Prefer Alt 1 in order to maintain the homogeneity and clarity of the specification |
| Futurewei | Prefer Alt 1, and suggest to further discuss. |
| CATT | Alt 1 is preferred. |
| Huawei, HiSilicon | Support Alt1. Note that in current QCL type categorization, QCL typeB/C are also a subset of type A. When some QCL parameters are dropped as in Alt 2, the QCL type would also need to be counted as a new one, as UE has to have a different channel estimation by excluding the dropped parameters. |
| Mod | It seems we need further discussion. Let’s plan to conclude on this issue in the next RAN1 meeting. |

### Issue #2-4 (Indication of the carrier frequency for UL)

Regarding indication of the carrier frequency for UL transmission. Several companies discussed this issue and provided their views, which are summarized below:

Issue#2-4: Indication of carrier frequency for uplink transmission in TRP-based pre-compensation schemes

- Option 1 Implicit from RAN1#102-e agreement

  - Supported by: OPPO, CATT, CMCC, Qualcomm, Intel, Samsung, [Futurewei], [vivo], [Lenovo / Motorola Mobility], [NTT DOCOMO], …

- Option 2 Explicit from RAN1#102-e agreement

  - Supported by: Ericsson, Sony, Intel, Nokia / NSN (if supported), [NTT DOCOMO], …

Companies are invited to share their views regarding indication option of the carrier frequency for UL transmission.

#### Round-1

Proposal #2-4:

- TBD

| Company | Comment |
| --- | --- |
| OPPO | Support Option 1. |
| Vivo | Support Option 1. |
| ZTE | Support Option 1 |
| Spreadtrum | Support Option 1. |
| Huawei, HiSilicon | Support Option 1 |
| Samsung | Support Option 1 |
| Apple | Option 1 |
| CATT | Support Option 1 |
| Nokia/NSB | Option 2. For option 1, please clarify if any specification impact exists. It should be clarified, if separate SRS resources should be transmitted to each TRP. Also, RAN4 study should guarantee the performance of frequency offset estimation via SRS. |
| Lenovo/MotM | Support Option 1. The path to specifying Option 2 is not clear, since introducing a new feedback parameter in CSI report would require several CSI enhancements related to reporting quantities, UCI design, time-domain behavior of CSI reporting configuration, and others. It is not clear whether/how this should be coordinated with AI 8.1.4. |
| QC | Suport Option 1. SRS enhancement to improve Doppler estimation should be discussed. As explained in our tdoc and also highlighted by Nokia, the current SRS is not designed to enable accurate Doppler shift/spread measurement which may lead to large frequency error that affect pre-compensation accuracy. |
| Ericsson | Option2: Explicit indication from UE shall be supported. We are also open for SRS enhancement. |
| LG | Support Option 1. |
| Futurewei | Support Option 1 |
| NEC | Support Option 1. |

#### Round-2

| Company | Comment |
| --- | --- |
| Moderator | Proponents of explicit reporting, i.e. Option 2, please clarify why the existing Doppler shift estimation used for UL demod is not sufficient to support TRP based pre-compensation. |
| Moderator 2 | The issue is currently captured as part of FFS for Issue #2-1 |
| Moderator 3 | No more discussion |

### Issue #2-5 (QCL-like association between DL and UL RS)

Regarding support of QCL-like association between DL and UL RS, e.g. for carrier frequency indication in UL. Several companies provided their views whether it requires specification support or can be up to UE implementation. Company’s preferences on this issue are summarized below:

Issue#2-5: Whether to support QCL-like association between DL and UL RS?

- Option 1: QCL-like association of the resource(s) received in the 1st step with UL signal transmitted in the 2nd step is supported by specification. FFS between the following alternatives:

  - Alt-1: Explicit indication of the DL RS for QCL-like association

  - Alt-2: Implicit indication of DL RS for QCL-like association

  - Supported by: Futurewei, CMCC, Qualcomm, [ZTE], [Lenovo/MotMobility], Sony, …

- Option 2: QCL-like association of the resource(s) received in the 1st step with UL signal transmitted in the 2nd step is supported by implementation without specification impact

  - Supported by: vivo, CATT, Ericsson (if supported), Samsung, Intel, OPPO, [LGE], [NEC], [Nokia/NSB], …

Companies are invited to share their preference on QCL-like association between DL and UL RS.

#### Round-1

Proposal #2-5:

- TBD

| Company | Comment |
| --- | --- |
| OPPO | Support Option 2. |
| Vivo | Support Option 2. |
| ZTE | Support Opiton 2 |
| Spreadtrum | Support Option 2. |
| Samsung | Support Option 2. |
| Apple | Option 2 |
| CATT | Support Option 2. |
| Nokia/NSB | Support Option 2. |
| Lenovo/MotM | Support Option 1, Alt-2. Explicit indication via unified TCI framework is still under discussion in AI 8.1.1, not clear how an agreement can be made that is based on an incomplete framework in another AI. We also believe specification is needed to ensure Doppler reciprocity, otherwise pre-compensation would not be precise |
| Ericsson | Option 2 shall be supported first. Option 1 can be considered if the pre-compensation is to compensate the measured frequency offset per TRP, instead of the difference between the two TRPs. |
| LG | Support option 2. |
| Futurewei | Support Option 1 |
| NEC | Support Option 2. |

#### Round-2

Proposal #2-5a:

- QCL-like association of the resource(s) received in the 1st step with UL signal transmitted in the 2nd step is supported by implementation without specification impact

| Company | Comment |
| --- | --- |
| Moderator | This is tentative proposal pending resolution for Issue #2-1 |

#### Round-3

Proposal #2-5a:

- QCL-like association of the resource(s) received in the 1st step with UL signal transmitted in the 2nd step is supported by implementation without specification impact

| Company | Comment |
| --- | --- |
| Moderator | Please indicate if you have any concern. To me it should be straightforward to agree on the proposal at least for FR1 |
| OPPO | Support. |
| LG | Support |
| vivo | Support. |
| Nokia/NSB | According to UL frequency offset estimation option, this is different. When single UL RS is used for the frequency offset estimation, the association is required, while explicit CSI report is used, it is upto implementation. |
| ZTE | Support |
| Ericsson | If either Alternative A, B or C is selected, this should be fine. We may clarify that condition in the proposal. |
| Lenovo/MotM | In our understanding, this issue is related to the need of switching the reference TRS when the train is moving towards/away from a TRP. We prefer implicit indication of the resource(s) which are associated with QCL Type A (e.g., in Variants A/C) |
| Futurewei | Fine with the majority view |
| CATT | Support |

#### Round-4

Proposal #2-5b:

- For Variant A, B or C (if supported)

  - QCL-like association of the resource(s) received in the 1st step with UL signal transmitted in the 2nd step for frequency offset pre-compensation is supported by implementation without specification impact

| Company | Comment |
| --- | --- |
| QC | Support |
| ZTE | Support |
| LG | Support |
| OPPO | Support |
| vivo | Support |
| Sony | Support |
| Spreadtrum | Support |
| Samsung | Support |
| CATT | Support |

### Issue #2-6 (Switching of TRP pre-compensation scheme)

Some companies have provided their views regarding configuration of TRP pre-compensation scheme and support of dynamic switching with legacy schemes. Company’s views on this issue are summarized below:

Issue#2-6: How to support switching/configuration of TRP pre-compensation with legacy Rel-15/Rel-16 schemes?

- Alt-1: Dynamic (DCI-based)

  - FFS which legacy schemes should support dynamic switching

  - Supported by: Qualcomm (scheme 1), Nokia/NSB (scheme 1), NTT DOCOMO, ZTE, Huawei …

- Alt-2: Semi-static (RRC-based)

  - Supported by: [InterDigital], , Apple, …

Companies are invited to share their preference on indication of TRP pre-compensation scheme.

#### Round-1

Proposal #2-6:

- TBD

| Company | Comment |
| --- | --- |
| OPPO | It depends on the supported variant for TRP pre-compensation. |
| DOCOMO | Support Alt.1 |
| vivo | The same as scheme 1 for consistence, since they are both SFN schemes |
| ZTE | The same as scheme 1 for consistence |
| Huawei, HiSilicon | Support Alt.1 |
| Samsung | The same as scheme 1 for consistency. |
| Apple | Alt-2 |
| CATT | Pre-compensation scheme can be indentified by indicating the new QCL type, e.g., QCL variant A, B or E, in TCI. |
| Nokia/NSB | Switching with S-TRP only based on TCI indication. Support of the scheme should be separately configured if supported. |
| Lenovo/MotM | Defer decision until pre-compensation scheme details are finalized |
| QC | Support Alt.1 Scheme1 and Pre-compensation share common UE processing and procedures. |
| Ericsson | At least support dynamic switch between S-TRP and TRP pre-compensation. |
| LG | We have the same view with vivo/ZTE/Samsung. |
| Futurewei | Support Alt-1 |

#### Round-2

| Company | Comment |
| --- | --- |
| Moderator | It seems there is majority that supports Alt 1. Proponents of Alt 2, please clarify problem of supporting dynamic switching for TRP based pre-compensation scheme. |
| Apple | No DCI based switching since it will change the QCL of DMRS/PDSCH. The fundamental question - Make it mandatory for operator and infra-vendor to support is before we can commit - How can gNB toggle between different scheme witin 0.5ms, based on what? Also, the same company who opposes pre-compensation, not support dynamic switching. What is the logic? Where is the sicnerecity? We have thought about it for a which, the only solution to make thing mutually mandatory for both infra-vendor and UE or just for infra-vendor for the discuission to become normal. |
| OPPO | We think the conlcusoin should be made after there is agreement on similar issue for scheme 1. Otherwise similar argument will occur again and again. |
| LG | Share the same view with OPPO |
| Moderator | Postpone until conclusion on support of dynamic switching is made for scheme 1 |

### Other issues

This section contains other issues that companies want to highlight for discussion regarding support of TRP-based pre-compensation scheme.

| Company | Comment |
| --- | --- |
| Futurewei | We think depending on the implementation, two different types of Variant E are needed: - Separate TRSs with two different Doppler shifts, and the DMRS with two different Doppler shifts (i.e., no gNB compensation for DMRS); - Separate TRSs with two different Doppler shifts, but the DMRS with only one Doppler shift (i.e., with gNB compensation for DMRS). Can this be discussed? Further details can be found in our tdoc. |
| Moderator | I suggest we take these aspects after finalizing Issue #2-2 |

## SFN transmission of PDCCH

### Issue #3-1 (MAC CE indication of TCIs for CORESET)

Regarding MAC CE indication. Several companies provided additional details on support of MAC CE based activation of two TCI states for PDCCH. Based on the discussion, the following proposal is made:

#### Round-1

Proposal #3-1:

- Introduce enhanced MAC CE signaling for PDCCH activating two TCI states for SFN-based PDCCH transmission

  - The corresponding MAC CE includes at least the following fields

    - Serving cell ID

    - CORESET ID

    - Two TCI state IDs

  - FFS whether for CA scenario additionally support RRC configured set of the serving cells which can be addressed by a single MAC CE

- Send LS to RAN2 to inform about agreement on support of enhanced MAC CE for CORESET in Rel-17

Companies to provide their preference on the proposal above.

| Company | Comment |
| --- | --- |
| OPPO | Support the proposal with the following restriction: - The enhanced MAC CE signaling is not applied to a CORESET configured with CORESETPoolindex. For CORESRTs grouped via different values of CORESETPoolindex, it is not needed to further activate two TCI states for a CORESET. Otherwise, UE may need to receive PDSCHs with more than two TCI states simultaneously if PDSCH follows the TCI states of PDCCH. |
| DOCOMO | Support. |
| Vivo | Support. |
| ZTE | Support. Regarding OPPO’s comment, it should be further discussed. |
| Xiaomi | Support the proposal |
| Spreadtrum | Support |
| Huawei, HiSilicon | We are fine with the proposal. |
| Samsung | Support |
| Apple | Okay |
| Sony | We are okay with the FL proposal. |
| CATT | Support FL’s proposal. Whether SFN-based PDCCH can be used in M-DCI needs further discussion. |
| Convida Wireless | Support. |
| Nokia/NSB | Support. |
| Lenovo/MotM | Support |
| APT/FGI | Support FL proposal |
| QC | Support |
| Ericsson | We are fine with FL’s proposal |
| LG | Support |
| Futurewei | Support |
| NEC | Support the proposal. |
| Moderator | The proposal is updated by adding second FFS to address comment from OPPO. |

Proposal #3-1a:

- Introduce enhanced MAC CE signaling for PDCCH activating two TCI states for SFN-based PDCCH transmission

  - The corresponding MAC CE includes at least the following fields

    - Serving cell ID

    - CORESET ID

    - Two TCI state IDs

  - FFS whether for CA scenario additionally support RRC configured set of the serving cells which can be addressed by a single MAC CE

  - FFS whether or not enhanced MAC CE signaling is applicable to a CORESET configured with CORESETPoolindex

- Send LS to RAN2 to inform about agreement on support of enhanced MAC CE for CORESET in Rel-17

### Issue #3-2 (Default TCI for single-TRP PDSCH)

Regarding default beam assumption for PDSCH. In the context of supporting two TCI states for PDCCH, several companies proposed to consider the issue of the default beam (TCI state) in different scenarios. In particular, default TCI state for Rel-15 single-TRP PDSCH reception. Based on the company’s contributions the following proposal is made.

#### Round-1

Proposal #3-2:

- If a CORESET is indicated with two TCI states, for Rel-15 single TRP PDSCH define rule(s) to determine one of the TCI states of the CORESET used as default beam for PDSCH reception

  - FFS the exact rule

| Company | Comment |
| --- | --- |
| OPPO | Support the proposal in principle. If neither of enableDefaultTCIStatePerCoresetPoolIndex and enableTwoDefaultTCI-States is configured, it is a typical S-TRP scenario for PDSCH, and one TCI state should be applied. |
| DOCOMO | Support. |
| Vivo | Support. |
| ZTE | Support in principle |
| Xiaomi | Support the proposal |
| Spreadtrum | Support |
| Samsung | Support in principle. |
| Apple | Okay |
| CATT | Support FL’s proposal. |
| Convida Wireless | Support, but suggest to use “activated with” instead of “indicated with”. |
| Nokia/NSB | The scenario should be justified before making decision. This is only for the case when SFN PDCCH + non-SFN S-TRP PDSCH are supported, and the use case should be justified first. We think that if SFN PDCCH is supported, PDSCH should be at least supporting SFN or M-TRP schemes. If then, the existing default QCL assumption for Rel-15 and Rel-16 can be used. |
| Lenovo/MotM | Support |
| APT/FGI | Support |
| LG | For the clarification, how can UE distinguish if it is for Rel-15 single TRP PDSCH? If neither of enableDefaultTCIStatePerCoresetPoolIndex and enableTwoDefaultTCI-States is configured, the UE can follow Rel-15 principle, i.e., the same TCI state(s) of the lowest CORESET ID in the latest slot or of the scheduling PDCCH |
| Futurewei | Support in principle |
| NEC | Support. |

#### Round-2

Proposal #3-2a:

- If a CORESET is indicated with two TCI states, for Rel-15 single TRP PDSCH support the following rule(s) to determine one of the TCI states of the CORESET used as default beam for PDSCH reception

  - If neither of enableDefaultTCIStatePerCoresetPoolIndex and enableTwoDefaultTCI-States is configured

    - Alt1: gNB ensures the lowest CORESET ID in the latest slot only configured one TCI state by implementation

    - Alt2: Modify the definition of the lowest CORESET ID in the latest slot, e.g. the lowest CORESET ID among the CORESETs associated with one TCI state in the latest slot.

    - Alt3: QCL assumption associated with one of TCI states, e.g. always selects the first or the second TCI state

| Company | Comment |
| --- | --- |
| Moderator | Let’s focus on the next level of details and clarify possible alternatives. Proponents of the proposal #3-2, please also provide response to Nokia question on the use case of SFN PDCCH and single-TRP PDSCH. |
| QC | How Alt4 is used with single TRP PDSCH? |
| ZTE | @Nokia At least we have fall back DCI which relies on CORESET for beam determination. When the CORESET is SFN, it is reasonable to choose one TCI for PDSCH scheduled by the fall back DCI. It is not proper to force UE receiving fall back PDSCH with SFN scheme. Even when SFN is enabled by RRC, STRP transmission can be still scheduled. One of two TCI states should be selected. |
| OPPO | We think the transmission schemes for PDCCH and PDSCH should be separate and determined individually by gNB. For example, when URLLC schemes are configured for PDSCH, it doesn’t mean that PDCCH repeeition should also be configured for PDCCH at the same time. It is up to gNB scheduling. On the proposal, we support Alt2. |
| Samsung | It seems Alt1 is too restrictive. We are fine Alt2 or Alt3. |
| Xiaomi | We are fine with Alt 2 or Alt 3. |
| Nokia/NSB | We still don’t see the use case. Do not support the proposal. @ZTE, for fallback DCI, if CORESET is non-SFN, PDSCH will follow the TCI/QCL assumption applied to the CORESET. UE can be configured with SFN CORESET and non-SFN CORESET. This is upto CORESET QCL. |
| Vivo | Slightly prefer Alt3. Alt 2 would introduce extra restriction, where the network must have a CORESET associated with one TCI state. |
| APT/FGI | We would like to clarify which scenario is assumed for the discussed default beam? Scheduling DCI without TCI field or scheduling offset below threshold? |
| Convida Wireless | Prefer Alt 3. For Alt 2, what if there is no CORESET associated with one TCI state in the latest slot? |
| Lenovo/MotM | We support the proposal in principle. We request updating Alt3 as follows Alt3: QCL assumption associated with one of TCI states, e.g. always selects the first or the second TCI state or the TCI state with a lower ID. We are concerned that Alt 1 may lead to flexibility limitations, since the lowest CORESET ID in the latest slot is not always a certain CORESET, it could be any CORESET. For example, in time t1 the lowest CORESET ID in the latest slot could be CORESET #1 and in time t2 the lowest CORESET ID in the latest slot could be CORESET #2. Besides, the TCI state of a CORESET is activated in MAC-CE level, if Alt 1 is adopted maybe TCI state of most CORESETs will be activated with only one. Therefore we support either Alt 2 or Alt 3. |
| Moderator | Updated proposal based on the feedback above and comments for issue #3-3: Proposal #3-2b: - If a CORESET is indicated with two TCI states, for Rel-15 single TRP [and Rel-16 scheme 3/4] PDSCH support the following rule(s) to determine one of the TCI states of the CORESET used as default beam for PDSCH reception - If neither of enableDefaultTCIStatePerCoresetPoolIndex and enableTwoDefaultTCI-States is configured down-select one alternative - Alt1: gNB ensures the lowest CORESET ID in the latest slot only configured one TCI state by implementation - Alt2: Modify the definition of the lowest CORESET ID in the latest slot, e.g. the lowest CORESET ID among the CORESETs associated with one TCI state in the latest slot. - Alt3: QCL assumption associated with one of TCI states, e.g. always selects the first or the second TCI state or the TCI state with a lower ID |
| CATT | Alt 3 is preferred. Alt 1 too restrictive to NW. So If neither of enableDefaultTCIStatePerCoresetPoolIndex and enableTwoDefaultTCI-States is configured, UE assumes that the QCL of PDSCH is associated with the first one of two TCI states of the CORESET. |

#### Round-3

Proposal #3-2b:

- If a CORESET is indicated with two TCI states, for Rel-15 single TRP [and Rel-16 scheme 3/4] PDSCH support the following rule(s) to determine one of the TCI states of the CORESET used as default beam for PDSCH reception

  - If neither of enableDefaultTCIStatePerCoresetPoolIndex and enableTwoDefaultTCI-States is configured down-select one alternative

    - Alt1: gNB ensures the lowest CORESET ID in the latest slot only configured one TCI state by implementation

    - Alt2: Modify the definition of the lowest CORESET ID in the latest slot, e.g. the lowest CORESET ID among the CORESETs associated with one TCI state in the latest slot.

    - Alt3: QCL assumption associated with one of TCI states, e.g. always selects the first or the second TCI state or the TCI state with a lower ID

| Company | Comment |
| --- | --- |
| Moderator | Propose to take Proposal #3-2b as offline agreement. |
| Apple | No need for this agreement in this meeting This is about the sTRP/TDM schemes with HST-SFN PDCCH, very strange deployment Why this has anything to do with “enableDefaultTCIStatePerCoresetPoolIndex” which is for the mDCI mTRP. But the main bullet is for sTRP for sDCI mTRP operation Or we can just for further study the issue. |
| OPPO | Re Apple’s question, the condition “If neither of enableDefaultTCIStatePerCoresetPoolIndex and enableTwoDefaultTCI-States is configured” is because only in this case, the PDSCH will follow the TCI state(s) of CORESET with the lowest ID, which can be two in R17. Otherwise, it will follow the TCI state for S-DCI or M-DCI based M-TRP, e.g. acquired from MAC CE or CORESETPoolindex. |
| QC | Similar views with Apple. We need to discuss the use-case and deployment scenario in which SFN PDCCH schedules sTPR PDSCH. In our views, if the PDCCH is SFN (Scheme-1), then PDSCH should be SFN as well. It is hard to understand why scheme-1 is applied only to PDCCH and not applied to PDSCH. |
| LG | We have similar view with Apple/QC. In our perspective, before making the decision, it seems that more clarification is needed because alternatives in proposal #3-2b and Alt2 in proposal #3-4 would make different UE behavior. - Case A: If neither of enableDefaultTCIStatePerCoresetPoolIndex and enableTwoDefaultTCI-States is configured - Case A-1: scheduling offset < timeDurationForQCL - Need decision for the case that the CORESET with the lowest controlResourceSetId in the latest slot is configured with two TCI states - Case A-2: scheduling offset >= timeDurationForQCL & scheduling PDCCH without TCI field - Need decision for the case that the scheduling PDCCH is configured with two TCI states - Case B: If either of enableDefaultTCIStatePerCoresetPoolIndex and enableTwoDefaultTCI-States is configured - Case B-1: scheduling offset < timeDurationForQCL - Need decision for the case that the CORESET with the lowest controlResourceSetId in the latest slot is configured with two TCI states when enableDefaultTCIStatePerCoresetPoolIndex is configured - Case B-2: scheduling offset >= timeDurationForQCL & scheduling DCI without TCI field - Need decision for the case that the scheduling PDCCH is configured with two TCI states We should consider default beam behavior for four cases above. If we support proposal #3-2b, only one of the two TCI states configured in CORESET can be applied to PDSCH for the case of Alt3, and only CORESET in which a single TCI state is configured can be used to determine PDSCH TCI state for the case of Alt1/2. But, if we support Alt2 in proposal #3-4, one or two TCI states can be applied to PDSCH depending on the number of TCI states configured to CORESET. So, I think proposal #3-2b and #3-4 are different alternatives. If my understanding is correct, we cannot support both Alt2 in proposal #3-4 and one of alts in proposal #3-2. Please let me know if my understanding is not correct. |
| Vivo | Support, but please cancel the [] for ‘and Rel-16 scheme 3/4’ if no companies have the concern about adding ‘and Rel-16 scheme 3/4’. |
| Nokia/NSB | No need to consider this option. |
| ZTE | Support FL proposal |
| OPPO | We support the FL proposal. If gNB wants to configure SFNed PDSCH, it can configure enableTwoDefaultTCI-States and activate two TCI states for at least one TCI codepoint, in which case two TCI states will be applied by default. Furthermore, if only one TCI state is activated for all the TCI codepoints, then using two TCI states as default TCI states will mandate UE to support dynamic switching between S-TRP and SFNed transmission, since the number of TCI states is different for cases of smaller than threshold and larger than threshold. We prefer to decoupling transmission of PDCCH and PDSCH, which gives gNB more flexibility. |
| DOCOMO | Support, and support Alt.3. We understand this section of 2.3. SFN transmission of PDCCH is not limited with HST, but for reliability enhancement. Default TCI state for PDSCH is needed for SFN PDCCH. |
| Ericsson | We are fine to discuss this later. Our preference is Alt 3. |
| Moderator | Re Apple Per Chairman SFN PDCCH discussion in this agenda item should cover not only HST, but also generic deployment scenario. |
| Lenovo/MotM | Support proposal. Prefer Alt 3 |
| Xiaomi | Support the proposal, and we prefer alt 2 or alt 3 |
| CATT | Alt 3 is preferred. |

### Issue #3-3 (Default TCI for Rel-16 multi-TRP PDSCH)

Several companies have proposed to define default TCI state for Rel-16 multi-TRP PDSCH schemes with repetition when scheduled by PDCCH transmitted from CORESET indicated with two TCI states. Based on the company’s contributions the following proposal is made.

#### Round-1

Proposal #3-3:

- If a CORESET is indicated with two TCI states, for Rel-16 multi-TRP PDSCH schemes 3,4 define rule to determine default beam for PDSCH reception

  - FFS the exact rule

Companies to provide their views on the proposal above.

| Company | Comment |
| --- | --- |
| OPPO | If enableTwoDefaultTCI-States is not configured, it is a single TRP scenario, and should be discussed in issue #3-2. If enableTwoDefaultTCI-States is configured, the default TCI state will be the TCI states corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states. Hence, it is unclear to us whether additional rule is needed. |
| DOCOMO | Support. We assume the both of two TCI states are applied to the PDSCH repetition. |
| Vivo | Support. |
| ZTE | There is no spec impact for this case. |
| Xiaomi | Support the proposal |
| Spreadtrum | Share the same view with OPPO, no enhancement is needed. |
| Samsung | There is no spec impact since the default beam behavior for PDSCH schemes 3 and 4 was defined regardless of the number of TCI states for CORESET. |
| Apple | Okay |
| CATT | Same view as OPPO, ZTE Spreadtrum and Samsung. |
| Convida Wireless | Question for clarification: Is the intention to support using the two activated CORESET TCI states for the TDMed PDSCH when the TCI field is not present in the DCI? |
| Nokia/NSB | Agree with OPPO’s comment. |
| Lenovo/MotM | We believe the situation depends on whether a TCI field present in DCI. If DCI contains TCI field, whether a scheduled PDSCH transmitted from a single TRP or multi-TRP, the default beam for multi-TRP PDSCH follows the lowest TCI codepoint containing two TCI states which was specified in Rel-16 already. If DCI doesn’t contain TCI field, the default beam for PDSCH reception needs to be determined for all multi-TRP PDSCH schemes, not only schemes 3,4. |
| APT/FGI | We are OK to discuss it. |
| LG | There is no spec impact. |
| Futurewei | OK to discuss |
| NEC | Support. |

#### Round-2

| Company | Comment |
| --- | --- |
| Moderator | Proponents of the proposal #3-3. Please clarify the specification impact. |
| Vivo | If enableTwoDefaultTCI-States is not configured, scheme 3/4 have the same problem as STRP-based PDSCH. We suggest merging issue #3-3 to issue #3-2, e.g. add the wording ‘scheme 3/4’ in the proposal #3-2: - If a CORESET is indicated with two TCI states, for Rel-15 single TRP PDSCH and Rel-16 scheme 3/4, support the following rule(s) to determine one of the TCI states of the CORESET used as default beam for PDSCH reception - If neither of enableDefaultTCIStatePerCoresetPoolIndex and enableTwoDefaultTCI-States is configured - Alt1: gNB ensures the lowest CORESET ID in the latest slot only configured one TCI state by implementation - Alt2: Modify the definition of the lowest CORESET ID in the latest slot, e.g. the lowest CORESET ID among the CORESETs associated with one TCI state in the latest slot. - Alt3: QCL assumption associated with one of TCI states, e.g. always selects the first or the second TCI state |
| Convida Wireless | Generally fine with the proposal, but it would be beneficial to clarify cases in which an enhancement is needed, e.g. TCI isn’t present in the DCI, offset is greater than threshold, etc. |
| Lenovo/MotM | If DCI has no TCI field, we don’t see any specification impact. If DCI is without TCI field and scheduling offset is equal to or larger than timeDurationForQCL, in Rel-16, the default TCI of PDSCH always follows the scheduling CORESET. However, in Rel-16 only one TCI state could be activated for a CORESET. Clarification is needed for Rel-17, e.g., whether one of the two TCI states or both TCI states of the scheduling CORESETs are used as the default TCI state for PDCSH reception |
| Moderator | Merged discussion with Issue #3-2. No more discussion. |

### Issue #3-4 (Default TCI for Rel-17 SFN PDSCH)

Several companies have proposed to define new default TCI state rule for Rel-17 enhanced SFN PDSCH transmission scheme, when scheduled by PDCCH transmitted from CORESET indicated with two TCI states. Based on the company’s contributions the following proposal is made.

#### Round-1

Proposal #3-4:

- If a CORESET is indicated with two TCI states for Rel-17 SFN PDSCH scheme

  - Alt 1. Define new rule to determine default beam for PDSCH

    - FFS the exact rule

  - Alt 2. Reuse Rel-16 rule to determine default beam for PDSCH

Companies to provide their views on the proposal above including preference for the specific alternative.

| Company | Comment |
| --- | --- |
| OPPO | Support Alt 2. Rel-16 rule, e.g. the default TCI state is the TCI states corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states, is sufficient for Rel-17 SFN. |
| DOCOMO | We’d like to know the difference between Alt.1 and Alt.2. Is it correct understanding that Alt.2 does not change the rule to determine which CORESET is selected as default beam, and we only specify how to select one of the two TCI states configured in the CORESET? If so, we prefer this (Alt.2). |
| vivo | Slightly prefer Alt 2. |
| ZTE | The same view as OPPO |
| Xiaomi | Support Alt 2. If the TCI field is not present, and the time offset is equal to or greater than a threshold, the default TCI state is the TCI states correspongding to the CORESET. If the time offset is less than a threshold, the default TCI state is the TCI states correspongding to the lowest codepoint with two TCI states. |
| Spreadtrum | Support Alt.2 |
| Apple | Alt 2 |
| CATT | Support Alt 2. |
| Convida Wireless | A question for OPPO, if I may (for my understanding): Is the assumption that the needs to support defaultQCL-TwoTCI-r16 to support a Rel-17 SFN PDSCH scheme? A question for Xiaomi, if I may (for my understanding): For the first case you describe, it seems the Rel-16 spec does not consider the case that two TCI states are activated for the CORESET: “the UE assumes that the TCI state or the QCL assumption for the PDSCH is identical to the TCI state or QCL assumption whichever is applied for the CORESET used for the PDCCH transmission”? |
| Nokia/NSB | Support Alt 2, using Rel-15 or Rel-16 default QCL according to MAC-CE inclusion of TCI codepoint with two TCI states. |
| Lenovo/MotM | Support Alt 2 |
| APT/FGI | We share similar questions as Convida. More clarications/discussions would be needed. |
| LG | If either of enableDefaultTCIStatePerCoresetPoolIndex and enableTwoDefaultTCI-States is configured, the UE can follow Rel-16 principle. But, if neither of them is configured, the UE can follow Rel-15 principle, i.e., the same TCI state(s) of the lowest CORESET ID in the latest slot or of the scheduling PDCCH |
| Futurewei | Prefer Alt 2 |
| NEC | Support Alt 2. |

#### Round-2

| Company | Comment |
| --- | --- |
| Moderator | It seems most of the companies prefer to reuse the existing default TCI state for this scenario. Proponents of Alt 1, please clarify the necessity of the default TCI state change for Rel-17 SFN PDSCH. |
| Moderator | No more discussion |
| QC | We are fine with re-using Rel-16 rules. The proposal covers only the default beam. Further clarification is needed for the case when TCI field not present and the time offset is equal to or greater than a threshold. In such case, the SFN PDSCH is to assume the two TCI states of the CORESET. |

### Issue #3-5 (Default TCI for aperiodic CSI-RS)

Regarding default beam assumption for aperiodic CSI-RS. Several companies proposed to define new beam assumptions for aperiodic CSI-RS reception in Rel-17 when CORESET is indicated with two TCI states. Based on the company’s contributions the following proposal is made.

#### Round-1

Proposal #3-5:

- If a CORESET is indicated with two TCI states, and scheduling offset for AP CSI-RS is less than the threshold

  - Alt 1: Use one of two TCI states as default beam for aperiodic CSI-RS reception

  - Alt 2: Use two TCI states of CORESET as default beam for aperiodic CSI-RS reception

Companies to provide their views on the proposal above including preference for the specific alternative.

| Company | Comment |
| --- | --- |
| OPPO | Alt 2 can’t be implemented by UE acturally. For example, for AP CSI-RS for CSI feedback, different TCI states will lead to different measurement results. How can UE use both TCI states for CSI measurement? |
| Vivo | Support Alt 1. It seems Alt 2 implies that SFN-based CSI-RS is transmitted. However, TRS is TRP-specific, CSI-RS should also be TRP-specific in HST-SFN. |
| ZTE | Alt 2 cannot be supported as CSI-RS cannot be SFN based. This proposal is not used when there is no ther signals in the same symbol as AP CSI-RS and UE does not support Rel-16/17 default beam feature |
| Xiaomi | Alt 1 and alt 2 may be applied in different cases respectively. |
| Spreadtrum | Support Alt 1 |
| Samsung | Support Alt1. As ZTE mentioned, whether there is other signals in the same symbol as AP CSI-RS or not should be considered. |
| Apple | Alt 1. |
| CATT | Support Alt 1. |
| Convida Wireless | Support Alt 1. |
| Nokia/NSB | Slightly prefer Alt 2, but we can discuss later after M-TRP CSI in AI8.1.4 completed. To vivo, QCL assumption doesn’t imply SFN CSI transmission. It is related UE’s assumption for RX operation not to miss reception any of beams before PDCCH decoding. gNB can transmit one or two of two TCIs for AP CSI-RS. |
| Lenovo/MotM | Support Alt 1 |
| LG | Support Alt1 |
| Futurewei | Ok to discuss further |
| NEC | Support Alt 1. |

#### Round-2

Proposal #3-5a:

- If a CORESET is indicated with two TCI states, and scheduling offset for AP CSI-RS is less than the threshold

  - Use one of two TCI states as default beam for aperiodic CSI-RS reception using the same principles as for default TCI state for Rel-15 single TRP PDSCH case

  - FFS Use two TCI states of CORESET as default beam for aperiodic CSI-RS reception

| Company | Comment |
| --- | --- |
| Moderator | Seems Alt 1 has more support. Let’s agree on Alt 1 and continue discussion on other details including whether Alt 2 should be also supported. |
| Apple | Which one? In our view, it is just up for UE implementation. |
| QC | support |
| ZTE | Support. Following Rel-15, 16 principle, one of TCI states should be used for AP CSI-RS. We don’t think it should be up to UE implementation since the used beam for UE and gNB should be aligned for accuracy CSI measurement. |
| Ericsson | The proposal is unclear and incomplete. There are scenarios of overlapping and non-overlapping with any signals. The proposal seems to deal with the non-overlapping case. Even in that case, there are rules defined in Rel-16 when two TCI states are activated to at least one codepoint of TCI field in DCI and with/without enable default TCI states and rules defined in Rel-15 for s-TRP with/without CORESET in the same BWP. The two TCI states activated for a CORESET in Rel-17 affects only the Rel-15/16 rule related to a CORESET, i.e. if a UE is not configured with enableTwoDefaultTCIStates and if at least one CORESET is configured for the BWP in which the aperiodic CSI-RS is received, when receiving the aperiodic CSI-RS, the UE applies the QCL assumption of one of the two TCI state activated for the CORESET associated with a monitored search space with the lowest controlResourceSetId in the latest slot in which one or more CORESETs within the active BWP of the serving cell are monitored. |
| Samsung | Support the FL proposal in principle but whether there are other DL signals overlapped in the same symbol as AP CSI-RS or not should be clarified. Is the proposal for no overlapped DL signal case? |
| Xiaomi | Support |
| Nokia/NSB | Still unclear with alt 1. If two TCIs are indicated for PDCCH, either TCIs can be scheduled for AP CSI-RS. UE assumption can be both, but one of them may be received. |
| Convida Wireless | Support |
| Lenovo/MotM | Support. In Rel 15/16, only one TCI state is used for AP CSI-RS reception, we prefer following the same principle. On the determination of one TCI state, we believe same rule in issue #3-2 could be reused. |
| Moderator | The proposal is updated to clarify that proposed default TCI state for the case of no overlapping DL signal. Proposal #3-5b: - If a CORESET is indicated with two TCI states, and scheduling offset for AP CSI-RS is less than the threshold and there is no other overlapping DL signal with indicated TCI state - Use one of two TCI states as default beam for aperiodic CSI-RS reception using the same principles as for default TCI state for Rel-15 single TRP PDSCH case - FFS Use two TCI states of CORESET as default beam for aperiodic CSI-RS reception |
| CATT | Support Proposal#3-5b. |

#### Round-3

Proposal #3-5b:

- If a CORESET is indicated with two TCI states, and scheduling offset for AP CSI-RS is less than the threshold and there is no other overlapping DL signal with indicated TCI state

  - Use one of two TCI states as default beam for aperiodic CSI-RS reception using the same principles as for default TCI state for Rel-15 single TRP PDSCH case

  - FFS Use two TCI states of CORESET as default beam for aperiodic CSI-RS reception

| Company | Comment |
| --- | --- |
| Moderator | Proponents please focus on fine tuning of the wording for the proposal. In case there are concerns, I suggest we take this for study for the next meeting. |
| Nokia/NSB | Though I don’t have strong concern on Rel-15 based approach, it is somewhat unclear. For example, if UE receives with TCI1+TCI2 for PDCCH, and TCI1 is a default beam for AP CSI-RS. If train approaches to TCI2, then should we switch beams to TCI2+TCI1 and TCI2 becomes the default beam in the case? We still have question why PDCCH is received by two TCI but only one TCI should be assumed for AP CSI-RS. UE is already receiving two beams for PDCCH, then keeping this reception for AP CSI-RS is natural choice. AP-CSI scheduling can be one of two TCI states but UE should assume any one of two beams can be received. |
| ZTE | Support |
| OPPO | Support. If two TCI states are applied, is which TCI state to use up to UE implementation? Then how can gNB know which TCI state is applied at UE. |
| DOCOMO | Support |
| Lenovo/MotM | Support |
| Xiaomi | Support |
| CATT | Support |

### Issue #3-6 (Default spatial relation for single-TRP PUSCH/PUCCH/SRS)

In the context of supporting two TCI states for CORESET, several companies have mentioned the issue of default uplink beam(s) and PL-RS for dedicated-PUSCH/PUCCH/SRS transmission to a single TRP. Based on the company’s contributions the following proposal is made.

#### Round-1

Proposal #3-6:

- If a CORESET is indicated with two TCI states, for single-TRP PUSCH/PUCCH/SRS transmission define rule(s) to determine one of the TCI states of the CORESET used as default beam and PL-RS

  - FFS the exact rule

Companies to provide their preference on the proposal above.

| Company | Comment |
| --- | --- |
| OPPO | This issue can be discussed after there is conclusion on PDSCH. |
| DOCOMO | Support. We support to select one of two TCI states of the CORESET used as default beams for M-TRP PUSCH/PUCCH/SRS transmission. |
| Vivo | Support. |
| ZTE | Discuss it later |
| Xiaomi | Support the proposal |
| Spreadtrum | Support |
| Samsung | Support FL’s proposal |
| Apple | We think this should be low priority, i.e., default beam for UL |
| CATT | Support |
| Convida Wireless | It can be discussed later. |
| Nokia/NSB | Discuss it later. This can be discussed later after AI 8.1.1 decision. |
| Lenovo/MotM | Support |
| APT/FGI | Support FL proposal |
| Futurewei | Ok to discuss it later |
| NEC | Support the proposal. |
| Moderator | It seems no technical concern on the proposal itself, but some companies prefer to discuss this topic later. |

### Issue #3-7 (Default spatial relation for Rel-17 multi-TRP PUSCH/PUCCH)

If a CORESET is indicated with two TCI states, several companies proposed to define rule to determine default beam for Rel-17 multi-TRP PUSCH/PUCCH transmission scheme with repetition. Based on the discussion the following proposal is made.

#### Round-1

Proposal #3-7:

- If a CORESET is indicated with two TCI states, support two TCI states of the CORESET used as default beams for Rel-17 Multi-TRP PUSCH/PUCCH repetition scheme

  - FFS the exact rule

Companies to provide their views on the proposal above.

| Company | Comment |
| --- | --- |
| OPPO | This issue can be discussed after there is conclusion on PDSCH. |
| DOCOMO | Support. We support both two TCI states of the CORESET used as default beams for each transmission of M-TRP PUSCH/PUCCH repetition. |
| Vivo | Discuss it later |
| ZTE | Discuss it later |
| Xiaomi | Discuss it later |
| Spreadtrum | Discuss it later |
| Samsung | Support. It is also fine for discussing in later. |
| Apple | We think this should be low priority |
| CATT | Discuss it later |
| Convida Wireless | Discuss it later |
| Nokia/NSB | Discuss it later. |
| Lenovo/MotM | Support |
| APT/FGI | Support |
| Futurewei | Ok to discuss it later |
| NEC | Discuss it later. |
| Moderator | It seems majority of companies prefer to discuss this topic later. |

### Issue #3-8 (Issues related to BFR support)

Several companies have mentioned several BFR issues that should be addressed for the UE configured with CORESET associated with two TCI states. Based on the company’s contributions the following proposal is made.

#### Round-1

Proposal #3-8:

- When two TCI states are configured for a CORESET, support the following enhancements to BFR procedures

  - Configuration of RS for BFD

    - Implicit configuration

      - Alt 1-1: RS of CORESETs with only single active TCI states are used

      - Alt 1-2: RS of CORESETs with both single and two TCI states are used

    - Explicit configuration

      - Alt 2-1 Support defining CSI-RS resource or SSB pairs

        - FFS other details

      - Alt 2-2 Reuse the existing approach for BFD RS configuration

  - Assumptions for hypothetical BLER calculation for PDCCH

      - Alt 3-1: RS in the two TCI states or CSI-RS / SSB pairs (if supported) are directly used as the BFD RS

      - Alt 3-2: UE calculates one hypothetical BLER under SFN assumption of BFD RS pairs

  - Configuration of NBI RS

      - Alt 4-1: Reuse the existing Rel-15 NBI configuration based on single CSI-RS resource

      - Alt 4-2: Introduce two new beam identification CSI-RS resource sets or new beam identification CSI-RS resource pairs

  - FFS applicability of the BFR enhancements above to

    - Rel-15 BFR

    - Rel-16 BFR

    - Rel-17 BFR

  - FFS UE behavior on monitoring the PDCCH candidate after BFD

- FFS applicability of some enhancements to RLM procedures

- FFS other aspects

Companies to provide their views on the proposal above including necessity of the BFR enhancements and refinements of the proposals.

| Company | Comment |
| --- | --- |
| DOCOMO | Configuration of RS for BFD - Support Alt.1-2. When a CORESET is indicated with two TCI states, for RLM and per cell BFD, RSs in both TCI states of the CORESET can be used as detection RS so that channel condition between UE and both TRPs are taken into account. - Support Alt.2-1. When RLM/BFD RS set is explicitly provided, for a CORESET indicated with two TCI states, we’d like to study how to ensure the detection RS set includes RSs in both TCI states of a CORESET. Assumptions for hypothetical BLER calculation for PDCCH - Support Alt 3-1. Configuration of NBI RS - Support Alt 4-2 Applicability of the BFR enhancements - Shoul be applied to all of Pcell/PSCell BFR, Scell BFR, and per TRP BFR. Enhancements to RLM procedures - RLM RS should be also considered in the proposa. The issue of explicit/implicit RLM RS configuration is the same as explicit/implicit BFD RS configuration. |
| ZTE | Discuss it later |
| Xiaomi | Support the proposal |
| Spreadtrum | Discuss it later |
| Samsung | We can discuss this later. |
| Apple | Discuss it later |
| CATT | This issue can be discussed under AI 8.1.2.3. |
| Convida Wireless | Agree with CATT. |
| Nokia/NSB | Discuss it later. |
| Lenovo/MotM | Support Alt.2-1, Alt.3-2, Alt.4-2 on account that new BFR mechanism is defined to align with newly specified SFN based PDCCH transmission scheme. |
| APT/FGI | Support in principle. This issue may be related to TRP BFR in 8.1.2.3. In addttion, on the first sub-bullet, we may also need to discuss whether there is one or two BFD-RS set. |
| Futurewei | Ok to discuss it later |
| NEC | We support the discuss the issues, and fine to disucss them later. |

#### Round-2

| Company | Comment |
| --- | --- |
| Moderator | I suggest we take the proposal #3-8 as offline agreement. Please indicate, if there are any concerns. Meanwhile I’ll also check with Mr. Chairman, which agenda item should continue discussion on the BFR aspects when CORESET is indicated with two TCI states. |
| Apple | The main body says support The sub-bullets list alternatives We are confused. |
| QC | Lets clarify first. |
| ZTE | Study is fine. No rush to agree these options. |
| Samsung | Okay to study but lower priority. |
| LG | For the clarification, in Proposal #3-8, are two different values of coresetPoolIndex taken into account or not? If we consider only single coresetPoolIndex in this issue, can I ask the motivation of Alt 4-2? |
| Xiaomi | Support the proposal |
| Nokia/NSB | This is out of this AI’s scope. This should be discussed under BM AI 8.1.2.3. Also, it should be aligned with the agreement of AI8.1.2.1 PDCCH enhancement. |
| Ericsson | The intention here is to study the alternatives and downselect? If so “supporting the following enhancements to BFR procedures’ in the main bullet is not appropriate. Better to say “study” than “supporting”. |
| Vivo | Prefer to clarify first. Alt 3-1 and 3-2 are not very clear for us, what is the difference between them? |
| APT/FGI | We support studying it. |
| Convida Wireless | We’d like to have some more time to study this. |
| Lenovo/MotM | Support studying |
| QC | Suggest adding one more alternative for implicit configuration: Alt 1-3: RS of CORESET(s) with only two TCI states are used |
| CATT | It should be clarified about which A.I. should this issue be discussed first. |

#### Round-3

Proposal #3-8a:

- When two TCI states are configured for a CORESET, study the following enhancements to BFR procedures

  - Configuration of RS for BFD

    - Implicit configuration

      - Alt 1-1: RS of CORESETs with only single active TCI states are used

      - Alt 1-2: RS of CORESETs with both single and two TCI states are used

      - Alt 1-3: RS of CORESET(s) with only two TCI states are used

    - Explicit configuration

      - Alt 2-1 Support defining CSI-RS resource or SSB pairs

        - FFS other details

      - Alt 2-2 Reuse the existing approach for BFD RS configuration

  - Assumptions for hypothetical BLER calculation for PDCCH

      - Alt 3-1: RS in the two TCI states or CSI-RS / SSB are directly used as the BFD RS

      - Alt 3-2: UE calculates one hypothetical BLER under SFN assumption of BFD RS pairs

  - Configuration of NBI RS

      - Alt 4-1: Reuse the existing Rel-15 NBI configuration based on single CSI-RS resource

      - Alt 4-2: Introduce two new beam identification CSI-RS resource sets or new beam identification CSI-RS resource pairs

  - FFS applicability of the BFR enhancements above to

    - Rel-15 BFR

    - Rel-16 BFR

    - Rel-17 BFR

  - FFS UE behavior on monitoring the PDCCH candidate after BFD

- FFS applicability of some enhancements to RLM procedures

- FFS other aspects

| Company | Comment |
| --- | --- |
| Moderator | Let’s check whether we can agree on the above proposal |
| QC | We are fine with the study proposal Question to Moderator: has it been clarified that BFR aspects of two TCI sates CORESET will be handled in 8.1.2.4 agenda? |
| Vivo | Thanks for Moderator to clarify Alt 3-1 and 3-2, now we are ok with the proposal. |
| Nokia/NSB | Again, this is out of scope, and similar discussion is happening in AI8.1.2.3 (M-TRP BM) |
| Moderator | Re QC Based on current discussion with Chairman the BFR enhancement related to HST scenario should be discussed in HST agenda item. |
| ZTE | We are fine to study |
| DOCOMO | Support |
| Lenovo/MotM | Support |
| Xiaomi | Support |
| CATT | Support to study. |
| ZTE2 | Perhaps, it is sufficient to agree the main bullet as follows When two TCI states are configured for a CORESET, study the following enhancements to BFR procedures |
| Huawei, HiSilicon | Not support. Agree with Nokia that it is out of scope. Before we study the specific solutions, we should discuss whether it is necessary for discussing BFR here. |
| Sony | We can live with studying BFR for SFN. However, our comment on it would since there are a few progresses in AI 8.1.2.3 BFR for M-TRP, we better to strive to reuse it for HST-SFN to avoid duplicated work. In our view, BFR for SFN transmission seems not the main target in AI 8.1.2.4. |
| Nokia/NSB | Support E///’s clarification when GTW session, which clarifies this is applicable when SFN PDCCH is used. When two TCI states are configured activated for a CORESET. |

#### Round-4

Proposal #3-8b:

- When two TCI states are activated for a CORESET, study the following enhancements to BFR procedures

  - Configuration of RS for BFD

    - Implicit configuration

      - Alt 1-1: RS of CORESETs with only single active TCI states are used

      - Alt 1-2: RS of CORESETs with both single and two TCI states are used

      - Alt 1-3: RS of CORESET(s) with only two TCI states are used

    - Explicit configuration

      - Alt 2-1 Support defining CSI-RS resource or SSB pairs

        - FFS other details

      - Alt 2-2 Reuse the existing approach for BFD RS configuration

  - Assumptions for hypothetical BLER calculation for PDCCH

      - Alt 3-1: RS in the two TCI states or CSI-RS / SSB are directly used as the BFD RS

      - Alt 3-2: UE calculates one hypothetical BLER under SFN assumption of BFD RS pairs

  - Configuration of NBI RS

      - Alt 4-1: Reuse the existing Rel-15 NBI configuration based on single CSI-RS resource

      - Alt 4-2: Introduce two new beam identification CSI-RS resource sets or new beam identification CSI-RS resource pairs

  - FFS applicability of the BFR enhancements above to

    - Rel-15 BFR

    - Rel-16 BFR

    - Rel-17 BFR

  - FFS UE behavior on monitoring the PDCCH candidate after BFD

- FFS applicability of some enhancements to RLM procedures

- FFS other aspects

### Issue #3-9 (Identification of SFN PDCCH transmission)

Several companies have mentioned the issue of identification of the Rel-17 enhanced SFN PDCCH. Based on this discussion, the following proposal is made:

Issue #3-9:

- Identification of the enhanced SFN PDCCH transmission scheme (scheme 1 and TRP-based pre-compensation)

  - Alt 1: RRC parameter

    - Supported by: Qualcomm, …

  - Alt 2: MAC CE with two TCI states

    - Supported by: Vivo, …

#### Round-1

Proposal #3-9:

- TBD

Companies to provide their preference on the alternatives above.

| Company | Comment |
| --- | --- |
| OPPO | Alt 1 is preferred. |
| vivo | Support Alt 2 |
| ZTE | In our view, both RRC and MACCE are needed. RRC is to enable this Rel-17 feature, and MACCE is to activate/update two TCI states |
| Samsung | Both are needed. |
| Apple | Alt 1 |
| CATT | This can be discussed later. Similar approach, i.e., based on indicated QCL type, as in PDSCH can be considered. |
| Convida Wireless | Agree with ZTE. |
| Lenovo/MotM | Support Alt.2-1, Alt.3-2, Alt.4-2 on account that new BFR mechanism is defined to align with newly specified SFN based PDCCH transmission scheme. |
| LG | Support Alt2. We already agreed on MAC CE enhancement, and this MAC CE can be used for identification of SFN PDCCH. |
| Futurewei | Agree with ZTE |
| NEC | Agree with ZTE. |

#### Round-2

Proposal #3-9a:

- Both RRC and MAC CE with two TCI states are used for identification of the enhanced SFN PDCCH transmission scheme

| Company | Comment |
| --- | --- |
| Moderator | Companies to provide additional details of RRC configuration for the modified proposal #3-9a |
| QC | Support. RRC parameter configuration per CORESET |
| ZTE | OK |
| OPPO | Support |
| Samsung | Support |
| Ericsson | Alt 2, MAC CE only is sufficient. Additional RRC signaling is not needed. |
| Nokia/NSB | Support. |
| vivo | Don’t support the proposal #3-9a. As MAC CE indicating two TCI states is a unique feature of SFN PDCCH, which can differentiate with other MTRP PDCCH schemes such as TDM PDCCH, it’s redundant to configure an additional RRC parameter. If we finally specify RRC parameters for both SFN PDSCH and PDCCH, there would be four, even more, RRC parameters for SFN indication, considering both scheme 1 and pre-compensation. |
| APT/FGI | Support |
| Convida Wireless | Support |
| Lenovo/MotM | Support |
| Moderator | I think RRC parameter is better to have to provide sufficient time for the UE to prepare hardware for two TCI state processing. The proposal is further updated based on the clarification from Qualcomm Proposal #3-9b: - Both RRC parameter configured per CORESET and MAC CE with two TCI states are used for identification of the enhanced SFN PDCCH transmission scheme |
| CATT | Support |

#### Round-3

Proposal #3-9b:

- RRC parameter configured per CORESET and MAC CE with two TCI states are used for identification of the enhanced SFN PDCCH transmission scheme

| Company | Comment |
| --- | --- |
| Moderator | Propose to take proposal #3-9b for offline agreement |
| Apple | We are fine |
| OPPO | Support |
| QC | Support |
| LG | If my understanding is correct, in Rel-16, there is no additional RRC signaling to enable two TCI states for PDSCH. So, it seems that MAC CE is also sufficient for PDCCH, and the motivation and benefit of additional RRC signaling is not clear to us. |
| vivo | To Moderator, thanks for the analysis for the benefit for UE to prepare hardware for two TCI state processing. But anyway, PDCCH should be supported to switch between SFN transmission and STRP transmission by MCE CE. We can’t understand why companies can support dynamic(DCI-based) switching between SFN PDSCH and STRP PDSCH, but can’t support pure MAC CE-based switching between SFN PDCCH and STRP PDCCH, which provides more time for UE to prepare its processing than DCI. In our view, both cases should be supported naturally. Therefore, we support only MAC CE with two TCI states without the additional new RRC parameter. |
| Nokia/NSB | Support the proposal. |
| ZTE | Support the FL proposal. |
| DOCOMO | We’d like to clarify whether it is possible for MAC CE to switch between SFN CORESET (with two TCI states) and non-SFN PDCCH (with one TCI state), without RRC reconfiguration. |
| Ericsson | We share same view as vivo and LG. Additional RRC configuration is redundant. |
| Moderator | Re DOCOMO Our understanding the Rel-15 MAC CE activating single TCI state can be used to support switching to non SFN-PDCCH based on one TCI state |
| Lenovo/MotM | Support |
| CATT | Support |

#### Round-4

Proposal #3-9b:

- RRC parameter configured per CORESET and MAC CE with two TCI states are used for identification of the enhanced SFN PDCCH transmission scheme

| Company | Comment |
| --- | --- |
| Moderator | Similar to the comment for identification of scheme 1 (PDSCH), RRC parameter is required for scheme 1 (PDCCH) from UE implementation perspective, where MAC CE based enabling of scheme 1 may not be feasible to support. In addition, the benefits of not supporting RRC parameter for identification of scheme 1 is not clear. |
| QC | Support |
| ZTE | We are fine with ether FL proposal 3-9b or purely use MACCE |
| LG | We support MAC CE based identification. However, if there is majority view to support both RRC and MAC CE based identification, we are ok with proposal for the progress. |
| OPPO | Support the proposal. |
| Huawei, HiSilicon | Support the proposal. |
| vivo | In our view, a new RRC parameter for SFN PDSCH is used to support semi-static (RRC-based) switching of scheme 1 (PDSCH) with Rel-16 scheme 1a. Without the RRC parameter, the semi-static (RRC-based) switching couldn’t be achieved. Form this perspective, we agree that we need a new RRC parameter for SFN PDSCH. However, in the case of SFN PDCCH, we think the situation is different. Our reasons are as follows. - MAC CE with two TCI states is a unique feature of SFN PDCCH, which is a feasible way to differentiate SFN PDCCH with other MTRP/STRP PDCCH schemes; - When switching from SFN PDCCH to STRP PDCCH, no matter with or without a new RRC parameter for SFN PDCCH, a MAC CE would be used to indicate a codepoint with one TCI state for STRP PDCCH anyway. Then UE would switch its receive processing after receiving the MAC CE. From this perspective, a RRC parameter couldn’t bring the benefits for UE to prepare hardware. Even, we already reached an agreement to support dynamic switching between STRP PDSCH and SFN PDSCH. Therefore, we believe UE implementation is not the bottleneck. - When switching from STRP PDCCH to SFN PDCCH, assuming a new RRC parameter is used to indicate SFN PDCCH, then RRC reconfiguration is needed. That would cause larger RRC overhead for switching, which is mentioned by many companies when we discussed the switching of SFN PDSCH before. According to the above reasons, we don’t support RRC parameter for SFN PDCCH. |
| Sony | We think it would be okay to support RRC parameter for configuring SFN PDCCH. But should the granularity of RRC configured SFN PDCCH be on a per CORESET level? In our view, we could reuse the RRC parameter of SFN PDSCH for SFN PDCCH either per BWP or per CC and allow MAC CE to tune number of TCI state(s) per CORESET. Apparently, 1 TCI state for S-TRP and 2 TCI states for SFN. |
| Spreadtrum | Support |
| Samsung | Support the proposal in principle. We can further discuss the granularity of the RRC parameter per CORESET/BWP/CC. |

### Issue #3-10 (PDCCH monitoring with different QCL-TypeD)

Two companies proposed to discuss priority rule for PDCCH monitoring of PDCCH candidates in overlapping monitoring occasion with different QCL-TypeD when CORESET is indicated with two TCI states.

#### Round-1

Proposal #3-10:

- When a CORESET is activated with two TCI states, study new rule for PDCCH monitoring of PDCCH candidates in overlapping monitoring occasion with different QCL-TypeD

Companies to provide their views on the proposal above.

| Company | Comment |
| --- | --- |
| DOCOMO | Support. In Rel-15/16, if PDCCH candidates in overlapping PDCCH monitoring occasions in multiple CORERSETs have different QCL-TypeD, UE monitors PDCCH only in a CORESET having the same QCL-TypeD as the CORESET determined from the priority rule that CSS has higher priority than USS and SS set with lower index has higher priority. If a CORESET can be activated with two TCI states, the rule for PDCCH monitoring in multiple CORESETs with different QCL-TypeD needs to be studied. For example, if Rel-16 priority rule is reused, if the CORESET determined from the priority rule is activated with one TCI state, it need to be studied whether another CORESET activated with two TCI states in overlapping monitoring occasion can be monitored. |
| ZTE | Support. The same view as DOCOMO |
| Spreadtrum | Support |
| Samsung | Support the proposal. Priority rule would be enhanced to monitor CORESET with two TCI states. |
| Apple | Discsuss later |
| CATT | Support |
| Lenovo/MotM | Support |
| APT/FGI | Support |
| LG | Support |
| Futurewei | Ok to study |
| NEC | Support. |

#### Round-2

| Company | Comment |
| --- | --- |
| Moderator | No technical concern is raised. I suggest we take proposal #3-10 as offline agreement to facilitate the study of this scenario in the next meeting. |
| QC | Okay to study |
| ZTE | OK |
| OPPO | Support |
| LG | Support |
| Xiaomi | Support |
| Nokia/NSB | OK to study. |
| APT/FGI | Okay to study |
| Lenovo/MotM | Support to study |
| CATT | Support |

#### Round-3

| Company | Comment |
| --- | --- |
| Moderator | Propose to take the proposal #3-10 as offline agreement |
| Apple | We are okay to study |
| QC | Support the study proposal |
| LG | Support |
| vivo | Support |
| Nokia/NSB | Support. |
| ZTE | Support |
| DOCOMO | Support |
| Ericsson | Okey to study. |
| Lenovo/MotM | Support |
| Xiaomi | Support |
| CATT | Support |
| Huawei, HiSilicon | We are ok to study. |

## Other issues

This section contains other issues the companies want to highlight for discussion regarding support of SFN PDCCH transmission.

| Company | Comment |
| --- | --- |

## Issue #4-1 (Other non-categorized proposals)

The proposals supported by one company are provided below for consideration in the next RAN1 meetings.

- A new definition on QCL association relationship of one antenna port and one antenna port group

- Support of small delay CDD with a properly adjusted delay offset between TRPs

- Support configuration of combination of SFN and TDM based PDCCH simultaneously

- Introduce new QCL type-E with loose Doppler shift relationship between the target and source RS.

- Study zone-based configuration for TCI/QCL information to mitigate potential high signaling overhead.

- Support variable-rate TRS transmission for HST deployment scenario.

- TCI states configured in non-serving cell(s) with PCI either explicitly configured or implicitly associated

- DMRS adaptation for HST SFN scenario

- UE assisted DMRS adaptation for DL, in which UE provides an indication of the most convenient DMRS configuration

- Study PTRS design in case of SFN transmission scheme

- Dynamic DMRS configuration signaling to enable DMRS adaptation

- New SRS pattern for UL Doppler estimation purpose

- SRS allocation for Doppler measurements multiplexing with any UL or DL channel for the addressed UE

- Support transmitting DMRS REs for one antenna port in FDM fashion from both TRPs

- Study TA issue in HST scenario

- Support one joint TCI state in DCI to trigger SP/AP-TRS in DL which is automatically followed by SP/AP-SRS in UL.

- Support configuration/activation of one or two TCI States for different search spaces in a CORESET for PDCCH SFN transmission.

# Other issues

This section contains other issues the companies want to highlight.

| Company | Comment |
| --- | --- |

# References

[1] RP-193133, New WID: Further enhancements on MIMO for NR, Samsung 3GPP TSG RAN Meeting #86, Sitges, Spain, December 9-12, 2019.

[2]	R1-2102337	Enhancements on high speed train for multi-TRP in Rel-17, Huawei, HiSilicon

[3]	R1-2102382	Enhancements on HST-SFN deployment, OPPO

[4]	R1-2102436	Multi-TRP Transmission for HST-SFN, InterDigital, Inc.

[5]	R1-2102445	Discussion on enhancements on HST-SFN deployment, Spreadtrum Communications

[6]	R1-2102510	Further discussion and evaluation on HST-SFN schemes, vivo

[7]	R1-2102602	Enhancements for HST-SFN deployment, CATT

[8]	R1-2102664	Discussion on Multi-TRP HST enhancements, ZTE

[9]	R1-2102728	Discussion of enhancements on HST-SFN deployment	Asia Pacific Telecom, FGI

[10] R1-2102764	Enhancement to support HST-SFN deployment scenario, FUTUREWEI

[11] R1-2102881	Enhancements on HST-SFN deployment, CMCC

[12] R1-2102963	Enhancements on HST-SFN operation for multi-TRP PDCCH transmission, Xiaomi

[13] R1-2103018	Enhancements to HST-SFN deployments, Intel Corporation

[14] R1-2103092	Views on Rel-17 HST enhancement, Apple

[15] R1-2103154	Enhancements on HST-SFN deployment, Qualcomm Incorporated

[16] R1-2103198	Enhancement on HST-SFN deployment, Ericsson

[17] R1-2103225	Enhancements on HST-SFN, Samsung

[18] R1-2103291	Considerations on HST-SFN operation for multi-TRP, Sony

[19] R1-2103369	Enhancements for HST-SFN deployment, Nokia, Nokia Shanghai Bell

[20] R1-2103508	Enhancements on HST-SFN deployment, LG Electronics

[21] R1-2103524	Discussion on HST-SFN deployment, NEC

[22] R1-2103563	Discussion on HST-SFN deployment, NTT DOCOMO, INC.

[23] R1-2103606	Enhancements for HST-SFN deployment, Lenovo, Motorola Mobility

# Appendix (Summary of the agreements)

The agreements made in RAN1#102e, RAN1#103e and RAN1#104e meetings are provided below.

RAN1#102-e meeting agreements

Agreement For the discussion purpose consider the following categorization of the enhanced DL transmission schemes - Scheme 1: - TRS is transmitted in TRP-specific / non-SFN manner - DM-RS and PDCCH/PDSCH from TRPs are transmitted in SFN manner - Scheme 2: - TRS and DM-RS are transmitted in TRP-specific / non-SFN manner - PDSCH from TRPs is transmitted in SFN manner Agreement Study the following aspects of the enhanced transmission schemes: - For scheme 1: - Target DL physical channels, i.e., PDSCH only or PDSCH + PDCCH - Whether more than 2 QCL/TCI states are required and corresponding signaling details - Whether and how to indicate scheme 1 for differentiation with Rel-16 non-SFNed transmission schemes with multiple QCL/TCI states - QCL relationship between TRS and DMRS ports - Note: Other schemes/aspects are not precluded - For scheme 2: - Association of each MIMO layer of PDSCH to DM-RS antenna ports - Whether more than 2 QCL/TCI states are required and corresponding signaling details - Whether and how to indicate scheme 2 for differentiation with Rel-16 non-SFNed transmission schemes with multiple QCL/TCI states Note: Other schemes/aspects are not precluded

Agreement Study TRP-based frequency offset pre-compensation including the following aspects: - Aspects related to indication of the carrier frequency determined based on the received TRS resource(s) in the 1st step - Option 1: Implicit indication of the Doppler shift(s) using uplink signal(s) transmitted on the carrier frequency acquired in the 1st step - Indication for QCL-like association of the resource(s) received in the 1st step with UL signal transmitted in the 2nd step - Type of the uplink reference signals / physical channel used in the 2nd step, necessity of new configuration and corresponding signaling details - Option 2: Explicit reporting of the Doppler shift(s) acquired in the 1st step using CSI framework - FFS: Indication for QCL-like association of the resource(s) received in the 1st step with UL signal transmitted in the 2nd step - CSI reporting aspects, configuration, quantization, signalling details, etc. - New QCL types/assumption for TRS with other RS (e.g., SS/PBCH), when TRS resource(s) is used as target RS in TCI state - New QCL types/assumptions for TRS with other RS (e.g., DM-RS), when TRS resource(s) is used as source RS in the TCI state - Target physical channels (e.g., PDSCH only or PDSCH/PDCCH) and reference signals that should be supported for pre-compensation - Signalling/procedural details on whether/how the pre-compensation is applied to target channels - Whether multiple sets of TRS and pre-compensation on TRS is needed in 3rd step. Note: Other aspects/schemes are not precluded

RAN1#103-e meeting

Agreement Support at least the following configuration for HST scenario in Rel-17 - The same DMRS port(s) can associate with multiple TCI states - FFS other details Note: DMRS and PDCCH/PDSCH from different TRPs are transmitted in SFN manner Agreement At most two TCI states are supported for HST scenario in Rel-17 - FFS: Whether to support more than two TCI states for FR2 - FFS configuration/signalling details of the TCI states Note: DMRS and PDCCH/PDSCH from different TRPs are transmitted in SFN manner Agreement When the same DMRS port(s) are associated with two TCI states containing TRS as source reference signal, at least one variant is supported for Rel-17 HST-SFN scenario based on further evaluations - Variant A: One of the TCI state can be associated with {average delay, delay spread} and another TCI states can be associated with {average delay, delay spread, Doppler shift, Doppler spread} (i.e., QCL-TypeA) - Variant B: One of the TCI state can be associated with {average delay, delay spread} and another TCI state with {Doppler shift, Doppler spread} (i.e., QCL-TypeB) - Variant C: One of the TCI state can be associated with {delay spread} and another TCI states can be associated with {average delay, delay spread, Doppler shift, Doppler spread} (i.e., QCL-TypeA) - Variant E: Both TCI states can be associated with {average delay, delay spread, Doppler shift, Doppler spread} (i.e., QCL-TypeA) - FFS: Indication method to apply QCL, e.g., via new QCL-type, or reuse existing QCL-type while UE to ignore certain QCL properties - Note: Each TCI state in the above variants may be additionally associated with {Spatial Rx parameter} (i.e., QCL-TypeD) - Note: Companies are encouraged to provide evaluation results for the above variants based on agreed EVM from RAN1#102e meeting - Note: Above variants are applicable to scheme 1 and/or TRP based pre-compensation as a reference for evaluation. - This agreement is for the purpose of evaluation and does not imply the support or lack of support of scheme 1 and/or TRP based pre-compensation

Agreement For PDCCH reliability enhancements, support SFN scheme + Alt 1-1. - FFS: TCI state activation for CORESET, impact on default beam, BFD resource for BFR Where the Alt 1-1 is agreed as: Alt 1-1: One PDCCH candidate (in a given SS set) is associated with both TCI states of the CORESET.

RAN1#104-e meeting

Agreement Scheme 1 is supported in Rel-17 - TRS is transmitted in TRP-specific / non-SFN manner - DM-RS and PDCCH/PDSCH from TRPs are transmitted in SFN manner - FFS other details Agreement For scheme 1 and SFN transmission of PDCCH support Variant E for QCL assumption in TCI state when TRS is used as source RS Agreement Two TCI states are supported for scheme 1 in FR2 Agreement - Support MAC CE activation of two TCI states for PDCCH - FFS other details Conclusion The decision on support of specification based TRP pre-compensation scheme for HST-SFN scenario to be made in RAN1#104-e-bis meeting. To facilitate RAN1 decision, companies are encouraged to provide evaluation results according to the agreed evaluation assumptions. The evaluations not compliant with agreed assumptions will not be considered by RAN1 in the decision process. Agreement For HST-SFN scenario: - Support semi-static (RRC based) switching of scheme 1 (PDSCH) with 2a, 2b, 3, 4 - FFS all other details including RRC signaling, possible RAN4 impact (if any), etc.