3GPP TSG-RAN WG1 #104b-e		R1-2104100

eMeeting, Apr 12 – 20, 2021

Source: 	Moderator (Ericsson)

Title:      	Summary#4 of Email discussion [104b-e-NR-DSS-01]

Agenda item:	8.13.1

Document for:	Discussion and Decision

# 1 Introduction

This document summarizes the discussions for email thread [104b-e-NR-DSS-01] under agenda item 8.13.1 on Cross-carrier scheduling (from SCell to PCell) for the Rel17 WI on NR Dynamic spectrum sharing (DSS).

# 2. Discussion

## 2.1 Moderator Summary

Below is a short moderator summary based on tdocs [1-21] submitted for RAN1#104-e

### 2.1.1	PDCCH monitoring and BD/CCE limit handling

Following aspects were discussed related to PDCCH monitoring and BD/CCE limit handling when CCS from sSCell to PCell/PSCell is configured

- In RAN1#103-e and RAN1#104-e, high level agreements on PDCCH monitoring on P(S)Cell and/or sSCell to support CCS from sSCell to P(S)Cell were made. For this meeting, below detailed alternatives were discussed in the tdocs (details of Alt 2-1 to 2-4 are in the RAN1#103-e agreement in the Annex)

  - Alt 2-1 – [1],[4],[5],[7?][11],[16],[17],[19],[20]

  - Alt 2-2 – [2],[3],[10],[14],[18]

  - Alt 2-3 –[19 (for non-fallback DCI formats x_2)]

  - Alt 2-4 – [6],[7?],[18],[19 (for non-fallback DCI formats x_1)]

  - Modified Alt 2-4 (now called Alt 2-4a) – [15]

    - A UE does not expect to monitor PDCCH [including CSS and USS candidates for primary cell scheduling] on both the primary cell and the sSCell in a same slot

  - Modified Alt 2-2 (now called Alt 2-5) – [8]

    - Dynamic switching of PDCCH monitoring of DCI formats 0_1,1_1,0_2,1_2 between monitoring on Pcell/PSCell USS sets and monitoring on sSCell USS sets is supported

      - FFS: Details of switching mechanism

    - UE may does not monitor DCI formats 0_1,1_1,0_2,1_2 on both Pcell USS set(s) and sSCell USS sets simultaneously in certain scenarios, e.g. when the UE falls back to monitoring USS set on the Pcell, it is not forbidden from monitoring the USS set on the sSCell.

  - Alt2-2+Alt 2-4 (now called Alt 2-6!) – [8]

    - The USS set(s) on PSCell/PCell and the USS set(s) on sSCell are configured such that UE does not monitor DCI formats 0_1,1_1,0_2,1_2 on both PCell USS set(s) and sSCell USS set(s) simultaneously

    - SS Group Swithcing is employed for dynamic switching mechanism

- BD/CCE limit handling and overbooking discussed in [1],[3],[4],[5],[7],[9],[11],[13],[14],[15],[17],[20],[21].

  - As part of the discussions, it is beneficial to clarify the intended BD limit handling for a corresponding PDCCH monitoring option (i.e., the Alt 2-1,2-2, etc. discussed above). To initiate such discussion, it can be first clarified whether BD limits are according to Option A or Option B below (similar principle can be applied for CCE limits later)

    - Assuming below notation

      - USS(s) => USS(s) that can schedule PDSCH/PUSCH on P(S)Cell)

      - x1(m) is #BDs for PDCCH CSS(s) candidates monitored on P(S)Cell slot m

      - x2(m) is #BDs for PDCCH USS(s) candidates monitored on P(S)Cell slot m for USS(s)

      - y(m) is #BDs for PDCCH USS(s) candidates monitored on sSCell in sSCell slot(s) that overlap slot m of P(S)Cell

    - Option A

      - max of (x1(m1)+x2(m1))+max of y(m2) corresponding to any P(S)Cell slots m1 and m2 is less than or equal to BD limit Z1

        - FFS Z1 related details including value of Z1and any related signaling

    - Option B

      - For P(S)Cell slot m,

        - x1(m)+x2(m)+y(m) is less than or equal to BD limit Z2

          - FFS Z2 related details including value of Z2 and any related signaling

      - max of (x1(m1)+x2(m1)) + max of y(m2) corresponding to any P(S)Cell slots m1 and m2 can be larger than BD limit Z2

  - Whether to allow PDCCH overbooking on sSCell was discussed in

    - Not allow overbooking on sSCell – [1],[3],[5?],[6],[7?],[9],[13],[19],[20]

    - Allow overbooking on the sSCell – [4],[5?],[7?],[11],[14],[15],[17]

- Handling of DCI formats 0_0 and 1_0 on USS for scheduling PCell/PSCell PDSCH/PUSCH (this was also discussed in RAN1#104-e)

  - only on PCell/PSCell as in Rel15/16 – [3],[4],[5],[11],[12],[15],[16],[19]

  - follows for non-fallback handling – [13]

  - FFS – [17],

- DCI format 2-5 (also discussed in RAN1#104-e)

  - follows Rel16 – [4],[6],[10],[11],[12],[15],[17]

- DCI format 2-6 (also discussed in RAN1#104-e)

  - Follows Rel16 handling – [3],[4],[6],[10],[12],[17]

  - Can be sent also on sSCell – [1?],[5],[11],[15],

- Impact on DCI size budgets– [3],[4],

- Impact on #DL and UL unicast DCI per monitoring occasion/span – [4],[7],[16],[19]

- PDCCH in SS set provided by recoverySearchSpaceId can be monitored on the sSCell – [21]

- Separate config of UL and DL DCI formats – [19]

### 2.1.2	Configuration details for CCS from sSCell to P(S)Cell

- CIF handling, hashing function – [3],[4],[11],[13],

- Search space ID linking and config of (PeriodicityAndOffset, SymbolsWithinSlot, and duration) for sSCell  PCell/PSCell scheduling and PCell/PSCell  PCell/PSCell scheduling – [3],[4],[13],[21]

- RRC configuration details for CCS from sSCell to PCell/PSCell (How to indicate using CrossCarrierSchedulingConfig) – [3],[4],[5],[6],[9],[15],

### 2.1.3	Remaining details on scheduling framework

Following aspects were discussed related to scheduling framework when CCS from sSCell to PCell/PSCell is configured

- Dynamic activation/activation/switching of sSCell  PCell/PSCell scheduling

  - Support – [4],

  - Not support – [5],[6],[7]

  - FFS – [16]

  - Handling when sSCell is deactivated/dormant – [17],[[19],

- Dormancy supported for sSCell?

  - Supported – [3],[4],

  - FFS –

- DCI or MAC CE based switching of sSCell – [15]

### 2.1.4	Other aspects

- SCell to PCell/PSCell scheduling has no impact on PUCCH or PUSCH/SRS for non-CA – [8]

- Whether sSCell can be unlicensed band? – [19]

- BFR on sSCell – [21]

- SCell to Pcell scheduling also supported for multi-cast broadcast PDSCH – [21]

Below are some proposals for discussion

## 2.2	Proposals

### Discussion point 1

- Companies are encouraged to indicate their preferred joint framework considering

  - the alternatives for PDCCH monitoring (i.e., Alt 2-1, 2-2, 2-3, 2-4, 2-4a, 2-5, 2-6 as discussed in section 2.1.1 above, point 1), and

  - the options for defining BD/CCE limits (i.e., a limit Z1 based on Option A, or Z2 based on Option B as discussed in section 2.1.1 above, point 2), and

  - whether PDCCH overbooking handling is needed or not for sSCell

Companies are requested to indicate their view in the Table below

| Company Name | Preferred combination (PDCCH monitoring Alt, BD/CCE limit option, yes/no for ssCell overbooking handling) | Comments (Discussion point 1) |
| --- | --- | --- |
| Moderator notes |  | Intention is to identify joint framework considering PDCCH monitoring approach, corresponding BD/CCE limits and requirement for overbooking handling on sSCell. In addition to clarifying Option A vs. Option B for BD/CCE limits, companies can also provide example limits for the corresponding Option. |
| Qualcomm |  | First of all, we would like to request to add a note that the discussion here is, for the time being, for Ues that can be configured to monitor DCI formats 0_1/1_1/0_2/1_2 that schedule PDSCH/PUSCH on PCell/PSCell on PCell/PSCell USS set(s), and/or on sSCell USS set(s). The other type of Ues, i.e., (Alt.1) Ues that cannot be configured to monitor these DCI formats on the PCell/PSCell, will be discussed separately. PDCCH monitoring (sub-)Alternative With the above understanding, our preference is Alt.2-1, subject to appropriate solutions of BD/CCE determination and PDCCH overbooking. BD/CCE limit option for Alt.2-1 Option A. Z1 should be 44 PDCCH candidates for the PCell SCS 15kHz. This number comes from the fact that the PDCCH capability is dimensioned not from a scheduling cell perspective; but from a scheduled cell perspective. For PDSCH/PUSCH scheduling on the PCell with SCS 15kHz, 44 PDCCH candidates in 1ms slot should be sufficient. Yes/no for ssCell overbooking handling for Alt.2-1 No. PDCCH overbooking on the PCell has been supported since Rel.15 to address the issue where the number of PDCCH candidates on the PCell could vary across slots due to different monitoring periodicities for USSs and CSSs (for SI, paging, RA). For Rel.17 cross-carrier scheduling for DSS, since we have agreed to keep these CSSs on the PCell, it is not necessary to enable PDCCH overbooking on the sSCell. In order to enable PDCCH overbooking on the PCell for Alt.2-1, we propose to introduce an RRC parameter that gives a BD limit for the PCell. The BD limit for the PCell should not be less than the maximum number of BDs required for CSSs across all the slots on the PCell (so that overbooking for CSSs is not required), and should not be more than Z1. Based on the configured BD limit for the PCell, PDCCH overbooking on USS sets on the PCell is enabled. For PDCCH monitored on the sSCell, no PDCCH overbooking shall be required. |
| Samsung |  | In resolving the FFS from the last meeting and considering Alt. 2-4, the baseline operation that enables DSS for Rel-16 Ues is for a UE to not monitor PDCCH in a same slot on both PCell and sSCell. There is no issue as the sSCell is TDD and Type3-CSS can be on either the PCell or the sSCell (DCI 2_6 does not matter). SI updates may be received twice per frame on PCell as in LTE. Obviously there is no spec impact for BD/CCE partitioning. Alt 2-1 and Alt 2-2 can be considered as enhancements. Alt 2-2 may be viewed as enhancement to Alt 2-4 but also applied to Alt 2-1. The spec impact includes the BD/CCE split/determination. Both Option A and Option B can work depending on particular alternative – e.g. Option A can work with Alt 2-1, Option B can work with Alt 2-2/2-4. Several more options can be considered, including introducing a virtual cell (e.g. x1(m)+x2(m) <= Z3 AND y(m) <= Z4, while Z3+Z4 <= Z5 or max of (x1(m1)+x2(m1))<= Z3 AND max of y(m2) <= Z4, while Z3+Z4 <= Z5, …). Instead of that, it would be better to discuss specific proposals for partitioning BDs/CCEs under Alt 2-1 – basically, the main question is how to determine MPDCCHtotal,slot,μ/CPDCCHtotal,slot,μ. SS overbooking on sSCell should be supported. To clearly see this, consider Alt. 1 where all USS sets are moved to the sCell (in that sense, Alt. 1 is not DSS as there is no ‘D’ in it). With the exception of infrequent SI updates (CSS on PCell), the sSCell can/will also include the Type-3 CSS and is practically the Rel-16 PCell. |
| Huawei |  | For PDCCH monitoring there has been a working assumption that implies separate UE capabilities for whether or not simultaneously monitoring the PDCCH from both cells. From network point of view it is not desirable to have further finer granularity for restricted approach in between Alt 2-1 and Alt-2, thus Alt 2-1 is the preferred target for this objective of R17 DSS. While to proceed it might be easier to discuss the BD/CCE limit first given the working assumption content, and in that case work based on Alt2-1 is proper. Our preference is not either Option A or Option B - either option is referring to Pcell while it was true in Rel16 X-carrier scheduling where Scell is not monitored, the need to keep the limit for each cell/SCS when sSCell scheduling PCell is raised, under Alt 2-1. So the determination needs to be extended considering the scheduling source numbers, as that happens in Multi-TRP operation. Our proposal is to add Option C (though Option B may still work if the SCS between two cells are same). - Option C - For P(S)Cell slot m1, - max of (x1(m1)+x2(m1)) is less than or equal to BD limit Z3, and - For sSCell slot m2, - max of (y(m2)) is less than or equal to BD limit Z4 - FFS Z3/Z4 related details including value of Z3/Z4 and any related signaling We don’t see strong need to support overlooking on sScell. |
| ZTE | 1) Alt.2-1 2) Option B 3) Overbook on across PCell + SCell | Based on our understanding, the potential UE implementation complexity mainly comes down to the BD/CCE budget split/sharing and/or overbooking between PCell and sSCell instead of these different alternatives. For Alt.2-2, 2-5 and 2-6, RAN1 needs to define the switching command and switching mechanism. Besides, switching delay is required, which makes it impossible to perform frequent switching between PCell and sSCell scheduling to adapt to the dynamic channel condition. For Alt.2-3, one DCI format is restricted on one carrier. This restricts the network scheduling flexibility. For example, if DCI format 1-2/0-2 is restricted on sSCell, it is impossible to switch PCell scheduling URLLC smoothly. For Alt.2-4 and 2-4a, it is not clear what’s the benefit to stagger the CSS/USS on PCell and USS on SCell. Regarding BD/CCE limit handling, Option A refers to separate limit for PCell and sSCell while Option B refers to a joint limit for PCell and sSCell. Based on our understanding, Option B can fully make use of the UE BD/CCE capability without any waste. Regarding overbooking, legacy overbooking mechanism is for a particular scheduled cell. It is preferred to perform overbooking across PCell and SCell. Thus, 1) Alt.2-1 + 2) Option B + 3) Overbook on across (PCell + SCell) would be our preferred solution. However, we understand that UE vendors may have concern on Option B and overbooking across PCell+SCell, we are also open to some compromised solutions for Alt.2-1. |
| Vivo | - Alt. 2-1 - Option A - Overbooking on Pcell CSS + Pcell USS only | As discussed in our contribution, the UE implementation complexity is evaluated by combining BD/CCE budget and overbooking options and provided below: Note that Option A and Option B is Option 1/2 and Option 3 respectively in the above table. For UE complexity evaluation, it is determined based on how to allocate PDCCH blind decoding resource (i.e. no change, long term change, or dynamic) in terms of UE implementation. It is clearly observed that UE complexity depends on mainly the BD/CCE handling not on the various alternatives. Even for Alt. 1, if Option B is adopted, the PDCCH monitoring resource unit allocation is changed slot by slot. So, in our understanding, there is no need to have different UE capability on Alt. 1 and Alt. 2.1. Actually allowing Alt. 2.1 is the most flexible since Alt. 1 and Alt. 2.4 are special configuration case of Alt. 2.1. Then the question boils down to whether we need such configuration restriction as Alt. 1 and Alt. 2.4. However, according to UE complexity analysis, such restriction doesn’t bring any benefit on implementation complexity. In this sense, Alt. 2.1 is preferred to allow more configuration flexibility. To reduce the UE complexity, Option A and not support Scell overbooking is preferred. For Alt. 2.2, it introduces SS switching mechanism and is beneficial to save some RRC re-configuration effort. We are open to discuss this switching mechanism and it could be configurable between Alt. 2.1 and Alt. 2.2. |
| MTK |  | As mentioned by HW and Samsung, Alt. 2.4 is the baseline according to previous working assumption on separate UE capabilities for whether or not simultaneously monitoring the PDCCH from both cells. If UE is capable of simultaneously monitoring the PDCCH from both cells, then we prefer Alt. 2.1. For Alt 2.1, we prefer Option A and Z1 should be 44 PDCCH candidates for the PCell SCS 15kHz. For overbooking, we prefer to not have overbooking on SCell. |
| CMCC | Alt. 2-1 Option B Overbooking only on PCell/PSCell | As the working assumption in last meeting, the USS set(s) scheduling PCell/PSCell can be monitored on PCell/PSCell and/or on sSCell, there is no need to further restrict to dynamically or semi-statically split the USS set(s) monitoring on PCell/PSCell or sSCell. In addition, Alt 2-1 also gives NW more flexibility. Regarding the BD/CCE limit, as the USS set(s) can be simultaneously monitored on PCell/PSCell and sSCell, it is much simple to take into count the total BD/CCE limits of PCell/PSCell and sSCell in Option B. Regarding the PDCCH overbooking, following current framework i.e., only on PCell/PSCell is preferred. |
| CATT | - First preference is alt 2-4 but can compromise to alt 2-1. - Option B if alt 2-1 is adopted - No overbooking on the sSCell | - For PDCCH monitoring From our perspective, the dynamic mechanisms are not preferred. They are too complicated to be applied to the real world. The configuration of USS on both sSCell and Pcell/PSCell is in semi-static manner. The motivation of dynamic switching of PDCCH monitoring is unclear. Furthermore, there is a risk of miss detection for the dynamic signaling, which leads to the miss understanding between network and UE. Considering the USSs configured on Pcell and sSCell are both used to schedule PDSCH/PUSCH on the Pcell, it is a bit abundant to mandate a UE to monitor USS on both cells. Hence alt 2-4 is our first preference. Furthermore, both option A and option B will be unnecessary as the Pcell and SCell are totally separate. We don’t need to further consider the partition of BD/CCE between Pcell and SCell. Accordingly, the current mechanism can be fully reused. - For the BD/CCE limit From our understanding, what really matters is how to calculate MPDCCHtotal,slot,μ/CPDCCHtotal,slot,μ. The current mechanism captured in the specification can be reused here, i.e. - Calculate the total number of BD and CCEs across the scheduling cell based on the pdcch-BlindDetectionCA, the number of configured cells, the per-CC limit for the target SCS, the number of CCs with the same SCS as the scheduling cell - Based on the total number of BD/CCE and the per-CC limit for the scheduled cell, determines the maximum number of BD/CCE for the scheduled cell From this perspective, option B fit the current logic better if alt 2-1 is adopted. - Overbooking on sSCell The overbooking operation is mainly used to protect the CSS. Considering there is no CSS configured on the sSCell, we don’t see the necessity to support overbooking on the sSCell. |
| Xiaomi | 1) Alt 2-2 2) Option A 3) Overbooking on PCell/PSCell only | 1) From the perspective of UE processing complexity, UE monitoring both on Pcell/PSCell and sSCell in the same slot is not preferred. It should be a feature that can be activated/deactivated by network under the configuration of the feature, instead of always activating the feature when it is configured. By this way, network can have more scheduling flexibility on UE-specific search space, such as switch the scheduling cell back to the ‘Pcell/PSCell’ in the case of beam failure on the sSCell, and switch the scheduling cell to only the sSCell for UE to reduce power consumption. 2) To reduce the UE complexity, Option A is preferred. 3) It is unnecessary to support overbooking on the sSCell. |
| ETRI | 1) Alt.2-1 2) Option B 3) Overbook on across PCell/PSCell + sSCell (i.e., overbooking on sSCell allowed) | We share the same view as ZTE. We basically think the existing BD/CCE limit and the overbooking rule can be applied the same over the union of PCell/PSCell and sSCell, without any split between them. We expect that the UE complexities of Option A and Option B are comparable, while Option B can reuse the existing mechanism as much as possible. |
| Apple | 1) Alt 2-4a | Regarding 1) We have strong concern on the toggling of USS between SpCell and sSCell or even simultaneous monitoring. You either have DSS problem on SpCell or you do not. Pick one, and have a clean solution Question 2 and 3 depends on the outcome of 1), so it should come after we resolve the first question |
| Nokia, NSB |  | PDCCH Monitoring: As indicated in our contribution, as it has become evident over the past meetings that 2-1 cannot be adopted as the solution we can design the system around, we prefer Alt 2-5 or Alt 2-6, or in the end any other mechanism that allows for having USS configured on both cells and there is a controlled way to manage which cell USS is being monitored at any given time. The dynamic switching the mechanism needs to be further detailed if that direction is the one preferred by the group. Alt 2-5(Modified Alt 2-2) - Dynamic switching of PDCCH monitoring of DCI formats 0_1,1_1,0_2,1_2 between monitoring on Pcell/PSCell USS sets and monitoring on sSCell USS sets is supported OR Alt 2-6 (Alt2-2+Alt 2-4) - The USS set(s) on PSCell/PCell and the USS set(s) on sSCell are configured such that UE does not monitor DCI formats 0_1,1_1,0_2,1_2 on both PCell USS set(s) and sSCell USS set(s) simultaneously BD/CCE limits : The BD/CCE limits are counted separately for each scheduled cell monitored on the active DL BWP of the scheduling cell. We somewhat prefer option A, as it provides flexibility, but we can accept option B as well. Overbooking on sScell : We can accept not to support overbooking on sScell to simplify the UE PDCCH reception burden if we can otherwise converge to a meaningful overall concept. |
| InterDigital | 1) Alt 2-2 or Alt 2-1 2) Option B for Alt 2-2 and Option A for Alt2-1 3) Overbooking on SCell is supported | For PDCCH monitoring, our preference is Alt2-2 and Alt2-1. For BD limit, if simultaneous monitoring of USS sets on both PCell and SCell is supported, separate limit should be set per scheduling cell. If dynamic switching is supported (Alt2-2), a maximum limit can be set across PCell and SCell. We support overbooking on SCell. Because different USS can have different periodicity/offset and instead of configuring the PDCCH candidates based on the worst case where all the USS occur in the same slot, overbooking can be used to maximize the usage of control resources on SCell. |
| Lenovo, Motorola Mobility | Alt 2-1+ option B + no overbooking on sSCell | Option B for PDCCH monitoring seems to align with Rel-16 mechanisms for BD/CCE limits. - For option A, we think it might quite limit scheduling flexibility in slots/spans wherein not all search spaces exist. - For both options A and B, wondering if Z1 and Z2 are considered to be larger than 44? Regarding Alt 2_x variants, in general, we prefer to allow URLLC scheduling (e.g., DCI format x_2) for PCell to be possible from PCell and not only from sSCell as the sSCell may become unavailable, and hence may impact URLLC performance. |
| LG Electronics | Alt 2-1 Option B Overbooking handling on PCell + sSCell | Our preference on PDCCH monitoring is Alt 2-1. More specifically, cross-carrier scheduling or self-carrier scheduling can be configured per each USS set. Regarding Option A vs. Option B, Option B is preferred since slot-by-slot handling for BD/CCE limit can provide more flexibility to assign PDCCH monitoring occasions. |
| OPPO | 1). Alt 2-2 2). Max of (x1(m1)+x2(m1)) over all m1 is no larger than Z3; and max of y(m2) over all m2 is no larger than Z4. 3). Overbooking on P(S)Cell only. | PDCCH monitoring With Alt2-2, the configuration is flexible and the implementation complexity is not increased. BD/CCE limit option Z1/Z2 is the maximum number BD in a slot for PCell (as scheduled cell), similar with the BD limit for a single cell. The number of PDCCH candidates may differ in different slots, it is not necessary to limit the maximum number of PDCCH candidates cross slots to satisfy the maximum number of BD for a single slot. It seems Option B is more suitable than Option A. On the other hand, why not to define separate limitations for PCell and sSCell? The number of PDCCH candidates needs to be counted across PCell and sSCell when there is only one limitation (Z1/Z2). It is easier to count separately for each scheduling cell if they have their own BD limitation. BD limitation is calculated in a slot or a span. If the SCS is different between PCell and sSCell, the time interval for a slot or a span is different and it need to unify the reference time interval used for BD limitation. Separate BD limitations can avoid mismatching of slot or span with different SCS. Yes/no for ssCell overbooking handling Because the CSS and USS have different monitoring period, the total number of PDCCH candidates changes with large variation cross slots. PDCCH overbooking for PCell is required. There is only USS in sSCell, no PDCCH overbooking is required. The maximum of BD for PCell(M1), not larger than Z1 or Z2, is needed to handle overbooking for PCell, where M1 should be equal or larger than the maximum PDCCH candidates of CSS across all slots in order to ensure the monitoring capability for CSS. If the BD limitations are defined separately for PCell and sSCell, M1 is the limitation for PCell. |
| NTT DOCOMO | Alt 2-2 Option B | Considering UE complexity, it may be better to avoid monitoring both P(S)Cell and sSCell simultaneously. |
| Ericsson | Alt 2-1, Option A or B, no overbooking on sSCell | We support Alt 2-1. Option B based BD limit provides more flexibility (i.e., Z2=X results in more constrained BD allocation than Z1=X for a given X). However, we are OK with Option A if it is paired with Alt 2-1. Our preference is to set Z1 based on counting sSCell (s-p) as an additional scheduling cell, subject to the BD budget given by e.g. pdcch-BlindDetectionCA. We are not OK with limiting CSS slots on DSS Pcell as proposed in 2-4a – it is very restrictive and impacts legacy Ues. Regarding overbooking, if the (p-p) BD/CCE limit can be separated from (s-p) BD/CCE limit, additional PDCCH overbooking handling for sSCell can be avoided. |
| Intel | Alt 2-1 Option B Overbooking on sSCell | Alt 2-1 is the most flexible without any real complexity Option A is to hardly split maximum number of BD/CCE to Pcell and sSCell. As a result, for the configured BD/CCE for CSS on Pcell, thought CSS is not monitored in a MO or slot, the allowable number of BD/CCE on sSCell cannot be increased. This cause limitation on gNB scheduling flexibility. Option B can avoid such issue For overbooking, if the total numbers of BD/CCE exceed the maximum numbers, a USS set on the sSCell can be dropped |
| Spreadtrum | 1) Alt 2-2 2) Option A, Z1 is 44. 3)No Overbooking on SCell | PDCCH monitoring First, Alt2-2 has more flexibility and less complexity. Since UE does not monitor DCI formats 0_1,1_1,0_2,1_2 on both Pcell USS set(s) and sSCell USS sets simultaneously, it is with less UE complexity. Furthermore, Alt 2-2 has the significant benefit when considering sSCell activation/deactivation and sSCell dormancy. Such as when sSCell is activation or in non-dormancy BWP, UE can monitoring USS sets in sSCell which can increase the PDCCH capacity on Pcell/PSCell. Otherwise, when sScell is deactivated or switched into dormancy BWP, UE can monitoring USS sets on Pcell/PSCell. Considering PDCCH capacity limits, these USS sets can with less PDCCH candidates or smaller aggregation levels. We do not think Alt 2-1 is necessary if considering the use case of sSCell scheduling P(S)Cell, due to the limitation of PDCCH capacity in NR cell. BD/CCE limit option Our first preference is Option A. And Z1 equals 44 when Pcell SCS is 15kHz. |
| Moderator notes |  | Below is my reading of comments for discussion point - Alt 2-1 - no overbooking on sSCell (different views on detailed design of BD/CCE limits) - QC (clarify this is not for ‘Alt1 only Ues’), HW, [Samsung?], vivo, MTK (for Ues that are not ‘Alt1 only’), CMCC, CATT, Lenovo/MotM, Ericsson - overbooking on sSCell - ZTE, ETRI, Interdigital, LG, Intel - Alt 2-2 - no overbooking on sSCell (different views on detailed design of BD/CCE limits) - OPPO, Docomo, Xiaomi, Docomo, Spreadtrum, Interdigital, OPPO, [Samsung?] - Alt 2-4 - no overbooking on sSCell - CATT (1st preference), MTK? - Alt 2-4a - overbooking on sSCell - Samsung - Apple (no view expressed on overbooking) - Alt 2-5 or 2-6 - no overbooking on sSCell - Nokia (open to ‘other mechanism that allows for having USS configured on both cells and there is a controlled way to manage which cell USS is being monitored at any given time’) |

### Proposal 1

- For Ues configured to monitor non-fallback USS set(s) on sSCell and/or P(S)Cell

  - Alt 2-1 is supported

  - PDCCH overbooking on sSCell USS set(s) is not allowed

  - BD/CCE limit handling is as follows

    - PDCCH monitoring candidates on P(S)Cell and/or sSCell are configured such that max of (x1(m1)+x2(m1))+max of y(m2) corresponding to any P(S)Cell slots m1 and m2 is less than or equal to Z1

      - At least the case of Z1 = 44 is supported for P(S)Cell SCS 15kHz

        - FFS if Z1 larger than above can also be supported based on UE capability (e.g. similar to BDFactorR in Rel16)

      - FFS signalling details on how the limit Z1 is realized

        - E.g. configure separate BD limits/scaling factors for #BDs for PDCCH USS(s) candidates monitored on P(S)Cell, #BDs for PDCCH USS(s) candidates monitored on sSCell, separate BdfactorR for P(S)Cell/sSCell etc.

      - Note

        - x1(m) is #BDs for PDCCH CSS(s) candidates monitored on P(S)Cell slot m

        - x2(m) is #BDs for PDCCH USS(s) candidates monitored on P(S)Cell slot m

        - y(m) is #BDs for PDCCH USS(s) candidates monitored on sSCell in sSCell slot(s) that overlap slot m of P(S)Cell

        - USS(s) => USS(s) that can schedule PDSCH/PUSCH on P(S)Cell)

    - Similar principle as BD handling is used for CCE limits

- For Ues supporting functionality of only Alt-1, and configured with non-fallback USS set(s) only on sSCell

  - FFS support of PDCCH overbooking on sSCell

  - FFS BD/CCE limit handling

Companies are requested to indicate their view about the above proposal in the Table below

| Company Name | support/not support | Comments (Proposal 1) |
| --- | --- | --- |
| Moderator notes |  | Thank you for the inputs for Discussion point 1. Majority views are in direction of Alt 2-1. There are diverse views on BD/CCE limit handling and hopefully a harmonized solution can be identified with the text in the Proposal as a starting point. Majority companies prefer to avoid PDCCH overbooking handling for sSCell but some prefer to support it. This has to be chosen one way or other. Comments welcome. |
| Qualcomm | Support | We think the FL proposal is a good starting point for Ues configured to monitor non-fallback USS set(s) on sSCell and/or P(S)Cell. One missing piece is USS set(s) for fallback DCI formats that schedules PDSCH/PUSCH on the P(S)Cell. For the Ues supporting Alt.2-1, it is natural to enable monitoring the USS set(s) for fallback DCI formats on the P(S)Cell. For the UE only supporting Alt.1, this should be added as an FFS under the 2nd bullet. If it is difficult to get consensus on this, it is OK to leave it to the next opportunity of discussion. |
| Samsung | Not support | The starting point should be no specification impact so that a Rel-16 UE can support DSS without hardware change. That is Alt 2-4. The scheduling flexibility impact is minimal, especially since the sSCell is TDD (and Alt 2-4 results naturally). The scheduling flexibility is also improved as the maximum of x(m1) + y(m2) can be larger than 44 (non-overlapping slots) without requiring a new UE capability (BTW, there is no reason to use x1 and x2 – just x for the Pcell suffices). Alt 2-1 can then be considered as an enhancement. Also, we do not have an agreement that there will be Ues supporting functionality of only Alt-1. There are multiple reasons, but a basic one is that Alt-1 is not even DSS – scheduling is always on the sSCell and only when there is infrequent CSS on the Pcell, an even more infrequent fallback scheduling can apply. For that reason, we do not support confirming the WA with the inclusion of Alt-1. Overbooking on the sSCell should be supported – taking Alt-1 just for the sake of an example, everything can be moved to the sSCell (except the rare non-Type 3 CSS). How is that different from the Pcell in Rel-15? |
| Apple | Not support | First of all, by default, 2-4 should be supported which minimizes the specification impact even though it may have limited UE complexity reduction depending on the architecture of the UE., i.e., how fast UE can toggle DCI monitoring between two cells. Secondly, we should avoid a jumbo agreement. Based on the reading of the proposals, we have some confusion - What is “max of (x1(m1)+x2(m1))+max of y(m2)” - Do we allow sSCell to have different SCS, i.e., 30kHz? In which case there will be multiple m2 overlapping with SpCell. If we allow, why we take max when UE need try all the BD, CCEs? - What is max of (x1(m1)+x2(m1)), it is max over what? |
| Nokia, NSB | Not support | Even though it is clear that option 1 is a subset of any option 2 flavour, it is not the same thing that we have agreed to option 1 being a UE capability as implied by the last part of the proposal. If we would allow for Option 1 and option 2-1 as separate UE capabilities, we can just end the discussion as it has been evident in the past meetings that the UE/chip vendors have no intention to support simultaneous USS monitoring. The working assumption made in the last meeting on specs supporting also Alt-1 only operation is not the same as we have an agreement for option 1 UE capability. At most the option 1 is a subset of option 2-x. I would really ask for not fragmenting the spec, but aiming for a single, acceptable compromise that is the only mode of operating here. That is why we have tried to find a compromise between the system flexibility and UE implementation complexity and would request all the parties to work towards this goal. |
| CATT | Not support | For the first sub-bullet, we agree with Apple that alt 2-4 can be supported as a default solution. We can live with alt 2-1 only for sake of progress. We support the second sub-bullet of the proposal. But we are not OK with the BD/CCE limit handling part. During the first round of discussion, many companies support option B for BD/CCE limit handling. However, the proposed proposal seems capture option A. As expressed by several companies, option A puts lots of restrictions on the PDCCH configuration. It takes the maximum number of PDCCH candidates on Pcell and SCell across slots as the upper bound. We don’t understand why we need this mechanism. Currently, the BD/CCE limit is defined on basis of slot or span, instead of across slots. We think the current way should be respected. Considering this is a huge proposal actually including three aspects, it may be better to proceed step by step. |
| ETRI | Not support | We prefer Alt-1 for better flexibility. But for overbooking on sSCell, we think it is too early to decide not to allow it. Since the PDCCH mapping/dropping is semi-statically determined, the UE complexity to implement overbooking additionally on sSCell (for CCS to P(S)Cell) would be comparable to Rel-16 overbooking. Also, if overbooking on sSCell is not allowed, some portion of the BD/CCE capability may be wasted and it may also require additional BD/CCE limit handling, which we want to avoid as much as possible. |
| MTK | Partially support | To our understanding: - RAN1 #104e WA allow Ues supporting functionality of only Alt-1 For Alt 2 series, we think Alt 2-4 should be the default (basic) scenario for UE to support. UE can further report support of Alt 2-1 using capability, and FL proposal 1 is about the detailed BD/CCE setting for Alt 2-1 (where we can support the details). Therefore, for this DSS feature, there should be 3 levels of UE support: - Alt 1 (RAN1 #104e WA) Alt 2-4 Alt 2-1 which is the first preference by us. However, if the majority view is to only keep 2 levels of UE support: - Alt 2-4 Alt 2-1 we can also accept that. |
| Huawei |  | Just our suggestion given the current comments from others (we don’t mind to continue but it does not seem to be able to reach anywhere). It would be good to explore more details focus on BD/CCE limit first, which is the part of may having impact on UE implementation most. After that part being clearer we can discussion alternatives of PDCCH monitoring - it appears the difference now is whether we want more split of the feature or not. We are not ok with the BD/CCE limit part. Having larger BD in our view has little additional contribute compared to what has been offered by Alt2-1. |
| Huawei2 |  | When we comment larger BD, it refers to the situation that may happen even with Z1 = 44. This issue happens due to the current proposal only take Pcell as reference then when Scell is involved, unlike R16 X-carrier scheduling, scheduling cell with different SCS will be on different cells thus different limit is required. As an example, the SCS of Pcell and SCell is 15KHz and 30KHz respectively. PDCCH monitoring occasions 1 and 2 on the SCell are used for cross-scheduling Pcell. PDCCH monitoring occasions 3 and 4 on the Pcell are used for Pcell self-scheduling. When there are max 44 PDCCH candidates on MO 1 and no PDCCH candidate on MO 2, 3 and 4, which satisfy the Z1 as proposed for Pcell@15KHz, the BD number for SCell@30KHz will exceed 36 in the first slot of SCell. Since the UE implementation for BD/CCE limit is related to SCS per cell, the above case needs some consideration. Perhaps the first question is whether this case is allowed or not? i.e. shall the limit of BD/CCE for any SCS/cell shall be met simultaneously or not? |
| Vivo | Support in principle | Regarding the above comments from other company, it seems that some companies think the implementation complexity of Alt 2-4 is smaller than Alt. 2-1. Let’s take the following two schemes for comparison by incorporating BD/CCE budget handling and overbooking Scheme A: - SS configuration: Alt. 2-1 - BD/CCE budget: max of (x1(m1)+x2(m1))+max of y(m2) corresponding to any P(S)Cell slots m1 and m2 is less than or equal to Z1 - Overbooking: PDCCH overbooking on sSCell USS set(s) is not allowed Scheme B: - SS configuration: Alt. 2-4 - BD/CCE budget: x1(m)+x2(m) or x1(m)+y(m) is less than or equal to BD limit Z2 - Overbooking: PDCCH overbooking on sSCell USS set(s) is allowed For Scheme A, the PDCCH monitoring resource could be semi-statically allocated for scheduling Pcell slot by slot, i.e. max of (x1(m1)+x2(m1)) fro Pcell self scheduling, max of y(m2) for Scell scheduling Pcell; For Scheme B, the PDCCH monitoring resource need to be switched between Pcell and Scell. For example, in slot 1 configured with Pcell CSS and USS only, the total BD Z2 (e.g. 44) will be allocated in Pcell; in slot 2 configured with Scell USS only, the total BD Z2 (e.g. 44) will be allocated in Scell. In this sense, the PDCCH monitoring resource will be switched between Pcell and Scell. In our understanding, Scheme B will need more implementation complexity. We agree that the BD/CCE budget handling in current proposal may not fully use the total budget in some slots. So we are also open to support Option B BD/CCE budget handling and overbooking on sSCell as an optional capability. Regarding the UE capability between Alt. 1 and Alt. 2-1 implied in this proposal, some clarification on the UE implementation complexity difference is needed. Based on our understanding, the BD/CCE budget handling will be dominant factor to impact UE complexity but not whether to have USS in Pcell self-scheduling. |
| ZTE |  | Based on our understanding, Alt.1 is a sub-set of Alt.2-1. In last meeting, we only agreed that specification supports Alt.1 only UE, but it doesn’t necessary mean this will be a UE capability at this stage. After further checking, it seems the main concern from UE vendors (at least some of them) is dynamic BD/CCE sharing between Pcell and sSCell, instead of whether to monitor DCI in USS on both Pcell and sSCell. In this sense, one compromised way forward would be to support two kinds of UE. Type-1) Alt.2-1 + Option A, no overbooking on sSCell Type-2) Alt.2-1 + Option B, overbooking across Pcell and sSCell Type-2 is more advanced UE feature compared with Type-1. With these two different types of UE, there is no need to have a separate UE feature for Alt.1 only UE. Below are some detailed comments for the current proposal. 1. If the intention of “(x1(m1)+x2(m1))+max of y(m2)” is to say one BD/CCE limit for Pcell and sSCell, respectively, we would prefer to make it clear as it seems companies have different understandings on this formula based on the comments above. 2. We suggest to delete “At least the case of Z1 = 44 is supported for P(S)Cell SCS 15kHz” or make it as an FFS. Because we have discussed how to set the BD/CCE limit for P(S)Cell SCS 15kH in case of different SCSs case. For example, in case of Pcell (15KHz) + sSCell (30KHz), another way is to determine BD/CCE budget based on 30KHz. If companies prefer to make Alt.2-4 (instead of Alt.1) as the baseline, we would also be ok with that. |
| InterDigital | Partially support | We are ok to go with Alt 2-1, but we don’t see a good reason for not supporting overbooking on the sSCell. We suggest the following modification to the proposal: - For Ues configured to monitor non-fallback USS set(s) on sSCell and/or P(S)Cell - Alt 2-1 is supported - FFS PDCCH overbooking on sSCell USS set(s) is not allowed |
| Intel |  | We support Alt 2-1. However, the proposed BD/CCE handling ‘max of (x1(m1)+x2(m1))+max of y(m2)’ results in much scheduling restriction. In this proposal, a number of BD/CCE, ‘max of (x1(m1)+x2(m1))’ is hardly reserved for Pcell. As a result, in most/all slots, the exact number of BD/CCE for Pcell scheduling (CSS/USS on Pcell + USS scheduling Pcell on sSCell) is less than Z1. Consequently, the scheduling flexibility will worse than self-scheduling on Pcell in legacy NR, since the number of monitored PDCCH candidates is reduced in proposal 1 in DSS To allow a better scheduling flexibility, overbooking for BD/CCE on sSCell should be supported. That is, if total number of BD/CCE exceeds Z1 in a slot, USS set on sSCell can be dropped. By this way, more BD/CCE on sSCell is possible if there is no/limited CSS/USS on Pcell. If companies have concern on overbooking on sSCell, we are OK to define different UE capability for different options of overbooking. |
| Lenovo, Motorola Mobility |  | We think the BD handling option (option A) could limit scheduling flexibility as mentioned earlier. In addition, it would be good to clarify that if BD limit corresponding to an SCS per cell is applicable (we think it is applicable, and in Huawei’s example, no more than 36 candidates can be configured to be monitored in the 1st slot of sSCell for Pcell scheduling). |
| OPPO | Not support. | Our preference is Alt 2-2 due to implementation complexity concern. Further, the BD/CCE limit handling as proposed is also not OK to us. If SCS is different between Pcell and sSCell, an implicit BD/CCE limit is needed for the cell with larger SCS. For an example, Pcell is configured with 15kHz, and sSCell is with 30kHz, Z1=44(using the SCS of Pcell as reference SCS), when max(x1+x2) is less than 8, the maximum value of y should be limited to 36, because of the single cell capability limit for SCS = 30kHz. When max(x1+x2) is 44, there is no space for BD in sSCell. It needs to extend the value of Z1 and to set additional limitation for each cell to solve the above problems. That is to say, separate BD/CCE limitations should be defined for each scheduling cell, e.g. max(x1(m1)+x2(m2)) =Z3=44, max(y(m2))=Z4 =36. This is our preference. |
| Spreadtrum | Partially support | We can support Alt 2-1 for progress. However, we would like to add a FFS point like FFS whether dnamic switching of PDCCH monitoring of DCI formats 0_1,1_1,0_2,1_2 between monitoring on Pcell/PSCell USS sets and monitoring on sSCell USS sets can be supported in additionally. We support PDCCH overbooking on sSCell USS set(s) is not allowed. For BD/CCE limit handling, a clarification for y(m), does it mean the sum of #BD/CCE on all the slots of sSCell overlapping with slot m of P(S)Cell? E.g. sSCell is 30kHz, Pcell is 15kHz, then y(m)=y(1)+y(2)? |
| CMCC | Partially support | We support alt 2-1, and also open to alt 2-4. We support no PDCCH overbooking and sScell USS set(s). Regarding the BD/CCE limit, we have the similar comment as Spreadtrum about the definition of y(m2) |
| Ericsson | Support | We support Alt 2-1. We are OK with not allowing PDCCH overbooking on sSCell. We are OK with the PDCCH BD/CCE handling as in our understanding it can allow the NW flexibility to configure and utilize search spaces on P(S)Cell and SCell appropriately according to Alt 2-1, while also providing reduction in UE complexity. While our preference is to have a larger Z1 value, we would be OK with the proposal to have it as FFS. |
| Moderator Notes |  | @Huawei -- Z1 is an additional limit and the 36 limit is still followed in the example figure of your comment (as Lenovo/MotM also commented) @ZTE – “ max of (x1(m1)+x2(m1))+max of y(m2) corresponding to any P(S)Cell slots m1 and m2 is less than or equal to Z1 “ implies this – say PCell has two slots slot m1 and slot m2 and n1 and n2 #BDs are monitored by UE on PCell on those slots respectively. Then say UE monitors on sSCell p1 #BDs in sSCell slot(s) that overlap with PCell slot m1 and p2 #BDs in sSCell slot(s) that overlap with PCell slot m2. Then any combination n1+p1, n1+p2, n2+p1, n2+p2 should be less than or equal to Z1. So, for example if Z1=44 and PCell SCS is 15kHz. Then, if UE monitors say 22 #BDs in a PCell slot, then the max #BDs UE monitors on any sSCell slots per 1ms is <= 44-22 = 22. Hope this clarifies. @Apple – I hope above explanation provides clarification. @Spreadtrum – yes, it is all overlapping sSCell slots “y(m) is #BDs for PDCCH USS(s) candidates monitored on sSCell in sSCell slot(s) that overlap slot m of P(S)Cell” |
| Apple |  | We the explanation from ZTE, we have even more concern. Using 15kHz + 30kHz SCS as example. This is allowed for BD if we understand the explanation correctly n1=22, p1=22, p2=22 That means, before DSS enhancement, we need to have 44 BD per 1ms. Now we need to have 66 BD per 1ms. What is the reason for that? Also the equation is the wrong formulation of what ZTE described. We think the discussion should be focused on the selection of one of the two followings - Simultaneous SpCell/sSCell PDCCH monitoring based - TDM SpCell/sSCell PDCCH monitoring based |

### Proposal 1a

- PDCCH overbooking on sSCell USS set(s) is not allowed

Above proposal already discussed during Friday(UTC) GTW session and corresponding agreement is copied below

Agreement

PDCCH overbooking on sSCell USS set(s) is not allowed

### Proposal 1b

- Further discuss PDCCH BD/CCE limit handling and down-select from following PDCCH monitoring alternatives in RAN1#105e

  - Alt 2-1

    - From RAN1#103 agreement

  - Alt 2-2

    - From RAN1#103 agreement

  - Alt 2-4

    - From RAN1#103 agreement

  - Al 2-4a

    - A UE does not expect to monitor PDCCH (including CSS and USS candidates) for primary cell scheduling on both the primary cell and the sSCell in a same slot

  - Alt 2-5

    - Dynamic switching of PDCCH monitoring of DCI formats 0_1,1_1,0_2,1_2 between monitoring on PCell/PSCell USS sets and monitoring on sSCell USS sets is supported

      - FFS: Details of switching mechanism

    - UE may does not monitor DCI formats 0_1,1_1,0_2,1_2 on both PCell USS set(s) and sSCell USS sets simultaneously in certain scenarios, e.g. when the UE falls back to monitoring USS set on the PCell, it is not forbidden from monitoring the USS set on the sSCell.

  - Alt 2-6

    - The USS set(s) on PSCell/PCell and the USS set(s) on sSCell are configured such that UE does not monitor DCI formats 0_1,1_1,0_2,1_2 on both PCell USS set(s) and sSCell USS set(s) simultaneously

    - SS Group Swithcing is employed for dynamic switching mechanism

Above proposal discussed during Friday (UTC) GTW session.

### Proposal 1c

- Further discuss PDCCH monitoring and BD/CCE limit handling in RAN1#105e considering below BD/CCE limit handling options

  - Option A

    - PDCCH monitoring candidates on P(S)Cell and/or sSCell are configured such that max of (x1(m1)+x2(m1))+max of y(m2) corresponding to any P(S)Cell slots m1 and m2 is less than or equal to Z1

    - At least the case of Z1 = 44 is supported for P(S)Cell SCS 15kHz

      - FFS if Z1 larger than above can also be supported based on UE capability (e.g. similar to BDFactorR in Rel16)

    - FFS signalling details on how the limit Z1 is realized, e.g.

      - RRC configured BD limit/scaling factor-based limit for max(x1(m)+x2(m))

      - Separate RRC configured BD limits/scaling factor-based limits for max(x1(m)+x2(m)) and max(y(m))

      - separate BdfactorR for P(S)Cell and sSCell

    - For sSCell scheduling P(S)Cell, the UE is not required to monitor on the active DL BWP with SCS configuration μ of the sSCell more than MPDCCHmax,slot,μ PDCCH candidates per slot of sSCell.

  - Option B

    - For P(S)Cell slot m, PDCCH monitoring candidates on P(S)Cell and/or sSCell are configured such that x1(m)+x2(m)+y(m) is less than or equal to BD limit Z2

    - At least the case of Z2 = 44 is supported for P(S)Cell SCS 15kHz

      - FFS if Z2 larger than above can also be supported based on UE capability (e.g. similar to BDFactorR in Rel16)

    - max of (x1(m1)+x2(m1)) + max of y(m2) corresponding to any P(S)Cell slots m1 and m2 can is allowed to be larger than BD limit Z2

    - FFS signalling details on how the limit Z2 is realized

    - For sSCell scheduling P(S)Cell, the UE is not required to monitor on the active DL BWP with SCS configuration μ of the sSCell more than MPDCCHmax,slot,μ PDCCH candidates per slot of sSCell.

  - Note

    - x1(m) is #BDs for PDCCH CSS(s) candidates monitored on P(S)Cell slot m

    - x2(m) is #BDs for PDCCH USS(s) candidates monitored on P(S)Cell slot m

    - y(m) is #BDs for PDCCH USS(s) candidates monitored on sSCell in all sSCell slot(s) that overlap slot m of P(S)Cell

    - USS(s) => USS(s) that can schedule PDSCH/PUSCH on P(S)Cell)

Companies are requested to indicate their view about the above proposal in the Table below

| Company Name | support/not support | Comments (Proposal 1c) |
| --- | --- | --- |
| Moderator notes |  | Intention of the proposal to progress more on BD/CCE discussion based on inputs received so far. I added some clarifications to reflect comments above wrt BD/CCE including addressing questions from HW and Spreadtrum. Please check and provide comments e.g. on any additional clarifications needed, any proposals on detailed signaling to realize Z1 or Z2, etc. |
| Samsung | Not support | The main issue to first address is whether it is necessary to introduce a new UE capability for simultaneously monitoring PDCCH from two scheduling cells for a scheduled cell in order for a UE to support DSS. Also, if introduced, whether that capability needs to be mandatory. For the proposal, there is no need to have “at least Z1/Z2=44 is supported” at this moment. There are also aspects that are unclear. For example, why is there x1(m) and x2(m) instead of a single x(m)? |
| LG |  | For clear understanding on Option B, we would like to check if there is misunderstanding in our side on FL’s intention, based on some examples below which are provided based on our current understanding. For an example, - SCS of Pcell is 15K, and SCS of sScell is 30K - # of BDs in Pcell slot (m) is A - # of BDs in sScell slot (n) is B1, and # of BDs in sScell slot (n+1) is B2 - Pcell slot (m) is overlapping with both sScell slot (n) and sScell slot (n+1) Then, for Pcell slot (m), {A+B1+B2} is less than or equal to BD limit Z2. Is this correct understanding on Option B? For another example, - SCS of Pcell is 30K, and SCS of sScell is 15K - # of BDs in Pcell slot (m) is A1, and # of BDs in Pcell slot (m+1) is A2 - # of BDs in sScell slot (n) is B - sScell slot (n) is overlapping with both Pcell slot (m) and Pcell slot (m+1) Then, {A1+B} <= Z2 for Pcell slot (m), and {A2+B} <= Z2 for Pcell slot (m+1). Is this correct understanding on Option B? |
| OPPO | Not support this version but could compromise to a modification | We would be fine to proposal if the case having max(x1+X2)<=Z3 and max(y)<=Z4 is included as a special case of Option A, as stated as following (marked in red): - Option A - PDCCH monitoring candidates on P(S)Cell and/or sSCell are configured such that max of (x1(m1)+x2(m1))+max of y(m2) corresponding to any P(S)Cell slots m1 and m2 is less than or equal to Z1. - This may include a configuration having max(x1(m1)+x2(m1))≤Z3 and max(y(m2))≤Z4 for Z3+Z4≤Z1 ….… We have the similar question as from LG. It seems necessary to clarify which subcarrier spacing (the one one PCell or the one on sSCell) is/are taken as reference to count slot/span for slot m. This is also the reason for us to add above (red) sub-sub-bullet, to simplify the SCS issue. |
| Qualcomm | Support | We are OK to discuss the two options in the next RAN1 meeting. Regarding SCS, we do not think SCS of P(S)Cell other than 15kHz is considered for this feature. There is no case where SCS of P(S)Cell is higher than SCS of sSCell. |
| ZTE | Partial support | Thanks for moderator’s clarification and further update. We have the following comments. 1. Comments for clarification. We have agreed that overbooking on sSCell is not allowed. That agreements only precludes the case of overbooking on sSCell itself. It is our understanding that overbooking across PCell and sSCell is still open, that’s why we have Option B here. 2. For Option A, it is not clear whether Z1 refers to MPDCCHmax,slot,μ or MPDCCHtotal,slot,μ. Seems we are discussing MPDCCHmax,slot,μ here, but the BDFactorR seems to be used in MPDCCHtotal,slot,μ calculation in Rel-16. 3. For the second bullet “At least the case of Z1 = 44 is supported for P(S)Cell SCS 15kHz” in Option A, we still think it is too early to confirm that “Z1” is determined by PCell SCS. We would suggest to delete it for now or update it as below. At least the case of Z1 = 44 is supported for P(S)Cell SCS 15kHz and sSCell SCS 15KHz 4. Regarding OPPO’s comments to add another sub-bullet under the 1st bullet of Option A, we think there are multiple ways to define Z4 here, it is better to make it open for now, e.g., defining Z4 per slot or defining Z4 per x slot (x=2^u_sSCell/2^u_PCell), etc.. This seems also related to LG’s question above. |
| vivo | Support with modification | We are generally fine for the proposal. 1st comment on both options: the proposal seems to assume Pcell SCS as the reference SCS to determine the BD/CCE budget. We agree that SCS of P(S)Cell is typically not larger than that of sScell in DSS case. However, it is better to have a design which could be easily extended to general case. To this end, we suggest the following change (Hope this modification could resolve some concerns in the above comments): - Option A - PDCCH monitoring candidates on P(S)Cell and/or sSCell are configured such that max of (x1(m1)+x2(m1))+max of y(m2) corresponding to any reference cell P(S)Cell slots m1 and m2 is less than or equal to Z1 - FFS reference cell (e.g P(S)Cell, P(S)Cell/Scell with smaller SCS) - Option B - For reference cell P(S)Cell slot m, PDCCH monitoring candidates on P(S)Cell and/or sSCell are configured such that x1(m)+x2(m)+y(m) is less than or equal to BD limit Z2 - FFS reference cell (e.g P(S)Cell, P(S)Cell/Scell with smaller SCS) 2nd comment on FFS part of Option A: Another way to determine the separate limit is to derive it from search space configuration. Then the following change is proposed: - FFS signalling details on how the limit Z1 is realized, e.g. - SS configuration-based BD limit for max(x1(m)+x2(m)) and max(y(m)) - RRC configured BD limit/scaling factor-based limit for max(x1(m)+x2(m)) - Separate RRC configured BD limits/scaling factor-based limits for max(x1(m)+x2(m)) and max(y(m)) - separate BdfactorR for P(S)Cell and sSCell |
| CATT | Support | We are fine with the proposal for the pending discussion in the next meeting. We share the same views as companies that it is better to keep the reference SCS open. Additionally, we propose the following modification to make the proposal a little bit clearer For the FFS points under Option A: - FFS signalling details on how the limit Z1 is realized, e.g. - RRC configured BD limit/scaling factor-based limit for max(x1(m)+x2(m)) + max(y(m)) - Separate RRC configured BD limits/scaling factor-based limits for max(x1(m)+x2(m)) and max(y(m)) - separate BdfactorR for P(S)Cell and sSCell For the last sub-bullet for both option A and option B, the upper bound for PDCCH candidates should take the MPDCCHtotal,slot,μ into account: - For sSCell scheduling P(S)Cell, the UE is not required to monitor on the active DL BWP with SCS configuration μ of the sSCell more than min(MPDCCHmax,slot,μ , MPDCCHtotal,slot,μ) PDCCH candidates per slot of sSCell. |
| Xiaomi | Partial support | We are OK to discuss the two options in the next RAN1 meeting. The reference SCS can be FFS. |
| MTK | Partial support Need more clarification | We are generally ok for the high level concept of FL proprosal. However, we share the same view with ZTE that - For Option A, it is not clear whether Z1 refers to MPDCCHmax,slot,μ or MPDCCHtotal,slot,μ. Seems we are discussing MPDCCHmax,slot,μ here, but the BDFactorR seems to be used in MPDCCHtotal,slot,μ calculation in Rel-16. Besides, both Option A and Option B have the bullet below: - For sSCell scheduling P(S)Cell, the UE is not required to monitor on the active DL BWP with SCS configuration μ of the sSCell more than MPDCCHmax,slot,μ PDCCH candidates per slot of sSCell. Comparing the bullet above with current 38.213 spec: - For each scheduled cell from the NcellsDL,μ downlink cells, the UE is not required to monitor on the active DL BWP with SCS configuration μ of the scheduling cell more than minMPDCCHmax,slot,μ,MPDCCHtotal,slot,μ PDCCH candidates or more than minCPDCCHmax,slot,μ,CPDCCHtotal,slot,μ non-overlapped CCEs per slot. It is not evident for us why we need to add the yellow highlighted bullet. |
| ETRI | Support | We are generally fine with the proposal, assuming that further clarification/modification on the options can be made as needed in the next meeting. |
| Apple | Not support | We think the formulation is still questionable. For example, for Alt1 Using 15kHz + 30kHz SCS as example. This is allowed for BD if we understand the explanation correctly n1=22, p1=22, p2=22 That means, before DSS enhancement, we need to have 44 BD per 1ms. Now we need to have 66 BD per 1ms. What is the reason for that? Also the equation is the wrong formulation of what ZTE described. We think the discussion should be focused on the selection of one of the two followings - Simultaneous SpCell/sSCell PDCCH monitoring based - TDM SpCell/sSCell PDCCH monitoring based The current agreement implies that we already support simultaneous SpCell/sSCell PDCCH monitoring based |
| Intel |  | For Option A and B, it needs to clarify what is the relation between MPDCCHmax,slot,μ (in the last sub-bullet) and Z1 or Z2. The proposed Option A and Option B both rely on a reference SCS to define max BD/CCE in case PCell and sSCell have different SCS. However, it is not easily justified either PCell SCS or sSCell SCS should be selected. Therefore, we think one simple way is to separately treat PCell and sSCell - For PCell, a max BD/CCE can be derived by the PCell SCS - For sSCell, another max BD/CCE can be derived by the sSCell SCS Therefore, we propose to consider following option. - Option C - PDCCH monitoring candidates on P(S)Cell are configured such that max of (x1(m1)+x2(m1)) is less than or equal to Z3 - Z3 is derived by the PDCCH monitoring capability of PCell - PDCCH monitoring candidates on sSCell are configured such that max of y(m2) is less than or equal to Z4 - Z4 is derived by the PDCCH monitoring capability of sSCell - FFS details to define Z3 and Z4 - For sSCell scheduling P(S)Cell, the UE is not required to monitor on the active DL BWP with SCS configuration μ of the sSCell more than Z4 PDCCH candidates per slot of sSCell. |
| Nokia, NSB |  | We have a preference on Option A, but believe both options can work. We don’t see it necessary to have every nut and bolt ironed our on both options before a decision can be taken. Samsung note seems true though, depending on the decision on simultaneous PDCCH monitoring there may be implications on what the correct decision here would be. |
| Huawei |  | Don’t get the point of Samsung comment - it seems the WA in the last meeting already support such direction of simultaneous monitoring PDCCH (with/without a separate UE capabilty). Is the point on “in the same slot” or? To us, based on the assumption of “in the same slot” – FFS which slot is fine for the time being, we would support to address BD/CCE limit firtst as the total limit decision would lead us to understand how “simultaneous” can be achieved - e.g. if any condition or restriction needed. We support Intel’s Option C – similar to what we proposed previously as Option C. |
| Ericsson | Support with updates | In general OK with Proposal. We propose to include following option in the FFS list on detailed BD/CCE configuration to realize Z1-- “counting ‘sSCell-to-P(S)Cell’ scheduling as an additional scheduling cell with numerology given by sSCell numerology in determining the BD/CCE limits”. |

Above Proposal updated to 1cv2 below based on comments on [104b-e-NR-DSS-01] email thread

### Proposal 1cv2

- Further discuss PDCCH monitoring and BD/CCE limit handling in RAN1#105e considering below BD/CCE limit handling options

  - Option A

    - At least when P(S)Cell SCS is not higher than sSCell SCS, PDCCH monitoring candidates on P(S)Cell and/or sSCell are configured such that max of (x1(m1)+x2(m1))+max of y(m2) corresponding to any P(S)Cell slots m1 and m2 is less than or equal to Z1

    - At least the case of Z1 = 44 is supported for P(S)Cell SCS 15kHz

      - FFS if Z1 larger than above can also be supported based on UE capability (e.g. similar to BDFactorR in Rel16)

    - FFS signalling details on how the limit Z1 is realized, e.g.

      - RRC configured BD limit/scaling factor-based limit for max(x1(m)+x2(m))

      - Separate RRC configured BD limits/scaling factor-based limits for max(x1(m)+x2(m)) and max(y(m))

      - separate BdfactorR for P(S)Cell and sSCell

      - SS configuration-based BD limit for max(x1(m)+x2(m)) and max(y(m))

      - RRC configured BD limit/scaling factor-based limit for max(x1(m)+x2(m))+ max(y(m))

      - Counting ‘sSCell-to-P(S)Cell’ scheduling as an additional scheduling cell with numerology given by sSCell numerology in determining the BD/CCE limits

    - FFS reference SCS to use when P(S)Cell has higher SCS than sSCell

    - For sSCell scheduling P(S)Cell, the UE is not required to monitor on the active DL BWP with SCS configuration μ of the sSCell more thanMPDCCHmax,slot,μ PDCCH candidates per slot of sSCell.

      - FFS how limitMPDCCHtotal,slot,μ is computed and applied when CCS from sSCell to P(S)Cell is configured

  - Option B

    - At least when P(S)Cell SCS is not higher than sSCell SCS, For P(S)Cell slot m, PDCCH monitoring candidates on P(S)Cell and/or sSCell are configured such that x1(m)+x2(m)+y(m) is less than or equal to BD limit Z2

    - At least the case of Z2 = 44 is supported for P(S)Cell SCS 15kHz

      - FFS if Z2 larger than above can also be supported based on UE capability (e.g. similar to BDFactorR in Rel16)

    - max of (x1(m1)+x2(m1)) + max of y(m2) corresponding to any P(S)Cell slots m1 and m2 can is allowed to be larger than BD limit Z2

    - FFS signalling details on how the limit Z2 is realized

    - FFS reference SCS to use when P(S)Cell has higher SCS than sSCell

    - For sSCell scheduling P(S)Cell, the UE is not required to monitor on the active DL BWP with SCS configuration μ of the sSCell more thanMPDCCHmax,slot,μ PDCCH candidates per slot of sSCell.

      - FFS how limitMPDCCHtotal,slot,μ is computed and applied when CCS from sSCell to P(S)Cell is configured

  - [Option C

    - PDCCH monitoring candidates on P(S)Cell are configured such that max of (x1(m1)+x2(m1)) is less than or equal to Z3

      - Z3 is derived by the PDCCH monitoring capability of PCell

    - PDCCH monitoring candidates on sSCell are configured such that max of y(m2) is less than or equal to Z4

      - Z4 is derived by the PDCCH monitoring capability of sSCell

    - FFS details to define Z3 and Z4

    - For sSCell scheduling P(S)Cell, the UE is not required to monitor on the active DL BWP with SCS configuration μ of the sSCell more than Z4 PDCCH candidates per slot of sSCell.]

  - Note

    - x1(m) is #BDs for PDCCH CSS(s) candidates monitored on P(S)Cell slot m

    - x2(m) is #BDs for PDCCH USS(s) candidates monitored on P(S)Cell slot m

    - y(m) is #BDs for PDCCH USS(s) candidates monitored on sSCell in all sSCell slot(s) that overlap slot m of P(S)Cell

    - USS(s) => USS(s) that can schedule PDSCH/PUSCH on P(S)Cell)

Above Proposal updated to 1cv3 below based on comments on [104b-e-NR-DSS-01] email thread

### Proposal 1cv3

- Further discuss PDCCH monitoring and BD/CCE limit handling in RAN1#105e considering below BD/CCE limit handling options

  - Option A

    - At least when P(S)Cell SCS is not higher than sSCell SCS, PDCCH monitoring candidates on P(S)Cell and/or sSCell are configured such that max of (x1(m1)+x2(m1))+max of y(m2) corresponding to any P(S)Cell slots m1 and m2 is less than or equal to Z1

    - At least the case of Z1 = 44 is supported for P(S)Cell SCS 15kHz

      - FFS if Z1 larger than above can also be supported based on UE capability (e.g. similar to BDFactorR in Rel16)

    - FFS signalling details on how the limit Z1 is realized, e.g.

      - RRC configured BD limit/scaling factor-based limit for max(x1(m)+x2(m))

      - Separate RRC configured BD limits/scaling factor-based limits for max(x1(m)+x2(m)) and max(y(m))

      - separate BdfactorR for P(S)Cell and sSCell

      - SS configuration-based BD limit for max(x1(m)+x2(m)) and max(y(m))

      - RRC configured BD limit/scaling factor-based limit for max(x1(m)+x2(m))+ max(y(m))

      - Counting ‘sSCell-to-P(S)Cell’ scheduling as an additional scheduling cell with numerology given by sSCell numerology in determining the BD/CCE limits

    - FFS reference SCS to use when P(S)Cell has higher SCS than sSCell (if supported)

    - For sSCell scheduling P(S)Cell, the UE is not required to monitor on the active DL BWP with SCS configuration μ of the sSCell more thanMPDCCHmax,slot,μ PDCCH candidates per slot of sSCell.

      - FFS how limitMPDCCHtotal,slot,μ is computed and applied when CCS from sSCell to P(S)Cell is configured

  - Option B

    - At least when P(S)Cell SCS is not higher than sSCell SCS, For P(S)Cell slot m, PDCCH monitoring candidates on P(S)Cell and/or sSCell are configured such that x1(m)+x2(m)+y(m) is less than or equal to BD limit Z2

    - At least the case of Z2 = 44 is supported for P(S)Cell SCS 15kHz

      - FFS if Z2 larger than above can also be supported based on UE capability (e.g. similar to BDFactorR in Rel16)

    - max of (x1(m1)+x2(m1)) + max of y(m2) corresponding to any P(S)Cell slots m1 and m2 can is allowed to be larger than BD limit Z2

    - FFS signalling details on how the limit Z2 is realized

    - FFS reference SCS to use when P(S)Cell has higher SCS than sSCell (if supported)

    - For sSCell scheduling P(S)Cell, the UE is not required to monitor on the active DL BWP with SCS configuration μ of the sSCell more thanMPDCCHmax,slot,μ PDCCH candidates per slot of sSCell.

      - FFS how limitMPDCCHtotal,slot,μ is computed and applied when CCS from sSCell to P(S)Cell is configured

  - Option C

    - PDCCH monitoring candidates on P(S)Cell are configured such that max of (x1(m1)+x2(m1)) is less than or equal to Z3

      - Z3 is derived by the PDCCH monitoring capability of PCell

    - PDCCH monitoring candidates on sSCell are configured such that max of y(m2) is less than or equal to Z4

      - Z4 is derived by the PDCCH monitoring capability of sSCell

    - FFS details to define Z3 and Z4, e.g.

      - Separate RRC configured BD limits/scaling factor-based limits for max(x1(m)+x2(m)) and max(y(m))

    - For sSCell scheduling P(S)Cell, the UE is not required to monitor on the active DL BWP with SCS configuration μ of the sSCell more than Z4 PDCCH candidates per slot of sSCell

  - Note

    - x1(m) is #BDs for PDCCH CSS(s) candidates monitored on P(S)Cell slot m

    - x2(m) is #BDs for PDCCH USS(s) candidates monitored on P(S)Cell slot m

    - y(m) is #BDs for PDCCH USS(s) candidates monitored on sSCell in all sSCell slot(s) that overlap slot m of P(S)Cell

    - USS(s) => USS(s) that can schedule PDSCH/PUSCH on P(S)Cell)

### Proposal 2

- When CCS from sSCell to Pcell/PSCell is configured

  - CIF=0 used for sSCell self-scheduling, and CIF for sSCell to Pcell cross-carrier scheduling is explicitly configured using CrossCarrierSchedulingConfig

Above proposal already discussed during Monday GTW session and corresponding agreement is copied below

Agreement

- When CCS from sSCell to Pcell/PSCell is configured

  - CIF=0 used for sSCell self-scheduling, and CIF for sSCell to Pcell cross-carrier scheduling is explicitly configured using RRC signalling

### Proposal 3 (for conclusion)

- When CCS from an SCell (sSCell) to Pcell/PSCell is configured DCI format 2_5 handling is same as in Rel16

Above proposal already discussed during Monday GTW session.

### Discussion point 4

- Companies are encouraged to provide comments on below search space linking framework for CCS from sSCell to P(S)Cell including preference between Alt1, Alt 2 or other alternative.

- When CCS from sSCell to Pcell/PSCell is configured

  - UE is configured with search space set Ssa on P(S)Cell with search space ID = X1

  - If UE is configured with a search space set SSb on sSCell with same search space ID = X1 (i.e., ‘linked SS sets’ approach as in Rel16)

    - DCI formats monitored by UE on SSb with n_CI corresponding to CIF P(S)Cell are used for P(S)Cell PUSCH/PDSCH scheduling

    - For monitoring the DCI formats on SSb used for P(S)Cell PUSCH/PDSCH scheduling

      - Alt 1

        - nrofCandidates is derived from Ssa configuration

        - monitoringSlotPeriodicityAndOffset, monitoringSymbolsWithinSlot, duration are derived from SSb configuration and same parameters are also used for self-scheduling on sSCell (i.e., same approach as in Rel16)

      - Alt 2

        - nrofCandidates is derived from Ssa configuration

        - monitoringSlotPeriodicityAndOffset, monitoringSymbolsWithinSlot, duration are configured independently from the parameters used for self-scheduling on sSCell

          - FFS Details

      - Alt 3

        - other

Companies are requested to provide comments in the Table below

| Company Name | Comments (Discussion point 4) |
| --- | --- |
| Qualcomm | We think it is natural to re-use “linked SS sets” approach as in Rel.15/16. This is also friendly to enable mixed operation of cross-carrier scheduling from sSCell to Pcell/PSCell and sSCell to another SCell(s). Among the alternatives, we think Alt.2 is a flexible solution especially when the SCSs of Pcell/PSCell and sSCell are different. This flexibility is also good to address the restriction comes from the BD budget Z1 – e.g., suppose 44 PDCCH candidates have to be distributed across a slot of Pcell with SCS 15kHz and overlapping two slots of sSCell with SCS 30kHz. With Alt.2, a UE can be configured to monitor sSCell USS for Pcell scheduling only on the first slot of the two sSCell slots with keeping monitoring sSCell USS for sSCell scheduling on both sSCell slots. |
| Samsung | Alt. 2 – provides better functionality for DSS and in general. But we can take this opportunity and fix the RAN2 bug from Rel-15 and generalize Alt. 2 as follows Alt. 2.1: separate SS configuration and SS ID for scheduling cell (sSCell) and scheduled cell (Pcell) |
| Huawei | Alt 2 seems good. But can also be determined in RAN2 later. |
| ZTE | Based on our understanding, Alt.1 is the legacy mechanism, which is preferred by us. Alt.2 seems to offer more flexibility, we can also accept Alt.2 if it is favored by most companies. But we may also need to clarify that Alt.2 is only for sSCell scheduling Pcell, or can also be applied to legacy cross-carrier scheduling, e.g., sSCell scheduling other SCell. |
| Vivo | Reusing “linked SS sets” approach as in Rel.15/16 is preferred by us. Alt. 1 is slightly preferred by us since it is the same as Rel.15/16. We are also open to Alt. 2 if it is beneficial. Another related issue is how to differentiate USS configuration for Pcell self-scheduling and cross carrier scheduling from sScell. For example, only the USS with non-complete SS configuration could be linked to cross carrier scheduling from sScell. Otherwise, it is used for Pcell self-scheduling. |
| MTK | We prefer Alt. 2 for the better flexibility as mentioned by Qualcomm and agree with ZTE to clarify Alt.2 is only for sSCell scheduling Pcell, or can also be applied to legacy cross-carrier scheduling, e.g., sSCell scheduling other SCell. |
| CMCC | Alt 1 can re-use current cross-carrier linked search space configuration framework. But we are also open to Alt 2 to give NW more flexibility. |
| CATT | Alt-2 provides more flexibility which is preferred from our perspective. |
| Xiaomi | Alt. 1 is slightly preferred since it is the same as Rel.15/16. We are also open to Alt. 2 if benefits are observed. |
| ETRI | We slightly prefer Alt. 1 as it is the legacy approach. Alt. 2 seems a general enhancement for cross-carrier scheduling rather than a DSS-specific one. But we are also open to discuss Alt. 2. |
| Apple | Alt 1 is preferred Alt 2 does provide some configuration flexibility, we need some example about what this can be used since we need to access the potential impact on UE implementation. |
| Nokia, NSB | Would prefer Alt-2 as it provides more flexibilility in configuration when the SCS of P(S)cell and sScell are different. |
| InterDigital | Alt 2 provides more flexibility and it is our preference. |
| Lenovo, Motorola Mobility | We are fine in principle with Alt1/2, however, would slightly prefer to discuss it (e.g., decide between Alt-1 and Alt-2) after decision on other high-level aspects (e.g., dormancy procedure of sSCell). |
| LG Electronics | Alt 1, prefer to keep current rule as is. |
| OPPO | Alt-1 is preferred. In R16, both searchSpaceId and nrofCandidates are indicated in the RRC signaling SearchSpace on scheduled cell and other configuration is derived from the associated SS on scheduling cell. It can help to meet the capability budget for PCell by configuring a suitable PDCCH candidate number for SS on sSCell. Furthermore, the SS configured with only searchSpaceId and nrofCandidates on PCell is used only for cross-carrier scheduling from sSCell. There is no problem for R17 to have the same approach for cross-carrier scheduling from sSCell to PCell. |
| Ericsson | We are OK to discuss improvements of existing SS configuration. If modifications are done to the Rel-15/16 SS-linking, then rather than Alt 2, configuring a separate search space for sSCell-to-PCell seems like a cleaner approach, e.g. configure nrofCandidates, monitoringSlotPeriodicityAndOffset, monitoringSymbolsWithinSlot, duration independently for s-p in that search space set configuration. Additionally, it should also be possible to separately configure BDs for uplink and downlink DCI formats, i.e. configuration of individual DCI format(s) (e.g. only DCI format 0-1, only DCI format 1-1) should be supported as part of the corresponding search space set configuration. |
| Intel | Alt 1 is preferred to maximize reusing existing CCS framework. |
| Qualcomm2 | We do not prefer to re-design the framework of cross-carrier scheduling configuration (i.e., “linked SS sets” approach as in Rel.15/16). This would cause different versions of general cross-carrier scheduling frameworks. Alt.2 basically does not change the framework, except the 38.331 description of RRC IE SearchSpace, highlighted by red below. “The IE SearchSpace defines how/where to search for PDCCH candidates. Each search space is associated with one ControlResourceSet. For a scheduled cell in the case of cross carrier scheduling, except for nrofCandidates, all the optional fields are absent (regardless of their presence conditions)” If majority wants Alt.1, it is fine for us. However, we think Alt.2 offers benefits with minimal impact. |
| Nokia, NSB | We prefer option 2 for better configurability with no real additional impact. We would support Samsung’s amendment to Alt 2. |

# 3 Conclusions

Following agreements were made in RAN1#104b-e based on discussions in [104b-e-NR-DSS-01] email thread and GTW sessions.

Agreement

- When CCS from sSCell to PCell/PSCell is configured

  - CIF=0 used for sSCell self-scheduling, and CIF for sSCell to PCell cross-carrier scheduling is explicitly configured using RRC signalling

Agreement

PDCCH overbooking on sSCell USS set(s) is not allowed

Following was captured in RAN1 Chairman notes

For RAN1#105-e, companies are encouraged to consider:

- Further discuss PDCCH monitoring and BD/CCE limit handling in RAN1#105e considering below BD/CCE limit handling options

  - Option A

    - At least when P(S)Cell SCS is not higher than sSCell SCS, PDCCH monitoring candidates on P(S)Cell and/or sSCell are configured such that max of (x1(m1)+x2(m1))+max of y(m2) corresponding to any P(S)Cell slots m1 and m2 is less than or equal to Z1

    - At least the case of Z1 = 44 is supported for P(S)Cell SCS 15kHz

      - FFS if Z1 larger than above can also be supported based on UE capability (e.g. similar to BDFactorR in Rel16)

    - FFS signalling details on how the limit Z1 is realized, e.g.

      - RRC configured BD limit/scaling factor-based limit for max(x1(m)+x2(m))

      - Separate RRC configured BD limits/scaling factor-based limits for max(x1(m)+x2(m)) and max(y(m))

      - separate BdfactorR for P(S)Cell and sSCell

      - SS configuration-based BD limit for max(x1(m)+x2(m)) and max(y(m))

      - RRC configured BD limit/scaling factor-based limit for max(x1(m)+x2(m))+ max(y(m))

      - Counting ‘sSCell-to-P(S)Cell’ scheduling as an additional scheduling cell with numerology given by sSCell numerology in determining the BD/CCE limits

    - FFS reference SCS to use when P(S)Cell has higher SCS than sSCell (if supported)

    - For sSCell scheduling P(S)Cell, the UE is not required to monitor on the active DL BWP with SCS configuration μ of the sSCell more thanMPDCCHmax,slot,μ PDCCH candidates per slot of sSCell.

      - FFS how limitMPDCCHtotal,slot,μ is computed and applied when CCS from sSCell to P(S)Cell is configured

  - Option B

    - At least when P(S)Cell SCS is not higher than sSCell SCS, For P(S)Cell slot m, PDCCH monitoring candidates on P(S)Cell and/or sSCell are configured such that x1(m)+x2(m)+y(m) is less than or equal to BD limit Z2

    - At least the case of Z2 = 44 is supported for P(S)Cell SCS 15kHz

      - FFS if Z2 larger than above can also be supported based on UE capability (e.g. similar to BDFactorR in Rel16)

    - max of (x1(m1)+x2(m1)) + max of y(m2) corresponding to any P(S)Cell slots m1 and m2 can is allowed to be larger than BD limit Z2

    - FFS signalling details on how the limit Z2 is realized

    - FFS reference SCS to use when P(S)Cell has higher SCS than sSCell (if supported)

    - For sSCell scheduling P(S)Cell, the UE is not required to monitor on the active DL BWP with SCS configuration μ of the sSCell more thanMPDCCHmax,slot,μ PDCCH candidates per slot of sSCell.

      - FFS how limitMPDCCHtotal,slot,μ is computed and applied when CCS from sSCell to P(S)Cell is configured

  - Option C

    - PDCCH monitoring candidates on P(S)Cell are configured such that max of (x1(m1)+x2(m1)) is less than or equal to Z3

      - Z3 is derived by the PDCCH monitoring capability of PCell

    - PDCCH monitoring candidates on sSCell are configured such that max of y(m2) is less than or equal to Z4

      - Z4 is derived by the PDCCH monitoring capability of sSCell

    - FFS details to define Z3 and Z4, e.g.

      - Separate RRC configured BD limits/scaling factor-based limits for max(x1(m)+x2(m)) and max(y(m))

    - For sSCell scheduling P(S)Cell, the UE is not required to monitor on the active DL BWP with SCS configuration μ of the sSCell more than Z4 PDCCH candidates per slot of sSCell

  - Note

    - x1(m) is #BDs for PDCCH CSS(s) candidates monitored on P(S)Cell slot m

    - x2(m) is #BDs for PDCCH USS(s) candidates monitored on P(S)Cell slot m

    - y(m) is #BDs for PDCCH USS(s) candidates monitored on sSCell in all sSCell slot(s) that overlap slot m of P(S)Cell

    - USS(s) => USS(s) that can schedule PDSCH/PUSCH on P(S)Cell)

# 4 References

- R1-2102309	Discussion on SCell PDCCH scheduling P(S)Cell PDSCH or PUSCH	Huawei, HiSilicon

- R1-2102416	Discussion on cross-carrier scheduling from SCell to PCell	OPPO

- R1-2102471	Discussion on cross-carrier scheduling from SCell to Pcell	Spreadtrum Communications

- R1-2102503	Discussion on Cross-Carrier Scheduling from SCell to PCell	ZTE

- R1-2102544	Discussion on Scell scheduling Pcell	vivo

- R1-2102611	Discussion on cross-carrier scheduling from Scell to Pcell	CATT

- R1-2102684	On Cross-Carrier Scheduling from SCell to PCell/PSCell	MediaTek Inc.

- R1-2102803	Om cross-carrier scheduling from SCell to Pcell	Nokia, Nokia Shanghai Bell

- R1-2102902	Discussion on cross-carrier scheduling from SCell to Pcell	CMCC

- R1-2102968	Discussion on Cross-carrier scheduling from SCell to PCell	Xiaomi

- R1-2103052	On SCell scheduling PCell transmissions	Intel Corporation

- R1-2103126	Views on Rel-17 DSS SCell scheduling PCell	Apple

- R1-2103188	Cross-carrier scheduling from an SCell to the PCell/PSCell	Qualcomm Incorporated

- R1-2103202	Search space monitoring in sSCell and PCell	InterDigital, Inc.

- R1-2103262	Cross-carrier scheduling from SCell to PCell	Samsung

- R1-2103333	Cross-carrier scheduling from SCell to Pcell	ETRI

- R1-2103359	Discussion on cross-carrier scheduling from SCell to Pcell	LG Electronics

- R1-2103596	Discussion on cross-carrier scheduling enhancements for NR DSS	NTT DOCOMO, INC.

- R1-2103609	Cross-carrier scheduling (from Scell to Pcell)	Lenovo, Motorola Mobility

- R1-2103645	Enhanced cross-carrier scheduling for DSS	Ericsson

- R1-2103659	Discussion on cross-carrier scheduling from sSCell to PCell/PSCell	ASUSTeK

# 5 Annex A – Agreements from previous meetings

## Agreements from RAN1#102-e

Agreements:

- Following scheduling combinations are allowed/not allowed when cross-carrier scheduling from an SCell to PCell/PSCell is configured

  - self-scheduling on PCell/PSCell is allowed

  - cross-carrier scheduling from PCell/PSCell to another SCell is not allowed

  - self-scheduling on the ‘SCell used for scheduling PCell/PSCell’ is allowed

  - cross-carrier scheduling from the ‘SCell used for scheduling PCell/PSCell’ to another serving cell is allowed

  - cross-carrier scheduling from another serving cell to the ‘SCell used for scheduling PCell/PSCell’ is not allowed

- FFS: Search space and DCI format handling for the allowed cases above

Agreements:

- Configuring 2 or more Scells to schedule the PCell/PSCell is not allowed

## Agreements from RAN1#103-e

Conclusion

- When CCS from sSCell to PCell/PSCell is configured, the configuration of Type 3 CSS set for DCI formats 2_0, 2_1, 2_2, 2_3, 2_4 and applicability of the information in the DCI formats are the same as in Rel-15/Rel-16

  - FFS: DCI format 2_5 and DCI Format 2_6 handling

- Note: The SCell configured with CCS to Pcell/PSCell is referred to as ‘sSCell’

Conclusion

- When the PCell/PSCell and sSCell use different numerologies, the PDSCH reception preparation time between the PDCCH on the sSCell and the PDSCH on the PCell/PSCell is applied (i.e., as specified in TS38.214 Section 5.5).

Agreements:

- When CCS from an SCell (sSCell) to PCell/PSCell is configured, UE monitors Type 0/0A/1/2 CSS sets (for the DCI formats associated with those SS sets) only on the PCell/PSCell and not on the sSCell

  - Note: UE monitors Type 0/0A/2 CSS only on PCell while Type 1 CSS can be monitored on PCell/PSCell

Agreements:

- Discuss in RAN1#104-e how to handle ‘DCI formats 0_1,1_1,0_2,1_2 scheduling PDSCH/PUSCH on PCell/PSCell’ from USS set(s), when CCS from sSCell to PCell/PSCell is configured.. Below alternatives can be considered in the discussion (other alternatives are not precluded)

- Below alternatives can be considered in the discussion (other alternatives are not precluded)

  - Alt 1: When CCS from sSCell to PCell/PSCell is configured, UE cannot be configured to monitor DCI formats 0_1,1_1,0_2,1_2 on PCell/PSCell USS set(s), and can be configured to monitor them only on the sSCell USS set(s)

  - Alt 2: When CCS from sSCell to PCell/PSCell is configured, UE can be configured to monitor DCI formats 0_1/1_1/0_2/1_2 on PCell/PSCell USS set(s), and/or on sSCell USS set(s). The PDCCH monitoring is based on following alternatives (other alternatives are not precluded)

    - Alt 2-1:

      - UE can monitor DCI formats 0_1,1_1,0_2,1_2 on both PCell USS set(s) and sSCell USS sets simultaneously

        - FFS activation/deactivation of scheduling from sSCell to PCell/PSCell

    - Alt 2-2:

      - Dynamic switching of PDCCH monitoring of DCI formats 0_1,1_1,0_2,1_2 between monitoring on PCell/PSCell USS sets and monitoring on sSCell USS sets is supported

        - FFS: Details of switching mechanism (e.g. based on SS group switching, based on BWP switching,…)

      - UE does not monitor DCI formats 0_1,1_1,0_2,1_2 on both PCell USS set(s) and sSCell USS sets simultaneously

    - Alt 2-3:

      - UE does not monitor the same DCI format on both PCell USS set(s) and sSCell USS sets simultaneously. UE can monitor some DCI formats on sSCell USS sets and other DCI formats on PCell/PSCell USS sets simultaneously

    - Alt 2-4:

      - The USS set(s) on PSCell/PCell and the USS set(s) on sSCell are configured such that UE does not monitor DCI formats 0_1,1_1,0_2,1_2 on both PCell USS set(s) and sSCell USS set(s) simultaneously

- FFS following aspects

  - Impact of sSCell activation/deactivation and sSCell dormancy

  - Impact on BD/CCE limit handling including considering PDCCH monitoring on CSS sets and PDCCH monitoring of ‘DCI formats 0_0, 1_0 scheduling PUSCH/PDSCH on PCell/PSCell’

  - Whether PDCCH overbooking on sSCell is supported or not supported and impact (if any) on overbooking handling on PCell/PSCell

  - Impact from different numerologies between PDCCH on the PCell/PSCell and that on the sSCell

  - Whether or not to have mechanism for activation/deactivation of scheduling from sSCell to PCell/PSCell

  - USS configuration details (e.g. handling of USS type (self-scheduling, cross carrier scheduling) for a configured USS set configured for scheduling of in PCell/PSCell)

## Agreements from RAN1#104-e

Agreement

When CCS from sSCell to PCell/PSCell is configured,

- Out of order scheduling is not allowed between a) PDSCH on PCell/PSCell scheduled by PDCCH on PCell/PSCell and b) PDSCH on PCell/PSCell scheduled by PDCCH on sSCell

- Out of order scheduling is not allowed between a) PUSCH on PCell/PSCell scheduled by PDCCH on PCell/PSCell and b) PUSCH on PCell/PSCell scheduled by PDCCH on sSCell

FFS: Whether this agreement requires RAN1 specification impact.

Agreement

When CCS from sSCell to PCell/PSCell is configured,

- Simultaneous reception of a) unicast PDSCH on PCell/PSCell scheduled from PCell/PSCell and b) unicast PDSCH on PCell/PSCell scheduled from sSCell is not allowed

- Simultaneous transmission of a) PUSCH on PCell/PSCell scheduled from PCell/PSCell and b) PUSCH on PCell/PSCell scheduled from sSCell is not allowed

- Note: Simultaneous implies full/partial time overlapping

FFS: Whether this agreement requires RAN1 specification impact.

Agreement

- When CCS from sSCell to PCell/PSCell is configured, CA activation/deactivation operation for the sSCell is supported

Working Assumption

- When CCS from sSCell to PCell/PSCell is configured, UE can be configured to monitor DCI formats 0_1/1_1/0_2/1_2 that schedule PDSCH/PUSCH on PCell/PSCell on PCell/PSCell USS set(s), and/or on sSCell USS set(s)

- The WA to be confirmed after agreements are made on PDCCH BD/CCE handling and PDCCH overbooking handling for CCS from sSCell to PCell/PSCell

- Specs also allow UEs supporting functionality of only Alt-1. Capability signaling details, if any, can be handled during the UE capability discussion for Rel17

- FFS: Whether the UE can monitor PDCCH from both cells in the same slot.

Agreement

- When CCS from sSCell to PCell/PSCell is configured, UE monitors ‘DCI formats 0_0 and 1_0 in CSS that schedule PDSCH/PUSCH on PCell/PSCell’ only on the PCell/PSCell and not on the sSCell