3GPP TSG RAN WG1 #104bis-e	R1-2104018

e-Meeting, April 12th – 20th, 2021

Title:	Further Reply LS on power control for NR-DC

Response to:	R4-2103373 (Further Reply LS on power control for NR-DC)

Release:	Release 16

Work Item:	LTE_NR_DC_CA_enh-Core

Source:	RAN1

To:	RAN4

Cc:	RAN2

Contact person:	Hong He		hhe5@apple.com

Zichao Ji 	zichao.ji@vivo.com

Attachments:	None

# 1	Overall description

According to the current TS38.213, if a UE is configured with both MCG and SCG using NR radio access in FR2, the maximum power for FR2 for transmissions in MCG (PMCG) is given by p-NR-FR2 corresponding to MCG, and the maximum power for FR2 for transmissions in SCG (PSCG) is given by p-NR-FR2 corresponding to SCG. Consequently, not introducing p-NR-FR2 is not consistent with current RAN1 specifications and would result in undefined power control for both uplink CCs of MCG in FR2 and uplink CCs of SCG in FR2.

RAN1 is discussing below changes as one possible solution to define power control for uplink CCs of MCG in FR2 and uplink CCs of SCG in FR2 in NR-DC case without requiring p-NR-FR2

----start TP to sub clause 7.6.2 of TS 38.213----------

If a UE is configured with an MCG using NR radio access in FR1 or in FR2 and with a SCG using NR radio access in FR2 or in FR1, respectively, or if a UE is configured with an MCG using NR radio access in FR2 and with a SCG using NR radio access in FR2 and is not configured with p-NR-FR2, the UE performs transmission power control independently per cell group as described in Clauses 7.1 through 7.5.

--------- end TP ----------------

However, RAN1 cannot determine if it is a feasible solution and would like to ask RAN4 inputs about the feasibility, for the following two possible cases:

1) uplink CCs of MCG and uplink CCs of SCG are in different frequency bands in FR2.

2) uplink CCs of MCG and uplink CCs of SCG are in the same frequency band in FR2.

In [R1-2007509], RAN4 informed RAN1 that discussion on p-UE-FR2 is postponed to Rel17. RAN1 would like to know if RAN4 expected to discuss p-NR-FR2 also in Rel17.

# 2	Actions

To 3GPP RAN4:

ACTION: RAN1 respectfully requests that RAN4 takes the above into account and provides feedback on the feasibility of independent power control for uplink CCs of MCG in FR2 and uplink CCs of SCG in FR2 for NR-NR Dual Connectivity.

# 3	Dates of next TSG RAN WG1 meetings

TSG-RAN WG1 Meeting #105-e			May 19th – May 27th, 2021		E-meeting.

TSG-RAN WG1 Meeting #106-e			Aug 16th  – Aug 27th, 2021 	    	E-meeting