3GPP TSG RAN WG1 #104bis-e			R1-2103936

e-Meeting, April 12th – 20th, 2021

Agenda item:	7.2.12

Source: 	Moderator (NTT DOCOMO, INC.), Nokia, Nokia Shanghai Bell, Thales

Title: 	TP on round trip delay drift rate for NTN scenarios

Document for:	Discussion and Decision

- Introduction

This contribution provides text proposal on round trip delay drift rate for NTN scenarios for TR38.821.

- Text proposal for TR 38.821

<Unchanged parts are omitted>

7.1	Requirements and key issues

7.1.1	Delay

In order to reduce the standardization work, the table here below identifies the worst case NTN scenarios to be considered for the delay constraint.

Table 7.1-1: NTN scenarios versus delay constraints, Source [2]

| NTN scenarios | A | B | C1 | C2 | D1 | D2 |
| --- | --- | --- | --- | --- | --- | --- |
|  | GEO transparent payload | GEO regenerative payload | LEO transparent payload |  | LEO regenerative payload |  |
| Satellite altitude | 35786 km |  | 600 km |  |  |  |
| Relative speed of Satellite with respect to earth | negligible |  | 7.56 km per second |  |  |  |
| Min elevation for both feeder and service links | 10° for service link and 10° for feeder link |  |  |  |  |  |
| Typical Min / Max NTN beam foot print diameter (note 1) | 100 km / 3500 km |  | 50 km / 1000 km |  |  |  |
| Maximum propagation delay contribution to the Round Trip Delay on the radio interface between the gNB and the UE | 541.46 ms (Worst case) | 270.73 ms | 25.77 ms |  | 12.89 ms |  |
| Minimum propagation delay contribution to the Round Trip Delay on the radio interface between the gNB and the UE | 477.48 ms | 238.74 ms | 8 ms |  | 4 ms |  |
| Maximum Round Trip Delay variation as seen by the UE (note 2) | Negligible |  | Up to +/- 93.0 µs/sec (Worst case) |  | Up to +/- 47.6 µs/sec |  |
| NOTE 1: The beam foot print diameter are indicative. The diameter depends on the orbit, earth latitude, antenna design, and radio resource management strategy in a given system. NOTE 2: The delay variation measures how fast the round trip delay (function of UE-satellite-NTN gateway distance) varies over time. The worst case is when the satellite moves towards/away from the UE at 10⁰ elevation angle, assuming the UE speed is 1200 km/h. It is expressed in µs/s and is negligible for GEO scenario. NOTE 3: Void NOTE 4: Speed of light used for delay calculation is 299792458 m/s. |  |  |  |  |  |  |

<Unchanged parts are omitted>