3GPP TSG RAN WG1 #105-e	R1-2104195

e-Meeting, May 19-27, 2021

Agenda Item:	8.12.1

Source:	Futurewei

Title:	Group Scheduling Aspects for Connected UEs

Document for:	Discussion/Decision

# Introduction

RAN1 has considered the standardization needed to support group scheduling mechanisms for MBS including the group common PDCCH/PDSCH, transmission and retransmission schemes, DCI, Search Space and CORESET designs. Following up on the progress made in RAN1#104-bis-e, in the contribution, we present our views on the remaining issues that have been identified that needed further discussions.

# Discussion

## Common Frequency Resource

There have been extensive discussions in previous meetings [1] [2] on the design and definition of the common frequency resource for the group common PDCCH and PDSCH. Various aspects such as the configurability of the common frequency resource (CFR) were discussed again in the RAN1#104-bis-e but with no conclusion:

Agreement:

The down-selection of Option 2A and Option 2B for CFR for multicast of RRC-CONNECTED UEs will be made before the end of RAN1#105-e.

In [1], the CFR for multicast of RRC-CONNECTED UEs is defined as confined within the frequency resource of a dedicated unicast BWP with the same numerology (SCS and CP) and configurable starting PRB and the number of PRBs. Also, separate PDSCH-config, PDCCH-config and SPS-config are defined for MBS, and in the absence of MBS traffic, unicast traffic can be scheduled in this CFR.

The CFR as agreed in the above retains similarity with BWP without needing for extensive configurations (e.g., BWP switching) that the BWP framework provides. The CFR can be configured with only with the starting PRB and the number of PRBs within the unicast BWP in a straightforward manner through the SIB. Additional configuration using RRC can also be considered. If SIB signaling is used, then it is expected that such signaling is sent regardless of whether the CFR is equal to the unicast BWP or not. As for the RRC, it is up to network implementation whether to send additional signaling when such needs arise.

Both starting location and the length can be jointly encoded to reduce overhead in the signaling.

Proposal 1: The starting PRB and the number of PRBs of the CFR within the unicast BWP is signaled in the SIB as a baseline. Additional configuration using RRC can also be considered. In the absence of SIB signaling, the starting PRB and the number of PRBs of the CFR equal the unicast BWP.

Proposal 2: Both starting location and the length can be jointly encoded to reduce overhead in the signaling.

Number of CFR

It has been agreed that one CFR is configured per dedicated unicast BWP for multicast of RRC-CONNECTED UEs with the agreement below.

Agreement:

One CFR is supported per dedicated unicast BWP for multicast of RRC-CONNECTED UEs.

- FFS: Whether more than one CFR is supported per dedicated unicast BWP

- FFS: Whether multicast can be supported or not in a dedicated unicast BWP when no CFR is configured for that BWP

Agreement:

If a CFR is configured for multicast in RRC-CONNECTED state and confined within a dedicated unicast BWP, further study the following options.

- Option 1: the CORESET configured in PDCCH-config for unicast in the dedicated unicast BWP can be used for multicast transmission if the CORESET is fully contained in the CFR in frequency domain, and the CORESET configured in PDCCH-config for MBS in the CFR can be used for unicast transmission.

- Option 2: the CORESET configured in PDCCH-config for unicast in the dedicated unicast BWP cannot be used for multicast transmission even if the CORESET is fully contained in the CFR in frequency domain, and the CORESET configured in PDCCH-config for MBS in the CFR cannot be used for unicast transmission.

- Option 3: the CORESET configured in PDCCH-config for unicast in the dedicated unicast BWP can be used for multicast transmission if the CORESET is fully contained in the CFR in frequency domain, but the CORESET configured in PDCCH-config for MBS in the CFR cannot be used for unicast transmission.

- Option 4: the CORESET configured in PDCCH-config for unicast in the dedicated unicast BWP cannot be used for multicast transmission even if the CORESET is fully contained in the CFR in frequency domain, but the CORESET configured in PDCCH-config for MBS in the CFR can be used for unicast transmission.

The remaining question is why more than one CFR needs to be configured per UE, especially now that it has already been agreed that the CFR shares the same numerology as the associated unicast BWP. In addition, flexibility is there to ensure that sufficient capacity can be configured for the single CFR.

Proposal 3: Only 1 CFR per unicast BWP per UE can be configured.

It was concluded in RAN1#104-bis-e that it is up to gNB implementation to schedule unicast on the frequency resources covered by CFR configured for multicast. One reason to support this is the fact that unicast is a subset for multicast. However, the reverse (i.e., supporting multicast in unicast BWP) should not be supported. Only upon configuration a multicast CFR, it is considered active and supports both unicast and multicast. Without such configuration, multicast reception by the UE should not be supported.

Proposal 4: Without CFR configured, multicast reception by default is not supported. In combination with Proposal 3, the support of Option 4 (listed above) is proposed i.e., the CORESET configured in PDCCH-config for unicast in the dedicated unicast BWP cannot be used for multicast transmission even if the CORESET is fully contained in the CFR in frequency domain, but the CORESET configured in PDCCH-config for MBS in the CFR can be used for unicast transmission.

In [3], RAN2 is requesting information about the configuration of the transmission bandwidth for MCCH reception. Since the PDSCH carries the MCCH and the transmission of the PDSCH is within the CFR, the reply LS should provide agreements about any CFR configuration.

## CORESET and PDCCH

In this section, we consider the agreements made in RAN1#104-bis-e and provide our views on the FFS that have been identified.

Search Space

It was agreed that for the search space set of group-common PDCCH of PTM scheme 1 for multicast in RRC_CONNECTED state, the CCE indices are common for different UEs in the same MBS group. In addition, we have also the following agreements in RAN1#104-e and RAN1#104-bis-e, respectively:

Agreements: For search space set of group-common PDCCH of PTM scheme 1 for multicast in RRC_CONNECTED state, further study the following options.

- Option 1: Define a new search space type specific for multicast

- Option 2: Reuse the existing CSS type(s) in Rel-15/16

  - FFS: whether modifications are needed for multicast

- Option 3: Reuse the existing USS in Rel-15/16 with necessary modifications for MBS

  - FFS: detailed modifications

Agreement:

For CSS of group-common PDCCH of PTM scheme 1 for multicast in RRC_CONNECTED state, down-select from the following alternatives (to be decided in RAN1#105):

- Alt 1: support Type-3 CSS

  - The monitoring priority of Type-3 CSS for group-common PDCCH is the same as existing Rel-15/16 CSS, regardless of which DCI format of group-common PDCCH is configured in Type-3 CSS

- Alt 2: support a new Type-x CSS

  - The monitoring priority of new Type-x CSS is determined based on the search space set indexes of the new Type-x CSS set and USS sets, regardless of which DCI format of group-common PDCCH is configured in the new Type-x CSS.

- Alt 3: support both Alt 1 and Alt 2

For a group-common PDCCH, reusing the common search space (CSS) in existing Rel-15/16 specifications (Option 2) means minimal standardization effort. The search space should support a UE receiving both multicast and broadcast services in the Connected state through a single search space. This can be achieved by sharing the same CSS type (e.g., Type3) allowing the UE to access the multicast and broadcast with different RNTI for group-common PDCCH for multicast and broadcast transmissions.

It was agreed to at least support CSS for the SS set of group-common PDCCH for PTM scheme 1 while receiving multicast traffic. The type of CSS to be adopted would have implications on the monitoring priority. Adopting legacy rules where the CSS is mapped before USS would result in ascending order of the SS indices i.e., PTM scheme 1 have higher monitoring priority relative to traffic scheduled over USS.

Proposal 5: The monitoring priority of group-common PDCCH is based on search space set indices and a new CSS type is defined to avoid fixed higher PTM scheme 1 priority over USS.

In the LS [3], RAN2 discussed agreements for broadcast session. Because UEs in the connected state can also receive broadcasts, discussion of the LS is also appropriate in this agenda item. The RAN2 agreements include defining a new RNTI for scheduling MCCH (downlink channel used for transmitting MBS control information from the network to the UE). The group-common PDCCH should support this new RNTI. An action item for RAN1 is to decide the RNTI and DCI format used for MCCH change notifications.

For the CORESETs, RAN2 is also asking for details of the CSS used for MCCH beyond the PDCCH-config for MBS. Additional agreements about the CSS are needed before sending a reply LS to RAN2.

Number of CORESET(s)

There were discussions and questions during RAN1#104-e on whether CORESET(s) for CFR in addition to existing CORESETs for UE dedicated BWP is needed or not. In the meeting before that, we have agreed on the following baseline:

Agreements: For PTM transmission scheme 1, the CORESET for group-common PDCCH is configured within the common frequency resource for group-common PDSCH.

- FFS: number of CORESET(s) for group-common PDCCH within the common frequency resource for group-common PDSCH

For PTM transmission scheme 2, the CORESET for the UE-specific PDCCH is configured within the unicast BWP and can be outside the MBS frequency region. In Release 15, the maximum number of CORESETs per BWP per cell is 3 and confirmed for 10 as the maximum number of search space sets per BWP per cell.

The number of CORESET(s) for group-common PDCCH within the common frequency resource for group-common PDSCH is a question on the capability of an MBS-enabled UE and should be discussed as part of the overall UE capability discussion.

Observation 1: The number of CORESET(s) for group-common PDCCH within the common frequency resource for group-common PDSCH should be considered during UE capability discussions for MBS-enabled UE.

Blind Decodings/CCE

In RAN1#103-e, two options have been identified on the number of BDs and CCEs:

Agreements: Down select from the two options for BDs/CCEs limit for Rel-17 MBS

- Option 1: the maximum number of monitored PDCCH candidates and non-overlapped CCEs per slot per serving cell defined in Rel-15 is kept unchanged for Rel-17 MBS.

- Option 2: For UEs supporting CA capability, the budget of BDs/CCEs of an unused CC can be used for group-common PDCCH to count the number of BDs/CCEs, which is similar to the method used for multi-DCI based multi-TRP in Rel-16.

The above two options can both be adopted instead of selecting one or the other. For UEs without CA capability, Option 1 applies where the Rel-15 rules apply. At the same time, for CA-capable UEs, Option 2 can be applied. In the last meeting, there have been continuing discussion on this, and one of the questions is whether a new UE capability is needed if Option 2 above is agreed.

## Group Scheduling and Retransmission / HARQ processes

In the RAN1#104b-e meeting, it was agreed on the support of initial and retransmission based on PTM scheme 1 and PTP for multicast and SPS transmission, respectively.

Agreement:

The same HARQ process ID and NDI are used for PTM scheme 1 (re)transmissions and PTP retransmissions of the same TB.

Conclusion:

The maximum number of HARQ processes per cell, currently supported for unicast, is kept unchanged for UE to support multicast reception.

- How to allocate HARQ processes between unicast and multicast is up to gNB.

From previous meetings, we also have the following agreements:

Agreements:

For RRC_CONNECTED UEs, if initial transmission for multicast is based on PTM transmission scheme 1, at least support retransmission(s) can use PTM transmission scheme 1.

- FFS: whether to support PTP transmission for retransmission(s).

- FFS: whether to support PTM transmission scheme 2 for retransmission(s).

- FFS: How to indicate the association between PTM scheme 1 and PTP transmitting the same TB.

FFS: If multiple retransmission schemes are supported, then can different retransmission schemes be supported simultaneously for different UEs in the same group?

Agreement:

For RRC_CONNECTED UEs, if ACK/NACK based HARQ-ACK feedback is supported for PTM scheme 1, and if initial transmission for multicast is based on PTM transmission scheme 1, support retransmission(s) using PTP transmission.

- The HARQ process ID and NDI indicated in DCI is used to associate the PTM scheme 1 and PTP transmitting the same TB.

Similarly, in RAN1#104b-e, PTP retransmission is supported for SPS group-common PDSCH:

Agreement:

The retransmission scheme for a given SPS group-common PDSCH can be either PTM scheme 1 or PTP.

- FFS: Whether PTM scheme 1 retransmission and PTP retransmission can be used simultaneously for different UEs in the same MBS group

There are some details needed on the remaining issues that have been raised during the discussions in the previous meeting:

- If multiple retransmission schemes are supported, then can different retransmission schemes be supported simultaneously for different UEs in the same group?

- Whether the UE is expected to receive a new TB#2 transmitted by PTM scheme 1 for a given HPN before the end of the expected transmission of HARQ-ACK of the previous TB#1, which is initially transmitted by PTM scheme 1, for that HPN.

- Whether the UE is expected to receive unicast TB by PTP for a given HPN between the end of the transmission of HARQ-ACK of the previous TB#1, which is initially transmitted by PTM scheme 1, and the start of a new TB#2 transmitted by PTM scheme 1 for that HPN

There exists a scenario where the PTP retransmission is sent to UE #1 simultaneously (same slot) while PTM scheme 1 retransmission is sent to other UEs, sending the same TB. See Figure 1.

Initial transmission

PTM scheme 1

HARQ ID: x0

Retransmission

For Other UEs:

PTM scheme 1

HARQ ID: x0

For UE#1:

PTP

HARQ ID: x0

Fig. 1. HARQ process(es) for initial transmission and retransmissions

Since the same HARQ process ID is used and the NDI has not been toggled, UE #1 is still able to receive the PTM scheme 1 retransmission. Therefore UE #1 is able to simultaneously receive both retransmissions and combine them for decoding of the TB. This provides the benefit for UE#1 to soft combine both the PTM scheme 1 and PTP retransmissions.

Proposal 6: Different retransmission schemes (e.g., PTM scheme 1 and PTP) can be used simultaneously for different UEs in the same group.

- The same HARQ process ID and NDI bit (not toggled) are used to signal transmission of the same TB.

- The soft combining of the same TB from the PTM scheme 1 and PTP retransmissions is left up to UE implementation.

UE Behavior with PTM Scheme 1 and PTP Retransmissions

Considering from the UE perspective, after the transmission of HARQ-ACK for the previous TB#1 with PTM scheme 1, depending on whether an ACK or NACK is sent, the following cases can be expected:

- A new TB#2 by PTM scheme 1, or

- A retransmission of TB#1 by PTM scheme 1, or

- A retransmission of TB#1 by PTP, or

- A new TB#2 by PTM scheme 1 and retransmission of TB#1 by PTP

All three Cases 1-3 can be considered as normal expected behavior.

For Case 4, per legacy HARQ operations the NDI is toggled to signal transmission of a new TB#2. From the UE perspective, it is not expected to receive a new TB#2 before the end of the expected transmission of HARQ-ACK of the previous multicast TB#1. In Case#4, when the UE receives the new TB#2 (with toggled NDI) and a retransmission of TB#1 with the same HARQ ID, then it is a signal to the UE that the previous TB#1 is aborted.

Observation 2: Further study of the UE timeline with regards to the transmission of HARQ-ACK feedback for a previous multicast TB#1 and reception of a new multicast TB#2 is needed.

Finally, we see limited use scenarios that would benefit from the supporting PTM transmission scheme 2 for initial transmission of MBS in the RRC_CONNECTED state.

Proposal 7: PTM transmission scheme 2 for initial transmission of MBS is not supported for Connected UEs.

### SPS Scheduling

For SPS group common PDSCH for MBS, it was agreed that G-CS-RNTI is defined for SPS group-common PDSCH and activation/deactivation of SPS group-common PDSCH, different from CS-RNTI for unicast SPS PDSCH.

Agreement:

Define G-CS-RNTI at least for SPS group-common PDSCH and activation/deactivation of SPS group-common PDSCH, different from CS-RNTI for unicast SPS PDSCH.

- G-CS-RNTI is used for PTM scheme 1 based dynamic retransmission of SPS group-common PDSCH

- FFS: Whether CS-RNTI can be used for PTP retransmission of SPS group-common PDSCH.

- FFS: Number of G-CS-RNTI.

Agreement:

For RRC_CONNECTED UE supporting MBS, support up to 8 configured SPS configurations in a BWP of a serving cell for unicast and MBS in total.

- It is up to gNB implementation to configure the SPS configuration indexes for unicast and MBS, respectively.

Agreement:

The retransmission scheme for a given SPS group-common PDSCH can be either PTM scheme 1 or PTP.

- FFS: Whether PTM scheme 1 retransmission and PTP retransmission can be used simultaneously for different UEs in the same MBS group

Agreement:

Confirm the working assumption:

For activation/deactivation of SPS group-common PDSCH for MBS in RRC_CONNECTED state,

- At least group-common PDCCH is supported

  - FFS: Whether and how to address the missed activation and deactivation

- FFS: Whether UE-specific PDCCH is supported for activation/deactivation

Still unresolved, noted as FFS above, is on the support for different UEs in the same MBS group to receive either PTM scheme 1 or PTP retransmissions. We see similarity in performance benefits in terms of handlings UEs in bad channel conditions by allowing these UEs to receive PTP retransmissions that are targeted for it and leveraging soft combining at the UE.

Proposal 8: The retransmission scheme for a given SPS group-common PDSCH can be either PTM scheme 1 or PTP for different UEs in the same group.

For the activation of SPS group common PDSCH, it was agreed that at least group-common PDCCH is supported for activation. While there may not be strong motivations for UE-specific PDCCH activation since the scheduling of SPS should be applicable to the whole group, there would be instances where the deactivation would be beneficial if it is UE-specific. For example, when the UE is completing a handoff, in order to avoid disrupting other UEs on that SPS process, it is better to send the SPS deactivation from the UE-specific search space.

The frequency of activation/deactivation of SPS group common PDSCH is service dependent. For some services such as video, there may be a need to deactivate frequently if video data is not available in time. Other services may need to activate/deactivate infrequently. By allowing UEs to skip monitoring the PDCCH for SPS activation and deactivation, one benefit is power savings.

Proposal 9: At least UE-specific PDCCH is supported for deactivation of SPS group-common PDSCH.

To support different MBS traffic with different requirements and deployment scenarios, more than one SPS configuration should be supported.

Proposal 10: Support of more than one SPS group-common PDSCH configuration.

# Conclusion

In this contribution, we discussed and presented our views on group scheduling for MBS. The following observation and proposals have been made.

Proposal 1: The starting PRB and the number of PRBs of the CFR within the unicast BWP is signaled in the SIB as a baseline. Additional configuration using RRC can also be considered. In the absence of SIB signaling, the starting PRB and the number of PRBs of the CFR equal the unicast BWP.

Proposal 2: Both starting location and the length can be jointly encoded to reduce overhead in the signaling.

Proposal 3: Only 1 CFR per unicast BWP per UE can be configured.

Proposal 4: Without CFR configured, multicast reception by default is not supported. In combination with Proposal 3, the support of Option 4 (listed above) is proposed i.e., the CORESET configured in PDCCH-config for unicast in the dedicated unicast BWP cannot be used for multicast transmission even if the CORESET is fully contained in the CFR in frequency domain, but the CORESET configured in PDCCH-config for MBS in the CFR can be used for unicast transmission.

Proposal 5: The monitoring priority of group-common PDCCH is based on search space set indexes and a new CSS type is defined to avoid fixed higher PTM scheme 1 priority over USS.

Observation 1: The number of CORESET(s) for group-common PDCCH within the common frequency resource for group-common PDSCH should be considered during UE capability discussions for MBS-enabled UE.

Proposal 6: Different retransmission schemes (e.g., PTM scheme 1 and PTP) can be used simultaneously for different UEs in the same group.

- The same HARQ process ID and NDI bit (not toggled) are used to signal transmission of the same TB.

- The soft combining of the same TB from the PTM scheme 1 and PTP retransmissions is left up to UE implementation.

Observation 2: Further study of the UE timeline with regards to the transmission of HARQ-ACK feedback for a previous multicast TB#1 and reception of a new multicast TB#2 is needed.

Proposal 7: PTM transmission scheme 2 for initial transmission of MBS is not supported for Connected UEs.

Proposal 8: The retransmission scheme for a given SPS group-common PDSCH can be either PTM scheme 1 or PTP for different UEs in the same group.

Proposal 9: At least UE-specific PDCCH is supported for deactivation of SPS group-common PDSCH.

Proposal 10: Support of more than one SPS group-common PDSCH configuration.

# References

Chairman notes, RAN1#104-bis-e, April 12-20, 2021.

Chairman notes, RAN1#104e, Jan 25 – Feb 5, 2021.

R1-2104165, “LS on broadcast session delivery and MCCH design”, RAN1#105, May 10-27, 2021