3GPP TSG RAN WG1 Meeting #105-e	R1-2104256

E-meeting, May 10th – May 27th, 2021

Agenda Item:	8.4.2

Source:	Huawei, HiSilicon

Title:	Discussion on UL time and frequency synchronization enhancement for NTN

Document for:	Discussion and Decision

Introduction

Agreement:

The Timing Advance applied by an NR NTN UE in RRC_IDLE/INACTIVE and RRC_CONNECTED is given by:

TTA=NTA+NTA,UE-specific+NTA,common+NTA,offset×Tc

Where:

- NTA  is defined as 0 for PRACH and updated based on TA Command field in msg2/msgB and MAC CE TA command.

  - FFS: details of NTA update/accumulation.

- NTA,UE-specific  is UE self-estimated TA to pre-compensate for the service link delay.

- NTA,common is network-controlled common TA, and may include any timing offset considered necessary by the network.

- NTA,common with value of 0 is supported.

  - FFS:  details of signaling including granularity.

- NTA,offset is a fixed offset used to calculate the timing advance.

Note-1: Definition of NTA is different from that in RAN1#103-e agreement.

Note-2: UE might not assume that the RTT between UE and gNB is equal to the calculated TA for Msg1/Msg A.

Note-3: NTA,common is the common timing offset X as agreed in RAN1 #103-e.

Agreement:

Support serving-satellite ephemeris broadcast based on one or more of the following:

- Set 1: Satellite position and velocity state vectors:

  - position X,Y,Z in ECEF (m)

  - velocity VX,VY,VZ in ECEF (m/s)

- Set 2: At least the following parameters in orbital parameter ephemeris format:

  - Semi-major axis α [m]

  - Eccentricity e

  - Argument of periapsis ω [rad]

  - Longitude of ascending node Ω [rad]

  - Inclination i [rad]

  - Mean anomaly M [rad] at epoch time to

    - FFS: Whether pre-provisioned ephemeris based on orbital elements can be used as reference. Thereby, only delta corrections can be broadcast in order to reduce the overhead

- FFS: The field size for each parameter

- FFS: The impact on signaling due to the required accuracy of serving-satellite ephemeris

- FFS: Whether down-selection is needed or both sets are supported

Conclusion:

The orbital propagator model to be used at UE side can be left to implementation.

In RAN1#104bis-e, the following agreements were reached to enhance the uplink timing and frequency synchronization for NTN [1]

In this contribution, we discuss the key remaining issues of time and frequency synchronization in NTN and also provide our views on satellite ephemeris.

Discussion

UL timing synchronization

Indication of common TA

It was agreed in RAN1#104bis-e that the Timing Advance applied by an NR NTN UE in RRC_IDLE/INACTIVE and RRC_CONNECTED is given by:

TTA=NTA+NTA,UE-specific+NTA,common+NTA,offset×Tc

Where:

- NTA  is defined as 0 for PRACH and updated based on TA Command field in msg2/msgB and MAC CE TA command.

  - FFS: details of NTA update/accumulation.

- NTA,UE-specific  is UE self-estimated TA to pre-compensate for the service link delay.

- NTA,common is network-controlled common TA, and may include any timing offset considered necessary by the network.

- NTA,common with value of 0 is supported.

  - FFS:  details of signaling including granularity.

- NTA,offset is a fixed offset used to calculate the timing advance.

Note-1: Definition of NTA is different from that in RAN1#103-e agreement.

Note-2: UE might not assume that the RTT between UE and gNB is equal to the calculated TA for Msg1/Msg A.

Note-3: NTA,common is the common timing offset X as agreed in RAN1 #103-e.

The value of  NTA,common is dependent on the location of reference point (RP) where uplink and downlink frames are aligned and the value is determined by the RTD between RP and the satellite. As NTA,common is network-controlled common TA, there exist three scenarios for the RP location.

Figure 1. RP for common TA located at the gNB

The first scenario is that RP is located at the gNB. As shown in Figure 1 (a), a large TA which is the sum of service link RTD and feeder link RTD is applied at the UE side and the UL frame timing and DL frame timing is aligned at the gNB. With this option, UE with GNSS can calculate the RTD of service link based on its GNSS acquired position and satellite position (obtained from serving satellite ephemeris). Meanwhile, the RTD of feeder link should be broadcast as the common TA. In this scenario, the UE can obtain the distances between the satellite and the gateway according to the full feeder link RTD. As shown in Figure 1(b), UE can collect d1, d2, d3 and derive the location of the gateway.

Observation 1: There will be a large timing offset between the DL and UL frame timing at the UE side when reference point for NTA,common is at the gNB.

Figure 2 RP for common TA located at the satellite

The second scenario is that RP is located at the satellite. In this scenario, the delay compensated by network is equal to the feeder link RTD, so UE only takes care of propagation delay introduced by service link which can be calculated by a GNSS UE with its position and satellite ephemeris. As shown in Figure 2, the feeder link RTD compensated by network is time-variant when RP is located at the satellite. As a result, the timing offset between the DL and UL frame timing at the gNB is time-variant. As the variation is equal to the time drift rate of feeder link, the implementation complexity for UL reception as well as gNB scheduler is high.

Observation 2: The timing drift rate between the DL and UL frame at the gNB is equal to the timing drift rate of feeder link when reference point for NTA,common is at the satellite.

Observation 3: The high timing offset variation between the DL and UL frame timing will introduce more implementation complexities to the gNB.

Figure 3 RP for common TA is under network control

(a) RP at feeder link (b) RP at service link

The third scenario is that RP is located either at the service link or at the feeder link, as illustrated in Figure 3. When RP is located at the feeder link part (shown in Figure 3 (a)), the network compensates part of feeder link propagation delay, the common TA indicate the rest of the feeder link propagation delay to UE. Then, the common TA is added to the calculated service link delay, and UE can get its TA value, which is applied to adjust the uplink timing. When the network compensates feeder link propagation delay and part of service link propagation delay as shown in Figure 3 (b), the common TA can indicate the compensated part of the service link propagation delay to the UE. The TA value can be obtained by removing the indicated common TA from the calculated service link delay. Another benefit is that the RP can be selected to ensure that compensated timing drift at the gNB is constant within a period of time instead of time-variant feeder link RTD to resolve the implementation complexities at the gNB.

Proposal 1: The delay compensated by the gNB should consider the gNB implementation complexity.

In summary, the common TA can be determined with the following steps:

- The network determines its compensated delay Z.

- The network calculates the feeder link RTD Y from the gateway/gNB position and satellite position.

- Common timing offset NTA,common= Y – Z

Depending of the value of Z, the value of NTA,common can be either positive or negative. Take one extra bit for the sign of  NTA,common, the maximum bits for LEO-600km with 10° elevation is 19 bits with granularity of 16∙64/2μ.

Proposal 2: The common TA is determined as the RTD from the reference point to the satellite, i.e. by subtracting the delay compensated at the gNB from the feeder link RTD.

Proposal 3: NTA,common can be a positive or negative value to support the corresponding reference point at both service link and feeder link.

Indication of location of reference point

An alternative solution for common TA indication is that the position of RP is broadcast and NTA,common is calculated by UE according the satellite and RP positions. One benefit of this solution is that when the RP is located gNB or a RU of the network, the information of RP will not update as frequent as NTA,common, which can avoid frequent SIB reading for UL synchronization.

Proposal 4: Support indication of the location of reference point as an alternative solution for common TA indication.

The need and indication of common TA drift rate

In NTN, the propagation delay from the network to UE location can change very quickly, e.g. the maximum delay variation of service link or feeder link can be up to +/- 47.6µs/sec for LEO-600km. The existing TA maintenance scheme in NR which is based on closed loop MAC-CE indication will result in a large DL signaling overhead if it is applied in NTN to track the large TA variation caused by satellite movement.

The UE specific TA from service link can be derived by UE location and satellite ephemeris, however, the common TA drift from feeder link part cannot be obtained at the UE. Therefore, it is beneficial to apply a common TA drift rate to reduce the signaling overhead and avoid frequent SIB1 decoding as the period of validity of common TA drift rate is much longer than common TA. The residual TA error can be compensated by closed loop TA command.

Observation 4: Appling a common TA drift rate at the UE can reduce the closed loop signaling overhead and avoid frequent SIB1 decoding.

For Msg1/MsgA transmission, TA drift rate can be used to compensate the TA drift between SIB decoding and preamble transmission. For example, UE receives common TA offset and ephemeris at t0 and transmit Msg1/MsgA at t1 , the applied TA can be expressed as

TA(t1 )= NTA,UE-specific(t1)+NTA, offset+NTA, common with margin(t1)×Tc

NTA, common with margint1=NTA, common with margint0+NTA, common,rate∙(t1-t0)

where NTA, common,rate is the common TA drift rate derived from gNB indication.

For UEs in RRC_CONNECTED state, the TA maintenance will be applied in a similar way by using common TA drift rate as open loop TA adjustment and the existing closed loop timing control to compensate the residual timing error. Then, for the uplink transmission after Msg1/MsgA transmission, the applied TA is

NTA,new=NTA,old++NTA,UE-specific(t3)+NTA, common,rate∙∆T+NTA

∆T is the time interval between UL transmission and the reception of NTA, common,rate

The first reception of TAC is carried by the Msg2/MsgB and NTA is calculated by NTA= TA∙16∙64/2μ, where TA =0, 1, 2, ..., 3846, is indicated by the 12 bits TAC field. For other UL transmissions, NTA=TA-31.16∙64/2μ where TA = 0, 1, 2,..., 63 is indicated by 6bits TAC field of MAC CE. The timing for the TA adjustment is shown in Figure 5.

Figure 4. TA update with TA drift rate

Depending on the system information periodicity and propagation delay, the indicated common TA drift rate can be a pre-compensated value by gNB implementation.

(a)                                                                             (b)

Figure 5. Common TA error vs TA rate valid duration

The validity of TA rate is dependent on the elevation and the worst case is when the UE is at the nadir point. To reduce the maximum TA error within one TA drift rate period, the indicated and applied TA drift is the one at the middle of period, as show in Figure 5 (a). The maximum TA error is near the boundary of the updating period. To demonstrate the validity of TA drift rate, the maximum common TA error assuming different periodicity of common TA drift rate is shown in Figure 5 (b). Here LEO-600km and TA drift rate of feeder link part is considered. For SSB and uplink signal with 15kHz SCS, the timing error limit is 12Ts as given in Table 7.1.2-1 in [3]. It can be observed that the validity of TA drift rate is around 2.5s when TA error is 12Ts. The validity can be much longer with increase/decrease of elevation as the change of common TA trend to be linear.

Observation 5: The validity of common TA drift rate is related to elevation and the worst case of validity is over 1s with LEO600.

In RAN1#104bis-e, it was proposed that the common TA variation rate, i.e. the second order of common TA, is additionally signaled so that the validity of common TA can be even longer. This can be viewed as a further optimization to reduce the periodicity of the common TA.

Proposal 5: At least support indicating a common TA drift rate as part of the common TA applied by UE.

Proposal 6: For Msg1/MsgA transmission and TA maintenance, use a common TA drift rate to compensate the TA drift between SIB decoding and UL transmission.

UL frequency alignment

Indication of frequency compensation on DL

Figure 6. UL frequency alignment scheme

The UL frequency alignment scheme is illustrated in Figure 6 and the process is as follows:

1) gNB will apply frequency pre-compensation in DL according to the ephemeris and service area to reduce the residual frequency offset Fd at the UE side.

2) UE estimate the DL frequency offset by tracking the DL reference signal. The calculated DL frequency offset is Fd+Fo, where Fo is total local oscillator frequency deviation of UE FUE and gNB FgNB.

3) If UE cannot differentiate Fd and Fo, it will pre-compensate the Fd+Fo as UE specific frequency offset for UL and the corresponding frequency offset generated at the gNB is approximately two times the total local oscillator frequency errors, i.e. 2‧ Fo.

If Fo is large and not known by the UE, the frequency offset at the gNB will affect the performance of UL. However, if the value of pre-compensation is indicated, the UE can firstly derive the full Doppler frequency offset based on the GNSS-acquired UE position and the serving satellite ephemeris and then derive Fd based on the indicated frequency pre-compensation. As a result, the frequency offset of UL reception at the gNB will be minimized.

Observation 6: With the information of frequency pre-compensation on DL, the frequency offset at the gNB of UL reception will be minimized.

To reduce the DL detection complexity at the UE side, frequency pre-compensation should be applied to minimize the residual frequency offset. For the indication of frequency pre-compensation, which may have a large range (+/-48kHz for 600km LEO at 2 GHz with 10° minimum elevation), the granularity should be less than the tolerance of frequency offset for UL. It is reasonable to normalize the value of frequency pre-compensation to subcarrier spacing. As a result, the granularity is set to be a fraction of subcarrier spacing.

Proposal 7: The indicate DL frequency pre-compensation is normalized to a predefined subcarrier spacing.

Indication of frequency compensation on UL

For UL transmissions, the frequency offset is from service link part and feeder link part and it can be pre-compensated by UE side and post-compensated by gNB side. If post-compensation is applied at the gNB side. The values of frequency offset compensated at the gNB should be known by the UE to determine how much frequency offset should be pre-compensated for UL transmission. As there is almost no difference of complexity for UE to do frequency pre-compensation with different values, it is straight forward to adopt a scheme with as less indication as possible.

Proposal 8: Adopt frequency post/pre-compensation with minimum indication overhead for UL frequency alignment.

To minimize the signaling overhead of frequency indication for UL alignment, there are two solutions to avoid the post-compensation indication, also with the minimum impact to the existing specification. The first solution is that the post-compensated common frequency offset for UL transmission is equivalent to the pre-compensated common frequency offset applied for DL due to the same geometry of UL and DL. For example, the Doppler shift at the UE side caused by satellite movement is calculated by FDL/c‧V‧cos Θ  and FUL/c‧V‧c cos Θ for DL and UL, respectively. FDL and FUL is the central frequency of DL and UL, 
and Θ is the angle between the direction of satellite velocity and satellite-to-user orientation. The post frequency compensation of UL can be derived from DL frequency pre-compensation Fpre-DL as 
(FUL / FDL)‧Fpre-DL. The second solution is that the frequency offset from the feeder link part is compensated by the satellite and the service link part is compensated by UE according to the position and ephemeris. With the two solutions, only one indication of DL pre-compensation is required.

Proposal 9: To reduce the signaling overhead, only DL pre-compensation indication is needed and sufficient for UL frequency alignment.

UL Frequency adjustment for UE in RRC_CONNECTED state

Based on the frequency offset estimated by tracking DL reference signal and indicated frequency offset, a GNSS UE can calculate the frequency offset for its UL transmissions in RRC connected mode. The accuracy of estimated frequency offset for UL frequency adjustment can be ensured even without closed-loop frequency compensation.

Proposal 10: For GNSS UE, closed-loop UL frequency compensation is not needed.

Consideration on satellite ephemeris indication

For the purpose of frequency/timing synchronization and mobility management, the indication of satellite ephemeris is required. There were some discussions in both RAN1 and RAN2 regarding how the satellite ephemeris can be used as well as the satellite ephemeris format. The key factors that needs to be taken into account include the accuracy in order to meet the synchronization requirement, signaling overhead as well as its applicability to different scenarios.

As agreed in the RAN1#4-bis-e, the satellite ephemeris can either be indicated in form of state vector (x, y, z, vx, vy, vz) or Keplerian orbit elements (a, e, ω, Ω, i, M0). The state vector indicate the instant satellite position and velocity in ECEF coordinator and the physical meaning of Keplerian orbit elements are as follows

- Semi-major axis α [m]

- Eccentricity e

- Argument of periapsis ω [rad]

- Longitude of ascending node Ω [rad]

- Inclination i [rad]

- Mean anomaly M [rad] at epoch time t0

Although other parameters are included in the current GNSS ephemeris for the orbital based format, 6 parameters are enough for UL synchronization according to our evaluation.

In this following, the performance of the orbit elements-based model is analyzed via the true satellite orbital data provided by Eutelsat. The ephemeris prediction error comes from two parts: random quantization error and propagator error [2]. For quantization error analysis, the dynamic range and signaling overhead of each Keplerian orbit element is given in Table 1. For semi-major axis, the dynamic range [6500, 43000] km could cover all the LEOs, MEOs and GEOs. For eccentricity, the dynamic range [0, 0.015] is suitable for satellites operating in near circular orbit. The orbit propagator input quantization noise is defined by the allocated bits number for each element.

Table 1 Signalling overhead and dynamic range of Keplerian orbit elements

|  | a | E | ω | Ω | I | M0 |
| --- | --- | --- | --- | --- | --- | --- |
| Bit allocation | 33bit | 19bits | 24bits | 21bits | 20bits | 24bits |
| Range | [6500, 43000]km | [0,0.015] | [0,2π] | [-180°, +180°] | [0°, 90°] | [0,2π] |

As the relative location and velocity between UE and satellite is not only depended on the radial position and velocity of the satellite, both radical and absolute performance are evaluated. Table 2 lists the mean value of the radical and absolute prediction errors at 10s, 20s and 30s by 1000 independent trails.

Table 2 Predication errors by Keplerian orbit elements format

| Period | Radical position error, m | Radical velocity error, m/s | Absolute position error, m | Absolute velocity error, m/s |
| --- | --- | --- | --- | --- |
| 10s | 1.415243964 | 0.002064304 | 6.627862341 | 0.019901675 |
| 20s | 2.556054744 | 0.004017807 | 8.887820108 | 0.032020696 |
| 30s | 3.455422236 | 0.00582607 | 11.03350865 | 0.042707769 |

The evaluation results show that with 140 bits overhead the satellite position and velocity can be predicted with very small error and the degradation is not serious with the increase of signaling period. Note that 1) given satellite physical parameters like mass and geometry could further improve the accuracy of a propagator physical model and lead to a more accurate ephemeris prediction, 2) the along-track direction error is more sensitive to quantization and propagator error, which should be addressed during the performance analysis, and 3) quantization bring in random quantization noise, thus only statistics could present the performance.

Considering the signaling overhead, Keplerian orbit elements based solution can be further optimized, as each element can be quantized with different resolutions. However the state vector has both position and velocity group and should be respectively quantized with the same resolution. To further reduce the overhead or increase the resolution of Keplerian orbit elements, the value of semi-major axis a can be confined to a much smaller range with some bits indicating the satellite type or orbit high range.

For HAPS and ATG scenarios, which has no orbital information, the ephemeris can be indicated by the equivalent Keplerian orbit elements transformed from the instant position and velocity. In summary, orbital based ephemeris can support all the scenarios.

Observation 7: Orbital based ephemeris can support all the scenarios including HAPS and ATG.

Observation 8: The bit allocation for Keplerian orbit elements indication can be optimized to reduce the overall signalling overhead.

In addition, when a list of neighbour satellites or cells are provided in the system information for cell reselection, providing orbit elements for neighbour satellites will reduce the signalling overhead as the neighbour satellites usually locate at the same or adjacent orbits of the service satellite. For example, only orbit elements of M0 need to be indicated for the satellites in the same orbit. In summary, another solution for ephemeris is that both PVT and orbital based elements are broadcast in the system information. The PVT part provide the position and velocity of serving satellite for UL time and frequency synchronization and the orbital part provide the neighboring satellites or cells for mobility management.

Proposal 11: At least Satellite ephemeris format based on the following orbital elements is supported

  - Semi-major axis α [m]

  - Eccentricity e

  - Argument of periapsis ω [rad]

  - Longitude of ascending node Ω [rad]

  - Inclination i [rad]

  - Mean anomaly M [rad] at epoch time t0

Conclusion

In this contribution, we further discuss the issue of UL time and frequency synchronization enhancement for NTN, the following observations and proposals are presented:

Observation 1: There will be a large timing offset between the DL and UL frame timing at the UE side when reference point for NTA,common is at the gNB.

Observation 2: The timing drift rate between the DL and UL frame at the gNB is equal to the timing drift rate of feeder link when reference point for NTA,common is at the satellite.

Observation 3: The high timing offset variation between the DL and UL frame timing will introduce more implementation complexities to the gNB.

Observation 4: Appling a common TA drift rate at the UE can reduce the closed loop signaling overhead and avoid frequent SIB1 decoding.

Observation 5: The validity of common TA drift rate is related to elevation and the worst case of validity is over 1s with LEO600.

Observation 6: With the information of frequency pre-compensation on DL, the frequency offset at the gNB of UL reception will be minimized.

Observation 7: Orbital based ephemeris can support all the scenarios including HAPS and ATG.

Observation 8: The bit allocation for Keplerian orbit elements indication can be optimized to reduce the overall signalling overhead.

Proposal 1: The delay compensated by the gNB should consider the gNB implementation complexity.

Proposal 2: The common TA is determined as the RTD from the reference point to the satellite, i.e. by subtracting the delay compensated at the gNB from the feeder link RTD.

Proposal 3: NTA,common can be a positive or negative value to support the corresponding reference point at both service link and feeder link.

Proposal 4: Support indication of the location of reference point as an alternative solution for common TA indication.

Proposal 5: At least support indicating a common TA drift rate as part of the common TA applied by UE.

Proposal 6: For Msg1/MsgA transmission and TA maintenance, use a common TA drift rate to compensate the TA drift between SIB decoding and UL transmission.

Proposal 7: The indicate DL frequency pre-compensation is normalized to a predefined subcarrier spacing.

Proposal 8: Adopt frequency post/pre-compensation with minimum indication overhead for UL frequency alignment.

Proposal 9: To reduce the signaling overhead, only DL pre-compensation indication is needed and sufficient for UL frequency alignment.

Proposal 10: For GNSS UE, closed-loop UL frequency compensation is not needed.

Proposal 11: At least Satellite ephemeris format based on the following orbital elements is supported

  - Semi-major axis α [m]

  - Eccentricity e

  - Argument of periapsis ω [rad]

  - Longitude of ascending node Ω [rad]

  - Inclination i [rad]

  - Mean anomaly M [rad] at epoch time t0

- References

- RAN1 #104-bis-e, chairman notes.

- Mason, James. "Development of a MATLAB/STK TLE Accuracy Assessment Tool, in Support of the NASA Ames Space Traffic Management Project." arXiv preprint arXiv:1304.0842 (2013).

- 3GPP TS 38.133 “Requirements for support of radio resource management”.