3GPP TSG RAN WG1 Meeting #105-e  									R1-2104301

e-Meeting, May 10th – 27th, 2021

Agenda Item:	8.4.2

Source:	Thales

Title:	Considerations on UL timing and frequency synchronization in NTN

Document for:	Discussion and decision

# Introduction

The agreements related to UL time and frequency synchronization in NTN achieved during RAN1 Meeting #104-bis-e are captured in FL Summary on enhancements on UL time and frequency synchronization for NR NTN [R1-2104076].

In this contribution, we make observations and proposals to address issues identified and discussed during RAN1#104-bis e-meeting.

# UL timing synchronization in NTN

## NTN UE Timing Advance

The Timing Advance to be applied by an NR NTN UE in the different RRC sates was discussed in RAN1 Meeting #104-bis-e and the following agreement was made:

The Timing Advance applied by an NR NTN UE in RRC_IDLE/INACTIVE and RRC_CONNECTED is given by:

TTA=NTA+NTA,UE-specific+NTA,common+NTA,offset×Tc

Where:

- NTA  is defined as 0 for PRACH and updated based on TA Command field in msg2/msgB and MAC CE TA command.

  - FFS: details of NTA update/accumulation.

- NTA,UE-specific  is UE self-estimated TA to pre-compensate for the service link delay.

- NTA,common is network-controlled common TA, and may include any timing offset considered necessary by the network.

- NTA,common with value of 0 is supported.

  - FFS:  details of signaling including granularity.

- NTA,offset is a fixed offset used to calculate the timing advance.

Note-1: Definition of NTA is different from that in RAN1#103-e agreement.

Note-2: UE might not assume that the RTT between UE and gNB is equal to the calculated TA for Msg1/Msg A.

Note-3: NTA,common is the common timing offset X as agreed in RAN1 #103-e.

The following section will address the second FFS and provide more details on the Common TA granularity and signalling.

## Common TA characterization

### Common TA equations

When Common TA is indicated by the Network, the UE needs to compute NTA,common before each uplink transmission (including the PRACH transmission). NTA,common will be derived from the broadcast Common TA values.

To assist the UE to calculate the NTA,common  at time t, the gNB shall broadcast the Common TA (NTA,common t0 ) and Common TA drift rate ( TACommondrift , t0 ) regularly (e.g. every 160ms). The UE can then estimate the  NTA,Common (t) based  on a first-order approximation using Equation 1 hereafter, expressed at unit of Tc:

Equation 1:

NTA,Common t= NTA,common t0+   TACommondrift, t0×(t-t0+δt0)

Further, the timing drift rate variation may need also to be indicated by the network so the UE can keep track of quick common delay changes without the need of frequent decoding of system information. Thus, if the gNB broadcast the TACommondrift_variation_rate , The UE can then estimate the  NTA,Common (t) based  on a second-order approximation using Equation 2 below, expressed at unit of Tc:

Equation 2:

NTA,Common t= NTA,common t0+   TACommondrift, t0×t-t0+δt0+12 TACommondrift_variation_rate, t0×t-t0+δt02

NTA,common t0, TACommondrift, t0 and TACommondrift_variation_rate, t0 denote respectively the Common TA, Common TA drift rate and Common TA drift variation rate acquired by the UE at SIB period t0.

δt0 denotes one-way propagation delay from gNB to the UE at time t0 (UE’s timing). This needs to be considered  in both equations to take into account the ageing of NTA,common t0 values (the NTA,common t0 at time t0 is outdated by δt0 at the reception by the UE).

### Minimum required time period for the acquisition of the common-TA related parameters

The cyclic prefix must accommodate the round trip propagation delay and the propagation channel delay spread. Therefore, for PRACH transmission the UE shall be able to estimate its TA with an accuracy better than  ±CP-delay spread2, depending on the PRACH format. Similarly, in connected state, the UE shall be able to update its TA with an accuracy better than ±CP-delay spread2, depending on the numerology in use.

Further, the maximum allowed error of the round trip propagation delay estimation can be equally split between the estimation of  NTA,UE-specific and NTA,common. Therefore the UE shall be able to self-estimate the Common TA within an error range Common TAerror which at least satisfies the following requirement (RAN4 may define different requirement; e.g based on Time transmission error limit Te):

Common TAerror<CP-Delay spread4  .

And so, for the following cases, if the delay spread is neglected, the maximum tolerable error on common TA calculation should be less than:

- ± 24.20 μs, in case of long PRACH Format0 and 3.

- ± 2.34 μs, in case of short PRACH format A1 with 15 kHz subcarrier spacing (SCS)

- ± 1.17 μs, in case of connected state and 15 kHz SCS

As already discussed in the previous section, the UE should be provided by the Network with other assistance information i.e. Common TA drift rate and Common TA drift variation rate. Otherwise, the UE will need to acquire the Common TA broadcast values very frequently. For example, to be able to apply the Common TA with sufficient accuracy a new Common TA value shall be available at the UE during the last 500ms before PRACH Format0/3 transmission. And when engaged in uplink transmission using 15 kHz SCS, the UE shall acquire new Common TA value almost every radio frame if the Common TA drift rate is not indicated.

Therefore, the Network shall jointly broadcast the Common TA values and assistance information (including at least the Common TA drift rate) to be used for computation of Common TA.

The UE needs to derive the actual Common TA values based on broadcast Common TA and  other assistance information indicated by the Network i.e. Common TA drift rate and Common TA drift variation rate.

To derive the common TA, the UE needs to acquire the periodically broadcasted Common TA, Common TA drift rate and the Common TA drift variation rate. Then,  it  shall perform a first-order approximation of Common TA using Equation 1or a second-order approximation Common TA using Equation 2.

The assistance information acquired in the SIB  will be valid only during a validity time period (equals to CTA_validity_time_max) which  depends on the maximum tolerable error on common TA estimation and the order of the Common TA approximation to be carried out by the UE. At the reception of new assistance information, the UE shall start/restart related timer CTA_validity_time. At expiry of this timer a new acquisition of Common TA estimation assistance information is required.

AS shown in the following graphs, by acquiring necessary assistance information indicated by the network, the UE can calculate the common TA during CTA_validity_time_max which is of the order of tens of seconds:

- In Idle or Inactive state, when first-order approximation of Common TA is used the UE needs to acquire via the SIB new assistance information at most 9 seconds before PRACH Format0 and 3 transmission as shown in Figure 1. And at most 28 seconds when second-order approximation of Common TA is used as shown in Figure 2.

- In Connected state, in case of  15 kHz SCS, the UE needs to acquire new assistance information at most 1.8 seconds before each uplink transmission when first-order approximation of Common TA is used as shown in Figure 3. And at most 9.8 seconds when second-order approximation of Common TA is used as shown in Figure 4.

For better accuracy of self-estimated Common TA a second-order approximation may be needed, thereby, both Common TA drift rate and Common TA drift variation rate may need to be jointly broadcast by the Network.

A validity timer configured for Common TA defines the maximum time during which the UE can track the Common RTD without having to perform new assistance information acquisition to be used for Common TA self -estimation.

Figure 1 First-order approximation of Common TA for PRACH format 0/3 transmission

Figure 2 Second-order approximation of Common TA for PRACH format 0/3 transmission

Figure 3 First-order approximation of Common TA in Connected state – SCS 15 kHz

Figure 4 Second-order approximation of Common TA in Connected state – SCS 15 kHz

Based on simulation results, the maximum time during which the UE can track the Common RTD without having acquired new assistance information is determined for long/short PRACH transmission and PUSCH/PUCCH transmission using 15kHs SCS as recapped in Table 1.

The following assumptions were considered:  LEO @600 km scenario and the satellite flies by the GW (worst case). Also, it is assumed that the gNB knows the satellite trajectory with a maximum error on satellite position of 30m (orbit prediction error will be discussed in section 4.3).  That is, the maximum error on the Common TA estimated by the gNB considered in the simulations is equal to 0.2 µs.  Note that the values provided in Table 1 do  not take into account the error due to the quantization which will be considered in the coming section.

The maximum time during which the UE can track the Common RTD without having acquired new assistance information shall take into account the maximum error on the Common RTD estimated by the gNB (before being broadcast) and the error of  quantization of assistance information.

Table 1 Validity time period of Common TA related assistance information

|  | Maximum time during which the UE can track the Common RTD without having acquired new assistance information: |  |  |
| --- | --- | --- | --- |
|  | Broadcast of Instantaneous Common TA DRIFT W/O Common TA drift variation indication | Broadcast of averaged Common TA Drift W/O Common TA drift variation indication | Broadcast of Instantaneous Common TA Drift With Common TA drift variation indication |
| PRACH Format 0/3 | 9 s | 18 s | 28 s |
| PRACH format A1 with 15 kHz SCS | 2.6 s | 5.3 s | 12.8 s |
| Connected state and 15 kHz SCS | 1.8 s | 3.6 s | 9.8 s |

Based on the above results, it can be seen that apart from indicating the instantaneous Common TA Drift rate it would be beneficial to indicate as well the Common TA drift variation rate. This will avoid to some degree frequent decoding of system information and most importantly will enhance the accuracy of Common TA derived by the UE.

Alternatively, to assist the UE to calculate the Common TA, without indicating Common TA drift variation rate, the network may indicate an averaged Common TA Drift rate (instead of  Instantaneous Common TA Drift rate). Common TA Drift rate can be averaged over a sliding window X as follows:

TACommondrift(t)= CommonTAt+X-CommonTA(t)  X

The averaging period X can be determined based on the maximum tolerable error on common TA estimation.

To assist the UE in self-estimating the Common TA, without indicating Common TA drift variation rate, the network may indicate an averaged Common TA Drift rate (instead of  Instantaneous Common TA Drift rate).

### Granularity and signalling of Common TA

The granularity of Common TA maybe set to be the same as the granularity of NTA i. e. 16∙64/2μ. Thereby, NTA,Common  values shall be indicated in SIB by index values of  TA_common = 0, 1, 2, ..., p. where an amount of the common TA with subcarrier spacing of kHz is given by:

NTA,Common =

p being the maximum range of TA_common. Thus, the maximum  common TA for numerology µ is given by :

NTA,Common, max =p*16*64/2μ, where  NTA,Common, max is calculated for 120kHz subcarrier spacing and  TA_common = p.

Therefore, when the reference point for uplink time synchronization is at the Radio Unit (RU) collocated with GW:

p= NTA,Common, max =maximum RTD on the feeder (ms) 1000.Tc.

The maximum RTD on the feeder link that shall be supported in NTN is 270.73 ms in case GEO based non-terrestrial access network, 12.89 ms (600km), 20.89 ms (1200km) in case of LEO based non-terrestrial access network

With the above considerations in mind, the maximum range of TA-Common as well as the number of bits to be used for TA-Common in SIB are given in the following table:

Table 2 Payload of common TA

| Scenarios | GEO based NTN | LEO based NTN |
| --- | --- | --- |
| maximum range of TA-Common | 4158413 | 197990 (600km), 320870 (1200km) |
| Payload (bits) | 23 bits | 19 bits (600km), 19 bits (1200Km) |

Further, as shown in the graph hereafter, using a 21 bits field in the SIB to indicate the Common TA will significantly reduce the quantization errors. Therefore,  using a granularity of 4∙642μ Tc for Common TA is beneficial.

Figure 5 Common TA with finer granularity, e.g., 4∙64/2^μ∙Tc

Proposal 1: The granularity of Common TA is equal to 4∙64/2μ*Tc

Moreover, it was observed through the simulations that in case of LEO-600, a field of 13 bits will be needed for Common TA drift rate indication and a field of 6 bits for Common TA drift variation rate.

Proposal 2:

The Network may periodically broadcast:

- in case of GEO based non-terrestrial access network:

  - Common TA in a field of 23 bits

- in case of LEO/MEO based non-terrestrial access network:

  - Common TA in a field of 21 bits

  - Common TA drift rate in a field of 13 bits

  - Common TA drift rate variation in a field of 6 bits

## The need and the indication of TA margin

UE can either underestimate or overestimate its TA. In case of overestimation, the PRACH sequence will be received at gNB side in advance w.r.t. the RACH occasion leading to unwanted interference with previous OFDM symbols. To avoid such interference, the TA_margin is used by the UE for pre-adjustment/pre-refinement of the autonomously acquired initial TA as shown in Figure 6:

Figure 6  Pre-adjustment of autonmously acquired initial TA

Further, if initial TA does not include a margin for maximum TA estimation error. A bipolar TA command is needed in msg2. i.e support negative TA adjusting in RAR. It is preferable to avoid such specification impact.

if initial TA does not include a margin for maximum TA estimation error. A bipolar TA command is needed in msg2. i.e support negative TA adjusting in RAR

The value to be considered for TA-margin in the cell depends on maximum allowed TA estimation error. And as such, it is related to the discussions for timing synchronization requirements in RAN4.

Proposal 3: RAN1 to discuss the indication of TA-margin after RAN4 response to the LS on timing synchronization requirements.

## TA update in connected mode

### TA update based on closed loop

For TA update in RRC_CONNECTED state, closed loop based on TA Command  field in msg2/msgB and MAC CE TA command will dynamically update the component  NTA. The TA command is transmitted on a per-need basis with a granularity in the step size of 0.52 μs.

When TAC (TA) in msg2/msgB is received,  UE makes the first adjustment and NTA is updated as follows:

NTA= TA× 16×64, where, TA is the TAC field in msg2/msgB.  If initial TA does not include a margin for maximum TA estimation error, a bipolar TA command is needed in msg2. i.e support negative TA adjusting in RAR.

When TACs (TA) provided within the MAC CE is received, NTA is updated as follows:

NTA_new=NTA_old+TA-31.16.642μ, where, TA is the TAC field received in MAC CE  command.

Proposal 4:

Existing NTA update in RRC_CONNECTED based on TA Command  field in msg2/msgB and MAC CE TA command is used for UL timing alignment correction without specification change

### TA update based on open loop

In our view, in RRC_Connected state the NTN UE shall update  NTA,UE-specific+NTA,common so that the cyclic prefix could absorb the propagation delay variation due to satellite movement.

As discussed in section 2.2.2 the UE could autonomously predict and correct the common delay within 0,15 µs provided that an updated Common TA related assistance information is available at the UE during the last 5s when both Common TA drift rate  and Common drift variation rate are indicated.

Moreover, as will be discussed in section 4.5 the UE could autonomously predict and correct the delay on service link within a timing error range of 0.15 µs provided that an updated ephemeris data is available at the UE during the last 45 s.

As a result, by updating NTA,UE-specific+NTA,common the UE could autonomously track the RTD variation on both service link and feeder link to keep the residual timing error within a maximum tolerable range that could be absorbed by the cyclic prefix when using 120kHz SCS.

Figure 7	TA maintenance based on open loop

### Time Alignment Timers

The legacy timeAlignmentTimer shall be used for closed loop: when the UE receives a MAC TAC, it updates its existing NTAand restarts the timeAlignmentTimer which defines the maximum time the UE can remain uplink synchronized without having received a TAC from the gNB. If this timer expires the UE assumes that it has lost uplink synchronization.

Regarding NTA,common update, the assistance information (Common TA, Common TA drift rate and drift variation rate) acquired in the SIB will be valid only during a validity time period which depends on the maximum tolerable error on common TA estimation and the order of the Common TA approximation to be carried out by the UE. At the reception of new assistance information, the UE shall start/restart related timer. At expiry of this timer a new acquisition of Common TA estimation assistance information is required.

Proposal 5:

A validity timer configured for Common TA defines the maximum time during which the UE can track the Common RTD without having acquired new assistance information to be used for Common TA calculation.

- This timer is restarted each time the UE receives new assistance information.

- The UE assumes that it has lost uplink synchronization if this timer expires and assistance information is not available.

# UL frequency synchronization in NTN

Common Doppler shift pre/post compensation on the feeder links

As working assumption, RAN1 to consider that the GW pre/post compensates common Doppler shift / Doppler shift variation over the feeder link and any transponder frequency error at the satellite in a transparent way to the UE.

Proposal 6: The Doppler shift over the feeder link and any transponder frequency error for both Downlink and Uplink is compensated in a way transparent to UE.

Common Doppler shift pre compensation on the DL service link.

It was observed via simulations [2] that for DL initial synchronization, robust performance can be provided by the SSB design in Rel-15 in case of LEO with beam specific pre-compensation of common frequency shift.

In RAN#104-e, RAN1 made the following conclusion:

If DL frequency compensation for the service link Doppler is applied, indication of the amount of frequency compensation is necessary.

- FFS: support of DL frequency compensation for the service link Doppler.

In case of Earth fixed cell, if DL frequency compensation for the service link Doppler is applied, gNB needs to periodically update and broadcast the DL pre-compensated frequency offset (FO). This pre-compensated  FO is time-varying as show in Figure 8 below:

Figure 8 Pre-compensated Frequency Offset w.r.t to the center of an Earth-fixed beam- LEO@600 km

The pre-compensated  FO will be used by the UE to perform its own estimation of the residual Doppler experienced on the DL. The UE needs to know this residual Doppler to be able to generate  UL frequency carrier for its uplink transmission. Therefore, the UE shall acquire the broadcast pre-compensated  FO and performs a first order approximation to self-calculate the pre-compensated  FO. Hence, the network needs to also indicate the drift rate of common pre-compensated FO. The error on self-calculated pre-compensated FO should be within a specified threshold .e.g 10% 0,1 ppm, that is 0.01 ppm.

Based on simulation results, in case of LEO @600 km, by using the indicated amount of frequency compensation and the associated FO drift rate the UE can self-calculate and predict pre-compensated FO 2.5 seconds ahead if quantization error is not taken into account. UE self-calculated pre-compensated FO is shown in Figure 9.

Figure 9 Self-calculated pre-compensated FO

By considering the quantization error, as shown in Figure 10 the UE will be able to predict the pre-compensated FO 2.3 seconds ahead when the  amount of frequency compensation is indicated in a field of 14 bits and the drift rate of common pre-compensated FO in a field of 5 bits.

Figure 10 Maximum pre-compensated FO error

If DL frequency compensation for the service link Doppler is supported, in case of Earth-fixed cell, the UE needs to frequently acquire the SIB to retrieve common pre-compensated FO parameters.

At least 19 bits are needed to indicate the amount of frequency compensation and associated drift rate.

Proposal 7  Deprioritize support of DL frequency compensation for the service link Doppler in Release 17.

# Satellite orbit determination and prediction

The aim of this section is to provide some insights on how and with which accuracy satellite position and velocity can be determined and predicted.

Satellite orbit determination

The satellite orbit determination (OD) consists in estimating the most likely track or orbit which have been taken by a satellite based on past and noisy measurements of its position and velocity. In general, the satellite orbit determination is performed as follows:

Step 1: Measurements of the satellite position and velocity are made and dated. Most of the time, these measurements are GNSS-based measurements performed on-board. The accuracy of GNSS measurements depends on several factors such as:

- The GNSS receiver characteristics (mono-frequency vs bi-frequency, single vs multi-constellation, quality of the GNSS receiver)

- The satellite orbit, and the number of GNSS satellites in view

Step 2: The measurements are collected in the NTN Control Center (NCC) where the satellite OD will be performed. In general, the satellite GNSS measurements are delivered to the ground via dedicated telemetry channels between the satellite and the NTN GW network. The reporting period of new GNSS measurements is implementation specific but it is directly related to the accuracy target associated to the instantaneous knowledge of the satellite position and velocity in the system.

Step 3: The satellite OD is performed in the NCC. This operation can be more or less complex depending on the models considered, the quantity of measurements available and the algorithms used. Traditionally, two filtering techniques are used on input measurements: Kalman filtering or least square filtering. Least square filtering usually provides more accurate results but requires more computing power.

An overview of a typical satellite system architecture is given on 4 where the locations of each OD step are explicitly represented.

The satellite OD is useful for several reasons. It helps estimating what was the satellite past trajectory so one can identify whether the satellite is diverging from a reference orbit and if needed schedule satellite station keeping manoeuvres to get it back on track.

Figure 9 : System Architecture Overview

Hereafter, we provide several examples of orders of magnitude of precision which can be expected for orbit determination. This performance shall not be confused with orbit prediction performances which concern the future satellite trajectory and are discussed in the next section.

One shall distinguish between orbit determination performance based on past measurements of the satellite trajectory and orbit prediction performance which concern the future satellite trajectory.

Case 1 : “Best” Precision Orbit Determination

For satellite systems designed for navigation purposes delivering very accurate and abundant GNSS measurements, it is possible to achieve the following performance:

- 3D Position RMS Error = 0.05 m

- 3D Velocity RMS Error = 0.05 mm/s

However, in commercial telecommunication satellites such performance cannot be met because the cost would be prohibitive and such OD precision is not required for the system to work.

Case 2 : “Typical” Precision Orbit Determination

For telecommunication satellites, it is more typical to consider the following values:

- 3D Position RMS Error = 0.5 m

- 3D Velocity RMS Error = 0.5 mm/s

Case 3 : “Low Quality” Precision Orbit Determination

Finally, for low cost system the following performance can be considered:

- 3D Position RMS Error = 5 m

- 3D Velocity RMS Error =5 mm/s

Note: As a rule of thumb; it can be assumed that there is a factor of 1000 between the position error (in m) and the velocity error (in m/s). This is important to keep in mind when defining accuracy targets for satellite position and velocity.

As a rule of thumb; it can be assumed that there is a factor of 1000 between the position error (in m) and the velocity error (in m/s). This is important to keep in mind when allocating an error budget for satellite position and velocity estimations.

Satellite orbit prediction

The satellite orbit determination can also be used to derive updated satellite ephemeris. The satellite ephemeris are useful to predict future satellite positions and velocity vectors. This can be useful for various reasons such as:

- Determine when the satellite is going to fly by over a specific zone of interest

- Anticipate collision avoidance operations

- Anticipate future feeder link switch over operations within the NTN gateways network.

- Perform Doppler compensation (for instance on the feeder links between the satellite and the serving NTN GW)

Of course, the accuracy of the orbit prediction required to perform such operations can be significant different from an application to another. Finally, the NCC can share the satellite ephemeris (under various formats) with other entities in the system responsible for performing such operations (e.g. with the NTN GW).

Then, one can wonder with which accuracy one can predict the future satellite position and velocity vectors and over what time horizon.

The orbit prediction accuracy may depend on several factors:

- The accuracy of the orbit determination used to derive the satellite ephemeris: this will drive the initial errors of the propagation model.

- The accuracy of the orbit propagation model: there are many propagation models which takes into account more or less precisely the physical effects on the satellite movement (J2/4 perturbations, aero-dynamical drag, anticipated station keeping manoeuvres)

- The time horizon over which the prediction is made: The initial errors combined with the propagation model errors tend to increase the prediction errors when the prediction period increases.

The orbit prediction accuracy depends on:

  - -	The accuracy of the orbit determination used to derive the satellite ephemeris

  - -	The accuracy of the orbit propagation model

  - -	The time horizon over which the prediction is made

## Orbit prediction error at NTN Control Center

In this study, we tried to quantify the propagation error evolution depending on the initial PV errors. The main assumptions and results of this study are summarized in Table 3.

A simplified relative position-velocity model for circular orbits (Clohessy-Wiltshire) has been considered. The CW equations are given in Figure 10 where the z-axis is along the radius vector of the target spacecraft, the y-axis is along the angular momentum vector of the target spacecraft, and the x-axis completes the right handed system.	These equations are usually used for rendez-vous or docking relative position evaluations.

However, they are used here to evaluate the propagation error evolution between the reference orbit and the predicted orbit which progressively moves away from the reference orbit due to propagation errors.

Figure 10: Clohessy-Wiltshire equations of motion

This study characterizes only the impact of the initial PV error on the prediction errors. The additional errors related to the orbit propagation model limits are not simulated here. However, when considering short prediction period (i.e. inferior to the orbital period), the propagation errors induced by the propagator models limitations (J2/J4 perturbation, air drag, …) may be considered negligible w.r.t. to the prediction errors induced by the initial PV errors.

Effect of initial errors on tangential (X), normal (Y), radial (Z) axes in position and velocity gives very different results. The worst case effects are observed when the initial error is applied on Z axis for position and X axis for velocity.

For illustrative purpose, the propagation errors over 2 revolution orbits for each worst case initial errors on position (dz0 = 1 m) and on velocity (dvx0 = 1mm/s) have been illustrated on Figure 11 and Figure 12 respectively. It can be observed that the velocity error is not an increasing function of time.

Figure 11 Propagation errors – dZ0 = 1 m

Figure 12 Propagation errors – dvX0 = 1 mm/s

For initial PV errors, the POD assumptions presented in the previous section have been considered.

Finally, one shall define a PV accuracy target to be guaranteed. When the propagation errors exceed the PV accuracy target then it means that a new orbit determination shall be performed and updated ephemeris shall be provided. The PV accuracy target is going to be overshoot after a more or less long prediction period depending on the initial PV error assumption.

In this study, the target PV accuracy has been driven by the accuracy expected to perform Doppler pre/post-compensation on feeder links. The Doppler compensation residual errors experienced on the feeder link shall be almost transparent to the gNB/UE. Then, it shall be an order of magnitude lower than the frequency error budget allocated at UE side (~0.1 ppm). Moreover, the carrier frequencies used on the feeder links may be significantly higher than the one used on the service link so additional margin shall be considered to counter shift this effect. As a consequence, it is proposed to assume that the satellite radial velocity w.r.t the NTN GW should be known with an accuracy better than +/- 0.3 m/s when performing Doppler compensation on the feeder link. It has been demonstrated by simulation (see Figure 13 and Table 4) that when the following assumptions on the PV errors are met the previous requirement is guaranteed :

- Gaussian position error distribution with 3D Position RMS Error = 10 m => 3sigma = 30 m

- Gaussian velocity error distribution with 3D Velocity RMS Error = 10 mm/s => 3sigma = 30 mm/s

Therefore, it is proposed to consider the following PV accuracy target:

- Position error < 30 m

- Velocity error < 30 mm/s

The simulation results are summarized in Table 4. It can be observed that the maximal propagation time before overshooting the PV accuracy target is significantly driven by the initial error. More precisely, depending on the quality of the orbit determination, the maximal propagation time can go from 8 hours to less than 10 minutes. The less accurate is the orbit determination, the more often orbit determination shall be performed based on more recent measurements and updated satellite ephemeris shall be distributed.

It can be also observed that the velocity error degradation is negligible w.r.t the position error. As a consequence, the radial velocity (or Doppler) error is mainly driven by the position error degradation.

In conclusion, it has been demonstrated that even for a satellite system with “low quality” orbit determination Algorithm, challenging operations relying on accurate prediction of satellite trajectories such as Doppler compensation can be performed reliably. The limiting condition is that the system should be designed such that new orbit determination and satellite ephemeris update can be performed frequently enough.

Even for a satellite system with “low quality” orbit determination algorithm, challenging operations relying on accurate prediction of satellite trajectories such as Doppler compensation can be performed reliably.

Figure 13 Monte Carlo simulation to evaluate radial velocity error [m/s] distribution assuming a PV Gaussian errors with (3D Position RMS Error = 10 m / 3D Velocity RMS Error = 10 mm/s)

Table 3 : Monte Carlo simulation parameters

| Satellite orbit | Polar orbit (i = 90°) |
| --- | --- |
| GW position | Equatorial ground station Min elevation = 30° |
| Error on position/velocity | Gaussian distribution on norm - 3D Position RMS Error = 10 m - 3D Velocity RMS Error = 10 mm/s Uniform distribution on direction |

Table 4 : Results of propagation error study

| Model | Kepler Model Clohessy-Wiltshire equations |
| --- | --- |
| Orbit | Circular @ 600 km Period = 5801 s |
| Initial PV Errors | Case 1 : “Best” Precision Orbit Determination - 3D Position RMS Error = 0.05 m - 3D Velocity RMS Error = 0.05 mm/s Case 2 : “Typical” Precision Orbit Determination - 3D Position RMS Error = 0.5 m - 3D Velocity RMS Error = 0.5 mm/s Case 3 : “Low Quality” Precision Orbit Determination - 3D Position RMS Error = 5 m - 3D Velocity RMS Error =5 mm/s |
| PV accuracy target | Position error < 30 m Velocity error < 30 mm/s |
| Prediction period before overshooting PV accuracy target | Case 1 : “Best” Precision Orbit Determination Propagation to 5 orbital periods before overshooting PV accuracy target ~ 8 h - Statistics on max position error - Mean = 4.35 m - Std dev = 4.10 m - Max = 35.9 m - Statistics on max velocity error - Mean = 0.3 mm/s - Std dev = 0.3 mm/s - Max = 2.6 mm/s Case 2 : “Typical” Precision Orbit Determination Propagation to 0.5 period before overshooting PV accuracy target ~ 48 mn - Statistics on max position error - Mean = 4.63 m - Std dev = 4.23 m - Max = 36.7 m - Statistics on max velocity error - Mean = 0.3 mm/s - Std dev = 0.3 mm/s - Max = 2.6 mm/s Case 3 : “Low Quality” Precision Orbit Determination Propagation to 0.1 period before overshooting PV accuracy target ~ 10 mn - Statistics on max position error - Mean = 5.65 m - Std dev = 3.61 m - Max = 28.9 m - Statistics on max velocity error - Mean = 7.5 mm/s - Std dev = 5.1 mm/s - Max = 46 mm/s |

## Satellite orbit format

Serving satellite ephemeris broadcast to UEs is required to enable autonomous adjustments of UL TA and frequency at UE side based on the satellite predicted position and velocity vectors. The satellite ephemeris format to be used was discussed in RAN1#104-bis-e and the following agreement was made:

Support serving-satellite ephemeris broadcast based on one or more of the following:

- Set 1: Satellite position and velocity state vectors:

  - position X,Y,Z in ECEF (m)

  - velocity VX,VY,VZ in ECEF (m/s)

- Set 2: At least the following parameters in orbital parameter ephemeris format:

  - Semi-major axis α [m]

  - Eccentricity e

  - Argument of periapsis ω [rad]

  - Longitude of ascending node Ω [rad]

  - Inclination i [rad]

  - Mean anomaly M [rad] at epoch time to

    - FFS: Whether pre-provisioned ephemeris based on orbital elements can be used as reference. Thereby, only delta corrections can be broadcast in order to reduce the overhead

- FFS: The field size for each parameter

- FFS: The impact on signaling due to the required accuracy of serving-satellite ephemeris

- FFS: Whether down-selection is needed or both sets are supported

The required accuracy for the satellite ephemeris format shall be driven based on the prediction budget error on satellite position and velocity vectors and the foreseen  prediction time horizon. It can be further discussed once the timing and frequency synchronization requirements are agreed.

The signalling overhead introduced by satellite ephemeris broadcast shall be carefully quantified.

There are two ways to provide a time reference or Epoch time to the satellite ephemeris broadcast. The first approach is to transmit this time explicitly. It is the simplest approach but it increases signalling overhead. It can also be done in an implicit way to limit the signalling overhead. For instance, the date can be considered equal to the reception time of the frame including the satellite ephemeris information.

Proposal 8: Epoch time is implicitly known as a reference time linked to DL slot where satellite ephemeris is broadcast

## Orbit prediction error at UE

Depending on adopted ephemeris format; set 1 or set 2, different propagator models can be considered for implementation at UE side. Some have been developed for orbital elements and others for PV state vectors. To enhance the propagator model additional elements may be included in the ephemeris data.

RAN1made the following conclusion in RAN1#104-bis-e:

Conclusion

The orbital propagator model to be used at UE side can be left to implementation.

The adopted ephemeris format can strongly orientate the choice of model (e.g. TLE format and TLE propagation models).

It was observed based on simulation that the prediction time period before overshooting the delay and Doppler pre-compensation target is linked to the broadcast satellite format and to the propagation model used in the UE. As shown in the table below  prediction 45 s ahead for UE pre-compensation is accurate within 0.29 µs for round trip delay error and within 45.93 Hz for Doppler error (at FR2) when Kepler model is used. This is sufficient to accommodate UE pre-compensation requirement when using 120 kHz SCS. Prediction 180 s ahead for UE pre-compensation is accurate within 1.74 µs for round trip delay error and within 17.60 Hz (in S band) for Doppler error when Kepler model is in use. This is sufficient to accommodate UE pre-compensation requirement when using 15 kHz SCS.

Using other propagation models in the UE will significantly enhance UE pre-compensation accuracy during a larger prediction time period. For example, by using analytical Eckstein-Hechler model, prediction 5 minutes ahead for UE pre-compensation is accurate within 0.29 µs for round trip delay error and within 6.20 Hz (in FR2) for Doppler error.

It is worth noting that adequate fitting can be performed based on the reference orbit and the prediction time horizon considered to further enhance UE pre-compensation accuracy. Of course, this is possible only if the propagator model to be used at UE side is known by the network. But the orbital propagator model to be used at UE side can be left to implementation as per the conclusion made in RAN1#104-bis-e.

Table 6 Orbit propagation model comparison

|  |  |  | Maximum errors |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Without fitting | Model | Duration of prediction (minute) | DP (m) | UE specific TA Error (µs) | DV (m/s) | Doppler error (Hz) |
|  | Kepler | 3 | 261 | 1.74 | 2.64 | 17.60 |
|  | Eckstein Hechler (J6) | 21 | 303 | 2.02 | 0.48 | 3.20 |
|  | 2x2 num | 22,5 | 301.4 | 2.01 | 0.42 | 2.80 |
|  | 6x6 num | 27 | 302.5 | 2.02 | 0.44 | 2.93 |
|  |  |  | Maximum errors |  |  |  |
|  | Model | Duration of prediction (second) | DP (m) | UE specific TA Error (µs) | DV (m/s) | Doppler error (Hz) |
| Without fitting | Kepler | 45 | 43.4 | 0.29 | 0.689 | 45.93 |
|  | Eckstein Hechler (J6) | 300 | 43.4 | 0.29 | 0.093 | 6.20 |
|  | 2x2 num | 300 | 44.3 | 0.30 | 0.1 | 6.67 |
|  | 6x6 num | 350 | 44.2 | 0.29 | 0.084 | 5.60 |
| With fitting | TLE | 370 | 44,1 | 0.29 | 0.1 | 6.67 |
|  | Eckstein Hechler (J6) | 390 | 44,2 | 0.29 | 0.083 | 5.53 |
|  | 2x2 num | 375 | 44 | 0.29 | 0.092 | 6.13 |
|  | 6x6 num | 395 | 44,2 | 0.29 | 0.079 | 5.27 |

Based on simulation results, prediction 45 s ahead for UE pre-compensation is accurate within 0.29 µs for round trip delay error and within 45.93 Hz for Doppler error (at FR2) when Kepler model is used. This is sufficient to accommodate UE pre-compensation requirement when using 120 kHz SCS

# Conclusion

In this contribution, the following proposal have been made:

Proposal 1: The granularity of Common TA is equal to 4∙64/2^μ*Tc

Proposal 2: The Network may periodically broadcast:

- in case of GEO based non-terrestrial access network:

  - Common TA in a field of 23 bits

- in case of LEO/MEO based non-terrestrial access network:

  - Common TA in a field of 21 bits

  - Common TA drift rate in a field of 13 bits

  - Common TA drift rate variation in a field of 6 bits

Proposal 3: RAN1 to discuss the indication of TA-margin after RAN4 response to the LS on timing synchronization requirements

Proposal 4: Existing N_TA update in RRC_CONNECTED based on TA Command  field in msg2/msgB and MAC CE TA command is used for UL timing alignment correction without specification change

Proposal 5: A validity timer configured for Common TA defines the maximum time during which the UE can track the Common RTD without having acquired new assistance information to be used for Common TA calculation.

- This timer is restarted each time the UE receives new assistance information.

- The UE assumes that it has lost uplink synchronization if this timer expires and assistance information is not available.

Proposal 6:The Doppler shift over the feeder link and any transponder frequency error for both Downlink and Uplink is compensated in a way transparent to UE.

Proposal 7:  Deprioritize support of DL frequency compensation for the service link Doppler in Release 17.

Proposal 8: Epoch time is implicitly known as a reference time linked to DL slot where satellite ephemeris is broadcast

In this contribution, the following observations have been made:

Observation 1.	The UE needs to derive the actual Common TA values based on broadcast Common TA and  other assistance information indicated by the Network e.g. Common TA drift rate and Common TA drift variation rate.

Observation 2.	For better accuracy of self-estimated Common TA a second-order approximation may be needed, thereby, both Common TA drift rate and Common TA drift variation rate may need to be jointly broadcast by the Network.

Observation 3.	A validity timer configured for Common TA defines the maximum time during which the UE can track the Common RTD without having to perform new assistance information acquisition to be used for Common TA self -estimation.

Observation 4.	The maximum time during which the UE can track the Common RTD without having acquired new assistance information shall take into account the maximum error on the Common RTD estimated by the gNB (before being broadcast) and the error of  quantization of assistance information.

Observation 5.	To assist the UE in self-estimating the Common TA, without indicating Common TA drift variation rate, the network may indicate an averaged Common TA Drift rate (instead of  Instantaneous Common TA Drift rate).

Observation 6.	if initial TA does not include a margin for maximum TA estimation error. A bipolar TA command is needed in msg2. i.e support negative TA adjusting in RAR

Observation 7.	If DL frequency compensation for the service link Doppler is supported, in case of Earth-fixed cell, the UE needs to frequently acquire the SIB to retrieve common pre-compensated FO parameters

Observation 8.	At least 19 bits are needed to indicate the amount of frequency compensation and associated drift rate.

Observation 9.	One shall distinguish between orbit determination performance based on past measurements of the satellite trajectory and orbit prediction performance which concern the future satellite trajectory.

Observation 10. As a rule of thumb; it can be assumed that there is a factor of 1000 between the position error (in m) and the velocity error (in m/s). This is important to keep in mind when allocating an error budget for satellite position and velocity estimations.

Observation 11. The orbit prediction accuracy depends on:

a.	-	The accuracy of the orbit determination used to derive the satellite ephemeris

b.	-	The accuracy of the orbit propagation model

c.	-	The time horizon over which the prediction is made

Observation 12. Even for a satellite system with “low quality” orbit determination algorithm, challenging operations relying on accurate prediction of satellite trajectories such as Doppler compensation can be performed reliably.

Observation 13. Based on simulation results, prediction 45 s ahead for UE pre-compensation is accurate within 0.29 µs for round trip delay error and within 45.93 Hz for Doppler error (at FR2) when Kepler model is used. This is sufficient to accommodate UE pre-compensation requirement when using 120 kHz SCS.

# Bibliography

| [1] | Thales, FL Summary on enhancements on UL time and frequency synchronization for NR NTN, R1-2104076, April 2021 |
| --- | --- |
| [2] | TR38.821 Solutions for NR to support non-terrestrial networks. |
| [3] | TR38.213 Physical layer procedures for control. |
| [4] | Thales, "RP-201256 Solutions for NR to support non-terresttrial networks (NTN)," RAN#88e, June 2020. |