3GPP TSG RAN WG1 #105-e                              	                     R1- 2104432

e-Meeting,  May 10th – 27th, 2021

Agenda Item:	8.7.1.1

Source:	Spreadtrum Communications

Title:	Discussion on potential paging enhancements for UE power saving

Document for:	Discussion and decision

# Introduction

There are two types of PEI proposed, i.e. DCI-based and sequence-based (SSS or TRS/CSI-RS) PEI.

In RAN1#104e [1], some assumptions and metrics for further evaluation to compare the DCI-based or sequence-based Paging Early Indication (PEI) were agreed.

This contribution mainly focuses on the further evaluation to compare DCI-based and sequence-based PEI.

# Sequence-based or DCI-based PEI

It was agreed in RAN1#104e [1] that there are two different UE behaviors corresponding to DCI-based and sequence-based PEI.

Agreements: For the evaluation and comparison of PEI candidate designs based on PDCCH, TRS/CSI-RS and SSS, the following are assumed: - Behv-A: - PEI indicates UE should monitor a PO if UE’s group/subgroup is paged - UE is not required to monitor a PO if UE does not detect PEI at all PEI occasion(s) for the PO - Behv-B: - PEI indicates whether or not UE should monitor a PO - UE is required to monitor a PO if UE does not detect PEI at all PEI occasion(s) for the PO

Generally, Behv-A corresponds to the PEI supporting DTX, and Behv-B corresponds to the PEI not supporting DTX. The design principles for sequence-based and DCI-based PEI are different in either Behv-A or Behv-B.

- Behv-A:

  - Sequence-based PEI: The presence of the target sequence means UE should monitor a PO.

  - DCI-based PEI: The presence of the target DCI (target monitoring occasion, DCI format and RNTI) means UE should monitor a PO.

- Behv-B:

  - Sequence-based PEI: The bit 1 in the target sequences indicates UE should monitor a PO, and the bit 0 in the target sequences indicates UE should not monitor a PO.

  - DCI-based PEI: The bit 1 in the target DCI indicates UE should monitor a PO, and the bit 0 in the target DCI indicates UE should not monitor a PO.

## Power saving gain

PEI can reduce the wakeup energy overhead. As summarized by FL in RAN1#102e,

- “Without early indication, in order to decode PDSCH successfully, UE needs to receive multiple SS burst for AGC and time/frequency tracking before PO. And due to the distributive SS bursts, UE needs to wake up multiple times before PO. The high wake-up energy overhead results in UE power consumption waste if the UE is not paged. This method can effectively save UE power by allowing UE to save unnecessary SSB processing if there is no need to receive paging data”.

It is common understanding that PEI can reduce the unnecessary SSB processing or equivalently shorten the time duration a priory to PO.

### Evaluation

Without UE subgrouping

Our evaluation of power saving gain for DCI-based PEI can be found in Appendix 6.1. According to the evaluation, in general cases, the power saving gain of PEI is about 20%. In our evaluation, the UE processing timeline for DCI-based PEI and sequence-based PEI are the same, and power model for DCI-based PEI and sequence-based PEI are the same, and thereby the power saving gain for them are similar.

Observation 1: Without UE subgrouping, the power saving gain of DCI-based PEI and sequence-based PEI are similar.

With UE subgrouping

In the evaluation of Appendix 6.1, we assume group paging rate is 10%. When group paging rate is low enough, the additional power saving gain for UE subgrouping may be too marginal. From RAN2 LS [4], it can be found that RAN2 adopted network controlled subgrouping as the assumption of UE subgrouping, instead of randomization based subgrouping (UE-ID based).

- We adopt network controlled subgrouping (based on individual UE characteristics, not specified or limited to paging probability as EUTRA, possibly with additional randomization). - If the network chooses to not provide specific subgrouping information, there will be configuration option where subgrouping can be supported by randomization (by UE-ID). - RAN2 also briefly discussed the number of UE subgroups per PO, and, considering it depends on which and how many UE characteristics above network controlled subgrouping will support, RAN2 could only provide an indication that the maximum number of UE subgroups per PO should be at least 8. However, RAN2 understands that it should also depend on observed power saving gain and the signalling method for indicating the subgroups (PEI and/or paging DCI) and so, potentially on the PEI design itself, therefore RAN2 leaves it to RAN1 to decide the final value.

Under assumption of network controlled subgrouping, group paging rate may not be simply assumed as 10%. For example, when a UE group (in a PO) has the large number of UEs, e.g. 80 (corresponding to about 55% group paging rate for 1% per UE paging rate), network can divide a group of UEs into 8 subgroups and each subgroup has 10 UEs (corresponding to about 10% group paging rate for 1% per UE paging rate). When group gaping rate is reduced from 55% down to 10%, obviously there is power saving gain.

With UE subgrouping, we do not see the different power saving gain between DCI-based PEI and sequence-based PEI.

Observation 2: With UE subgrouping, the power saving gain of DCI-based PEI and sequence-based PEI are similar.

### Related to RRM measurement relaxation

If RRM measurem ent for serving cell is relaxed, the power saving gain of sequence-based PEI is higher than that of DCI-based PEI. It could be the reason why NB-IoT/eMTC adopted sequence-based PEI. For instance, WUS configuration is a condition for RRM measurement relaxation for serving cell.

The UE shall meet the requirement defined for the DRX cycle length of N*DRX_cycle in Section 4.6.2.1, provided the following conditions are met: - WUS has been configured in the serving NB-IoT cell using WUS-Config-NB-r15, and

But in NR, RRM measurement for serving cell cannot be relaxed, so the power saving gain of sequence-based PEI may not be higher than that of DCI-based PEI.

Observation 3: In NR, RRM measurement for serving cell cannot be relaxed, so the power saving gain of sequence-based PEI may not be higher than that of DCI-based PEI.

## Resource overhead

### Evaluation

Our updated evaluation can be found in Appendix 6.2. From the evaluation results, we find that the resource overhead of sequence-based PEI and DCI-based PEI are comparable for Behv-A, and the resource overhead of DCI-based PEI is much smaller than that of sequence-based PEI for Behv-B.

More generally, according to Table 2 in Appendix, the resource overhead per group/subgroup can be formulated in the following table.

Table 1: The resource overhead per group/subgroup

|  |  | Without UE subgrouping | With UE subgrouping |
| --- | --- | --- | --- |
| Behv-A | Sequence-based PEI | X_A*(paging rate of group) | X_A*(paging rate of subgroup) |
|  | DCI-based PEI | Y_A*(paging rate of group) | Y_A*(paging rate of group)/(number of subgroups per PEI) |
| Behv-B | Sequence-based PEI | X_B | X_B |
|  | DCI-based PEI | Y_B/(number of groups per PEI) | Y_B/(number of subgroups per PEI) |

Note: X_A is the required number of REs per PEI for Behv-A for sequence-based PEI; Y_A is the required number of REs per PEI for Behv-A for DCI-based PEI; X_B is the required number of REs per PEI for Behv-B for sequence-based PEI; Y_B is the required number of REs per PEI for Behv-B for DCI-based PEI.

It can be observed that for Behv-A, DCI-based PEI has comparable resource overhead compared to sequence-based PEI. Further, UE subgrouping is more suitable for DCI-based PEI, because DCI can accommodate more bits for wakeup indication of UE subgroup(s).

It can be also observed that for Behv-B, DCI-based PEI has lower resource overhead than sequence-based PEI.

Observation 4: For Behv-A, DCI-based PEI has comparable resource overhead compared to sequence-based PEI. For Behv-B, DCI-based PEI has lower resource overhead than sequence-based PEI.

### Dynamic resource sharing b/w PEI and legacy signals/channels

In RAN1#104bis-e, dynamic resource sharing between PEI and legacy signals/channels was widely discussed, and the conclusion is that dynamic resource sharing between PEI and legacy signals/channels is supported by all PEI candidate schemes. We still think the dynamic resource sharing is too optimistic regarding gNB scheduling.

For collision handling between PEI and PDCCH of legacy UE, gNB should hold the PDCCH transmission to avoid the collision, and the legacy UE needs to detect the legacy PDCCH “absent” under risk of increase of false alarm rate. In the discussion of collision handing between SSS-based PEI and PDCCH of legacy UE in RAN1#104bis-e, it was argued that SSS-based PEI can be mapped to CORESET#0 like a PDCCH candidate. However, CCE-to-REG bundle mapping in CORESET#0 is in interleaving manner, so SSS-based PEI is scattered in different PDCCH candidates, and then the risk of false alarm rate is higher.

For resource sharing between PEI and PDSCH of legacy UE, gNB should inform the legacy UE to perform PDSCH rate matching around PEI and gNB should schedule the PDSCH with DCI format 1_1. It is still restrictive.

In our view, semi-static resource sharing between PEI and legacy signals/channels is better, and Behv-B is suitable.

Observation 5: Semi-static resource sharing between PEI and legacy signals/channels is better, and Behv-B is suitable.

### Dynamic resource sharing b/w PEI and additional TRS/CSI-RS

It is common understanding that the additional TRS/CSI-RS for power saving is dynamically valid to save the resource overhead.

For Behv-A, to avoid the reserved resource for PEI, PEI and the additional TRS/CSI-RS should share the resource dynamically. The availability of the additional TRS/CSI-RS is based on the presence of connected UEs, but the availability of PEI is based on presence of paging message in PDSCH, so the dynamic resource sharing b/w PEI and the additional TRS/CSI-RS is problematic.

For Behv-B, the semi-static resource sharing b/w PEI and the additional TRS/CSI-RS is good enough.

Observation 6: For Behv-B, the semi-static resource sharing b/w PEI and the additional TRS/CSI-RS is good enough.

### Collision with SS burst

When POs are dense, e.g. 4 POs in a PF, and close to an SS burst, e.g. in the same PF, how to place PEI together with the SS burst in the same PF should be discussed, especially for the extreme case where 4 POs all have paging message (i.e. no DTX of PEIs).

For Behv-A, 1-to-1 PEI-to-PO mapping is always assumed, and resource of 4 PEIs is easy to collide with the SS burst within the CORESET0 bandwidth.

For Behv-B and DCI-based PEI (here PEI means a PEI PDCCH instead of a bit in PEI PDCCH), 1-to-N PEI-to-PO mapping can be configured, and resource of 1 PEI can be semi-statically configured to avoid collision with the SS burst within the CORESET0 bandwidth.

Observation 7: For Behv-B and DCI-based PE, PEI can be semi-statically configured to avoid collision with SS burst.

As conclusion, Behv-B is more robust for resource sharing or collision handling between PEI and other signals/channels.

Observation 8: Behv-B is more robust for resource sharing or collision handling between PEI and other signals/channels.

## Complexity of receiver

In the previous discussion, it is evaluation assumption that UE should detect PEI under residual frequency error ±0.5ppm. In companies’ evaluation, coherent receiver is used in DCI-based PEI, i.e. channel estimation and equalization are applied in the receiver, but non-coherent receiver is used in sequence-based PEI, i.e. no channel estimation or equalization.

For DCI-based PEI, the coherent receiver is realized in frequency domain. The coherent receiver will perform channel estimation, equalization, QPSK demodulation and decoding. The receiver is robust for the residual frequency error (possibly plus the residual timing error) with protection of equalization, QPSK demodulation and decoding.

For SSS-based PEI, the non-coherent receiver is usually realized in time domain. The non-coherent receiver can try multiple hypotheses for the residual frequency error (possibly plus the residual timing error) to improve performance. In our view, the overall complexity of the non-coherent receiver for SSS-based PEI is similar to that of the coherent receiver for DCI-based PEI.

For TRS/CSI-RS based PEI, the non-coherent receiver may be realized in frequency domain. The residual frequency error (possibly plus the residual timing error) is hard to be overcome in frequency domain if equalization is not applied. Hence, the non-coherent receiver for TRS/CSI-RS based PEI has higher complexity.

Observation 9: The non-coherent receiver for TRS/CSI-RS based PEI has higher complexity.

## Summary

From above discussion, we prefer DCI-based PEI.

Proposal 1: Behv-B is prioritized.

Proposal 2: If two PEI candidate schemes should be down selected, DCI-based PEI and SSS-based PEI are selected.

Proposal 3: DCI-based PEI is supported in R17.

# DCI format for DCI-based PEI

During the discussion in the previous meetings, the candidate DCI format for DCI-based PEI could be

- Candidate-1: Extending DCI format 1_0;

- Candidate-2: Extending DCI format 2_6;

- Candidate-3: New DCI format.

The DCI format for DCI-based PEI is related to assumption of Behv-A and Behv-B.

## Behv-A

For Behv-A, gNB can perform DTX for PEI transmission, and a given UE should only detect the PEI a prior to its own PO, and all DCI bits in a PEI-PDCCH are dedicated to the UE. Therefore, DCI format 2_6 is not suitable. DCI format 1_0 is usually used for scheduling and has DCI size larger than 12 bits. Hence, DCI format 1_0 is also not suitable.

Proposal 3: If DCI-based PEI is supported in R17 with Behv-A, new DCI format is a starting point.

## Behv-B

For Behv-B, gNB cannot perform DTX for PEI transmission, and to save the resource overhead gNB can transmit a PEI corresponding to multiple POs, and a given UE can only get a part of DCI bits like DCI format 2-6 in R16. Therefore, DCI format 2_6 is more suitable.

But DCI format 2_6 is designed for connected mode UE, in which the bit segmentation for a UE can be dedicated RRC configured. Dedicated RRC cannot configure the bit segmentation for an idle mode UE, so there could be some standardization efforts to extend DCI format 2-6 from connected mode to idle mode.

Proposal 4: If DCI-based PEI is supported in R17 with Behv-B, extending DCI format 2_6 is a starting point.

# Conclusion

We have the following proposals.

Proposal 1: Behv-B is prioritized.

Proposal 2: If two PEI candidate schemes should be down selected, DCI-based PEI and SSS-based PEI are selected.

Proposal 3: DCI-based PEI is supported in R17.

Proposal 4: If DCI-based PEI is supported in R17 with Behv-A, new DCI format is a starting point.

Proposal 5: If DCI-based PEI is supported in R17 with Behv-B, extending DCI format 2_6 is a starting point.

# Reference

- 3GPP RAN1#104e Chairman’s Notes, Jan. 25th – Feb. 5th, 2021.

- R1-2009753, “Summary for potential paging enhancements”, MediaTek, Oct. 26th – Nov. 13th, 2020.

- 3GPP RAN1#104bis-e Chairman’s Notes, April 12th – 20th, 2021.

- R2-2104356 (R1-2104229), “Reply LS on UE Sub-grouping for Paging Enhancement”, MediaTek, Apr. 12 - 20, 2021.

# Appendix

## Evaluation for power saving gain

### Initial proposal for UE processing timeline

Initial Proposal 3: When SINR is not high or for reduced capability UEs, the following reference UE processing timeline for a paging cycle and the corresponding evaluation are utilized: Operation in sequence; Purpose; Time duration (ms); Energy contribution (power * time + energy overhead). SSB proc.; AGC, coarse synchronization, serving-cell/intra-freq. RRM measurement; 2; 60 * 2. Light sleep; Power saving; 18; 20 * 18 + 100. SSB proc.; Coarse/fine synchronization, (additional serving-cell/intra-freq. RRM measurement); 2; 50 * 2. Light sleep; Power saving; 18; 20 * 18 + 100. SSB proc.; Fine synchronization, (additional serv.-cell/intra-freq. RRM measurement); 2; 50 * 2. Light sleep; Power saving; 8; 20 * 8 + 100. PDCCH or PDCCH+PDSCH; Paging control proc. and data proc. (if paged); 8 slots for diversity reception; 4; Not paged: 50 * 4 Paged: 120 * 4 (subject to group paging rate, P). Light sleep; Power saving; 6; 20 * 6 + 100. Inter-freq. RRM measurement; Inter-frequency/inter-RAT neighbor cell measurement; 5; 60 * 5. Deep sleep; Power saving; 1215; 1 * 1215 + 450. Total; 1280; 200 * (1-P) + 480 * P + 3685. Average Power Consumption = Total energy / page cycle (1280). Initial Proposal 4: When SINR is high, the following reference UE processing timeline for a paging cycle and the corresponding evaluation are utilized: Operation in sequence; Purpose; Time duration (ms); Energy contribution (power * time + energy overhead). SSB proc.; AGC, Coarse/fine synchronization, serving-cell/intra-freq. RRM measurement. Note: Due to no a prior of SINR change, assume the same 1st SSB timing w.r.t. PO as the case where SINR is not high; 2; 60 * 2. Light sleep; Power saving; 48; 20 * 48 + 100. PDCCH or PDCCH+PDSCH; Paging control proc. and data proc. (if paged); 2 slots for more efficient reception; 1; Not paged: 50 * 1 Paged: 120 * 1 (subject to group paging rate, P). Deep sleep; Power saving; 1229; 1 * 1229 + 450. Total; 1280; 50 * (1-P) + 120 * P + 2859. Average Power Consumption = Total energy / page cycle (1280).

### Power model

In RAN1#102e, the power model in idle/inactive mode was updated as follows.

Agreements: The following power consumption model for FR1 is utilized for the evaluations of Rel-17 UE power saving enhancements in idle/inactive mode. - FFS: FR2 power consumption model for idle/inactive mode operations Power State; Relative Power (FR1 reference from TR 84.840); Relative Power (Idle/inactive-mode operation with reception bandwidth 20 MHz). Deep Sleep (PDS); 1; 1. Light Sleep (PLS); 20; 20. Micro sleep (PMS); 45; 45. PDCCH-only (PPDCCH); 100; 50Note. PDCCH + PDSCH (PPDCCH+PDSCH); 300; 120. PDSCH-only (PPDSCH); 280; 112. SSB/CSI-RS proc. (PSSB); 100 (synchronization or serving cell measurement); 50. Intra-frequency RRM measurement (Pintra); 150 (synchronous case, N=8, measurement only; Pintra, meas-only) 200 (combined search and measurement; Pintra, search+meas); [60] (synchronous case, N=8, measurement only; Pintra, meas-only) [80] (combined search and measurement; Pintra, search+meas). Inter-frequency RRM measurement (Pinter); 150 (measurement only per freq. layer; Pinter, meas-only) 150 (neighbor cell search power per freq. layer; Pinter, search-only) Micro sleep power assumed for switch in/out a freq. layer; [60] (measurement only per freq. layer; Pinter, meas-only) [150] (neighbor cell search power per freq. layer; Pinter, search-only) Micro sleep power assumed for switch in/out a freq. layer. Note: Power scaling to 20MHz reception bandwidth follows the rule in Section 8.1.3 of TR 38.840, i.e., max{reference power * 0.4, 50}..

### Baseline assumptions

The baseline assumptions are also agreed in RAN1#102e, e.g. SS burst configuration, paging configuration.

Agreements: For study of Rel-17 paging enhancement, the following are assumed as a baseline for FR1 and FR2: - Reference configuration for FR1/FR2 as specified in Section 8.1.1/8.1.2 of TR 38.840 - Note: the setting for some PDSCH parameters may not be applicable for RedCap UEs - Baseline paging cycle length: [1.28] second - SS burst related assumptions: - 20 ms periodicity - 2 ms duration for serving cell RRM measurement, which can overlap with the one for synchronization before PO - FFS time/frequency tracking - Measurement related assumptions: - 20 ms SMTC periodicity - 2 ms SMTC window for intra-frequency RRM measurement, assuming synchronized deployment - [5 ms SMTC window and 6 ms measurement gap for inter-frequency RRM measurement] - Note: RAN4 requirement assumes one frequency layer per measurement gap, and 0.5 ms is assumed for switch in/out a frequency layer - Note: the inclusion of potential TRS/CSI-RS occasions can be considered Agreements: Group paging rate of 10% is assumed for the evaluation of Rel-17 paging enhancement - FFS: Another group paging rate > 10% - Note: If UE sub-grouping is applied, the sub-group paging rate can be reduced w.r.t. the total sub-group number for a PO

### Initial proposal for UE processing timeline

According the above baseline assumptions, UE processing timeline is shown in Appendix 5.1 as FL proposed initial proposal 3 and 4.

### Time offset between PEI and PO

In [2], companies are encouraged to declare the time offset between PO and the start of the nearest SS burst.

Proposal: For characterizing the power saving gains with a PEI design without UE sub-grouping, companies are encouraged to report the power saving gains w.r.t. Rel-16 for the following cases, assuming at least 10% group paging rate, - When the time offset between PO and the start of the nearest SS burst is T (ms): - S1 (%) where the baseline assumes 1 SS burst for synchronization before PO reception - S2 (%) where the baseline assumes 2 SS bursts for synchronization before PO reception - S3 (%) where the baseline assumes 3 SS bursts for synchronization before PO reception - Note: Considered value(s) for T is by company report.

In our view, there could be two different UE implementation if PEI indicates UE to wake up.

- Implementation-1: UE should process the 2 SS bursts immediately after PEI and enter light sleep until PO.

- Implementation-2: UE should enter light sleep and wake up to process the 2 SS bursts close to PO.

For Implementation-1, the time offset between PO and the start of the nearest SS burst does not affect power consumption. Hence, the time offset between PEI and PO will influence the evaluation results.

In the following evaluations, different time offsets between PEI and PO are assumed.

### Power consumption for “no PEI”

For evaluation purpose, we assume UE should process 3 SS bursts before PO and 1 SS burst before PEI.

Table 1: UE processing timeline and power consumption for legacy operation (no PEI)

| Operation in sequence | Purpose | Time duration (ms) | Energy contribution |
| --- | --- | --- | --- |
|  |  |  | (power * time + energy overhead) |
| SSB proc. | AGC, coarse synchronization, serving-cell/intra-freq. RRM measurement | 2 | 60 * 2 |
| Light sleep | Power saving | 18 | 20 * 18 + 100 |
| SSB proc. | Coarse/fine synchronization, (additional serving-cell/intra-freq. RRM measurement) | 2 | 50 * 2 |
| Light sleep | Power saving | 18 | 20 * 18 + 100 |
| SSB proc. | Fine synchronization, (additional serv.-cell/intra-freq. RRM measurement) | 2 | 50 * 2 |
| Light sleep | Power saving | 8 | 20 * 8 + 100 |
| PDCCH or PDCCH+PDSCH | Paging control proc. and data proc. (if paged); 8 slots for diversity reception | 4 | Not paged: 50 * 4 |
|  |  |  | Paged: 120 * 4 (subject to group paging rate, P) |
| Light sleep | Power saving | 6 | 20 * 6 + 100 |
| Inter-freq. RRM measurement | Inter-frequency/inter-RAT neighbor cell measurement | 5 | 60 * 5 |
| Deep sleep | Power saving | 1215 | 1 * 1215 + 450 |
| Total |  | 1280 | 200 * (1-P) + 480 * P + 3685 |

When P = 10%, the total power consumption is about 3913.

### Power consumption when PEI indicates not wake up

Table 3: UE processing timeline and power consumption, when PEI indicates not wake up

| Operation in sequence | Purpose | Time duration (ms) | Energy contribution |
| --- | --- | --- | --- |
|  |  |  | (power * time + energy overhead) |
| SSB proc. | AGC, coarse synchronization, serving-cell/intra-freq. RRM measurement | 2 | 60 * 2 |
| Light sleep | Power saving | 16 | 20 * 16 + 100 |
| PEI | Early indicate whether the UE is paged | 2 | 50 * 2 |
| SSB proc. | Inter-frequency/inter-RAT neighbor cell measurement | 5 | 60 * 5 |
| Deep sleep | Power saving | 1255 | 1 * 1255 + 450 |
| Total |  | 1280 | 2645 |

### Power consumption when PEI indicates wake up and the time offset between PEI and PO is small

In the following evaluation, we assume the time offset between PEI and PO is small and equal to about 40ms (2 SS bursts periodicities).

Table 4: UE processing timeline and power consumption, when PEI indicates wake up and the time offset between PEI and PO is small

| Operation in sequence | Purpose | Time duration (ms) | Energy contribution |
| --- | --- | --- | --- |
|  |  |  | (power * time + energy overhead) |
| SSB proc. | AGC, coarse synchronization, serving-cell/intra-freq. RRM measurement | 2 | 60 * 2 |
| Light sleep | Power saving | 16 | 20 * 16 + 100 |
| PEI | Early indicate whether the UE is paged | 2 | 50 * 2 |
| SSB proc. | Coarse/fine synchronization, (additional serving-cell/intra-freq. RRM measurement) | 2 | 50 * 2 |
| Light sleep | Power saving | 18 | 20 * 18 + 100 |
| SSB proc. | Fine synchronization, (additional serv.-cell/intra-freq. RRM measurement) | 2 | 50 * 2 |
| Light sleep | Power saving | 8 | 20 * 8 + 100 |
| PDCCH+PDSCH | Paging control proc. and data proc. (if paged); 8 slots for diversity reception | 4 | 120 * 4 |
| Light sleep | Power saving | 6 | 20 * 6 + 100 |
| Inter-freq. RRM measurement | Inter-frequency/inter-RAT neighbor cell measurement | 5 | 60 * 5 |
| Deep sleep | Power saving | 1215 | 1 * 1215 + 450 |
| Total |  | 1280 | 5665 |

When P = 10%, the total power consumption is about 2645*0.9+5665*0.1, and the power saving gain is about 25%.

### Power consumption when PEI indicates wake up and the time offset between PEI and PO is large

In the following evaluation, we assume the time offset between PEI and PO is large and equal to about 100ms (5 SS bursts periodicities).

Table 5: UE processing timeline and power consumption, when PEI indicates wake up and the time offset between PEI and PO is large

| Operation in sequence | Purpose | Time duration (ms) | Energy contribution |
| --- | --- | --- | --- |
|  |  |  | (power * time + energy overhead) |
| SSB proc. | AGC, coarse synchronization, serving-cell/intra-freq. RRM measurement | 2 | 60 * 2 |
| Light sleep | Power saving | 16 | 20 * 16 + 100 |
| PEI | Early indicate whether the UE is paged | 2 | 50 * 2 |
| SSB proc. | Coarse/fine synchronization, (additional serving-cell/intra-freq. RRM measurement) | 2 | 50 * 2 |
| Light sleep | Power saving | 18 | 20 * 18 + 100 |
| SSB proc. | Fine synchronization, (additional serv.-cell/intra-freq. RRM measurement) | 2 | 50 * 2 |
| Light sleep | Power saving | 8+60=68 | 20 * 68 + 100 |
| PDCCH+PDSCH | Paging control proc. and data proc. (if paged); 8 slots for diversity reception | 4 | 120 * 4 |
| Light sleep | Power saving | 6 | 20 * 6 + 100 |
| Inter-freq. RRM measurement | Inter-frequency/inter-RAT neighbor cell measurement | 5 | 60 * 5 |
| Deep sleep | Power saving | 1155 | 1 * 1215 + 450 |
| Total |  | 1280 | 6865 |

When P = 10%, the total power consumption is about 2645*0.9+6865*0.1, and the power saving gain is about 22%. It can be observed that when the time offset between PEI and PO is large, the power saving gain is reduced.

From above observation, if PEI is too far away from PO, UE would skip PEI monitoring. As well, if PEI is too close to PO, and UE cannot process 2 SS bursts after PEI, UE would skip PEI monitoring.

Furthermore, we notice there are some factors, e.g. group paging rate, affecting power saving gain. These factors are involved in the discussion for evaluation purpose. These factors may be biased due to wireless environment.

Therefore, it can be up to UE implementation whether to monitor PEI, if UE predicts monitoring PEI is power consumed, e.g., the time offset between PEI and PO is too large, or group paging rate is too high.

## Evaluation of resource overhead

It was agreed in RAN1#104e [3] for the evaluation methodology of resource overhead for PEI.

Agreements: For the evaluation of resource overhead with PEI candidate designs based on PDCCH, TRS/CSI-RS and SSS, companies to provide estimated overheads for PEI candidate designs based on the following factors: - Assumption of Behv-A/B - Required #REs from performance evaluations - 10% group paging rate per PO as baseline; other group paging rates can be optionally considered and based on the following assumptions with justification (up to each company) - Whether and how coexistence with legacy UEs is considered - Whether and how indication(s) to multiple POs and/or UE subgroups by one PEI is considered - Whether and how multi-beam transmission is considered

It is common understanding that the resource overhead can be modeled as follows.

Resource overhead per group/subgroup = (the required number of REs per PEI) × (PEI transmission probability) ÷ (the number of groups/subgroups per PEI).

### Assumption of the required number of REs per PEI

There are two alternatives agreed in RAN1#104e [3] for performance requirement (MDR and FAR).

Agreement: - Take Alt 1 as mandatory, and Alt 2 as optional Alt 1 For the performance evaluations of PEI candidate designs based on PDCCH, TRS/CSI-RS and SSS, - The following are assumed, at the SNR where the Miss-Detection Rate (MDR) of paging PDSCH is 1%, - When Behv-A is assumed: - The joint miss-detection rate (MDR) of PEI and paging PDCCH defined below should be no worse than 1%: MDR_Joint_A = MDR_PEI + (1 – MDR_PEI) MDR_PagingPDCCH - The False-Alarm Rate (FAR) of PEI should be no larger than [1%] - When Behv-B is assumed: - The joint miss-detection rate (MDR) of PEI and paging PDCCH defined below should be no worse than 1%: MDR_Joint_B = FAR_PEI + (1 – FAR_PEI) MDR_PagingPDCCH - The MDR of PEI should be no larger than [1%] - Note: The CFO is modeled at the input of PEI detection and based on LLS assumptions agreed in RAN1 #102-e. Companies should justify the applied random range for the CFO. - Companies to provide: - Information on the utilized detection method for each PEI candidate design (e.g., non-coherent detection or coherent detection) - The required #REs to comply with the performance assumptions - The maximum number of subgroups that can be carried in PEI, subject to the performance assumptions Alt 2 For the performance evaluations of PEI candidate designs based on PDCCH, TRS/CSI-RS and SSS, - The following are assumed, at the SNR where the Miss-Detection Rate (MDR) of paging DCI is 1%, - When Behv-A is assumed: - The MDR of PEI should be no larger than 0.1% - The False-Alarm Rate (FAR) of PEI should be no larger than 1% - When Behv-B is assumed: - The FAR of PEI should be no larger than 0.1% - The MDR of PEI should be no larger than 1% - Note: The CFO is modeled at the input of PEI detection and based on LLS assumptions agreed in RAN1 #102-e. Companies should justify the applied random range for the CFO. - Companies to provide: - Information on the utilized detection method for each PEI candidate design (e.g., non-coherent detection or coherent detection) - The required #REs to comply with the performance assumptions - The maximum number of subgroups that can be carried in PEI, subject to the performance assumptions

It seems there is no essential difference between Alt-1 and Alt-2. As Alt-1 is mandatory, we regard Alt-1 as the performance requirement.

- For Behv-A

  - MDR_PEI=(MDR_Joint_A-MDR_PagingPDCCH)/(1-MDR_PagingPDCCH)

  - If the MDR of paging PDCCH is 0.9%, to achieve the joint MDR equal to 1%, the MDR of PEI should be 0.1%.

  - The FAR of PEI should be no larger than 1%.

- For Behv-B

  - FAR_PEI=(MDR_Joint_B-MDR_pagingPDCCH)/(1-MDR_PagingPDCCH)

  - For sequence-based PEI

    - If the MDR of paging PDCCH is 1%, to achieve the joint MDR equal to 1%, the FAR of PEI should be 0. The FAR is too low for sequence-based PEI, so we use the optional assumption (Alt 2) and FRA of PEI should be 0.1%.

  - For DCI-based PEI:

    - If the MDR of paging PDCCH is 1%, to achieve the joint MDR equal to 1%, the FAR of PEI should be 0. We consider FAR equal to 2^(-21) for PDCCH (CRC length 24) approaching to 0.

  - The MDR of PEI should be no larger than 1%.

According to the above performance requirement, the requirement number of REs in one PEI (monitoring occasion) are listed as follows.

- Behv-A

  - For sequence-based PEI:

    - The MDR of PEI is 0.1%, and the FAR of PEI is 1%.

    - The required number of REs in one PEI is X_A, which is equal to about 200.

  - For DCI-based PEI:

    - The MDR of PEI is 0.1%, and the FAR of PEI is 1%.

    - The required number of REs in one PEI is Y_A, which is equal to about 288. It is assumed that AL is 4, and DCI size is 12 bits. The 12 bits DCI can be used for wakeup indication of one UE group/subgroup, TRS availability etc.

- Behv-B

  - For sequence-based PEI:

    - The FAR of PEI is 0.1%, and the MDR of PEI is 1%.

    - The required number of REs in one PEI is X_B, which is equal to about 200.

  - For DCI-based PEI:

    - The FAR of PEI is close to 0, and the MDR of PEI is 1%.

    - The required number of REs in one PEI is Y_B, which is equal to about 288. It is assumed that AL is 4, and DCI size is 12 bits. Each bit in 12 bits DCI can be used for wakeup indication of one UE group/subgroup.

It should be noted that the required numbers of REs in one PEI for sequence-based and DCI-based are estimated under the same channel condition and SINR point. In our estimation, the SINR point is chosen to satisfy the joint MDR of PDSCH and PDCCH under assumption of “PDSCH: MCS0, TB scaling 1.0, PDCCH: AL8, 41-bit payload”.

Some companies thought that sequence-based PEI can be transmitted in Single-Frequency Network (SFN) way to form an omni-directional transmission. But the RSRP or SINR at UE side will be degraded in SFN way, and it will also influence the sequence detection which could be non-coherent detection. For fair comparison, we assume the single beam (one SSB beam) for both sequence-based PEI and DCI-based PEI.

In NB-IoT/eMTC, the UE subgroups can be differentiated by multiple sequences, i.e. single-sequence CDM mechanism (only single sequence is transmitted in a time/frequency domain resource). The MDR of multiple sequences will be raised if keeping FAR of detection of multiple sequences unchanged. To keep the MDR low enough, the resource overhead should increase. In our view, the increase of the resource overhead is nearly proportional to the number of multiple sequences. Therefore, it is hard to achieve the reduction of the resource overhead by using the single-sequence CDM mechanism.

The cell ID in the sequence or scrambling in all PEI candidate schemes (including SSS-based PEI) cannot be used to differentiate UE group/subgroup, since the cell ID is used to combat the inter-cell interference already. Otherwise, the MDR/FAR will be degraded significantly if the inter-cell interference is modeled or the system level simulation is conducted.

### Assumption of PEI transmission probability

The PEI transmission probability is related to co-existence b/w PEI and legacy channels/signals.

It was agreed in RAN1#104e [3] that companies should declare whether/how PEI design can co-exist with existing channels/signals.

Agreements: For the evaluation and comparison of PEI candidate designs, companies to report - Description of how PEI design can co-exist with existing channels/signals, and impact to legacy UEs. - Rel-15 designs for multiplexing PEI with legacy channels/signals are assumed as baseline o Other multiplexing method with legacy channels/signals can be additionally reported with justification

The above co-existence (resource sharing) issue was extensively discussed in RAN1#104bis-e. For co-existence b/w PEI and PDCCH, there was no consensus for the description. Companies thought both sequence-based and DCI-based PEI can occupy the resource of PDCCH and the R15 UE may not be influenced in detection of PDCCH, even if there is risk of false alarm. For co-existence b/w PEI and PDSCH, the consensus was achieved that rate matching around both the sequence-based and DCI-based PEI for the legacy PDSCH can be realized, which is shown as follow.

Agreement: Observation 1a: For the evaluation and comparison of PEI candidate designs, the following observations for coexistence with legacy PDSCH are identified: - For coexistence with legacy PDSCH, semi-static resouce sharing by configuring RB-symbol-level or RE-level rate-matching patterns covering PEI REs is supported for all PEI candidate designs. - For coexistence with legacy PDSCH, dynamic resource sharing can be realized for all PEI candidates if PDSCH is scheduled by DCI format 1_1 - For PDCCH based PEI, CORESET-level rate matching can be realized for the PDSCH as per mandatory capability - For SSS-based PEI, CORESET-level rate matching may be realized for the PDSCH as per mandatory capability, depending on the design of SSS-based PEI and UE capability regarding number of supported CORESETs - For TRS/CSI-RS based PEI, RE-level rate matching can be realized for the PDSCH as per mandatory capability - When PDSCH is not scheduled by DCI format 1_1, it is up to gNB implementation whether and how PEI is transmitted in PDSCH resource

In our view, the PEI transmission probability for Behv-A and Behv-B are different.

- Behv-A

  - For sequence-based PEI:

    - Dynamic rate-matching in PDSCH. The PEI transmission probability is equal to paging rate of subgroup.

  - For DCI-based PEI:

    - PEI is transmitted as a Rel-15 PDCCH in a CORESET when a UE group is paged, or dynamic rate-matching in PDSCH. The PEI transmission probability is equal to paging rate of group. It is assumed that wakeup indication of UE subgroup is carried by DCI bits.

- Behv-B

  - For sequence-based PEI:

    - Semi-static rate-matching in PDSCH. The PEI transmission probability is equal to 1.

  - For DCI-based PEI:

    - PEI is always transmitted as a Rel-15 PDCCH in a CORESET, or semi-static rate-matching in PDSCH. The PEI transmission probability is equal to 1.

### Assumption of the number of groups per PEI without subgrouping

Without UE subgrouping, the assumption of the number of groups per PEI is listed as follows.

- Behv-A

  - For sequence-based PEI:

    - There is 1 group corresponding to one PEI. We assume there is one PEI associated to a PO.

  - For DCI-based PEI:

    - There is 1 group corresponding to one PEI. We assume there is one PEI associated to a PO.

- Behv-B

  - For sequence-based PEI:

    - There is 1 group corresponding to one PEI. We assume there is one PEI associated to a PO.

  - For DCI-based PEI:

    - There is 12 group corresponding to one PEI. We assume there is one PEI associated to 12 POs.

### Evaluation results without subgrouping

The resource overhead can be evaluated as follows.

- Behv-A

  - For sequence-based PEI:

    - The required number of REs in one PEI could be X_A = 200.

    - The PEI transmission probability is paging rate of group = 0.1.

    - There is 1 group corresponding to one PEI.

    - Resource overhead per group = X_A*(paging rate of group) = 200*0.1 = 20.

  - For DCI-based PEI:

    - The required number of REs in one PEI could be Y_A = 288.

    - The PEI transmission probability is paging rate of group = 0.1.

    - There is 1 group corresponding to one PEI.

    - Resource overhead per group = Y_A*(paging rate of group) = 288*0.1 = 28.8.

- Behv-B

  - For sequence-based PEI:

    - The required number of REs in one PEI could be X_B = 200.

    - The PEI transmission probability is 1.

    - There is 1 group corresponding to one PEI.

    - Resource overhead per group = X_B = 200.

  - For DCI-based PEI:

    - The required number of REs in one PEI could be Y_B = 288.

    - The PEI transmission probability is 1.

    - There is 12 groups corresponding to one PEI.

    - Resource overhead per group = Y_B/(number of groups per PEI) = 288/12 = 24.

The following table summarize the resource overhead per group/subgroup in the above evaluation.

Table 2: The resource overhead per group/subgroup in the above evaluation

| Behv-A | Sequence-based PEI | X_A*(paging rate of group) = 200*0.1 = 20 |
| --- | --- | --- |
|  | DCI-based PEI | Y_A*(paging rate of group) = 288*0.1 = 28.8 |
| Behv-B | Sequence-based PEI | X_B = 200 |
|  | DCI-based PEI | Y_B/(number of groups per PEI) = 288/12 = 24 |

### Assumption of the number of groups/subgroups per PEI with subgrouping

With UE subgrouping, the assumption of the number of groups/subgroups per PEI is listed as follows.

- Behv-A

  - For sequence-based PEI:

    - There is 1 subgroup corresponding to one PEI. We assume there is one PEI associated to a UE subgroup.

  - For DCI-based PEI:

    - There is 3 subgroup corresponding to one PEI. We assume there is one PEI associated to 3 UE subgroups, and wakeup indication of UE subgroup is carried by 3 bits.

- Behv-B

  - For sequence-based PEI:

    - There is 1 subgroup corresponding to one PEI We assume there is one PEI associated to a UE subgroup..

  - For DCI-based PEI:

    - There is 12 subgroup corresponding to one PEI. We assume there is one PEI associated to 4 PO each including 3 UE subgroups, and wakeup indication of UE subgroup is carried by 12 bits.

### Evaluation results with subgrouping

The resource overhead can be evaluated as follows.

- Behv-A

  - For sequence-based PEI:

    - The required number of REs in one PEI could be X_A = 200.

    - The PEI transmission probability is paging rate of subgroup = 0.1.

    - There is 1 subgroup corresponding to one PEI.

    - Resource overhead per subgroup = X_A*(paging rate of subgroup) = 200*0.1 = 20.

  - For DCI-based PEI:

    - The required number of REs in one PEI could be Y_A = 288.

    - The PEI transmission probability is paging rate of group = 0.27, which corresponds to paging rate of subgroup equal to 0.1 and there are 3 UE subgroups per PO. In another case, the PEI transmission probability is paging rate of group = 0.57, which corresponds to paging rate of subgroup equal to 0.1 and there are 8 UE subgroups per PO

    - There is 3 subgroups corresponding to one PEI.

    - Resource overhead per subgroup = Y_A*(paging rate of group)/(number of subgroup per PEI) = 288*0.27/3 = 26. In another case, resource overhead per subgroup = Y_A*(paging rate of group)/(number of subgroup per PEI) = 288*0.57/8 = 20.52.

- Behv-B

  - For sequence-based PEI:

    - The required number of REs in one PEI could be X_B = 200.

    - The PEI transmission probability is 1.

    - There is 1 subgroup corresponding to one PEI.

    - Resource overhead per subgroup = X_B = 200.

  - For DCI-based PEI:

    - The required number of REs in one PEI could be Y_B = 288.

    - The PEI transmission probability is 1.

    - There is 12 subgroups corresponding to one PEI.

    - Resource overhead per group = Y_B/(number of subgroup per PEI) = 288/12 = 24.

The following table summarize the resource overhead per group/subgroup in the above evaluation.

Table 3: The resource overhead per group/subgroup in the above evaluation

| Behv-A | Sequence-based PEI | X_A*(paging rate of subgroup) = 200*0.1 = 20 |
| --- | --- | --- |
|  | DCI-based PEI | Y_A*(paging rate of group)/(number of subgroups per PEI) = 288*0.27/3 = 26, or 288*0.57/8 = 20.52 |
| Behv-B | Sequence-based PEI | X_B = 200 |
|  | DCI-based PEI | Y_B/(number of subgroups per PEI) = 288/12 = 24 |