3GPP TSG RAN WG1 #105-e	R1-2104557

e-Meeting, May 19th – May 27th, 2021

Agenda item:      8.14.3

Source:	Nokia, Nokia Shanghai Bell

Title:	Performance results in indoor hotspot and dense urban deployments of CG and VR applications

Document for:     Discussion

# Introduction

RAN#91 approved a revised SID on XR Evaluations for NR [1]. The XR SI is now ongoing with focus on related system-level performance evaluation to assess how well the currently standardized NR system can support such services. In this document, we present our preliminary results of XR and CG services in 5G NR. Two deployment scenarios are considered for evaluating the performance of XR and CG services, as agreed in [2]: (1) Indoor Hotspot (InH) and (2) Dense Urban (UMa). In these two deployments, we evaluate the DL system capacity of Cloud Gaming and Virtual Reality/Augmented Reality applications according to the DL traffic models discussed in [3].

# Statistical Models and KPI for XR Traffic

In this section, we discuss the parameters we used in our performance evaluation for the statistical model for XR traffic. The parameters have been set according to the discussion in [2,4,5]. Table 1 and Table 2 illustrate the parameters for generating DL traffic (i.e., video traffic).

Table 1 – Parameters of frame size distribution for DL traffic model computed as a function of data rate.

| Parameter | Proposed value |  |  |
| --- | --- | --- | --- |
| Application | CG | CG/VR/AR | VR/AR |
| Frame Rate | 60 fps |  |  |
| Frame size distribution | Truncated Gaussian |  |  |
| Bitrate | 8 Mbps | 30 Mbps | 45 Mbps |
| Frame size (avg) | 16667 Bytes | 62500 Bytes | 93750 Bytes |
| Frame size (std) | 1750 Bytes (10.5% of mean value) | 6563 Bytes (10.5% of mean value) | 9844 Bytes (10.5% of mean value) |
| Frame size (max) | 25000 Bytes (150% of mean value) | 93750 Bytes (150% of mean value) | 140625 Bytes (150% of mean value) |
| Frame size (min) | 8334 Bytes (50% of mean value) | 31250 Bytes (50% of mean value) | 46875 Bytes (50% of mean value) |
| Transport protocol | UDP |  |  |
| Maximum transport packet size | 25000 Bytes 1 frame = 1 PDU | 93750 Bytes 1 frame = 1 PDU | 140625 Bytes 1 frame = 1 PDU |

Table 2 – Parameters of the interarrival time distribution for DL traffic model computed as a function of frame rate and jitter.

| Parameter | Proposed value |
| --- | --- |
| Frame Rate | 60 fps |
| Inter-arrival time distribution | Truncated Gaussian |
| Interarrival time (avg) | 16.67 ms |
| Interarrival time (std) | 2 ms |
| Interarrival time (max) | 20.67 ms |
| Interarrival time (min) | 12.67 ms |

As agreed in [2,4,5], the system capacity is measured in terms on “number of satisfied UEs”. More specifically, a UE is marked as “satisfied UE” if more than X% of packets are successfully transmitted over the air interface within the PDB. The system capacity is defined as the maximum number of UEs per cell with at least Y% of UEs marked as satisfied UEs. In our evaluation, we adopt X=99% as the percentage of frames to be successfully received within the PDB to identify satisfied UEs, while Y=90% as the percentage of satisfied UEs to measure the system capacity.

# Capacity Evaluation of Indoor Hotspot

## Simulation settings

The carrier frequency is set to 4 GHz and 30 GHz for FR1 and FR2, respectively. System bandwidth is assumed to be 100 MHz for FR1 and for FR2 to compare achievable system capacity when propagation and antenna configuration change. Time division duplexing (TDD) is configured according to the first option, thus using “DDDSU” as radio frame. The smallest schedulable radio resource is the physical radio blocks (PRBs) of 12 subcarriers, each is of 30 kHz and 120 kHz for FR1 and FR2, respectively. The TTI size is set to 14 OFDM symbols, with one control symbol, always placed at the start of each TTI. The asynchronous HARQ Chase combing is adopted with maximum 3 HARQ retransmission before a packet is dropped (i.e., marked with an infinite radio latency). The transmit power of gNBs is set as follows: 31dBm with 100MHz (24dBm per 20MHz) in FR1, and 24dBm with (23dBm per 80MHz) in FR2. Table 3 lists the main parameters of the Indoor Hotspot deployment that are considered in this study.

Table 3 – Main parameters for Indoor Hotspot (InH) deployment

| Parameter | Value |  |  |
| --- | --- | --- | --- |
| Layout | - 120m x 50m, Single layer (indoor floor, open office) - 12 cells/TRPs - ISD: 20m |  |  |
| Channel model | InH |  |  |
| Carrier frequency | FR1: 4 GHz | FR2: 30 GHz |  |
| Subcarrier spacing | FR1: 30 kHz | FR2: 120 kHz |  |
| System bandwidth | FR1: 100 MHz | FR2: 100 MHz |  |
| BS height | 3 m |  |  |
| UE height | 1.5 m |  |  |
| BS noise figure | FR1: 5 dB | FR2: 7 dB |  |
| UE noise figure | FR1: 9 dB |  |  |
| BS receiver | LMMSE-IRC |  |  |
| UE receiver | LMMSE-IRC |  |  |
| Channel estimation | Ideal |  |  |
| UE speed | 3 km/h |  |  |
| MCS | Up to 256QAM |  |  |
| BS Tx power | FR1: 31dBm (24dBm per 20MHz) | FR2: 24 dBm (23dBm per 80MHz) |  |
| UE Tx max power | 23 dBm |  |  |
| TDD Frame structure | DDDSU |  |  |
| Cell Selection | RSRP Slow Fading |  |  |
| BS antenna configuration | - Pattern: Ceiling-mount antenna radiation pattern - Gain: 5 dBi - Downtilt: 90° |  |  |
|  | Configuration in FR1: - 32 TxRU - (M, N, P, Mg, Ng) = (4,4,2,1,1) - (dH, dV) = (0.5, 0.5)λ - (Mp, Np) = (4,4) |  | Configuration in FR2: - 2 TxRU - (M, N, P, Mg, Ng) = (16, 8, 2,1,1) - (dH, dV) = (0.5, 0.5) λ - (Mp, Np) = (1,1) Grid of Beams: - Azimuth angles (degrees): {90, 90, 90, 112.5, 112.5, 112.5, 67.5, 67.5, 67.5, 140, 140, 140, 40, 40, 40} - Elevation angles (degrees): {-30, 0, 30, -30, 0, 30, -30, 0, 30, -30, 0, 30, -30, 0, 30} |
| UE antenna configuration | - Pattern : Omni-directional, - Gain : 0 dBi, - Configuration : 2T/4R (M, N, P, Mg, Ng) = (1,2,2,1,1) (dH, dV) = (0.5, 0)λ (Mp, Np) = (1,2) |  | - Pattern: UE radiation pattern model 1 (TR 38.901) - Gain: 5 dBi - Configuration : (M, N, P) = (1, 4, 2), (dH, dV) = (0.5, 0)λ (Mp, Np) = (1,1) 3 panels (left, right, top) |
| Scheduler | SU-MIMO, guaranteed bitrate in time domain and delay-aware in frequency domain |  |  |
| CSI acquisition | Periodic CQI on 2 ms period |  |  |
| PHY processing delay | PDSCH decoding: 4OFDM symbols |  |  |
| PDCCH overhead | Modelled |  |  |
| Target BLER | 10% for first transmission |  |  |
| Max HARQ transmission | 3 |  |  |
| HARQ scheme | Chase combining |  |  |

## Simulation results

In this section, we present the results obtained with our system level simulator in the Indoor Hotspot deployment scenario for FR1 and FR2. In Section 3.2.1 we show the system capacity for UEs running CG applications at 30Mbps and 8Mbps, whereas in Section 3.2.2 we focus on UEs running VR applications at 30Mbps and 45Mbps. UEs are randomly dropped in the simulated area while guaranteeing that all cells have the same number of connected UEs, this is to guarantee that the cells are running under similar load conditions.

### Cloud Gaming: DL Capacity

Figure 1 shows the ratio of satisfied UEs for CG computed with 99% of frames received within the PDB of 15ms as a function of the average cell load, which is measured in number of UEs per cell. Results correspond to a system bandwidth of 100MHz for both FR1 and FR2, and for data rates of 30Mbps and 8Mbps. The dashed black line indicates the 90% limit on the number of satisfied UEs to measure the system capacity. According to the 90% limit, we conclude that in the Indoor Hotspot deployment we can support up to 4 UEs per cell (i.e., 48 UEs in the network) in FR1, with the capacity showing a significant increase in FR2 with more than 8 satisfied UEs per cell (i.e., more than 96 UEs in the network). The higher capacity for FR2 is mainly due to the use of the more sophisticated gNB GoB configuration. When lowering the data rate to 8Mbps more than 8 UEs per cell (i.e., 120 UEs in the network) can be supported for both FR1 and FR2.

| (a) CG in FR1 at 30Mbps with X=99%. | (b) CG in FR2 at 30Mbps with X=99%. |
| --- | --- |
| (c) CG in FR1 at 8Mbps with X=99%. | (d) CG in FR2 at 8Mbps with X=99%. |

Figure 1 – DL capacity evaluation of CG traffic in Indoor Hotspot in FR1 and FR2 with X=99% of frames received within PDB, 100MHz system bandwidth and cells evenly loaded. The lower bound on the percentage of satisfied user is Y=90% (dashed line).

Observation 1: When we consider CG traffic model at 30Mbps and 8Mbps for an InH deployment, we can support the number of UEs per cell in DL indicated in Table 4.

Table 4 – System capacity in DL for CG (PDB=15ms) at 30Mbps [#UEs/cell] in an InH deployment

|  | 30Mbps | 8Mbps |
| --- | --- | --- |
| FR1 | 4 | ≥8 |
| FR2 | ≥8 | ≥8 |

### Augmented/Virtual Reality: DL Capacity

In this section, we illustrate the results on the system capacity obtained for VR/AR applications, namely with PDB=10ms. Figure 2 shows the ratio of satisfied UEs computed with 99% of frames received within the PDB as a function of the average cell load, which is measured in number of UEs; results correspond to a system bandwidth of 100MHz for both FR1 and FR2, and data rates of 30Mbps and 45Mbps. The dashed black line indicates the 90% limit on the number of satisfied UEs to measure the system capacity. According to the 90% limit, we can conclude that in the Indoor Hotspot deployment in FR1 we can support 3 UEs per cell running VR/AR applications at 30Mbps (i.e., 36 UEs in the network), with such capacity increasing to more than 8 UEs per cell (i.e., more than 96 UEs in the network) for the same data rate when using FR2. For a 45Mbps data rate, in FR1 we can support 2 UEs per cell (i.e., 24 UEs in the network) whereas in FR2 up to 6 UEs per cell (i.e., 72 UEs in the network) can be supported.

| (a) VR/AR in FR1 at 30Mbps with X=99%. | (b) VR/AR in FR2 at 30Mbps with X=99%. |
| --- | --- |
| (c) VR/AR in FR1 at 45Mbps with X=99%. | (d) VR/AR in FR2 at 45Mbps with X=99%. |

Figure 2 –  DL capacity evaluation of VR/AR traffic in Indoor Hotspot in FR1 and FR2 with X=99% of frames received within PDB, 100MHz system bandwidth and cells evenly loaded. The lower bound on the percentage of satisfied user is Y=90% (dashed line).

Observation 2: When we consider VR traffic model at 30Mbps and 45Mbps for an InH deployment, we can support the number of UEs per cell in DL indicated in Table 5.

Table 5 – System capacity in DL for VR (PDB=10ms) at 30Mbps [#UEs/cell] in an InH deployment

|  | Rate: 30Mbps | Rate: 45Mbps |
| --- | --- | --- |
| FR1 | 3 | 2 |
| FR2 | ≥8 | 6 |

# Power Saving Evaluation in Indoor Hotspot

## Simulation settings

The same configuration of the Indoor Hotspot deployment described in Section 3.1 has been considered for the evaluation of UE power consumption (please refer to Table 3 for a detailed description of main simulation parameters). Only DL traffic on CG and VR applications at 30Mbps has been considered in this analysis (baseline data rate for both applications). Furthermore, the following three CDRX configurations have been considered to evaluate the trade-off between power saving and achievable capacity:

- CDRX 1: (DRX long cycle, inactivity timer value, On duration timer value)=(4, 2, 2)

- CDRX 2: (DRX long cycle, inactivity timer value, On duration timer value)=(8, 4, 4)

- CDRX 3: (DRX long cycle, inactivity timer value, On duration timer value)=(16, 8, 8)

## Simulation results

In the following, we illustrate the measured power saving and capacity in Indoor Hotspot deployment when CDRX is enabled. The three proposed configurations are compared to the results with no DRX shown in Section 3 (labeled “Always ON” in this section) in terms of average power saving gain and capacity loss.

### Cloud Gaming: DL Capacity vs Power Consumption

In this section, we present the system capacity results obtained for CG application at 30Mbps, with PDB=15ms.  Figure 3 illustrates the absolute values of capacity and mean power consumption (across all UEs in the system) considering UEs always ON (no DRX is used) and the three CDRX configurations described above. Table 6 reports the average power gain and system capacity loss of the three considered CDRX configurations with respect to Always ON policy.

| - Capacity - Always ON. | (b) UE Mean Power - Always ON. |
| --- | --- |
| - Capacity - DRX (4,2,2). | (d) UE Mean Power - DRX (4,2,2). |
| (e) Capacity - DRX (8,4,4). | (f) UE Mean Power - DRX (8,4,4). |
| (g) Capacity - DRX (16,8,8). | (h) UE Mean Power - DRX (16,8,8). |

Figure 3 – Capacity and UE mean power consumption in Indoor Hotspot in FR1 for CG at 30Mbps with X=99%           (DL only).

Table 6 – Evaluation of UE power saving schemes for {Indoor Hotspot, CG, DL Only, 30Mbps, FR1}

| Power Saving Scheme | Power Saving Gain (PSG) compared to ‘Always ON’ |  |  |  | #satisfied UEs per cell with PS / #satisfied UEs per cell w/o PS |
| --- | --- | --- | --- | --- | --- |
|  | Baseline | Optional |  |  |  |
|  | Mean PS gain | PS gain of 5%-tile UE in PSG CDF | PS gain of 50%-tile UE in PSG CDF | PS gain of 95%-tile UE in PSG CDF |  |
| Always ON | - | - | - | - | 4 / 4 |
| (4, 2, 2) | 73 % | 75 % | 74 % | 70 % | 3 / 4 |
| (8, 4, 4) | 66 % | 69 % | 67 % | 58 % | 3 / 4 |
| (16, 8, 8) | 50 % | 55 % | 52 % | 40 % | 3 / 4 |

Observation 3: Power saving gain and DL capacity loss with different CDRX configurations for CG applications in Indoor Hotspot deployment in FR1 are indicated in Table 6.

Observation 4: CDRX reduces UE power consumption by up to 74% but it also affects the DL system capacity for CG applications, reducing it by 1 UEs/cell for all considered DRX configurations.

Observation 5: The CDRX configuration that achieves the best trade-off between power saving gain and DL capacity loss for CG applications in Indoor Hotspot deployment in FR1 is (DRX long cycle, inactivity timer value, On duration timer value)=(4, 2, 2).

### Virtual Reality: DL Capacity vs Power Consumption

In this section, we illustrate the results for the system capacity and power consumption obtained for VR applications at 30Mbps, with PDB=10ms. Figure 4 shows the absolute values of capacity and mean power consumption (across all UEs in the system) considering UEs always ON (no DRX) and the three CDRX configurations described above. Table 7 reports the average power gain and system capacity loss of the three considered CDRX configurations with respect to Always ON policy.

| (a) Capacity - Always ON. | (b) UE Mean Power - Always ON. |
| --- | --- |
| (c) Capacity - DRX (4,2,2). | (d) UE Mean Power - DRX (4,2,2). |
| (e) Capacity - DRX (8,4,4). | (f) UE Mean Power - DRX (8,4,4). |
| (g) Capacity - DRX (16,8,8). | (h) UE Mean Power - DRX (16,8,8). |

Figure 4 – Capacity and UE mean power consumption in Indoor Hotspot in FR1 for VR at 30Mbps with X=99% (DL-only).

Table 7 – Evaluation of UE power saving schemes for {Indoor Hotspot, VR, DL Only, 30Mbps, FR1}

| Power Saving Scheme | Power Saving Gain (PSG) compared to ‘Always ON’ |  |  |  | #satisfied UEs per cell with PS / #satisfied UEs per cell w/o PS |
| --- | --- | --- | --- | --- | --- |
|  | Baseline | Optional |  |  |  |
|  | Mean PS gain | PS gain of 5%-tile UE in PSG CDF | PS gain of 50%-tile UE in PSG CDF | PS gain of 95%-tile UE in PSG CDF |  |
| Always ON | - | - | - | - | 3 / 3 |
| (4, 2, 2) | 73 % | 75 % | 74 % | 70 % | 3 / 3 |
| (8, 4, 4) | 66 % | 69 % | 67 % | 58 % | 2 / 3 |
| (16, 8, 8) | 50 % | 55 % | 52 % | 40 % | 0 / 3 |

Observation 6: Power saving gain and DL capacity loss with different CDRX configurations for VR applications in Indoor Hotspot deployment in FR1 are indicated in Table 7.

Observation 7: CDRX reduces UE power consumption by up to 75% but it also strongly affects the DL system capacity for VR applications, reducing it to 0 in the worst case due to the low delay budget with respect to the simulated retransmission delay.

Observation 8: The CDRX configuration that archives the best trade-off between power saving gain and DL capacity loss for VR applications in Indoor Hotspot deployment in FR1 is (DRX long cycle, inactivity timer value, On duration timer value)=(4, 2, 2).

# Capacity Evaluation in Dense Urban

## Simulation settings

The carrier frequency is set to 4 GHz and 30GHz for FR1 and FR2, respectively. System bandwidth has been fixed to 100MHz for both FR1 and FR2 to compare achievable system capacity when propagation and antenna configuration change. Time division duplexing (TDD) is configured according to the first option, thus using “DDDSU” as radio frame. The smallest schedulable radio resource is the physical radio blocks (PRBs) of 12 subcarriers, each is of 30kHz and 120kHz for FR1 and FR2, respectively. The TTI size is set to 14 OFDM symbols, with one control symbol, always placed at the start of each TTI. The asynchronous HARQ Chase combing is adopted with maximum 3 HARQ retransmission before a packet is dropped (i.e., marked with an infinite radio latency). The transmit power of gNBs is set to 50 dBm (i.e., 43dBM for 20MHz). Table 8 lists the main parameters of the Dense Urban deployment that are considered in this study.

Table 8– Main parameters for Dense Urban (UMa) deployment

| Parameter | Value |  |
| --- | --- | --- |
| Layout | 21 cells with wraparound (ISD: 200m) |  |
| Channel model | UMa |  |
| Carrier frequency | FR1: 4 GHz | FR2: 30 GHz |
| Subcarrier spacing | FR1: 30 kHz | FR2: 120 kHz |
| System bandwidth | FR1: Option 1: 100 MHz | FR2: Option 1: 100 MHz |
| BS height | 25m |  |
| UE height | hUT = 3(nfl – 1) + 1.5 |  |
|  | Outdoor: nfl = 1 | Indoor: - nfl ~ uniform(1,Nfl) - Nfl ~ uniform(4,8) |
| BS noise figure | FR1: 5 dB |  |
| UE noise figure | FR1: 9 dB |  |
| BS receiver | MMSE-IRC |  |
| UE receiver | MMSE-IRC |  |
| Channel estimation | Ideal |  |
| UE speed | 3 km/h |  |
| MCS | Up to 256QAM |  |
| BS Tx power | 44 dBm per 20 MHz 51 dBm per 100 MHz |  |
| UE Tx max power | FR1: 23 dBm |  |
| TDD Frame structure | Option 1: DDDSU |  |
| Mechanical Downtilt | Baseline: 6 degrees |  |
| Cell Selection | RSRP Slow Fading |  |
| BS antenna configuration | FR1: 32TxRUs (M, N, P, Mg, Ng; Mp, Np) = (8,2,2,1,1,8,2), (dH, dV) = (0.5λ, 0.5λ) | FR2: 2TxRUs (M, N, P, Mg, Ng; Mp, Np) = (4,8,2,2,2;1,1), (dH, dV) = (0.5λ, 0.5λ) Grid of Beams - Azimuth angles: {33.75, 56.25, 78.75, 101.25, 123.75, 146.25, 33.75, 56.25, 78.75, 101.25, 123.75, 146.25} degrees - Elevation angles: {-22.5, -22.5, -22.5, -22.5, -22.5, -22.5, -67.5, -67.5, -67.5, -67.5, -67.5, -67.5} degrees |
| UE antenna configuration | FR1: 2T/4R, (M, N, P, Mg, Ng; Mp, Np) = (1,2,2,1,1;1,2), (dH, dV) = (0.5λ, 0.5λ) | FR2: (M, N, P)=(1, 4, 2), 3 panels (left, right, top) (Mp, Np)=(1, 1) |
| Power control parameter | Open loop, Alpha = 1, P0 = -106 dBm |  |
| Scheduler | SU-MIMO, guaranteed bitrate in time domain and delay-aware in frequency domain |  |
| CSI acquisition | Periodic CQI on 2 ms period |  |
| PHY processing delay | PDSCH decoding: 6 OFDM symbols |  |
| PDCCH overhead | Modelled |  |
| Target BLER | 10% for first transmission |  |
| Max HARQ transmission | 3 |  |
| HARQ scheme | Chase combining |  |

## Simulation results

In the following, we illustrate the results obtained with our system level simulator in Dense Urban deployment scenario presented in the previous section. We illustrate the DL system capacity when UEs run either CG (Section 5.2.1) or VR/AR (Section 5.2.2) applications. We selected the even load random placement of UEs in the simulation area to be sure that all cells get the same number of connected UEs. Specifically, UEs are randomly dropped in the simulated area one by one according to a spatial uniform distribution. This process is repeated for all UEs until each cell served exactly the same number of UEs. Note that cell selection for a UE is based on RSRP.

### Cloud Gaming: DL Capacity

In this section, we illustrate the results on the system capacity obtained for CG application at 30Mbps, with PDB=15ms. Figure 5 shows the ratio of satisfied UEs when cells are evenly loaded. The ratio of satisfied UEs has been computed with 99% of frames received within the PDB both in FR1 and FR2 configurations as a function of the average cell load, which is measured in number of UEs. The dashed black line indicates the 90% limit on the number of satisfied UEs to measure the system capacity. We can observe that up to 7 and 8 UEs/cell running CG services can be supported in DL in FR1 and FR2 respectively.

| (a) CG in FR1 at 30Mbps with X=99%. | (b) CG in FR2 at 30Mbps with X=99%. |
| --- | --- |
| (a) CG in FR1 at 8Mbps with X=99%. | (b) CG in FR2 at 8Mbps with X=99%. |

Figure 5 – DL Capacity evaluation of CG traffic in FR1 and FR2 with X=99% of frames received within PDB and cells evenly loaded. The lower bound on the percentage of satisfied user is Y=90% (dashed line).

Observation 9: When we consider baseline CG traffic model for DL for Dense Urban deployment, we can support the number of UEs per cell indicated in Table 9.

Table 9 – System capacity in DL for CG (PDB=15ms) at 30Mbps and 8Mbps [#UEs/cell]

|  | Rate: 30Mbps | Rate: 8Mbps |
| --- | --- | --- |
| FR1 | 7 | ≥ 26 |
| FR2 | 8 | ≥ 26 |

### Virtual Reality: DL Capacity

In this section, we illustrate the results on the DL system capacity obtained for VR application, namely with PDB=10ms. As in the previous analysis, UE are placed in the simulation area to obtain evenly loaded cells in terms of connected UEs.

| (a) VR in FR1 at 45Mbps with X=99%. | (b) VR in FR2 at 45Mbps with X=99%. |
| --- | --- |

Figure 6 shows the ratio of satisfied UEs when cells are evenly loaded. The ratio of satisfied UEs has been computed with 99% of frames received within the PDB both in FR1 and FR2 configurations as a function of the average cell load, which is measured in number of UEs. The dashed black line indicates the 90% limit on the number of satisfied UEs to measure the system capacity. Given the smaller PDB, we can observe that up to 5 UEs/cell running VR services can be supported both in DL in FR1 and FR2.

| (a) VR in FR1 at 30Mbps with X=99%. | (b) VR in FR2 at 30Mbps with X=99%. |
| --- | --- |
| (a) VR in FR1 at 45Mbps with X=99%. | (b) VR in FR2 at 45Mbps with X=99%. |

Figure 6 – DL capacity evaluation of VR traffic in FR1 and FR2 with X=99% of frames received within PDB and cells evenly loaded. The lower bound on the percentage of satisfied user is Y=90% (dashed line).

Observation 10: When we consider baseline VR traffic model for DL for Dense Urban deployment, we can support a number of UEs per cell indicated in Table 10.

Table 10 – System capacity in DL for VR/AR (PDB=10ms) at 30Mbps [#UEs/cell]

|  | Rate: 30Mbps | Rate: 45Mbps |
| --- | --- | --- |
| FR1 | 5 | 3 |
| FR2 | 5 | 3 |

# Power Saving Evaluation in Dense Urban

## Simulation settings

The same configuration of the Dense Urban deployment described in Section 5.1 has been considered for the evaluation of UE power consumption (please refer to Table 8 for a detailed description of main simulation parameters). Only DL traffic on CG and VR applications at 30Mbps has been considered in this analysis (baseline for CG and VR). Furthermore, the following three CDRX configurations have been considered to evaluate the trade-off between power saving and achievable capacity:

- CDRX 1: (DRX long cycle, inactivity timer value, On duration timer value)=(4, 2, 2)

- CDRX 2: (DRX long cycle, inactivity timer value, On duration timer value)=(8, 4, 4)

- CDRX 3: (DRX long cycle, inactivity timer value, On duration timer value)=(16, 8, 8)

## Simulation results

In the following, we illustrate the measured power saving and capacity in Dense Urban deployment when CDRX is enabled. The three proposed configurations are compared to the results illustrated in Section 5 (labeled “Always ON” in this section) in terms of average power saving gain and capacity loss.

### Cloud Gaming: DL Capacity vs Power Consumption

In this section, we illustrate the results on the system capacity obtained for CG application at 30Mbps, with PDB=15ms.  Error! Reference source not found. illustrates the absolute values of capacity and mean power consumption (across all UEs in the system) considering UEs always ON and the three CDRX configurations described above. Table 11 reports the average power gain and system capacity loss of the three considered CDRX configurations with respect to Always ON policy.

| (a) Capacity - Always ON. | (b) UE Mean Power - Always ON. |
| --- | --- |
| (c) Capacity - DRX (4,2,2). | (d) UE Mean Power - DRX (4,2,2). |
| (e) Capacity - DRX (8,4,4). | (f) UE Mean Power - DRX (8,4,4). |
| (g) Capacity - DRX (16,8,8). | (h) UE Mean Power - DRX (16,8,8). |

Figure 7 – Capacity and UE mean power consumption in Dense Urban in FR1 for CG at 30Mbps with X=99% (DL-only).

Table 11 – Evaluation of UE power saving schemes for {Dense Urban, CG, DL Only, 30Mbps, FR1}

| Power Saving Scheme | Power Saving Gain (PSG) compared to ‘Always ON’ |  |  |  | #satisfied UEs per cell with PS / #satisfied UEs per cell w/o PS |
| --- | --- | --- | --- | --- | --- |
|  | Baseline | Optional |  |  |  |
|  | Mean PS gain | PS gain of 5%-tile UE in PSG CDF | PS gain of 50%-tile UE in PSG CDF | PS gain of 95%-tile UE in PSG CDF |  |
| Always ON | - | - | - | - | 7 / 7 |
| (4, 2, 2) | 19 % | 22 % | 19 % | 13 % | 5 / 7 |
| (8, 4, 4) | 28 % | 31 % | 29 % | 20 % | 6 / 7 |
| (16, 8, 8) | 31 % | 32 % | 32 % | 25 % | 6 / 7 |

Observation 11: Power saving gain and DL capacity loss with different CDRX configurations for CG applications in Dense Urban deployment in FR1 are indicated in Table 10.

Observation 12: CDRX reduces UE power consumption by up to 32% but it also affects the DL system capacity for CG applications, reducing it by 1 or 2 UEs/cell depending on the DRX configuration.

Observation 13: The CDRX configuration that archives the best trade-off between power saving gain and DL capacity loss for CG applications in Dense Urban deployment in FR1 is (DRX long cycle, inactivity timer value, On duration timer value)=(16, 8, 8).

### Virtual Reality: DL Capacity vs Power Consumption

In this section, we illustrate the results on the system capacity and power consumption obtained for VR application at 30Mbps, with PDB=10ms. Figure 8 illustrates the absolute values of capacity and mean power consumption (across all UEs in the system) considering UEs always ON and the three CDRX configurations described above. Table 12 reports the average power gain and system capacity loss of the three considered CDRX configurations with respect to Always ON policy.

| (a) Capacity - Always ON. | (b) UE Mean Power - Always ON. |
| --- | --- |
| (c) Capacity - DRX (4,2,2). | (d) UE Mean Power - DRX (4,2,2). |
| (e) Capacity - DRX (8,4,4). | (f) UE Mean Power - DRX (8,4,4). |
| (g) Capacity - DRX (16,8,8). | (h) UE Mean Power - DRX (16,8,8). |

Figure 8 – Capacity and UE mean power consumption in Dense Urban in FR1 for VR at 30Mbps with X=99% (DL-only).

Table 12 – Evaluation of UE power saving schemes for {Dense Urban, VR, DL Only, 30Mbps, FR1}

| Power Saving Scheme | Power Saving Gain (PSG) compared to ‘Always ON’ |  |  |  | #satisfied UEs per cell with PS / #satisfied UEs per cell w/o PS |
| --- | --- | --- | --- | --- | --- |
|  | Baseline | Optional |  |  |  |
|  | Mean PS gain | PS gain of 5%-tile UE in PSG CDF | PS gain of 50%-tile UE in PSG CDF | PS gain of 95%-tile UE in PSG CDF |  |
| Always ON | - | - | - | - | 5 / 5 |
| (4, 2, 2) | 19 % | 22 % | 19 % | 13 % | 0 / 5 |
| (8, 4, 4) | 28 % | 31 % | 29 % | 20 % | 3 / 5 |
| (16, 8, 8) | 31 % | 32 % | 32 % | 25 % | 4 / 5 |

Observation 14: Power saving gain and DL capacity loss with different CDRX configurations for VR applications in Dense Urban deployment in FR1 are indicated in Table 12.

Observation 15: CDRX reduces UE power consumption by up to 32% but it also strongly affects the DL system capacity for VR applications, reducing it to 0 in the worst case due to the low delay budget with respect to the simulated retransmission delay.

Observation 16: The CDRX configuration that archives the best trade-off between power saving gain and DL capacity loss for VR applications in Dense Urban deployment in FR1 is (DRX long cycle, inactivity timer value, On duration timer value)=(16, 8, 8).

# Summary

In this contribution we have discussed performance results in indoor and outdoor deployments and bottlenecks that could be improved to increase 5G NR system capacity and UE power saving. Those naturally require further studies and discussions. In summary, following observations have been made for system capacity evaluation:

The following observations have been made for system capacity evaluation in Indoor Hotspot:

Observation 1: When we consider CG traffic model at 30Mbps for an InH deployment, we can support the number of UEs per cell indicated in Table 4.

Observation 2: When we consider VR traffic model at 30Mbps for an InH deployment, we can support the number of UEs per cell indicated in Table 5.

The following observations have been made for UE  power saving evaluation in Indoor Hotspot:

Observation 3: Power saving gain and DL capacity loss with different CDRX configurations for CG applications in Indoor Hotspot deployment in FR1 are indicated in Table 6.

Observation 4: CDRX reduces UE power consumption by up to 74% but it also affects the DL system capacity for CG applications, reducing it by 1 UEs/cell for all considered DRX configurations.

Observation 5: The CDRX configuration that achieves the best trade-off between power saving gain and DL capacity loss for CG applications in Indoor Hotspot deployment in FR1 is (DRX long cycle, inactivity timer value, On duration timer value)=(4, 2, 2).

Observation 6: Power saving gain and DL capacity loss with different CDRX configurations for VR applications in Indoor Hotspot deployment in FR1 are indicated in Table 7.

Observation 7: CDRX reduces UE power consumption by up to 75% but it also strongly affects the DL system capacity for VR applications, reducing it to 0 in the worst case due to the low delay budget with respect to the simulated retransmission delay.

Observation 8: The CDRX configuration that archives the best trade-off between power saving gain and DL capacity loss for VR applications in Indoor Hotspot deployment in FR1 is (DRX long cycle, inactivity timer value, On duration timer value)=(4, 2, 2).

The following observations have been made for system capacity evaluation in Dense Urban:

Observation 9: When we consider baseline CG traffic model for DL for Dense Urban deployment, we can support a number of UEs per cell indicated in Table 9.

Observation 10: When we consider baseline VR traffic model for DL for Dense Urban deployment, we can support a number of UEs per cell indicated in Table 10.

The following observations have been made for UE  power saving evaluation in Dense Urban:

Observation 11: Power saving gain and DL capacity loss with different CDRX configurations for CG applications in Dense Urban deployment in FR1 are indicated in Table 10.

Observation 12: CDRX reduces UE power consumption by up to 32% but it also affects the DL system capacity for CG applications, reducing it by 1 or 2 UEs/cell depending on the DRX configuration.

Observation 13: The CDRX configuration that archives the best trade-off between power saving gain and DL capacity loss for CG applications in Dense Urban deployment in FR1 is (DRX long cycle, inactivity timer value, On duration timer value)=(16, 8, 8).

Observation 14: Power saving gain and DL capacity loss with different CDRX configurations for VR applications in Dense Urban deployment in FR1 are indicated in Table 12.

Observation 15: CDRX reduces UE power consumption by up to 32% but it also strongly affects the DL system capacity for VR applications, reducing it to 0 in the worst case due to the low delay budget with respect to the simulated retransmission delay.

Observation 16: The CDRX configuration that archives the best trade-off between power saving gain and DL capacity loss for VR applications in Dense Urban deployment in FR1 is (DRX long cycle, inactivity timer value, On duration timer value)=(16, 8, 8).

All previous observations on system capacity and UE power saving gains are captured and summarized by the following tables:

|  |  | Cloud Gaming |  | Virtual/Augmented Reality |  |
| --- | --- | --- | --- | --- | --- |
| Deployment | Frequency Range | Rate: 30Mbps | Rate: 8Mbps | Rate: 30Mbps | Rate: 45Mbps |
| Indoor Hotspot | FR1 | 4 | ≥ 8 | 3 | 2 |
|  | FR2 | ≥8 | ≥ 8 | ≥8 | 6 |
| Dense Urban | FR1 | 7 | ≥ 26 | 5 | 3 |
|  | FR2 | 8 | ≥ 26 | 5 | 3 |

|  |  | Cloud Gaming |  | Virtual/Augmented Reality |  |
| --- | --- | --- | --- | --- | --- |
| Deployment | PS Scheme | Mean PS gain | Capacity | Mean PS gain | Capacity |
| Indoor Hotspot (FR1) | Always ON | - | 4 / 4 | - | 3/ 3 |
|  | CDRX (4, 2, 2) | 73 % | 3 / 4 | 73 % | 3 / 3 |
|  | CDRX (8, 4, 4) | 66 % | 3 / 4 | 66 % | 2 / 3 |
|  | CDRX (16, 8, 8) | 50 % | 3 / 4 | 50 % | 0 / 3 |
| Dense Urban (FR1) | Always ON | - | 7 / 7 | - | 5 / 5 |
|  | CDRX (4, 2, 2) | 19 % | 5 / 7 | 19 % | 0 / 5 |
|  | CDRX (8, 4, 4) | 28 % | 6 / 7 | 28 % | 3 / 5 |
|  | CDRX (16, 8, 8) | 31 % | 6 / 7 | 31 % | 4 / 5 |

# References

- RP-210460 Revised SID on XR Evaluations for NR, RAN#91

- R1-2009812 FL Summary of RAN1 103-e agreements and email discussions on Rel-17 SI on XR evaluations for NR

- RAN1 Chairman’s Notes (v018) 3GPP TSG RAN WG1 Meeting #104-e e-Meeting, January 25th – February 5th, 2021

- R1-2101866 Summary of [104-e-NR-XR-02]_Other Eval Methodology.

- Chair notes RAN1, 104-bis-e Chair_notes_104b-e