3GPP TSG RAN WG1 Meeting #105-e  	R1-2104569

e-Meeting, May 10th – 27th, 2021

Agenda Item: 8.15.3

Source: MediaTek Inc.

Title: Timing relationship enhancements for IoT-IoT NTN

Document for: Discussion and Decision

# Introduction

At the RAN#86 meeting, a new Study Item was approved for IoT Non Terrestrial Network (NTN) and revised in RAN#91 [1]. There was an email discussion on [91E][42][NTN_IoT_Roadmap] In RAN#91 with moderator summary and final proposal for GTW input in [2].

In RAN#91-e GTW session, the Chairman endorsed a Way Forward Proposal in [3] on email discussion on [50][New_proposals_approval]. This included guidance from RAN Chairman for NTN NR and NTN IoT as follows

- RAN#92E (June) to finalize the scope and project plan to deliver the essential minimum functionality of both NTN NR and NTN IoT (both NB-IoT and eMTC) within the existing TU allocations

- Detailed scoping exercise (NTN NR WID revision, NTN IoT WID approval) to be undertaken at RAN#92E (June)

# Timing relationships requiring enhancement

This section addresses Section 2.1 Timing relationships requiring enhancements in FL Summary [4].  RAN1#104bis-e made the following agreements for IoT NTN:

The following NB-IoT timing relationships need enhancing for essential minimum functionality of IoT NTN:

- NPDCCH to NPUSCH format 1

- RAR grant to NPUSCH format 1

- NPDSCH to HARQ-ACK on NPUSCH format 2

- Timing advance command activation

- FFS: NPDCCH order to NPRACH

- FFS: Other NB-IoT timing relationships

The enhancement based on extending the timing relationship, by e.g. Koffset, adopted in NR NTN should be the starting point for enhancement of NB-IoT timing relationships in IoT NTN. Details can be further discussed considering IoT NTN.

We discuss timing enhancements case by case for NB-IoT below:

NPDCCH to NPUSCH format 1:

This case is illustrated in Figure 1. A UE needs to get UL grant in DCI Format N0 on NPDCCH at time n for data transmission on NPUSCH Format 2 at time n + ko , but needs to advance its UL transmission before receiving NPDCCH at time n. eNB is not aware of UE estimated delay which needs to be taken into when scheduling UL grant to avoid collisions between UL transmissions of UEs experiencing different RTDs. As in NR NTN, a solution is to introduce UL scheduling K_offset. For the transmission timing of DCI scheduled NPUSCH Format N0, the slot allocated for the NPUSCH can be modified to be n+ko+Koffset.

Proposal 1: For NB-IoT, on receiving UL grant on DCI format N0 in slot n, NPUSCH Format 1 is transmitted in subframe  n+k0+K_offset.

Figure 1: UL Grant for NPUSCH Format 1

RAR grant to NPUSCH format 1:

This case is illustrated in Figure 3. UE needs to get UL grant in RAR at time n for MSG3 transmission on NPUSCH Format 2 at time n + ko, but needs to advance its UL transmission before receiving RAR at time n. eNB is not aware of UE estimated delay which needs to be taken into when scheduling UL grant in RAR for to avoid collisions between UL transmissions of UEs experiencing different RTDs. A solution is to introduce UL scheduling offset Koffset. For the transmission timing of RAR grant scheduled NPUSCH Format 1, the UE transmits the NPUSCH format 1  in slot n+ko+Koffset.

Proposal 2: For NB-IoT, on receiving a NPDSCH with a RAR message in slot n, message 3 is transmitted on NPUSCH format 1in subframe  n+k0+K_offset.

Figure 3: UL Grant for RAR on NPUSCH Format 1

NPDSCH to HARQ-ACK on NPUSCH format 2:

This case is illustrated in Figure 2. A UE needs to report UL HARQ feedback on NPUSCH Format 2 at time n + ko as indicated in DCI Format N1 on NPDCCH at time n, but needs to advance its UL transmission before receiving NPDCCH at time n. eNB is not aware of UE estimated delay which needs to be taken into when scheduling NPUSCH Format 2 to avoid collisions between UL transmissions of UEs experiencing different RTDs. A solution is to introduce UL scheduling offset K_offset. For the transmission timing of HARQ-ACK on NPUSH Format 2, the UE provides corresponding HARQ-ACK information in a NPUSH transmission within slot n+k0+Koffset.

Proposal 3: For NB-IoT, on receiving DL assignment on DCI format N1 in slot n, HARQ-ACK on NPUSH Format 2 is transmitted in subframe  n+k0+K_offset.

Figure 2: UL HARQ feedback on NPUSCH Format 2

NPDCCH / MPDCCH order to NPRACH:

This case is still under discussion in NR NTN WI. The main issue still for discussion is whether it is acceptable to have blind detection in gNB. The maximum differential delay within a cell is 10.3 ms assuming max foot print size (edge to edge) regardless of the elevation angle of 3500 km (TR 38.821 Table 4.2-2). The eNB can then assume that the maximum ambiguity for the next RO would be upper bounded by 10.3 ms assuming the common TA and further assuming the RTT to the nearest beam edge. It is up to the gNB to configure ROs for NPDCCH / MPDCCH order RACH. If eNB configure ROs once every 10 ms in normal coverage, and longer in deep coverage or extreme coverage, the ambiguity can be removed all together; if it configure ROs once every 5 ms in normal coverage, then two blind decoding attempts may be needed. Hence, it can be left to the network to configure NPDCCH / MPDCCH ordered RACH resources to mitigate blind detection attempts at the gNB.

Proposal 4: For NB-IoT / eMTC, blind detection of NPDCCH / MPDCCH ordered RACH is supported without new enhancements.

NPDCCH / MPDCCH timing advance command activation:

This case is still under discussion in NR NTN WI. The main issue is whether to prioritize NTN designs that support systems where DL and UL are aligned at the gNB. What needs to be discussed is the common TA which is not constant for DL-UL subframe timing alignment at the gNB. The UE will need to get indication on common TA in NTN SIB at time n  and predict and pre-compensate the change in common TA for UL transmission at time n+K. This discussed in our contribution in AI 8.4.2 on “Enhancements on UL Time and Frequency Synchronisation for NR-NTN”  [4].

Proposal 5: For NB-IoT / eMTC, blind detection of NPDCCH / NPDCCH ordered RACH is supported without new enhancements.

# Preamble retransmission

This section addresses Section 2.5 Preamble re-transmission in FL Summary [4].  RAN1#104bis-e made the following agreements

For NB-IoT over NTN, the following timing relationship needs to be studied to check whether enhancement is necessary and beneficial:

- PRACH preamble retransmission

For eMTC over NTN, the following timing relationship needs to be studied to check whether enhancement is necessary and beneficial:

- PRACH preamble retransmission

In TS 36.213, Section 16.3.2 mentions that for NB-IoT:

b) If a random access response is received and the corresponding DL-SCH transport block ending in subframe n does not contain a response to the transmitted preamble sequence, the UE shall, if requested by higher layers, be ready to transmit a new preamble sequence no later than the NB-IoT UL slot starting 12 milliseconds after the end of subframe n.

c) If no NPDCCH scheduling random access response is received in subframe n, where subframe n is the last subframe of the random access response window, the UE shall, if requested by higher layers, be ready to transmit a new preamble sequence no later than the NB-IoT UL slot starting 12 milliseconds after the end of subframe n.

d) If an NPDCCH scheduling random access response with associated RA-RNTI is detected and the corresponding DL-SCH transport block reception ending in subframe n cannot be successfully decoded, the UE shall, if requested by higher layers, be ready to transmit a new preamble sequence no later than the NB-IoT UL slot starting 12 milliseconds after the end of subframe n.

Hence, to our understanding with start of RAR window offet to subframe n, then legacy specifications can be re-used for RACH preamble transmissions without need for enhancements. We do not see an issue with RACH preamble re-transmission assuming that the start of the RAR window is offset by UE-sat-gNB RTT as per RAN2#113bis-e agreement in NR NTN.

Proposal 6: Release 15 RACH preamble re-transmission is used for NTN with the start of the RAR window offset by UE-sat-gNB RTT.

# GNSS measurements

This section addresses Section 2.7 GNSS measurements in FL Summary [4].  RAN1#104e made the following agreements for IoT NTN:

AI 8.15.3 for Timing relationship enhancements

- Study the impact on any timing relationships for IoT-NTN due to the need to perform GNSS measurements for time and frequency synchronization.

AI 8.15.2. for Enhancements on Time and frequency synchronization

- Discuss whether GNSS measurement window is needed and beneficial for initial access.

A main difference with NR NTN, is that in IoT NTN SID it is stated that “simultaneous GNSS and NTN NB-IoT/eMTC operation is not assumed” [1].

In idle mode, a maximum eDRX of 43.69 min for eMTC and 2.91 hours for NB-IoT can be configured, where eDRX cycle consist of an integral multiple of length of a single H-SFN. The minimum eDRX cycle is 5.12 s for eMTC and 20.48 s for NB-IoT [Table 10.5.5.32, 6].    .

In connected mode, a maximum DRX of 2.56 s and a maximum eDRX of 10.24s can be configured in MAC-MainConfig information element. Rel-12 Power Saving Mode (PSM) with a maximum of 12.1 days can be configured with T3412 configuration.

| Connected UE | Max DRX=2.56 s / eDRX = 10.24 s |
| --- | --- |
| Idle UE | Min eDRX = 5.12 s (eMTC) Min eDRX = 20.48 s (NB-IoT) |
|  | Max eDRX = 43.69 min (eMTC) Max eDRX = 2.91 hours (NB-IoT) Max PSM = 12.1 days (NB-IoT) |

Table 1:Connect DRX and idle DRX / PSM

It is sufficient to re-use legacy procedures for UE acquisition of GNSS position fix assuming simultaneous GNSS and NTN NB-IoT/eMTC operation is not used in the device

- Re-use  legacy paging procedure with adequate configuration of paging timer for mobile-terminated calls

- For idle UE, GNSS measurement for position fix can be done in idle DRX / eDRX / PSM with a GNSS TTFF with hot start or warm start before moving to connected for mobile-originated calls

- For connected UE, GNSS measurement for position fix can be done in connected DRX / eDRX with a GNSS TTFF with hot start.

In Typical IoT applications with intermittent delay-tolerant small packets, the UE position may be fixed (e.g. IoT sensor on a gas pipeline) or the UE position needs to be reported by application layer (e.g. asset tracking, vehicle tracking). A UE may only need a new GNSS position solely for UE pre-compensation for UL synchronization in corner case scenarios where (i) it is not fixed; (ii) reporting of the GNSS position is not needed by application layer.

It is up to UE implementation when to switch on its GNSS module to acquire its position with a GNSS TTFF with a hot start or warm start when a new position fix is needed during idle DRX / eDRX / PSM before moving to connected for mobile-terminated calls and for mobile-originated calls. For connected UE, GNSS measurement for position fix can be done in connected DRX / eDRX with a GNSS TTFF with hot start.

Observation 1: With implementation for GNSS measurements re-suing paging and DRX procedures, timing relationships defined for idle DRX / eDRX / PSM and for connected DRX / eDRX can be re-used.

Proposal 7: Re-use timing relationships for legacy paging and DRX procedures for UE acquisition of GNSS position fix assuming simultaneous GNSS and NTN NB-IoT/eMTC operation is not used in the device

# Timing offset for the start of RAR window

This section addresses Section 2.8 Timing offset for the start of RAR window in FL Summary [4].  The following feature lead recommendation was made in RAN1#104bis-e:

- Study the value for the RAR window offset and how it is determined both at UE and eNB for discussion at next meeting.

RAN2#113bis-e has made the following agreement on IoT NTN:

Enhancements to ra-ResponseWindow and mac-ContentionResolutionTimer are essential. R2 assume that design can follow NR NTN agreements as baseline.

RAN2#113bis-e has made the following agreement on NR NTN:

- RAN2 wait for RAN1’s progress and postpone the discussion on how to broadcast parameters, if any, for TA pre-compensation.

- RAN2 send an LS to RAN1, focusing on below aspects:

- Ask RAN1 to prioritize the TA pre-compensation work on whether and/or what parameters to broadcast for TA pre-compensation, and when broadcasted, how often the broadcasted parameters are expected to change over time;

- RAN2 has agreed to use UE-gNB RTT as the offset to start some UP timers (e.g. drx-HARQ-RTT-TimerDL). Ask RAN1 to provide inputs on (i) how UE acquires UE-gNB RTT and (ii) what additional information needs to be broadcasted other than that for TA pre-compensation, if any.

Based on RAN2#11bis-e agreements, it is our understanding that we should wait for NR NTN agreements on enhancements to ra-ResponseWindow and mac-ContentionResolutionTimer in NR NTN. This includes use UE-gNB RTT as the offset to start some UP timers, and (i) how the UE acquires UE-gNB RTT and (ii) what additional information needs to be broadcasted other than that for TA pre-compensation, if any.

Proposal 8: RAN1 wait for NR NTN agreements on enhancements to ra-ResponseWindow and mac-ContentionResolutionTimer in NR NTN.

# Conclusion

In this contribution, we summarize issues and discuss impact on specifications for solutions for NR timing relationships.

Proposal 1: For NB-IoT, on receiving UL grant on DCI format N0 in slot n, NPUSCH Format 1 is transmitted in subframe  n+k0+K_offset.

Proposal 2: For NB-IoT, on receiving a NPDSCH with a RAR message in slot n, message 3 is transmitted on NPUSCH format 1in subframe  n+k0+K_offset.

Proposal 3: For NB-IoT, on receiving DL assignment on DCI format N1 in slot n, HARQ-ACK on NPUSH Format 2 is transmitted in subframe  n+k0+K_offset.

Proposal 4: For NB-IoT / eMTC, blind detection of NPDCCH / MPDCCH ordered RACH is supported without new enhancements.

Proposal 5: For NB-IoT / eMTC, blind detection of NPDCCH ordered RACH is supported without new enhancements.

Proposal 6: Release 15 RACH preamble re-transmission is used for NTN with the start of the RAR window offset by UE-sat-gNB RTT.

Observation 1: With implementation for GNSS measurements re-suing paging and DRX procedures, timing relationships defined for idle DRX / eDRX / PSM and for connected DRX / eDRX can be re-used.

Proposal 7: Re-use timing relationships for legacy paging and DRX procedures for UE acquisition of GNSS position fix assuming simultaneous GNSS and NTN NB-IoT/eMTC operation is not used in the device

Proposal 8: RAN1 wait for NR NTN agreements on enhancements to ra-ResponseWindow and mac-ContentionResolutionTimer in NR NTN.

# References

- RP-210868, “New Study WID on NB-IoT/eTMC support for NTN”, MediaTek, RAN#91-e, March 2021

- RP-210915, “Moderator's summary for email discussion [91E][42][NTN_IoT_roadmap]”, Ericsson (RAN1 Vice-Chair)RAN#91-e, March 2021

- RP-210906, Way forward on new proposals, Nokia (RAN Chair), RAN#91-e, March 2021

- R1-2104083, Moderator (Sony), FL summary#4 of AI 8.15.3, RAN1#104bis-e, April 2021