3GPP TSG RAN WG1 #105-e		R1-2104752

e-Meeting, May 10th – 27th, 2021

Source:	OPPO

Title:	Remaining open issues and corrections for physical layer procedure

Agenda Item:	7.2.4

Document for:	Discussion and Decision

Introduction

This is to discuss how to determine the transmission timing of PSFCH in case the TX timing and RX timing are not synchronized from the PSFCH transmitters (i.e. PSSCH receiver) perspective.

Discussion

According to sync procedure of R16 NR-V2X, specified in TS 38.331 section 5.8.6, UE derives transmission timing based on the sync source: gNB/eNB, GNSS or UE. In the following 2 cases, UE1 and UE2 have asynchronized timing.

- Case 1: gNB as sync source, and gNB1/gNB2 are not time aligned.

- Case 2: UE as sync source, and UE3 and UE4 are not time aligned.

- Observation 1: Based on the R16 NR-V2X sync procedure, it is possible that two UEs communicating via PC5 adopt asynchronized sync sources.

(a) Case 1

(b) Case 2

Figure 1 Illustration of asynchronization between UEs with SL communication

In LTE SL or NR SL, gNB/eNB can configure either gNB/eNB or GNSS has higher priority. For case 1, if there is asynchronization between gNB1 and gNB2, gNB can configure GNSS has higher priority so that all UEs (including UE1 and UE2) are time aligned. While it is not applicable to case 2, since the sync source (UE3 and UE4) is based on internal clock and they are not time aligned.

Observation 2: If independent UE is taken as sync source, asynchronization among UEs cannot be solved by configuration.

For the PSCCH/PSSCH communication, even UE1 and UE2 are not time aligned, they can communicate with each other based on reception of SLSS. For example, from UE1 perspective

- For the PSCCH/PSSCH transmission from UE1 to UE2, it uses UE1 Tx pool and follows the timing derived from UE1’s sync source (i.e., UE3).

- For the PSCCH/PSSCH transmission from UE2 to UE1, it uses UE1 Rx pool and follows the timing derived from UE2 (i.e., based on SLSS from UE2).

The two pools are not necessarily sync-ed with each other, so even if the sync source of two UEs are asynchronized,  they can communicate with each other and there is no need to align the sync of the two UEs in a PC5 link.

While for NR, since there is PHY feedback introduced, i.e., PSFCH, there seems a need to align the sync of the two UEs in a PC5 link, i.e. from UE1 perspective .

- For the PSCCH/PSSCH transmission from UE1 to UE2, it uses UE1 Tx pool and follows the timing derived from UE1’s sync source (i.e., UE3).

- For the PSFCH transmission from UE2 to UE1, it uses UE1 Tx pool;

Which sync timing is used for UE1 to receive PSFCH from UE2, sync-1(its own sync source, i.e., UE3) or sync-2 (SLSS from UE2), in case there is timing difference between sync-1 and sync-2?

- If sync-2 is used for PSFCH reception by UE1, the time/frequency resources of UE1 Tx pool is not aligned with sync-2;

- If sync-1 is used for PSFCH reception by UE1, that requires UE2 to perform PSFCH transmission in a time/frequency resource which is not aligned with its Tx sync (i.e., UE4)

- Observation 3: In case of asynchronization between TX UE and RX UE, that will cause issue for PSFCH transmission/reception.

How to determine the transmission timing for PSFCH in case of asynchronization is not explicitly discussed in RAN1 till now. Considering the above case, some clarification or confirmation of PSFCH transmission timing is needed.

Observation 4:  Clarification or confirmation of PSFCH transmission timing is needed in RAN1.

There is the following description in TS 38.331

5.8.6.1	General

The purpose of this procedure is to select a synchronisation reference and used when transmitting NR sidelink communication.

Considering that at current stage, reuse existing sync mechanism is more preferred than introducing new behavior to address above issue. Therefore, we suggest to confirm that the sync procedure specified in TS 38.331 section 5.8.6 is also applicable to PSFCH transmission.

Proposal 1: The sync procedure specified in TS 38.331 section 5.8.6 is applicable to PSFCH transmission.

Conclusion

How to determine the transmission timing of PSFCH in case of asynchronization between unicast pairing UEs is discussed. The following observations and proposal are given.

- Observation 1: Based on the R16 NR-V2X sync procedure, it is possible that two UEs communicating via PC5 adopt asynchronized sync sources.

Observation 2: If independent UE is taken as sync source, asynchronization among UEs cannot be solved by configuration.

- Observation 3: In case of asynchronization between TX UE and RX UE, that will cause issue for PSFCH transmission/reception.

Observation 4:  Clarification or confirmation of PSFCH transmission timing is needed in RAN1.

Proposal 1: The sync procedure specified in TS 38.331 section 5.8.6 is applicable to PSFCH transmission.