3GPP TSG RAN WG1 #105-e			 R1-2105208

e-Meeting, May 10th – 27th, 2021

Agenda Item:	8.4.1

Source: 	ITL

Title: 	Timing relationship enhancements for NTN

Document for:	Discussion and decision

Introduction

In RAN1#104bis-e meeting, RAN1 made the following agreements related to the timing relationship enhancements for NTN[1].

Agreement: For updating K_offset after initial access, at least one of the following options is supported: - Option 1: RRC reconfiguration - Option 2: MAC CE FFS: Other options Agreement: - For determination of cell-specific K_offset in system information, down-select one option from below: - Option 1: Signal one offset value for K_offset - Note: For example, the value is expected to cover the RTT of service link plus the RTT between serving satellite and reference point - Option 2: Signal a first offset value and a second offset value. K_offset is equal to the sum of the two offset values - Note: For example, the first offset value is expected to cover the RTT between serving satellite and reference point or is determined by common TA, and the second offset value is expected to cover RTT of service link Agreement: Confirm the following working assumption: Introduce K_offset to enhance the adjustment of uplink transmission timing upon the reception of a corresponding timing advance command. Agreement: When UE is not provided with K_offset value other than the one signaled in system information, the K_offset value signaled in system information is used for all timing relationships that require K_offset enhancement. Agreement: UE can be provided by network with a K_mac value. - When UE is not provided by network with a K_mac value, UE assumes K_mac = 0.

In this contribution, we further discuss technical issues of the timing relation enhancements based on the conclusions from previous RAN1 meeting.

Discussion

K_offset value after initial access

For the K_offset after initial access, it may require additional signaling for gNB and UE to update their timing relationship, so that more exact timing relationship should be tracked between UE and gNB for NTN. Further update of K_offset value can be UE-specific or even beam-specific as proposed by several companies. For the update of K_offset value after initial access, two options have been mainly considered as shown in the agreements of previous RAN1 meeting. There are obviously pros and cons for each of the options. It seems Option 1 (i.e. RRC reconfiguration) is natural way and stable option as usual that is, both gNB and UE would have same understanding on the timing relation based on the RRC reconfiguration procedure. It can cover UE-gNB RTT with K1/K2. Given that scheduling timing information (e.g. K1 or K2) are provided by a DCI on top of the K_offset value, fine adjustment of the timing relationship may be available, which may lead to less RRC reconfigurations. As a price it may cause longer latency than option 2.

Meanwhile, Option 2 is to use MAC CE signaling different from Option 1 relying on RRC reconfiguration. faster update of the K_offset value is possible when either UE or satellite is moving fast and channel conditions is thus frequently changed. Also, since it may be derived from the common timing offset (Common TA), signaling overhead would be saved.

In summary, we think option 1 may easily handle most of cases even with the worst case where the NTN channel condition is frequently changed by NW implementation and proper K_offset configuration with scheduling flexibility (K1/K2). Also option 1 would much simpler way than option 2 that use separate parameters (e.g. common TA and UE-calculated TA).

Proposal 1. It is preferred to adopt RRC reconfiguration for updating K_offset value after initial access.

For K_offset update after initial access procedure, there are two ways in our considerations, i.e. gNB controlled and UE initiated. In some cases, UE may have TA information or RTT-related timing information which means that UE may decide to update the K_offset value depending on those timing factors. Then, UE can report the necessity of K_offset update to the gNB so that the gNB can update K_offset value by using RRC reconfiguration as discussed in above. Alternatively, the gNB can configure the new updated K_offset value based on the satellite velocity, ephemeris and location.

Proposal 2. gNB controlled and UE-initiated mechanisms can be supported for update of K_offset value.

For the applicability of K_offset value, it has been considered with at least two different K_offset values from either SIB or UE-specific signaling. From system perspective, it needs to be clear which one of K_offset values are assumed per UE, in order to avoid potential collision of UL transmission. So, in previous RAN1 meeting, following agreement was made:

Agreement in RAN1#104bis-e: When UE is not provided with K_offset value other than the one signaled in system information, the K_offset value signaled in system information is used for all timing relationships that require K_offset enhancement.

In addition, how to handle remaining case where UE is provided with K_offset value other than the one signaled in system information needs to be clarified. According to the discussion in last RAN1 meeting, we think that when the updated K_offset value is provided to a UE, the updated K_offset value is used for following cases:

  - The transmission timing of DCI scheduled PUSCH (including CSI on PUSCH).

  - The transmission timing of HARQ-ACK on PUCCH.

  - The CSI reference resource timing.

  - The transmission timing of aperiodic SRS.

  - The first transmission opportunity of PUSCH in Configured Grant Type 2

  - MAC CE activation timing

The K_offset value signaled in system information is used for

  - HARQ-ACK on PUCCH to MsgB/Msg4

  - RAR or fallbackRAR grant scheduled PUSCH

Proposal 3. When UE is provided with K_offset value other than the one signaled in system information,

  - for UL transmission related to random access procedure, K_offset value signaled in system information is used regardless of RRC states while for other purposes, K_offset value signaled in system information is used until new updated K_offset value is provided by network.

Determination of K_offset

In RAN1#104bis-e, following agreement is made for determination of cell-specific K_offset in system information:

Agreement: - For determination of cell-specific K_offset in system information, down-select one option from below: - Option 1: Signal one offset value for K_offset - Note: For example, the value is expected to cover the RTT of service link plus the RTT between serving satellite and reference point - Option 2: Signal a first offset value and a second offset value. K_offset is equal to the sum of the two offset values - Note: For example, the first offset value is expected to cover the RTT between serving satellite and reference point or is determined by common TA, and the second offset value is expected to cover RTT of service link

Regarding determination of K_offset value, it seems to us that option 1 is much simple way than option 2 that require some association with like common TA for first offset value as seen in above agreement. We fail to find strong justification to adopt option 2 even if it can save signaling overhead. Also, if option 2 is adopted, it is concerned that it would make some relation with TA discussion discussed in section 8.4.2 as there seems no motivation for this function with separate parameters. So, option 1 can be simply adopted for this function and easily address the UE-gNB RTT as sum of feeder link and service link by signaling proper K_offset value.

Proposal 4. Single one offset value for K_offset can be adopted.

Beam-specific K_offset configuration for initial access procedure

In RAN1#103-e meeting, it was agreed to introduce at least cell-specific K_offset configuration used in initial access. As remaining discussion, it needs to be clarified whether beam specific K_offset configured in system information should be also supported in initial access procedure. We think it is beneficial to configure beam-specific K_offset during initial access especially for reducing latency. However, it would introduce complexity of specification (e.g. when to use cell specific K_offset or beam-specific K_offset) and higher signaling overhead (e.g. multiple values of beam-specific K_offset in addition to cell-specific K_offset value). Considering those factors, cell-specific K_offset is sufficient in initial access.

Proposal 5. Cell-specific K_offset is only supported in initial access procedure.

On K1 range extension

In RAN1#104bis-e, following aspects on the functionality of K1 range extension was discussed to complete K1 range extension for NTN:

1) Extension to FDD

2) Relevant RRC parameter

3) DCI impact

Regarding 1), there was an agreement to extend the K1 range from 0~15 to 0~31 for TDD. Accordingly one remaining question will be whether it is also applied to FDD. In our understanding, it seems there would be no motivation to consider it in FDD as well. So, it is not desirable to extend the K1 range for FDD.

For 2), we think this issue needs to be discussed in RAN2 since the determination of relevant RRC parameters and signaling structure are under RAN2 work.

For 3), it is basically preferred not to change the DCI size that will result in additional complexity and specification works, since it is sufficient to use the current number of bits in the DCI field for K1 value range while the extended value range configured by new RRC parameter can be configured.

Proposal 6. Followings on K1 range extension issue are proposed:

- It is not supported to extend the K1 range for FDD

- DCI field range related to the K1 range extension should not be increased.

RAR window starting time

In NR TN, the RAR window starts at the first symbol of the earliest CORESET, that is at least one symbol, after the last symbol of the PRACH occasion corresponding to the PRACH transmission in case of 4-step RA or after the last symbol of PUSCH occasion corresponding to the PRACH transmission.

In NR NTN, it is clear that the large TA value should be considered in determining the RAR window starting time. Due to the large RTT between gNB and UE, UE specific RTT has been mainly considered to determine the start of PDCCH monitoring for RAR window, which seems be aligned with the RAN2 working assumption that ra-ResponseWindow and msgB-ResponseWindow are accurately compensated by UE-gNB RTT. Moreover, it has been discussed that two cases where DL and UL frame timing are aligned at gNB and not aligned at gNB may have different assumption in determining the UE specific RTT. The case where DL and UL frame timing is not aligned at gNB seems to need more handling and signaling in order to help UE to derive the RAR reception timing. For example, without timing information related to the feeder link RTT at a UE side, the UE may not accurately derive the UE-specific RTT when determining the RAR window starting time. Hence, the UE may need to know the feeder link RTT for the RAR reception timing.

In addition, it seems there may be possible dependence between K_offset value and the UE-specific RTT. If a gNB indicates the K_offset, it may be used for deriving the UE-specific RTT (i.e. UE-gNB RTT) although how the K_offset is determined is not completed yet. Hence, depending on the conclusion of the K_offset determination issue, the UE-specific RTT may be derived from K_offset value.

Proposal 7. The RAR window starting time is determined based on UE-specific RTT. Also, consider how the K_offset is used for deriving the UE-specific RTT.

Conclusion

In this contribution, we focus on technical design issues related to timing relationship and enhancements. Observations and proposals are as followings:

Proposal 1. It is preferred to adopt RRC reconfiguration for updating K_offset value after initial access.

Proposal 2. gNB controlled and UE-initiated mechanisms can be supported for update of K_offset value.

Proposal 3. When UE is provided with K_offset value other than the one signaled in system information,

  - for UL transmission related to random access procedure, K_offset value signaled in system information is used regardless of RRC states while for other purposes, K_offset value signaled in system information is used until new updated K_offset value is provided by network.

Proposal 4. Single one offset value for K_offset can be adopted.

Proposal 5. Cell-specific K_offset is only supported in initial access procedure.

Proposal 6. Followings on K1 range extension issue are proposed:

- It is not supported to extend the K1 range for FDD

- DCI field range related to the K1 range extension should not be increased.

Proposal 7. The RAR window starting time is determined based on UE-specific RTT. Also, consider how the K_offset is used for deriving the UE-specific RTT.

Reference

RAN1#104bis-e Meeting Report