3GPP TSG RAN WG1 Meeting #105					          R1-2105474

e-Meeting, May 10th – 27th, 2021

Agenda item:	8.7.1.1

Source:	Panasonic

Title:	On Paging Enhancement

Document for:	Discussion/Decision

# Introduction

In the RAN1 #104b-e meeting, agreements for comparison of PEI candidate designs were achieved [1], which are attached in the appendix.

This contribution provides our views in below aspects on the paging enhancement for Rel.17 UE power saving.

- Support of Behv-A and/or Behv-B

- Subgrouping design

- Paging Early Indication (PEI) physical layer design

# Paging enhancement

## Support of Behv-A and/or Behv-B

High level comparison between Behv-A and Behv-B:

- If group paging rate (GPR) is low, by implementation, gNB may skip some PEI transmission in order to save more resource in case of adopting Behv-A than Behv-B, as most of the instances there is no paging by network.

- Following similar principle, if group paging rate is high, by implementation, gNB can skip some PEI transmission in case of adopting Behv-B

Observation 1: If UE does not detect any PEI, the supported UE behavior can largely impact the system overhead caused by PEI transmission under different GPR assumptions.

The system overhead introduced by PEI depends on multiple factors, i.e. PEI physical channel design (PDCCH, TRS/CSI-RS, or SSS-based), supported UE behavior (Behv-A or B) in case of miss detection, and assumed group paging rate. In the evaluation, the typical GPR used is 10%. Some higher values from 20% to 60% are used to explore more power saving gain from sub-grouping mechanism. However, the high range of the GPR is not always typical for actually network deployment. Thus, Behv-A should be supported at least.

According to [2], RAN2 adopted network controlled subgrouping (based on individual UE characteristics, not specified or limited to paging probability as EUTRA, possibly with additional randomization). Our understanding is the situation of GPR does not change in spite of the network controlled subgrouping as the reasonable network would configure the sub-grouping to make GPR to the order of RAN1 evaluation for reduced power consumption taking into account the difference of the paging rate of the UE characteristics (URLLC, V2X, and so on).

Proposal 1: Behv-A should be supported.

For IDLE/INACTIVE mode UE, when UE is uncertain about SIB configuration of PEI e.g. during the SI modification period or before to obtain the SIB related to PEI, the UE behavior should be discussed separately. In addition, the TRS/CSI-RS for time/frequency tracking in AI 8.7.1.2 is also agreed to be configured by SIB signaling. Hence, when UE is within SI modification period, it is possible that the reception of the TRS/CSI-RS for time/frequency tracking is not reliable or the previous configuration of PEI is not reliable. In this case, it is possible that gNB has actually transmitted PEI but UE fails to detect it. Thus more reasonable UE behavior would be to continue monitoring paging PDCCH. Besides, the availability of TRS/CSI-RS for tracking should also be taken into account for UE to evaluation whether current reception of PEI is reliable or not.

Proposal 2: When UE is certain about the SIB configuration of PEI, if UE does not detect PEI, UE is not required to continue monitoring paging PDCCH in the PO after PEI.

Proposal 3: When UE is not certain about the SIB configuration of PEI, e.g. during SI modification period or before obtaining SIB configuration related to PEI and/or TRS/CSI-RS for time/frequency tracking availability status, UE should continue monitoring paging PDCCH in the PO after PEI.

## Subgrouping design

The sub-grouping index need to be specified, in opinions from both RAN1 and RAN2 [2]. A UE’s sub-group index could be associated with a certain UE characteristic and/or UE ID and can be indicated by the PEI. Our expectation is the certain UE is associated with which sub-group would be managed by CN signaling of UE dedicated configuration similar to UE specific DRX configured by upper layers in RRC_IDLE. We expect all are to be discussed in RAN2. Therefore, physical layer only to know which sub-group the UE belongs.

Proposal 4: Sub-grouping information could be carried in the PEI.

Proposal 5: Which sub-grouping UE belongs are informed from the higher layer.

After UE detects PEI and checks whether it belongs to the sub-group intended for paging, UE may choose to continue or stop the paging monitoring procedure in this paging cycle. If sequence-based PEI is supported, the sub-grouping indication is not flexible enough to accurately point to the paged UE. It is true that sequence-based design is not friendly to support relatively large sub-group numbers in case the UE load is high in a PO. In this case, the further more dedicated sub-grouping information can be indicated by paging DCI. When PEI is not configured by the network, this operation also allows sub-grouping gain of the power reduction.

Proposal 6: Sub-grouping information can also be carried in the paging DCI. When PEI is configured, more refined sub-grouping indication is achieved. When PEI is not configured, just sub-grouping indication within paging DCI can also serve the function.

The detailed subgrouping design should firstly look at the maximumly supported number of subgroups. In the evaluation, the number is from 2 to 16 for a PO, which corresponds to 1 to 4 bits.

As we prefer to support subgrouping by PEI and paging DCI in a configurable way, some options can be as below table, which can be configured via SIB.

Table 1. Example of configurable subgrouping indication by PEI and paging DCI

| Channel carrying subgrouping index | Number of bits of the subgrouping index carried by each channel. |  |  |
| --- | --- | --- | --- |
| PEI | 0 | 1 | 2 |
| Paging DCI | 0 or 1 | 0 | 0, 1 or 2 |

## Paging Early Indication (PEI) physical layer design candidates

As shown by the agreed observations in the appendix, the evaluation assumptions from different sources are still not aligned. The main metric to compare is the average resource overhead per PO in terms of (REs). Actually, no matter for which candidate out of PDCCH, SSS, and TRS/CSI-RS based PEI design, for each PO, the indication may consist of positive/negative PO, and/or sub-grouping indication. Therefore, the ultimate key comparison is about the average resource overhead per bit of indication in PEI.

Observation 2: To check the average resource overhead in terms of REs for PEI design candidates, the metric should be RE numbers per bit of indication.

To deliver certain number of bits by a control channel in DL, both channel encoding and sequence modulation/selection are supported in current NR design, e.g. PBCH and PDCCH for former design, PSS/SSS and PBCH DMRS modulating SSB index for later design. One of the essential differentiations is the modulated bit number. From the evaluation results, to fulfill the performance requirement of MDR/FAR, for PDCCH-based design, AL8 is required. Therefore, For a PEI with several bits of indication, utilizing PDCCH of AL8 and overhead of DMRS is basically less efficient than sequence-based modulation/selection. On the other hand, for an indication with tens of bits, DCI-based design is more preferable than SSS or TRS/CSI-RS based design. Therefore, it would be helpful to get consensus on the supported functionalities and number of bits carried by PEI, before decision on physical channel design.

Proposal 7: To agree on functionalities of PEI and the number of bits supported by PEI, before agreeing on PDCCH, SSS or TRS/CSI-RS based design.

# Conclusion

In this contribution, views on paging enhancement schemes for Rel.17 UE power saving enhancement are provided. The following conclusions are highlighted:

Observation 1: If UE does not detect any PEI, the supported UE behavior can largely impact the system overhead caused by PEI transmission under different GPR assumptions.

Proposal 1: Behv-A should be supported.

Proposal 2: When UE is certain about the SIB configuration of PEI, if UE does not detect PEI, UE is not required to continue monitoring paging PDCCH in the PO after PEI.

Proposal 3: When UE is not certain about the SIB configuration of PEI, e.g. during SI modification period or before obtaining SIB configuration related to PEI and/or TRS/CSI-RS for time/frequency tracking availability status, UE should continue monitoring paging PDCCH in the PO after PEI.

Proposal 4: Sub-grouping information could be carried in the PEI.

Proposal 5: Which sub-grouping UE belongs are informed from the higher layer.

Proposal 6: Sub-grouping information can also be carried in the paging DCI. When PEI is configured, more refined sub-grouping indication is achieved. When PEI is not configured, just sub-grouping indication within paging DCI can also serve the function.

Observation 2: To check the average resource overhead in terms of REs for PEI design candidates, the metric should be RE numbers per bit of indication.

Proposal 7: To agree on functionalities of PEI and the number of bits supported by PEI, before agreeing on PDCCH, SSS or TRS/CSI-RS based design.

# Reference

[1] RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #104b-e

[2] R1-2104229/R2-2104356, Reply LS on UE Sub-grouping for Paging Enhancement, RAN2

# Appendix: Agreements for paging enhancement in Rel.17 Power Saving Enhancement from RAN1#104b-e meeting

Agreement:

Observation 1a:

For the evaluation and comparison of PEI candidate designs, the following observations for coexistence with legacy PDSCH are identified:

- For coexistence with legacy PDSCH, semi-static resouce sharing by configuring RB-symbol-level or RE-level rate-matching patterns covering PEI REs is supported for all PEI candidate designs.

- For coexistence with legacy PDSCH, dynamic resource sharing can be realized for all PEI candidates if PDSCH is scheduled by DCI format 1_1

  - For PDCCH based PEI, CORESET-level rate matching can be realized for the PDSCH as per mandatory capability

  - For SSS-based PEI, CORESET-level rate matching may be realized for the PDSCH as per mandatory capability, depending on the design of SSS-based PEI and UE capability regarding number of supported CORESETs

  - For TRS/CSI-RS based PEI, RE-level rate matching can be realized for the PDSCH as per mandatory capability

  - When PDSCH is not scheduled by DCI format 1_1, it is up to gNB implementation whether and how PEI is transmitted in PDSCH resource

Observation 2a:

For the evaluation and comparison of PEI candidate designs, the following summarize the identified configurations of PEI candidate designs, including pairs of the minimum required resource and maximum UE (sub)group indication capacity per PEI, that can comply with the mandatory performance metrics agreed in RAN1 #104-e:

- If Behv-A is assumed,

| Paging Setting | PEI candidate design | Physical-layer configuration and resource | UE (sub)group indication capcity | Number of companies providing performance results |
| --- | --- | --- | --- | --- |
| PDSCH: MCS0, TB scaling 1.0 PDCCH: AL8, 41-bit payload | PDCCH-based PEI | AL4 PDCCH with 12-bit payload, occupying 288 REs | 12 bits | 5 (HW/HiSi, OPPO, ZTE, CATT, MTK) |
|  |  | AL8 PDCCH with 12-bit payload, occupying 576 REs | 12 bits | 7 (Xiaomi, Intel, QC, Samsung, IDCC, Ericsson, vivo) |
|  |  | AL8 PDCCH with 41-bit payload, occupies 576 REs | 41 bits | 1 (CATT) |
|  | SSS-based PEI | 1-symbol SSS, occupying 132 REs (11 RB x 1 symbol) | 3 bits | 1 (IDCC) |
|  |  | 2-symbol SSS, occupying 264 REs (11 RB x 2 symbols) | 1 bit | 6 (HW/HiSi, vivo, ZTE, CATT, QC, Samsung) |
|  |  |  | 3 bits | 1 (IDCC) |
|  |  | 3-symbol SSS, occupying 396 REs (11 RB x 3 symbols) | 4 bits | 1 (MTK) |
|  | TRS/CSI-RS-based PEI | 1-slot 24-RB TRS, occupying 144 REs (24 RB x 3 REs per RB x 2 symbols) | ≥ 8 bits | 1 (Intel) |
|  |  | 1-slot 28-RB TRS, occupying 168 REs (28 RB x 3 REs per RB x 2 symbols) | 1 bit | 1 (HW/HiSi) |
|  |  | 1-slot 36-RB TRS, occupying 216 REs (36 RB x 3 REs per RB x 2 symbols) | 1 bit | 1 (Samsung) |
|  |  | 1-slot 48-RB TRS, occupying 288 REs (48 RB x 3 REs per RB x 2 symbols) | 1 bit | 3 (vivo, ZTE, Ericsson) |
|  |  |  | 6 bits | 1 (CATT) |
|  |  | 1-slot 50-RB TRS, occupying 300 REs (50 RB x 3 REs per RB x 2 symbols) | 1 bit | 2 (OPPO, QC) |
|  |  |  | 4 bits | 1 (MTK) |
| PDSCH: MCS0, TB scaling 0.5; PDCCH: AL16, 41-bit payload | PDCCH-based PEI | AL8 PDCCH with 12-bit payload, occupying 576 REs | 12 bits | 4 (OPPO, ZTE, MTK, Intel) |
|  | SSS-based PEI | 3-symbol SSS, occupying 396 REs (11 RB x 3 symbols) | 4 bits | 1 (MTK) |
|  | TRS/CSI-RS-based PEI | 1-slot 24-RB TRS, occupying 144 REs (24 RB x 3 REs per RB x 2 symbols) | 3 bits | 1 (Intel) |
|  |  | 1-slot 36-RB TRS, occupying 216 REs (36 RB x 3 REs per RB x 2 symbols) | 8 bits | 1 (Intel) |
|  |  | 1-slot 50-RB TRS, occupying 300 REs (50 RB x 3 REs per RB x 2 symbols) | 1 bit | 1 (OPPO) |
|  |  |  | 4 bits | 1 (MTK) |

- If Behv-B is assumed,

| Paging Setting | PEI candidate design | Physical-layer configuration | UE (sub)group indication capcity | Number of companies providing performance results |
| --- | --- | --- | --- | --- |
| PDSCH: MCS0, TB scaling 1.0 PDCCH: AL8, 41-bit payload | PDCCH-based PEI | AL4 PDCCH with 12-bit payload, occupying 288 REs | 12 bits | 4 (HW/HiSi, OPPO, ZTE, MTK) |
|  |  | AL8 PDCCH with 12-bit payload, occupying 576 REs | 12 bits | 2 (vivo, Samsung) |
|  | SSS-based PEI | 2-symbol SSS, occupying 264 REs (11 RB x 2 symbols) | 1 bit | 3 (HW/HiSi, vivo, ZTE) |
|  |  | 3-symbol SSS, occupying 396 REs (11 RB x 3 symbols) | 1 bit | 1 (MTK) |
|  | TRS/CSI-RS-based PEI | 1-slot 28-RB TRS, occupying 168 REs (28 RB x 3 REs per RB x 2 symbols) | 1 bit | 1 (HW/HiSi) |
|  |  | 1-slot 48-RB TRS, occupying 288 REs (48 RB x 3 REs per RB x 2 symbols) | 1 bit | 2 (vivo, ZTE) |
|  |  |  | 6 bits | 1 (CATT) |
|  |  | 1-slot 50-RB TRS, occupying 300 REs (50 RB x 3 REs per RB x 2 symbols) | 1 bit | 1 (OPPO) |
|  |  |  | 2 bits | 1 (MTK) |
| PDSCH: MCS0, TB scaling 0.5 PDCCH: AL16, 41-bit payload | PDCCH-based PEI | AL8 PDCCH with 12-bit payload, occupying 576 REs | 12 bits | 3 (OPPO, ZTE, MTK) |
|  | SSS-based PEI | 3-symbol SSS, occupying 396 REs (11 RB x 3 symbols) | 1 bit | 1 (MTK) |
|  | TRS/CSI-RS-based PEI | 1-slot 50-RB TRS, occupying 300 REs (50 RB x 3 REs per RB x 2 symbols) | 1 bit | 1 (OPPO) |
|  |  |  | 2 bits | 1 (MTK) |

Observation 3a:

For the evaluation and comparison of PEI candidate designs, the following summarize average resource overheads per PO for PEI candidate designs, considering the configurations identified from performance observation.

- The average overhead results are based on PO settings without impact from UE sub-grouping indication within the PO.

- Note: For comparison purpose, single-beam transmission for PEI is assumed, and results with multi-beam transmission for PEI is scaled. This doesn’t preclude any beam-forming related design for PEI.

- If Behv-A is assumed:

| Paging Setting | PEI candidate design |  | Physical-layer configuration and resource | UE (sub)group indication capacity | Number of companies providing performance results |  | Average resource overhead per PO (REs) |  |  |  | PO and PEI related assumptions | Resource sharing assumption |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PDSCH: MCS0, TB scaling 1.0 PDCCH: AL8, 41-bit payload | PDCCH-based PEI |  | AL4 PDCCH with 12-bit payload, occupying 288 REs | 12 bits | 5 (HW/HiSi, OPPO, ZTE, CATT, MTK) |  | 17.2 |  | OPPO |  | 1 PEI for up to 12 PO's | PEI is transmitted as a Rel-15 PDCCH in a CORESET when a UE group is paged |
|  |  |  |  |  |  |  | 17.2 |  | ZTE |  | 1 PEI for up to 12 PO's |  |
|  |  |  |  |  |  |  | 17.6 |  | HW/HiSi |  | 1 PEI for up to 12 PO's |  |
|  |  |  |  |  |  |  | 21.8 |  | MTK |  | 1 PEI for up to 12 PO's; averaged all PO settings for 1.28-sec cycle |  |
|  |  |  |  |  |  |  | 28.8 |  | CATT |  | 1 PEI for 1 PO | Dynamic rate-matching in PDSCH |
|  |  |  |  |  |  |  | 288.0 |  | CATT |  | 1 PEI for 1 PO | Semi-static rate-matching in PDSCH |
|  |  |  | AL8 PDCCH with 12-bit payload, occupying 576 REs | 12 bits | 7 (Xiaomi, Intel, QC, Samsung, IDCC, Ericsson, vivo) |  | 49.5 |  | vivo |  | 1 PEI for 4 PO | PEI is transmitted as a Rel-15 PDCCH in a CORESET when a UE group is paged |
|  |  |  |  |  |  |  | 57.6 |  | vivo |  | 1 PEI for 1 PO |  |
|  |  |  |  |  |  |  | 57.6 |  | QC |  | 1 PEI for 1 PO |  |
|  |  |  |  |  |  |  | 57.6 |  | Samsung |  | 1 PEI for 1 PO; PEI RE# scaled w.r.t. 1-beam |  |
|  |  |  | AL8 PDCCH with 41-bit payload, occupies 576 REs | 41 bits | 1 (CATT) |  | 57.6 |  | CATT |  | 1 PEI for 1 PO | Dynamic rate-matching in PDSCH |
|  |  |  |  |  |  |  | 576.0 |  | CATT |  | 1 PEI for 1 PO | Semi-static rate-matching in PDSCH |
|  | SSS-based PEI |  | 1-symbol SSS, occupying 132 REs (11 RB x 2 symbols) | 3 bits | 1 (IDCC) |  |  |  |  |  |  |  |
|  |  |  | 2-symbol SSS, occupying 264 REs (11 RB x 2 symbols) | 1 bit | 6 (HW/HiSi, vivo, ZTE, CATT, QC, Samsung) |  | 25.4 |  | Samsung |  | 1 PEI for 1 PO; PEI RE# scaled w.r.t. 1-beam | Dynamic rate-matching in PDSCH |
|  |  |  |  |  |  |  | 25.4 |  | vivo |  | 1 PEI for 1 PO |  |
|  |  |  |  |  |  |  | 26.4 |  | ZTE |  | 1 PEI for 1 PO |  |
|  |  |  |  |  |  |  | 28.8 |  | CATT |  | 1 PEI for 1 PO |  |
|  |  |  |  |  |  |  | 28.8 |  | QC |  | 1 PEI for 1 PO |  |
|  |  |  |  |  |  |  | 254.0 |  | HW/HiSi |  | 1 PEI for 1 PO | Semi-static rate-matching in PDSCH |
|  |  |  |  |  |  |  | 264.0 |  | ZTE |  | 1 PEI for 1 PO |  |
|  |  |  |  |  |  |  | 288.0 |  | QC |  | 1 PEI for 1 PO |  |
|  |  |  |  | 3 bits | 1 (IDCC) |  |  |  |  |  |  |  |
|  |  |  | 3-symbol SSS, occupying 396 REs (11 RB x 3 symbols) | 4 bits | 1 (MTK) |  | 34.0 |  | MTK |  | 1 PEI for up to 4 PO's; averaged all PO settings for 1.28-sec cycle | Dynamic rate-matching in PDSCH |
|  |  |  |  |  |  |  | 437.0 |  | MTK |  | 1 PEI for up to 4 PO's; averaged all PO settings for 1.28-sec cycle; RB-symbol rate-matching pattern period up to 40 ms | Semi-static rate-matching in PDSCH |
|  | TRS/CSI-RS-based PEI |  | 1-slot 24-RB TRS, occupying 144 REs (24 RB x 3 REs per RB x 2 symbols) | ≥ 8 bits | 1 (Intel) |  | 14.4 |  | Intel |  | 1 PEI for 1 PO | Dynamic rate-matching in PDSCH |
|  |  |  | 1-slot 28-RB TRS, occupying 168 REs (28 RB x 3 REs per RB x 2 symbols) | 1 bit | 1 (HW/HiSi) |  | 123.4 |  | HW/HiSi |  | 1 PEI for 1 PO | Dynamic rate-matching in PDSCH |
|  |  |  |  |  |  |  | 168.0 |  | HW/HiSi |  | 1 PEI for 1 PO | Semi-static rate-matching in PDSCH |
|  |  |  | 1-slot 36-RB TRS, occupying 216 REs (36 RB x 3 REs per RB x 2 symbols) | 1 bit | 1 (Samsung) |  | 21.6 |  | Samsung |  | 1 PEI for 1 PO; PEI RE# scaled w.r.t. 1-beam | Dynamic rate-matching in PDSCH |
|  |  |  | 1-slot 48-RB TRS, occupying 288 REs (48 RB x 3 REs per RB x 2 symbols) | 1 bit | 3 (vivo, ZTE, Ericsson) |  | 28.8 |  | vivo |  | 1 PEI for 1 PO | Dynamic rate-matching in PDSCH |
|  |  |  |  |  |  |  | 28.8 |  | ZTE |  | 1 PEI for 1 PO |  |
|  |  |  | 1-slot 48-RB TRS, occupying 288 REs (48 RB x 3 REs per RB x 2 symbols) | 6 bits | 1 (CATT) |  | 28.8 |  | CATT |  | 1 PEI for 1 PO | Dynamic rate-matching in PDSCH |
|  |  |  |  |  |  |  | 288.0 |  | CATT |  | 1 PEI for 1 PO | Semi-static rate-matching in PDSCH |
|  |  |  | 1-slot 50-RB TRS, occupying 300 REs (50 RB x 3 REs per RB x 2 symbols) | 1 bit | 2 (OPPO, QC) |  | 30.0 |  | OPPO |  | 1 PEI for 1 PO | Dynamic rate-matching in PDSCH |
|  |  |  |  |  |  |  | 30.0 |  | QC |  | 1 PEI for 1 PO |  |
|  |  |  |  |  |  |  | 300.0 |  | QC |  | 1 PEI for 1 PO | Semi-static rate-matching in PDSCH |
|  |  |  |  | 4 bits | 1 (MTK) |  | 26.0 |  | MTK |  | 1 PEI for up to 4 PO's | Dynamic rate-matching in PDSCH |
| PDSCH: MCS0, TB scaling 0.5; PDCCH: AL16, 41-bit payload | PDCCH-based PEI | AL8 PDCCH with 12-bit payload, occupying 576 REs |  | 12 bits |  | 4 (OPPO, ZTE, MTK, Intel) | 34.4 | OPPO |  | 1 PEI for up to 12 PO's |  | PEI is transmitted as a Rel-15 PDCCH in a CORESET when a UE group is paged |
|  |  |  |  |  |  |  | 43.6 | MTK |  | 1 PEI for up to 12 PO's; averaged all PO settings for 1.28-sec cycle |  |  |
|  |  |  |  |  |  |  | 57.6 | Intel |  | 1 PEI for 1 PO |  | PEI is transmitted as a Rel-15 PDCCH in a CORESET when a UE group is paged |
|  | SSS-based PEI | 3-symbol SSS, occupying 396 REs (11 RB x 3 symbols) |  | 4 bits |  | 1 (MTK) | 34.0 | MTK |  | 1 PEI for up to 4 PO's; averaged all PO settings for 1.28-sec cycle |  | Dynamic rate-matching in PDSCH |
|  |  |  |  |  |  |  | 437.0 | MTK |  | 1 PEI for up to 4 PO's; averaged all PO settings for 1.28-sec cycle; RB-symbol rate-matching pattern period up to 40 ms |  | Semi-static rate-matching in PDSCH |
|  | TRS/CSI-RS-based PEI | 1-slot 24-RB TRS, occupying 144 REs (24 RB x 3 REs per RB x 2 symbols) |  | 3 bits |  | 1 (Intel) | 14.4 | Intel |  | 1 PEI for 1 PO |  | Dynamic rate-matching in PDSCH |
|  |  | 1-slot 36-RB TRS, occupying 216 REs (36 RB x 3 REs per RB x 2 symbols) |  | 8 bits |  | 1 (Intel) | 21.6 | Intel |  | 1 PEI for 1 PO |  | Dynamic rate-matching in PDSCH |
|  |  | 1-slot 50-RB TRS, occupying 300 REs (50 RB x 3 REs per RB x 2 symbols) |  | 1 bit |  | 1 (OPPO) | 30.0 | OPPO |  | 1 PEI for 1 PO |  | Dynamic rate-matching in PDSCH |
|  |  |  |  | 4 bits |  | 1 (MTK) | 26.0 | MTK |  | 1 PEI for up to 4 PO's; averaged all PO settings for 1.28-sec cycle |  | Dynamic rate-matching in PDSCH |

- If Behv-B is assumed:

| Paging Setting | PEI candidate design | Physical-layer configuration | UE (sub)group indication capacity | Number of companies providing performance results | Average resource overhead per PO (REs) |  | PO and PEI related assumptions | Coexistence assumption |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PDSCH: MCS0, TB scaling 1.0 PDCCH: AL8, 41-bit payload | PDCCH-based PEI | AL4 PDCCH with 12-bit payload, occupying 288 REs | 12 bits | 4 (HW/HiSi, OPPO, ZTE, MTK) | 24.0 | HW/HiSi | 1 PEI for up to 12 PO's | PEI is ALWAYS transmitted as a Rel-15 PDCCH in a CORESET |
|  |  |  |  |  | 24.0 | OPPO | 1 PEI for up to 12 PO's |  |
|  |  |  |  |  | 24.0 | ZTE | 1 PEI for up to 12 PO's |  |
|  |  |  |  |  | 51.0 | MTK | 1 PEI for up to 12 PO's; averaged all PO settings for 1.28-sec cycle |  |
|  |  | AL8 PDCCH with 12-bit payload, occupying 576 REs | 12 bits | 2 (vivo, Samsung) | 518.4 | vivo | 1 PEI for 1 PO | PEI is ALWAYS transmitted as a Rel-15 PDCCH in a CORESET |
|  |  |  |  |  | 518.4 | Samsung | 1 PEI for 1 PO; PEI RE# scaled w.r.t. 1-beam |  |
|  | SSS-based PEI | 2-symbol SSS, occupying 264 REs (11 RB x 2 symbols) | 1 bit | 3 (HW/HiSi, vivo, ZTE) | 228.6 | vivo | 1 PEI for 1 PO | Dynamic rate-matching in PDSCH |
|  |  |  |  |  | 228.6 | ZTE | 1 PEI for 1 PO |  |
|  |  |  |  |  | 254.0 | HW/HiSi | 1 PEI for 1 PO | Semi-static rate-matching in PDSCH |
|  |  |  |  |  | 264.0 | ZTE | 1 PEI for 1 PO |  |
|  |  | 3-symbol SSS, occupying 396 REs (11 RB x 3 symbols) | 1 bit | 1 (MTK) | 561.0 | MTK | 1 PEI for 1 PO; average over all PO settings for 1.28-sec cycle; RB-symbol rate-matching pattern period up to 40 ms | Semi-static rate-matching in PDSCH |
|  | TRS/CSI-RS-based PEI | 1-slot 28-RB TRS, occupying 168 REs (28 RB x 3 REs per RB x 2 symbols) | 1 bit | 1 (HW/HiSi) | 168.0 | HW/HiSi | 1 PEI for 1 PO | Semi-static rate-matching in PDSCH |
|  |  | 1-slot 48-RB TRS, occupying 288 REs (48 RB x 3 REs per RB x 2 symbols) | 1 bit | 2 (vivo, ZTE) | 259.2 | ZTE | 1 PEI for 1 PO | Dynamic rate-matching in PDSCH |
|  |  |  |  |  | 259.2 | vivo | 1 PEI for 1 PO |  |
|  |  |  | 6 bits | 1 (CATT) | 288.0 | CATT | 1 PEI for 1 PO | Semi-static rate-matching in PDSCH |
|  |  | 1-slot 50-RB TRS, occupying 300 REs (50 RB x 3 REs per RB x 2 symbols) | 1 bit | 1 (OPPO) | 279.0 | OPPO | 1 PEI for 1 PO | Semi-static rate-matching in PDSCH |
|  |  |  | 2 bits | 1 (MTK) | 150.0 | MTK | 1 PEI for 2 PO's | Semi-static rate-matching in PDSCH |
| PDSCH: MCS0, TB scaling 0.5 PDCCH: AL16, 41-bit payload | PDCCH-based PEI | AL8 PDCCH with 12-bit payload, occupying 576 REs | 12 bits | 3 (OPPO, ZTE, MTK) | 48.0 | OPPO | 1 PEI for up to 12 POs | PEI is ALWAYS transmitted as a Rel-15 PDCCH in a CORESET |
|  |  |  |  |  | 102.0 | MTK | 1 PEI for up to 12 POs |  |
|  | SSS-based PEI | 3-symbol SSS, occupying 396 REs (11 RB x 3 symbols) | 1 bit | 1 (MTK) | 561.0 | MTK | 1 PEI for 1 PO; RB-symbol rate-matching pattern period up to 40 ms | Semi-static rate-matching in PDSCH |
|  | TRS/CSI-RS-based PEI | 1-slot 50-RB TRS, occupying 300 REs (50 RB x 3 REs per RB x 2 symbols) | 1 bit | 1 (OPPO) | 270.0 | OPPO | 1 PEI for 1 PO | Semi-static rate-matching in PDSCH |
|  |  |  | 2 bits | 1 (MTK) | 150.0 | MTK | 1 PEI for 2 PO's | Semi-static rate-matching in PDSCH |