3GPP TSG RAN WG1#105-e	 	 	R1-2105702

e-Meeting, May 10th – 27th, 2021

Source:	NTT DOCOMO, INC.

Title:	Discussion on multipath/NLOS mitigation for NR positioning

Agenda Item:	8.5.5

Document for: 	Discussion and Decision

- Introduction

At the RAN#91-e meeting, updated WID on NR positioning was agreed [1]. The work item includes RAN1-led objectives as follows:

The objective of this work item is to specify solutions to enable RAT dependent (for both FR1 and FR2) and RAT independent NR positioning enhancements for improving positioning accuracy, latency, network and/or device efficiency. The objective of this work item also includes the support of GNSS enhancements.

The specific objectives of this work are:

- Specify methods, measurements, signalling, and procedures for improving positioning accuracy of the Rel-16 NR positioning methods by mitigating UE Rx/Tx and/or gNB Rx/Tx timing delays, including [RAN1, RAN2, RAN3, RAN4]

  - DL, UL and DL+UL positioning methods

  - UE-based and UE-assisted positioning solutions

- Specify the procedure, measurements, reporting, and signalling for improving the accuracy of [RAN1, RAN2, RAN3]

  - UL AoA for network-based positioning solutions.

  - DL-AoD for UE-based and network-based (including UE-assisted) positioning solutions.

- Specify the enhancements of signalling, and procedures for improving positioning latency of the Rel-16 NR positioning methods, for DL and DL+UL positioning methods, including:

  - Latency reduction related to the request and response of location measurements or location estimate and positioning assistance data; [RAN2, RAN3, RAN1]

  - Latency reduction related to the time needed to perform UE measurements; [RAN1, RAN4]

  - Latency reduction related to the measurement gap; [RAN1, RAN4, RAN2]

- Study and specify, if agreed, the enhancements of information reporting from UE and gNB for multipath/NLOS mitigation [RAN1, RAN2, RAN3]

Notes:

Solutions for RAT-dependent positioning enhancements are designed to operate in both frequency ranges (i.e. FR1 & FR2)

RAN working groups will take into account the work from other working group SA/CT which involves same objectives.

- The WID is subject to further update in RAN #91 for RAN1/2/3/4 scoping.

In this contribution, we present our view on potential enhancements of information reporting from UE and gNB for multipath/NLOS mitigation for Rel-17 NR positioning.

- Potential enhancements for multipath/NLOS mitigation

In the current NR, the angle based positioning methods are supported such as DL-AoD and UL-AoA. In these angle based positioning methods, a UE/NW measures RSRP of PRS as positioning metric. When we consider the case that the UE is located to NLOS environment from a TRP, angle based positioning relying on RSRP reporting may be insufficient. For example, when a UE is located to NLOS or multipath effective environment, the UE/NW may calculate positioning metric by using an inappropriate direction of arrival beam and it may impact on positioning accuracy since RSRP information may be insufficient to identify whether the beam is NLOS or not. Therefore, introducing multipath/NLOS mitigation scheme may be beneficial to improve the accuracy of angle based positioning.

Observation 1:

- Introducing multipath/NLOS mitigation scheme may be beneficial to improve the accuracy of angle based positioning

In order to improve an accuracy of angle based positioning, RSRP measurement related to first arrival path has been discussed in other agenda items (i.e. enhancements for DL-AoD/UL-AoA). However, introducing measurement reporting related to first arrival path may not be enough to improve an accuracy of angle based positioning. A LMF does not understand whether the reported beam is LOS path or NLOS path since the measurement reporting from a UE/gNB to the LMF does not include LOS/NLOS information. For example, when the UE is located to NLOS environment from a TRP, the LMF may misinterpret that the reported beam related to first arrival path is LOS path. Introducing LOS/NLOS indictor (e.g. an indicator to signal whether the reported beam is LOS path or NLOS path) may be helpful to enable to select the TRP(s) and/or switch angle based positioning method to other positioning method.

Observation 2:

- Introducing LOS/NLOS indicator may be helpful to enable to select the TRP(s) and/or switch angle based positioning method to other positioning method

- Conclusion

In this contribution, we discussed on potential enhancements for multipath/NLOS mitigation for Rel-17 NR positioning. Based on the discussion, we made following observations.

Observation 1:

- Introducing multipath/NLOS mitigation scheme may be beneficial to improve the accuracy of angle based positioning

Observation 2:

- Introducing LOS/NLOS indicator may be helpful to enable to select the TRP(s) and/or switch angle based positioning method to other positioning method

Reference

- RP-210903, 3GPP RAN#91-e, “Revised WID on NR Positioning Enhancements”, Electronic Meeting, Mar. 2021.