3GPP TSG RAN WG1 Meeting #104-e	R1-2105764

e-Meeting, May 10th – 27th 2021

Agenda item:	8.10.2

Source: 	Lenovo, Motorola Mobility

Title: 	Timing, interference, and power control in enhanced IAB systems

Document for:	Discussion and Decision

# Introduction

In a companion paper ‎[1], we provide our views on resource multiplexing in eIAB systems. In this paper, we discuss enabling features such as timing alignment, power control, and interference management.

# Discussion

## Timing alignment

It was agreed in RAN1#102-e to support 	Case-6 timing for Case A multiplexing and Case-7 timing for Case B multiplexing. Furthermore, the following agreements were made in RAN1#103-e and RAN1#104-e ‎[2]‎[3]:

Agreement

Case 6 timing mode operation at an IAB-node is controlled by the parent node to which the UL transmission is intended for.

Agreement

Case 7 timing is supported with symbol level alignment without explicit support for slot level alignment

Agreement

Switching between Case 1, Case 6, and Case 7 timing is supported.

- FFS whether Case 6 and Case 7 timing shall be restricted to certain resources, e.g. excluding resources used for access or TDM backhaul

- FFS details on switching including the switching conditions

- FFS relationship between switching timing modes with the usage/indication of different resource multiplexing modes

- FFS whether Rel-16 OTA synchronization shall be enhanced to support switching timing modes

Since Case-1 timing alignment does not specify timing relationships for uplink, IAB nodes in Rel-16 normally follow the legacy timing advance (TA) procedure for uplink transmissions. For an IAB node with one antenna panel/RF frontend connected to a baseband unit, symbol boundaries should be aligned for simultaneous transmissions (Case-6 timing alignment) and simultaneous receptions (Case-7 timing alignment).

First, whether timing alignment is needed for the case of simultaneous TX (Case A) or simultaneous RX (Case B) in an IAB node depends on the IAB node’s capability. Particularly if the IAB node has multiple IFFT/FFT windows for OFDM processing, it does not need to perform timing alignment further than Case-1, which can improve the possibility of simultaneous operations significantly. Therefore, we propose that the IAB node’s capability for timing alignment be reported to the IAB donor and/or the parent node.

Support a capability signalling to indicate whether the IAB node requires timing alignment between IAB-MT and IAB-DU operations. If negative, the IAB node can transmit unaligned OFDM symbols (Case A) and receive/process unaligned OFDM symbols (Case B).

With respect to restrictions on applicability of enhanced timing alignment schemes, for example excluding resources on access links and TDM backhaul links, we propose to leave the matter to implementation as much as possible. For example, a capability signaling from an IAB node can indicate whether the IAB node supports enhanced timing alignment. The rest can be left to implementation to save standardization efforts.

Support a capability signaling to indicate whether the IAB node supports enhanced timing alignment. If the feature is not optional, the capability signaling may be the parameter that indicates capability of enhanced (Rel-17) operation by the IAB node.

Next, for an IAB node with a single antenna panel/RF frontend connected to a baseband unit that has a single IFFT/FFT window, Case-6 timing alignment should be applied when UL resources are configured for IAB-MT and DL resources are configured for IAB-DU; and conversely, Case-7 timing alignment should be applied when DL resources are configured for IAB-MT and UL resources are configured for IAB-DU.

Therefore, we propose to specify timing alignment configuration for IAB nodes. The configuration can indicate a timing alignment for a number of slots, a semi-static scheduled (SPS) channel, and so on. Configurations work well for hard DL/UL resources, but with flexible and/or soft resources, a timing alignment mode may or may not need to be applied at a particular time slot. Hence, lower layer control signaling can be further employed to activate or trigger a timing alignment mode.

Support configuration and control signaling for applying Case-6 and Case-7 timing alignment.

It was agreed in the previous meeting that switching between Case-1, Case-6, and Case-7 timing will be supported. Case-1 applies to Rel-16 and Rel-17 IAB nodes while Case-6 and Case-7 will be supported by Rel-17 IAB nodes only. Since it is desired to support operation of IAB systems with mixed Rel-16 and Rel-17 IAB nodes, signaling and application of enhanced timing alignment schemes should be transparent to Rel-16 IAB nodes.

One issue with switching timing modes is the possibility of overlapping the last symbol of a TX/RX with one timing mode and the first symbol of the next TX/RX with another timing mode. Guard symbols can be designated for switching between timing modes if needed.

Guard symbols can be used to separate consecutive TX/RX operations when switching timing alignment modes.

Case-6 timing alignment:

In order to realize Case-6, an IAB node should align its uplink transmission with downlink transmission. Since downlink transmissions should be aligned across IAB nodes according to Case-1, the IAB node needs no additional information to apply Case-6.

Case-7 timing alignment:

In order to realize Case-7, the IAB node does need additional information. Consider the scenario depicted in ‎Figure 1.

. Example scenario

The goal is for the parent node (PN) and the child node (CN) to transmit signals simultaneously to the IAB node (N) in a way that symbol boundaries are aligned at N. Given that the downlink transmission from PN to N is already determined in Case-7 similar to Case-1, the only degree of freedom is to adjust the uplink transmission time.

Let us define the following notations:

- Tp,u: the propagation delay between PN and N

- Ttx,u: downlink transmission time by PN

- Trx,u=Ttx,u+Tp,u: downlink reception time at N from PN

- Tp,d: the propagation delay between CN and N

- Ttx,d: uplink transmission time by CN

- Trx,d=Ttx,d+Tp,d: uplink reception time at N from CN

A scheme based on Case-7 should set:

Trx,d≔Trx,u   ⇒   Ttx,d≔Ttx,u+Tp,u-Tp,d

This is the transmission timing that should be applied by CN. We can assume that Trx,d is already known to CN as Case-1 already aligns all downlink transmission times across PN, N, and CN. However, additional signaling is needed to inform CN of the propagation delay term in the round brackets.

Define signaling to communicate information of the parent link propagation delay to child IAB nodes.

Unified timing alignment:

It was proposed in the previous meeting to support a unified timing alignment framework. This approach can simplify signaling of timing alignment and provide higher flexibility for future extensions.

In order to realize a unified scheme, we should note that the uplink timing for legacy TA, Case-6, and Case-7 can be written in the following canonical form:

TTA= a.NTA+b.Tdelta+c.NATA+d⋅Tc

In this equation, NTA is obtained by legacy TA command, Tdelta is obtained for Case-1, which in turn determines uplink transmission time for Case-6, and NATA is the additional TA for Case-7 as derived earlier. By setting the constant coefficients a, b, c, d, the IAB node can determine the timing advance TTA for any of legacy TA (Case-1), Case-6, and Case-7 modes.

Support a unified framework for uplink timing alignment.

With respect to the FFS point on enhancing Rel-16 OTA synchronization, we prefer to avoid any modification to the specification of Case-1 timing in order to allow backward compatibility and coexistence of Rel-16 and Rel-17 IAB nodes in IAB systems.

No modification to Case-1 timing alignment.

## Power control

The following agreements were made on DL/UL power control in RAN1#103-e and RAN1#104-e ‎‎[2][3]:

Agreement

Further study requirement of enhanced DL and UL Tx power control mechanism considering the following:

- DL/UL power control with assistance information from the child node.

- DL/UL power control with assistance information from the parent node.

- Central (e.g. by CU) power control coordination (e.g. semi-static max DL/UL Tx power limits).

- Coexistence of different power control mechanisms within an IAB node and in the network.

Note. Any power control mechanism should consider the following aspects:

- Existing base station design principles (e.g. power control and dynamic range capability, etc.) related to transmission power.

Network constraints in regard to transmitted reference signals with constant power.

Agreement

RAN1 to further study whether the legacy UL power control mechanism (including PHR) is sufficient for an IAB-node operating in an enhanced multiplexing mode.

- FFS: if not (i.e., the legacy mechanism is not sufficient), support an IAB-node indicating information to assist with its UL power control.

Agreement

Support an IAB-node indicating information to assist with the DL power control of its parent-node towards the IAB-node without mandating an expected behavior at the parent node.

- Note: At least the assistance information is for supporting the simultaneous operation within the IAB-node to avoid power imbalance

- FFS: type of assistance information (e.g., desired received power, power adjustment, preferred CSI-RS resource)

- FFS: whether this information is provided to the parent-node, the CU, or both.

- FFS: applicability of the assistance information (e.g. relation to beams or multiplexing modes)

- FFS: the channel carrying this assistance information

Consider the example scenario illustrated in ‎Figure 1. In that scenario, uplink transmit power of the IAB node (N) is controlled by the parent IAB node (PN). Consider Case A duplexing where PN schedules a PUSCH for N, and N schedules a simultaneous PDSCH for the child IAB node (CN). If the instantaneous uplink transmit power by N has a large offset with respect to its target downlink transmit power, N will be compelled to either alter its downlink transmit power or drop the transmission. This may be caused by a large power imbalance between upstream and downstream transmissions for N, or by a constraint on the total transmit power, and so on. However, prior information of the transmit power could help N decide whether it wants to schedule any simultaneous downstream transmissions in the first place.

A similar issue may be raised for Case B duplexing. If the IAB node N does not obtain prior knowledge of the received power from its parent node PN, it may not be able to send TPC commands to its child node CN for a simultaneous communication in order to maintain a balanced receive power for signals received from PN and CN.

Semi-static resource configuration (partitioning) provides a reliable means to coordinate on otherwise-complex aspects such as timing alignment, interference management, and so on. Power control in uplink and downlink is another aspect that can be handled by semi-static configurations in addition to L1/L2 control signalling. In this approach, power control processes are configured in a semi-persistent manner for each resource partition. This provides an early assessment of transmit and receive powers for IAB nodes on each resource partition so as to allow the IAB nodes to schedule simultaneous operations, signal transmit power control to child nodes, and so on. Such signalling can be controlled by the donor IAB-CU and further complemented by L1/L2 signalling.

Support power control configurations at least for Case A and Case B multiplexing.

Although power control configurations as described above are helpful for resource planning, scheduling, interference management, etc., dynamic L1/L2 control signaling is still needed to handle variations due to mobility and beamforming among other factors. Such signaling is expected to require power control in both uplink and downlink communications. IAB-MT uplink power control can follow similar principles as UE uplink power control and possibly enjoy enhancements, for example, for Case A multiplexing.

IAB-DU downlink power control can also be specified to enable Case B multiplexing at a child node. For example, the child node can request a serving IAB-DU for a change in its DL Tx power such that the child node’s Rx power on its MT and DU remain within a threshold. Given constraints such as intended coverage and dynamic range, the serving IAB-DU can then respond by changing the DL Tx power as requested by the child node.

It was agreed in the previous meeting to support indication of downlink power control information to the parent node. We propose a request-response signaling by which the IAB node requests a change of transmit power by the parent node towards the IAB node, followed by an optional response signaling informing the IAB node, for example, whether the power change request was accepted. The response signaling can assist the IAB node to perform UL power control signaling with its child nodes and UEs, schedule simultaneous PUSCH for the child nodes and UEs, and so on.

The request signaling should contain the requested value of power offset, for example in dB. Reference power and spatial information such as reference to SSB or CSI-RS resources can also be included in the request signaling.

In response, the parent node can optionally send a response signaling to the IAB node including information of whether the request is granted and/or by what ratio in dB. Similar to the request signaling, reference power and spatial information such as reference to SSB or CSI-RS resources can also be included in the response signaling.

The downlink power control signaling from the IAB node to the parent node includes a power offset value as well as reference power and spatial information.

Parent node can respond to the IAB node indicating a granted power offset as well as reference power and spatial information.

## Interference management

The following was agreed in RAN1#103-e and RAN1#104-e [1][2]:

Agreement

Interference management for the following IAB interference scenarios should be discussed:

- Inter-IAB scenarios, including:

  - MT to MT, DU to DU, DU to MT, and MT to DU.

- Interference to non-IAB nodes, including:

  - IAB-DU to non-IAB-DU

  - IAB-MT to non-IAB-DU

- Intra-IAB-node (self-interference) scenarios (Interference between a DU and MT of an IAB-node).

This agreement does not necessarily mean that specification support is needed for any of the scenarios.

Agreement

Use the Rel-16 interference management frameworks (e.g. CLI, RIM) to handle IAB interference scenarios, and discuss if any of the following enhancements are needed (not an exhaustive list):

- FFS: extend the information exchange (e.g. the resource configuration, result of CLI measurements, etc.) among different entities (e.g. between parent-child nodes, adjacent IAB nodes, between network and IAB-node, etc.)

- FFS: required enhancements on CLI measurement accuracy (e.g. via timing adjustment, etc.)

FFS: required enhancements on CLI measurements (e.g. introducing short-term measurements, multi-beam measurements, etc.)

Conclusion

In Rel-17, RAN1 will not specify specific mechanisms for intra-IAB-node interference (self-interference) management.

- Self-interference can be handled by the implementation or via using the available techniques defined, or to be defined in Rel-17, that can commonly be used for other interference scenarios as well.

Agreement

RAN1 to select among the following options to support DU-to-DU measurement and report.

- For DU-to-DU CLI measurement:

  - Option 1.1. no specific mechanism is specified (e.g., it is handled by the implementation, or the available techniques)

  - Option 1.2. enhanced legacy DU-based measurement procedures (e.g., enhanced Rel-16 RIM)

  - Option 1.3. enhanced MT-based measurements (e.g., MT-based CLI, MT RRM measurements)

- For DU-to-DU CLI report:

  - Option 2.1. no specific mechanism is specified (e.g., it is handled by the implementation, or the available techniques)

  - Option 2.2. enhanced legacy DU-based report (e.g., enhanced Rel-16 RIM)

  - Option 2.3. enhanced MT-based report (e.g., MT-based CLI, MT RRM measurements)

Agreement

RAN1 to decide whether to enhance interference mitigation through information exchange to support beam-management at the parent or child node in RAN1#104bis-e

  - FFS: reporting of desired beams for reception in DL or desired beams for transmission in UL by the IAB node for a given multiplexing mode

  - FFS: indicating applicable beams in DL or beams in UL for a given multiplexing mode.

Different types of interference, including cross-link interference (CLI) among adjacent IAB nodes and self-interference (SI) within an IAB node can be handled by configuring reference signals that other IAB nodes in the vicinity can measure. These reference signals are naturally CSI-RS for DL resources and SRS for UL resources. This approach works well at least for handling interference among IAB nodes connected to the same IAB donor, because information of these designated CSI-RS/SRS configurations can be provided to all the IAB nodes by the IAB-CU. The case of interference among IAB nodes connected to different IAB donors can be further studied. Furthermore, CLI where either the aggressor or the victim is a UE can also be handled by existing CLI specifications.

CLI management was specified in Rel-16. Two types of CLI measurements were specified: SRS-RSRP and CLI-RSSI. Given that a typical IAB deployment may allow IAB nodes to be in close proximity compared to a typical cellular system, especially in mobile IAB scenarios where a continuous CLI measurement and a flexible resource allocation for reference signals is needed, several aspects of CLI can be enhanced for application in IAB systems. For example, semi-persistent configurations where IAB nodes can activate or deactivate reference signal transmissions and measurements can be introduced so as to allow multi-hop IAB systems to gain control over measurements and reporting.

Support enhanced DU-based procedures for measurement and reporting. Further consider MT-based procedures.

Another matter is timing adjustment to improve accuracy of CLI measurements. According to Rel-16 CLI, the victim IAB node receives configuration information for SRS from the aggressor IAB node. But the timing of the SRS transmission is unknown to the victim node, hence potentially reducing the measurement accuracy if left to implementation. Compared to UE-to-UE CLI, the matter might be worse in the case of IAB due to various timing alignment modes in the uplink.

Addressing this issue for UE-to-UE CLI could be complicated, possibly requiring inter-gNB signaling for conveying timing information. However, since the aggressor node transmits SSBs, the victim node can obtain the timing and utilize that for timing adjustment.

Support timing adjustment for CLI measurements at the victim node based on timing obtained by receiving SSB from the aggressor node.

Companies were encouraged in the previous meetings to further consider resource partitioning scenarios. One of the main aspects is how to handle interference and whether that affects DL/UL resources and backhaul/access links.

We propose to consider both downlink and uplink resources to cover all the four duplexing cases at least for the backhaul link. The different types of interference, including cross-link interference (CLI) and self-interference (SI), among adjacent IAB nodes and within an IAB node, can be handled by configuring reference signals that other IAB nodes in the vicinity can measure. These reference signals are naturally CSI-RS for DL resources and SRS for UL resources. This approach works well at least for handling interference among IAB nodes connected to the same IAB donor, because information of these designated CSI-RS/SRS configurations can be provided to all the IAB nodes by the IAB-CU. The case of interference among IAB nodes connected to different IAB donors can be further studied.

Furthermore, CLI where either the aggressor or the victim is a UE can also be handled by existing CLI specifications. Therefore, it can be argued that resource partitioning for duplexing cases need not be discriminated based on resource type (DL/UL) or link type (access/backhaul).

Support CLI for downlink and uplink resources of backhaul links and access links.

Advantages pursued by deploying IAB systems are reducing cell planning efforts and support for mobile small cells. IAB systems are expected to measure interference frequently and respond appropriately in a timely manner. That includes cross-link interference (CLI) and inter-cell interference (ICI) as well as self-interference (SI) in the case of enhanced duplexing. Enhancements to interference management in IAB systems is further encouraged by the possibility of more effective coordination among IAB nodes that are connected to the same IAB donor; responding to prohibitive interference by proper beamforming and power control; and so on.

Consider the scenario in ‎Figure 2.

. Example scenario for CLI and SI

For MT to MT, DU to DU, DU to MT, and MT to DU interference between different IAB nodes, existing CLI and ICI mechanisms can be taken as a starting point for enhancements. Particularly, for managing interference between IAB nodes served by the same IAB donor, it is possible for the IAB donor to configure CSI-RS or SRS to measure downlink or uplink interference from an IAB-DU or IAB-MT, respectively, and share this information with IAB nodes in the vicinity. Since the interference from the DU or MT on other IAB nodes changes with variations in beamforming, Tx power, mobility, etc., the reference signal can be transmitted while such parameters are applied according to upcoming transmissions, hence allowing other IAB nodes to measure the resulting interference.

Support configuration of reference signals for measuring CLI according to the aggressor node’s current beamforming, Tx power, etc.

A similar situation applies to the case of self-interference for enhanced duplexing. Since beamforming can change the amount of SI between DU and MT of the IAB node, and since this interference generally depends on the environment, which may be varying in the case of mobile IAB or changing scattering environment, it is desired to devise similar mechanisms for SI measurement. In order to save resource overhead, same reference signals can be used for CLI and SI. Therefore, we propose to consider the different types of interference under the same framework.

Support interference management, including CLI and SI, at least among IAB nodes connected to the same IAB donor. CLI and SI management can be specified under the same framework in order to reduce specification effort, improve implementation flexibility, and save resource overhead for reference signals.

Another interference scenario discussed in the last meeting is the case of interference from IAB nodes on non-IAB cells. This scenario is also of importance because IAB systems are distributed in nature, hence causing varying interference on nearby cells. Existing TDD coordination signaling falls short in conveying this information to nearby cells for interference management. Signaling enhancements between IAB and non-IAB cells can be considered to enable resource coordination and interference management among non-IAB cells and IAB systems. We propose to introduce proper signaling so as to avoid need for “IAB-MT transmission in DL access slots.”

Support interference management among non-IAB cells and IAB systems. No need to introduce IAB-MT transmission in DL access slots in the specification.

# Conclusions

In this contribution, we discussed other enhancements for simultaneous operation of child and parent links of an IAB-node and made the following proposals:

Proposal 1:	Support a capability signalling to indicate whether the IAB node requires timing alignment between IAB-MT and IAB-DU operations. If negative, the IAB node can transmit unaligned OFDM symbols (Case A) and receive/process unaligned OFDM symbols (Case B).

Proposal 2:	Support a capability signaling to indicate whether the IAB node supports enhanced timing alignment. If the feature is not optional, the capability signaling may be the parameter that indicates capability of enhanced (Rel-17) operation by the IAB node.

Proposal 3:	Support configuration and control signaling for applying Case-6 and Case-7 timing alignment.

Proposal 4:	Guard symbols can be used to separate consecutive TX/RX operations when switching timing alignment modes.

Proposal 5:	Define signaling to communicate information of the parent link propagation delay to child IAB nodes.

Proposal 6:	Support a unified framework for uplink timing alignment.

Proposal 7:	No modification to Case-1 timing alignment.

Proposal 8:	Support power control configurations at least for Case A and Case B multiplexing.

Proposal 9:	The downlink power control signaling from the IAB node to the parent node includes a power offset value as well as reference power and spatial information.

Proposal 10:	Parent node can respond to the IAB node indicating a granted power offset as well as reference power and spatial information.

Proposal 11:	Support enhanced DU-based procedures for measurement and reporting. Further consider MT-based procedures.

Proposal 12:	Support timing adjustment for CLI measurements at the victim node based on timing obtained by receiving SSB from the aggressor node.

Proposal 13:	Support CLI for downlink and uplink resources of backhaul links and access links.

Proposal 14:	Support configuration of reference signals for measuring CLI according to the aggressor node’s current beamforming, Tx power, etc.

Proposal 15:	Support interference management, including CLI and SI, at least among IAB nodes connected to the same IAB donor. CLI and SI management can be specified under the same framework in order to reduce specification effort, improve implementation flexibility, and save resource overhead for reference signals.

Proposal 16:	Support interference management among non-IAB cells and IAB systems. No need to introduce IAB-MT transmission in DL access slots in the specification.

# References

- R1-2105763, “Resource multiplexing in enhanced IAB systems,” Lenovo, Motorola Mobility, RAN1#105-e, Electronic Meeting, May 2021.

- RAN1#103-e Chairman’s Notes.

- RAN1#104-e Chairman’s Notes.