3GPP TSG RAN WG1 #105	R1-2105769

e-Meeting, May 10th – 27th, 2021

Agenda item:	8.3.3

Source: 	Lenovo, Motorola Mobility

Title: 	Intra-UE multiplexing enhancement for IIoT/URLLC

Document for:	Discussion

# Introduction

During RAN1 #104bis-e meeting, RAN1 made following agreements regarding intra-UE multiplexing enhancement for  Industrial Internet of Things (IoT) and ultra-reliable and low latency communication (URLLC) [1]:

Agreements:

For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, when the total number of LP and HP HARQ-ACK bits is more than 2, support separate coding for the two HARQ-ACKs.

- FFS for HP HARQ-ACK or LP HARQ-ACK of 1-2 bit(s).

- (working assumption) Drop CSI (including part 1 and part2, if exist) if CSI would multiplex on a PUCCH which has HP A/N.

  - FFS Strive to let HP A/N reuse the encoder, rate matching equation, and RE mapping rules in Rel-15 for A/N+CSI-1.

  - FFS Strive to let LP A/N reuse the encoder, rate matching equation, and mapping rules in Rel-15 for CSI-2.

Agreement:

For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUSCH in R17, support separate coding for the two HARQ-ACKs.

- It is understood that it is intended that the number of encoding chains for all UCI multiplexing combinations in Rel-17 should not exceed that in Rel-15/16.

In this document, we discuss further details for multiplexing UCI of different priorities on PUCCH and on PUSCH, such as determination of a PUCCH resource for mixed priority UCI, determination of the number of PRBs for PUCCH with mixed priority UCI, determination of a PUSCH resource for mixed priority UCI, and handling of SR and CSI in PUSCH.

# Multiplexing mixed priority UCI in PUCCH

## Multiplexing mixed priority UCI in PUCCH formats 0/1

When a UE multiplexes UCI of mixed priorities including up to 2bit HARQ-ACK information with/without a positive HP SR in a PUCCH resource of PUCCH format 0 or 1 (PF0 or PF1) in a slot/sub-slot, the PUCCH resource can be determined based on

- a last DCI format among DCI formats that indicates the slot/sub-slot for the PUCCH transmission and a higher priority index (e.g. the priority indicator field of the DCI format set to 1), or

- a last DCI format among DCI formats indicating the slot/sub-slot for the PUCCH transmission if no DCI format indicating a higher priority index is detected, or

- a PUCCH resource configured for UCI of mixed priorities for up to 2 HARQ-ACK bits with a positive SR, when there is no corresponding DCI format.

In the determined PUCCH resource, LP/HP HARQ-ACK bits and additionally HP SR can be indicated in a same way that HARQ-ACK bits of the same priority and additionally SR of the same priority are indicated in PUCCH format 0 or 1, according to Subclause 9.2.3 and 9.2.5.1 of TS 38.213 [2]. For low-latency HARQ-ACK feedback, the PUCCH resource can be selected from a PUCCH resource set configured by the second PUCCH-Config associated with HARQ-ACK codebook of a higher priority index, even when no DCI format indicating the higher priority index is detected. Further, the UE may receive configuration of a list of PUCCH resources to be used for transmitting UCI of mixed priorities including HARQ-ACK when there is no corresponding DCI format, where the list of PUCCH resources are configured by the second PUCCH-Config.

Proposal 1: A PUCCH resource configured by the second PUCCH-Config for multiplexing UCI of mixed priorities including up to 2bit HARQ-ACK information with/without a positive HP SR is determined based on:

- a last DCI format indicating a higher priority index, or

- a last DCI format if no DCI format indicating a higher priority index is detected, or

- a PUCCH resource configured for UCI of mixed priorities for up to 2 HARQ-ACK bits with a positive SR, when there is no corresponding DCI format.

## Multiplexing mixed priority UCI in PUCCH formats 2/3/4

When a UE would transmit HP UCI in a PUCCH of higher priority index that would overlap in time with a transmission of a PUCCH of lower priority index including LP HARQ-ACK and when the total number of UCI bits is larger than 2 and the UE determines a PUCCH resource of a PUCCH configuration with PUCCH format 2 or PUCCH format 3 for multiplexing at least a part of LP UCI with the HP UCI, the UE can determine the number of PRBs of the PUCCH resource by selecting the minimum number NPRB, minPUCCH of the NPRBPUCCH PRBs satisfying

OHP UCI+OHP,CRC/RHP UCImax/Qm+OLP UCI+OLP,CRC/RLP UCImax/Qm≤NPRB,minPUCCH∙Nsc, ctrlRB∙Nsymb, UCIPUCCH, where

- OHP UCI and OHP, CRC are the number of multiplexed HP UCI bits and the number of HP UCI CRC bits, respectively, for transmission on a PUCCH carrying multiplexed UCI bits of mixed priorities. If the payload size is not larger than a threshold value (e.g. 11 bits), CRC bits may not be attached.

- OLP UCI and OLP, CRC are the number of multiplexed LP UCI bits and the number of LP UCI CRC bits, respectively, for transmission on a PUCCH carrying multiplexed UCI bits of mixed priorities. If the payload size is not larger than a threshold value (e.g. 11 bits), CRC bits may not be attached.

- MPRBPUCCH is the configured maximum number of PRBs for PUCCH format 2 or PUCCH format 3.

- RHP UCImax is the configured maximum PUCCH coding rate for HP UCI.

- RLP UCImax is the configured maximum PUCCH coding rate for LP UCI.

- Nsymb,UCIPUCCH is the number of PUCCH symbols used for UCI transmission (excluding a number of symbols used for DM-RS transmission for PUCCH format 3 or for PUCCH format 4) in a PUCCH.

- Qm is the modulation order of a PUCCH.

- Nsc, ctrlRB is the number of effective subcarriers per resource block, taking into account a spreading overhead or DMRS overhead.

The UE may receive information of one nrofPRBs value for a corresponding PUCCH format (e.g. PUCCH format 2, PUCCH format 3) of the PUCCH configuration, for the multiplexed HP and LP UCI bits. With separate encoding of HP UCI and LP UCI, if UE receives information of separate values of maxCodeRate, a first value for HP UCI and a second value for LP UCI in the PUCCH configuration, the UE may apply corresponding maxCodeRate values to determine the number of REs required for the HP UCI and the number of REs for the multiplexed LP UCI, respectively. If UE receives information of one value of maxCodeRate, the UE may assume that the one value of maxCodeRate is applicable to both the HP UCI and the multiplexed LP UCI.

Proposal 2: Support indicating two values of maxCodeRate, a first value for HP UCI and a second value for LP UCI in a PUCCH configuration corresponding to a high priority index, in order to support different coding rates for HP UCI and LP UCI that are multiplexed in a PUCCH.

If the number of mixed UCI bits satisfies the following (i.e. the total payload consisting of LP UCI and HP UCI is larger than a supportable payload based on configured parameters of the maximum code rates and the maximum number of PRBs), the UE may not multiplex LP UCI with HP UCI in the PUCCH resource at all.

OHP UCI+OHP,CRC/RHP UCImax/Qm+OLP UCI+OLP,CRC/RLP UCImax/Qm>NPRBPUCCH∙Nsc, ctrlRB∙Nsymb, UCIPUCCH

That is, UE may determine whether to multiplex LP UCI (i.e. at least LP HARQ-ACK) in a PUCCH resource of PUCCH format 2, 3, or 4 of higher priority index, based on

- whether the total payload consisting of LP UCI and HP UCI is less than a supportable payload based on configured parameters of the maximum code rate(s) and the maximum number of PRBs of a given PUCCH format; and/or

- whether the PUCCH of lower priority index carrying LP UCI is confined within a sub-slot where the transmission of the PUCCH of higher priority index would occur.

For the case that LP HARQ-ACK is not multiplexed with HP UCI due to the large payload size, if the PUCCH of lower priority index carrying LP HARQ-ACK is not confined within the sub-slot (e.g. sub-slot n) where the transmission of the PUCCH of higher priority index carrying HP UCI would occur, the UE may further determine a separate PUCCH resource for LP HARQ-ACK that would not overlap (e.g., in time) with the PUCCH of higher priority index for HP UCI and performs a separate transmission on the separate PUCCH resource.

For example, the separate PUCCH resource for LP HARQ-ACK is in a sub-slot (e.g. sub-slot n+x, where x is a positive integer) following the sub-slot where the transmission of the PUCCH of higher priority index occurs. If the PUCCH of low priority index overlaps with a transmission of another PUCCH of higher priority index for another HP UCI in the next sub-slot (e.g. sub-slot n+1), decision for multiplexing may be based on the total UCI payload size in sub-slot (n+1) and/or whether the PUCCH of lower priority index is confined within the sub-slot (n+1). Thus, the UE can handle the case that the PUCCH of lower priority index overlaps in time with more than one PUCCH of higher priority index.

Proposal 3: UE determines whether to multiplex LP HARQ-ACK with HP UCI in a PUCCH resource of PUCCH format 2, 3, or 4 of higher priority index, based on the total UCI payload size and configured max. code rate/max PRB parameters.

Proposal 4: If LP HARQ-ACK not multiplexed due to payload size limitation, UE can further check possible multiplexing in the next sub-slot, unless a PUCCH of low priority index for LP HARQ-ACK is limited up to a current sub-slot.

# Multiplexing mixed priority UCI in PUSCH

## PUSCH resource determination

When a UE would transmit HP UCI and LP UCI in overlapping (in time domain) PUCCHs, respectively, and when a PUCCH resource determined to multiplex HP UCI and at least a part of LP UCI overlaps in time with one or multiple PUSCHs, the UE multiplexes the HP UCI and the LP UCI into a PUSCH of the one or multiple PUSCHs. If the one or multiple PUSCHs include at least one HP PUSCH (i.e. a PUSCH of a higher physical layer priority index), the PUSCH is selected from the at least one HP PUSCH. If the one or multiple PUSCHs consist of only LP PUSCHs (i.e. PUSCHs of a lower physical layer priority index), the PUSCH is selected from the LP PUSCHs according to Rel-15 PUSCH selection rules.

Proposal 5: If a PUCCH resource to multiplex HP UCI and LP UCI overlaps in time with one or multiple PUSCHs including at least one HP PUSCH, a PUSCH to multiplex the HP UCI and the LP UCI is selected from the at least one HP PUSCH.

Proposal 6: If a PUCCH resource to multiplex HP UCI and LP UCI overlaps in time with one or multiple PUSCHs without at least one HP PUSCH, a PUSCH to multiplex the HP UCI and the LP UCI is selected according to Rel-15 PUSCH selection rules.

## Handling of SR and CSI

If a UE would transmit UL-SCH on a selected PUSCH of a given physical layer priority (i.e. selected for multiplexing mixed priority UCI of PUCCH), the UE may not multiplex SR of the given physical layer priority into the PUSCH of the given physical layer priority but may multiplex SR of a physical layer priority different than the given physical layer priority into the PUSCH. For example, the UE may jointly encode SR and HARQ-ACK of the priority index 0 and multiplex them in the PUSCH of the priority index 1.

Proposal 7: UE does not multiplex SR of a given physical layer priority into a PUSCH of the given physical layer priority but may multiplex SR of a physical layer priority different than the given physical layer priority into the PUSCH.

If a UE would transmit semi-persistent or aperiodic CSI on a selected PUSCH of a given physical layer priority (i.e. selected for multiplexing mixed priority UCI of PUCCH), the UE may multiplex the semi-persistent or aperiodic CSI with the mixed priority UCI of PUCCH in the selected PUSCH. The priority of the semi-persistent or aperiodic CSI is determined based on the priority of the selected PUSCH. When multiplexing HP UCI (e.g. HP HARQ-ACK, SR, CSI, and/or CG-UCI) and LP UCI (e.g. LP HARQ-ACK, SR, CSI, and/or CG-UCI) in the selected PUSCH, the following alternatives to encode mixed priority UCI can be considered:

- Alt 1: the UE performs first encoding jointly with HP HARQ-ACK/SR/CG-UCI, second encoding jointly with LP HARQ-ACK/SR/CG-UCI, and third encoding with CSI (including both CSI-part1 and CSI-part2, if CSI consists of two parts);

- Alt 2:

  - If the semi-persistent or aperiodic CSI is of a lower physical layer priority (i.e. PUSCH of priority index 0), the UE performs first encoding jointly with HP HARQ-ACK/SR, second encoding jointly with LP HARQ-ACK/CG-UCI/LP CSI-part1, and third encoding jointly with LP CSI-part2.

  - If the semi-persistent or aperiodic CSI is of a higher physical layer priority (i.e. PUSCH of priority index 1), the UE performs first encoding jointly with HP HARQ-ACK/CG-UCI, second encoding with HP CSI-part1, and third encoding jointly with LP HARQ-ACK/SR/HP CSI-part2.

The UE performs rate-matching of coded bits according to the resource allocation priority order of 1st priority for first bits from the first encoding, 2nd priority for second bits from the second encoding, and 3rd priority for third bits from the third encoding.

Proposal 8: If a UE would transmit semi-persistent or aperiodic CSI on a PUSCH determined for multiplexing mixed priority UCI of PUCCH, the UE may multiplex the semi-persistent or aperiodic CSI with the mixed priority UCI of PUCCH in the PUSCH.

Proposal 9: Further study encoding and rate-matching when multiplexing semi-persistent/aperiodic CSI of a PUSCH with mixed priority UCI of a PUCCH in the PUSCH.

# Conclusion

In summary, we observe and propose the followings for intra-UE multiplexing enhancement for Rel-17 IIoT/URLLC:

- Proposal 1: A PUCCH resource configured by the second PUCCH-Config for multiplexing UCI of mixed priorities including up to 2bit HARQ-ACK information with/without a positive HP SR is determined based on:

  - a last DCI format indicating a higher priority index, or

  - a last DCI format if no DCI format indicating a higher priority index is detected, or

  - a PUCCH resource configured for UCI of mixed priorities for up to 2 HARQ-ACK bits with a positive SR, when there is no corresponding DCI format.

- Proposal 2: Support indicating two values of maxCodeRate, a first value for HP UCI and a second value for LP UCI in a PUCCH configuration corresponding to a high priority index, in order to support different coding rates for HP UCI and LP UCI that are multiplexed in a PUCCH.

- Proposal 3: UE determines whether to multiplex LP HARQ-ACK with HP UCI in a PUCCH resource of PUCCH format 2, 3, or 4 of higher priority index, based on the total UCI payload size and configured max. code rate/max PRB parameters.

- Proposal 4: If LP HARQ-ACK not multiplexed due to payload size limitation, UE can further check possible multiplexing in the next sub-slot, unless a PUCCH of low priority index for LP HARQ-ACK is limited up to a current sub-slot.

- Proposal 5: If a PUCCH resource to multiplex HP UCI and LP UCI overlaps in time with one or multiple PUSCHs including at least one HP PUSCH, a PUSCH to multiplex the HP UCI and the LP UCI is selected from the at least one HP PUSCH.

- Proposal 6: If a PUCCH resource to multiplex HP UCI and LP UCI overlaps in time with one or multiple PUSCHs without at least one HP PUSCH, a PUSCH to multiplex the HP UCI and the LP UCI is selected according to Rel-15 PUSCH selection rules.

- Proposal 7: UE does not multiplex SR of a given physical layer priority into a PUSCH of the given physical layer priority but may multiplex SR of a physical layer priority different than the given physical layer priority into the PUSCH.

- Proposal 8: If a UE would transmit semi-persistent or aperiodic CSI on a PUSCH determined for multiplexing mixed priority UCI of PUCCH, the UE may multiplex the semi-persistent or aperiodic CSI with the mixed priority UCI of PUCCH in the PUSCH.

- Proposal 9: Further study encoding and rate-matching when multiplexing semi-persistent/aperiodic CSI of a PUSCH with mixed priority UCI of a PUCCH in the PUSCH.

# References

- “RAN1 Chairman’s Notes”, RAN1 104bis-e meeting, April 12th – 20th, 2021.

- 3GPP TS 38.213 V16.4.0 (2020-12), “Physical layer procedures for control (Release 16)”.

- 3GPP TS 38.212 V16.2.0 (2020-06), “Multiplexing and channel coding (Release 16)”.