3GPP TSG-RAN WG1 Meeting #105-e	R1-2105920

# e-Meeting, May 10 – May 27, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [DRAFT] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | XXX | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on resource exclusion for other TBs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Core |  |  |  |  |  | Date: |  |  | 2021-05-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | During the resource exclusion procedure for mode 2 in TS 38.214, a candidate single-slot resource Rx,y would be excluded from the set of all the candidate single-slot resources due to non-monitored slots and the slots on which UE received an SCI from another UE. For the slots which UE has not monitored, i.e., UE transmitted on these slots, the hypothetical reservation considers any periodicity value allowed by the higher layer parameter sl-ResourceReservePeriodList. In this case, both the reservation from the sensing UE itself and the reservation from the other transmitting UEs are included. Hence, only the resources which have been selected and periodically reserved for different TBs can be excluded by step 5). The resources which have been pre-selected but not reserved, and the resources which have not been periodically reserved, are not excluded by the current resource determination procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a condition which considers the remaining pre-selected resources for other TBs after step 5). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Collision caused by the resources which have been pre-selected but not reserved, and the resources which have not been periodically reserved cannot be avoided. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

--------------------------- Start of Text Proposal for TS 38.214 ------------------------

<Unchanged parts omitted>

8.1.4    UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2

In resource allocation mode 2, the higher layer can request the UE to determine a subset of resources from which the higher layer will select resources for PSSCH/PSCCH transmission. To trigger this procedure, in slot n, the higher layer provides the following parameters for this PSSCH/PSCCH transmission:

<Unchanged parts omitted>

-	if the higher layer requests the UE to determine a subset of resources from which the higher layer will select resources for PSSCH/PSCCH transmission as part of re-evaluation or pre-emption procedure, the higher layer provides a set of resources (r0,r1,r2,…) which may be subject to re-evaluation and a set of resources (r0',r1',r2',…) which may be subject to pre-emption.

-	it is up to UE implementation to determine the subset of resources as requested by higher layers before or after the slot ri'' - T3, where ri'' is the slot with the smallest slot index among (r0,r1,r2,…) and (r0',r1',r2',…) , and T3 is equal to Tproc,1SL, where Tproc,1SL  is defined in slots in Table 8.1.4-2 where μSL is the SCS configuration of the SL BWP.

- the higher layer provides a set of resources (r0,r1,r2,…) which were selected before n for PSSCH/PSCCH transmission.

The following steps are used:

<Unchanged parts omitted>

4)	The set SA is initialized to the set of all the candidate single-slot resources.

5)	The UE shall exclude any candidate single-slot resource Rx,y from the set SA if it meets all the following conditions:

-	the UE has not monitored slot t'mSL in Step 2.

-	for any periodicity value allowed by the higher layer parameter sl-ResourceReservePeriodList and a hypothetical SCI format 1-A received in slot t'mSL with 'Resource reservation period' field set to that periodicity value and indicating all subchannels of the resource pool in this slot, condition c in step 6 would be met.

5-1)	For remaining resources from the set (r0,r1,r2,…) which have not been excluded by step 5-1), the UE shall exclude any candidate single-slot resource Rx,y from the set SA if it meets all the following conditions:

a) Remaining resource from the set (r0,r1,r2,…)  overlaps with Rx,y+j×Prsvp_TX' for j=0, 1, …, Cresel-1

<Unchanged parts omitted>

------------------------------------End of Text Proposal -------------------------------