3GPP TSG RAN WG1 Meeting #105-e                                                                     R1-2106102

e-Meeting, May 10th – 27th, 2021

Source:              ZTE

Title:                   Draft Text Proposals for 38.212 on TEI-17 CW mapping

Agenda Item:     8.16

Document for:   Discussion and Decision

Introduction

In Rel-17 TEI agenda, we propose to enhance the current NR codeword mapping for PUSCH transmission. The proposal is shown as follows.

Proposal: Support 2 codewords with 2 MCS for rank 2-4 uplink transmission (i.e. LTE-like CW mapping)

The detailed test case, simulation results and analysis can be found in our companion contribution [1]. Based on the above TEI proposal, we provide the corresponding text proposals for 38.212 [2] in section 2 of this contribution.

Proposed TPs

- 7.3.1.1.2	Format 0_1

DCI format 0_1 is used for the scheduling of one or multiple PUSCH in one cell, or indicating CG downlink feedback information (CG-DFI) to a UE.

<Unchanged part is omitted>

-	1 bit according to Table 7.3.1.1.1-3 otherwise, only applicable to resource allocation type 1, as defined in Clause 6.3 of [6, TS 38.214].

For transport block 1:

-	Modulation and coding scheme – 5 bits as defined in Clause 6.1.4.1 of [6, TS 38.214]

-	New data indicator – 1 bit if the number of scheduled PUSCH indicated by the Time domain resource assignment field is 1; otherwise 2, 3, 4, 5, 6, 7 or 8 bits determined based on the maximum number of schedulable PUSCH among all entries in the higher layer parameter pusch-TimeDomainAllocationListForMultiPUSCH, where each bit corresponds to transport block 1 of one scheduled PUSCH as defined in clause 6.1.4 in [6, TS 38.214].

-	Redundancy version – – number of bits determined by the following:

-	2 bits as defined in Table 7.3.1.1.1-2 if the number of scheduled PUSCH indicated by the Time domain resource assignment field is 1;

-	otherwise 2, 3, 4, 5, 6, 7 or 8 bits determined by the maximum number of schedulable PUSCHs among all entries in the higher layer parameter pusch-TimeDomainAllocationListForMultiPUSCH, where each bit corresponds to transport block 1 of one scheduled PUSCH as defined in clause 6.1.4 in [6, TS 38.214] and redundancy version is determined according to Table 7.3.1.1.2-34.

For transport block 2:

-	Modulation and coding scheme – 0 bits if EnableTwoCodewords-R17 is not configured, otherwise 5 bits as defined in Clause 6.1.4.1 of [6, TS 38.214].

-	New data indicator – 0 bits if EnableTwoCodewords-R17 is not configured, otherwise,

-     1 bit if the number of scheduled PUSCH indicated by the Time domain resource assignment field is 1;

-    otherwise 2, 3, 4, 5, 6, 7 or 8 bits determined based on the maximum number of schedulable PUSCH among all entries in the higher layer parameter pusch TimeDomainAllocationListForMultiPUSCH, where each bit corresponds to transport block 2 of one scheduled PUSCH as defined in clause 6.1.4 in [6, TS 38.214].

-	Redundancy version – – number of bits determined by the following:

-   0 bit if EnableTwoCodewords-R17 is not configured, otherwise

-  2 bits as defined in Table 7.3.1.1.1-2 if the number of scheduled PUSCH indicated by the Time domain resource assignment field is 1;

-	  otherwise 2, 3, 4, 5, 6, 7 or 8 bits determined by the maximum number of schedulable PUSCHs among all entries in the higher layer parameter pusch-TimeDomainAllocationListForMultiPUSCH, where each bit corresponds to transport block 2 of one scheduled PUSCH as defined in clause 6.1.4 in [6, TS 38.214] and redundancy version is determined according to Table 7.3.1.1.2-34.

-	HARQ process number – 4 bits

-	1st downlink assignment index – 1, 2 or 4 bits:

-	1 bit for semi-static HARQ-ACK codebook;

<Unchanged part is omitted>

Conclusion

In Rel-17 TEI agenda, we propose to support 2 codewords with 2 MCS for rank 2-4 uplink transmission (i.e. LTE-like CW mapping) for NR.

In this contribution, we provide the corresponding text proposals for 38.212 in section 2.

References

- R1-2104596  TEI-17 proposal on NR codeword mapping  ZTE, Sanechips, CMCC, China Telecom, China

Unicom, SoftBank, vivo, NTT DOCOMO

- TS 38.212 g50, Multiplexing and channel coding (Release 16), v16.5.0.