3GPP TSG-RAN WG1 Meeting #106-e	Tdoc R1-2107629

e-Meeting, August 16th – 27th, 2021

Agenda Item:	8.14.1

Source:	Ericsson

Title:	Traffic model for XR

Document for:	Discussion

# 1	Introduction

A new SI on XR evaluations for NR was approved at RAN#86 [1]. The objectives of this study item are as follows.

- Confirm XR and Cloud Gaming applications of interest

- Identify the traffic model for each application of interest taking outcome of SA WG4 work as input, including considering different upper layer assumptions, e.g. rendering latency, codec compression capability etc.

- Identify evaluation methodology to assess XR and CG performance along with identification of KPIs of interest for relevant deployment scenarios

- Once traffic model and evaluation methodologies are agreed, carry out performance evaluations towards characterization of identified KPIs.

In this contribution, we discuss the remaining issues of XR traffic modelling.

# 2	XR traffic models

RAN1 has worked throughout the study item to define traffic models for XR. These traffic models should capture important characteristics of “XR traffic”, while still being simple enough to be useful in RAN1 evaluations. To describe video traffic, RAN1 has agreed to use quasi-periodic packet arrivals, where the packet size follows a truncated gaussian distribution. This video traffic model is controlled by a single parameter: the (average) data rate. Agreeing to this compact representation of the video traffic is a great achievement in RAN1. Together with the per-packet latency requirements, this traffic model will stress the radio interface, to discover bottlenecks that can be removed in future NR releases.

Of course, the traffic model and the associated KPIs are not perfect. There are properties of “XR traffic” that are overlooked, properties that will have a large impact on the radio network performance. Here we list a few:

- The chosen bitrates are in general quite high: it is possible to get quite good end-user experience with much lower bitrates. Moreover, end-point rate adaptation with reduce the bitrate in case where the end-user performance drops. As a concrete example, a VR application will not keep sending packets at 30Mbps if too many packets are lost or delayed.

- Application layer feedback is ignored. The applications react to lost or delayed packets by requesting additional information from the encoder. This will lead to bi-directional traffic and will have a large impact on the statistics of the video stream: there will be a correlation in packet sizes.

- The KPIs for lost/delayed packets are at best good guesses. In reality, it may very well be possible to get good end-user quality with more

Both these aspects will have large impact on the end-user quality, and it would of course be beneficial to include them in a video traffic model. However, it would be impossible to perform evaluations in RAN1 using such complicated models.

We thus realize that for good reasons, important traffic properties have been discarded. Smaller refinements of the traffic models in other areas are then less valuable, since the larger inaccuracies remain.

Since traffic properties that greatly impact RAN performance have already been discarded, the value of providing more accurate modelling of DL video is very limited.

In RAN1#105-e, one such smaller refinement was proposed and agreed:

Agreement

For the optional evaluation scenario, two streams of I-frame and P-frame for DL video stream (option 1), the traffic models described in the below table are assumed.

- FFS: Parameter values of α, A, B, C, D, E, F, G, H

  - Including the possibility of using multiple set of parameter values

- For companies who are evaluating this option, it is recommended to evaluate at least the following scenario: AR/VR, 30Mbps, Dense Urban for FR1 and InH for FR2.  It is encouraged to evaluate additional baseline/optional scenarios/configurations.

| Two data streams, i.e. M1 = 2 | Option 1A: slice-based |  |  |  | Option 1B: GOP-based |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | I-stream |  | P-stream |  | I-stream |  | P-stream |
| Packet modelling | Slice-level |  |  |  | Frame-level |  |  |
| Traffic pattern | Both streams are periodic at 60 fps with the same jitter model as for single stream. |  |  |  | Follow the GOP structure, where GOP size K = 8 with the same jitter model as for single stream. |  |  |
| Number of packets per stream at a time | 1 |  | N-1 |  | I-frame: 1 or 0 P-frame: 0 or 1 At each time instant, there is either only one I-stream packet or only one P-stream packet |  |  |
|  | N = 8: the number of slices per frame. |  |  |  |  |  |  |
| Average data rate per stream |  |  |  |  | RI=R*αK-1+α |  | RP=R*K-1K-1+α |
|  | - R: average data rate of a single stream video - α: average size ratio between one I-frame/slice and one P-frame/slice, e.g. α = 1.5, 2, 3 |  |  |  |  |  |  |
| Packet size distribution | Truncated Gaussian distribution |  |  |  |  |  |  |
|  | Mean = RIFPS | Mean = RPFPS*(N-1) |  |  | Mean = RI*KFPS | Mean = RP*KFPS*(K-1) |  |
|  | - [STD, Max, Min]: [10.5, 150, 50]% of Mean packet size - FPS is the frame rate of the single stream video |  |  |  |  |  |  |
| PER, PDB | [PER_I, PER_P] = [A %, B %] [PDB_I, PDB_P] = [C ms, D ms] |  |  | [PER_I, PER_P] = [E %, F %] [PDB_I, PDB_P] = [G ms, H ms] |  |  |  |

Keeping in mind that important factors have already been overlooked, the focus should be on minimizing the additional complexity of the models. This should be considered when we address the FFS: we should focus on simplicity. To this end, we propose

For A, B, C, D, E, F, G, and H, the same values are used as for DL video without I- and P-frame differentiation.

This means that

- For CG and AR/VR, A=B=E=F=1%

- For CG, C=D=G=H=15ms

- For AR/VR, C=D=G=H=10ms

The remaining parameter is α. In reality, α can vary a lot: it depends on the type of content. Again, there is little point in aiming for the highest fidelity. It makes more sense to aim for a value that may trigger new bottlenecks or lead to different results. This would suggest that we use a large value of α, and hence we propose

The average size ratio between one I-frame/slice and one P-frame/slice is 3.

# Conclusion

In the previous sections we made the following observations:

Observation 1	Since traffic properties that greatly impact RAN performance have already been discarded, the value of providing more accurate modelling of DL video is very limited.

Based on the discussion in the previous sections we propose the following:

Proposal 1	For A, B, C, D, E, F, G, and H, the same values are used as for DL video without I- and P-frame differentiation.

Proposal 2	The average size ratio between one I-frame/slice and one P-frame/slice is 3.

# References

RP-193241, “New SID on XR Evaluations for NR”, Qualcomm, 3GPP TSG RAN #86, Dec 2019.

TR 26.928 v16.0.0, “Extended Reality in 5G”, March 2020.