3GPP TSG RAN WG1 #106b-e                                                                                                  R1-2108736

e-Meeting, October 11th – 19th, 2021

Agenda Item:	8.14.1

Source:	Huawei, HiSilicon

Title:	Performance evaluation results for XR and Cloud Gaming

Document for:	Discussion and Decision

# Introduction

During RAN1#103-e ~ RAN1#106-e, some agreements on XR (eXtended Reality) and CG (Cloud Gaming) have been made such as traffic model, KPIs, evaluation assumptions/methodologies, etc. [1] [2] [3] [4] [5].

In this contribution, the system level evaluation results on capacity, power and coverage based on the agreed evaluation assumptions/methodologies are provided. The simulation results are also filled into the template excel as attached.

# Simulation Setting

## Traffic Model

### Single-stream model

We consider the statistical traffic models with different parameters for XR and CG services. The packet size and the jitter of the packet arrival time follow the truncated Gaussian distribution. According to previous agreements, the details of the single-stream model for downlink (DL) and uplink (UL) in our simulations are summarized in Table 1.

Table 1. The single-stream traffic model

| Application |  | DL video | UL pose/control | UL video |
| --- | --- | --- | --- | --- |
| Frame per second/Sampling Frequency |  | 60 FPS | 250 Hz | 60 FPS |
| Average data rate |  | 30 Mbps, 45 Mbps | 0.2 Mbps | 10 Mbps |
| Packet size | Distribution | Truncated Gaussian Distribution | Fixed Size | Truncated Gaussian Distribution |
|  | Mean | 62500 Bytes, 93750 Bytes | - | 20833 Bytes |
|  | STD | 0.105*mean | - | 0.105*mean |
|  | Max | 1.5*mean | - | 1.5*mean |
|  | Min | 0.5*mean | - | 0.5*mean |
| Jitter | Distribution | Truncated Gaussian Distribution | - |  |
|  | Mean | 0 ms |  |  |
|  | STD | 2 ms |  |  |
|  | Range | [-4, 4] ms |  |  |

### Multi-stream model

Agreement:

For the optional evaluation scenario, two streams of I-frame and P-frame for DL video stream (option 1), the traffic models described in the below table are assumed.

- FFS: Parameter values of α, A, B, C, D, E, F, G, H

  - Including the possibility of using multiple set of parameter values

- For companies who are evaluating this option, it is recommended to evaluate at least the following scenario: AR/VR, 30Mbps, Dense Urban for FR1 and InH for FR2.  It is encouraged to evaluate additional baseline/optional scenarios/configurations.

| Two data streams, i.e. M1 = 2 | Option 1A: slice-based |  |  |  | Option 1B: GOP-based |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | I-stream |  | P-stream |  | I-stream |  | P-stream |
| Packet modelling | Slice-level |  |  |  | Frame-level |  |  |
| Traffic pattern | Both streams are periodic at 60 fps with the same jitter model as for single stream. |  |  |  | Follow the GOP structure, where GOP size K = 8 with the same jitter model as for single stream. |  |  |
| Number of packets per stream at a time | 1 |  | N-1 |  | I-frame: 1 or 0 P-frame: 0 or 1 At each time instant, there is either only one I-stream packet or only one P-stream packet |  |  |
|  | N = 8: the number of slices per frame. |  |  |  |  |  |  |
| Average data rate per stream | RI=R*αN-1+α |  | RP=R*N-1N-1+α |  | RI=R*αK-1+α |  | RP=R*K-1K-1+α |
|  | - R: average data rate of a single stream video - α: average size ratio between one I-frame/slice and one P-frame/slice, e.g. α = 1.5, 2, 3 |  |  |  |  |  |  |
| Packet size distribution | Truncated Gaussian distribution |  |  |  |  |  |  |
|  | Mean = RIFPS | Mean = RPFPS*(N-1) |  |  | Mean = RI*KFPS | Mean = RP*KFPS*(K-1) |  |
|  | - [STD, Max, Min]: [10.5, 150, 50]% of Mean packet size - FPS is the frame rate of the single stream video |  |  |  |  |  |  |
| PER, PDB | [PER_I, PER_P] = [A %, B %] [PDB_I, PDB_P] = [C ms, D ms] |  |  | [PER_I, PER_P] = [E %, F %] [PDB_I, PDB_P] = [G ms, H ms] |  |  |  |

Agreement

For evaluation of separate streams of I-frame and P-frame that is an optional evaluation scenario,

- Alpha value: 2.0 and 1.5, Other values, e.g., 3.0 can be optionally evaluated

  - This alpha value assumption applies to both Option 1A (slice-based) and Option 1B (GOP-based) evaluations

In RAN1#105-e [3] and RAN1#106-e [5], the following agreement of the DL video with multiple streams is achieved.

In this contribution, we simulate both Option 1A and 1B. Due to the limited time, in this meeting, we provide simulation results for 30 Mbps and 45 Mbps, and alpha=1.5 and 2.0. We’ll continue providing simulation results for more cases in future meeting.

## KPI

For the DL single-stream model, according to the agreement in RAN1#104b-e [2], the following (PER, PDB) combinations are used to evaluate the capacity.

- KPI of  DL single-stream model for VR/AR and CG service

| (PER, PDB) of DL single stream |  |
| --- | --- |
| VR/AR | CG |
| (1%, 10ms) | (1%, 15ms) |
| (0.5%, 10ms) |  |
| (5%, 10ms) |  |
| (1%, 7ms) |  |
| (1%, 13ms) |  |

For the UL single-stream model, according to the agreement in RAN1#105-e [3], the following (PER, PDB) combinations are used to evaluate the capacity.

- KPI of  UL single-stream model for VR/AR and CG service

| (PER, PDB) of UL single stream |  |
| --- | --- |
| Pose/control | Video |
| (1%, 10ms) | (1%, 10ms) |
|  | (1%, 15ms) |
|  | (1%, 30ms) |
|  | (1%, 60ms) |
|  | (5%, 30ms) |
|  | (10%, 30ms) |

For the DL multi-stream model, based on the agreement in RAN1#106-e [5] and discussion in our companion paper [6], the following (PER, PDB) combinations are used as KPI.

- KPI of  DL multi-stream model for VR/AR service

[PER_I, PER_P, PDB_I, PDB_P]

VR/AR

[1%, 1%, 10ms, 10ms]

[0.5%, 5%, 10ms, 10ms]

[1%, 1%, 15ms, 9ms]

[1%, 5%, 10ms, 10ms]

[1%, 1%, 15ms, 10ms]

[1%, 5%, 15ms, 10ms]

[1%, 1%, 17ms, 9ms]

[1%, 1%, 17ms, 10ms]

[1%, 5%, 17ms, 10ms]

## Details and Parameters

We present the evaluation results for FR1 Dense Urban and Urban Macro scenarios. Both DL and UL transmission are dynamically scheduled in our simulation. In the uplink slot, users send ACK/NACK over PUCCH according to the DL decoding result. The overall simulation assumptions are listed in Appendix A.

# Evaluation Results for DL Capacity

According to the agreement in RAN1#103-e [4], system capacity is defined as the maximum number of users per cell with at least 90% (baseline) or 95% (optional) of users being satisfied. Therefore, 90% and 95% user satisfaction ratio is considered in our evaluation. The evaluation is performed over different average numbers of users per cell. Note that the average number of users per cell means the average load per cell. The actual number of users for each cell may be unequal. The system level evaluation results for capacity of VR/AR and CG in Dense Urban and Urban Macro scenarios are shown as follows.

## Single-stream model

### Baseline Performance

#### Results in Dense Urban

This part provides the baseline capacity results in Dense Urban scenario. From Figure 1, it is seen that the network capacity decreases with data rate. For example, in SU-MIMO, the network capacity of VR/AR with data rate 30Mbps is 5.1 users per cell while the network capacity of VR/AR with 45Mbps is 2.1 users per cell. Besides, it is observed that the user satisfaction ratio reduces as the average number of users per cell increases. This is mainly due to the fact that more users per cell results in larger queuing delay.

- SU-MIMO

- MU-MIMO

Figure 1. Capacity results of the single-stream model in Dense Urban

Observation 1: In FR1 Dense Urban, the network capacity of the DL single-stream model is summarized in Table 2.

Table 2. Capacity of the DL single-stream model in FR1 Dense Urban

| Average number of supported users per cell | VR/AR @30Mbps (PER=1%, PDB=10ms) | CG @30Mbps (PER=1%, PDB=15ms) | VR/AR @45Mbps (PER=1%, PDB=10ms) |
| --- | --- | --- | --- |
| SU-MIMO | 5.1 | 7.6 | 2.1 |
| MU-MIMO | 11.5 | 16.1 | 5.3 |

#### Results in Urban Macro

Figure 2 shows the capacity results in Urban Macro scenario. By comparing Figure 1 and Figure 2, it is observed that the network capacity in Urban Macro scenario is generally less than that in Dense Urban scenario due to the larger pathloss in Urban Macro case.

- SU-MIMO

- MU-MIMO

Figure 2. Capacity results of the single-stream model in Urban Macro

Observation 2: In FR1 Urban Macro, the network capacity of the DL single-stream model is summarized in Table 3.

Table 3. Capacity of the DL single-stream model in FR1 Urban Macro

| Average number of supported users per cell | VR/AR @30Mbps (PER=1%, PDB=10ms) | CG @30Mbps (PER=1%, PDB=15ms) | VR/AR @45Mbps (PER=1%, PDB=10ms) |
| --- | --- | --- | --- |
| SU-MIMO | 4.5 | 6.5 | 1.8 |
| MU-MIMO | 9.3 | 12.4 | 4.0 |

### Impact on capacity

#### PER and PDB

Figure 3 gives the capacity results of VR/AR with 30Mbps for different PER requirement under PDB=10ms in Dense Urban MU-MIMO scenario. It is observed that the network can support 11.5 users per cell with PER=1%, PDB=10ms. If the PER requirement is more stringent, i.e. 0.5%, the network capacity decreases to 9.9 users per cell. If the PER requirement is looser, i.e. 5%, the network capacity is larger than 15 users per cell.

Figure 3. Capacity results of the single-stream model with different PER

Figure 4 gives the capacity results of VR/AR with 30Mbps for different PDB under PER=1% in Dense Urban MU-MIMO scenario. It is observed that the network can support 11.5 users per cell with PER=1%, PDB=10ms. If the PDB requirement is more stringent, i.e. 7ms, the network capacity decreases to 6.4 users per cell. If the PDB requirement is looser, i.e. 13ms, the network capacity is 14.6 users per cell.

Figure 4. Capacity results of the single-stream model with different PDB

Observation 3: Relaxing PDB and PER requirement can increase the network capacity as shown in Table 4.

Table 4. Capacity of the DL single-stream model with different (PER, PDB) in FR1 Dense Urban MU-MIMO, AR/VR 30 Mbps

| (PER, PDB) | Average number of supported users per cell |
| --- | --- |
| (0.5%, 10ms) | 9.9 |
| (1%, 10ms) | 11.5 |
| (5%, 10ms) | >15 |
| (1%, 7ms) | 6.4 |
| (1%, 13ms) | 14.6 |

#### Jitter

This part shows the impact of jitter on the network capacity. As shown in Figure 5, if jitter is considered, the network can support 11.5 VR/AR users per cell in Dense Urban MU-MIMO scenario while it can support 11.6 users per cell for no jitter case. The slight difference of the capacity is mainly because jitter may reduce the interval time of two frames and the transmission of the latter frame might be delayed by the former one.

Figure 5. Capacity results of the single-stream model with/without jitter

Observation 4: Jitter will slightly reduce the network capacity as shown in Table 5.

Table 5. Capacity of the DL single-stream model with different jitter configuration in FR1 Dense Urban MU-MIMO, AR/VR 30 Mbps, (1%, 10ms)

| Jitter Configuration | Average number of supported users per cell |
| --- | --- |
| With jitter | 11.5 |
| Without jitter | 11.6 |

#### Benefit of frame level integrated transmission (FLIT)

Agreement:

For XR/CG capacity evaluation, a packet is considered as lost when it has exceeded the PDB, such that it will be added to the PER and the data of the packet is discarded.

- It is up to company to report the details for the packet when it has exceeded the PDB, e.g.

  - Option 1: The packet exceeding the delay is still delivered to the other side

  - Option 2: The packet (including the non-transmitted part) is discarded at the transmitter (at the gNB for DL packets and at the UE for UL packets)

  - Other options are not precluded

- Note: This is for the purpose of evaluation

In RAN1#104b-e, the following agreements on transmission scheme in XR/CG capacity evaluation were achieved [2].

Generally, a video frame can only be decoded and reconstructed correctly if all its associated content have been correctly received within PDB. Inspired by this characteristic of XR/CG services, we further consider an advanced transmission scheme, which takes the frame integrity into consideration:

- Frame Level Integrated Transmission (FLIT): Denote Sall as the size of the current frame for a user, and Ssent as the size of the already sent part of the current frame. Denote Tremaining as the remaining delivery time of the current frame based on PDB. These factors are considered to calculate the scheduling priority of the users, e.g.,

Priority = f(Sall, Ssent, Tremaining,  others).

where “others” represents other conventional parameters used in gNB scheduler such as instantaneous data rate indicated by channel quality indicator, and historical data rate.

The motivation to consider these factors are as follows:

- Sall-Ssent represents the remaining bits of the video frame under transmitting. When Sall-Ssent is small, it means this frame is close to completion and is easier to be transmitted successfully, so that this frame should be prioritized.

- When Tremaining is small, it means this frame is close to deadline and should be prioritized for transmission.

It is expected that a scheduler considering these factors can increase capacity. More details about FLIT can be found in our companion paper [7].

Figure 6 and Figure 7 show the capacity improvement induced by FLIT in Dense Urban SU-MIMO scenario and Dense Urban MU-MIMO scenario respectively.

Figure 6. Capacity results with different transmission schemes in Dense Urban SU-MIMO scenario

Figure 7. Capacity results with different transmission schemes in Dense Urban MU-MIMO scenario

Observation 5: Frame Level Integrated Transmission (FLIT), which considers the following factors during scheduling, can improve the network capacity by 20~30% as shown in Table 6.

- The size of the frame

- The size of the already sent part of the frame

- The remaining delivery time of the frame

Table 6. Capacity of the DL single-stream model of different transmission schemes in Dense Urban

| Cases | Option 2 (Discard frames exceeding PDB) | FLIT (Frame Level Integrated Transmission) | Capacity Improvement of FLIT over Option 2 |
| --- | --- | --- | --- |
| SU-MIMO & VR/AR 30Mbps | 5.1 | 6.4 | 25.5% |
| SU-MIMO & VR/AR 45Mbps | 2.1 | 2.7 | 28.6% |
| MU-MIMO & VR/AR 30Mbps | 11.5 | 14 | 21.7% |
| MU-MIMO & VR/AR 45Mbps | 5.3 | 6.6 | 24.5% |

## Multi-stream model: I/P-stream

This sub-section gives our evaluation results of the multi-stream model of I/P-frame in FR1 Dense Urban MU-MIMO scenario.

### Option 1B: GOP-based traffic model

#### Impact of PER and PDB on capacity

Figure 8. Capacity results of the multi-stream model Option 1B (GOP-based) with different PER and PDB

Figure 8 shows the capacity results of the multi-stream model Option 1B (GOP-based) under different PER and PDB. In the following, we analyze the impact of relaxing the QoS requirement, i.e., PER and PDB, of I-frame and P-frame respectively on the network capacity.

The impact of relaxing PER and PDB of I-frame on capacity

By comparing the results of Case 1 and Case 3, it can be observed that by relaxing the PER requirement of I-frame, the network capacity increases from 6.0 to 6.7 users per cell. By comparing the results of Case 0 Case 4 and Case 7, it is seen that the network capacity is significantly improved from 6.7 users to 10.5 users by relaxing the PDB requirement of I-frame.

Observation 6: The network capacity under DL multi-stream model Option 1B (GOP-based traffic model) can be improved by relaxing the QoS requirement of I-frame:

- relaxing the PER of I-frame can improve the network capacity by ~12%,

- relaxing the PDB of I-frame can significantly improve the network capacity by ~57%.

Table 7. The improvement of the capacity of the DL multi-stream model Option 1B (GOP-based traffic model) by relaxing the QoS requirement of I-frame

| [PER_I, PER_P, PDB_I, PDB_P] | Average number of supported users per cell | Capacity improvement |
| --- | --- | --- |
| Case 1 [0.5%, 5%, 10ms, 10ms] | 6.0 | 11.7% (relaxing PER of I-frame from Case 1 to 3) |
| Case 3 [1%, 5%, 10ms, 10ms] | 6.7 |  |
| Case 0 [1%, 1%, 10ms, 10ms] | 6.7 | 56.7 % (relaxing PDB of I-frame from Case 0 to 7) |
| Case 4 [1%, 1%, 15ms, 10ms] | 9.1 |  |
| Case 7 [1%, 1%, 17ms, 10ms] | 10.5 |  |

The impact of relaxing PER and PDB of P-frame on capacity

By comparing the results of Case 0 and Case 3 where the QoS requirement of I-frame is stringent (i.e. I-frame PDB=10ms), it is observed that relaxing the PER requirement of P-frame almost cannot improve the network capacity. This is because the QoS requirement of I-frame is already stringent, so that I-frame is the bottleneck of capacity. In this case, relaxing the PER requirement of P-frame does not matter so much.

While for the Case 4 and Case 5 (or Case 7 and Case8) where the QoS requirement of I-frame is loose (i.e. I-frame PDB=15ms or 17ms), I-frame is less dominating the capacity, then it is observed relaxing the PER requirement of P-frame has some benefit on the network capacity.

Besides, from Case 2 and Case 4 (or Case 6 and Case 7), it is observed that relaxing the PDB requirement of P-frame can also improve the network capacity.

Observation 7: The network capacity under DL multi-stream model Option 1B (GOP-based traffic model) might be improved by relaxing the QoS requirement of P-frame:

- when the QoS requirement of I-frame is stringent, relaxing the PER requirement of P-frame almost cannot improve the network capacity,

- when the QoS requirement of I-frame is loose, relaxing the PER/PDB requirement of P-frame can improve the network capacity by 3~12%.

Table 8. The improvement of the capacity of the DL multi-stream model Option 1B (GOP-based traffic model) by relaxing the QoS requirement of P-frame

| [PER_I, PER_P,PDB_I, PDB_P] | Average number of supported users per cell | Capacity improvement |
| --- | --- | --- |
| Case 0 [1%, 1%, 10ms, 10ms] | 6.7 | 0% (when QoS of I-frame is stringent, relaxing PER of P-frame from Case 0 to 3) |
| Case 3 [1%, 5%, 10ms, 10ms] | 6.7 |  |
| Case 4 [1%, 1%, 15ms, 10ms] | 9.1 | 5.5% (when QoS of I-frame is loose, relaxing PER of P-frame from Case 4 to 5) |
| Case 5 [1%, 5%, 15ms, 10ms] | 9.6 |  |
| Case 7 [1%, 1%, 17ms, 10ms] | 10.5 | 12.4% (when QoS of I-frame is loose, relaxing PER of P-frame from Case 7 to 8) |
| Case 8 [1%, 5%, 17ms, 10ms] | 11.8 |  |
| Case 2 [1%, 1%, 15ms, 9ms] | 8.8 | 3.4% (when QoS of I-frame is loose, relaxing PDB of P-frame from Case 2 to 4) |
| Case 4 [1%, 1%, 15ms, 10ms] | 9.1 |  |
| Case 6 [1%, 1%, 17ms, 9ms] | 9.5 | 10.5% (when QoS of I-frame is loose, relaxing PDB of P-frame from Case 6 to 7) |
| Case 7 [1%, 1%, 17ms, 10ms] | 10.5 |  |

#### Benefit of prioritizing the transmission of the more important stream and FLIT

In this subsection, we consider three types of scheduling schemes for the multi-stream model:

- Scheme 1 (S1): Proportional fair (PF). The scheduling priority of each user is calculated as the ratio of the instantaneous data rate, indicated by channel quality indicator, over the historical data rate. The ratio is also named as the PF value. Larger PF value means higher scheduling priority.

- Scheme 2 (S2): Based on the PF scheme, the packets of the more important stream (i.e., I-stream in our simulation) are prioritized to be scheduled. That is, if the currently transmitting packet of a user belongs to Stream#1 (I-frame), the PF value of the user is scaled up with a factor.

- Scheme 3 (S3): This scheme applies FLIT technique and further prioritizes to transmit the packets of the more important stream.

Figure 9. Capacity results of the DL multi-stream model Option 1B (GOP-based traffic model) with VR/AR 30Mbps under different scheduling schemes

Figure 10. Capacity results of the DL multi-stream model Option 1B (GOP-based traffic model) with VR/AR 45Mbps under different scheduling schemes

Observation 8: The network capacity of the DL multi-stream Option 1B (GOP-based) can be improved by prioritizing the transmission of the more important stream and applying Frame Level Integrated Transmission (FLIT) as shown in Table 9 and Table 10 .

Table 9. Capacity of the DL multi-stream model Option 1B (GOP-based) with data rate 30Mbps in FR1 Dense Urban MU-MIMO

| Scheduling scheme | S1 (no prioritization between I/P streams) | S2 (prioritize I-stream) | S3 (prioritize I-stream and FLIT) |
| --- | --- | --- | --- |
| Average number of supported users per cell | 6.0 | 7.4 | 8.6 |
| Capacity improvement over S1 | - | 23.3% | 43.3% |

Table 10 Capacity of the DL multi-stream model Option 1B (GOP-based) with data rate 45Mbps in FR1 Dense Urban MU-MIMO

| Scheduling scheme | S1 (no prioritization between I/P streams) | S2 (prioritize I-stream) | S3 (prioritize I-stream and FLIT) |
| --- | --- | --- | --- |
| Average number of supported users per cell | 1.4 | 2.6 | 3.2 |
| Capacity improvement over S1 | - | 85.7% | 128.6% |

#### Size ratio between I-frame and P-frame

Figure 11 shows the capacity results of the multi-stream option 1B (GOP-based traffic model) at different size ratios between I-frame and P-frame. Here, we also plot a capacity result of single-stream model which is obtained from the simulation results of the multi-stream model with α=1 by using KPI (PER=1%, PDB=10ms).

It can be observed that the capacity of multi-stream model with α=1  (red line) is less than that of single-stream model (blue line). The decrease of network capacity is due to the fact that in the single-stream model, we do not distinguish I/P frame and calculate the PER of all packets from two streams, and a user is deemed to be satisfied if NI+NPNall_I+Nall_P≥99% , where NI, NP and Nall_I,Nall_P denote the numbers of successful I/P packets, and the numbers of all I/P packets of a UE, respectively. But in the multi-stream model, the PER of I-stream and P-stream are counted separately and a user is deemed satisfied if both streams are satisfied, i.e. NINall_I≥99% and NPNall_P≥99%, which is more stringent than that in single-stream case.

In addition, it is observed that the network capacity decreases with increasing α since the larger the packet size of I-frame, the harder it can transmitted successfully within the PDB.

Figure 11. Capacity results of the multi-stream option 1B (GOP-based traffic model) with different α

Observation 9: The network capacity of the DL multi-stream Option 1B (GOP-based traffic model) decreases as the size ratio between I-frame and P-frame becomes larger.

### Option 1A: slice-based traffic model

Figure 12 gives the baseline capacity result of multi-stream option 1A slice-based traffic model. From the figure, it is seen that the network can support 15 users per cell by using KPI [PER_I=1%, PER_P=1%, PDB_I=10ms, PDB_P=10ms].

Figure 12. Capacity results of the multi-stream option 1A (slice-based traffic model)

Observation 10: The network capacity of the DL multi-stream Option 1A (Sliced-based traffic model) is summarized in Table 11.

Table 11. Capacity of the DL multi-stream model Option 1A in FR1 Dense Urban MU-MIMO, AR/VR 30 Mbps

| [PER_I, PER_P,PDB_I, PDB_P] | Average number of supported users per cell |
| --- | --- |
| [1%, 1%, 10ms, 10ms] | 14.9 |
| [0.5%, 5%, 10ms, 10ms] | 15.6 |
| [1%, 5%, 10ms, 10ms] | 17.4 |

# Evaluation Results for UL Capacity

Similar to DL evaluation, UL system capacity is defined as the maximum number of users per cell with at least 90% (baseline) of users being satisfied. The evaluation is performed over different average numbers of users per cell. Note that the average number of user per cell means the average load per cell. The actual number of users for each cell may be unequal.  For the UL evaluations, the system level evaluation results for capacity of VR/AR and CG in FR1 Dense Urban and Urban Macro scenarios are shown as follows.

## Single-stream model

### Results for pose/control stream

In this part, pose/control stream for UL CG/VR applications is simulated for capacity evaluation. Figure 13 shows the satisfaction ratio of UEs for the agreed uplink pose/control traffic model parameters under Dense Urban and Urban Macro scenarios. It can be observed that more than 15 UEs per cell can be supported in both Dense Urban scenario and Urban Macro scenario.

Figure 13. Capacity results of the UL pose/control stream in different scenarios.

Observation 11: The network capacity of the UL pose/control stream is summarized in Table 12.

Table 12. Capacity of the UL pose/control stream in different scenarios.

| Scenario | Average number of supported users per cell |
| --- | --- |
| Dense Urban | >15 |
| Urban Macro | >15 |

### Results for video stream

According to the agreement in RAN1#104b-e [2], the results of the single UL video stream (i.e., Option 2) is provided. Figure 14 shows the user satisfaction ratio of AR UL video stream with PDB=30ms in both Dense Urban and Urban Macro scenarios. It can be observed that the network can support 8.1 users per cell in Dense Urban scenario while it can only support less than 1 UE per cell in Urban Macro scenario. This may be due to the larger inter site distance which results in larger path loss in Urban Macro scenario while the maximum transmission power of UEs is the same in these two scenarios.

Figure 14. Capacity results of the AR UL video stream in different scenarios.

Observation 12: The capacity result of UL video stream is summarized in Table 13.

Table 13. Capacity of the AR UL video stream in different scenarios

| Scenario | Average number of supported users per cell |
| --- | --- |
| Dense Urban | 8.1 |
| Urban Macro | < 1 |

### Impact of PER and PDB on capacity

The network capacity of AR UL video stream with different PDBs in Dense Urban scenario is provided in Figure 15. Generally, it can be observed that tighter PDB constraint results in lower capacity. In case of PDB < 30ms, the network capacity increases significantly if the PDB becomes looser. For example, the network can support less than 1 user with PDB=10ms while it can support 5.4 users with PDB=15ms. However, in case of PDB > 30ms, the improvement of network capacity is limited when the PDB becomes looser. For example, the network can support 8.1 users with PDB=30ms while it can only support 8.3 users with PDB=60ms. This is because when PDB < 30ms, the UL source within the PDB is limited. Many video frames cannot obtain enough transmission occasions so that they cannot finish the transmission within the given PDB. In this case, when PDB is looser, the capacity can be improved significantly. When PDB > 30ms, most of video frames already have enough transmission occasions. In this case, the UL system capacity is mainly limited by UEs that have bad channel quality. These UEs can only transmit few bits on each occasions so that they cannot finish transmission even if they obtain more transmission occasions by relaxing PDB to 60ms.

Figure 15. Capacity results of the AR UL video stream with different PDB.

Figure 16 shows the network capacity of AR UL video stream with different PER in Dense Urban scenario. It can be observed that the network can support 8.1 UEs when PER=1%, 8.3 UEs when PER=5%, and 8.4 UEs when PER=10%.

Figure 16. Capacity results of the AR UL video stream with different PER.

Observation 13: The capacity result of UL video stream with different PDB is summarized in Table 14.

- When PDB < 30ms, the network capacity increases significantly if the PDB becomes looser.

- When PDB > 30ms, the improvement of network capacity is limited when the PDB becomes looser.

Observation 14: The capacity result of UL video stream with different PER is summarized in Table 14. Relaxing PER can slightly increase the network capacity when PDB = 30ms.

Table 14. Capacity of the AR UL single video stream with different PER and PDB

| (PER, PDB) | Average number of supported users per cell |
| --- | --- |
| (1%, 10ms) | < 1 |
| (1%, 15ms) | 5.4 |
| (1%, 30ms) | 8.1 |
| (1%, 60ms) | 8.3 |
| (5%, 30ms) | 8.3 |
| (10%, 30ms) | 8.4 |

## Multi-stream model: pose/control + video

In this part, the network capacity of AR UL two-stream is evaluated in Dense Urban MU-MIMO scenario. Stream 1 is the pose/control stream and Stream 2 is the video stream. The parameters of these two streams are given in Table 1. UL pose/control and UL video have different arrival periodicity and PDB, i.e., UL pose/control arrives every 4ms and the PDB is 10ms, UL video arrives every 16.67ms and the PDB is 30ms (baseline). Considering such difference, we simulated two transmission schemes as follows:

- Scheme 1 (S1, UE is not aware of traffic difference): UE is not aware of the current packet belongs to UL pose/control or UL video, so that UL pose/control and UL video are transmitted in a first-in-first-out manner.

- Scheme 2 (S2, UE is aware of traffic difference): UE is aware of the current packet belongs to UL pose/control or UL video, so that UE prioritizes the transmission of UL pose/control since it has tighter PDB requirement.

As observed in Figure 17, under Scheme 1, where the UE is not aware of traffic difference, the network can only support 1.5 UEs per cell. This is mainly because due to the first-in-first-out manner under Scheme 1, many packets belonging to UL pose/control may fail to meet their PDB requirement. While under Scheme 2, where the UE is aware of traffic difference, the network can support up to 5.6 UEs per cell.

Figure 17. Capacity results of the UL multi-stream model (pose/control & video).

Observation 15: The capacity result of UL pose/control and video stream is summarized in Table 15. If the UE is aware of traffic difference, i.e., the UE prioritizes the transmission of UL pose/control over UL video, the network capacity can be largely increased.

Table 15. Capacity of the UL multi-stream model (pose/control & video)

| S1: UE is not aware of traffic difference | S2: UE is aware of traffic difference | Capacity Improvement of S2 over S1 |
| --- | --- | --- |
| 1.5 | 5.6 | 273.33% |

# Evaluation Results for UE power consumption for DL traffic

In this section, the system level evaluation results for UE power consumption of VR/AR/CG in Dense Urban scenario are provided. SU-MIMO is used in the evaluations. The power consumption is evaluated based on single-stream model. In the evaluations, the traffic model is listed as following:

- VR/AR: DL, 30Mbps @60fps, PDB=10ms, PER=1%

- CG: DL, 30Mbps @60fps, PDB=15ms, PER=1%

In this section, baseline (no C-DRX configuration) and C-DRX are evaluated. The existing integer C-DRX cycle cannot align with the XR traffic arrival. 10ms is the minimum value of long DRX cycle configuration supported by the current specification and it is considered to be evaluated for XR traffic. To be close to the periodicity of XR traffic, 16ms of DRX cycle is also considered. The parameters of C-DRX are shown in Table 16.

Table 16. C-DRX configurations

| DRX cycle (ms) | onDuration Timer (ms) | Inactivity Timer(ms) |
| --- | --- | --- |
| 10 | 5 | 4 |
| 10 | 8 | 4 |
| 16 | 8 | 8 |
| 16 | 14 | 4 |

For VR/AR@30Mbps, the network capacity for the baseline is 5.1 UEs on average per cell. For CG@30Mbps, the network capacity for the baseline is 7.6 UEs on average per cell. In the evaluations, N=3 or 5 UEs on average are assumed within a cell for VR/AR traffic and N=3 or 7 are assumed for CG traffic. In our evaluations, the power consumption of PDCCH-only, PDCCH+PDSCH, PUCCH for HARQ-ACK and different states of sleep are included. The power model of PDCCH-only, PDCCH+PDSCH and sleep state in TR 38.840 are reused. In our evaluations, it is assumed that power value for S slot is same as that for DL slot. For simplicity, it is assumed that the power value of PUCCH for HARQ-ACK is the value of long PUCCH at 23dBm. Based on the assumptions and C-DRX configurations mentioned above, the following observations are provided.

Observation 16: For DL VR/AR@30Mbps in dense urban, the power saving gains are summarized in Table 17.

Table 17. Power saving gain for dense urban, DL VR/AR@30Mbps

| Power saving scheme | mean PS gain when all UEs are considered | #satisfied UEs per cell / #UEs per cell |
| --- | --- | --- |
| Baseline | - | 4.6/5 |
| C-DRX(10,5,4) | 14.68% | 3.0/5 |
| C-DRX(10,8,4) | 5.53% | 4.4/5 |
| C-DRX(16,8,8) | 10.70% | 0/5 |
| C-DRX(16,14,4) | 3.46% | 4.5/5 |
| Baseline | - | 3.0/3 |
| C-DRX(10,5,4) | 15.24% | 2.4/3 |
| C-DRX(10,8,4) | 5.76% | 2.9/3 |
| C-DRX(16,8,8) | 11.01% | 0/3 |
| C-DRX(16,14,4) | 3.57% | 2.9/3 |

Observation 17: For DL CG@30Mbps in dense urban, the power saving gains are summarized in Table 18 .

Table 18. Power saving gain for dense urban, DL CG@30Mbps

| Power saving scheme | mean PS gain when all UEs are considered | #satisfied UEs per cell / #UEs per cell |
| --- | --- | --- |
| Baseline | - | 6.3/7 |
| C-DRX(10,5,4) | 13.83% | 5.5/7 |
| C-DRX(10,8,4) | 5.26% | 6.3/7 |
| C-DRX(16,8,8) | 9.71% | 5.2/7 |
| C-DRX(16,14,4) | 3.30% | 6.3/7 |
| Baseline | - | 3.0/3 |
| C-DRX(10,5,4) | 15.20% | 3.0/3 |
| C-DRX(10,8,4) | 5.75% | 3.0/3 |
| C-DRX(16,8,8) | 10.79% | 2.9/3 |
| C-DRX(16,14,4) | 3.57% | 3.0/3 |

# Evaluation Results for Coverage

In this section, the coupling gain of satisfied UEs is provided to analyze the coverage performance of XR traffic. Both optional methodology 1 and optional methodology 2 as agreed in RAN1#106-e [5] are considered and summarize as below:

- Optional methodology 1 (B UEs per cell): coverage is defined to be the A-percentile point in CDF of coupling gain for the “satisfied” UEs, with #UEs per cell = B, for a given XR application in a given deployment scenario. The CDF curves of coupling gain for the “satisfied” UEs with B=1 and B=capacity are provided, and the values at A=5 percentile point are reported. Note that when B=capacity, the average number of UEs per cell is set as the largest integer not greater than capacity, and this value might be different in different applications, scenarios, etc.

- Optional methodology 2 (One UE in the entire network): only one UE is dropped randomly in the entire network that is associated with one of the 3 center cells (or gNBs), i.e., only one of the center gNBs is activated. The CDF curves of coupling gain for the “satisfied” UEs for different XR services are provided, and the values at X=5%-tile point are reported.

As agreed for capacity evaluation, the system capacity is defined as the maximum number of users per cell with at least X% of users being satisfied. In this section, we consider X=90. The evaluation is performed over different average numbers of users per cell. Note that the average number of user per cell means the average load per cell. The actual number of users for each cell may be unequal. MU-MIMO is used in the evaluations. The evaluation results of coverage for DL and UL in Urban Macro scenario are shown as follows.

## DL Coverage

In this subsection, the coverage evaluation results for DL are provided.

### Results for Optional methodology 2 (One UE in the entire network)

The CDF of coupling gain for DL with Optional methodology 2 (One UE in the entire network) is shown in Figure 18.

Figure 18. CDF of coupling gain for DL with Optional methodology 2 (One UE in the entire network).

Observation 18: The 5% points of the coupling gain CDF for DL with Optional methodology 2 (One UE in the entire network) are summarized in Table 19.

Table 19. The 5% points of the coupling gain CDF for DL with Optional methodology 2 (One UE in the entire network).

| Case | 5% points of the Coupling gain |
| --- | --- |
| CG 30Mbps, (1%, 15ms), satisfied UE | -146.88 dB |
| VR/AR 30Mbps, (1%, 10ms), satisfied UE | -145.33 dB |
| VR/AR 45Mbps, (1%, 10ms), satisfied UE | -143.85 dB |

### Results for Optional methodology 1 (B UEs per cell)

Figure 19 shows the CDF of coupling gain for DL with Optional methodology 1 (B UEs per cell) when B=1, while Figure 20 shows the CDF of coupling gain for DL with Optional methodology 1 (B UEs per cell) when B=Capacity.

Figure 19. CDF of coupling gain for DL with Optional methodology 1 (B UEs per cell, B=1).

Figure 20. CDF of coupling gain for DL with Optional methodology 1 (B UEs per cell, B=Capacity).

Observation 19: The 5% points of the coupling gain CDF for DL with Optional methodology 1 (B UEs per cell) are summarized in Table 20.

Table 20. The 5% points of the coupling gain CDF for DL with Optional methodology 1 (B UEs per cell).

| Application | 5% points of the Coupling gain (B=1) | 5% points of the Coupling gain (B=Capacity) |
| --- | --- | --- |
| CG 30Mbps, (1%, 15ms), satisfied UE | -137.19 dB | -134.38 dB |
| VR/AR 30Mbps, (1%, 10ms), satisfied UE | -137.19 dB | -132.86 dB |
| VR/AR 45Mbps, (1%, 10ms), satisfied UE | -136.58 dB | -132.95 dB |

## UL Coverage

### Results for Optional methodology 2 (One UE in the entire network)

The CDF of coupling gain for UL with Optional methodology 2 (One UE in the entire network) is shown in Figure 21.

Figure 21. CDF of coupling gain for UL with Optional methodology 2 (One UE in the entire network).

Observation 20: The 5% points of the coupling gain CDF for UL with Optional methodology 2 (One UE in the entire network) are summarized in Table 21.

Table 21. The 5% points of the coupling gain CDF for Optional methodology 2 (One UE in the entire network).

| Application | 5% points of the Coupling gain |
| --- | --- |
| AR 10Mbps, (1%,15ms), satisfied UE | -119.85 dB |
| AR 10Mbps, (1%,30ms), satisfied UE | -122.57 dB |
| AR 10Mbps, (1%,60ms), satisfied UE | -123.21 dB |
| VR/CG 0.2Mbps, (1%, 10ms), satisfied UE | -137.81 dB |

### Results for Optional methodology 1 (B UEs per cell)

Figure 22 shows the CDF of coupling gain for all satisfied UEs for UL with Optional methodology 1 (B UEs per cell) when B=1. Since the network can only support less than 1 UE per cell in Urban Macro scenario, the CDF of coupling gain for B=Capacity is not provided.

Figure 22. CDF of coupling gain for UL with Optional methodology 1 (B UEs per cell, B=1).

Observation 21: The 5% points of the coupling gain CDF for UL with Optional methodology 1 (B UEs per cell, B=1) are summarized in Table 22.

Table 22. The 5% points of the coupling gain CDF for UL with Optional methodology 1 (B UEs per cell, B=1).

| Application | 5% points of the Coupling gain |
| --- | --- |
| AR 10Mbps, (1%,15ms), satisfied UE | -118.91 dB |
| AR 10Mbps, (1%,30ms), satisfied UE | -121.61 dB |
| AR 10Mbps, (1%,60ms), satisfied UE | -122.55 dB |
| VR/CG 0.2Mbps, (1%, 10ms), satisfied UE | -136.01 dB |

# Conclusions

In this contribution, the system level evaluation results based on the agreed evaluation assumptions/methodologies are provided. The following observations are made:

Observation 1: In FR1 Dense Urban, the network capacity of the DL single-stream model is summarized in Table 2.

Table 2. Capacity of the DL single-stream model in FR1 Dense Urban

| Average number of supported users per cell | VR/AR @30Mbps (PER=1%, PDB=10ms) | CG @30Mbps (PER=1%, PDB=15ms) | VR/AR @45Mbps (PER=1%, PDB=10ms) |
| --- | --- | --- | --- |
| SU-MIMO | 5.1 | 7.6 | 2.1 |
| MU-MIMO | 11.5 | 16.1 | 5.3 |

Observation 2: In FR1 Urban Macro, the network capacity of the DL single-stream model is summarized in Table 3.

Table 3. Capacity of the DL single-stream model in FR1 Urban Macro

| Average number of supported users per cell | VR/AR @30Mbps (PER=1%, PDB=10ms) | CG @30Mbps (PER=1%, PDB=15ms) | VR/AR @45Mbps (PER=1%, PDB=10ms) |
| --- | --- | --- | --- |
| SU-MIMO | 4.5 | 6.5 | 1.8 |
| MU-MIMO | 9.3 | 12.4 | 4.0 |

Observation 3: Relaxing PDB and PER requirement can increase the network capacity as shown in Table 4.

Table 4. Capacity of the DL single-stream model with different (PER, PDB) in FR1 Dense Urban MU-MIMO, AR/VR 30 Mbps

| (PER, PDB) | Average number of supported users per cell |
| --- | --- |
| (0.5%, 10ms) | 9.9 |
| (1%, 10ms) | 11.5 |
| (5%, 10ms) | >15 |
| (1%, 7ms) | 6.4 |
| (1%, 13ms) | 14.6 |

Observation 4: Jitter will slightly reduce the network capacity as shown in Table 5.

Table 5. Capacity of the DL single-stream model with different jitter configuration in FR1 Dense Urban MU-MIMO, AR/VR 30 Mbps, (1%, 10ms)

| Jitter Configuration | Average number of supported users per cell |
| --- | --- |
| With jitter | 11.5 |
| Without jitter | 11.6 |

Observation 5: Frame Level Integrated Transmission (FLIT), which considers the following factors during scheduling, can improve the network capacity by 20~30% as shown in Table 6.

Table 6. Capacity of the DL single-stream model of different transmission schemes in Dense Urban

| Cases | Option 2 (Discard frames exceeding PDB) | FLIT (Frame Level Integrated Transmission) | Capacity Improvement of FLIT over Option 2 |
| --- | --- | --- | --- |
| SU-MIMO & VR/AR 30Mbps | 5.2 | 6.4 | 23.1% |
| SU-MIMO & VR/AR 45Mbps | 2.1 | 2.7 | 28.6% |
| MU-MIMO & VR/AR 30Mbps | 11.5 | 14 | 21.7% |
| MU-MIMO & VR/AR 45Mbps | 5.3 | 6.6 | 24.5% |

Observation 6: The network capacity under DL multi-stream model Option 1B (GOP-based traffic model) can be improved by relaxing the QoS requirement of I-frame:

- relaxing the PER of I-frame can improve the network capacity by ~12%,

- relaxing the PDB of I-frame can significantly improve the network capacity by ~57%.

Table 7. The improvement of the capacity of the DL multi-stream model Option 1B (GOP-based traffic model) by relaxing the QoS requirement of I-frame

| [PER_I, PER_P, PDB_I, PDB_P] | Average number of supported users per cell | Capacity improvement |
| --- | --- | --- |
| Case 1 [0.5%, 5%, 10ms, 10ms] | 6.0 | 11.7% (relaxing PER of I-frame from Case 1 to 3) |
| Case 3 [1%, 5%, 10ms, 10ms] | 6.7 |  |
| Case 0 [1%, 1%, 10ms, 10ms] | 6.7 | 56.7 % (relaxing PDB of I-frame from Case 0 to 7) |
| Case 4 [1%, 1%, 15ms, 10ms] | 9.1 |  |
| Case 7 [1%, 1%, 17ms, 10ms] | 10.5 |  |

Observation 7: The network capacity under DL multi-stream model Option 1B (GOP-based traffic model) might be improved by relaxing the QoS requirement of P-frame:

Table 8. The improvement of the capacity of the DL multi-stream model Option 1B (GOP-based traffic model) by relaxing the QoS requirement of P-frame

| [PER_I, PER_P,PDB_I, PDB_P] | Average number of supported users per cell | Capacity improvement |
| --- | --- | --- |
| Case 0 [1%, 1%, 10ms, 10ms] | 6.7 | 0% (when QoS of I-frame is stringent, relaxing PER of P-frame from Case 0 to 3) |
| Case 3 [1%, 5%, 10ms, 10ms] | 6.7 |  |
| Case 4 [1%, 1%, 15ms, 10ms] | 9.1 | 5.5% (when QoS of I-frame is loose, relaxing PER of P-frame from Case 4 to 5) |
| Case 5 [1%, 5%, 15ms, 10ms] | 9.6 |  |
| Case 7 [1%, 1%, 17ms, 10ms] | 10.5 | 12.4% (when QoS of I-frame is loose, relaxing PER of P-frame from Case 7 to 8) |
| Case 8 [1%, 5%, 17ms, 10ms] | 11.8 |  |
| Case 2 [1%, 1%, 15ms, 9ms] | 8.8 | 3.4% (when QoS of I-frame is loose, relaxing PDB of P-frame from Case 2 to 4) |
| Case 4 [1%, 1%, 15ms, 10ms] | 9.1 |  |
| Case 6 [1%, 1%, 17ms, 9ms] | 9.5 | 10.5% (when QoS of I-frame is loose, relaxing PDB of P-frame from Case 6 to 7) |
| Case 7 [1%, 1%, 17ms, 10ms] | 10.5 |  |

Observation 8: The network capacity of the DL multi-stream Option 1B (GOP-based) can be improved by prioritizing the transmission of the more important stream and applying Frame Level Integrated Transmission (FLIT) as shown in Table 9 and Table 10 .

Table 9. Capacity of the DL multi-stream model Option 1B (GOP-based) with data rate 30Mbps in FR1 Dense Urban MU-MIMO

| Scheduling scheme | S1 (no prioritization between I/P streams) | S2 (prioritize I-stream) | S3 (prioritize I-stream and FLIT) |
| --- | --- | --- | --- |
| Average number of supported users per cell | 6.0 | 7.4 | 8.6 |
| Capacity improvement over S1 | - | 23.3% | 43.3% |

Table 10 Capacity of the DL multi-stream model Option 1B (GOP-based) with data rate 45Mbps in FR1 Dense Urban MU-MIMO

| Scheduling scheme | S1 (no prioritization between I/P streams) | S2 (prioritize I-stream) | S3 (prioritize I-stream and FLIT) |
| --- | --- | --- | --- |
| Average number of supported users per cell | 1.4 | 2.6 | 3.2 |
| Capacity improvement over S1 | - | 85.7% | 128.6% |

Observation 9: The network capacity of the DL multi-stream Option 1B (GOP-based traffic model) decreases as the size ratio between I-frame and P-frame becomes larger.

Observation 10: The network capacity of the DL multi-stream Option 1A (Sliced-based traffic model) is summarized in Table 11.

Table 11. Capacity of the DL multi-stream model Option 1A in FR1 Dense Urban MU-MIMO, AR/VR 30 Mbps

| [PER_I, PER_P,PDB_I, PDB_P] | Average number of supported users per cell |
| --- | --- |
| [1%, 1%, 10ms, 10ms] | 14.9 |
| [0.5%, 5%, 10ms, 10ms] | 15.6 |
| [1%, 5%, 10ms, 10ms] | 17.4 |

Observation 11: The network capacity of the UL pose/control stream is summarized in Table 12.

Table 12. Capacity of the UL pose/control stream in different scenarios.

| Scenario | Average number of supported users per cell |
| --- | --- |
| Dense Urban | >15 |
| Urban Macro | >15 |

Observation 12: The capacity result of UL video stream is summarized in Table 13.

Table 13. Capacity of the AR UL video stream in different scenarios

| Scenario | Average number of supported users per cell |
| --- | --- |
| Dense Urban | 8.1 |
| Urban Macro | < 1 |

Observation 13: The capacity result of UL video stream with different PDB is summarized in Table 14.

- When PDB < 30ms, the network capacity increases significantly if the PDB becomes looser.

- When PDB > 30ms, the improvement of network capacity is limited when the PDB becomes looser.

Observation 14: The capacity result of UL video stream with different PER is summarized in Table 14. Relaxing PER can slightly increase the network capacity when PDB = 30ms.

Table 14. Capacity of the AR UL single video stream with different PER and PDB

| (PER, PDB) | Average number of supported users per cell |
| --- | --- |
| (1%, 10ms) | < 1 |
| (1%, 15ms) | 5.4 |
| (1%, 30ms) | 8.1 |
| (1%, 60ms) | 8.3 |
| (5%, 30ms) | 8.3 |
| (10%, 30ms) | 8.4 |

Observation 15: The capacity result of UL pose/control and video stream is summarized in Table 15. If the UE is aware of traffic difference, i.e., the UE prioritizes the transmission of UL pose/control over UL video, the network capacity can be largely increased.

Table 15. Capacity of the UL multi-stream model (pose/control & video)

| S1: UE is not aware of traffic difference | S2: UE is aware of traffic difference | Capacity Improvement of S2 over S1 |
| --- | --- | --- |
| 1.5 | 5.6 | 273.33% |

Observation 16: For DL VR/AR@30Mbps in dense urban, the power saving gains are summarized in Table 17.

Table 17. Power saving gain for dense urban, DL VR/AR@30Mbps

| Power saving scheme | mean PS gain when all UEs are considered | #satisfied UEs per cell / #UEs per cell |
| --- | --- | --- |
| Baseline | - | 4.6/5 |
| C-DRX(10,5,4) | 14.68% | 3.0/5 |
| C-DRX(10,8,4) | 5.53% | 4.4/5 |
| C-DRX(16,8,8) | 10.70% | 0/5 |
| C-DRX(16,14,4) | 3.46% | 4.5/5 |
| Baseline | - | 3.0/3 |
| C-DRX(10,5,4) | 15.24% | 2.4/3 |
| C-DRX(10,8,4) | 5.76% | 2.9/3 |
| C-DRX(16,8,8) | 11.01% | 0/3 |
| C-DRX(16,14,4) | 3.57% | 2.9/3 |

Observation 17: For DL CG@30Mbps in dense urban, the power saving gains are summarized in Table 18 .

Table 18. Power saving gain for dense urban, DL CG@30Mbps

| Power saving scheme | mean PS gain when all UEs are considered | #satisfied UEs per cell / #UEs per cell |
| --- | --- | --- |
| Baseline | - | 6.3/7 |
| C-DRX(10,5,4) | 13.83% | 5.5/7 |
| C-DRX(10,8,4) | 5.26% | 6.3/7 |
| C-DRX(16,8,8) | 9.71% | 5.2/7 |
| C-DRX(16,14,4) | 3.30% | 6.3/7 |
| Baseline | - | 3.0/3 |
| C-DRX(10,5,4) | 15.20% | 3.0/3 |
| C-DRX(10,8,4) | 5.75% | 3.0/3 |
| C-DRX(16,8,8) | 10.79% | 2.9/3 |
| C-DRX(16,14,4) | 3.57% | 3.0/3 |

Observation 18: The 5% points of the coupling gain CDF for DL with Optional methodology 2 (One UE in the entire network) are summarized in Table 19.

Table 19. The 5% points of the coupling gain CDF for DL with Optional methodology 2 (One UE in the entire network).

| Case | 5% points of the Coupling gain |
| --- | --- |
| CG 30Mbps, (1%, 15ms), satisfied UE | -146.88 dB |
| VR/AR 30Mbps, (1%, 10ms), satisfied UE | -145.33 dB |
| VR/AR 45Mbps, (1%, 10ms), satisfied UE | -143.85 dB |

Observation 19: The 5% points of the coupling gain CDF for DL with Optional methodology 1 (B UEs per cell) are summarized in Table 20.

Table 20. The 5% points of the coupling gain CDF for DL with Optional methodology 1 (B UEs per cell).

| Application | 5% points of the Coupling gain (B=1) | 5% points of the Coupling gain (B=Capacity) |
| --- | --- | --- |
| CG 30Mbps, (1%, 15ms), satisfied UE | -137.19 dB | -134.38 dB |
| VR/AR 30Mbps, (1%, 10ms), satisfied UE | -137.19 dB | -132.86 dB |
| VR/AR 45Mbps, (1%, 10ms), satisfied UE | -136.58 dB | -132.95 dB |

Observation 20: The 5% points of the coupling gain CDF for UL with Optional methodology 2 (One UE in the entire network) are summarized in Table 21.

Table 21. The 5% points of the coupling gain CDF for Optional methodology 2 (One UE in the entire network).

| Application | 5% points of the Coupling gain |
| --- | --- |
| AR 10Mbps, (1%,15ms), satisfied UE | -119.85 dB |
| AR 10Mbps, (1%,30ms), satisfied UE | -122.57 dB |
| AR 10Mbps, (1%,60ms), satisfied UE | -123.21 dB |
| VR/CG 0.2Mbps, (1%, 10ms), satisfied UE | -137.81 dB |

Observation 21: The 5% points of the coupling gain CDF for UL with Optional methodology 1 (B UEs per cell, B=1) are summarized in Table 22.

Table 22. The 5% points of the coupling gain CDF for UL with Optional methodology 1 (B UEs per cell, B=1).

| Application | 5% points of the Coupling gain |
| --- | --- |
| AR 10Mbps, (1%,15ms), satisfied UE | -118.91 dB |
| AR 10Mbps, (1%,30ms), satisfied UE | -121.61 dB |
| AR 10Mbps, (1%,60ms), satisfied UE | -122.55 dB |
| VR/CG 0.2Mbps, (1%, 10ms), satisfied UE | -136.01 dB |

# References

Chairman notes, RAN1#104-e, February, 2021.

Chairman notes, RAN1#104b-e, April, 2021.

Chairman notes, RAN1#105-e, May, 2021.

Chairman notes, RAN1#103-e, November, 2020.

Chairman notes, RAN1#106-e, August, 2021.

R1-2108735, “Traffic model and evaluation methodology for XR and Cloud Gaming”, Huawei, HiSilicon, RAN1#106b-e, October 11th – 19th, 2021

R1-2109745, “Challenges and potential enhancements for XR and Cloud Gaming”, Huawei, HiSilicon, RAN1#106b-e, October 11th – 19th, 2021

# Appendix A: System Simulation Parameters

Table A-1. System level simulation assumption for FR1

| Parameter | Value |  |
| --- | --- | --- |
| Scenarios | Scenario-1: Dense Urban | Scenario-2: Urban Macro |
| Layout | 21cells with wraparound |  |
| Channel Model | UMa |  |
| Carrier frequency | 4.0 GHz |  |
| Bandwidth | 100 MHz |  |
| Subcarrier spacing | 30 KHz |  |
| Frame structure | [DDDSU DDDSU] Detailed S slot format is 10D:2F:2U. |  |
| Inter Site Distance | 200m | 500m |
| BS Antenna Height | 25m |  |
| BS Antennas | Option 1: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) |  |
| CSI-RS Port | 32 |  |
| UE Antenna Height | General equation: hUT=3(nfl – 1) + 1.5 - nfl for outdoor UEs: 1 - nfl for indoor UEs: nfl ~ uniform(1, Nfl ) where Nfl ~ uniform(4,8) |  |
| UE Antennas | Baseline: 2T/4R, (M, N, P, Mg, Ng; Mp, Np) = (1,2,2,1,1;1,2), (dH, dV) = (0.5, N/A)λ |  |
| BS antenna pattern | 3-sector antenna radiation pattern, 8 dBi |  |
| UE antenna pattern | Omnidirectional, 0 dBi |  |
| TX Power | BS : 44 dBm per 20MHz | BS : 49 dBm per 20MHz |
| UE MAX Power | 23dBm |  |
| Noise Figure | BS:5 dB, UE:9 dB |  |
| Scheduler | SU-MIMO Proportional Fair/MU-MIMO Proportional Fair |  |
| MCS | Up to 256QAM |  |
| UE distribution | 80% indoor, 20% outdoor |  |
| UE speed | 3 km/h |  |
| Channel Estimation | Realistic |  |
| Downtilt | 12 degree |  |
| CSI acquisition | CSI-RS: Period=5ms, Density=1, feedback delay=1ms, 4bit quantization, and modelling the error SRS: Period=5ms |  |
| Transmission scheme | Close loop rank adaptation |  |
| PHY processing delay | UE PDSCH processing Capability #1 |  |
| PDCCH overhead | 2 symbols |  |
| Target BLER | 10% |  |
| Max HARQ transmission | 4 |  |

Note 1: For frame structure, U symbol of S slot is not used for uplink transmission.